GERAN CIoT Link-to-System Mapping Methodology

(Revision of GPC-150242)

# Introduction:

Link to system mapping is essential for efficiently capturing the performance of the underlying physical layer in the system level simulator (SLS). In this contribution we provide a novel method that can be used for link to system mapping in Cellular IoT systems. The end goal of the proposed method is to provide a BLER curve that is matched to the interference scenario observed in the SLS simulator. This document is an update of [1].

# Effect of interference scenarios on BLER:

The observed interference scenario, i.e., the number of interferers and their received interference powers, has a direct impact on the BLER curve obtained through the link level simulations. Subsequently, this interference scenario will affect the performance observed by the SLS. In particular, for a given modulation coding scheme (MCS) and a particular interference-to-noise ratio (INR) value, different interference scenarios will incur different BLERs. For example, consider an EC-GSM communication scenario that utilizes MCS 2 without repetitions and incurs an INR=4 dB, whereby the noise power is 0dB. For such a scenario, exemplary BLER curves corresponding to different distributions of the interference power among the co-channel interferers (CCIs) are provided in Figure 1. In this figure, the 1-CCI corresponds to the single dominant interferer case, whereas the 18-CCIs correspond to case in which the interference power is divided equally among 18-CCIs. In the same figure, the remaining curves correspond to exemplary co-channel interference scenarios with different interference distributions; e.g., 3 interferers scenario with interference powers, I=[1.8 -2 -4.2]dB;

# Figure 1. Effect of the underlying interference scenario on BLER for EC-GSM MCS2.

In [2], it was shown that for communication systems without interference suppression, the performance of any interference scenario with a given INR that is observed in the SLS is upper and lower bounded by the 1-CCI and M-CCIs corresponding to the same INR, where M is a large integer. In a cellular communication system with a frequency reuse factor=1/3, this M will be approximately equal to 18; i.e., the number of interferers in the first two tiers. An example on these upper and lower bounds for the case of MCS 2 and INR=4dB is shown in Figure 1.

To accurately evaluate the performance of a communication system using system level simulations, each interference scenario must be mapped to a BLER curve for link level performance extraction [3]. The accuracy of this mapping procedure will directly impact the accuracy of the SLS. Hence, to increase this accuracy, in this contribution we provide a two-step link to system mapping method. In the first step, a set of reference interference scenarios curves is selected based on the incurred INR. Subsequently, in the second step, a mapping function is introduced to select the BLER curve corresponding to the nominal interference scenario that closely mimics the performance of the incurred interference scenarios. These two steps are discussed in details in the following sections.

# Selection of nominal interference scenarios:

In our simulations, we made two observations. First the IoT devices are most likely to have a dominant interferer due to the cellular layout; e.g., the frequency reuse factor and the antenna pattern. Second, the INR values and the strength of the dominant interferer are the main factors that affect the BLER curves. Utilizing these two observations, the nominal interference scenarios’ BLER curves were selected such that:

  - They come in sets, each representing a given nominal value of INR, wherein each of the BLER curves within a given set corresponds to a different dominant interference power, but with the same INR value.

  - For a set of BLER curves corresponding to a given INR value, the curves within this set must be uniformly distributed and the gap between any two consecutive curves is less than 0.5 dB.

  - The nominal INR values are selected such that their 1-CCI curves are upper and lower bounded by the sensitivity curve, the zero interference scenario, and the 1-CCI curve corresponding to the highest INR observable in the system, respectively. In addition, the difference between any two consecutive 1-CCI curves should be less than 0.5 dB. Note that for high INR values the BLER curves tend to approximately coincide. For example, in EC-GSM systems, the BLER curves tend to coincide after INR=5dB and thus it is sufficient to consider the lower bound on the INR to be 5dB and map any higher INR values to this bound.

Figure 3. Nominal interference scenarios with 1-CCI for EC-GSM MCS2. Note that in this example, the BLER curves were observed to coincide for INR values >0.5 dB.

# Scenario identification procedure:

Given the infinite possible interference scenarios that can be observed in the SLS, the link-to-system mapping method must include a scenario identification procedure. In particular, a procedure must be provided to match the observed interference scenario in the SLS with one of the nominal pre-selected interference scenarios.  In this proposal, we identify the matching nominal interference scenario that mimics the observed interference scenario in the SLS based on the following values:

INR=kIkN0 , and

DIR=ImaxkIk .

Where DIR is the dominant-interferer ratio, Imax is the dominant interferer power in the linear scale, Ikis the interferer power in the linear domain of the k-th interferer and N0 is the noise power. Based on these two values, a specific BLER performance curve that accurately reflects the observed interference scenario in the SLS is selected. The detailed steps leading to this selection are illustrated in details in the following section.

Figure 4. The nominal interference scenarios BLER curves for MCS4 and INR=4dB.

# Implementation of the proposed link to system mapping method in SLS:

## The proposed link to system mapping method can be implemented as follows:

  - Select the nominal interference scenarios according to the criteria in Section 3 and tabulate their BLER curves.

  - Identify the SINR value, noise power N0 and the interference vector I corresponding to the interference scenario observed in the SLS.

  - Calculate the INR and DIR values for the interference scenario observed in the SLS according to the formulas of section 4.

  - Select the set of BLER curves corresponding to the INR value closest to the one corresponding to the interference scenario observed in the SLS.

  - Select the BLER curve corresponding to the DIR value closest to the one corresponding to the interference scenario observed in the SLS.

  - Utilize the selected curve to obtain the BLER corresponding to the SINR value observed in the SLS.

# Verification of the proposed link to system mapping method:

In this section, we implement our proposed link to system method for a EC-GSM system utilizing MCS2 for verification. For this system, the nominal interference scenarios were selected based on the criteria of Section 3.

Now assuming that we observe an interference scenario in the SLS with interference powers I=[-0.22   -3.23  -11.01  -11.01] dB and N0=0dB. For this interference Scenario, the corresponding INR and DIR values are 2dB and 0.6, respectively. The BLER curves corresponding to this scenario and the selected nominal interference scenario are provided in Figure 5. From this figure, it can be seen that the two curves are relatively close with an error of 0.05dB at 10% BLER. The two curves are presented in Figure 5.

Figure 5. The BLER curves corresponding to the selected reference interference curve and the observed interference scenario in the SLS.

# Effect of adjacent channel interference:

In [4], it was shown that due to filtering, there exists an adjacent channel protection (ACP) of 18dB, 50dB and 58dB from adjacent Interferers with frequency separation of 200 KHz, 400 KHz, and 600 KHz, respectively. This protection can be even higher with the implementation of more advanced receiver types [5]. Since an EC-GSM system is assumed to have a frequency reuse 1/3, i.e., there exists a 600 KHz frequency separation between intra cell bands, intra-cell adjacent channel interference (intra-cell ACI) can be negligible.

# Link to system mapping in the UL:

Since the observations on the downlink interference scenarios are also valid for the uplink, the proposed link to system mapping method can be reused to select the BLER curves for the observed interference scenarios in the UL.

# References:

  - GPC-150227, “GERAN CIoT Link-to-System Mapping Methodology”, Intel Corporation, GERAN#65.

  - Arnold S. Rosenbaum, “Binary PSK Error Probabilities with Multiple Cochannel Interferences”, IEEE Transactions on Communication Technology, Vol. Com-18, No. 3, June 1970.

  - Mikko Saily et al., “GSM/EDGE: Evolution and Performance”, John Wiley & Sons, Inc.

  - 3GPP TS 05.05, “Radio transmission and reception”.

  - GP-150145, “NB M2M- On adjacent channel interference”, Ericsson, GERAN#65.