Ready State DRX for Cellular IoT

(Revision of GPC-150245)

1.	Introduction:

In some cellular IoT (CIoT)  use cases such as industrial CIoT, the devices may transmit/receive a packet very frequently (say every 1 min or 5 min). In such a case, if devices are moved to GMM Standby State after each packet activity, there may be a significant impact on network signalling overhead due to state transitions from Standby to Ready State and vice versa. The issue may become more critical in case of massive IoT devices scenario as thousands of devices may perform such state transitions in the network. One solution is to keep these devices always in Ready state. However, it will result in higher battery consumption. Keeping devices always in Ready state also saves paging resources as paging is limited to a cell in place of paging in all cells of a routing area (RA). This Paging resource saving can be very attractive especially for low or no mobility devices.

In Ready State device moves to packet Idle states after the TBF is completed. After TBF completion, the device monitors all downlink packet common control channels (PCCCHs) for a duration configured by non-DRX Timer. Once non-DRX Timer expires, devices moves to DRX mode of packet Idle state in the Ready state, where device monitors only its own paging group and is reachable with a higher latency depending on paging/DRX cycle. If DL packet arrives in DRX mode of Packet Idle State, network pages the device to move the device to packet transfer state. The device may not need to go through the contention based random access process to move to the data transfer state as the device is known to cell level. BSS can send DL immediate assignment to the device on the device’s group specific paging channel.  If timing advance is not valid, BSS triggers the mobile station to transmit a PACKET CONTROL ACKNOWLEDGEMENT message as four access bursts in the reserved uplink radio block specified by the RRBP (reserved radio block period) field. The PACKET CONTROL ACKNOWLEDGEMENT message (i.e. four radio bursts) is used to derive the timing advance in UL. In case of UL packet arrival, device performs packet access (random network access or channel request) immediately to move to the data transfer state. Frequent UL packet arrivals may cause significant Random access signalling.

In this contribution, we explore Ready State discontinuous reception (RS-DRX) as a possible solution to reduce signalling overhead by reducing state transitions while keeping the device power consumption lower for IoT devices with frequent traffic arrivals.

2.	Ready State with DRX:

A Ready state DRX (RS-DRX) for GERAN based CIoT can be beneficial for specific IoT use cases (such as Industrial IoT) where IoT devices are expected to get frequent traffic. The IoT devices will remain in the Ready State so that network can reach these devices with lower latency. In the proposed solution, IoT devices go to sleep in between data receptions/transmissions in the Ready state in order to save device power. During sleep, the device skips monitoring packet common control channel PCCCHs. The device periodically wakes up for a short duration to check for possible downlink (DL) assignment messages. During the short awake period, the device monitors all DL PCCCHs. When an incoming (DL) packet arrives for a device, network sends a downlink assignment message on any of the PCCCHs in that cell during the awake period of the device. This mechanism is referred as Ready State Discontinuous Reception (RS-DRX) in this contribution. The awake period is called RS-DRX ON duration and the sleep period is called RS-DRX Sleep. The proposed RS-DRX is applicable to EC-GSM as well as other Gb interface (GERAN) based solutions. The following descriptions are presented in reference to EC-GSM.

In case of DL packet arrival during packet Idle state (of Ready State), the device is known to cell level. As a result, the BSS may be able to send DL assignment to the device on the device’s group specific paging channel without asking device to go for the Random access process. If timing advance is not valid in existing packet Idle state (of Ready State), BSS can send an immediate assignment on the group specific paging channel with timing advance update request [3]. For example, BSS may trigger the mobile station to transmit a PACKET CONTROL ACKNOWLEDGEMENT message as four access bursts in the reserved uplink radio block specified by the RRBP (reserved radio block period) field. The reserved block is considered as a one block PACCH allocation. The PACKET CONTROL ACKNOWLEDGEMENT message (i.e. four radio bursts) is used to derive the timing advance in UL. However, scheduling DL assignment on their group paging channels for massive number of devices in the existing packet Idle mode of Ready state can be challenging. RS-DRX may provide higher flexibility to BSS in order to send DL assignment compared to that in existing packet Idle state (of Ready State), as device monitors all PCCCH in RS-DRX during awake period (RS-DRX ON). RS-DRX ON duration can be configured of variable length dynamically by BSS based on the network load.

Observation 1: Ready State DRX (RS-DRX) may provide higher flexibility to send DL assignment for DL packet Transmission compared to that in existing packet Idle state of Ready State.

In case of UL packet arrival during existing packet Idle State (of Ready State), the device performs packet access (on Random access channel which is contention based network access procedure) immediately. If the network access is successful, device gets UL immediate assignment to make packet resource request. In the RS-DRX, device may transmit UL data only during the DRX ON duration to save power. It helps saving device power as well as network signaling e.g., (i) by aggregating the UL packets for RS-DRX Sleep period, (ii) by aggregating the DL and UL packets for the RS-DRX sleep period, and (iii) waking up for DL and UL packet transmissions at the same time.

Observation 2: Ready State DRX (RS-DRX) can save device power as well as network signaling in case of frequent UL traffic.

In the following, we outlined the working details of RS-DRX.

2.1	Ready-State Discontinuous Reception (RS-DRX)

The major steps of the RS-DRX are described in the following steps:

- The IoT device always remains in Ready State by deactivating READY Timer function.

  - Based on TS 23.060 [2], if the timer length is set to all “1”s (binary), the READY timer function is deactivated, i.e. the timer no longer runs and the device remains in READY state.

  - Device can request for deactivating READY Timer to the network during Attach Request.

  - Alternately, when device requests for Ready State DRX (RS-DRX) configuration in the attach request, network deactivate the READY Timer for the device in its response.

- Eliminating Standby State to Ready State transitions saves a lot of network signaling as it reduces number of random access process triggers significantly and also avoids routing area (RA) level flooding of paging messages.

- The device asks for Ready State DRX (RS-DRX) configuration in the attach request through a new field in the attach request message.

  - BSS sends the RS-DRX parameters (RS-DRX ON duration and RS-DRX Sleep duration) in the attach response.

  - The device’s request for RS-DRX may be forwarded to the core network (e.g., SGSN) by the BS so that

    - Network (i.e. Core Network entity such as SGSN) sets READY Timer as deactivated for the device so that the device remains in Ready State with the RS-DRX mechanism running in the Ready state.

    - Network sets legacy non-DRX Timer as zero for the device and sends to the device.

- Upon being configured for RS-DRX, device stays in READY state with RS-DRX running in the READY state to save device power.

  - Although RS-DRX performs better in terms of signaling overhead reduction, the power consumption in RS-DRX Sleep may be slightly higher than the Standby mode sleep. Therefore, if the packet-inter arrival time is longer (e.g., in hours or days), benefits of keeping device always in Ready State with RS-DRX diminish.

- On completion of TBF, device does not start non-DRX Timer. The value of non-DRX Timer is considered 0. The device starts periodic RS-DRX cycles until next establishment of TBF (i.e., until next arrival of LLC PDU).

- After completion of TBF, device goes to RS-DRX ON duration where it monitors all DL PCCCHs to check for DL assignment message.

  - A short wait timer (e.g., smaller value of non-DRX Timer) may be defined such that non-DRX Timer starts after completion of transmission/reception of TBF. The device then moves to RS-DRX ON state on expiry of non-DRX Timer.

Fig. 1. Ready State device behaviors in existing and proposed systems.

  - RS-DRX ON duration is configured relatively very small compared to legacy non-DRX Timer to minimize device’s awake period in order to save device power.

- If no DL assignment message received during RS-DRX ON duration for DL packet arrival or if there was no UL packet arrival during the last RS-DRX Sleep, device goes for sleep for duration equal to RS-DRX Sleep. During this period device does not monitor any PCCCHs. It may also skip other power consuming activities such as serving/neighbor cell measurements.

  - If a DL packet arrives while device is in RS-DRX Sleep, network (such as BSS) stores/buffers the packet and sends DL assignment message in the next RS-DRX ON duration which follows current RS-DRX Sleep.

  - If a UL packet arrives while device is in RS-DRX Sleep, device waits to perform random access process in the next RS-DRX ON duration which follows current RS-DRX Sleep.

- Avoiding longer legacy non-DRX state saves device power as during this period device is always fully awake monitoring all PCCCH.

- RS-DRX ON and RS-DRX Sleep states alternate each other until the next packet (LLC PDU) arrival. A RS-DRX ON and a RS-DRX Sleep constitute a RS-DRX Cycle. Upon arrival of next packet, RS-DRX cycle is terminated and TBF is established. On completion of TBF, RS-DRX cycles start again.

- Fig. 1 presents a typical example showing device states and behaviors in existing and proposed systems.

- If the packet arrival for IoT device is periodic, RS-DRX cycle can be aligned in such a way that device is in DRX ON during or right after the packet arrival. It can be done by selecting DRX cycle in such a way that the packet arrival period is integer multiple of DRX cycle.

  - Fig. 2 shows an example where CIoT packet arrival period is aligned with RS-DRX cycle.

- A RS-DRX Start offset may also be defined to give BSS more flexibility for controlling distribution of RS-DRX ON periods of various devices. After end of TBF, device will wait for RS-DRX Start offset before entering RS-DRX cycle. During RS-DRX Start offset, device may monitor all DL PCCCH.

Fig.2. An Example showing RS-DRX aligned with periodic industrial CIoT packet arrival.

3.	Conclusions:

A Ready State discontinuous reception (RS-DRX) can be introduced in the EC-GSM and other GERAN based CIoT as a possible solution to reduce signaling overhead while keeping the device power consumption lower.  It can be beneficial for specific IoT use cases (such as Industrial IoT) when IoT devices are expected to get frequent traffic. The IoT devices remains in the low device power consumption Ready State so that network can reach these devices with lower latency and lower signaling.

4.	References:

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

3GPP TS 23.060 V13.2.0 (2015-03), General Packet Radio Service (GPRS); Service description; Stage 2 (Release 13)

3GPP TS 44.060 V12.3.0 (2014-11), Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control / Medium Access Control (RLC/MAC) protocol (Release 12).