# EC-GSM – Link-Level Performance of EC-SCH

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

One of the main objectives in FS_IoT_LC is to increase the coverage compared to existing GPRS services. In case of evolving GSM, a straightforward way is to make use of blind repetitions. This is what has been proposed for the Extended Coverage GSM (EC-GSM) concept (earlier known as GSM Evolution), see [2].

In this contribution, the link level performance of EC-SCH with blind repetitions is provided, and it shows that current repetition scheme may not be sufficient to satisfy the SNR requirement and may lead to high memory and complexity requirements.

# EC-SCH burst mapping with blind repetition

Figure 1 Modulating EC-SCH with GMSK using a negative modulation index on odd 51-multiframes.

As Figure 1 shows, total 14 blind repetitions is used for EC-SCH [3], [4]. And one EC-SCH block needs two 51-multiframes, 7 consecutive SCH bursts are located at TS1 of the first 7 frames. The 7 consecutive SCH bursts provide the possibility to perform IQ accumulation to improve the performance as shown in [3].

# Simulations

## Simulation assumptions

Simulations have been run for EC-SCH using different number of blind transmissions.

The simulation assumptions from [3] have been followed.

Some of the parameters are listed in Table 1.

Table 1. Simulation assumptions

| Parameter | Value |
| --- | --- |
| Logical Channel | EC-SCH |
| Channel | TU |
| Mobile speed | 1.2 km/h |
| Band | GSM 900 |
| Impairments | Typical RX and TX. Frequency offset=0,10,30,100,200,300 |
| FEFC | Disable/enable |
| Number of repetitions | 14/28 |
| Blind modulation detection | Real |
| FER requirement | 10% |

In the simulation, different combining schemes, i.e. with and without IQ accumulation, are compared for different number of blind repetitions and different frequency offset (FO) values.

## Results

Figure 2. Performance of EC-SCH with IQ Accumulation and no FEFC

As Figure 2 shows,  without Frequency Offset Estimation and Frequency Compensation (FEFC), the is very sensitive to the residual frequency offset, 5Hz frequency offset can lead 0.5dB degradation, and 30Hz frequency offset can ruin the performance .

Figure 3. Performance of EC-SCH with IQ Accumulation and FEFC

As Figure 3 shows,  FEFC makes the EC-SCH performance is not sensitive to the frequency offset. But there is still around 0.3dB gap to the -6.3dB SNR requirement if use 14 times blind repetitions.

This FEFC will need a well fine-tuned algorithm as shown in [3] and it will need more data memory since it needs to buffer at least 7 EC-SCH burst data.

If one burst need 320 IQ data with a 2X sample rate and the word length is 16 bits, then it will cost 320*2*7*2=8960 Bytes, and if buffer all 14 bursts data, the memory cost will be doubled. This will be a challenge for a very low cost product implementation.

Figure 4. Performance of EC-SCH without IQ Accumulation and FEFC

And as shown in figure 4, the EC-SCH performance is not sensitive to FO any more without IQ accumulation and FEFC, even in the presence of frequency offsets of up to 300Hz. And it need around 28 times blind repetition to achieve the -6.3dB SNR requirement.

# Conclusions

For EC-SCH, it may be difficult to satisfy the -6.3dB SNR requirement with the current 14 blind repetitions, and the link level simulation results show that it is very sensitive to the residual FO, thereby potentially requiring complicated FO estimation and compensation algorithm. Specifically, the need to compensate the FO before the equalizer can lead to high data memory cost, which is disadvantageous to a low cost and complexity implementation.

If IQ Accumulation is not used, 28 blind repetitions can satisfy the -6.3dB SNR requirement.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

GP-140882, “GSM Evolution for cellular IoT – On using blind repetitions”, source Ericsson LM. GERAN#64.

GPC150064, “EC-GSM, EC-SCH design, performance and mapping”, source Ericsson LM. GERAN Ad Hoc #1 on FS_IoT_LC.

GP-150317, “3GPP TR 45.820 V1.0.0 (2015-03)”