EC-GSM - Paging Group Determination

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things  [1] (WI code: FS_IoT_LC)  was approved.

One of the main objectives is to increase the coverage compared to existing GPRS services in which case logical channels need to operate supporting extended coverage, compared to current operation. These logical channels are referred to as EC-channels.

In this contribution the determination of paging groups on the EC-PCH channel is outlined.

Changes compared to the version presented at GERAN Ad Hoc#2 on FS_IoT_LC, see [3], are shown in red font.

# Mapping of paging groups

In order to avoid paging groups corresponding to different coverage classes being  spread out in time, the nominal paging groups supported within a given eDRX cycle for a given device (IMSI) are always identified, irrespective of the coverage class the device belongs to, by using the EC-PCH block associated with DL CC 1 (normal coverage).

To determine the nominal paging group for higher coverage classes, for the same device and eDRX cycle, the EC-PCH block of CCX  is always chosen to contain the EC-PCH block of CC1.

For example, if the EC-PCH resources used on the EC-PCH for coverage class 5 is defined as EC-PCHCC5, then EC-PCHCC1 shall be a subset of EC-PCHCC5.

This is illustrated in table 1 below where an example of how to map the logical EC-PCH channel onto physical resources within the 51-multiframe structure is shown. As can be seen in this example, the nominal paging group for the EC-PCH block for a specific CIoT device in CC1 is shown by black background color, and is mapped onto TDMA frame 21 and 22. As can be seen, the same frames are included in all other EC-PCH blocks used for the nominal paging groups of all higher coverage classes that could be used by the same device for a given eDRX cycle. This example is also illustrated in Figure 1.

For more information on the mapping of logical channels onto physical channels, see [2].

Table 1. Example of EC-PCH blocks used for the nominal paging group of a specific CIoT device for different coverage classes (CCs)

| Channel designation | Dir. | Repeat length TDMA frames | Coverage class (CC) | Interleaved block TDMA frame mapping |
| --- | --- | --- | --- | --- |
| EC-PCH | D | 51 | 1 | B0(19,20),B1(21,22),…,B15(49,50) |
|  |  | 51 | 2 | B0(19,…,22),B1(23,…,26),…,B7(47,…,50) |
|  |  | 51 | 3 | B0(19,…,26),B1(27,…,34),…,B3(43,…,50) |
|  |  | 51 | 4 | B0(19,…,34), B1(35,…,50) |
|  |  | 102 | 5 | B0(19,…,34 + 51N), B1(35,…,50 + 51N), N=0,1 |
|  |  | 204 | 6 | B0(19,…,34 + 51N), B1(35,…,50 + 51N), N=0,1,2,3 |

By this it is ensured that the set of EC-PCH blocks of interest for any given coverage class will always be located within 4 51-multiframes (i.e. the maximum duration of the paging group of the worse coverage class) from the EC-PCH CC1 block

Furthermore, a device changing its coverage class from a higher coverage class to a lower coverage class does not necessarily have to indicate this to the network since it is always ensured that if a network pages a device according to the higher coverage class, the lower coverage class (which the device would monitor if the coverage class was reduced) will always be included in a higher coverage class paging message transmission. Therefore the device will still start reading the paging group according to its old higher coverage class but will be able to read the paging message earlier.

### Paging Group Determination

When sending a paging request to a BSS, the SGSN includes an indication of the eDRX cycle, DL CC and IMSI associated with the target device thereby allowing the BSS to determine the next occurrence of the nominal paging group for that device within its eDRX cycle as follows:

-	N is the number of paging groups corresponding to a given DL CC within a given eDRX cycle and is determined based on EXTENDED_DRX_MFRMS, EC_PCH_BLKS_MFRM, and CC_EC_PCH_BLKS where:

- EXTENDED_DRX_MFRMS is the number of 51-multiframes per eDRX cycle determined as per Table 1 below.

- EC_PCH_BLKS_MFRM indicates the number of EC-PCH blocks (i.e. the number of 2 bursts blocks) per 51-multiframe. For EC-GSM this can be fixed at 16 which is the equivalent of the legacy PCH_BLKS_MFRM parameter indicating 8 PCH blocks per 51-multiframe.

- CC_EC_PCH_BLKS is the number of EC-PCH blocks required for a given DL CC (where the number of blind repetitions required for any given DL CC is pre-defined by the specifications).

-	The set of eDRX cycle lengths identified by Table 2 is selected such that each member of the set occurs an integral number of times within the full TDMA FN space.

-	N  = (EC_PCH_BLKS_MFRM x EXTENDED_DRX_MFRMS)/CC_EC_PCH_BLKS. The EC-PCH CC1 block read by a device using a given eDRX cycle is determined based on where the nominal paging group occurs for DL CC = 1 (i.e. CC_EC_PCH_BLKS = 1)

-	EC-PCH CC1 block = mod (IMSI, N) where N = (16 x EXTENDED_DRX_MFRMS)/1.

Table 2. Set of eDRX Cycles Supported

| eDRX Cycle Value (EXTENDED_DRX) | Target eDRX Cycle Length | Number of 51-MF per eDRX Cycle (EXTENDED_DRX_MFRMS) | eDRX Cycles per TDMA FN Space |
| --- | --- | --- | --- |
| 0000 | ~1.9 seconds | 8 | 6656 |
| 0001 | ~3.8 seconds | 16 | 3328 |
| 0010 | ~7.5 seconds | 32 | 1664 |
| 0011 | ~12.2 seconds | 52 | 1024 |
| 0100 | ~24.5 seconds | 104 | 512 |
| 0101 | ~49 seconds | 208 | 256 |
| 0110 | ~1.63 minutes | 416 | 128 |
| 0111 | ~3.25 minutes | 832 | 64 |
| 1000 | ~6.5 minutes | 1664 | 32 |
| 1001 | ~13 minutes | 3328 | 16 |
| 1010 | ~26 minutes | 6656 | 8 |
| 1011 | ~52 minutes | 13312 | 4 |
| Note 1: 53248 51-multiframes occur with the TDMA FN space (2715648 TDMA frames) Note 2: All remaining EXTENDED_DRX values are reserved |  |  |  |

Example 1:

- IMSI = 00000000  10001001  00110000  00000001 = 4796417 and EXTENDED_DRX_MFRMS = 6656 (i.e. the eDRX cycle ~ 26 minutes)

- N = 16 * 6656 = 106496

- CC1 Nominal Paging Group = mod (IMSI, 106496) = 4097 which occurs in the 4098th EC-PCH block of the eDRX cycle (i.e. in the 2nd EC-PCH block in 51-multiframe #257

- The nominal paging groups associated with other DL CC for the same IMSI and eDRX cycle length are as shown in Figure 1 (e.g. the nominal paging group for DL CC 2 occurs in the 1st and 2nd EC-PCH blocks of 51-multiframe #257)

Figure 1 – Coverage Class Specific Paging Groups per Example 1

As can be seen in Figure 1, using this method for establishing DL CC specific nominal paging groups for a given eDRX cycle ensures that for a given IMSI the nominal paging groups associated with all possible DL CC will fall within 4 51-multiframes of the EC-PCH CC1 block. As such, if a device triggers a cell update e.g. 5 seconds prior to the next occurrence of its nominal paging group the BSS will still be able to send a page in time for it to be received by the device monitoring according to its DL CC incremented by 1 level. With the ability to update its DL CC as late as a few seconds before the next occurrence of its nominal paging group, a device will thereby experience a substantially reduced probability of missing a page due to experiencing DL CC degradation since the last time it performed a RAU procedure.

### Reachability in EC-GSM Idle Mode

Upon completing an uplink transmission where the EC-PUAN indicates all uplink data blocks have been received, an EC-GSM device may be told to enter EC-GSM Idle mode (indicated by the EC-PUAN message). While in EC-GSM Idle mode it remains reachable according to a DRX cycle known as iDRX for the duration of the Ready timer as follows:

- The eDRX cycle length of 32 51-multiframes (~ 7.5 sec) can be used as the iDRX cycle length used in EC-GSM Idle mode for all coverage classes. This will ensure a very limited number of instances of reachability in the interest of battery conservation.

- The nominal paging group corresponding to a CC1 device for the iDRX cycle length is used as the basis for determining the set of EC-PCH blocks actually read by a device of a given coverage class during the iDRX cycle (see Figure 1).

- N is the number of paging groups corresponding to a CC1 device within the iDRX cycle length = 16*32 = 512.

- The nominal paging group corresponding to a CC1 device for a given iDRX cycle = mod (IMSI, N). Considering the example of DEVICE_CC =3, the CC1 row of Figure 1 conceptually indicates which of the N = 512 EC-PCH blocks per iDRX cycle applies to a given device (assuming it was a CC1 device) and the CC3 row of Figure 1 indicates the actual set of EC-PCH blocks comprising the nominal paging group of that CC3 device.

- The length of the Ready timer is indicated using the GPRS Timer IE of the Routing Area Update Accept message as per legacy operation. A device starts the Ready timer upon completing the transmission of an LLC PDU and as such, upon entering EC-GSM Idle mode it makes use of the iDRX mode for whatever time remains for the Ready timer.

- In the event the device does not receive the EC-PUAN and the BSS has not requested device transmission of a Packet Control Ack the device will simply timeout (waiting for an EC-PUAN) and return to EC-GSM Idle mode where it remains reachable once per iDRX cycle for the duration of its Ready timer.

### Reachability in EC-GSM Extended Uplink TBF mode

Upon completing an uplink transmission where the EC-PUAN indicates all uplink data blocks have been received, an EC-GSM device may be told to enter EC-GSM Extended Uplink TBF mode in which case it remains on its assigned EC-PDTCH resources and monitors the DL EC-PACCH (according to its coverage class).

- While in EC-GSM Extended Uplink TBF mode it monitors the downlink according to where EC-PACCH messages can be received based on its coverage class.

- It listens once every 8th instance of a 52-MF starting with the first 52-multiframe after the 52-multiframe in which it received the EC-PUAN + mod(IMSI, 8) 52-multiframes.

- Assuming there is a response to its uplink transmission (e.g. an application layer ack) it receives an EC-Packet Downlink Assignment (EC-PDA) message on the EC-PACCH and then receives the application layer ack on the corresponding downlink TBF.

- If it does not receive a matching EC-PACCH message after N instances of monitoring the EC-PACCH (according to its coverage class) then it shall release the uplink TBF, enter EC-GSM Idle mode and remain reachable once per iDRX cycle (see section 2.1.2) for the duration of its Ready timer. The value of N defaults to “1” unless otherwise indicated by the EC-PUAN.

# Conclusion

In the interest of supporting dynamic coverage class updates whereby the paging periodicity requirement of ensuring 1 page per eDRX cycle is maintained, the paging group determination described above is used for EC-GSM devices. In addition, the BSS is provided with the option of indicating whether a device is to enter EC-GSM Idle mode or Extended Uplink TBF mode when sending an EC-PUAN message confirming reception of all RLC data blocks of an uplink transmission. A corresponding pCR is provided in [3].

# References

- [1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

- [2]	GPC150055, “EC-GSM, Mapping of logical channels onto physical channels”, source Ericsson LM. GERAN Ad Hoc#1 on FS_IoT_LC.

- [3]	GP-150422, “pCR, EC-GSM - Paging Group Determination”, source Ericsson LM. GERAN#66.