# EC-GSM, EC-PDTCH UL system capacity evaluation
(update of GPC150280)

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

One objective of the study is to “Scale to support a massive number of MTC Mobile Stations”. This includes providing sufficient capacity on the traffic channels.

In this contribution the system capacity of EC-GSM is evaluated on system level for the EC-PDTCH UL channel.

This document is an update of GPC150280 (see [1]). The impact of interference from legacy CS users has been investigated.

# Uplink EC-PDTCH

## Coverage classes

There are in total six coverage classes defined. These are also used by the system level simulations. The determination of coverage class is based on the received signal strength. Further, this is assumed to be known by the mobile, i.e. in sensitivity limited scenario the estimation of the coverage class (or in this case signal strength) would be ideal. How sensitive the results are to estimation errors in coverage class is not covered in these simulations.

## Fixed allocation

The EC-GSM concept makes use of the concept of fixed UL allocation. In short, this implies that the device requests a limited number of resources in the random access attempt, and the network schedules resources by the use of EC-AGCH and/or EC-PACCH.

The fixed allocation in the performed system simulations is done in accordance with the EC-GSM concept. This means:

- Users that are in normal coverage will only be allocated one TS per TTI.

- Users in extended coverage performing two blind repetitions will be allocated two TS per TTI.

- Users in extended coverage performing four or more blind repetitions will be allocated four TS per TTI.

Furthermore, a user is not allowed to try to send its report for more than 20 seconds. If the report has not been transferred in 20 seconds the user will be kicked out from the simulation and logged as a “timed out user”. This maximum delay is currently arbitrarily chosen for these set of simulations.

# Simulations

## Simulation assumptions

The system level simulation assumptions in [3] have been followed. Other specific assumptions are shown in Table 1.

### System parameters

Table 1. Simulation assumptions, in addition to [3]

| Parameter | Value |
| --- | --- |
| General |  |
| Number of seeds | 1 |
| Simulation time | 100 s |
| System size | 192 cells |
| Direction | UL |
| Frequency band | 900 MHz |
| Frequency re-use on BCCH layer | 12 |
| BTS antenna diversity | MRC |
| CIoT parameters |  |
| EC-PDTCH timeslots per cell | 4 dedicated(note 1) |
| CIoT arrival rate per cell and second | 1, 5, 6.8(note 2), 9 |
| Fixed UL allocation | On |
| MCSs | MCS-4, MCS-1 |
| Minimum delay between subsequent transmissions (scaled per CC) | 3 radio blocks |
| Chase combining | On |
| Incremental Redundancy | Off |
| Power control | Off |
| IP header compression | Off |
| Device output power | 23, 33 dBm (100%) |
| BPL model(note 3) | 100 % CIoT devices subject to BPL |
| BPL scenario and inter-site correlation | Scenario 1, correlation 0.50 Scenario 2, correlation 0.75 |
| Device timeout | 20 seconds |
| CS parameters |  |
| CS timeslots per cell | 4 dedicated(note1) |
| CS load | 2 E per cell |
| CS output power | Max 33 dBm |
| CS power control | On |
| DTX | On |
| BPL model | CS devices are not subject to BPL |
| NOTE 1: The allocation of the CS and IoT timeslots in the TDMA frame are randomly offset per cell to model an unsynchronized network. NOTE 2: Derived from traffic model in [3]. 6.8 users per cell and second corresponds to the targeted number of devices per sector in the study. NOTE 3: According to model in [3] |  |

### Coverage classes

Table 2 gives the details of the coverage classes.

Table 2. MCS and number of repetitions per coverage class

| Coverage Class | Number of repetitions | MCS |
| --- | --- | --- |
| CC1 | 1 | MCS-4 |
| CC2 | 1 | MCS-1 |
| CC3 | 2 | MCS-1 |
| CC4 | 4 | MCS-1 |
| CC5 | 8 | MCS-1 |
| CC6 | 16 | MCS-1 |

### Control signaling

EC-AGCH and EC-PACCH (PUAN) are modeled in terms of delay but with a BLER of 0 %.

### Circuit switched users

The CS users are used as interferers on the remaining 4 TS on the used UL carrier. The traffic load for CS is 2 Erlang per cell, which gives an average blocking of approximately 13 %. The maximum output power for CS users is 33 dBm in all scenarios. For CS, power control and DTX are used, according to common assumptions, see [5]. No BPL is applied to the CS users.

### Simulated scenarios

Table 3 summarizes the simulated scenarios and clarifies the legends in the figures presented in section 3.2.

Table 3: Simulated scenarios

| Legend text | CIoT output power [dBm] | BPL scenario | BPL inter-site correlation coefficient | CS interferer load [Erlang/cell] | CS max output power [dBm] |
| --- | --- | --- | --- | --- | --- |
| 33 dBm BPL1 CS | 33 | 1 | 0.5 | 2 | 33 |
| 23 dBm BPL1 CS | 23 | 1 | 0.5 | 2 | 33 |
| 33 dBm BPL4 CS | 33 | 2 | 0.75 | 2 | 33 |
| 23 dBm BPL4 CS | 23 | 2 | 0.75 | 2 | 33 |

## Results

The results presented are:

- Delay

  - The delay is defined from successful decoding of the channel request to successful reception of the whole report at the BTS. This includes queuing delay in the scheduler.

  - The results are presented as a CDF of the delay at the target traffic load (6.8 users per cell and second), see Figure 1.

  - In addition the 95th percentile of the delay is plotted against the traffic load, see Figure 2.

  - Failed attempts are not included in the statistics.

- Resource utilization

  - This represents the average amount of EC-PDTCH UL resources required per cell in the system, plotted against the traffic load, see Figure 3.

- Failed attempts

  - This represents the percentage of the attempts that were not successful, i.e. did not manage to get the report through during 20 seconds.

  - The percentage of failed attempts is plotted against the traffic load, see Figure 4.

### Delay CDF

In Figure 1, the CDF over the delay for the traffic load 6.8 users per cell and second is shown. It can be seen that:

- 95 % of the users with 33 dBm output power and BPL scenario 1 with correlation coefficient 0.5 (denoted BPL1 in the legend) have a delay less than 1.8 seconds

- 95 % of the users with 33 dBm output power and for the most aggressive BPL scenario (scenario 2 with correlation coefficient 0.75, denoted BPL4 in the legend) have a delay less than 2.7 seconds.

- For users with output power 23 dBm 95 % of the users have a delay less than 2.9 seconds in BPL scenario 1

- 90 % of the users have a delay less than 7.3 seconds in the most aggressive BPL scenario. 95 % of the users with 23 dBm output power and the most aggressive BPL scenario have a delay less than 10.8 seconds.

Figure 1: CDF of delay for UL reports with 6.8 users per cell-second.

In Figure 2 the 95th percentile is plotted against the traffic load.

Figure 2: 95-percentile of delay for UL reports.

### Resource utilization

Figure 3 shows the resource utilization for the CIoT users against the CIoT traffic load. At the traffic load of 6.8 users per cell and second EC-GSM only utilizes approximately 1.1 time slots on average for 33 dBm output power and BPL scenario 1 for EC-PDCH UL. For 33 dBm output power and BPL scenario 2 the corresponding figure is 1.3 time slots. With 23 dBm output power, simulations show an average of 1.4 time slots for BPL scenario 1 and 2.1 for BPL scenario 2.

Figure 3: Timeslot utilization per cell and second.

### Failed attempts

The percentage of failed attempts (i.e., the report did not get delivered within 20 seconds) is shown in Figure 4. At the traffic load 6.8 users per cell and second, less than 0.01 % do not get their report through for 33 dBm output power. For 23 dBm output power and BPL scenario 1, approximately 0.04 % do not get the report through within 20 seconds. For BPL scenario 2 and only 23 dBm output power, approximately 3.4 % of the users do not get the report through within 20 seconds. The total number of users passing through the system for this simulation is approximately 120 000.

Figure 4: Timed out users per cell and second.

### Summary

The simulation results are summarized in Table 4.

Table 4: Summary of results

| Scenario | 95th percentile delay [s] | Average resource utilization [TS] | Failed attempts |
| --- | --- | --- | --- |
| 33 dBm, BPL scenario 1 (corr. 0.5) | 1.8 | 1.1 | <0.01 % |
| 33 dBm, BPL scenario 2 (corr. 0.75) | 2.7 | 1.3 | <0.01 % |
| 23 dBm, BPL scenario 1 (corr. 0.5) | 2.9 | 1.4 | 0.04 % |
| 23 dBm, BPL scenario 2 (corr. 0.75) | 10.8 | 2.1 | 3.4 % |

# Conclusions

In this contribution the system performance of EC-GSM uplink data traffic channels has been investigated in a mixed traffic scenario (CIoT and legacy CS). Performance at the target CIoT traffic load of 6.8 users per cell and second is summarized in Table 4 above.

E.g., with an MS output power of CIoT devices of 33 dBm in BPL scenario 1, and with interfering CS users with a traffic load of 2 Erlang per cell (giving approximately 13 % blocking):

- The 95th percentile delay is less than 1.8 seconds

- Less than 0.01 % of the reports time out, with a timeout limit of 20 seconds

- Only 1.1 EC-PDTCH UL timeslots per cell are needed on average to support the CIoT traffic

In summary, the results look very promising regarding the EC-PDTCH capacity for EC-GSM.

# References

- GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

- GPC150280, “EC-GSM, EC-PDTCH UL system capacity evaluation”, source Ericsson LM. GERAN Ad Hoc #2 on FS_IoT_LC.

- GP-150387, TR 45.820 Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (Release 13), v1.2.1

- GPC150055, “EC-GSM, Mapping of logical channels onto physical channels”, source Ericsson LM. GERAN Ad Hoc #1 on FS_IoT_LC.

- 3GPP TR 45.914, “Circuit switched voice capacity evolution for GERAN“