Source: 	Qualcomm Incorporated

Title:    	NB-CIoT – Downlink System Level Simulation Results

Agenda:	7.1.5.3.5.5, 7.2.5.3.4.5

Document for:	Discussion

# Introduction

A converged solution [1] built upon NB-M2M and NB-OFDMA candidates [2] was proposed at GERAN#66. The downlink physical layer design of this converged solution, namely, the Narrowband Cellular IoT candidate solution (NB-CIoT), is expanded in [3].

This paper presents the downlink system simulation results and performance metrics for NB-CIoT system and is an update of [7]. Evaluation methodology and traffic models are taken from [2] Section 5 and Annex E respectively.  Details are as follows.

# Simulation model

## Drop and System parameters

Key deployment parameters are summarized in Table 1.

Geometry of the deployment under these parameters is depicted in Figure 1. The cells are not assumed to be synchronous, therefore despite frequency reuse 3 there may be leakage between the neighbouring sectors with disjoint set of subcarriers [4]. The impact of this leakage on DL SINR is also depicted in Figure 1. Specifically, the average DL SINR on the outermost subcarriers – i.e., the subcarriers 14, 16, 31 and 33 that receive the highest ACI from the sectors using adjacent set of subcarriers – is also plotted, assuming fully loaded system.

System configuration is identical to [3]. Specifically, 5 slots per even-numbered subframe are taken by PBCH, 1 slot per subframe is taken by PSCH, 31 slots on 8 subcarriers per odd-numbered subframes are taken by PDCCH, and the remaining is available for PDSCH.  These PDCCH resources are further divided among four MCSs listed in [3, Table 7.3.2.3-3] to yield 16, 24, 12, and 2 PDCCH message locations per frame for PDCCH MCSs 1 to 4 respectively.

Table 1: Simulation parameters

| Parameter | Value |
| --- | --- |
| Simulation area | 19 cell-sites with 3 sectors per site, in 2-tier Hexagonal grid with wrap around |
| ISD | 1732 meters |
| System bandwidth | 1 carrier (200 KHz) |
| OFDM subcarrier spacing | 3.75 KHz; 48 total subcarriers in 180KHz |
| Frequency Reuse | 3; Sets {0,…,14}{16,…,22,24,…31}{33,…,47} |
| BS maximum transmit power per subcarrier | 43dBm – 10*log10(48) |
| UE distribution | Uniform density, varied from 48K to 66K UEs/sector |
| Propagation loss | 120.9 + 37.6log10(dKM); 900 MHz |
| Shadow Fading | Lognormal with σ=8dB; correlation ½ across cell-sites and 1 across sectors of each cell-site |
| Penetration loss | Build Penetration Loss Scenarios 1&2; inter-site correlation coeff. of 0.5 & 0.75, i.i.d across UEs |
| Antenna configuration | DL: 1x1 |
| Antenna pattern | 2D, 65deg H-plane (TR45.914) – 3 sectored |
| Antenna gains | BS: 18dB; UE: -4dB |
| BS cable loss | 3dB |
| Cell association | BS with strongest RSRP |
| MCL for association | 164dB |
| UE Max. TX Power | 23 dBm |
| UE Noise Figure | 5dB |
| BS Noise Figure | 3dB |

Figure 1: Reuse 3 geometry for inter-site BPL correlation of 0.75. Also shown is the average DL SINR for fully loaded system on the “outermost” subcarrier of each sector.

## Traffic model

Instead of a standalone simulation with each traffic type, all DL and UL traffic for its contribution to the load on DL control channels is simulated. Specifically, the traffic in the simulation comprises these two types:

- Mobile Autonomous Reporting (MAR) Periodic UL reports: 80% of UEs send MAR Periodic UL reports albeit with different intensity, as specified in [2, Table E.2-1].  The net intensity in total number of sessions per day per sector, as calculated in [6], is

λMAR= 8.96×NMS,

where NMS is the total UE population per sector. Moreover, 50% of these sessions generate a DL application-layer ACK, for which the latency is measured and reported in the results below.  The other 50% sessions contribute to DL load only through RACH response and UL grants transmitted on PDCCH.

- Network Command (NC): 20% of the UEs are sent NCs by the BS at various intensities. The net intensity of NC sessions is given by,

λNC= 2.24×NMS.

Moreover, 50% of these sessions generate an (application-layer) UL response that further contributes to load on DL control channels. The size of DL NC is fixed at 20 bytes.

No IP header compression is assumed; thus the total header size including CoAP/DTLS/UDP/IP is taken to be 65 bytes.

## MCS adaptation

MCS is selected based on filtered DL SINR and an MCS-specific margin of 0 to 4dB to obtain <10% BLER at first transmission.

# Simulation results

Figure 2 below gives throughput of DL reports as UE population is varied from about 48,000 UEs/sector to 66,000 UEs/sector. Note, the offered load is the number of DL reports generated per hour per cell-site.  DL reports include Network Commands and application-layer DL ACKs (APP-DL-ACKs) associated with MAR periodic UL reports.  As can be seen, the failure rate of DL reports is about 1%. Failed reports are not retransmitted from the application layer (in accordance with the model in [2, Annex E.2.2]).

Figure 2: Throughput of DL reports (NCs and APP-DL-ACKs) for BPL scenario 2. Red curve highlights the load and throughput at the target load of 52,547 UEs/sector. Throughput under BPL scenario 1 (not shown) is identical.

Scheduling and transmission latency of the APP-DL-ACKs is measured at the granularity of 1280ms, which is the standard frame length in NB-CIoT system, interspersed with four PDCCH bundles. Figure 3 gives the CDF of latency at the traffic load generated by 65,000 UEs/sector.  As can be seen, most APP-DL-ACKs are scheduled and complete transmission within 1.28seconds of their generation (generation/arrival at BS of APP-DL-ACKs is assumed instantaneous upon the reception of MAR UL report, in accordance with the model in [2, Annex E.2.2]).

Figure 3: Scheduling and transmission delay incurred by application-layer ACKs associated with MAR periodic UL reports

# Summary

System level simulation results for the DL of NB-CIoT solution are presented. More specifically, DL throughput and latency for the application-layer DL ACKs associated with MAR periodic UL reports are provided.  The results demonstrate that NB-CIoT solution exceeds by a wide margin the requirements stated in [2].

References

- GP-150524, “Proposed convergence of NB M2M and NB-OFDMA clean-slate solutions,” Qualcomm Incorporated, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., Neul Limited, u-blox. GERAN #66, May 2015.

- 3GPP TR 45.820, Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things, Release 13, version 1.3.0.

- GP-150810, “Narrowband CIoT Downlink physical layer design - Miscellaneous Corrections”, Qualcomm Incorporated, Neul Ltd, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., GERAN #67, August 2015.

- GPC150416, “NB-CIoT – Downlink Interference and Link-to-System Mapping,” Qualcomm Incorporated, July 2015.

- GPC150476, “pCR 45.820 NB M2M - Evaluations of Software Update and Reconfiguration,” Huawei Technologies Co., Ltd. , HiSilicon Technologies Co., Ltd., 3GPP TSG GERAN #66, May 2015.

- GPC150477, "NB M2M - System Level Simulation for Capacity and Latency Evaluation (update of GPC150261)," Huawei Technologies Co., Ltd. , HiSilicon Technologies Co., Ltd., 3GPP TSG GERAN #66, May 2015.

- GPC150417, "NB-CIoT – Downlink System Level Simulation Results," Qualcomm Incorporated, 3GPP TSG GERAN Ad-hoc #3, July 2015.