C-UNB Capacity evaluation

-

revision of GP-150715

# Scope

At GERAN#62, a new feasibility study on Cellular Internet of Things (CIoT) was approved [1]. The Cooperative Ultra Narrow Band (C-UNB) is a clean slate solution, whose general concept is described in [2]. C-UNB physical layer and link layer are defined in [3], [4] and [5].

This document evaluates the capacity of C-UNB networks.

It is a revision of GP-150715, where changes are in red.

For the sake of traceability, previous changes coming from a revision of GPC150312 are in green.

# Evaluation methodology

For network capacity evaluation, document [6] sets assumptions on traffic model and MS density and encourages using simulation tools for capacity evaluation. In the present contribution, network capacity is evaluated with an analytical model of the C-UNB radio access technology. Considering the simplicity of the C-UNB RAT, we propose to consider these results to be realistic enough to get preliminary results on the capacity. More detailed results based on simulation model are FFS.

## Traffic model

In [6], the traffic model per MS is defined with two parameters: the number of MAR per day and the distribution of the MAR size.

The number of MAR per day is defined with for values of the inter-arrival time. As there is always a large number of MS in a given cell, we propose to evaluate the network capacity with one "mean type" of MS, implementing a "mean value" of inter-arrival time (i.e. an average number of MAR per day). Table 1 gives the value used in the C-UNB analytical model.

Table 1: Average # of MAR per day used in C-UNB capacity analysis

| ratio | inter-arrival time | equivalent number of MAR per day | average number of MAR per day |
| --- | --- | --- | --- |
| 40% | 1 day | 1 | 11,2 |
| 40% | 2 hours | 12 |  |
| 15% | 1 hour | 24 |  |
| 5% | 30mn | 48 |  |

In [6], the second parameter for defining the traffic model is the size of application data carried by MARs. This size is defined as a Pareto distribution with min value of 20 bytes, max value of 200 bytes and a coefficient of 2.5. This formula is quite complex to evaluate in an analytical model, such as the one we use in C-UNB. Therefore we propose to split the calculation in four parts and made a weighted average at the end:

use four different values of MAR sizes,

evaluate the network capacity with these four MAR sizes

make a weighted average of rough capacity results with the Pareto ratios.

As there is a full decade between min and max values of the application data size, we propose to define the MAR size with a log scale. The four MAR sizes and corresponding Pareto's ratios are given in table 2.

Table 2: MAR sizes and corresponding Pareto ratios
used in network capacity evaluation

| index | MAR size (in bytes) | Pareto ratio |
| --- | --- | --- |
| 1 | 20 | 0,833 |
| 2 | 40 | 0,147 |
| 5 | 100 | 0,015 |
| 10 | 200 | 0,005 |

## Coverage

Annex E of document [6] defines hexagonal tiles for the cellular coverage and the cell site distance of tri-sectorized BTS (see figure 1). In common cellular system, a MS is attached to a given sector; hence the network capacity is evaluated per sector that is hexagon shape.

In the case of C-UNB, the cooperative reception feature requires an MS to be seen by several base stations. In our capacity evaluation, we have enlarged the coverage of a given sector up to the next BTS. Each sector has a diamond shape (see figure 2) giving three overlaps per sector.

Figure 1: Cell site sector area definition (from [6])

Figure 2: C-UNB sector definition (a: theoretical; b: used in analytical model)

## MAR segmentation assumption

Document [6] defines two protocol overheads (65 bytes normal and 29 bytes compressed) for the MARs. As a matter of fact, protocols overheads can be omitted over the C-UNB RAT because they can be generated by MS avatars hosted in C-UNB server (see document [7]). Nevertheless, capacity evaluation is given with and without headers for the sake of comparison.

As MAC-PDUs have limited payload size in C-UNB, table 3 details the number of segments and MAC-PDU size for three assumptions on the header size: 65, 29 and 0 bytes.

Table 3: segmentation with various headers (all size in bytes)

| Assumption | Data size | Application packet size | Segmentation | Number and size of MAC-PDUs |
| --- | --- | --- | --- | --- |
| full header of 65 bytes | 20 | 85 | 3x29 | 3x 46,5 |
|  | 40 | 105 | 4x27 | 4x 44,5 |
|  | 100 | 165 | 6x28 | 6x 45,5 |
|  | 200 | 265 | 9x30 | 9x 47,5 |
| compressed header of 29 bytes | 20 | 49 | 2x25 | 2x 42,5 |
|  | 40 | 69 | 3x23 | 3x 40,5 |
|  | 100 | 129 | 5x26 | 5x 43,5 |
|  | 200 | 229 | 8x29 | 8x 46,5 |
| header not transmitted over the air | 20 | 20 | 1x20 | 1x 37,5 |
|  | 40 | 40 | 2x20 | 2x 37,5 |
|  | 100 | 100 | 4x25 | 4x 42,5 |
|  | 200 | 200 | 7x29 | 8x 46,5 |

## Other assumptions

Document [6] defines other assumptions for the capacity evaluation, such as the ratio of MS transmitting MAR and receiving network command, the protocol overheads and the system level assumptions (annex D).

The specific features of C-UNB radio access technology allow us to simplify some of the assumptions without going against the objectives of the study. The extra assumptions made in the present contribution are:

network capacity is mainly limited by uplink and not by downlink in C-UNB

propagation loss and building penetration loss have no impact on the network capacity because base station can cope with highly dynamic signals.

# Results

Table 4 gives the maximum capacity of a C-UNB network defined as a number of MSs per sector. These figures are computed with:

a repetition rate (i.e. blind repetition) of 2 PHY-PDU per MAC-PDU

a link-level retransmission rate of 10% due to the BLER.

Table 4: C-UNB capacity (preliminary results from analytical model)

| Header | Max number of MS per sector |
| --- | --- |
| full size, 65 bytes | 21 460 |
| compressed, 29 bytes | 34 640 |
| not over the air, 0 byte | 75 900 |

# Conclusion

This contribution deals with capacity evaluation of the C-UNB radio access technology when loaded with MAR traffic. Preliminary results, given by analytical model of the C-UNB RAT, show that the required capacity (i.e. 52 547 MSs per sector) is exceeded when transmission over the air interface is optimized, that is to say, when protocol overhead is not transmitted over the air, but managed by the MS avatars located in the C-UNB server.

Capacity evaluation based on simulation tools is FFS.

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62, 26-29 May'14, Valencia.

[2]	GP-150708, "Cooperative Ultra Narrow Band - General Description", SIGFOX, GERAN meeting #67, 10-14 August 2015, Yin Chuan, China

[3] 	GP-150709, "C-UNB technology for Cellular IoT - Downlink Physical Layer Design", SIGFOX, GERAN meeting #67, 10-14 August 2015, Yin Chuan, China

[4] 	GP-150710, "C-UNB technology for Cellular IoT - Uplink Physical Layer Design", SIGFOX, GERAN meeting #67, 10-14 August 2015, Yin Chuan, China

[5] 	GP-150711, "C-UNB Link Layer Aspects", SIGFOX, GERAN meeting #67, 10-14 August 2015, Yin Chuan, China

[6]	TR 45.820 v1.4.0 (2015-07), "Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things"

[7]	GPC150368, "C-UNB architecture", 3GPP GERAN meeting#66, 25-29 May 2015, Vilnius, Lithuania