3GPP TSG-GERAN #68 	GP-151097

# Anaheim, USA, 16-19 November, 2015			   							(revision of GPE150071)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.002 | CR | 0185 | rev | 2 | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EC-EGPRS, Physical channels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2015-11-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | EC-EGPRS is a feature based on EGPRS that introduces a streamlined protocol implementation, ensuring energy efficient operation and improved security while supporting extended coverage up to [20 dB] compared to GPRS/EGPRS operation. The feature is not yet introduced in the GERAN specifications. Compared to GPE150031, comments received in discussion following the presentation have been taken into account, highlighted in green and marked with “EAB r1” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of specific information regarding physical channel operation for EC-EGPRS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EC-EGPRS feature will not be introduced in the GERAN specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 5.2.5, 5.2.7, 5.6, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 43.013 0135 TS 43.022 0035 TS 43.064 CR 0089,0090,0091,0092 TS 44.018 CR 1025, 1026 TS 44.060 CR 1609, 1610, 1611, 1612 TS 45.001 CR 0079,0080 TS 45.002 CR 0183,0184, 0186 TS 45.003 CR 0135 TS 45.004 CR 0020 TS 45.005 CR 0578 TS 45.008 CR 0626,0627 |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 51010 ... CR ... TS 51021 ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modification

## 4.1	General

The physical resource available to the radio sub-system is an allocation of part of the radio spectrum. This resource is partitioned both in frequency and time. Frequency is partitioned by radio frequency channels (RFCHs) divided into bands as defined in 3GPP TS 45.005. Time is partitioned by timeslots, TDMA frames, and (for COMPACT) time groups and 52-multiframe number as defined in subclause 4.3 of this Technical Specification.

2nd modification

### 5.2.5	Synchronization Burst (SB)

| Bit Number |  | Length | Contents | Definition |
| --- | --- | --- | --- | --- |
| (BN) |  | of field | of field |  |
| 0 | - 2 | 3 | tail bits | (below) |
| 3 | - 41 | 39 | encrypted bits (e0 . e38) | 45.003 |
| 42 | - 105 | 64 | extended training sequence bits | (below) |
| 106 | - 144 | 39 | encrypted bits (e39 .. e77) | 45.003 |
| 145 | - 147 | 3 | tail bits | (below) |
| (148 | - 156 | 8,25 | guard period (bits) | subclause 5.2.8) |

-	where the "tail bits" are defined as modulating bits with states as follows:

(BN0, BN1, BN2) 	= (0, 0, 0)	and

(BN145, BN146, BN147)	= (0, 0, 0)

-	where the "extended training sequence bits" are defined as modulating bits with states as follows:

(BN42, BN43 .. BN105)	=	(1, 0, 1, 1, 1, 0, 0, 1, 0, 1, 1, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 1, 0,
		0, 0, 0, 0, 0, 1, 1, 1, 1, 0, 0, 1, 0, 1, 1, 0, 1, 0, 1, 0, 0, 0, 
		1, 0, 1, 0, 1, 1, 1, 0, 1, 1, 0, 0, 0, 0, 1, 1, 0, 1, 1)

except for CTS synchronization bursts where states are as follows:

(BN42, BN43 .. BN105)	=	(1, 1, 1, 0, 1, 1, 1, 0, 0, 1, 1, 0, 1, 0, 1, 1, 0, 0, 1, 0, 1, 0, 0, 0,

0, 0, 1, 1, 1, 1, 1, 0, 1, 1, 1, 1, 0, 1, 0, 0, 0, 1, 1, 1, 1, 1, 1, 0,

1, 1, 0, 0, 1, 0, 1, 1, 0, 0, 0, 1, 0, 1, 0, 1)

except for COMPACT synchronization bursts furthermore, where states are as follows:

(BN42, BN43 .. BN105)		=		(1, 1, 1, 0, 1, 1, 0, 0, 0, 0, 1, 1, 0, 1, 1, 1, 0, 1, 0, 1, 0, 0, 0, 1, 0, 1,

0, 1, 1, 0, 1, 0, 0, 1, 1, 1, 1, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 1, 1, 0, 1, 0, 0, 1, 1, 1, 0)

except for EC-EGPRS synchronization bursts where states are as follows:

(BN42, BN43 .. BN105)		=				(TBD)

3rd modification

### 5.2.7	Access burst (AB)

| Bit Number |  | Length | Contents | Definition |
| --- | --- | --- | --- | --- |
| (BN) |  | of field | of field |  |
| 0 | - 7 | 8 | extended tail bits | (below) |
| 8 | - 48 | 41 | synch. sequence bits | (below) |
| 49 | - 84 | 36 | encrypted bits (e0..e35) | 45.003 |
| 85 | - 87 | 3 | tail bits | (below) |
| (88 | - 156 | 68,25 | extended guard period (bits) | subclause 5.2.8) |

-	where the "extended tail bits" are defined as modulating bits with the following states:

(BN0, BN1, BN2 .. BN7)	= (0, 0, 1, 1, 1, 0, 1, 0)

-	where the "tail bits" are defined as modulating bits with the following states:

(BN85, BN86, BN87)	= (0, 0, 0)

-	where the "synch. sequence bits" are defined as modulating bits with the following states, unless explicitly stated otherwise (see  3GPP TS 44.060):

(BN8, BN9 .. BN48)	=	(0, 1, 0, 0, 1, 0, 1, 1, 0, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 1, 1, 0, 0,
		 1, 1, 0, 1, 0, 1, 0, 1, 0, 0, 0, 1, 1, 1, 1, 0, 0, 0)

in case alternative training (synchronization) sequence "TS1" is used, the "synch. sequence bits" shall be defined as modulating bits with the following states:

(BN8, BN9 .. BN48)	=	(0, 1, 0, 1, 0, 1, 0, 0, 1, 1, 1, 1, 1, 0, 0, 0, 1, 0, 0, 0, 0, 1, 1,
		 0, 0, 0, 1, 0, 1, 1, 1, 1, 0, 0, 1, 0, 0, 1, 1, 0, 1)

in case alternative training (synchronization) sequence "TS2" is used, the "synch. sequence bits" shall be defined as modulating bits with the following states:

(BN8, BN9 .. BN48)	=	(1, 1, 1, 0, 1, 1, 1, 1, 0, 0, 1, 0, 0, 1, 1, 1, 0, 1, 0, 1, 0, 1, 1,
		 0, 0, 0, 0, 0, 1, 1, 0, 1, 1, 0, 1, 1, 1, 0, 1, 1, 1)

-	in case of EC-EGPRS operation on the RACH or EC-RACH the "synch. sequence bits" shall be defined as one of the following sets of modulating bits depending on the used uplink Coverage Class (CC), and in case of CC1 also supported set of MCSs:

For operation on RACH or EC-RACH:

-	CC1 and MCS-1-4 supported: (BN8, BN9 .. BN48)	=	(TBD)

-	CC1 and MCS-1-9 supported: (BN8, BN9 .. BN48)	=	(TBD)

For operation on EC-RACH:

-	CC2: (BN8, BN9 .. BN48)	=	(TBD)

-	CC3: (BN8, BN9 .. BN48)	=	(TBD)

-	CC4: (BN8, BN9 .. BN48)	=	(TBD)

4th modification

## 5.6	Parameters for channel definition and assignment

### 5.6.1	General

This subclause describes the set of parameters necessary to describe fully the mapping of any logical channel onto a physical channel. These parameters may be divided into general parameters, that are characteristic of a particular base transceiver station, and specific parameters, that are characteristic of a given physical channel.

### 5.6.2	General parameters

These are:

i)	the set of radio frequency channels used in the cell (CA), together with the identification of the BCCH carrier.

ii)	the TDMA frame number (FN), which can be derived from the reduced TDMA frame number (RFN) which is in the form T1, T2, T3', see 3.3.2.

These parameters are broadcast (or derived from parameters broadcast) in the BCCH and SCH.

For COMPACT, these are:

i)	the set of radio frequency channels used in the cell (CA), together with the identification of the COMPACT CPBCCH carrier (primary COMPACT carrier).

ii)	the TDMA frame number (FN), which can be derived from the reduced TDMA frame number (RFN) which is in the form R1 and R2, see 3.3.2.

iii)	the time group number (TG)

These parameters are broadcast (or derived from parameters broadcast) in the COMPACT CPBCCH and CSCH.

For EC-EGPRS, these are:

i)	the set of radio frequency channels used in the cell (CA), together with the identification of the BCCH carrier.

ii)	the TDMA frame number (FN), which can be derived from the reduced TDMA frame number per quarter hyperframe (RFNQH) which is in the form T1', T2', T2'', T3'', see 3.3.2.2.3 and information provided within assignment messages sent on the EC-AGCH (see 3GPP TS 44.018).

These parameters are broadcast (or derived from parameters broadcast) in the EC-BCCH and EC-SCH. or included within assignment messages sent on the EC-AGCH.

### 5.6.3	Specific parameters

These parameters define a particular physical channel in a base transceiver station. They are:

o)	the training sequence Code (TSC);

i)	the timeslot number (TN);

ii)	the mobile radio frequency channel allocation (MA);

iii)	the mobile allocation index offset (MAIO);

iv)	the hopping sequence number (HSN);

v)	the type of logical channel;

vi)	the sub-channel number (SCN).

The last two parameters allow the determination of the frame sequence.

End of modifications