3GPP TSG-GERAN #68	GP-151136

# Anaheim, USA, 16-19 November, 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 44.003 | CR | 00xx | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EC-EGPRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2015-11-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | EC-EGPRS is a feature based on EGPRS that introduces a streamlined protocol implementation, ensuring energy efficient operation and improved security while supporting extended coverage up to [20 dB] compared to GPRS/EGPRS operation. The feature is not yet introduced in the GERAN specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of channel types for EC-EGPRS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EC-EGPRS feature will not be introduced in the GERAN specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6, 7.1.1, 7.1.2, 7.1.3, 8, 9, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 43.013 0135 TS 43.022 0035 TS 43.064 CR 0089,0090,0091,0092 TS 44.018 CR 1025,1026 TS 44.060 CR 1609,1610,1611,1612 TS 45.001 CR 0080 TS 45.002 CR 0183,0184,0185,0186 TS 45.003 CR 0135 TS 45.004 CR 0020 TS 45.005 CR 0578 TS 45.008 CR 0626,0627 |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 51.021 TS 51.010 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modification

# 6	Channel types and their use: Packet data traffic channels

Packet data traffic channels are used to carry a wide variety of information streams, including user information and signalling information for, e.g. Session Management (SM) and Mobility Management (MM) in packet mode. A distinguishing characteristic is that a packet data traffic channel allows a plurality of information streams, associated with different users, to be multiplexed in a pre-emptive and dynamic fashion. Signalling functions between the MS and the BSS are carried out over other types of channels, namely the control channels.

Uni-directional information streams are carried on the packet data traffic channel on an alternate, or simultaneous basis, consistent with the packet data traffic channel carrying capability. The packet data traffic channel uses the radio resources referred to as PDTCH or Extended Coverage PDTCH (EC-PDTCH) (see 3GPP TS 45.002).

2nd modification

# 7	Channel types and their use: Control channels

NOTE:	The term "Dm channel" may be used to refer to the controls channels used by a Mobile Station at a given moment, independently of their type. (The term "Dm channel" in conjunction with the packet control channels shall be avoided.)

Control channels are used to provide all active Mobile Stations with a continuous frame oriented means of communication across the MS-BS interface.

A Mobile Station Channel Configuration contains one or more control channels. These control channels may change in time, with the channel configuration. Access management signalling functions are used to insure the continuity when a change in the control channels occurs.

Control channels are classified by control channel types, which have common characteristics. These control channel types are specified in sub-clause 7.1.

The control channels are primarily intended to carry signalling information for Connection Management (CM), Mobility Management (MM) and Radio Resource (RR) management.

In addition to signalling information control channels may also be used to carry other data, including those relating to Short Message Services (SMS).

## 7.1	Control channel types

### 7.1.1	Broadcast Control Channel

A broadcast control channel is a point-to-multipoint uni-directional control channel, from the fixed sub-system to the Mobile Stations. Broadcast control channels are physically sub-divided into the Broadcast Control CHannel (BCCH) , Packet Broadcast Control CHannel (PBCCH), Compact Packet Broadcast cControl CHannel (CPBCCH),  and Extended Coverage Broadcast Control CHannel (EC-BCCH).

BCCH, PBCCH, CPBCCH and EC-BCCH are intended to broadcast a variety of information to MSs, including information necessary for MS to register in the system (e.g. synchronization data).

BCCH, PBCCH, CPBCCH and EC-BCCH use a protocol specified in Technical Specifications in 3GPP TS 44-Series.

### 7.1.2	Common Control Channel

A common control channel is a point-to-multipoint bi-directional control channel. Common control channels are physically sub-divided into the Common Control CHannel (CCCH), the Packet Common Control CHannel (PCCCH), the Compact Packet Common Control CHannel (CPCCCH), and the Extended Coverage Common Control Channel (EC-CCCH)

CCCH, PCCCH, CPCCCH and EC-CCCH are primarily intended to carry signalling information necessary for access management functions (e.g., allocation of dedicated channels or radio resource on a packet data traffic channel). The CCCH can be used for other signalling purposes.

CCCH, PCCCH, CPCCCH and EC-CCCH use a layered protocol according to Technical Specifications in 3GPP TS 44-Series. In particular the multipoint to point management is achieved through random access techniques.

The following terms may be used when the context requires it:

-	The RACH (Random Access CHannel) is the uplink (MS to network) part of the CCCH.

-	The PRACH (Packet Random Access CHannel) is the uplink part of the PCCCH.

-	The CPRACH (Compact Packet Random Access CHannel) is the uplink part of the CPCCCH.

-	The EC-RACH (Extended Coverage Random Access CHannel) is the uplink part of the EC-CCCH.

-	The AGCH (Access Grant CHannel) is the part of the downlink (network to MS) part of the CCCH reserved for assignment messages.

-	The PAGCH (Packet Access Grant CHannel) is the part of the downlink part of the PCCCH used for assignment messages.

-	The CPAGCH (Compact Packet Access Grant CHannel) is the part of the downlink part of the CPCCCH used for assignment messages.

-	The EC-AGCH (Extended Coverage Access Grant CHannel) is the downlink part of the EC-CCCH used for assignment messages.

-	The NCH (Notification CHannel) is the part of the downlink part of the CCCH reserved for voice group and/or voice broadcast calls notification messages.

-	PCH (Paging CHannel) is the remaining part of the downlink part of the CCCH.

-	PPCH (Packet Paging CHannel) is the remaining part of the downlink part of the PCCCH.

-	CPPCH (Compact Packet Paging CHannel) is the remaining part of the downlink part of the CPCCCH.

-	EC-PCH (Extended Coverage Paging CHannel) is the downlink part of the EC-CCCH used for paging messages.

### 7.1.3	Dedicated Control Channel (DCCH)

A Dedicated Control CHannel (DCCH) is a point-to-point bi-directional or uni-directional control channel.

DCCHs exist with a variety of bit rates.

DCCHs are further classified as follows according to some technical particularities:

A SDCCH (Stand-alone DCCH) is a bi-directional DCCH whose allocation is not linked to the allocation of a TCH. The bit rate of a SDCCH is 598/765 kbit/s.

A FACCH (Fast Associated DCCH) is a bi-directional DCCH obtained by pre-emptive dynamic multiplexing on respectively a TCH/F or a TCH/H channel. The allocation of a FACCH is obviously linked to the allocation of a TCH. The bit rate of a FACCH is 9 200 bit/s or 4 600 bit/s.

A SACCH (Slow Associated DCCH) is either a bi-directional or uni-directional DCCH of rate 115/300 kbit/s or a bi-directional DCCH of rate 299/765 kbit/s. An independent SACCH is always allocated together with a TCH or a SDCCH. The co-allocated TCH and SACCH shall be either both bi-directional or both uni-directional.

NOTE 1:	A Multislot Configuration (described in sub-clause 9) is an example of a case where uni-directional SACCHs may be used.

The terms Bm, or Bm + ACCHs can be used to refer to a Bm channel together with the corresponding FACCH and the co-allocated SACCH when the context avoids any ambiguities. Similar remarks apply to the terms Lm, Lm + ACCHs, Lm + Lm, Lm + Lm + ACCHs. The term SDCCH can be used to refer specifically to a SDCCH together with the co-allocated SACCH when the contexts avoids any ambiguities.

NOTE 2:	TCH/F is sometimes used to designate Bm associated with its control channel (FACCH and SACCH). TCH/H is sometime used to designate Lm associated with its control channel (FACCH and SACCH).

A PACCH (Packet Associated Control CHannel) is a bi-directional DCCH obtained by pre-emptive dynamic multiplexing on a PDTCH.

An EC-PACCH (Extended Coverage Packet Associated Control CHannel) is a bi-directional DCCH obtained by multiplexing on a PDTCH wherein allowed EC-PACCH occasions are determined according to certain restrictions.

A PTCCH (Packet Timing Control CHannel) is a bi-directional DCCH carrying synchronization data for a group of up to 16 MSs in packet transfer state (see 3GPP TS 44.060).

The DCCHs use a layered protocol according to Technical Specifications in 3GPP TS 04- and 05-series.

3rd modification

# 8	BS access capability

The BS access capability is composed of:

one BCCH;

one CCCH physically related to the BCCH;

{{0 to 3 additional CCCHs; and a global resource.

OR:

BCCH, CCCH plus 4 SDCCHs and a global resource.}}

The global resource can be used to accommodate:

i)	n1 (Bm + FACCH + SACCH);

ii)	2n2 (Lm + FACCH + SACCH);

iii)	8n3 (SDCCH of rate 598/765 kbit/s + SACCH);

iv)	n4 (Bm + SACCH);

v)	n5 (PBCCH + PCCCH + PDTCH + PACCH + PTCCH);

vi)	n6 (PCCCH + PDTCH + PACCH + PTCCH);

vii)	n7 (PBCCH + PCCCH); and

viii)	n8 (PDTCH + PACCH + PTCCH);

with the constraints:	n5 = 0 or 1;

n5 > 0 implies that n7 = 0;

n7 > 0 implies that n5 = 0 and n6 = 0; and

n1 + n2 + n3 + n4 + n5 + n6 + n7 + n8 lower than some value characterizing the BS access capability.

The exact use of the global resource may vary in time.

For Compact, the BS access capability is composed of:

CPBCCH;

CPCCCH physically related to the CPBCCH;

{{and a global resource.}}

The global resource can be used to accommodate:

i)	n9 (PDTCH + PACCH + PTCCH);

n9 lower than some value characterizing the BS access capability.

The exact use of the global resources may vary in time.

For EC-EGPRS capable BS, the BS access capability is composed of:

one BCCH and one EC-BCCH;

CCCH physically related to the BCCH;

EC-CCCH physically related to the EC-BCCH;

{{0 to 3 additional CCCHs and/or EC-CCCHs; and a global resource.}}

The global resource can be used to accommodate:

i)	n10 (EC-PDTCH + EC-PACCH);

n10 lower than some value characterizing the BS access capability.

The exact use of the global resources may vary in time.

4th modification

# 9	Channel configurations

At a given moment, a Mobile Station accesses only a limited number of channels appearing on its radio interface. Different compositions for the accessed channels set are identified, and specified below.

Different channel configurations are:

i)	BCCH;

ii)	CCCH;

iii)	CCCH + BCCH;

iv)	SDCCH + SACCH;

v)	Bm + FACCH + SACCH;

vi)	Lm + FACCH + SACCH;

vii)	Lm + Lm + FACCH + SACCH;

viii)	( n + m ) Bm + FACCH + ( n + m ) SACCH;

where n is the number of bi-directional channels and m is the number of uni-directional channels (n = 1..8, m = 0..7, n + m = 1..i);

ix)	PCCCH + PBCCH;

x)	( n + m ) PDTCH + PACCH + PTCCH

where n is the number of channel allowing information streams in both directions and m is the number of channels allowing information streams in one direction (n = 0..8, m = 0..8, n + m = 1..8).

xi)	CPBCCH;

xii)	CPCCCH;

xiii)	CPCCCH + CPBCCH

xiv)	EC-BCCH;

xv)	EC-CCCH;

xvi)	EC-CCCH + EC-BCCH

Configurations i), xi) and xiv)are normally used only in the phase when the physical connection is not set (i.e. just after switch-on, or after a too long interruption of the physical connection due to poor propagation conditions).

Configurations ii) or iii) are used by active but idle MS or MS in packet wait state (see GSM 44.060).

Configurations iv) is used in phases when only a dedicated control channel is needed.

Configurations v) to viii) are used in particular when a circuit-switched communication is in progress.

Configuration viii) is a Multislot Configuration. Possible combinations of bi- and uni-directional channels are defined in 3GPP TS 45.002.

Configurations ix), xii), and xiii) are used by MS in packet wait state.

Configuration x) is a Multislot Configuration on packet data traffic channels. Possible combinations of bi- and uni- directional channels are defined in 3GPP TS 45.002.

In addition, a MS of GPRS MS class A (see 3GPP TS 22.060) may combine one of the configurations i) to viii) (to support GSM circuit switched services and SMS) with one of the configurations ii), iii), ix), or x) (to support GSM GPRS services).

Configurations xiv) or xv) are used by active but idle EC-EGPRS MS or an EC-EGPRS MS in packet wait state.

End of modifications