3GPP TSG-GERAN #68 	GP-151176

# Anaheim, USA, 16-19 November, 2015								     (Revision of GP-151094)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.001 | CR | 0080 | rev | 3 | Current version: | 12.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EC-EGPRS, Coding interleaving and performance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2015-11-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | EC-EGPRS is a feature based on EGPRS that introduces a streamlined protocol implementation, ensuring energy efficient operation and improved security while supporting extended coverage up to [20 dB] compared to GPRS/EGPRS operation. The feature is not yet introduced in the GERAN specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of coding interleaving and performance for EC-EGPRS. Compared to GPE150068 the use of Extended TTI has been removed (see GP-151105), and instead current burst mapping applies also for UL when using blind physical layer repetitions. It can be noted that compared to the previous version of the CR the changes related to ETTI are only removal of text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EC-EGPRS feature will not be introduced in the GERAN specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7,11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 43.013 CR 0003 TS 43.022 CR 0036 TS 43.064 CR 0090,0091, 0092, 0093 TS 44.018 CR 1027, 1026 TS 44.060 CR 1617,1610,1614,1612,1613,1615 TS 45.001 CR 0079, 0081 TS 45.002 CR 0183,0184,0185,0186 TS 45.003 CR 0135 TS 45.004 CR 0020 TS 45.005 CR 0578 TS 45.008 CR 0626,0627 TS 45.010 CR 0065 TS 48.058, CR 0022 |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 51.021 TS 51.010 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Compared to GP-151094, comments received in discussion have been highlighted in green. |  |  |  |  |  |  |  |  |

1st modification

# 7	Coding and interleaving

## 7.1	General

A brief description of the coding schemes that are used for the logical channels mentioned in clause 2, plus the synchronization channel (SCH, see subclause 5.2), is made in the following tables. For all the types of channels the following operations are made in this order:

-	external coding (block coding);

-	internal coding (convolutional or turbo coding);

-	interleaving.

After coding, the different channels (except RACH, SCH, CTSBCH-SB and CTSARCH) are constituted by blocks of coded information bits plus coded header (the purpose of the header is to distinguish between TCH and FACCH blocks). These blocks are interleaved over a number of bursts. The block size and interleaving depth are channel dependent. All these operations are specified in 3GPP TS 45.003.

For the adaptive speech traffic channels, a signaling codeword is attached to the block of coded information bits before interleaving. The signaling codeword is a block code representation of a 2-bits inband information word (rate ¼ for the adaptive full rate speech traffic channels, ½ for the adaptive half rate speech traffic channels, rate 1/6 for the adaptive half rate 8-PSK speech and 8-PSK wideband speech traffic channels, and rate 1/12 for the adaptive full rate 8-PSK wideband speech traffic channels).

| Type of channel | bits/block data+parity+tail1 | convolutional code rate | coded bits per block | interleaving depth |
| --- | --- | --- | --- | --- |
| TCH/FS |  |  | 456 | 8 |
| class I2 | 182 + 3 + 4 | ½ | 378 |  |
| class II | 78 + 0 + 0 | - | 78 |  |
| TCH/EFS |  |  | 456 | 8 |
| class I2 | 170 + 15 + 4 | ½ | 378 |  |
| class II | 74 + 4 + 0 | - | 78 |  |
| TCH/HS |  |  | 228 | 4 |
| class I3 | 95+3+6 | 104/211 | 211 |  |
| class II | 17+0+0 |  | 17 |  |
| TCH/AFS12.24 |  |  | 456 | 8 |
| Class I5 | 244 + 6 + 4 | 127/224 | 448 |  |
| TCH/AFS10.24 |  |  | 456 | 8 |
| Class I6 | 204 + 6 + 4 | 107/224 | 448 |  |
| TCH/AFS7.954 |  |  | 456 | 8 |
| Class I7 | 159 + 6 + 6 | 171/448 | 448 |  |
| TCH/AFS7.44 |  |  | 456 | 8 |
| Class I8 | 148 + 6 + 4 | 79/224 | 448 |  |
| TCH/AFS6.74 |  |  | 456 | 8 |
| Class I9 | 134 + 6 + 4 | 9/28 | 448 |  |
| TCH/AFS5.94 |  |  | 456 | 8 |
| Class I10 | 118 + 6 + 6 | 65/224 | 448 |  |
| TCH/AFS5.154 |  |  | 456 | 8 |
| Class I11 | 103 + 6 + 4 | 113/448 | 448 |  |
| TCH/AFS4.754 |  |  | 456 | 8 |
| Class I12 | 95 + 6 + 6 | 107/448 | 448 |  |
| TCH/AHS7.9513 |  |  | 228 | 4 |
| Class I14 | 123 + 6 + 4 | 133/188 | 188 |  |
| Class II | 36+0+0 |  | 36 |  |
| TCH/AHS7.413 |  |  | 228 | 4 |
| Class I15 | 120 + 6 + 4 | 65/98 | 196 |  |
| Class II | 28+0+0 |  | 28 |  |
| TCH/AHS6.713 |  |  | 228 | 4 |
| Class I16 | 110 + 6 + 4 | 3/5 | 200 |  |
| Class II | 24+0+0 |  | 24 |  |
| TCH/AHS5.913 |  |  | 228 | 4 |
| Class I17 | 102 + 6 + 4 | 7/13 | 208 |  |
| Class II | 16+0+0 |  | 16 |  |
| TCH/AHS5.1513 |  |  | 228 | 4 |
| Class I18 | 91 + 6 + 4 | 101/212 | 212 |  |
| Class II | 12+0+0 |  | 12 |  |
| TCH/AHS4.7513 |  |  | 228 | 4 |
| Class I19 | 83 + 6 + 6 | 95/212 | 212 |  |
| Class II | 12+0+0 |  | 12 |  |
| (continued) |  |  |  |  |

| (continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type of channel | bits/block data+parity+tail1 | convolutional code rate | coded bits per block | interleaving depth |
| TCH/WFS12.654 |  |  | 456 | 8 |
| Class I23 | 253 + 6 + 4 | 263/448 | 448 |  |
| TCH/WFS8.854 |  |  | 456 | 8 |
| Class I24 | 177 + 6 + 4 | 187/448 | 448 |  |
| TCH/WFS6.64 |  |  | 456 | 8 |
| Class I25 | 132 + 8 + 4 | 9/28 | 448 |  |
| O-TCH/WFS23.8527 |  |  | 1368 | 8 |
| Class I23 | 477 + 6 + 6 | 163/448 | 1344 |  |
| O-TCH/WFS15.8527 |  |  | 1368 | 8 |
| Class I23 | 317 + 6 + 6 | 47/192 | 1344 |  |
| O-TCH/WFS12.6527 |  |  | 1368 | 8 |
| Class I23 | 253 + 6 + 6 | 14/71 | 1344 |  |
| O-TCH/WFS8.8527 |  |  | 1368 | 8 |
| Class I24 | 177 + 6 + 6 | 9/64 | 1344 |  |
| O-TCH/WFS6.627 |  |  | 1368 | 8 |
| Class I28 | 132 + 6 + 6 | 3/28 | 1344 |  |
| O-TCH/WHS12.6526 |  |  | 684 | 4 |
| Class I23 | 253 + 6 + 6 | 265/672 | 672 |  |
| O-TCH/WHS8.8526 |  |  | 684 | 4 |
| Class I24 | 177 + 6 + 6 | 9/32 | 672 |  |
| O-TCH/WHS6.626 |  |  | 684 | 4 |
| Class I28 | 132 + 6 + 6 | 3/14 | 672 |  |
| O-TCH/AHS12.226 |  |  | 684 | 4 |
| Class I5 | 244 + 6 + 6 | 8/21 | 672 |  |
| O-TCH/AHS10.226 |  |  | 684 | 4 |
| Class I6 | 204 + 6 + 6 | 9/28 | 672 |  |
| O-TCH/AHS7.9526 |  |  | 684 | 4 |
| Class I7 | 159 + 6 + 6 | 57/224 | 672 |  |
| O-TCH/AHS7.426 |  |  | 684 | 4 |
| Class I8 | 148 + 6 + 6 | 5/21 | 672 |  |
| O-TCH/AHS6.726 |  |  | 684 | 4 |
| Class I9 | 134 + 6 + 6 | 73/336 | 672 |  |
| O-TCH/AHS5.926 |  |  | 684 | 4 |
| Class I10 | 118 + 6 + 6 | 65/336 | 672 |  |
| O-TCH/AHS5.1526 |  |  | 684 | 4 |
| Class I11 | 103 + 6 + 6 | 115/672 | 672 |  |
| O-TCH/AHS4.7526 |  |  | 684 | 4 |
| Class I12 | 95 + 6 + 6 | 107/672 | 672 |  |
| TCH/F14.4 TCH/F9.6 | 290 + 0 + 4 4*60 + 0 + 4 | 294/456 244/456 | 294/456 456 | 19 19 |
| TCH/F4.8 | 60 + 0 + 16 | 1/3 | 228 | 19 |
| TCH/H4.8 | 4*60 + 0 + 4 | 244/456 | 456 | 19 |
| TCH/F2.4 | 72 + 0 + 4 | 1/6 | 456 | 8 |
| TCH/H2.4 | 72 + 0 + 4 | 1/3 | 228 | 19 |
| (continued) |  |  |  |  |

| (continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type of channel | bits/block data+parity+tail1 | convolutional code rate | coded bits per block | interleaving depth |
| FACCH/F | 184 + 40 + 4 | ½ | 456 | 8 |
| E-FACCH/F | 184 + 40 + 4 | ½ | 456 | 4 |
| FACCH/H | 184 + 40 + 4 | ½ | 456 | 6 |
| O-FACCH/F | 184 + 40 + 6 | 1/6 | 1368 | 8 |
| O-FACCH/H | 184 + 40 + 6 | 1/6 | 1368 | 6 |
| SDCCHs,SACCHs20,BCCH, EC-BCCH, NCH, AGCH, PCH,CBCH | 184 + 40 + 4 | ½ | 456 | 4 |
| EC-AGCH, EC-PCH | 88+18+0 | 106/116 | 116 | 1 |
| EC-PACCH, downlink | 80+18+0 | 98/114 | 114 | 1 |
| EC-PACCH, uplink | 64+18+0 | 82/116 | 116 | 1 |
| SACCH/TP SACCH/MP | 184 + 18 + 6 + 4021 | ½ | 456 | 4 |
| E-IACCH | 3 | 1/822 | 24 | 4 |
| EPCCH | 3 | ¼22 | 12 | 1 |
| RACH | 8 + 6 + 4 | ½ | 36 | 1 |
| EC-SCH | 27 + 10 + 4 | ½ | 78 | 1 |
| SCH | 25 + 10 + 4 | ½ | 78 | 1 |
| CTSBCH-SB | 25 + 10 + 4 | ½ | 78 | 1 |
| CTSPCH | 184 + 40 + 4 | ½ | 456 | 4 |
| CTSARCH | 25 + 10 + 4 | ½ | 78 | 1 |
| CTSAGCH | 184 + 40 + 4 | ½ | 456 | 4 |
| (continued) |  |  |  |  |

(continued)

NOTE 1: The tail bits mentioned here are the tail bits of the convolutional code.

NOTE 2: The 3 parity bits for TCH/FS detect an error on 50 bits of class I.

NOTE 3: The 3 parity bits for TCH/HS detect an error on 22 bits of class I.

NOTE 4: For TCH/AFS and TCH/WFS an 8 bits in band signalling codeword is attached to the block of coded information before interleaving. A dedicated block structure to carry the comfort noise information associated with the adaptive full rate speech traffic channels is also specified in 3GPP TS 45.003.

NOTE 5: The 6 parity bits for TCH/AFS12.2 and O-TCH/AHS12.2 detect an error on 81 bits of class I.

NOTE 6: The 6 parity bits for TCH/AFS10.2 and O-TCH/AHS10.2 detect an error on 65 bits of class I.

NOTE 7: The 6 parity bits for TCH/AFS7.95 and O-TCH/AHS7.95 detect an error on 75 bits of class I.

NOTE 8: The 6 parity bits for TCH/AFS7.4 and O-TCH/AHS7.4 detect an error on 61 bits of class I.

NOTE 9: The 6 parity bits for TCH/AFS6.7 and O-TCH/AHS6.7 detect an error on 55 bits of class I.

NOTE 10: The 6 parity bits for TCH/AFS5.9 and O-TCH/AHS5.9 detect an error on 55 bits of class I.

NOTE 11: The 6 parity bits for TCH/AFS5.15 and O-TCH/AHS5.15 detect an error on 49 bits of class I.

NOTE 12: The 6 parity bits for TCH/AFS4.75 and O-TCH/AHS4.75 detect an error on 39 bits of class I.

NOTE 13: For TCH/AHS a 4 bits in band signalling codeword is attached to the block of coded information before interleaving A dedicated block structure to carry the comfort noise information associated with the adaptive half rate speech traffic channels is also specified in 3GPP TS 45.003.

NOTE 14: The 6 parity bits for TCH/AHS7.95 detect an error on 67 bits of class I.

NOTE 15: The 6 parity bits for TCH/AHS7.4 detect an error on 61 bits of class I.

NOTE 16: The 6 parity bits for TCH/AHS6.7 detect an error on 55 bits of class I.

NOTE 17: The 6 parity bits for TCH/AHS5.9 detect an error on 55 bits of class I.

NOTE 18: The 6 parity bits for TCH/AHS5.15 detect an error on 49 bits of class I.

NOTE 19: The 6 parity bits for TCH/AHS4.75 detect an error on 39 bits of class I.

NOTE 20: with the exception of SACCH/TP and SACCH/MP

NOTE 21: 40 uncoded dummy bits are inserted for the mapping of the enhanced power control signalling

NOTE 22: block code is applied

NOTE 23: The 6 parity bits for TCH/WFS12.65, O-TCH/WFS23.85, O-TCH/WFS15.85, O-TCH/WFS12.65 and O-TCH/WHS12.65 detect an error on 72 bits of class I.

NOTE 24: The 6 parity bits for TCH/WFS8.85, O-TCH/WFS8.85 and O-TCH/WHS8.85 detect an error on 64 bits of class I.

NOTE 25: The 8 parity bits for TCH/WFS6.60 detect an error on 54 bits of class I.

NOTE 27: For O-TCH/WFS a 24 bits in band signalling codeword is attached to the block of coded information before interleaving. A dedicated block structure to carry the comfort noise information associated with the adaptive full rate 8PSK wideband speech traffic channels is also specified in 3GPP TS 45.003.

NOTE 28: The 6 parity bits for O-TCH/WFS6.6 and O-TCH/WHS6.6 detect an error on 54 bits of class I.

| Type of channel | bits/block data+parity+tail1 | Reed-Solomon code rate | convolutional code rate | coded bits per block | interleaving depth |
| --- | --- | --- | --- | --- | --- |
| E-TCH/F43.2 E-TCH/F32.0 E-TCH/F28.8 | 870 + 0 + 6 640 + 0 + 6 580 + 0 + 6 | N/A N/A 73/85 | 876/1368 646/1392 686/1368 | 1368 1392 1368 | 19 12 19 |

## 7.2	Packet Traffic and Control Channels

All packet traffic and control channels, except PRACH, use rectangular interleaving of one Radio Block over four bursts in consecutive TDMA frames of one PDCH. As an exception, in RTTI configuration, the PDTCH uses rectangular interleaving of one Radio Block over four bursts in two consecutive TDMA frames of a PDCH-pair.

### 7.2.1	Channel coding for PDTCH

#### 7.2.1.1	Channel coding for GPRS PDTCH

Four different coding schemes, CS-1 to CS-4, are defined for the GPRS Radio Blocks carrying RLC data blocks. For the Radio Blocks carrying RLC/MAC Control blocks code CS-1 is always used. The exceptions are messages that use the existing Access Burst (see 3GPP TS 45.003, e.g. Packet Channel Request). An additional coding scheme is defined for the Access Burst that includes 11 information bits.

The first step of the coding procedure is to add a Block Check Sequence (BCS) for error detection. For CS-1 - CS-3, the second step consists of pre-coding USF (except for CS-1), adding four tail bits and a convolutional coding for error correction that is punctured to give the desired coding rate. For CS-4 there is no coding for error correction.

The details of the codes are shown in the table below, including:

-	the length of each field;

-	the number of coded bits (after adding tail bits and convolutional coding);

-	the number of punctured bits;

-	the data rate, including the RLC header and RLC information.

| Scheme | Code rate | USF | Pre-coded USF | Radio Block excl. USF and BCS | BCS | Tail | Coded bits | Punctured bits |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS-1 | ½ | 3 | 3 | 181 | 40 | 4 | 456 | 0 |
| CS-2 | »2/3 | 3 | 6 | 268 | 16 | 4 | 588 | 132 |
| CS-3 | »3/4 | 3 | 6 | 312 | 16 | 4 | 676 | 220 |
| CS-4 | 1 | 3 | 12 | 428 | 16 | - | 456 | - |

CS-1 is the same coding scheme as specified for SDCCH. It consists of a half rate convolutional code for FEC and a 40 bit FIRE code for BCS (and optionally FEC). CS-2 and CS-3 are punctured versions of the same half rate convolutional code as CS-1 for FEC. CS-4 has no FEC.

The USF has 8 states, which are represented by a binary 3 bit field in the MAC Header.

All coding schemes are mandatory for MSs supporting GPRS. Only CS-1 is mandatory for the network.

#### 7.2.1.2	Channel coding for EGPRS and EGPRS2 PDTCH

Nine different modulation and coding schemes, MCS-1 to MCS-9, are defined for the EGPRS Radio Blocks (4 bursts, 20ms in case of BTTI configuration or 10ms in case of RTTI configuration) carrying RLC data blocks. Further, 16 more modulation and coding schemes, DAS-5 to DAS-12 and DBS-5 to DBS-12, are defined for EGPRS2 downlink radio blocks (4 bursts, 20ms in case of BTTI configuration or 10ms in case of RTTI configuration) carrying RLC data blocks and 13 more modulation and coding schemes, UAS-7 to UAS-11 and UBS-5 to UBS-12, are defined for EGPRS2 uplink radio blocks (4 bursts, 20ms in case of BTTI configuration or 10ms in case of RTTI configuration) carrying RLC data blocks. For the Radio Blocks carrying RLC/MAC Control blocks code CS-1 is always used, except in the case of RTTI configuration, when code MCS-0 may be used in the downlink (see subclause 7.2.2). The exceptions are messages that use the existing Access Burst (see 3GPP TS 45.003, e.g. Packet Channel Request). An additional coding scheme is defined for the Access Burst that includes 11 information bits.

To ensure strong header protection, the header part of the Radio Block is independently coded from the data part of the Radio Block (8 bit CRC calculated over the header -excl. USF- for error detection, followed by rate 1/3 convolutional coding – and possibly puncturing – for error correction). Additionally, for transmission using FANR, a PAN field is included which is independently coded from the header and data parts of the Radio Block (the coding is the same as for the header part).

For EGPRS (see 3GPP TS 44.060), the MCSs are divided into different families A, B and C. Each family has a different basic unit of payload (see 3GPP TS  43.064). Different code rates within a family are achieved by transmitting a different number of payload units within one Radio Block. For families A and B, 1, 2 or 4 payload units are transmitted, for family C, only 1 or 2 payload units are transmitted.

For EGPRS2 (see 3GPP TS 44.060), the coding schemes are divided into different families A, B and C. Each family has a different basic unit of payload (see 3GPP TS 43.064). Different code rates within a family are achieved by transmitting a different number of payload units within one Radio Block. For family A, 1, 2, 4, 6 or 8 payload units are transmitted, for family B, 1, 2, 3, 4, 6 or 9 payload units are transmitted, for family C, only 1 or 2 payload units are transmitted.

For EGPRS, when 4 payload units are transmitted (MCS-7, MCS-8 and MCS-9), these are splitted into two separate RLC blocks (i.e. with separate sequence numbers and block check sequences).

For EGPRS2, when 4 payload units are transmitted (UAS-7 to UAS-9, UBS-7, UBS-8, DAS-8, DAS-9, DBS-7 and DBS-8), these are split into two separate RLC blocks (i.e. with separate sequence numbers and block check sequences). When 6 payload units are transmitted (UAS-10, UAS-11, UBS-9, UBS-10, DAS-10, DAS-11, DBS-9 and DBS-10), these are split into three separate RLC blocks (i.e. with separate sequence numbers and block check sequences), except for DAS-10 for which they are split into two separate RLC blocks. When 8 payload units are transmitted (DBS-11, DBS-12, UBS-11 and UBS-12), these are split into four separate RLC blocks (i.e. with separate sequence numbers and block check sequences). When 9 payload units are transmitted (DAS-12), these are split into three separate RLC blocks (i.e. with separate sequence numbers and block check sequences).

# The first step of the coding procedure is to add a Block Check Sequence (BCS) for error detection.

In case of convolutional coding, the second step consists of adding six tail bits (TB) and a 1/3 rate convolutional coding for error correction that is punctured to give the desired coding rate.

In case of turbo coding, the second step consists of 1/3 rate turbo coding for error correction (see 3GPP TS 45.003) that is punctured to give the desired coding rate.

The USF has 8 states, which are represented by a binary 3 bit field in the MAC Header. The USF is encoded to 12 symbols for bursts using legacy symbol rate and 16 symbols for bursts using higher symbol rate, (this results in12 bits for GMSK modes, 32 bits for QPSK modes, 36 bits for 8PSK modes, 48 bits for 16-QAM modes at legacy symbol rate (See 3GPP TS 45.004), 64 bits for 16-QAM at higher symbol rate (see 3GPP TS 45.004), 60 bits for 32-QAM at legacy symbol rate and 80 bits for 32-QAM at higher symbol rate).

MSs supporting EGPRS shall support MCS-1 to MCS-9 in downlink and MCS-1 to MCS-4 in uplink. In case an MS supporting EGPRS is 8-PSK capable in uplink, it shall also support MCS-5 to MCS-9 in uplink. A network supporting EGPRS may support only some of the MCSs.

MSs supporting EGPRS2 (see 3GPP TS 44.060) shall support EGPRS coding schemes MCS-1 to MCS-9 in downlink and uplink.

MSs supporting EGPRS2-A (see 3GPP TS 44.060) in the downlink shall support DAS-5 to DAS-12.

MSs supporting EGPRS2-B (see 3GPP TS 44.060) in the downlink shall support DAS-5 to DAS-12 and DBS-5 to DBS-12.

MSs supporting EGPRS2-A in the uplink shall support UAS-7 to UAS-11.

MSs supporting EGPRS2-B in the uplink shall support UAS-7 to UAS-11 and UBS-5 to UBS-12.

A network supporting EGPRS2-A in downlink (respectively uplink) may support only some of DAS-5 to DAS-12 (respectively UAS-7 to UAS-11).

A network supporting EGPRS2-B in downlink (respectively uplink) may support only some of DBS-5 to DBS-12 (respectively UBS-5 to UBS-12).

The details of the EGPRS coding schemes are shown in Table 7.2.1.2.1. An exhaustive description of the EGPRS coding schemes can be found in 3GPP TS 45.003.

# Table 7.2.1.2.1 - Coding parameters for the EGPRS coding schemes

| Scheme | Code rate (Note 2) | Header Code rate | PAN Code rate (if present) | Modulation | RLC blocks per Radio Block (20ms) | Raw Data within one Radio Block (Note 1) | Family | BCS | Tail payload | HCS | PCS (if present) | Data rate kb/s (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS-9 | 1,0 | 0,36 | n/a | 8PSK | 2 | 2x592 | A | 2x12 | 2x6 | 8 | 10 | 59,2 |
| MCS-8 | 0,92 (0,98) | 0,36 | 0,42 |  | 2 | 2x544 | A |  |  |  |  | 54,4 |
| MCS-7 | 0,76 (0,81) | 0,36 | 0,42 |  | 2 | 2x448 | B |  |  |  |  | 44,8 |
| MCS-6 | 0,49 (0,52) | 1/3 | 0,39 |  | 1 | 592 48+544 | A | 12 | 6 |  |  | 29,6 27,2 |
| MCS-5 | 0,37 (0,40) | 1/3 | 0,39 |  | 1 | 448 | B |  |  |  |  | 22,4 |
| MCS-4 | 1,0 | 0,53 | n/a | GMSK | 1 | 352 | C |  |  |  |  | 17,6 |
| MCS-3 | 0,85 (0,96) | 0,53 | 0,63 |  | 1 | 296 48+248 and 296 | A |  |  |  |  | 14,8 13,6 |
| MCS-2 | 0,66 (0,75) | 0,53 | 0,63 |  | 1 | 224 | B |  |  |  |  | 11,2 |
| MCS-1 | 0,53 (0,60) | 0,53 | 0,63 |  | 1 | 176 | C |  |  |  |  | 8,8 |
| Note 1: The italic caption indicates the 6 octets of padding when retransmitting MCS-8 block with MCS-3 or MCS-6. For MCS-3, the 6 octets of padding are sent every second block (see 3GPP TS 44.060). Note 2: The number in bracket indicates the coding rate for transmission using FANR, when the PAN is present. Note 3: These data rates are applicable for BTTI configuration. The data rates are doubled in case of RTTI configuration. |  |  |  |  |  |  |  |  |  |  |  |  |

The details of the EGPRS2 coding schemes are shown in the table below. An exhaustive description of the EGPRS2 coding schemes can be found in 3GPP TS 45.003.

# Table 7.2.1.2.2: Coding parameters for the EGPRS2 coding schemes

| Scheme | Code rate (Note 5) | Header Code rate | PAN code rate (if present) | Modulation | RLC blocks per Radio Block (20ms) | Raw Data within one Radio Block | Family | BCS | Tail payload | HCS | PCS | Data rate kb/s (Note 6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAS-12 | 0,96 (0,99) | 0,38 | 0.38 | 32QAM (Note 4) | 3 | 658 | B | 3x12 | (Note 7) | 8 | 10 | 98,4 |
| DAS-11 | 0,80 (0,83) | 038 | 0.38 |  | 3 | 546 | A | 3x12 | (Note 7) |  |  | 81,6 |
| DAS-10 | 0,64 (0,66) | 0,33 | 0.38 |  | 2 | 658 | B | 2x12 | (Note 7) |  |  | 65,6 |
| DAS-9 | 0,68 (0,70) | 0,34 | 0.38 | 16QAM (Note 4) | 2 | 546 | A | 2x12 | (Note 7) |  |  | 54,4 |
| DAS-8 | 0,56 (0,58) | 0,34 | 0.38 |  | 2 | 450 | B | 2x12 | (Note 7) |  |  | 44,8 |
| DAS-7 | 0,54 (0,57) | 033 | 0.38 | 8PSK (Note 4) | 1 | 658 | B | 12 | (Note 7) |  |  | 32,8 |
| DAS-6 | 0,45 (0,48) | 0,33 | 0.38 |  | 1 | 546 | A | 12 | (Note 7) |  |  | 27,2 |
| DAS-5 | 0,37 (0,39) | 0,33 | 0.38 |  | 1 | 450 | B | 12 | (Note 7) |  |  | 22,4 |
| DBS-12 | 0,98 (1,00) | 0,37 | 0.54 | 32QAM (Note 3) | 4 | 594 | A | 4x12 | (Note 7) |  |  | 118,4 |
| DBS-11 | 0,91 (0,93) | 0,37 | 0.38 |  | 4 | 546 | A | 4x12 | (Note 7) |  |  | 108,8 |
| DBS-10 | 0,72 (0,75) | 0,34 | 0.38 |  | 3 | 594 | A | 3x12 | (Note 7) |  |  | 88,8 |
| DBS-9 | 0,71 (0,.73) | 0,34 | 0.38 | 16QAM (Note 3) | 3 | 450 | B | 3x12 | (Note 7) |  |  | 67,2 |
| DBS-8 | 0,60 (0.63) | 0,31 | 0.38 |  | 2 | 594 | A | 2x12 | (Note 7) |  |  | 59,2 |
| DBS-7 | 0,47 (0,48) | 0,31 | 0.38 |  | 2 | 450 | B | 2x12 | (Note 7) |  |  | 44,8 |
| DBS-6 | 0,63 (069) | 0,31 | 0.38 | QPSK (Note 3) | 1 | 594 | A | 1x12 | (Note 7) |  |  | 29,6 |
| DBS-5 | 0,49 (0,53) | 0,31 | 0.38 |  | 1 | 450 | B | 1x12 | (Note 7) |  |  | 22,4 |
| UAS-11 | 0,95 (0,99) | 0,36 | 0.38 | 16QAM (Note 2) | 3 | 514 | A | 3x12 | 3x6 |  |  | 76,8 |
| UAS-10 | 0,84 (0,87 | 0,36 | 0.38 |  | 3 | 450 | B | 3x12 | 3x6 |  |  | 67,2 |
| UAS-9 | 0,71 (0,74) | 0,36 | 0.38 |  | 2 | 594 | A | 2x12 | 2x6 |  |  | 59,2 |
| UAS-8 | 0,62 (0,64) | 0,36 | 0.38 |  | 2 | 514 | A | 2x12 | 2x6 |  |  | 51,2 |
| UAS-7 | 0,55 (0,57) | 0,36 | 0.38 |  | 2 | 450 | B | 2x12 | 2x6 |  |  | 44,8 |
| UBS-12 | 0,96 (0,99) | 0,35 | 0.38 | 32QAM (Note 1) | 4 | 594 | A | 4x12 | 4x6 |  |  | 118,4 |
| UBS-11 | 0,89 (0,91) | 0,35 | 0.38 |  | 4 | 546 | A | 4x12 | 4x6 |  |  | 108,8 |
| UBS-10 | 0,71 (0,73) | 0,35 | 0.36 |  | 3 | 594 | A | 3x12 | 3x6 |  |  | 88,8 |
| UBS-9 | 0,70 (0,72) | 0,32 | 0.36 | 16QAM (Note 1) | 3 | 450 | B | 3x12 | 3x6 |  |  | 67,2 |
| UBS-8 | 0,60 (0,61) | 0,33 | 0.38 |  | 2 | 594 | A | 2x12 | 2x6 |  |  | 59,2 |
| UBS-7 | 0,46 (0,47) | 0,33 | 0.38 |  | 2 | 450 | B | 2x12 | 2x6 |  |  | 44,8 |
| UBS-6 | 0,62 (0,67) | 0,35 | 0.38 | QPSK (Note 1) | 1 | 594 | A | 12 | 6 |  |  | 29,6 |
| UBS-5 | 0,47 (0,51) | 0,35 | 0.38 |  | 1 | 450 | B | 12 | 6 |  |  | 22,4 |
| Note 1: These coding schemes use the higher symbol rate (See 3GPP TS 45.004) and are only available in the uplink Note 2: These coding schemes use the normal symbol rate (See 3GPP TS 45.004) and are only available in the uplink Note 3: These coding schemes use the higher symbol rate (See 3GPP TS 45.004) and are only available in the downlink Note 4: These coding schemes use the normal symbol rate (See 3GPP TS 45.004) and are only available in the downlink Note 5: The number in bracket indicates the code rate for transmission using FANR, when the PAN is present Note 6: These data rates apply in case of BTTI configuration, the data rates are doubled in case of RTTI configuration. Note 7: The turbo code is terminated using 2x3 tail bits resulting in 12 coded bits per RLC block. |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.2.2	Channel coding for PACCH, PBCCH, PAGCH, PPCH, CPBCCH, CPAGCH, CPPCH, and CSCH

The channel coding for the PBCCH, PAGCH, PPCH, CPBCCH, CPAGCH, and CPPCH corresponds to the coding scheme CS-1. The channel coding for the PACCH corresponds to the coding scheme CS-1; in case of RTTI configuration, the channel coding for the downlink PACCH corresponds to the coding scheme MCS-0 if BTTI USF mode is used and may correspond to either CS-1 or MCS-0 if RTTI USF mode is used. The channel coding for the CSCH is identical to SCH.

### 7.2.3	Channel Coding for the PRACH and MPRACH

Two types of packet random access burst may be transmitted on the PRACH and MPRACH: an 8 information bits random access burst or an 11 information bits random access burst called the extended packet random access burst. The MS shall support both random access bursts. The channel coding used for the burst carrying the 8 data bit packet random access uplink message is identical to the coding of the random access burst on the RACH. The channel coding used for the burst carrying the 11 data bit packet random access uplink message is a punctured version of the coding of the random access burst on the RACH.

### 7.2.4	Channel coding for EC-PDTCH, EC-PACCH, EC-AGCH, EC-PCH, EC-BCCH and EC-SCH

The channel coding for the EC-BCCH corresponds to the coding scheme CS-1. EC-PACCH, EC-AGCH, and EC-PCH are all defined by separate coding schemes, see 3GPP TS 45.003. The channel coding for the EC-SCH is identical to SCH, apart from an applied cyclic shift of the encoded bits. The channel coding for the EC-PDTCH is identical to PDTCH for EGPRS, see subclause 7.2.1.2.

2nd modification

# 11	Performance

Under typical urban fading conditions (i.e. multipath delays no greater than 5 µs), the quality threshold for full-rate speech and PDTCH/CS-1 is reached at a C/I value of approximately 9 dB. The maximum sensitivity is approximately -104 dBm for base stations and GSM mobiles and -102 dBm for GSM small MSs and PCS 1900 MS s and -100 dBm for DCS 1 800 hand-helds (see 3GPP TS 45.005).

For EC-EGPRS, the logical channels operate in what is referred to as extended coverage, where, under typical urban fading conditions, the quality threshold for EC-PDTCH/MCS-1 is reached at a C/I of approximately [TBD] dB. The maximum sensitivity is approximately [TBD] dBm for base station and [TBD] dBm for EC-EGPRS MS.

Multi band MSs shall meet the requirements on each band of operation respectively.

End of modifications