3GPP TSG-GERAN #68 	GP-151182

# Anaheim, USA, 16th-19th of November 2015	(revision of GP-151100)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.004 | CR | 0020 | rev | 3 | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EC-EGPRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2015-11-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To introduce EC-EGPRS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of EC-EGPRS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EC-EGPRS will not be introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1.1, 2.6, 2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.013 0135 TS 43.022 0035 TS 43.064 CR 0089,0090,0091,0092 TS 44.018 CR 1025,1026 TS 44.060 CR 1609,1610,1611,1612 TS 45.001 CR 0079,0080 TS 45.002 CR 0183,0184,0185,0186 TS 45.003 CR 0135 TS 45.005 CR 0578 TS 45.008 CR 0629,0630 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.021 TS 51.010 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes compared to GPE150036 are highlighted in green. Changes compared to GP-151100 are highlighted in turquoise. |  |  |  |  |  |  |  |  |

*** First Change ***

# 1.1	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TS 45.001: “Physical Layer on the Radio Path (General Description)”.

[3]	3GPP TS 45.002: “Multiplexing and multiple access on the radio path”.

[4]	3GPP TS 45.005: “Radio transmission and reception”.

[5]	3GPP TS 45.010: “Radio subsystem synchronization”.

[6]	3GPP TS 44.060: “Radio Link Control/ Medium Access Control (RLC/MAC) protocol”.

[7]	3GPP TS 43.064: “General Packet Radio Service (GPRS)”.

[8]	3GPP TS 45.003: “Channel Coding”.

*** For information ***

## 2.5	Output phase

The phase of the modulated signal is:

where the modulating index h is 1/2 (maximum phase change in radians is /2 per data interval).

The time reference t' = 0 is the start of the active part of the burst as shown in figure 1. This is also the start of the bit period of bit number 0 (the first tail bit) as defined in 3GPP TS 45.002.

*** Next Change ***

## 2.6	Modulation

The modulated RF carrier, except for start and stop of the TDMA burst may therefore be expressed as:

where Ec is the energy per modulating bit, f0 is the centre frequency and 0 is a random phase and is constant during one burst.

In case of EC-EGPRS when using blind physical layer transmissions (see 3GPP TS 43.064 [7]) the modulated RF carrier, except for start and stop of the TDMA burst may, for each blind physical layer transmission of a burst for which coherent transmission is required (see 3GPP TS 45.005 [4]), be expressed as:

where t0 is a burst-specific time offset relative to the absolute time instant where t' = 0 for the first transmission of the burst in the set of blind physical layer transmissions. 0 for all blind physical layer transmissions of the same burst, see 3GPP TS 45.003 [8], shall be the same as 0 for the first transmission of that burst.

***Next Change ***

## 2.7	Overlaid CDMA

In the case of OLCDMA for EC-EGPRS the modulated RF carrier may be expressed with an additional term representing the OLCDMA code:

where OC equals 0 or 1, in accordance to the applied OLCDMA code, see 3GPP TS 45.002 [3], and where T is defined in subclause 2.1, t' is defined in subclause 2.5 and the remaining variables are defined in subclause 2.6.