# Impact on legacy services in a reduced BCCH spectrum allocation

# Introduction

At GERAN#67 a new work item named Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things (WI code: CIoT_EC_GSM) was approved, see [1]. One of the objectives of the work item is the following:

“Support for extended coverage GSM deployment in a reduced BCCH spectrum allocation, provided it is shown to be feasible, from 4/12 (2.4 MHz) frequency re-use to 3/9 (1,8 MHz) or 1/3 (600 kHz) frequency re-use, where legacy CS users might not be supported, and add respective normative changes, if any.”

The BCCH layer serves many purposes, which may be negatively impacted by a tightened reuse, including the following:

- Frequency and time synchronization (FCCH, SCH)

- Cell identification (reading BSIC on SCH) and received signal level measurements e.g. for cell reselection purposes

- System information acquisition (BCCH)

- Common control signaling (RACH, PCH, AGCH)

- User data traffic

In this contribution, item 5 is investigated for legacy services. Item 1 is investigated in [2].

# Assumptions

## Traffic generation

MTC traffic is generated according to the MAR periodic reporting and Network Command traffic models [2]. The split between these is 80 % MAR periodic and 20 % Network command.

No other traffic is generated in the cell.

## BCCH Power Savings

BCCH power savings can be used to reduce interference on the BCCH frequency layer.  With tighter BCCH frequency re-use the importance of this increases. BCCH power savings can be used with various levels of reduction and selections of what timeslots and channels it should be applied to. For simulator implementation a simple implementation for BCCH PS was used with a reduction of 6 dB for all TS not used for PDTCH or PACCH. In case PDTCH or PACCH is used on the DL, no power regulation is used.

# Simulations

## Simulation assumptions

The system level simulation assumptions in [2] have been followed. Other specific assumptions are shown in Table 1.

### System parameters

Table 1. Simulation assumptions, in addition to [2]

| Parameter | Value |
| --- | --- |
| General |  |
| Simulation time | 400 s |
| System size | 4/12 re-use: 108 cells 3/9 re-use: 81 cells |
| Direction | UL and DL |
| Frequency band | 900 MHz |
| Layer | BCCH |
| Frequency re-use | 12 and 9 |
| BTS antenna diversity | MRC |
| BTS output power | 43 dBm |
| Cell radius | 577.33 m |
| Legacy GPRS MTC parameters |  |
| PDTCH timeslots per cell | 4 PDCH(note 1) |
| Legacy GPRS MTC arrival rate per cell and second | 6.8 (100%)(note 2) |
| Fixed UL allocation | On |
| Coding schemes | CS-4, CS-3, CS-2, CS-1 |
| GPRS L2S model | Approximated by EGPRS L2S (MCS-1 to MCS-4) without incremental redundancy, see [3] |
| Minimum delay between subsequent transmissions on PDTCH and PACCH | 1 radio block |
| Incremental Redundancy | Off |
| Power control | Off |
| IP header compression | Off |
| Device output power | 33 dBm (100%) |
| BPL model(note 3) | No BPL is applied to the Legacy GPRS MTC users |
| Device timeout | 20 seconds |
| NOTE 1: The allocation of the timeslots in the TDMA frame are randomly offset per cell to model an unsynchronized network. NOTE 2: Derived from traffic models in [2]. 6.8 reports/commands per cell and second corresponds to the targeted number of devices per sector in the study. NOTE 3: According to model in [2]. |  |

### Cell selection and MCS selection

Cell reselection was based on the calculated path gain. A realistic model for cell reselection will be added in a future contribution.

In the simulations no link adaptation was used. Instead the coding scheme was selected solely on path gain and remained the same throughout the duration of the TBF. CS-1, CS-2, CS-3 and CS-4 were allowed.

### Control signaling

Packet uplink ACK/NACK (PUAN) is sent on PACCH/D to (negatively) acknowledge data sent in the UL and assign fixed allocations to the MS. In the simulations its performance is modeled with EGPRS MCS-1. If a PUAN is unsuccessfully received the negative acknowledged blocks will not be transmitted, i.e. the allocated MS will not transmit anything, but the radio block resources are consumed.

Packet downlink ACK/NACK (PDAN) is sent on PACCH/U to (negatively) acknowledge data sent in the DL. In the simulations its performance is modeled with EGPRS MCS-1.

### Simulated scenarios

Table 2 summarizes the simulated scenarios and clarifies the legends in the figures presented in section 4.2.

Table 2: Simulated scenarios

| Legend text | BCCH Re-use | BCCH Power Saving |
| --- | --- | --- |
| BCCH PS = 0 dB; Re-use = 12 | 12 | 0 dB |
| BCCH PS = 6 dB; Re-use = 12 | 12 | 6 dB |
| BCCH PS = 0 dB; Re-use = 9 | 9 | 0 dB |
| BCCH PS = 6 dB; Re-use = 9 | 9 | 6 dB |

  - Results

The results presented are:

- Resource (TS) utilization (section 3.2.1)

  - This represents the average amount of PDTCH UL and DL TS resources required per cell in the system, for the different scenarios, see Table 3 and Table 4.

- Latency of MAR periodic reports (section 3.2.2)

  - The latency includes time to transfer the message.

  - The results are presented as CDFs of the delay at the target traffic load (6.8 users per cell and second), see Figure 1.

  - Failed attempts are not included in the statistics (following the agreed methodology).

- Latency of DL application Ack (section 3.2.3)

  - Latency is measured from the time an application layer DL ACK is received at the base station till the time when the device has successfully received the application layer DL ACK

  - The results are presented as CDFs of the delay at the target traffic load (6.8 users per cell and second), see Figure 2.

- Failed attempts (section 3.2.4)

  - This represents the percentage of the attempts that were not successful, i.e. did not manage to get the report through during 20 seconds.

- Uplink capacity (section 3.2.5)

  - Uplink capacity is defined as “spectral efficiency in number of reports/200 kHz/hour”. Results are shown in Table 5.

### TS Utilization

The TS Utilization is shown in Table 3 and Table 4 for downlink and uplink respectively. On the downlink, the TS Utilization without BCCH Power Saving increases from 0.37 to 0.42 when the reuse is changed from 12 to 9. This is an increase of ~13% while the frequency reuse increase is 25%.

If BCCH Power Saving is activated, the TS utilization goes down to 0.32 for reuse 12 and 0.33 for reuse 9. I.e. the TS usage is lower with re-use 9 and BCCH PS compared to re-use 12 without BCCH PS (0.37 vs 0.33).

Table 3. TS Utilization for the downlink

| BCCH Re-use | BCCH Power Saving [dB] | TS usage [#TS] |
| --- | --- | --- |
| 12 | 0 dB | 0.37 |
| 12 | 6 dB | 0.32 |
| 9 | 0 dB | 0.42 |
| 9 | 6 dB | 0.33 |

For the uplink, the TS Utilization without BCCH Power Saving increases from ~0.49 to ~0.52 when the reuse is changed from 12 to 9. This is an increase of ~6% while the frequency reuse increase is 25%.

If BCCH Power Saving is activated, the TS utilization is reduced to ~0.47 TS for both 12-reuse and 9-reuse.

Table 4. TS Utilization for the uplink

| BCCH Re-use | BCCH Power Saving [dB] | TS usage [#TS] |
| --- | --- | --- |
| 12 | 0 dB | 0.49 |
| 12 | 6 dB | 0.47 |
| 9 | 0 dB | 0.52 |
| 9 | 6 dB | 0.47 |

### Latency of MAR periodic reports

In the absence of valid random access delay measurements, the latency of MAR periodic reports is here only represented by the latency of the data transfer. A few users will experience an increased delay as seen in Figure 1. The delay is positively impacted by the use of BCCH Power Saving.

Figure 1: Uplink Transmission Delay

### Latency of Downlink Application Ack

A few users will experience an increased delay as seen in Figure 2. However, also in this case, the delay is positively impacted by the use of BCCH PS.

Figure 2: Downlink Application Ack Delay

### Failed Attempts

At the traffic load 6.8 users per cell and second, the percentage of failed attempts (i.e., the report did not get delivered within 20 seconds) is found to be 0.00 % in all simulated scenarios.

### Capacity

In the TR [2] capacity is defined as “spectral efficiency in number of reports/200 kHz/hour”. This definition is made with a standalone CIoT system in mind. Since the system in this evaluation serves only one traffic type (MTC traffic), the capacity definition is more meaningful in this case than in the previous EC-EGPRS investigations in which mixed services were assumed. Still it should be noted that the measure is not really a capacity measure since it does not reflect the capacity limit of the system but rather at an assumed fixed load.

Capacity is here calculated as

(#sent reports per sector per hour)*(1 - failed attempts)/reuse

The capacity is shown in Table 5 for the simulated scenarios.

Table 5: Capacity

| BCCH Re-use | BCCH Power Saving | Capacity [reports/200kHz/hour] |
| --- | --- | --- |
| 12 | 0 dB | 2048 |
| 12 | 6 dB | 2048 |
| 9 | 0 dB | 2723 |
| 9 | 6 dB | 2723 |

As can be seen from the table, the 9-reuse scenario has a 33 % higher capacity than the 9-reuse scenario, as expected considering the change in re-use factor, and the fact that no reports fails to be delivered.

# Discussion and conclusions

This paper shows that the uplink seems to be almost unaffected by the 3/9 reuse compared to 4/12. The reason for this is that the simulated scenarios are noise limited on the uplink. The downlink however is somewhat affected. In both cases, the effect is however rather small compared to the 25% reduction of the required frequency spectrum, corresponding to a 33 % increase in spectral efficiency.

The benefits of BCCH Power Savings are clearly visible in all results, although it can be noted that a simplified version of BCCH PS is applied in these simulations.

# References

- GP-151039, “New Work Item on Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things”, source Ericsson LM, Intel, Gemalto N.V., MediaTek Inc., TeliaSonera AB, Sierra Wireless S.A., Telit Communications S.p.A., ORANGE, Nokia Networks, Alcatel-Lucent. GERAN#67.

- GP-151124, ”GPRS synchronization performance in a tight frequency reuse scenario”, source Ericsson LM. GERAN#68.

- GP-151104, ”Intended scope for reduced spectrum allocation on BCCH evaluation (update of GPE150043)”, source Ericsson LM. GERAN#68.

- GP-150762, “EC-GSM, Link modeling methodology for EC-PDTCH capacity evaluations (update of GPC150441)”, source Ericsson LM. GERAN#67.

In reality, the MS would likely send preemptive retransmissions, but this is not modelled in the simulator.

It should be noted that this is rather an increase in spectral efficiency than capacity since the same absolute number of reports are delivered in both scenarios.