# Impact on PDCH for EC-GSM-IoT in a reduced BCCH spectrum allocation (including carrier SS vs SINR measurements)

# Introduction

At GERAN#67 a new work item named Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things (WI code: CIoT_EC_GSM) was approved, see [1]. One of the objectives of the work item is the following:

“Support for extended coverage GSM deployment in a reduced BCCH spectrum allocation, provided it is shown to be feasible, from 4/12 (2.4 MHz) frequency re-use to 3/9 (1.8 MHz) or 1/3 (600 kHz) frequency re-use, where legacy CS users might not be supported, and add respective normative changes, if any.”

The BCCH layer serves many purposes, which may be negatively impacted by a tightened reuse, including the following:

- Frequency and time synchronization (EC-FCCH, EC-SCH)

- Cell identification (reading BSIC on EC-SCH) and received signal level measurements e.g. for cell reselection purposes

- System information acquisition (EC-BCCH)

- Common control signaling (EC-RACH, EC-PCH, EC-AGCH)

- User data traffic

In this contribution, item 5 is investigated for EC-GSM-IoT MTC services for BCCH layer spectrum allocations using 2.4 MHz down to 600 kHz.

# Assumptions

## Traffic generation

MTC traffic is generated according to the MAR periodic reporting and Network Command traffic models [3]. The split between these is 80 % MAR periodic and 20 % Network command.

## EC-RACH interference

Interference from EC-RACH without power control has been modelled.

## BCCH Power Savings

BCCH power savings can be used to reduce interference on the BCCH frequency layer.  With tighter BCCH frequency re-use the importance of this functionality increases. BCCH power savings can be used with various levels of reduction and selections of what timeslots and channels it should be applied to. For simulator implementation a simple implementation for BCCH PS was used with a reduction of 6 dB for timeslots not used for EC-PDTCH or EC-PACCH.

## Uplink Power Backoff

A power backoff of maximum 4 dB is used on EC-PDTCH and EC-PACCH in uplink. This power regulation is based on signal strength measurements. It is worth to note the following:

- The applied model follows the information provided in the EC-EGPRS CHANNEL REQUEST, see [6].

# Simulations

## Simulation assumptions

The system level simulation assumptions in [3] have been followed. Other specific assumptions are shown in Table 1.

### System parameters

Table 1. Simulation assumptions, in addition to [3]

| Parameter | Value |
| --- | --- |
| General |  |
| Simulation time | 100 s |
| System size | 108 cells (all frequency re-uses) |
| Direction | UL and DL |
| Frequency band | 900 MHz |
| Layer | BCCH |
| Frequency re-use | 4/12, 3/9, 1/3 |
| BTS antenna diversity | MRC |
| BTS output power | 43 dBm |
| Cell radius | 577.33 m |
| Number of seeds | 20 for 33 dBm, 1 for 23 dBm |
| EC-GSM-IoT MTC parameters |  |
| Number of repetitions | 1, 4, 8 and 16 |
| EC-PDTCH timeslots per cell | 6 PDCH(note 1) |
| EC-GSM-IoT MTC arrival rate per cell and second | 6.8 (100%)(note 2) |
| Fixed UL allocation | On |
| BT_Threshold_DL | -92, -101 and -103 dBm for carrier CC DL 9 dB for SINR CC DL |
| X (DL_Signal_Strength_Step_Size used in the channel request) | 3 and 6 dB |
| BT_Threshold_UL | -101 dBm |
| Coding schemes in DL | MCS-1, MCS-2, MCS-3 and MCS-4 |
| Coding scheme in UL | MCS-1 |
| EGPRS L2S model | Approximated by EGPRS L2S (MCS-1) with IR on the UL and without IR on the DL, see [4] |
| Minimum delay between subsequent transmissions on EC-PDTCH and EC-PACCH | 1 radio block |
| Incremental Redundancy | On (UL) Off (DL) |
| Power control | On. Power savings 6 dB DL if nothing to transmit on BCCH TS2-TS7. 0, 2 and 4 dB for EC-PDTCH/U and EC-PACCH/U depending on content of channel request |
| IP header compression | Off |
| Device output power | 33 dBm (100%) 23 dBm (100%) |
| BPL model(note 3) | BPL-1 is applied to the EC-GSM-IoT MTC users (scenario 1 and 0.5 raa) |
| Device timeout | 20 seconds |
| NOTE 1: The system simulator uses a network wide timeslot alignment with a random timeslot offset between cells. NOTE 2: Derived from traffic models in [3]. 6.8 reports/commands per cell and second corresponds to the targeted number of devices per sector in the study. NOTE 3: According to model in [3]. |  |

### Cell selection and uplink coverage class selection

Cell selection and uplink coverage class selection was based on carrier measurements according to the simulator model in [8] taking 5 samples per measured cell over 5 seconds.

No cell re-selection has been modeled. The users arrive in the system, perform measurements in idle mode to select a cell to camp on, and then connect to the network. As per the EC-GSM-IoT specification, no measurements for cell reselection are performed in packet transfer mode (PTM), and consequently no cell reselection is performed in PTM.

### Downlink coverage class selection and coding scheme selection

Downlink coverage class selection was based on either SINR or carrier measurements according to the model in [8] taking 5 samples per measured cell over 5 seconds. The BT_Threshold_DL and X used for the simulations are reported in Table 2.

Table 2: BT_Threshold_DL and X for Carrier and SINR CC DL.

| BCCH re-use | Carrier CC DL |  | SINR CC DL |  |
| --- | --- | --- | --- | --- |
|  | BT_Threshold_DL [dBm] | X [dB] | BT_Threshold_DL [dB] | X [dB] |
| 4/12 re-use | -103 | 6 | 9 | 3 |
| 3/9 re-use | -101 | 6 | 9 | 3 |
| 1/3 re-use | -92 | 6 | 9 | 3 |

In the simulations no link adaptation was used. Instead the coding scheme was intially selected to MCS-1, MCS-2, MCS-3 or MCS-4 depending on the measured SINR or carrier value reported in the EC-EGPRS CHANNEL REQUEST by the MS, see [6], and remained the same throughout the duration of the EC TBF. The MCS choice for carrier based and SINR based downlink coverage class selection are reported in Table 3 and Table 4 respectively and are based on the “DL Coverage Class” field reported by the MS in the channel request. This is a 3-bit field and hence 8 different code points can be communicated. The code points for the DL Coverage Class field are referred to as “CC CP”.

Table 3: MCS choice for carrier based downlink coverage class selection.

| CC CP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4/12 re-use | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-2 | MCS-4 |
| 3/9 re-use | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-2 | MCS-4 |
| 1/3 re-use | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-2 | MCS-3 | MCS-4 |

Table 4: MCS choice for SINR based downlink coverage class selection.

| CC CP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4/12 re-use | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-2 | MCS-4 | MCS-4 |
| 3/9 re-use | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-2 | MCS-4 | MCS-4 |
| 1/3 re-use | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-1 | MCS-2 | MCS-3 | MCS-4 |

### Control signaling

Packet uplink ACK/NACK (PUAN) is sent on EC-PACCH/D to (negatively) acknowledge data sent in the UL and assign fixed allocations to the MS. If a PUAN is unsuccessfully received the negative acknowledged blocks will not be transmitted, i.e. the allocated MS will not transmit anything, but the radio block resources are consumed and logged as such, contributing to the overall resource usage.

Packet downlink ACK/NACK (PDAN) is sent on EC-PACCH/U to (negatively) acknowledge data sent in the DL.

EC-PACCH specific Link to System mappings has been used for EC-PACCH/D and EC-PACCH/U.

### Simulated scenarios

The simulated scenarios are for downlink coverage class selection based on measured SINR and carrier signal strength for 4/12, 3/9 and 1/3 re-use, and tables and figures are presented in section 3.2.  The thresholds and the coverage class code point dependent DL MCS choice both for the SINR and carrier scenarios have been optimized to give low timeslot utilization, short delay and high capacity while aiming for an EC-PDTCH DL BLER target of 20 % for MCS-1 in order to ensure robustness of the system. For higher MCSs a higher BLER has been allowed, considering that the RCL/MAC header would still experience a low BLER level at the SINR where the higher MCSs are used.

## Results

The results presented are:

- Resource (TS) Usage (section 3.2.1)

  - This represents the average amount of EC-PDTCH DL and UL TS resources required on average per cell in the system, for the different scenarios, see Table 5.

- Latency of MAR periodic reports (section 3.2.2)

  - The latency includes time to transfer the message excluding common control signaling delay (presented in a separate evaluation, see [10]).

  - The results are presented as CDFs of the delay at the target traffic load (6.81 users per cell and second).

  - Failed attempts are not included in the statistics (following the agreed methodology).

- Latency of DL application Ack (section 3.2.3)

  - Latency is measured from the time an application layer DL ACK is received at the base station till the time when the device has successfully received the application layer DL ACK

  - The results are presented as CDFs of the delay at the target traffic load (6.8 users per cell and second).

- Failed attempts (section 3.2.4)

  - This represents the percentage of the attempts that were not successful, i.e. did not manage to get the report through during 20 seconds.

- Uplink capacity (section 3.2.5)

  - Uplink capacity is defined as “spectral efficiency in number of reports/200 kHz/hour”. Results are shown in Table 6.

### TS Usage

The TS Usage is shown in Table 5 for the downlink and uplink. On the downlink, the TS Usage increases from 0.35 TS to 0.70 TS for SINR based downlink coverage class selection and to 0.77 TS for carrier based downlink coverage class when the reuse is changed from 4/12 to 1/3. Thus, the timeslot utilization increases approximately 2.0 times for SINR and 2.2 times for carrier based downlink coverage class selection while the used frequency bandwidth is reduced four times.

Table 5. TS Usage for the downlink and uplink

| BCCH re- use | 33 dBm TS usage DL [#TS] |  | 33 dBm TS usage UL [#TS] |  |
| --- | --- | --- | --- | --- |
|  | SINR CC DL | Carrier CC DL | SINR CC DL | Carrier CC DL |
| 12 | 0.35 | 0.35 | 0.85 | 0.84 |
| 9 | 0.37 | 0.37 | 0.85 | 0.85 |
| 3 | 0.70 | 0.75 | 0.91 | 0.92 |

| BCCH re- use | 23 dBm TS usage DL [#TS] |  | 23 dBm TS usage UL [#TS] |  |
| --- | --- | --- | --- | --- |
|  | SINR CC DL | Carrier CC DL | SINR CC DL | Carrier CC DL |
| 12 | 0.35 | 0.36 | 1.60 | 1.59 |
| 9 | 0.37 | 0.38 | 1.59 | 1.60 |
| 3 | 0.68 | 0.73 | 1.69 | 1.68 |

It can be noted that the resource increase for the carrier based CC selection is mainly due to more conservative settings (see Table 2) when switching between coverage classes with the aim to roughly operate in the same BLER region irrespective of re-use. Generally it applies that the tighter the re-use the more interference in the system, the more conservative the coverage class thresholds (to lower operative BLER points by using blind transmissions), and the more resources are used. For SINR the same thresholds are used in all simulations (see Table 2) which will shift the coverage class distribution to more users in CC2 and above, when increasing the interference levels in the system (going to a tighter re-use).

Further, it can be noted that carrier based downlink coverage class selection gives approximately 7 % higher downlink TS usage than SINR based downlink coverage class selection in 1/3 re-use. This is however not the only benefit seen, as will be seen below. In actuality there is a trade-off between all metrics presented in this paper, e.g. a lower resource usage would have an impact on latency, CC distribution and failed attempts. All output need to be analyzed jointly.

### Latency of MAR periodic reports

The latency of MAR periodic reports is represented by the latency of the data transfer, i.e. the common control signaling delay, see [3], is not included. A few users will experience an increased delay as seen in Figure 1 and Figure 2. The delays are increasing with tighter frequency re-use.

Figure 1: Uplink Transmission Delay for 33 dBm.

Figure 2: Uplink Transmission Delay for 23 dBm.

### Latency of Downlink Application Ack

A few users will experience an increased Downlink Application Ack delay when going to tighter re-use as seen in Figure 3 and Figure 4. It can be noted that the Downlink Application Ack delay for 3/9 and 4/12 re-use is almost the same for the two downlink coverage class selection cases. However, for 1/3 re-use the delay is larger with carrier based selection compared to the SINR based selection.

Figure 3: Downlink Application Ack Delay for 33 dBm.

Figure 4: Downlink Application Ack Delay for 23 dBm.

### Failed Attempts

At the traffic load 6.8 users per cell and second and device output power of 33 dBm, the percentage of failed attempts (i.e., the report did not get delivered within 20 seconds) is found to be less than 0.1 % in all scenarios.

### Capacity

In the TR [2] capacity is defined as “spectral efficiency in number of reports/200 kHz/hour”. This definition is made with a standalone CIoT system in mind. Since the system in this evaluation serves only one traffic type (MTC traffic), the capacity definition is more meaningful in this case than in the previous EC-GSM-IoT investigations in which mixed services were assumed. Still it should be noted that the measure is not really a capacity measure since it does not reflect the capacity limit of the system but rather at an assumed fixed load.

Capacity is here calculated as

(#sent reports per sector per hour)*(1 - failed attempts)/reuse

The capacity is shown in Table 6 for the simulated scenarios.

Table 6: Capacity

| BCCH Re-use | 33 dBm Capacity [reports/200kHz/hour] |  | 23 dBm Capacity [reports/200kHz/hour] |  |
| --- | --- | --- | --- | --- |
|  | SINR CC DL | Carrier CC DL | SINR CC DL | Carrier CC DL |
| 12 | 2038 | 2038 | 2055 | 2055 |
| 9 | 2724 | 2725 | 2738 | 2738 |
| 3 | 8150 | 8150 | 8220 | 8219 |

As can be seen from the table, the 3-reuse scenario has four times capacity than the 12-reuse scenario, as expected considering the change in re-use factor, and the fact that almost no reports fails to be delivered. The capacity for the 23 dBm is a little higher than the capacity for the 33 dBm case and even higher than the theoretical capacity of 8172 for 6.81 users per cell and second due to randomization.

## DL and UL Coverage Class Distribution

Table 7 and Table 8 summarizes the DL and UL CC distribution for the 4/12 and 1/3 re-use scenarios for both measured SINR and Carrier thresholds. Approximately 98 % and 97 % of all mobiles uses coverage class 1 in downlink for the 4/12 re-use and 3/9 re-use scenarios respectively. For 1/3 re-use only 79 % for SINR based and 87 % for carrier based downlink coverage class selection of all mobiles uses coverage class 1 in downlink. Approximately 98 % and 84 % of all 33 dBm and 23 dBm mobiles respectively uses coverage class 1 in uplink irrespective of the re-use.

Table 7: DL and UL CC distribution for 33 dBm

| BCCH Re- use | Coverage Class | Distribution of users in DL [%] |  | Distribution of users in UL [%] |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SINR CC DL | Carrier CC DL | SINR CC DL | Carrier CC DL |
| 12 | 1 | 98.2 | 98.5 | 97.5 | 97.5 |
| 12 | 2 | 1.8 | 1.5 | 1.8 | 1.8 |
| 12 | 3 | <0.1 | <0.1 | 0.5 | 0.5 |
| 12 | 4 | <0.1 | <0.1 | 0.2 | 0.2 |
| 9 | 1 | 97.0 | 97.5 | 97.5 | 97.5 |
| 9 | 2 | 3.0 | 2.3 | 1.8 | 1.8 |
| 9 | 3 | <0.1 | 0.13 | 0.5 | 0.5 |
| 9 | 4 | <0.1 | <0.1 | 0.2 | 0.2 |
| 3 | 1 | 78.3 | 86.4 | 97.6 | 97.6 |
| 3 | 2 | 21.6 | 11.1 | 1.7 | 1.7 |
| 3 | 3 | 0.13 | 1.3 | 0.5 | 0.5 |
| 3 | 4 | <0.1 | 1.2 | 0.2 | 0.2 |

Table 8: DL and UL CC distribution for 23 dBm

| BCCH Re- use | Coverage Class | Distribution of users in DL [%] |  | Distribution of users in UL [%] |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SINR CC DL | Carrier CC DL | SINR CC DL | Carrier CC DL |
| 12 | 1 | 98.3 | 98.6 | 83.8 | 83.8 |
| 12 | 2 | 1.7 | 1.4 | 9.2 | 9.2 |
| 12 | 3 | <0.1 | <0.1 | 3.9 | 3.9 |
| 12 | 4 | <0.1 | <0.1 | 3.1 | 3.1 |
| 9 | 1 | 97.0 | 97.6 | 84.1 | 84.1 |
| 9 | 2 | 3.0 | 2.3 | 9.0 | 9.0 |
| 9 | 3 | <0.1 | 0.1 | 3.8 | 3.9 |
| 9 | 4 | <0.1 | <0.1 | 3.1 | 3.0 |
| 3 | 1 | 78.7 | 86.5 | 84.5 | 84.4 |
| 3 | 2 | 21.2 | 11.1 | 8.8 | 8.9 |
| 3 | 3 | 0.1 | 1.3 | 3.8 | 3.8 |
| 3 | 4 | <0.1 | 1.1 | 2.9 | 2.9 |

# Discussion and conclusions

## Impact from frequency re-use

This paper adds simulation results for 1/3 re-use and shows that EC-GSM-IoT MTC services may be accommodated on the PDCH of a single BCCH carrier network on as low BCCH spectrum allocations as 600 kHz. For 600 kHz there is less than 2.2 times increase in Downlink TS utilization compared to 2.4 MHz. The transmission delays are increased when going to a tighter re-use. The effect is however rather moderate compared to the 75% reduction of the required frequency spectrum, corresponding to four times the spectral efficiency. Failed rates are in all scenarios kept at a low level.

## SINR vs carrier based measurements

### General

This paper also compares performance between SINR based and carrier signal strength based downlink coverage class selection methods. It shows that downlink coverage class selection based on SINR has more potential than signal strength measurements, specifically in the 1/3 re-use. For 4/12 re-use the downlink TS utilization is the same for both selection methods, but also here there is a visible difference in the delay of the DL delivered reports. For 1/3 re-use, carrier based downlink coverage class selection gives approximately 7 % higher downlink TS utilization compared to SINR based downlink coverage class selection.

It should be mentioned that carrier based measurements and SINR based measurements need to be compared taking the full system impact into account. The coverage class settings will be different, and influence how devices behave in the network. Hence, all metrics investigated will be influenced when changing the CC selection method.

### Coverage class distribution

The main intention to go from carrier based measurements to SINR based measurements is to get a more accurate CC selection that better reflects the experienced SINR when transmitting the block (although the measurement to base the SINR CC selection on is taken at another point in time). In contrast, for carrier based selection, the selection will not take interference into account, and hence the thresholds need to be set more conservatively (more users in higher CC) when interference is increased in order to keep the timeslot utilization and delays low.

In Figure 5 and Figure 6 the coverage class distribution between carrier based and SINR based measurements are shown. As can be seen in Figure 5 and Figure 6 the downlink coverage class and coverage class code point distribution over experienced SINR is much wider for carrier measurement based than SINR measurement based downlink coverage class selection. That means that more mobiles will make a better downlink coverage class selection if SINR measurement based downlink coverage class selection is used.

It can be noted that for carrier measurements approximately 10 % of the mobiles experiencing a downlink SINR of only 0 dB will flag the highest coverage class code point 7.

It can also be noted that the width at half height for the distribution of coverage class code point 3 to 6 is approximately 7 to 8 dB for SINR measurements.

Figure 5: Downlink Coverage class distribution for 33 dBm. 
Carrier SS based (left), SINR based (right)

Figure 6: Downlink Coverage class distribution for 23 dBm. 
Carrier SS based (left), SINR based (right)

### BLER

To understand the system behavior it is of interest to look at the BLER performance. One way to look at it is to investigate the BLER at different coupling loss. At high coupling loss close to the coverage limit, both C based selection and SINR based selection will use higher CCs (for C based selection a MS will estimate itself to be below BT_Threshold_DL, and for SINR based selection, the SINR will be low enough even without added interference). However, for lower coupling loss ranges, a difference is expected, depending on the coverage class threshold settings for the different approaches. In Figure 7 the BLER versus Coupling Loss is shown at Coupling Loss 100 to 140 dB. As can be seen, even with a lower resource usage (as shown in Table 5) for SINR based selection, the BLER is significantly lower compared to the C based selection.

Figure 7: Average downlink BLER vs Coupling Loss for 33 dBm.

One can see that the BLER level is for some coupling losses higher than the aimed for 20%. However, in these regions MCS-4 can have been used, where the RLC/MAC header still would have a low BLER, and IR could be used (although not activated in the simulations). For SINR based selection, the BLER is low due to the limited MCSs usage (maximum MCS-4) and the limitation in power down-regulation on the BCCH carrier (max 6 dB). Hence, even if 8PSK MCSs have not been used in the simulations (MCS-5-9) the simulations show a potential of using these to minimize resource usage in the network and improve spectral efficiency also reducing resource usage further.

# Conclusion

EC-GSM-IoT MTC services may be accommodated on the PDCH of a single BCCH carrier network on as low BCCH spectrum allocations as 600 kHz.

Downlink coverage class selection based on SINR has more potential than signal strength measurements, specifically in the 1/3 re-use. It gives lower downlink timeslot utilization and lower delays thanks to increased probability to choose a coverage class and MCS that matches the experienced SINR.

# References

- GP-151039, “New Work Item on Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things”, source Ericsson LM, Intel, Gemalto N.V., MediaTek Inc., TeliaSonera AB, Sierra Wireless S.A., Telit Communications S.p.A., ORANGE, Nokia Networks, Alcatel-Lucent. GERAN#67.

- GP-151124, ”GPRS synchronization performance in a tight frequency reuse scenario”, source Ericsson LM. GERAN#68.

- GP-151104, ”Intended scope for reduced spectrum allocation on BCCH evaluation (update of GPE150043)”, source Ericsson LM. GERAN#68.

- GP-150762, “EC-GSM, Link modeling methodology for EC-PDTCH capacity evaluations (update of GPC150441)”, source Ericsson LM. GERAN#67.

- GP-151183, “Impact on legacy services in a reduced BCCH spectrum allocation”, source Ericsson LM. GERAN#68.

- GP-160036, “On the EC-EGPRS Channel Request content”, source Ericsson LM. GERAN#69.

- GP-160204, “CR 45.008-0631 Introduction of EC-EGPRS - Idle mode (Rel-13)”, source Ericsson LM

- GP-160033, “Received signal level measurements for EC-EGPRS (update of GP-151135)”, source Ericsson LM. GERAN#69.

- Cell (re)selection and Coverage Class selection for EC-GSM-IoT

- GP-160268, “Impact on common control channels for EC-GSM-IoT in a reduced BCCH spectrum allocation”, source Ericsson LM. GERAN#70.

It should be noted that this is rather an increase in spectral efficiency than capacity since the same absolute number of reports are delivered in both scenarios.