# Impact on common control channels for GPRS/EGPRS in a reduced BCCH spectrum allocation

# Introduction

At GERAN#67 a new work item named Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things (WI code: CIoT_EC_GSM) was approved, see [1]. One of the objectives of the work item is the following:

“Support for extended coverage GSM deployment in a reduced BCCH spectrum allocation, provided it is shown to be feasible, from 4/12 (2.4 MHz) frequency re-use to 3/9 (1.8 MHz) or 1/3 (600 kHz) frequency re-use, where legacy CS users might not be supported, and add respective normative changes, if any.”

The BCCH layer serves many purposes, which may be negatively impacted by a tightened reuse, including the following:

- Frequency and time synchronization (FCCH, SCH)

- Cell identification (reading BSIC on SCH) and received signal level measurements e.g. for cell reselection purposes

- System information acquisition (BCCH)

- Common control signaling (RACH, PCH, AGCH)

- User data traffic

In this contribution, item 4 is investigated for legacy GPRS/EGPRS devices for BCCH layer spectrum allocations using 2.4 MHz down to 600 kHz.

# Assumptions

## Link model

The link level model used in the simulator is described in [8].

## Network synchronization

The interference situation modeled by the simulation is limited to timeslot synchronized network. This means AGCH / RACH channels are both interfered by other CCCH channels, and PDTCH/PACCH interference in other cells.

## BCCH Power Savings

BCCH power savings can be used to reduce interference on the BCCH frequency layer.  With tighter BCCH frequency re-use the importance of this functionality increases. BCCH power savings can be used with various levels of reduction and selections of what timeslots and channels it should be applied to. For the simulator a simple implementation for BCCH PS was used with a reduction of 6 dB for 60% of the dummy bursts transmitted on the CCCH DL. I.e. no power control was applied to Immediate Assignment messages. The choice not to down-regulate all dummy bursts on the CCCH is to also include a more highly loaded network where not only AGCH but also PCH would be transmitted (assumed to be not power regulated).

## Frequency planning

The frequency planning simulated have been based on regular re-use clusters in a 4/12, 3/9, and 1/3 re-use.

# Simulations

## Simulation assumptions

The system level simulation assumptions in [3] have been followed. Other specific assumptions are shown in Table 1.

### System parameters

Table 1. Simulation assumptions, in addition to [3]

| Parameter | Value |
| --- | --- |
| Number of re-use clusters | 4/12, 3/9 has used 9 clusters. 1/3 has used 36 clusters. |
| Direction | UL and DL |
| Frequency band | 900 MHz |
| Layer | BCCH |
| Frequency re-use | 4/12,3/9,1/3 with regular frequency planning |
| BTS antenna diversity | MRC |
| BTS output power | 43 dBm |
| Cell radius | 577.33 m |
| MTC arrival rate per cell and second | 5.4, see [12] |
| Maximum attempts on EC-RACH per system access attempt | 6 |
| Power control, DL | 6 dB DL on 60% of dummy bursts. |
| Power control, UL | None |
| Device output power | 33 dBm |
| BPL model | None |
| RACH parameters | S=109, T=5 |

## Results

The results presented are:

- Resource Usage

  - Average amount of bursts used per user, including all transmissions per system access attempt.

- Common control signaling delay

  - The delay includes time from initial RACH transmission to a received matching Immediate Assignment.

- Failed attempts

  - This represents the percentage of the attempts that were not successful, after the maximum attempts.

### Resource Usage

The resource usage in terms of bursts is shown in Table 2.

Table 2: Resource Usage for the downlink and uplink, 33 dBm

| BCCH Re-use | Resource usage DL [#bursts] | Resource usage UL [#bursts] |
| --- | --- | --- |
| 12 | 4.0 | 1.0 |
| 9 | 4.0 | 1.0 |
| 3 | 4.6 | 1.2 |

As can be seen, the difference between 12 and 9 re-use is not visible, while the change from a 9 re-use factor to a 3 re-use factor has a clearly visible impact on the results.

### Common control channel delay

In Figure 1 the common control channel delay is shown. As can be seen, 95% of the users experience lower delay than 50 ms (granularity of logged delay CDF) in all cases.

Figure 1: Common Control Signaling Delay, 33 dBm

### Failed Attempts

The overall failed attempts are in all simulations well below 0.1%, but to avoid the risk of not running too long simulations to come up with a number with enough statistical significance, it can safely be assumed that less than 0.1% of the system access attempts fail.

# Discussion

The paper has investigated the performance of the CCCH in a tight BCCH re-use scenario. Frequency re-use factors from 12, 9 and 3 has been investigated using a regular frequency re-use cluster deployment.

BCCH power savings has been applied, but only on dummy bursts transmitted on the CCCH, and down-regulation has only been allowed in 60% of the bursts. This is to model a higher load on the CCCH, considering also for example PCH traffic would be present in a real network deployment.

# Conclusions

The paper has investigated the impact on the CCCH in a tight BCCH spectrum. The results are encouraging showing extremely low failed rates even in a very tight re-use pattern. The resource usage is increased by roughly 20% when going from 12 to 3 in frequency re-use. The overall common control signaling delay is slightly increased, as expected, but still the 95 percentile is around 50 ms for all cases

# References

- GP-151039, “New Work Item on Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things”, source Ericsson LM, Intel, Gemalto N.V., MediaTek Inc., TeliaSonera AB, Sierra Wireless S.A., Telit Communications S.p.A., ORANGE, Nokia Networks, Alcatel-Lucent. GERAN#67.

- GP-151124, ”GPRS synchronization performance in a tight frequency reuse scenario”, source Ericsson LM. GERAN#68.

- GP-151104, ”Intended scope for reduced spectrum allocation on BCCH evaluation (update of GPE150043)”, source Ericsson LM. GERAN#68.

- GP-160042, “Impact on EC-EGPRS synchronization performance in reduced spectrum allocation”, source Ericsson LM. GERAN #69.

- GP-160026, “Dependency on number of coverage classes for EC-EGPRS on system capacity (update of GP-151125)”, source Ericsson LM. GERAN Telco#4 on EC-GSM and eDRX

- GP-160030, “Alignment of Coverage Classes for EC-EGPRS”, source Ericsson LM. GERAN#69

- GP-160035, “Coverage Class Adaptation on EC-RACH”, source Ericsson LM. GERAN#69.

- GP-150435, “EC-GSM, Link modelling methodology for EC-RACH capacity evaluations (update of GPC150193)”, source Ericsson LM. GERAN#66

- GP-151126, System impact from power control settings on EC-RACH, source Ericsson LM. GERAN#68

- GP-150762, “EC-GSM, Link modeling methodology for EC-PDTCH capacity evaluations (update of GPC150441)”, source Ericsson LM. GERAN#67.

- GP-151183, “Impact on legacy services in a reduced BCCH spectrum allocation”, source Ericsson LM. GERAN#68.

- GP-160060, “Traffic model for legacy GPRS MTC”, source Orange. GERAN#69.