3GPP TSG-GERAN #70 	GP-160314

# Nanjing, PR China, May 23rd – 27th 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 44.060 | CR | 1625 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EC-GSM-IoT RLC/MAC Block Structure and Control messages, Miscellaneous corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G2 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2016-05-17 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Some errors and need for clarifications have been identified for EC-GSM-IoT in 3GPP TS 44.060. The proposed name changes in GP-160130, which was agreed at the GERAN#69 meeting, also need to be implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Miscellaneous corrections of errors and clarifications for EC-GSM-IoT are implemented in the specification, including: - Name changes for EC-GSM-IoT. - Update of the UL and DL Ctrl block to align with the correct block size - Retry bit removed - Alignment of TFI and PR fields for both EGPRS and EC-GSM-IoT in DL data headers - DL CC field removed from DL data header Type 1 and 2 - Stalling indicator field removed in UL data header Type 1 - 3 - Resent bit indicator field removed in UL data header Type 1 – 3 - A 4-bit field is included in the uplink RLC data block header to signal the estimated downlink Coverage Class and received signal level - Changes field alignment for the EC-PACCH/D - Addition of new message for ack/nack, contention resolution and FUA, and removal of requirement for NW to include the TLLI in the next EC PUAN after it has been received, according to GP-160320; - Size of DL_CC_EST field increased to align with 44.018; - Inclusion of the ALPHA parameter - Increased the number of bits for TIMESLOT_MULTIPLICATOR parameter |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect names and other errors will remain in the specifications. EC-GSM-IoT operation will not be correctly implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.0a, 10.0a.2, 10.0b.2, 10.0b.2.1, 10.0b.2.2, 10.0b.3, 10.0b.3.2, 10.3.1.3, 10.3.2, 10.3a, 10.3a.0, 10.3a.1, 10.3a.1.1 (new), 10.3a.1.2 (new), 10.3a.2, 10.3a.2.1 (new), 10.3a.2.2 (new), 10.3a.3, 10.3a.3.1, 10.3a.3.2, 10.3a.3.3, 10.3a.4, 10.3a.4.1, 10.3a.4.2, 10.3a.4.3, 10.4.2, 10.4.4, 10.4.4c, 10.4.5, 10.4.6, 10.4.6a, 10.4.7, 10.4.8a, 10.4.8a.1, 10.4.8a.2, 10.4.8a.3, 10.4.8b, 10.4.9f, 10.4.9g, 10.4.10a, 10.4.10b, 10.4.12, 10.4.14a, 10.4.20, 10.4.27, 10.4.28, 10.4.29 (new), 11.1.1.1, 11.1.4.2, 11.1.4.3.3, 11.1.4.3.4, 11, 11.2, 11.2.0.1, 11.2.0.2, 11.2.49, 11.2.50, 11.2.51, 11.2.52, 11.2.53, 11.2.54, 11.2.55, 11.2.56, 11.2.57, 11.2.58, 11.2.59 (new), 12.72, 12.73, 12.74, 12.75. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS 43.022 CR 0037,TS 43.059 CR 0080 TS 43.064 CR 0104,TS 44.004 CR 0019 TS 44.014 CR 0024,TS 44.018 CR 1035 TS 44.018 CR 1036,TS 44.060 CR 1624 TS 45.001 CR 0084,TS 45.002 CR 0192 TS 45.003 CR 0138,TS 45.004 CR 0024 TS 45.005 CR 0578,TS 45.008 CR 0635 TS 45.008 CR 0636,TS 45.010 CR 0069 TS 48.018 CR 0425 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS 51.021 CR 0276 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First modification *****

## 10.0a	RLC/MAC block structure

Different RLC/MAC block structures are defined for data transfers and for control message transfers. The RLC/MAC block structures for data transfers are different for GPRS and EGPRS/EC-GSM-IoT, whereas the RLC/MAC block structure used for control message transfers for EC-GSM-IoT is different than the one used for GPRS and EGPRS.

***** Next modification *****

### 10.0a.2	EGPRS and EC-GSM-IoT RLC/MAC block for data transfer

The RLC/MAC block for EGPRS, EC-GSM-IoT and EGPRS2 data transfer is shown in figure 10.0a.2.1 and consists of:

-	 a combined RLC/MAC header,

-	one or two RLC data blocks for EGPRS and EC-GSM-IoT, or up to four RLC data blocks for EGPRS2,

-	an optional PAN field which is included in case FANR is activated,

-	and an optional eTFI field which is included for the case where a mobile station in a DLMC configuration is assigned an eTFI for a downlink TBF. If an eTFI is assigned for a downlink TBF then an eTFI is included in an uplink RLC/MAC block for data transfer only if a PAN field corresponding to that downlink TBF is also included.

| RLC/MAC block |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| RLC/MAC header | RLC data block 1 | RLC data block 2 (conditional) | RLC data block 3 (conditional) | RLC data block 4 (conditional) | PAN (optional) | eTFI (optional) |

Figure 10.0a.2.1: RLC/MAC block structure for EGPRS, EC-GSM-IoT and EGPRS2 data transfer

Each RLC data block contain octets from one or more upper layer PDUs.

The PAN field may only be included within an EGPRS RLC/MAC block for data transfer within a TBF with FANR activated.

In EGPRS and EC-GSM-IoT, depending on the modulation and coding scheme (see 3GPP TS 44.004 and 3GPP TS 45.003) one or two RLC data blocks are contained in one RLC/MAC block. For MCS-1, MCS-2, MCS-3, MCS-4, MCS-5 and MCS-6 there is one RLC data block, whereas for MCS-7, MCS-8 and MCS-9 there are two RLC data blocks in the RLC/MAC block.

In EGPRS and EC-GSM-IoT, in each transfer direction, uplink and downlink, three different header types are defined. The header fields for a specific header type may differ between EGPRS and EC-GSM-IoT. Which header type that is used depends on the modulation and coding scheme (MCS):

Header type 1	is used with modulation and coding scheme MCS-7, MCS-8 and MCS-9.

Header type 2	is used with modulation and coding scheme MCS-5 and MCS-6.

Header type 3	is used with modulation and coding scheme MCS-1, MCS-2, MCS-3 and MCS-4.

In EGPRS2, depending on the modulation and coding scheme (see 3GPP TS 44.004 and 3GPP TS 45.003) one to four RLC data blocks are contained in one RLC/MAC block as follows:

-	One RLC data block per RLC/MAC block: MCS-1, MCS-2, MCS-3, MCS-4, MCS-5, MCS-6, DAS-5, DAS-6, DAS-7, DBS-5, DBS-6, UBS-5 and UBS-6;

-	Two RLC data blocks per RLC/MAC block: MCS-7, MCS-8, MCS-9, DAS-8, DAS-9, DAS-10, DBS-7, DBS-8, UAS-7, UAS-8, UAS-9, UBS-7 and UBS-8;

-	Three RLC data blocks per RLC/MAC block: DAS-11, DAS-12, DBS-9, DBS-10, UAS-10, UAS-11, UBS-9 and UBS-10;

-	Four RLC data blocks per RLC/MAC block: DBS-11, DBS-12, UBS-11 and UBS-12.

In EGPRS2, ten header types are used in the downlink direction. Which header type is used depends on the modulation and coding scheme:

Header type 1	is used with modulation and coding scheme MCS-7, MCS-8 and MCS-9

Header type 2	is used with modulation and coding scheme DAS-5, DAS-6 and DAS-7.

Header type 3	is used with modulation and coding scheme MCS-1, MCS-2, MCS-3 and MCS-4.

Header type 4	is used with modulation and coding scheme DAS-8 and DAS-9.

Header type 5	is used with modulation and coding scheme DAS-11 and DAS-12.

Header type 6	is used with modulation and coding scheme DBS-5 and DBS-6.

Header type 7	is used with modulation and coding scheme DBS-7 and DBS-8.

Header type 8	is used with modulation and coding scheme DBS-9 and DBS-10.

Header type 9	is used with modulation and coding scheme DBS-11 and DBS-12.

Header type 10	is used with modulation and coding scheme DAS-10.

In EGPRS2, eight header types are used in the uplink direction. Which header type is used depends on the modulation and coding scheme:

Header type 2	is used with modulation and coding scheme MCS-5 and MCS-6.

Header type 3	is used with modulation and coding scheme MCS-1, MCS-2, MCS-3 and MCS-4.

Header type 4	is used with modulation and coding scheme UAS-7, UAS-8 and UAS-9.

Header type 5	is used with modulation and coding scheme UAS-10 and UAS-11.

Header type 6	is used with modulation and coding scheme UBS-5 and UBS-6.

Header type 7	is used with modulation and coding scheme UBS-7 and UBS-8.

Header type 8	is used with modulation and coding scheme UBS-9 and UBS-10.

Header type 9	is used with modulation and coding scheme UBS-11 and UBS-12.

***** Next modification *****

### 10.0b.2	Assembling conventions

Different assembling conventions apply for GPRS RLC data blocks, RLC/MAC control blocks, 11-bit and 8-bit control messages and EGPRS/EC-GSM-IoT RLC data blocks.

#### 10.0b.2.1	Assembling convention for GPRS RLC data blocks and RLC/MAC control blocks, 11-bit and 8-bit control messages

The different components of an RLC/MAC block carrying a GPRS RLC data block or an RLC/MAC control block shall be assembled sequentially. Each component consists of an integer number of octets with an exception for the EC-PACCH/D RLC/MAC control block where the block is of integer number of octets but not the individual components (MAC header and Control message content). The assembling of components shall be performed progressively, starting in octet number 1 of the physical block.

The 11-bit and 8-bit control messages map directly into the corresponding physical block.

In this respect, an RLC/MAC control message, defined in sub-clause 11, or a segment of an RLC/MAC control message, see sub-clause 9.1.12a, shall be treated as a single field of either:

-	176 bits(22 octets, using the PBCCH/PCCCH downlink/CS-1 encoded PACCH block format),

-	304 bits (38 octets, using the CS-3 encoded PACCH block format),

-	11 bits or 8 bits (using the PRACH uplink/PACCH uplink short acknowledgement block formats, see 3GPP TS 44.004),

-	75 bits or 67 bits (using the EC-PACCH/D without or with the optional RLC/MAC header bits respectively),

-	64 bits (using the EC-PACCH/U).

The message contents defines a sequence of bits in decreasing order of value, i.e. the first bit of the message contents represents the highest order value and the last bit the lowest order value.

The RLC/MAC header and a GPRS RLC data block are components that consist of an integer number of octets. Each octet shall be treated as a separate field when mapped into the physical block. The lowest numbered bit represents the lowest order value.

The PDTCH block type 2 (CS-2), type 3 (CS-3) and type 4 (CS-4) formats (see 3GPP TS 44.004) do not have an integer number of octets. In these block types, bits number n to 1 of octet number N1 are spare bits.

#### 10.0b.2.2	Assembling convention for EGPRS and EC-GSM-IoT RLC data blocks

The different components of the RLC/MAC block carrying an EGPRS or EC-GSM-IoT RLC data block shall be assembled sequentially. A component may consist of a non-integer number of octets. Each octet shall be treated as a separate field when mapped into the physical block. The lowest numbered bit represents the lowest order value.

The assembling of components shall be performed progressively, starting with octet number 1 of the physical block. If the boundary between two components falls within an octet of the physical block, the components, or parts thereof, that are contained in that octet shall be assembled progressively, starting with bit number 1 of the octet. (i.e. going from bit number 1 to bit number 8, except in octet number N1, where components are assembled going from bit number 1 to bit number n).

### 10.0b.3	Field mapping conventions

Different field mapping conventions apply for GPRS RLC data blocks, RLC/MAC control blocks, 11-bit and 8-bit control messages and EGPRS/EC-GSM-IoT RLC data blocks.

#### 10.0b.3.1	Field mapping convention for GPRS RLC data blocks, CS-1 or CS-3 encoded RLC/MAC control blocks, EC-PACCH/D and EC-PACCH/U, 11-bit and 8-bit control messages

When a field within a GPRS RLC data block or a CS-1 encoded RLC/MAC control block, or a CS-3 encoded RLC/MAC control block, or EC-PACCH/D or EC-PACCH/U encoded RLC/MAC control block or an 11-bit or an 8-bit control message is contained within a single octet of the physical block, the lowest numbered bit of the field represents the lowest order value.

When a field spans more than one octet of the physical block, the order of bit values within each octet progressively decreases as the octet number increases. In that part of a field contained in a given octet, the lowest numbered bit represents the lowest order value.

#### 10.0b.3.2	Field mapping convention for EGPRS and EC-GSM-IoT RLC data blocks and MCS-0 encoded RLC/MAC control blocks

When a field within an EGPRS or EC-GSM-IoT RLC data block is contained within a single octet of the physical block, the lowest numbered bit of the field represents the lowest order value.

When a field spans more than one octet of the physical block, the order of bit values within each octet progressively increases as the octet number increases. In that part of a field contained in a given octet, the lowest numbered bit represents the lowest order value.

***** Next modification *****

#### 10.3.1.3	Blocks encoded for EC-PACCH/D

A coding block payload content of up to 80 bits, including MAC header, has been defined for use on EC-PACCH/D where the USF is encoded separately. Depending on the value of the Payload Type bit, the payload size is either 75 or 67 bits and is coded to generate punctured bits for EC-PACCH/D and designed to provide compatibility with GPRS and EGPRS. The EC-PACCH/D RLC/MAC control block for EC-GSM-IoT, including the MAC header, is formatted as shown in figure 10.3.1.3.1 or 10.3.1.3.2.

The USF is encoded separately, see 3GPP TS 45.003. A different USF value may be included in each block when several blind physical layer transmissions are used in the downlink. If only EC TBFs are assigned on a PDCH, the inclusion of the USF should be regarded as optional.

| Bit |  |  |
| --- | --- | --- |
| 3 | 2 | 1 |
|  | USF |  |

|  |  |  |  | Bit |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |
| PR | Payload Type |  | RRBP |  |  | S/P |  |  |  | MAC header (5 bits) |
|  |  |  |  |  |  |  |  |  |  | Octet 1 |
|  |  |  |  |  |  |  |  |  |  | Octet 2 |
| Control Message Contents |  |  |  |  |  |  |  |  |  | … |
|  |  |  |  |  |  |  |  |  |  | Octet 9 |

Figure 10.3.1.3.1: EC-GSM-IoT Downlink RLC/MAC control block together with its normal MAC header when Payload Type is set to 0

| Bit |  |  |
| --- | --- | --- |
| 3 | 2 | 1 |
|  | USF |  |

|  |  |  |  | Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 | 6 | 5 | 4 |  | 3 | 2 |  | 1 |  |  |
| PR | Payload Type |  | RRBP |  |  | S/P | PRe |  | RBSN |  | FS | MAC header (13 bits) |
| TFI |  |  |  |  |  |  |  |  |  |  |  | Octet 1 |
|  |  |  |  |  |  |  |  |  |  |  |  | Octet 2 |
| Control Message Contents |  |  |  |  |  |  |  |  |  |  |  | … |
|  |  |  |  |  |  |  |  |  |  |  |  | Octet 9 |

Figure 10.3.1.3.2: EC-GSM-IoT Downlink RLC/MAC control block together with its extended MAC header when Payload Type is set to 1

***** Next modification *****

### 10.3.2	Uplink RLC/MAC control block

The Uplink RLC/MAC control block together with its MAC header is formatted as shown in figure 10.3.2.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| Payload Type |  |  |  | spare |  |  | R | MAC header |
|  |  |  |  |  |  |  |  | Octet 1 |
|  |  |  |  |  |  |  |  | Octet 2 |
|  |  |  |  |  |  |  |  | Octet 3 |
| Control Message Contents |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  | Octet 21 |
|  |  |  |  |  |  |  |  | Octet 22 |

Figure 10.3.2.1: Uplink RLC/MAC control block together with its MAC header

A coding block payload content of 64 bits has been defined for use on EC-PACCH/U. The payload is coded to generate punctured bits for EC-PACCH/U and designed to provide compatibility with GPRS and EGPRS. The EC-PACCH/U RLC/MAC control block for EC-GSM-IoT is without MAC header and formatted as shown in figure 10.3.2.2.

|  |  |  |  |  |  |  |  |  |  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 1 |  |
| Control Message Contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  | … |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 8 |  |

Figure 10.3.2.2: EC-GSM-IoT Uplink RLC/MAC control block.

***** Next modification *****

## 10.3a	EGPRS and EC-GSM-IoT RLC data blocks and RLC/MAC headers

### 10.3a.0	General

The EGPRS and EC-GSM-IoT RLC data block consists of a FBI (downlink) or TI (uplink) field and an E field followed by an EGPRS RLC data unit. The EGPRS RLC data unit is a sequence of N2 octets that are numbered from 1 to N2.

NOTE:	The octets of an EGPRS RLC data unit are not necessarily aligned with the octets of the RLC/MAC block. An octet of the EGPRS RLC data unit may thus span across the boundary between two consecutive octets of the RLC/MAC block.

The RLC/MAC block format convention of sub-clause 10.0b for EGPRS and EC-GSM-IoT applies when the components of the EGPRS and EC-GSM-IoT RLC data block are assembled into the RLC/MAC block.

| E | FBI/TI | EGPRS RLC data unit |
| --- | --- | --- |

Figure 10.3a.0.1: Components of the EGPRS and EC-GSM-IoT RLC data block

The size of the EGPRS RLC data unit for each of the channel coding schemes is shown in table 10.3a.0.1.

Table 10.3a.0.1: EGPRS and EC-GSM-IoT RLC data unit size

| Channel Coding Scheme | EGPRS RLC data unit size (N2) (octets) | Family |
| --- | --- | --- |
| MCS-1 | 22 | C |
| MCS-2 | 28 | B |
| MCS-3 with padding | 31 | A padding |
| MCS-3 | 37 | A padding / A |
| MCS-4 | 44 | C |
| MCS-5 | 56 | B |
| MCS-6 with padding | 68 | A padding |
| MCS-6 | 74 | A |
| MCS-7 | 2x56 | B |
| MCS-8 | 2x68 | A padding |
| MCS-9 | 2x74 | A |
| NOTE 1: The four families of EGPRS RLC data blocks C, B, A and A padding based on a common size basis (22, 28, 37 and 68 octets respectively) enable link adaptation retransmission as described in sub-clause 9. NOTE 2: Modulation and coding schemes of family A padding are compatible with Family A padding6 defined for EGPRS2. |  |  |

The size of the RLC data unit for each of the channel coding schemes used in EGPRS2 is shown in tables 10.3a.0.3, 10.3a.0.4, 10.3a.0.5, and 10.3a.0.6.

Table 10.3a.0.3: RLC data unit size (EGPRS2-A downlink)

| Channel Coding Scheme | EGPRS RLC data unit size (N2) (octets) | Family |
| --- | --- | --- |
| MCS-1 | 22 | C |
| MCS-2 with padding | 26 | B padding2 |
| MCS-2 | 28 | B padding2 / B |
| MCS-3 with padding | 31 | A padding6 |
| MCS-3 | 37 | A padding6 |
| MCS-4 | 44 | C |
| DAS-5 | 56 | B |
| DAS-6 | 68 | A padding6 |
| MCS-6 | 74 | NOTE 2 |
| DAS-7 | 82 | B padding2 |
| MCS-7 | 2x56 | B |
| DAS-8 | 2x56 | B |
| MCS-8 | 2x68 | A padding6 |
| DAS-9 | 2x68 | A padding6 |
| DAS-10 | 2x82 | B padding2 |
| DAS-11 | 3x68 | A padding6 |
| DAS-12 | 3x82 | B padding2 |
| NOTE 1: The four families of RLC data blocks (C, B, A padding6, and B padding2) based on a common size basis (22, 28, 68 and 82 octets respectively) enable link adaptation retransmission as described in sub-clause 9. NOTE 2: MCS-6 is also used for EGPRS2-A downlink (see sub-clause 5.2.1). |  |  |

Table 10.3a.0.4: RLC data unit size (EGPRS2-B downlink)

| Channel Coding Scheme | EGPRS RLC data unit size (N2) (octets) | Family |
| --- | --- | --- |
| MCS-1 | 22 | C |
| MCS-2 | 28 | B |
| MCS-3 with padding | 31 | A padding6 |
| MCS-3 | 37 | A padding6 / A |
| MCS-4 | 44 | C |
| DAS-5 | 56 | B |
| DBS-5 | 56 | B |
| DAS-6 | 68 | A padding6 |
| MCS-6 | 74 | A |
| DBS-6 | 74 | A |
| MCS-7 | 2x56 | B |
| DAS-8 | 2x56 | B |
| DBS-7 | 2x56 | B |
| MCS-8 | 2x68 | A padding6 |
| DAS-9 | 2x68 | A padding6 |
| MCS-9 | 2x74 | A |
| DAS-10 with padding | 2x74 | A padding8 |
| DBS-8 | 2x74 | A |
| DBS-9 | 3x56 | B |
| DAS-11 | 3x68 | A padding6 |
| DAS-12 with padding | 3x74 | A padding8 |
| DBS-10 | 3x74 | A |
| DBS-11 | 4x68 | A padding6 |
| DBS-12 | 4x74 | A |
| NOTE 1: The five families of RLC data blocks (C, B, A, A padding6 and A padding8) based on a common size basis (22, 28, 37, 68 and 74 octets respectively) enable link adaptation retransmission as described in sub-clause 9. |  |  |

Table 10.3a.0.5: RLC data unit size (EGPRS2-A uplink)

| Channel Coding Scheme | EGPRS RLC data unit size (N2) (octets) | Family |
| --- | --- | --- |
| MCS-1 | 22 | C |
| MCS-2 | 28 | B |
| MCS-3 with padding | 27 | A padding10 |
| MCS-3 | 37 | A padding10 / A |
| MCS-4 | 44 | C |
| MCS-5 | 56 | B |
| MCS-6 with padding | 64 | A padding10 |
| MCS-6 | 74 | A |
| UAS-7 | 2x56 | B |
| UAS-8 | 2x64 | A padding10 |
| UAS-9 | 2x74 | A |
| UAS-10 | 3x56 | B |
| UAS-11 | 3x64 | A padding10 |
| NOTE 1: The four families of RLC data blocks (C, B, A, and A padding10) based on a common size basis (22, 28, 37 and 64 octets respectively) enable link adaptation retransmission as described in sub-clause 9. |  |  |

Table 10.3a.0.6: RLC data unit size (EGPRS2-B uplink)

| Channel Coding Scheme | EGPRS RLC data unit size (N2) (octets) | Family |
| --- | --- | --- |
| MCS-1 | 22 | C |
| MCS-2 | 28 | B |
| MCS-3 with padding | 31 | A padding6 |
| MCS-3 | 37 | A padding6 / A |
| MCS-4 | 44 | C |
| UBS-5 | 56 | B |
| UBS-6 with padding | 68 | A padding6 |
| UBS-6 | 74 | A |
| UBS-7 | 2x56 | B |
| UBS-8 with padding | 2x68 | A padding6 |
| UBS-8 | 2x74 | A |
| UBS-9 | 3x56 | B |
| UBS-10 with padding | 3x68 | A padding6 |
| UBS-10 | 3x74 | A |
| UBS-11 | 4x68 | A padding6 |
| UBS-12 | 4x74 | A |
| NOTE 1: The four families of RLC data blocks (C, B, A and A padding6) based on a common size basis (22, 28, 37 and 68 octets respectively) enable link adaptation retransmission as described in sub-clause 9. |  |  |

***** Next modification *****

### 10.3a.1	Downlink RLC data block

#### 10.3a.1.1	EGPRS downlink RLC data block

The EGPRS downlink RLC data blocks are formatted according to figure 10.3a.1.1.1.

|  |  |  | Bit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 1 |  |  |  |  |
|  |  |  | FBI | E |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| Length indicator |  |  |  |  |  |  | E | Octet 1 (note) (optional) |
| . . . |  |  |  |  |  |  |  | . . . |
| Length indicator |  |  |  |  |  |  | E | Octet M (optional) |
|  |  |  |  |  |  |  |  | Octet M+1 |
| RLC data |  |  |  |  |  |  |  | . . Octet K-1 |
| spare |  | DTR Blks |  | CI | TN/PDCH-pair |  |  | Octet K (optional) |
|  |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  | Octet N2-1 |
|  |  |  |  |  |  |  |  | Octet N2 |

NOTE:	If padding is used, then "Octet 1" shall be replaced by "Octet 7", see example in annex J.

Figure 10.3a.1.1.1: (EC-)EGPRS downlink RLC data block

#### 10.3a.1.2	EC-GSM-IoT downlink RLC data block

For an EC TBF, the downlink RLC data blocks are formatted according to figure 10.3a.1.2.1.

|  |  |  | Bit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 1 |  |  |  |  |
|  |  |  | FBI | E |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| Length indicator |  |  |  |  |  |  | E | Octet 1 (note) (optional) |
| . . . |  |  |  |  |  |  |  | . . . |
| Length indicator |  |  |  |  |  |  | E | Octet M (optional) |
|  |  |  |  |  |  |  |  | Octet M+1 |
| RLC data |  |  |  |  |  |  |  | . . Octet K |
|  |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  | Octet N2-1 |
|  |  |  |  |  |  |  |  | Octet N2 |

NOTE:	If padding is used, then "Octet 1" shall be replaced by "Octet 7", see example in annex J.

Figure 10.3a.1.2.1: EC-GSM-IoT downlink RLC data block

### 10.3a.2	Uplink RLC data block

#### 10.3a.2.1	EGPRS Uplink RLC data block

The EGPRS uplink RLC data block is formatted according to figure 10.3a.2.1.1.

|  |  |  | Bit |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 1 |  |  |  |  |  |
|  |  |  | TI | E |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |  |
| Length indicator |  |  |  |  |  |  |  | E | Octet 1 (note 1) (optional) |
| spare |  |  |  |  |  | Selected PLMN Index |  |  | Octet 2 (note 3) (optional) |
| . . . |  |  |  |  |  |  |  |  | . . . |
| Length indicator |  |  |  |  |  |  |  | E | Octet M (optional) |
|  |  |  |  |  |  |  |  |  | Octet M+1 \ |
|  |  |  | TLLI |  |  |  |  |  | Octet M+2 } (optional) |
|  |  |  |  |  |  |  |  |  | Octet M+3 / |
|  |  |  |  |  |  |  |  |  | Octet M+4 / |
| PFI |  |  |  |  |  |  |  | E | Octet M+5 / |  |
|  |  |  |  |  |  |  |  |  | Octet M+6 |
| RLC data |  |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  |  | Octet N2-1 |
|  |  |  |  |  |  |  |  |  | Octet N2 |

NOTE 1:	If padding is used, then "Octet 1" shall be replaced by "Octet 7", see example in annex J.

NOTE 2:	The field mapping convention for EGPRS (sub-clause 10.0b.3.2) applies. According to that, in particular regarding the TLLI field, the least significant byte of the TLLI value shall be mapped on octet M+1 and the most significant byte of the TLLI value shall be mapped on octet M+4 of the uplink EGPRS RLC data block.

NOTE 3:	This octet is only included if the value of the first instance of the Length indicator in the RLC data block is 123.

Figure 10.3a.2.1.1: Uplink EGPRS RLC data block

#### 10.3a.2.2	EC-GSM-IoT Uplink RLC data block

For an EC TBF, the uplink RLC data block is formatted according to figure 10.3a.2.2.1.

|  |  |  | Bit |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 1 |  |  |  |  |  |
|  |  |  | TI | E |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |  |
| Length indicator |  |  |  |  |  |  |  | E | Octet 1 (note 1) (optional) |
| spare |  |  |  |  |  | Selected PLMN Index |  |  | Octet 2 (note 3) (optional) |
| . . . |  |  |  |  |  |  |  |  | . . . |
| Length indicator |  |  |  |  |  |  |  | E | Octet M (optional) |
|  |  |  |  |  |  |  |  |  | Octet M+1 \ |
|  |  |  | TLLI |  |  |  |  |  | Octet M+2 } (optional) |
|  |  |  |  |  |  |  |  |  | Octet M+3 / |
|  |  |  |  |  |  |  |  |  | Octet M+4 / |
|  |  |  |  |  |  |  |  |  | Octet M+5 |
| RLC data |  |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  |  | Octet N2-1 |
|  |  |  |  |  |  |  |  |  | Octet N2 |

NOTE 1:	If padding is used, then "Octet 1" shall be replaced by "Octet 7", see example in annex J.

NOTE 2:	The field mapping convention for EC-GSM-IoT (sub-clause 10.0b.3.2) applies. According to that, in particular regarding the TLLI field, the least significant byte of the TLLI value shall be mapped on octet M+1 and the most significant byte of the TLLI value shall be mapped on octet M+4 of the uplink EC-GSM-IoT RLC data block.

NOTE 3:	This octet is only included if the value of the first instance of the Length indicator in the RLC data block is 123.

Figure 10.3a.2.2.1: Uplink EC-GSM-IoT RLC data block

***** Next modification *****

### 10.3a.3	EGPRS and EC-GSM-IoT Downlink RLC/MAC header

#### 10.3a.3.1	Header type 1: header for MCS-7, MCS-8 and MCS-9

For an EGPRS TBF without FANR activated, the EGPRS combined downlink RLC/MAC header for MCS-7, MCS-8 and MCS-9 (header type 1) shall be formatted according to figure 10.3a.3.1.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | RRBP |  | ES/P |  | USF |  |  | 1 |
| BSN1 |  | PR |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  | BSN1 | 4 |
| CPS |  |  |  |  | BSN2 |  |  | 5 |

Figure 10.3a.3.1.1: EGPRS downlink RLC data block header (FANR not activated)
for MCS-7, MCS-8 and MCS-9

For an EGPRS TBF with FANR activated and for an EGPRS2 TBF (see sub-clause 5.2.1), the downlink RLC/MAC header for MCS-7, MCS-8 and MCS-9 shall be formatted according to figure 10.3a.3.1.2.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | PANI | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  | PR |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  | BSN1 | 4 |
| CPS |  |  |  |  | BSN2 |  |  | 5 |

Figure 10.3a.3.1.2: EGPRS (FANR activated) / EGPRS2 downlink RLC data block header

For an EGPRS TBF without FANR activated in a DLMC configuration using an SNS of 8192, the EGPRS combined downlink RLC/MAC header for MCS-7, MCS-8 and MCS-9 (header type 1) shall be formatted according to figure 10.3a.3.1.3.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | BSN2 | RRBP | ES/P |  | USF |  |  | 1 |
| BSN1 |  |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  | BSN1 | 4 |
| CPS |  |  |  |  | BSN2 |  |  | 5 |

Figure 10.3a.3.1.3: DLMC EGPRS downlink RLC data block header (FANR not activated) 
for MCS-7, MCS-8 and MCS-9

For an EGPRS TBF with FANR activated in a DLMC configuration using an SNS of 8192 and for an EGPRS2 TBF in a DLMC configuration using an SNS of 8192, the downlink RLC/MAC header for MCS-7, MCS-8 and MCS-9 shall be formatted according to figure 10.3a.3.1.4.

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 |  | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | PANI | BSN2 | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  |  |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  | BSN1 | 4 |
| CPS |  |  |  |  |  | BSN2 |  |  | 5 |

Figure 10.3a.3.1.4: DLMC EGPRS (FANR activated) / DLMC EGPRS2 downlink RLC data block header for MCS-7, MCS-8 and MCS-9

For an EC TBF, the combined downlink RLC/MAC header for MCS-7, MCS-8 and MCS-9 (header type 1) shall be formatted according to figure 10.3a.3.1.5.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  |  | 7 |  | 6 |  |  |  | 5 |  | 4 |  |  | 3 |  | 2 |  | 1 |  |  | Octet |  |  |
| TFI |  |  | Spare |  |  |  | RRBP |  | ECS/P |  |  |  | USF |  |  |  |  |  |  | 1 |  |  |
| BSN1 |  |  |  |  |  | PR |  |  |  |  | TFI |  |  |  |  |  |  |  |  | 2 |  |  |
| Spare |  |  | BSN2 |  |  |  |  |  |  |  |  |  | BSN1 |  |  |  |  |  |  | 3 |  |  |
| Spare |  |  | RRBP |  |  |  |  |  | CPS |  |  |  |  |  |  |  |  |  |  | 4 |  |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |

Figure 10.3a.3.1.5: EC-GSM-IoT downlink RLC data block header for MCS-7, MCS-8 and MCS-9

#### 10.3a.3.2	Header type 2: header for MCS-6, MCS-5, DAS-5, DAS-6 and DAS-7

For an EGPRS TBF without FANR activated, the EGPRS combined downlink RLC/MAC header for MCS-5 and MCS-6 (header type 2) shall be formatted according to figure 10.3a.3.2.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | RRBP |  | ES/P |  | USF |  |  | 1 |
| BSN1 |  | PR |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
|  |  |  |  | CPS |  |  | BSN1 | 4 |

Figure 10.3a.3.2.1: EGPRS downlink RLC data block header (FANR not activated)
for MCS-5 and MCS-6

For an EGPRS TBF with FANR activated and for an EGPRS2 TBF (see sub-clause 5.2.1), the downlink RLC/MAC header for MCS-5 and MCS-6 shall be formatted according to figure 10.3a.3.2.2, whereas the RLC/MAC header for DAS-5, DAS-6 and DAS-7 shall be formatted as shown on figure 10.3a.3.2.2.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | PANI | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  | PR |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
|  |  |  |  | CPS |  |  | BSN1 | 4 |

Figure 10.3a.3.2.2: EGPRS (FANR activated) / EGPRS2 downlink RLC data block header
for MCS-5, MCS-6, DAS-5, DAS-6 and DAS-7

For an EGPRS TBF without FANR activated in a DLMC configuration using an SNS of 8192, the EGPRS combined downlink RLC/MAC header for MCS-5 and MCS-6 (header type 2) shall be formatted according to figure 10.3a.3.2.3.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | spare | RRBP | ES/P |  | USF |  |  | 1 |
| BSN1 |  |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
|  |  |  |  | CPS |  |  | BSN1 | 4 |

Figure 10.3a.3.2.3: DLMC EGPRS downlink RLC data block header (FANR not activated)
for MCS-5 and MCS-6

For an EGPRS TBF with FANR activated in a DLMC configuration using an SNS of 8192 and for an EGPRS2 TBF in a DLMC configuration using an SNS of 8192, the downlink RLC/MAC header for MCS-5, MCS-6, DAS-5, DAS-6 and DAS-7  shall be formatted according to figure 10.3a.3.2.4.

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 |  | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | PANI | spare | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  |  |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  | 3 |
|  |  |  |  |  | CPS |  |  | BSN1 | 4 |

Figure 10.3a.3.2.4: DLMC EGPRS (FANR activated) / DLMC EGPRS2 downlink RLC data block header
for MCS-5, MCS-6, DAS-5, DAS-6 and DAS-7

For an EC TBF, the EGPRS combined downlink RLC/MAC header for MCS-5 and MCS-6 (header type 2) shall be formatted according to figure 10.3a.3.2.5.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 | 5 | 4 | 3 |  | 2 |  | 1 | Octet |
| TFI | Spare |  | RRBP |  | ECS/P |  | USF |  |  |  |  | 1 |
| BSN1 |  |  | PR |  |  | TFI |  |  |  |  |  | 2 |
| RRBP |  |  | CPS |  |  |  | BSN1 |  |  |  |  | 3 |
|  |  |  |  |  |  | Spare |  |  |  |  |  | 4 |

Figure 10.3a.3.2.5: EC-GSM-IoT downlink RLC data block header for MCS-5 and MCS-6

#### 10.3a.3.3	Header type 3: header for MCS-4, MCS-3, MCS-2, MCS-1 and MCS-0 case

For an EGPRS TBF without FANR activated, the EGPRS combined downlink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 (header type 3) shall be formatted according to figure 10.3a.3.3.1.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI | RRBP |  |  |  |  | ES/P |  |  | USF |  |  |  | 1 |
| BSN1 |  |  | PR |  |  |  | TFI |  |  |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
|  |  | SPB |  |  | CPS |  |  |  |  |  | BSN1 |  | 4 |

Figure 10.3a.3.3.1: EGPRS downlink RLC data block header (FANR not activated)
for MCS-1, MCS-2, MCS-3 and MCS-4

For an EGPRS TBF with FANR activated and for an EGPRS2 TBF (see sub-clause 5.2.1), the downlink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 shall be formatted according to figure 10.3a.3.3.2.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI | PANI |  | CES/P |  |  |  |  |  | USF |  |  |  | 1 |
| BSN1 |  |  | PR |  |  |  | TFI |  |  |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
|  |  | SPB |  |  | CPS |  |  |  |  |  | BSN1 |  | 4 |

Figure 10.3a.3.3.2: EGPRS (FANR activated) / EGPRS2 downlink RLC data block header
for MCS-1, MCS-2, MCS-3 and MCS-4

For a TBF in RTTI configuration, the downlink RLC/MAC control block header for MCS-0 shall be formatted as defined in figure 10.3a.3.3.3.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| Payload type | Spare |  | RRBP | S/P | USF |  |  | 1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 |
|  | 0 | 0 | CPS |  |  |  | Spare | 4 |
| NOTE2: Field indicated as ‘0’ will be replaced by an 18 bit CRC during the channel coding, see sub-clause 5.1.4a.1.4 in 3GPP TS 45.003. |  |  |  |  |  |  |  |  |

Figure 10.3a.3.3.3: Downlink RLC/MAC control block header
for MCS-0.

For an EGPRS TBF without FANR activated in a DLMC configuration using an SNS of 8192, the EGPRS combined downlink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 (header type 3) shall be formatted according to figure 10.3a.3.3.4.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI | spare |  | RRBP |  | ES/P |  |  | USF |  |  |  | 1 |
| BSN1 |  |  |  |  |  | TFI |  |  |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  |  | 3 |
|  |  | SPB |  | CPS |  |  |  |  |  | BSN1 |  | 4 |

Figure 10.3a.3.3.4: DLMC EGPRS downlink RLC data block header (FANR not activated)
for MCS-1, MCS-2, MCS-3 and MCS-4

For an EGPRS TBF with FANR activated in a DLMC configuration using an SNS of 8192 and for an EGPRS2 TBF in a DLMC configuration using an SNS of 8192, the downlink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 shall be formatted according to figure 10.3a.3.3.2.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI | PANI |  | spare |  | CES/P |  |  |  | USF |  |  |  | 1 |
| BSN1 |  |  |  |  |  |  | TFI |  |  |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
|  |  | SPB |  |  | CPS |  |  |  |  |  | BSN1 |  | 4 |

Figure 10.3a.3.3.5: DLMC EGPRS (FANR activated) / DLMC EGPRS2 downlink RLC data block header
for MCS-1, MCS-2, MCS-3 and MCS-4

For an EC TBF, the EGPRS combined downlink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 (header type 3) shall be formatted according to figure 10.3a.3.3.6. Note that different USF values may be included in the different transmissions if blind physical layer transmissions are used in the downlink (when CC2, CC3 or CC4 is assigned for the transfer).

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  |  | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | Octet |  |
| TFI | Spare |  |  | RRBP |  | ECS/P |  |  | USF |  |  |  |  |  | 1 |  |
| BSN1 |  |  |  | PR |  |  | TFI |  |  |  |  |  |  |  | 2 |  |
| Spare | CC |  |  |  |  | SPB |  |  | BSN1 |  |  |  |  |  | 3 |  |
|  | Spare |  |  |  |  | RRBP |  |  | CPS |  |  |  |  |  | 4 |  |

Figure 10.3a.3.3.6: EC-GSM-IoT downlink RLC data block header for MCS-1, MCS-2, MCS-3 and MCS-4

***** Next modification *****

### 10.3a.4	EGPRS and EC-GSM-IoT Uplink RLC/MAC header

#### 10.3a.4.1	Header type 1: header for MCS-7, MCS-8 and MCS-9

The EGPRS combined uplink RLC/MAC header for MCS-7, MCS-8 and MCS-9 (header type 1) shall be formatted according to figure 10.3a.4.1.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  | 2 |
| BSN2 |  | BSN1 |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  | 4 |
| Spare | PI | RSB | CPS |  |  |  |  | 5 |
|  |  | Spare |  |  |  |  |  | 6 |

Figure 10.3a.4.1.1: EGPRS uplink RLC data block header (FANR not activated)
for MCS-7, MCS-8 and MCS-9.

For a EGPRS TBF with FANR activated, the uplink RLC/MAC header for MCS-7, MCS-8 and MCS-9 shall be formatted according to figure 10.3a.4.1.2.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  | 2 |
| BSN2 |  | BSN1 |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  | 4 |
| PANI | PI | RSB | CPS |  |  |  |  | 5 |
|  |  | Spare |  |  |  |  |  | 6 |

Figure 10.3a.4.1.2: EGPRS uplink RLC data block header (FANR activated)
for MCS-7, MCS-8 and MCS-9.

The EC-GSM-IoT combined uplink RLC/MAC header for MCS-7, MCS-8 and MCS-9 (header type 1) shall be formatted according to figure 10.3a.4.1.3.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI |  |  | Countdown Value |  |  |  |  |  | FOI |  | RI | 1 |
| BSN1 |  |  |  |  |  |  |  | TFI |  |  |  | 2 |
| Spare |  |  |  |  |  | BSN2 |  |  |  |  |  | 3 |
| DCCE |  |  |  | CPS |  |  |  |  |  |  |  | 4 |
| Spare |  |  |  | rTLLI |  |  |  |  |  | DCCE |  | 5 |
|  |  | Spare |  |  |  |  |  |  |  |  |  | 6 |

Figure 10.3a.4.1.3: EC-GSM-IoT uplink RLC data block header for MCS-7, MCS-8 and MCS-9.

#### 10.3a.4.2	Header type 2: header for MCS-6 and MCS-5

The EGPRS combined uplink RLC/MAC header for MCS-5 and MCS-6 (header type 2) shall be formatted according to figure 10.3a.4.2.1.

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI |  | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  |  | 2 |
| CPS |  | BSN1 |  |  |  |  |  |  | 3 |
| Spare |  |  |  |  | PI | RSB | CPS |  | 4 |
|  |  |  | Spare |  |  |  |  |  | 5 |

Figure 10.3a.4.2.1: EGPRS uplink RLC data block header (FANR not activated)
for MCS-5 and MCS-6

For an EGPRS TBF with FANR activated and for an EGPRS2 TBF, the uplink RLC/MAC header for MCS-5 and MCS-6 shall be formatted according to figure 10.3a.4.2.2.

| Bit |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI |  | Countdown Value |  |  |  |  | SI |  | R | 1 |
| BSN1 |  |  |  |  |  | TFI |  |  |  | 2 |
| CPS |  | BSN1 |  |  |  |  |  |  |  | 3 |
| Spare |  |  |  | PANI |  | PI | RSB | CPS |  | 4 |
|  |  |  | Spare |  |  |  |  |  |  | 5 |

Figure 10.3a.4.2.2: EGPRS (FANR activated) / EGPRS2 uplink RLC data block header 
for MCS-5 and MCS-6

The EC-GSM-IoT combined uplink RLC/MAC header for MCS-5 and MCS-6 (header type 2) shall be formatted according to figure 10.3a.4.2.3.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  |  | 7 |  | 6 | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI |  |  |  |  | Countdown Value |  |  |  |  | FOI |  | RI | 1 |
| BSN1 |  |  |  |  |  |  |  |  | TFI |  |  |  | 2 |
| Spare |  | DCCE |  |  |  |  |  |  | CPS |  |  |  | 3 |
| Spare |  |  |  |  |  |  | rTLLI |  |  |  |  |  | 4 |
|  |  |  |  |  |  | Spare |  |  |  |  |  |  | 5 |

Figure 10.3a.4.2.3: EC-GSM-IoT uplink RLC data block header for MCS-5 and MCS-6

#### 10.3a.4.3	Header type 3 : header for MCS-4, MCS-3, MCS-2 and MCS-1

The EGPRS combined uplink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 (header type 3) shall be formatted according to figure 10.3a.4.3.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  | 2 |
| CPS |  | BSN1 |  |  |  |  |  | 3 |
|  | Spare | PI | RSB | SPB |  | CPS |  | 4 |

Figure 10.3a.4.3.1: EGPRS uplink RLC data block header (FANR not activated)
for MCS-1, MCS-2, MCS-3 and MCS-4

For an EGPRS TBF with FANR activated and for an EGPRS2 TBF, the uplink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 shall be formatted according to figure 10.3a.4.3.2.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  | 2 |
| CPS |  | BSN1 |  |  |  |  |  | 3 |
|  | PANI | PI | RSB | SPB |  | CPS |  | 4 |

Figure 10.3a.4.3.2: EGPRS (FANR activated)/EGPRS2 uplink RLC data block header
for MCS-1, MCS-2, MCS-3 and MCS-4

The EC-GSM-IoT combined uplink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 (header type 3) shall be formatted according to figure 10.3a.4.3.3.

|  |  |  |  | Bit |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |  |
| TFI |  | Countdown Value |  |  |  | FOI | RI | 1 |  |
| BSN1 |  |  |  |  | TFI |  |  | 2 |  |
| DCCE |  |  | SPB |  | CPS |  |  | 3 |  |
|  | Spare |  | rTLLI |  |  |  | DCCE | 4 |  |

Figure 10.3a.4.3.3: EC-GSM-IoT uplink RLC data block header for MCS-1, MCS-2, MCS-3 and MCS-4

***** Next modification *****

### 10.4.2	Retry (R) bit

The Retry (R) bit shall indicate whether the mobile station transmitted the CHANNEL REQUEST message (see 3GPP TS 44.018), PACKET CHANNEL REQUEST message, EGPRS PACKET CHANNEL REQUEST message or MPRACH PACKET CHANNEL REQUEST message one time or more than one time during its most recent channel access. The mobile station shall send the same value for the R bit in each uplink RLC/MAC block of the TBF.

Table 10.4.2.1: Retry (R) bit

| bit 1 | Retry (R) bit |
| --- | --- |
| 0 | MS sent channel request message once |
| 1 | MS sent channel request message twice or more |

***** Next modification *****

### 10.4.4	Supplementary/Polling (S/P) Bit

The S/P bit is used to indicate whether the RRBP field is valid or not valid in downlink GPRS RLC data blocks and downlink RLC/MAC control blocks using the coding scheme of CS-1and in downlink EC-GSM-IoT RLC/MAC control blocks on EC-PACCH/D.

Table 10.4.4.1: Supplementary/Polling (S/P) bit - GPRS case and RLC/MAC control

| bit 4 | S/P |
| --- | --- |
| 0 | RRBP field is not valid |
| 1 | RRBP field is valid |

***** Next modification *****

### 10.4.4c	EC-GSM-IoT Supplementary/Polling (ECS/P) Field

The ECS/P field is used to indicate whether the RRBP field is valid or not valid and, if valid, if a quality report is to be included in the next uplink control block.

Table 10.4.4c.1: EC-GSM-IoT Supplementary/Polling (ECS/P) field

| bits 5 4 | ECS/P |
| --- | --- |
| 0 0 | RRBP field is not valid (no Polling) |
| 0 1 | RRBP field is valid, Ack/Nack report to be included |
| 1 0 | RRBP field is valid, Ack/Nack report to be included. If there is enough room in the RLC/MAC block, a channel quality report shall also be included. |
| 1 1 | Reserved |

***** Next modification *****

### 10.4.5	Relative Reserved Block Period (RRBP) field

The RRBP value specifies a single uplink block in which the mobile station shall transmit either a PACKET CONTROL ACKNOWLEDGEMENT message, an EC PACKET CONTROL ACKNOWLEDGEMENT message, a PACCH block or an EC-PACCH block to the network. If the RRBP field is received as part of an RLC/MAC block containing an RLC/MAC control block, the mobile station shall transmit a PACKET CONTROL ACKNOWLEDGEMENT or an EC PACKET CONTROL ACKNOWLEDGEMENT message in the uplink radio block specified, except if:

-	The received message is a Packet Paging Request, Packet Access Reject, or Packet Queuing Notification message, or

-	It is specified elsewhere that the mobile station shall not respond to the polling request.

If the RRBP field is received as part of an RLC/MAC block containing an RLC/MAC control block containing a Packet Paging Request, Packet Access Reject, or Packet Queuing Notification message, or it is specified elsewhere that the mobile station shall not respond to the polling request, the mobile station shall ignore this RRBP field. The mobile station shall only react on RLC/MAC control blocks containing a valid RRBP field if the mobile station is addressed either in the downlink RLC/MAC control block header or in the control message itself. If the control message is segmented into more than one downlink RLC/MAC control blocks the mobile station shall react only on RLC/MAC control blocks containing a valid RRBP field if the mobile station is addressed in the downlink RLC/MAC control block header.

If the mobile station receives two or more RLC/MAC blocks containing an RLC/MAC control message with different RRBP values such that they specify the same uplink block, the mobile station shall transmit one PACKET CONTROL ACKNOWLEDGEMENT or an EC PACKET CONTROL ACKNOWLEDGEMENT message in the specified uplink radio block.

If the RRBP field is received as part of a RLC/MAC block containing an RLC data block, the mobile station shall transmit a PACCH block or an EC-PACCH block in the specified uplink radio block. If the mobile station receives two or more RLC/MAC blocks containing an RLC data block with different RRBP values such they specify the same uplink radio block, the mobile station shall transmit one (EC-)PACCH block in the specified uplink radio block.

If the mobile station receives an RLC data block and an RLC/MAC control block with different RRBP values such that they specify the same uplink radio block, the mobile station shall transmit a PACKET CONTROL ACKNOWLEDGEMENT or an EC PACKET CONTROL ACKNOWLEDGEMENT message in the specified uplink radio block.

If no uplink control timeslot is assigned to the mobile station and EFTA is not used at the time the RRBP field is received, the mobile station shall transmit the uplink radio block on the same timeslot as the block where the RRBP was received (in case of a BTTI configuration) or on the corresponding uplink PDCH pair to the downlink PDCH pair where the block containing the RRBP was received (in case of a RTTI configuration). If EFTA is used at the time the RRBP field is received however, the mobile station shall instead transmit the uplink radio block according to the uplink radio block transmission order as described in Annex N regardless of the timeslot or PDCH-pair where the block containing the RRBP was received.

If an uplink control timeslot is assigned to the mobile station, the mobile station shall transmit the uplink radio block on this uplink control timeslot (in case of a BTTI configuration) or on the uplink PDCH pair the uplink control timeslot belongs to (in case of an RTTI configuration).

NOTE:	In case of an RTTI configuration, the network should not poll the mobile station on a downlink PDCH pair for which no corresponding uplink PDCH pair exists.

After receiving an RLC/MAC block containing a valid RRBP field the mobile station need not monitor the USF in the associated downlink RLC/MAC block appearing just before the uplink block it shall transmit. However, when Extended Dynamic Allocation or Shifted USF operation is used, the corresponding USF monitoring procedure shall apply as described in sub-clause 8.1.1.2.1 and sub-clause 8.1.1.2.4 respectively.

After receiving an RLC/MAC block containing a valid RRBP field for an EC TBF, the mobile station need not monitor the downlink RLC/MAC blocks during the radio block periods where it shall transmit the uplink response.

A polled control message shall always be sent in the uplink block specified by the corresponding valid RRBP field of a downlink RLC/MAC control block, and not in any other uplink block that may be allocated to the mobile station.

The network should use the RRBP field to schedule the transmission of a PACKET CONTROL ACKNOWLEDGEMENT or an EC PACKET CONTROL ACKNOWLEDGEMENT message or an uplink (EC-)PACCH block as follows:

-	For BTTI configuration, no later than the second last BTTI radio block, B(x-2) mod 12, before the first BTTI radio block, B(x), where the mobile station shall be ready to transmit and receive using a new assignment.

-	For RTTI configuration, no later than the second last RTTI radio block, B(x-2)b, before the first RTTI radio block, B(x)a, where the mobile station shall be ready to transmit and receive using a new assignment or no later than the second last RTTI radio block, B(x-1)a, before the first RTTI radio block, B(x)b, where the mobile station shall be ready to transmit and receive using a new assignment.

A mobile station that is scheduled an uplink block later than these times, may omit responding to the polling request.  If a new assignment specifies an uplink TBF, and the TTI configuration of the uplink TBF specified is the same as the TTI configuration of the ongoing uplink TBF that this assignment modifies, or if the assignment does not specify any uplink TBF, then the mobile station may delay the access using the new assignment in order to respond to the polling request.

The network should not use the RRBP field to schedule the transmission of PACKET CONTROL ACKNOWLEDGEMENT, EC PACKET CONTROL ACKNOWLEDGEMENT messages or uplink (EC-) PACCH blocks, in such way, that a mobile station has more than three such uplink blocks pending for transmission at any instant. A mobile station, that is scheduled such uplink blocks more frequent than that, may omit responding to the excessive polling requests.

The RRBP field shall be interpreted according to the TTI configuration in use for the mobile station on the PDCH (or on the PDCH pair) where the block containing the RRBP field is received at the time it is received, and, for a BTTI configuration, according to whether FANR is activated or not for the mobile station.

For non EC TBFs with FANR not activated, Table 10.4.5.1 specifies the coding of the RRBP field indicating the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block. The delay is relative to the first TDMA frame (N) of the downlink block containing the RRBP value. For definition of TDMA frame numbering, see 3GPP TS 45.002.

Table 10.4.5.1: Relative Reserved Block Period (RRBP) field (FANR not activated)

| bit 6-5/7-6 | Full-rate PDCH uplink block with TDMA frame number | Full-rate PDCH uplink block with TDMA frame number (DLMC configuration with SNS 2048) | Half-rate PDCH uplink block with TDMA frame number |
| --- | --- | --- | --- |
| 0 0 | (N+13) mod 2715648 | (N+13) mod 2715648 | reserved |
| 0 1 | (N+17 or N+18) mod 2715648 | (N+17 or N+18) mod 2715648 | (N+17 or N+18) mod 2715648 |
| 1 0 | (N+21 or N+22) mod 2715648 | (N+21 or N+22) mod 2715648 | reserved |
| 1 1 | (N+26) mod 2715648 | (N+26) mod 2715648 | (N+26) mod 2715648 |

If the mobile station is operating on a half-rate PDCH and it receives an RLC/MAC block with a reserved RRBP value, it shall regard the RRBP field as not valid and shall ignore the polling.

For TBFs with FANR activated, Table 10.4.5.2 specifies the coding of the RRBP field indicating the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block.

Table 10.4.5.2: Relative Reserved Block Period (RRBP) field (FANR activated)

| bit 6-5 | PDCH uplink block with TDMA frame number (BTTI) | PDCH uplink block with TDMA frame number (RTTI) |
| --- | --- | --- |
| 0 0 | (N+13) mod 2715648 | reserved |
| 0 1 | (N+8 or N+9) mod 2715648 | (N+8 or N+9) mod 2715648 |
| 1 0 | reserved | (N+6 or N+7) mod 2715648 |
| 1 1 | reserved | reserved |
| NOTE: Table 10.4.5.2 only applies to RLC/MAC control blocks encoded using CS-1, while the CES/P field is used in case of polling in RLC data blocks (see sub-clause 10.4.4b). |  |  |

In downlink RLC/MAC Control blocks encoded using MSC-0, a 1-bit RRBP is defined (see subclause 10.3a.3.3). In this case the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block is indicated in Table 10.4.5.3.

Table 10.4.5.3: Relative Reserved Block Period (RRBP) field – 1 bit field

| bit 5 | PDCH uplink block with TDMA frame number |
| --- | --- |
| 0 | (N+6 or N+7) mod 2715648 |
| 1 | (N+8 or N+9) mod 2715648 |

In a DLMC configuration using an SNS of 8192 for a downlink TBF where FANR is not activated, Table 10.4.5.4 specifies the RRBP field in the RLC data block header (see sub-clause 10.3a.3.1, 10.3a.3.2 and 10.3a.3.3) by indicating the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block. The delay is relative to the first TDMA frame (N) of the downlink block containing the RRBP value. For definition of TDMA frame numbering, see 3GPP TS 45.002.

Table 10.4.5.4: Relative Reserved Block Period (RRBP) field (FANR not activated) – DLMC Configuration

| bit 6 | PDCH uplink block with TDMA frame number |
| --- | --- |
| 0 | (N+6 or N+7) mod 2715648 |
| 1 | (N+8 or N+9) mod 2715648 |

For EC TBFs, Table 10.4.5.5 specifies the coding of the RRBP field if it is received as part of an RLC/MAC block containing an RLC/MAC data block indicating the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block according to its uplink Coverage Class. The delay is relative to the first TDMA frame (N) of the downlink block containing the last blind physical layer transmission of the EC-PACCH or EC-PDTCH message, according to the USED_DL_COVERAGE_CLASS field. For definition of TDMA frame numbering, see 3GPP TS 45.002 [13].

Table 10.4.5.5: Relative Reserved Block Period (RRBP) field (EC TBF mode) if received as part of an RLC/MAC block containing an RLC/MAC data block.

| bit 8 7 6 | Full-rate PDCH uplink block with TDMA frame number given UL CC1 or CC2 | Full-rate PDCH uplink block with TDMA frame number given UL CC3 | Full-rate PDCH uplink block with TDMA frame number given UL CC4 |
| --- | --- | --- | --- |
| 0 0 0 | (N+13) mod 2715648 | (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 | (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 or (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 |
| 0 0 1 | (N+17 or N+18) mod 2715648 | (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 | (N+30 or N+31) mod 2715648 or (N+34 or N+35) mod 2715648 or (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 |
| 0 1 0 | (N+21 or N+22) mod 2715648 | (N+30 or N+31) mod 2715648 or (N+34 or N+35 ) mod 2715648 | (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 or (N+56 or N+57) mod 2715648 or (N+60 or N+61) mod 2715648 |
| 0 1 1 | (N+26) mod 2715648 | (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 | (N+65) mod 2715648 or (N+69 or N+70) mod 2715648 or (N+73 or N+74) mod 2715648 or (N+78) mod 2715648 |
| 1 0 0 | (N+30 or N+31) mod 2715648 | (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 | (N+82 or N+83) mod 2715648 or (N+86 or N+87) mod 2715648 or (N+91) mod 2715648 or (N+95 or N+96) mod 2715648 |
| 1 0 1 | (N+34 or N+35) mod 2715648 | (N+56 or N+57) mod 2715648 or (N+60 or N+61) mod 2715648 | (N+99 or N+100) mod 2715648 or (N+104) mod 2715648 or (N+108 or N+109) mod 2715648 or (N+112 or N+113) mod 2715648 |
| 1 1 0 | (N+39) mod 2715648 | (N+65) mod 2715648 or (N+69 or N+70) mod 2715648 | (N+117) mod 2715648 or (N+121 or N+122) mod 2715648 or (N+125 or N+126) mod 2715648 or (N+130) mod 2715648 |
| 1 1 1 | (N+43 or N+44) mod 2715648 | (N+73 or N+74) mod 2715648 or (N+78) mod 2715648 | (N+134 or N+135) mod 2715648 or (N+138 or N+139) mod 2715648 or (N+143) mod 2715648 or (N+147 or N+148) mod 2715648 |
| NOTE: Each RRBP value in Table 10.4.5.5 will assign only one valid uplink block given the downlink coverage class used, i.e. when the poll is received, and the uplink coverage class used. See 3GPP TS 45.002 [13] for the coverage class dependent channel mappings. |  |  |  |

For EC TBFs, Table 10.4.5.6 specifies the coding of the RRBP field if it is received as part of an RLC/MAC block containing an RLC/MAC control block indicating the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block according to its uplink Coverage Class. The delay is relative to the first TDMA frame (N) of the downlink block containing the last blind physical layer transmission of the EC-PACCH or EC-PDTCH message, according to the USED_DL_COVERAGE_CLASS field. For definition of TDMA frame numbering, see 3GPP TS 45.002 [13].

Table 10.4.5.6: Relative Reserved Block Period (RRBP) field (EC TBF mode) if received as part of an RLC/MAC block containing an RLC/MAC control block

| bit 3 2 | Full-rate PDCH uplink block with TDMA frame number given UL CC1 or CC2 | Full-rate PDCH uplink block with TDMA frame number given UL CC3 | Full-rate PDCH uplink block with TDMA frame number given UL CC4 |
| --- | --- | --- | --- |
| 0 0 | (N+13) mod 2715648 | (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 | (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 or (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 |
| 0 1 | (N+17 or N+18) mod 2715648 | (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 | (N+30 or N+31) mod 2715648 or (N+34 or N+35) mod 2715648 or (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 |
| 1 0 | (N+21 or N+22) mod 2715648 | (N+30 or N+31) mod 2715648 or (N+34 or N+35 ) mod 2715648 | (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 or (N+56 or N+57) mod 2715648 or (N+60 or N+61) mod 2715648 |
| 1 1 | (N+26) mod 2715648 | (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 | (N+65) mod 2715648 or (N+69 or N+70) mod 2715648 or (N+73 or N+74) mod 2715648 or (N+78) mod 2715648 |
| NOTE: Each RRBP value in Table 10.4.5.6 will assign only one valid uplink block given the downlink coverage class used, i.e. when the poll is received, and the uplink coverage class used. See 3GPP TS 45.002 [13] for the coverage class dependent channel mappings. |  |  |  |

***** Next modification *****

### 10.4.6	Countdown Value (CV) field

The Countdown Value (CV) field is sent by the mobile station to allow the network to calculate the number of RLC data blocks remaining for the current uplink RLC entity. The CV value shall be calculated according to the process described in sub-clause 9.3.1. The CV field is 4 bits in length and is encoded as a binary number with range 0 to 15.

A mobile station with an uplink EC TBF is required to send the Countdown Value in an RLC uplink data block in order to inform the network that there are more remaining RLC data blocks for the uplink EC TBF than it has previously signalled to the network. When transmitting the last RLC data block, the CV equal to zero shall always be included. The Follow-On Indicator (FOI) shall indicate the presence of a valid CV field.

### 10.4.6a	Follow-On Indicator field (FOI)

The Follow-On Indicator bit is sent from the mobile station to the network to indicate the presence of a valid Countdown Value field in the uplink RLC data block header during an uplink EC TBF.

Table 10.4.6a.1: Follow On Indicator field

| bit 2 | FOI |
| --- | --- |
| 0 | Countdown Value not present |
| 1 | Countdown Value present |

***** Next modification *****

### 10.4.7	Payload Type field

The Payload Type field shall indicate the type of data contained in remainder of the RLC/MAC block. The encoding of the Payload Type field is shown in table 10.4.7.1.

Table 10.4.7.1: Payload Type field

| bit 8 7 | Payload Type |
| --- | --- |
| 0 0 | RLC/MAC block contains an RLC data block |
| 0 1 | RLC/MAC block contains an RLC/MAC control block that does not include the optional octets of the RLC/MAC control header |
| 10 | In the downlink direction, the RLC/MAC block contains an RLC/MAC control block that includes the optional first octet of the RLC/MAC control header. In the uplink direction, this value is reserved. |
| 1 1 | Reserved. In this version of the protocol, the mobile station shall ignore all fields of the RLC/MAC block except for the USF field |

In downlink RLC/MAC Control blocks encoded using MSC-0 the encoding of the Payload Type field is shown in table 10.4.7.2.

Table 10.4.7.2: Payload Type field, MCS-0

| bit 8 | Payload Type |
| --- | --- |
| 0 | RLC/MAC block contains an RLC/MAC control block that does not include the optional octets of the RLC/MAC control block message shown in figure 10.3.1.2.1 |
| 1 | RLC/MAC block contains an RLC/MAC control block that includes at least the first optional octet of the RLC/MAC control block message shown in figure 10.3.1.2.1 |

In downlink RLC/MAC Control blocks on EC-PACCH/D, the encoding of the Payload Type field is shown in table 10.4.7.3.

Table 10.4.7.3: Payload Type field on EC-PACCH/D

| bit 7 | Payload Type |
| --- | --- |
| 0 | The RLC/MAC control block, including the normal MAC header, is coded according to figure 10.3.1.3.1 |
| 1 | The RLC/MAC control block, including the extended MAC header, is coded according to figure 10.3.1.3.2 |

***** Next modification *****

### 10.4.8a	Coding and Puncturing Scheme indicator field (CPS)

In EGPRS and EC-GSM-IoT headers, the Coding and Puncturing Scheme indicator field is used to indicate the kind of channel coding and puncturing used for data blocks (see 3GPP TS 45.003).

#### 10.4.8a.1	Header type 1

Table 10.4.8a.1.1: Coding and Puncturing Scheme indicator field for Header type 1 in EGPRS TBF, EC TBF or downlink EGPRS2 TBF

| bits 54321 | CPS |
| --- | --- |
| 00000 | (MCS-9/P1 ; MCS-9/P1) (see NOTE 2) |
| 00001 | (MCS-9/P1 ; MCS-9/P2) (see NOTE 2) |
| 00010 | (MCS-9/P1 ; MCS-9/P3) (see NOTE 2) |
| 00100 | (MCS-9/P2 ; MCS-9/P1) (see NOTE 2) |
| 00101 | (MCS-9/P2 ; MCS-9/P2) (see NOTE 2) |
| 00110 | (MCS-9/P2 ; MCS-9/P3) (see NOTE 2) |
| 01000 | (MCS-9/P3 ; MCS-9/P1) (see NOTE 2) |
| 01001 | (MCS-9/P3 ; MCS-9/P2) (see NOTE 2) |
| 01010 | (MCS-9/P3 ; MCS-9/P3) (see NOTE 2) |
| 01011 | (MCS-8/P1 ; MCS-8/P1) |
| 01100 | (MCS-8/P1 ; MCS-8/P2) |
| 01101 | (MCS-8/P1 ; MCS-8/P3) |
| 01110 | (MCS-8/P2 ; MCS-8/P1) |
| 01111 | (MCS-8/P2 ; MCS-8/P2) |
| 10000 | (MCS-8/P2 ; MCS-8/P3) |
| 10001 | (MCS-8/P3 ; MCS-8/P1) |
| 10010 | (MCS-8/P3 ; MCS-8/P2) |
| 10011 | (MCS-8/P3 ; MCS-8/P3) |
| 10100 | (MCS-7/P1 ; MCS-7/P1) |
| 10101 | (MCS-7/P1 ; MCS-7/P2) |
| 10110 | (MCS-7/P1 ; MCS-7/P3) |
| 10111 | (MCS-7/P2 ; MCS-7/P1) |
| 11000 | (MCS-7/P2 ; MCS-7/P2) |
| 11001 | (MCS-7/P2 ; MCS-7/P3) |
| 11010 | (MCS-7/P3 ; MCS-7/P1) |
| 11011 | (MCS-7/P3 ; MCS-7/P2) |
| 11100 | (MCS-7/P3 ; MCS-7/P3) |
|  | All the other values are reserved for future use |
| NOTE 1: The bit numbering is relative to the field position. NOTE 2: MCS-9 shall only be used for an EGPRS TBF, for an EC TBF or for a downlink EGPRS2-B TBF. |  |

#### 10.4.8a.2	Header type 2

In EGPRS, EC-GSM-IoT and in EGPRS2-A uplink, the Coding and Puncturing Scheme indicator field indicates the kind of channel coding and puncturing used for data block as defined in table 10.4.8a.2.1.

Table 10.4.8a.2.1: Coding and Puncturing Scheme indicator field for Header type 2 in EGPRS TBF, EC TBF or uplink EGPRS2-A TBF

| bits 321 | (first block) CPS |
| --- | --- |
| 000 | MCS-6/P1 |
| 001 | MCS-6/P2 |
| 010 | MCS-6/P1 with 6 octets padding (see NOTE 2) |
| 011 | MCS-6/P2 with 6 octets padding (see NOTE 2) |
| 100 | MCS-5/P1 |
| 101 | MCS-5/P2 |
| 110 | MCS-6/P1 with 10 octets padding (see NOTE 3) |
| 111 | MCS-6/P2 with 10 octets padding (see NOTE 3) |
| NOTE 1: The bit numbering is relative to the field position. NOTE 2: MCS-6 with 6 octets padding shall only be used for an EGPRS TBF or, in case of an uplink EGPRS2-A TBF, for retransmission of blocks originally transmitted using EGPRS. NOTE 3: MSC-6 with 10 octets padding shall only be used for an uplink EGPRS2-A TBF or, in case of an uplink EGPRS TBF, for retransmission of blocks originally transmitted using EGPRS2-A. |  |

In EGPRS2 downlink, if the Downlink EGPRS Level assigned (see Table 11.2.7.1 and Table 12.10f.1) is EGPRS2-A, the Coding and Puncturing Scheme indicator field indicates the kind of channel coding and puncturing used for data block as defined in table 10.4.8a.2.2.

Table 10.4.8a.2.2: Coding and Puncturing Scheme indicator field for 
Header type 2 in downlink EGPRS2-A TBF

| bits 321 | CPS |
| --- | --- |
| 000 | MCS-6/P1 (see NOTE 2) |
| 001 | MCS-6/P2 (see NOTE 2) |
| 010 | DAS-5/P1 |
| 011 | DAS-5/P2 |
| 100 | DAS-6/P1 |
| 101 | DAS-6/P2 |
| 110 | DAS-7/P1 |
| 111 | DAS-7/P2 |
| NOTE 1: The bit numbering is relative to the field position. NOTE 2: In EGPRS2-A downlink, MCS-6 shall be used only for retransmissions of blocks originally transmitted using EGPRS. |  |

In EGPRS2 downlink, if the Downlink EGPRS Level assigned (see Table 11.2.7.1 and Table 12.10f.1) is EGPRS2-B, the Coding and Puncturing Scheme indicator field indicates the kind of channel coding and puncturing used for data block as defined in table 10.4.8a.2.3.

Table 10.4.8a.2.3: Coding and Puncturing Scheme indicator field for 
Header type 2 in downlink EGPRS2-B TBF

| bits 321 | CPS |
| --- | --- |
| 000 | MCS-6/P1 |
| 001 | MCS-6/P2 |
| 010 | DAS-5/P1 |
| 011 | DAS-5/P2 |
| 100 | DAS-6/P1 |
| 101 | DAS-6/P2 |
|  | All the other values are reserved for future use |
| NOTE: The bit numbering is relative to the field position. |  |

#### 10.4.8a.3	Header type 3

Table 10.4.8a.3.1: Coding and Puncturing Scheme indicator field for Header type 3 (non EC TBF)

| bits 4321 | First block CPS |
| --- | --- |
| 0000 | MCS-4/P1 |
| 0001 | MCS-4/P2 |
| 0010 | MCS-4/P3 |
| 0011 | MCS-3/P1 |
| 0100 | MCS-3/P2 |
| 0101 | MCS-3/P3 |
| 0110 | MCS-3/P1 with padding |
| 0111 | MCS-3/P2 with padding |
| 1000 | MCS-3/P3 with padding |
| 1001 | MCS-2/P1 |
| 1010 | MCS-2/P2 |
| 1011 | MCS-1/P1 |
| 1100 | MCS-1/P2 |
| 1101 | MCS-2/P1 with padding (see NOTE 2) |
| 1110 | MCS-2/P2 with padding (see NOTE 2) |
| 1111 | MCS-0 (see NOTE 3) |
| NOTE 1: The bit numbering is relative to the field position. NOTE 2: MCS-2 with padding shall only be used for a downlink EGPRS2-A TBF or, in case of a downlink EGPRS TBF, for retransmissions of blocks originally transmitted using EGPRS2-A. NOTE 3: MCS-0 shall only be used for a downlink TBF. |  |

The number of padding octets for uplink is indicated by the SPB field (see sub clause 10.4.8b).

In EC-GSM-IoT, the CPS field for MCS-1-4 in Header type 3 indicates the kind of channel coding and puncturing used for data block as defined in Table 10.4.8a.3.2.

Table 10.4.8a.3.2: CPS field for Header type 3 in EC TBF.

| bits 3 2 1 | CPS |
| --- | --- |
| 0 0 0 | MCS-4/P1 |
| 0 0 1 | MCS-4/P2 |
| 0 1 0 | MCS-3/P1 |
| 0 1 1 | MCS-3/P2 |
| 1 0 0 | MCS-3/P1 with padding |
| 1 0 1 | MCS-3/P2 with padding |
| 1 1 0 | MCS-2/P1 |
| 1 1 1 | MCS-1/P1 |

***** Next modification *****

### 10.4.8b	Split Block indicator field (SPB)

In EGPRS uplink, EC-GSM-IoT uplink and EGPRS2 uplink, the Split Block indicator is only used in header type 3 to indicate if some user data is retransmitted using 2 block resegmentation (see clause 9).

Table 10.4.8b.1: Split Block indicator field (uplink)

| bits 2 1 | SPB |
| --- | --- |
| 0 0 | No retransmission |
| 0 1 | Retransmission - first part of block with 10 octet padding |
| 1 0 | Retransmission - first part of block with no padding or 6 octet padding |
| 1 1 | Retransmission - second part of block |
| NOTE: The bit numbering is relative to the field position. |  |

In case of EGPRS, 10 octet padding shall be used only for retransmissions of blocks originally transmitted using EGPRS2-A. In case of EGPRS2-A, 6 octet padding shall be used only for retransmissions of blocks originally transmitted using EGPRS.

In EGPRS downlink, EC-GSM-IoT downlink and EGPRS2 downlink, the Split Block indicator is only used in header type 3 to indicate if some user data is retransmitted using 2 or 3 block resegmentation (see clause 9).

Table 10.4.8b.2: Split Block indicator field (downlink)

| bits 2 1 | SPB |
| --- | --- |
| 0 0 | No retransmission |
| 0 1 | Retransmission - third part of block (Note 2) |
| 1 0 | Retransmission - first part of block |
| 1 1 | Retransmission - second part of block |
| NOTE 1: The bit numbering is relative to the field position. NOTE 2: In EGPRS downlink this code point is only used when retransmiting blocks at EGPRS level change from EGPRS2-A to EGPRS. |  |

***** Next modification *****

### 10.4.9f	Reduced TLLI (rTLLI)

A mobile station in EC operation that has been indicated to use the enhanced access burst procedure shall only include the TLLI in the first RLC data block of the uplink EC TBF and the rTLLI in the RLC/MAC headers of the remaining RLC data blocks until the contention resolution is completed (see sub-clause 7a.2.1.2).

### 10.4.9g	Reduced TLLI Indicator (RI)

The RI bit is used to indicate whether the rTLLI field is valid or not valid.

Table 10.4.9g.1: Reduced TLLI Indicator (RI)

| bit 1 | RI |
| --- | --- |
| 0 | rTLLI field is not valid |
| 1 | rTLLI field is valid |

***** Next modification *****

### 10.4.10a	Power Reduction (PR) field

The Power Reduction (PR) field indicates the power level reduction of the current RLC block.

The coding of Power Reduction (PR) field is shown in Table 10.4.10a.1. There is one value of the PR field which indicates that the field shall be ignored by the MS.

If downlink power control is not used, the MS shall ignore the PR field.

If downlink power control is used and the PR field is not included in a downlink RLC/MAC control block, the MS shall act as if the block contained a usable PR field with value ‘0 0’.

Table 10.4.10a.1: Power Reduction (PR) field

| bit 6 5 | Power Reduction |
| --- | --- |
| 0 0 | 0 dB (included) to 3 dB (excluded) less than BCCH level - P0 |
| 0 1 | 3 dB (included) to 7 dB (excluded) less than BCCH level - P0 |
| 1 0 | 7 dB (included) to 10 dB (included) less than BCCH level - P0 |
| 1 1 | Not usable |

The Power Reduction (PR) field in a downlink EC-GSM-IoT control block indicates the power level reduction of the current RLC/MAC control block. When PRe is not included, the coding of Power Reduction (PR) field for a downlink EC-GSM-IoT control block is shown in Table 10.4.10a.2.

Table 10.4.10a.2: Power Reduction (PR) field for EC-GSM-IoT control blocks

| bit 8 | Power Reduction |
| --- | --- |
| 0 | 0 dB (included) to 3 dB (excluded) less than BCCH level - P0 |
| 1 | 3 dB (included) to 7 dB (excluded) less than BCCH level - P0 |

### 10.4.10b Power Reduction extension (PRe) field

The Power Reduction extension (PRe) field indicates the extended power level reduction of the current RLC/MAC control block. The PRe field is only included if the Payload Type, see table 10.4.7.3, is set to 1.

When included the PRe field, together with the PR field, gives the coding of the power reduction for a downlink EC-GSM-IoT control block as shown in Table 10.4.10b.1.

Table 10.4.10b.1: PR bit, PRe bit

| PR | PRe | Power Reduction |
| --- | --- | --- |
| 0 | 0 | 0 dB (included) to 3 dB (excluded) less than BCCH level - P0 |
| 1 | 0 | 3 dB (included) to 7 dB (excluded) less than BCCH level - P0 |
| 0 | 1 | 7 dB (included) to 10 dB (included) less than BCCH level - P0 |
| 1 | 1 | Reserved |

***** Next modification *****

### 10.4.12	Block Sequence Number (BSN) field

The Block Sequence Number (BSN) field carries the sequence absolute Block Sequence Number (BSN') modulo Sequence Number Space (SNS) (32 in EC-GSM-IoT, 128 in GPRS, 2 048 in EGPRS and 8192 in a DLMC configuration) of each RLC data block within the TBF.

In EC-GSM-IoT, the BSN is 5 bits in length and is encoded as a binary number with range 0 to 31.

In GPRS, the BSN is 7 bits in length and is encoded as a binary number with range 0 to 127.

In EGPRS, or in a DLMC configuration where a SNS of 2048 is used (see sub-clause 5.13), the BSN is 11 bits in length and is encoded as a binary number with range 0 to 2 047.

In a DLMC configuration where a SNS of 8192 is used (see sub-clause 5.13), the BSN is 13 bits in length and is encoded as a binary number with range 0 to 8191.

In case two to four RLC data blocks are sent within a RLC/MAC block, BSN2 to BSN4 are relative to BSN1, provided the difference between the second to fourth block number and the first block modulo SNS is less than Window Size (WS).

For example,

Second block number = [BSN1 + BSN2] modulo SNS

(e.g. SNS = 2 048, WS = 512, Block A block number = 10 and Block B block number= 2 000 then:

[Block A - Block B] modulo SNS = 58 < 512;

[Block B - Block A] modulo SNS = 1 990 > 512;

Then: Block #1 = Block B and Block #2 = Block A, BSN1 = 2 000 and BSN2 = 58).

In a DLMC configuration there are restrictions regarding the selection of RLC data blocks for inclusion within a given RLC/MAC block:

-		In the RLC/MAC header for MCS-7, MCS-8 and MCS-9 13 bits for BSN1 and 11 bits for BSN2 are defined. For these MCSs the difference between the two RLC data blocks (Block A and Block B) shall be less than 2048.

-	i.e. [Block A - Block B] modulo SNS < 2048 or [Block B - Block A] modulo SNS < 2048

-	In the RLC/MAC header for DAS-11 and DAS-12 13 bits for BSN1, 12 bits for BSN2 and 11 bits for BSN3 are defined. For these MCSs the difference between two of the RLC data blocks (i.e. two out of Block A, B or C) shall be less than 2048.

-	i.e. [Block X - Block Y] modulo SNS < 2048 or [Block Y - Block X] modulo SNS < 2048 where X is any one of Block A, B or C, and Y is any of the two remaining blocks.

For example,

SNS = 8192, WS = 4096:

BSN1=2000 (Block B block number=2000)

BSN2=58 (Block A block number=2058)

-	[Block A – Block B] modulo 8192 = 58 < 4096

-	[Block B – Block A] modulo 8192 = 8133 > 4096

then Block C block number shall fulfill [Block C - Block B] modulo SNS < 2048.

For example Block C block number = 4003, and BSN3=2003

-	[Block C – Block B] modulo 8192 = 2003 < 2048

***** Next modification *****

### 10.4.14a	Length Indicator (LI) field in EGPRS TBF mode, EC TBF mode and TCH TBF mode (Iu mode)

The Length indicator is used to delimit Upper Layer PDUs within the RLC data block. Additionally for Iu mode, a Length Indicator of value 127 may be used to indicate the presence of filler information within the RLC data block. If the first length indicator in the RLC data block is set to 127, then the entire RLC data block contains filler information.

The first Length Indicator (respectively, second length Indicator, if the first is of value 123, as described below) shall indicate the number of octets of the RLC data field belonging to the first Upper Layer PDU, the second Length Indicator (respectively third Length Indicator, if the first is of value 123, as described below) shall indicate the number of octets of the RLC data field belonging to the second Upper Layer PDU, etc. Only the last segment of any Upper Layer PDU, including those with only one segment, shall be identified with a Length Indicator. The length indicator shall be placed in the RLC data block corresponding to the last segment of the Upper Layer PDU, unless the Upper Layer PDU without the corresponding LI field fills the RLC data block precisely. In this singular case, the Length Indicator shall be placed as the first Length Indicator in the next in sequence RLC data block and take the value 0. The inclusion of a Length Indicator that indicates the presence of a Selected PLMN Index field (see sub-clause 10.4.27) will never correspond to an Upper Layer PDU and therefore will never result in this singular case.

If the Upper Layer PDU does not fill the current RLC data block, a Length Indicator with value 127 (111 1111) shall be included as the last Length Indicator of the current RLC data block, indicating that there is no following Upper Layer PDU in this RLC data block. If the Upper Layer PDU does not fill the RLC data block and there is only one octet left, then the Length Indicator corresponding to the Upper Layer PDU is the last Length Indicator field that shall be included in the RLC data block.

During RLC non-persistent mode of operation, or, during RLC unacknowledged mode of operation if the network has commanded the usage of "Indication of Upper Layer PDU Start for RLC UM" for the TBF, and if the current RLC data block starts with the first segment of a new Upper Layer PDU, then a Length Indicator taking the value 126 shall be placed as the first Length Indicator in the RLC data block unless a Length Indicator with the value 0, signalling that the last segment of the previous Upper Layer PDU precisely fills the previous in sequence RLC data block, shall be included.

If the network and the mobile station support dynamic timeslot reduction (see sub-clause 8.1.8), a Length Indicator with value 125 indicates the presence of dynamic timeslot reduction control information which shall be included after the last Upper Layer PDU.

If EMSR is enabled, the transmission of an Upper Layer PDU can be suspended to transmit a higher priority Upper Layer PDU by including an LI with a value of 124 as the first LI field within the next transmitted RLC data block where an octet containing the TFI value associated with the higher priority Upper Layer PDU immediately follows the LI field. The transmission of the suspended Upper Layer PDU is resumed by including an LI with a value of 124 followed by an octet containing the TFI (and eTFI if assigned, see sub-clause 5.13) associated with the Upper Layer PDU whose transmission is resumed after the LI associated with the last segment of the higher priority Upper Layer PDU whose transmission is completed. The transition from the last segment of an Upper Layer PDU to the first segment of a new Upper Layer PDU associated with a different PFC can also occur within an RLC data block (i.e. no suspend occurs) in which case the transition is indicated by including an LI with a value of 124 followed by an octet containing the TFI (and eTFI if assigned) associated with the new Upper Layer PDU after the LI associated with the last segment of the Upper Layer PDU whose transmission is completed. In addition:

-	If the last segment of the Upper Layer PDU without the corresponding LI field fills the RLC data block precisely and the next in sequence RLC data block is in the same radio block, then a Length Indicator with the value 0 shall be placed as the first Length Indicator in the next in sequence RLC data block.

-	If the last segment of the Upper Layer PDU without the corresponding LI field fills the RLC data block precisely and the next in sequence RLC data block is in a different radio block, then a Length Indicator with the value 0 shall be placed as the first Length Indicator in the next in sequence RLC data block and the TFI in the RLC/MAC header associated with the next in sequence RLC data block and, if eTFI is assigned, the eTFI indicated by the CRC of the  next in sequence RLC data block, shall correspond to the last segment of the Upper Layer PDU.

-	When a Length Indicator having the value 124 is included in the same RLC data block as the last segment of the Upper Layer PDU and fills the RLC data block precisely then the first octet of the RLC data field of the next in sequence RLC data block shall include the TFI (and eTFI if assigned) corresponding to the Upper Layer PDU whose transmission is resumed (if a suspend occurred) or to a new Upper Layer PDU (if no suspend occurred).

A Length Indicator of value 123 is used by a mobile station supporting network sharing to indicate the presence of the Selected PLMN Index field (see sub-clause 10.4.27) in the octet immediately following this Length Indicator. In this case  it shall always be the first Length Indicator in the RLC data block. The first Length Indicator used for the case of EMSR requires that at least one previous RLC data block has been sent for the uplink TBF whereas the first Length Indicator used to indicate the presence of the Selected PLMN Index field shall always occur within the very first RLC data block sent for the uplink TBF and as such they can never occur within the same RLC data block.

The final RLC data block of a TBF shall have a Length Indicator field corresponding to the final Upper Layer PDU unless the final Upper Layer PDU fills the RLC data block precisely. If the final Upper Layer PDU fills the final RLC data block precisely, the final Upper Layer PDU shall be sent without a corresponding Length Indicator field.

The Length Indicator field is 7 bits in length and shall be encoded as a binary number. The valid values are the values ranging from 0 to 74 for an EGPRS TBF, an EC TBF, an EGPRS2-A uplink TBF or an EGPRS2-B TBF, from 0 to 82 for an EGPRS2-A downlink TBF, from 0 to 103 in TCH TBF mode (Iu mode), and the values 124 to 127. All other values are reserved. A mobile station detecting a reserved Length Indicator value or an inconsistent encoding of the Length Indicator and E fields shall ignore the RLC data block.

The interpretation of the value contained in the length indicator with corresponding E bit is summarized in table 10.4.14a.1 and some examples are shown in annex B.

Table 10.4.14a.1: Interpretation of values of LI field and E bit

| Value of LI in a RLC data block | Value of E bit in the same octet | Interpretation |
| --- | --- | --- |
| k-th LI (k>0 integer): 0< value <75 (EGPRS/EC-GSM-IoT except EGPRS2-A downlink) 0< value <83 (EGPRS2-A downlink) 0< value <104 (TCH) |  | The value of the k-th LI is the number of octets of the k-th Upper Layer PDU, or the last segment of it, in the current RLC data block. |
|  | 0 | There is at least one Upper Layer PDU following the k-th Upper Layer PDU in the current RLC data block. |
|  | 1 | There is no more than one Upper Layer PDU following the k-th Upper Layer PDU in the current RLC data block. |
| 1st LI: value =0 | 0 | The last Upper Layer PDU of the previous in sequence RLC data block ends at the boundary of that RLC data block and it has no LI in the header of that RLC data block. Thus the current RLC data block contains the first segment of all included Upper Layer PDUs. |
| 1st LI: value = 126 | 0 | The current RLC data block contains the first segment of all included Upper Layer PDUs. |
| k-th LI (k>1 integer): 0< value <75 (EGPRS/EC-GSM-IoT except EGPRS2-A downlink) 0< value <83 (EGPRS2-A downlink) 0< value <104 (TCH) |  | The k-th LI contains the number of octets of the (k-1)-th Upper Layer PDU in the current RLC data block. |
|  | 0 | There is at least one Upper Layer PDU following the (k-1)-th Upper Layer PDU in the current RLC data block. |
|  | 1 | There is no more than one Upper Layer PDU following the (k-1)-th Upper Layer PDU in the current RLC data block. |
| k-th LI: value=127 | 1 | The octets between the end of the Upper Layer PDU indicated by the (k-1)-th LI and the end of the current RLC data block are filling octets. |
| 1st LI: value=0 | 1 | The previous RLC data block contains a Upper Layer PDU, or a part of it, that fills precisely the previous data block and for which there is no length indicator in that RLC data block. The current RLC data block contains a Upper Layer PDU that either fills the current RLC data block precisely or continues in the next RLC data block. |
| 1st LI: value = 126 | 1 | The current RLC data block contains the first segment of an Upper Layer PDU that either fills the current RLC data block precisely or continues in the next RLC data block. |
| 1st LI: value=127 (Iu mode only) | 1 | All octets of the RLC Data block contain filling information. |
| k-th LI: value=125 (this value is not supported in EC operation) | 0/1 | The current RLC data block contains the dynamic timeslot reduction control information. |
| k-th LI:value=124 (this value is only allowed if EMSR is enabled) | 0 | An LI value of 124 serves as a TFI transition indicator and is immediately followed by an octet containing the TFI associated with the next Upper Layer PDU segment in bit positions 1 to 5 (where bit 5 is the most significant bit) and reserved bits in bit positions 6 to 8. If eTFI is assigned then it shall be indicated by bit positions 6 to 8 (where bit 8 is the most significant bit). |
| 1 st LI:value=123 (this value is only allowed if network sharing is supported both by the network and the mobile station) | 0/1 | An LI value of 123 serves as a Selected PLMN Index field indicator and is immediately followed by an octet that contains the Selected PLMN Index field (see sub-clause 10.4.27). |
| No LI field present | n.a. | The Upper Layer PDU that starts with the current RLC data block either fills the current RLC data block precisely or continues in the following in-sequence RLC data block |

***** Next modification *****

### 10.4.20	Packet Flow Identifier (PFI) field

The PFI field contains a PFI value encoded as the contents of the PFI information element as defined in 3GPP TS 44.018. The use of PFI is not supported in EC operation.

***** Next modification *****

### 10.4.27	Selected PLMN Index field

This field contains the index of the PLMN selected by a mobile station supporting network sharing. The index value refers either to the identity of the Common PLMN broadcast in the SYSTEM INFORMATION TYPE 3/4 message, or EC SYSTEM INFORMATION TYPE 2 message for EC-GSM-IoT, or to the identity of an Additional PLMN broadcast in the SYSTEM INFORMATION TYPE 22 message, or EC SYSTEM INFORMATION TYPE 4 message for EC-GSM-IoT, as described in Table 10.4.27.1.

Table 10.4.27.1: Selected PLMN Index field

| bit 3 2 1 | Selected PLMN Index |
| --- | --- |
| 0 0 1 | PLMN identity of the Common PLMN broadcast in SYSTEM INFORMATION TYPE 3/4. For EC-GSM-IoT, the PLMN identity of the Common PLMN is broadcast in EC SYSTEM INFORMATION TYPE 2. |
| 0 1 0 | PLMN identity of the first Additional PLMN in the network sharing information broadcast in SYSTEM INFORMATION TYPE 22. For EC-GSM-IoT, the PLMN identity of the first Additional PLMN in the network sharing information is broadcast in EC SYSTEM INFORMATION TYPE 4. |
| 0 1 1 | PLMN identity of the second Additional PLMN in the network sharing information broadcast in SYSTEM INFORMATION TYPE 22. For EC-GSM-IoT, the PLMN identity of the second Additional PLMN in the network sharing information is broadcast in EC SYSTEM INFORMATION TYPE 4. |
| 1 0 0 | PLMN identity of the third Additional PLMN in the network sharing information broadcast in SYSTEM INFORMATION TYPE 22. For EC-GSM-IoT, the PLMN identity of the third Additional PLMN in the network sharing information is broadcast in EC SYSTEM INFORMATION TYPE 4. |
| 1 0 1 | PLMN identity of the fourth Additional PLMN in the network sharing information broadcast in SYSTEM INFORMATION TYPE 22. For EC-GSM-IoT, the PLMN identity of the fourth Additional PLMN in the network sharing information is broadcast in EC SYSTEM INFORMATION TYPE 4. |
|  | All other values are reserved |

***** Next modification *****

### 10.4.28	Coverage Class field (CC)

In EC operation, a 2-bit field is included in the downlink RLC data block header type 3 to signal the downlink Coverage Class used in the block. The Coverage Class indicates a predefined number of blind physical layer transmissions to be applied for each logical channel, where the use of blind physical layer transmissions can allow the mobile station to operate in extended coverage, see 3GPP TS 43.064.

Table 10.4.28.1: Coverage Class field

| bits 7 6 | CC |
| --- | --- |
| 0 0 | Coverage Class 1, CC1 |
| 0 1 | Coverage Class 2, CC2 |
| 1 0 | Coverage Class 3, CC3 |
| 1 1 | Coverage Class 4, CC4 |

***** Next modification *****

### 10.4.29	Downlink Coverage Class Estimate (DCCE)

In EC operation, a 4-bit field is included in the uplink RLC data block header to signal the estimated downlink Coverage Class, see 3GPP TS 43.064, and, when CC1 is estimated, the extent to which the estimated C value (see 3GPP TS 45.008) exceeds the value indicated by the BT_Threshold_DL parameter sent in EC SI2 (see 3GPP TS 44.018). The C value will be reported in steps of 3 dB above the BT_Threshold_DL where BT_Threshold_DL indicates when blind physical layer transmissions are used.

Table 10.4.29.1: DCCE field

| bits 4 3 2 1 | DCCE | bits 4 3 2 1 | DCCE |
| --- | --- | --- | --- |
| 0 0 0 0 | DL CC4 | 1 0 0 0 | DL CC1; BT_Threshold_DL+15 dB ≤ C value < BT_Threshold_DL+18 dB |
| 0 0 0 1 | DL CC3 | 1 0 0 1 | DL CC1; BT_Threshold_DL+18 dB ≤ C value < BT_Threshold_DL+21 dB |
| 0 0 1 0 | DL CC2 | 1 0 1 0 | DL CC1; BT_Threshold_DL+21 dB ≤ C value < BT_Threshold_DL+24 dB |
| 0 0 1 1 | DL CC1; BT_Threshold_DL ≤ C value < BT_Threshold_DL+3 dB | 1 0 1 1 | DL CC1; BT_Threshold_DL+24 dB ≤ C value < BT_Threshold_DL+27 dB |
| 0 1 0 0 | DL CC1; BT_Threshold_DL+3 dB ≤ C value < BT_Threshold_DL+6 dB | 1 1 0 0 | DL CC1; BT_Threshold_DL+27 dB ≤ C value < BT_Threshold_DL+30 dB |
| 0 1 0 1 | DL CC1; BT_Threshold_DL+6 dB ≤ C value < BT_Threshold_DL+9 dB | 1 1 0 1 | DL CC1; BT_Threshold_DL+30 dB ≤ C value < BT_Threshold_DL+33 dB |
| 0 1 1 0 | DL CC1; BT_Threshold_DL+9 dB ≤ C value < BT_Threshold_DL+12 dB | 1 1 1 0 | DL CC1; BT_Threshold_DL+33 dB ≤ C value < BT_Threshold_DL+36 dB |
| 0 1 1 1 | DL CC1; BT_Threshold_DL+12 dB ≤ C value < BT_Threshold_DL+15 dB | 1 1 1 1 | DL CC1; BT_Threshold_DL+36 dB ≤ C value < BT_Threshold_DL+39 dB |
| NOTE: The bit numbering is relative to the field position. |  |  |  |

***** Next modification *****

# 11	Message functional definitions and contents

This sub-clause defines the structure of the RLC/MAC control messages. These are non-standard L3 messages as defined in 3GPP TS 24.007. The formats for the messages are valid only for the PDCH. The format for RLC/MAC control messages for use on the CCCH are defined in 3GPP TS 44.018.

The RLC/MAC control messages defined in this sub-clause may be used also on DBPSCH and SBPSCH in Iu mode according to the requirements specified in 3GPP TS 44.160. A subset of these messages is used exclusively in Iu mode. Messages belonging to that subset are labelled as "Iu mode only" in this technical specification.

A subset of the Iu mode only messages is used exclusively on DBPSCH. These messages do not follow the general syntactical rules for the RLC/MAC control messages used on shared channels. The error handling defined for shared channels does not apply. Messages that may be sent from the network to the mobile station, and that belong to this subset, are classified as DBPSCH messages, see sub-clause 11.1.1.

Specific RLC/MAC control messages, sent on the EC-PACCH, are used for EC TBFs.

Each definition given in the present sub-clause includes:

-	a brief description of the message direction and use;

-	a CSN.1 description of the message information elements and fields (see CSN.1 Specification, Version 2.0). Definition of information elements may immediately follow the definition of the message. If the definition of an information element immediately follows the message definition, the information element name ends with 'struct'. Otherwise the information element name ends with 'IE' and the definition of the information element is defined in clause 12 or in 3GPP TS 44.018. The definition of a 'struct' is valid only within the table in which it is defined. No references shall be made to a 'struct' definition from outside of the table in which it is defined or from outside the present document. The definition of an information element is valid throughout clause 11 and clause 12 as well as in 3GPP TS 44.018;

-	a note specifying, where appropriate, conditions for information elements or fields with presence requirement C or O in the relevant message which together with other conditions specified in 3GPP TS 44.060 define when the information elements shall be included or not, what non-presence of such information elements or fields means, and - for IEs with presence requirement C - the static conditions for presence and/or non-presence of the information elements or fields (see 3GPP TS 24.007);

-	a table follows which contains a definition for each field referenced in the message definition or in an information element struct immediately following the message definition.

Bit fields within RLC/MAC messages shall have the highest numbered bit of the bit field in the highest numbered bit of the lowest number octet (see sub-clause 10.0b.3.1). The mapping of an 11 bit field is illustrated in figure 11.1.

| bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  |  |  |  |  |  |  |  | Octet N |
|  |  |  |  | bit 11 | bit 10 | bit 9 | bit 8 | Octet N+1 |
| bit 7 | bit 6 | bit 5 | bit 4 | bit 3 | bit 2 | bit 1 |  | Octet N+2 |
|  |  |  |  |  |  |  |  | Octet N+3 |

Figure 11.1: Field mapping within RLC/MAC messages

The length of an RLC/MAC control messages is an integer number of RLC/MAC control blocks. Padding bits are necessary to fill the message up to the desired length. The padding bits may be the 'null' string. Otherwise, the padding bits starts with bit '0', followed by 'spare padding'.

< padding bits > ::= { null ¦ 0 < spare padding > ! < Ignore : 1 bit** = < no string > > } ;

The padding sequence used for 'spare padding' in the present document, see 3GPP TS 24.007, is a repetition of octet '00101011', starting on an octet boundary.

***** Next modification *****

#### 11.1.1.1	Distribution messages

A distribution message is recognised by the most significant bit of the message type being set to bit '1'. The general format of a distribution message, except for distribution messages on the EC-PACCH, sent from the network to the mobile station is:

< Distribution message > ::= < MESSAGE_TYPE : 1 bit (5) > < Distribution contents > < padding bits > ;

The general format of a distribution message sent on the EC-PACCH from the network to the mobile station is:

< Distribution message > ::= < MESSAGE_TYPE : 1 bit (4) > < Distribution contents > < padding bits > ;

Any mobile stations may receive a distribution message. Depending on the protocol state of the mobile station, a distribution message shall be analysed as specified in sub-clauses 5, 6, 7, 8 and 9 of the present document.

The 'Distribution contents' of a distribution message contains Page Mode information and any specific distribution information according to the syntax defined for the message type. The ‘Distribution contents’ of a distribution message on the EC-PACCH contains Used DL Coverage Class information instead of the Page Mode information. The 'padding bits' of a distribution message can be reduced to the null string.

The general format of the 'Distribution contents', except for distribution messages sent on the EC-PACCH, is:

< Distribution contents > ::= < PAGE_MODE : bit (2) > < specific distribution information > ;

The encoding of the Page Mode information is defined in sub-clause 12.20.

The general format of the 'Distribution contents' for distribution messages sent on the EC-PACCH is:

< Distribution contents > ::= < USED_DL_COVERAGE_CLASS : bit (2) > < specific distribution information > ;

The encoding of the Used DL Coverage Class information is defined in sub-clause 12.71.

***** Next modification *****

#### 11.1.4.2	Message not compatible with current protocol state

When a non-distribution message is received, which is not expected by the addressed receiver in its current protocol state, the mobile station shall follow the procedures that are described in sub-clauses 5, 6, 7, 8 and 9 of the present document.

If no such reaction is specified, the mobile station shall ignore the message. If in packet transfer mode, the mobile station, which is identified by the address information, shall return a status message (PACKET MOBILE TBF STATUS message) with TBF_CAUSE #4, "Message not compatible with current protocol state", unless the mobile station has an ongoing EC TBF.

Unexpected distribution messages are ignored.

***** Next modification *****

##### 11.1.4.3.3	Messages with error label: 'Non-distribution part error'

For syntactically incorrect messages received with generic error label: 'Non-distribution part error', data corresponding to the description following the error label shall be recognised as erroneous data and be ignored.

The distribution contents preceding the error label may be analysed and treated as described in clause 5 and clause 7 of the present document.

The address information preceding the error label shall be analysed. In packet transfer mode, the mobile station identified by the address information shall return a PACKET MOBILE TBF STATUS message with TBF_CAUSE #2 "Syntactically incorrect message, non-distribution part error", unless the mobile station has an ongoing EC TBF.

***** Next modification *****

##### 11.1.4.3.4	Messages with error label: 'Message escape'

For syntactically incorrect messages with error label: 'Message escape', data corresponding to the description following the error label shall be recognised as erroneously received mandatory data and be rejected.

The distribution contents preceding the error label may be analysed and treated as described in clause 5 of the present document.

If the address information proceeds the error label and it is received correctly, it shall be analysed. In packet transfer mode, the mobile station identified by the address information shall return a PACKET MOBILE TBF STATUS message with TBF_CAUSE #3 "Syntactically incorrect message, message escape", unless the mobile station has an ongoing EC TBF.

***** Next modification *****

## 11.2	RLC/MAC control messages

Table 11.2.1 summarises the RLC/MAC control messages. Table 11.2.2 summarises the RLC/MAC control messages on the EC-PACCH. For each control message, the message type shall be a fixed number of bits from the beginning of the message.

Table 11.2.1: RLC/MAC control messages

| Uplink TBF establishment messages: | Reference |
| --- | --- |
| Packet Access Reject | 11.2.1 |
| Packet Channel Request | 11.2.5 |
| EGPRS Packet Channel Request | 11.2.5a |
| Packet Queuing Notification | 11.2.15 |
| Packet Resource Request | 11.2.16 |
| Packet Uplink Assignment | 11.2.29 |
| Multiple TBF Uplink Assignment | 11.2.29a |
| Additional MS Radio Access Capabilities | 11.2.32 |
| Downlink TBF establishment messages: | Reference |
| Packet DBPSCH Assignment | 11.2.5b |
| Packet Downlink Assignment | 11.2.7 |
| Multiple TBF Downlink Assignment | 11.2.7a |
| TBF release messages: | Reference |
| Packet TBF Release | 11.2.26 |
| Paging messages: | Reference |
| Packet Paging Request | 11.2.10 |
| RLC messages: | Reference |
| Packet Downlink Ack/Nack | 11.2.6 |
| EGPRS Packet Downlink Ack/Nack | 11.2.6a |
| Packet DBPSCH Downlink Ack/Nack | 11.2.6b |
| Packet DBPSCH Downlink Ack/Nack Type 2 | 11.2.6c |
| MBMS Downlink Ack/Nack | 11.2.6d |
| EGPRS Packet Downlink Ack/Nack Type 2 | 11.2.6e |
| EGPRS Packet Downlink Ack/Nack Type 3 | 11.2.6f |
| Packet Uplink Ack/Nack | 11.2.28 |
| Packet DBPSCH Uplink Ack/Nack | 11.2.28a |
| Packet DBPSCH Uplink Ack/Nack Type 2 | 11.2.28b |
| EGPRS Packet Downlink Ack/Nack DLMC | 11.2.48 |
| System information messages: | Reference |
| Packet System Information Type 1 | 11.2.18 |
| Packet System Information Type 2 | 11.2.19 |
| Packet System Information Type 3 | 11.2.20 |
| Packet System Information Type 3 bis | 11.2.21 |
| Packet System Information Type 3 ter | 11.2.21a |
| Packet System Information Type 3 quater | 11.2.21b |
| Packet System Information Type 5 | 11.2.23 |
| Packet System Information Type 6 | 11.2.23a |
| Packet System Information Type 7 | 11.2.23b |
| Packet System Information Type 8 | 11.2.24 |
| Packet System Information Type 13 | 11.2.25 |
| Packet System Information Type 14 | 11.2.25a |
| Packet System Information Type 15 | 11.2.25b |
| Packet System Information Type 16 | 11.2.25c |
| Miscellaneous messages: | Reference |
| Packet Control Acknowledgement | 11.2.2 |
| Packet Cell Change Continue | 11.2.2a |
| Packet Cell Change Failure | 11.2.3 |
| Packet Cell Change Notification | 11.2.3a |
| Packet Cell Change Order | 11.2.4 |
| Packet Downlink Dummy Control Block | 11.2.8 |
| Packet Uplink Dummy Control Block | 11.2.8b |
| Packet Measurement Report | 11.2.9 |
| Packet Measurement Order | 11.2.9b |
| Packet Mobile TBF Status | 11.2.9c |
| Packet Enhanced Measurement Report | 11.2.9d |
| Packet Neighbour Cell Data | 11.2.9e |
| Packet PDCH Release | 11.2.11 |
| Packet Polling Request | 11.2.12 |
| Packet Power Control/Timing Advance | 11.2.13 |
| Packet PRACH Parameters | 11.2.14 |
| Packet PSI Status | 11.2.17 |
| Packet Serving Cell Data | 11.2.17a |
| Packet SI Status | 11.2.17b |
| Packet Pause | 11.2.30a |
| Packet Timeslot Reconfigure | 11.2.31 |
| Multiple TBF Timeslot Reconfigure | 11.2.31a |
| Handover Access | 11.2.33 |
| Physical Information | 11.2.34 |
| Packet CS Request | 11.2.35 |
| Packet CS Command | 11.2.36 |
| Packet CS Release Indication | 11.2.37 |
| MBMS Service Request | 11.2.38 |
| MBMS Assignment (Non-distribution) MBMS Assignment (Distribution) | 11.2.39 11.2.39a |
| MBMS Neighbouring Cell Information | 11.2.40 |
| MBMS MS_ID Assignment | 11.2.41 |
| Packet MBMS Announcement | 11.2.42 |
| PS Handover Command | 11.2.43 |
| PS Handover Access | 11.2.44 |
| Packet Physical Information | 11.2.45 |
| DTM Handover Command | 11.2.46 |
| Packet Application Information | 11.2.47 |

Table 11.2.2: RLC/MAC control messages on EC-PACCH

| Uplink EC TBF establishment messages: | Reference |
| --- | --- |
| EC Packet Access Reject | 11.2.49 |
| EC Packet Uplink Assignment | 11.2.58 |
| Downlink EC TBF establishment messages: | Reference |
| EC Packet Downlink Assignment | 11.2.52 |
| EC TBF release messages: | Reference |
| EC Packet TBF Release | 11.2.56 |
| EC-GSM-IoT RLC messages: | Reference |
| EC Packet Downlink Ack/Nack | 11.2.51 |
| EC Packet Uplink Ack/Nack | 11.2.57 |
| EC Packet Uplink Ack/Nack and Contention Resolution | 11.2.59 |
| EC-GSM-IoT miscellaneous messages: | Reference |
| EC Packet Control Acknowledgement | 11.2.50 |
| EC Packet Polling Request | 11.2.54 |
| EC Packet Power Control/Timing Advance | 11.2.55 |
| EC Packet Downlink Dummy Control Block | 11.2.53 |

***** Next modification *****

#### 11.2.0.1	Downlink RLC/MAC messages

Downlink RLC/MAC control messages are received in RLC/MAC control block format. The different types of messages are distinguished by the MESSAGE_TYPE field.

< Downlink RLC/MAC control message > ::= < MESSAGE_TYPE : bit (6) == 1 00001 > < Packet Access Reject message content > ¦ < MESSAGE_TYPE : bit (6) == 0 00001 > < Packet Cell Change Order message content > ¦ < MESSAGE_TYPE : bit (6) == 0 00010 > < Packet Downlink Assignment message content > ¦ < MESSAGE_TYPE : bit (6) == 0 00011 > < Packet Measurement Order message content > ¦ < MESSAGE_TYPE : bit (6) == 1 00010 > < Packet Paging Request message content > ¦ < MESSAGE_TYPE : bit (6) == 1 00011 > < Packet PDCH Release message content > ¦ < MESSAGE_TYPE : bit (6) == 0 00100 > < Packet Polling Request message content > ¦ < MESSAGE_TYPE : bit (6) == 0 00101 > < Packet Power Control/Timing Advance message content > ¦ < MESSAGE_TYPE : bit (6) == 1 00100 > < Packet PRACH Parameters message content > ¦ < MESSAGE_TYPE : bit (6) == 0 00110 > < Packet Queueing Notification message content > ¦ < MESSAGE_TYPE : bit (6) == 0 00111 > < Packet Timeslot Reconfigure message content > ¦ < MESSAGE_TYPE : bit (6) == 0 01000 > < Packet TBF Release message content > ¦ < MESSAGE_TYPE : bit (6) == 0 01001 > < Packet Uplink Ack/Nack message content > ¦ -- PACCH only < MESSAGE_TYPE : bit (6) == 0 01010 > < Packet Uplink Assignment message content > ¦ < MESSAGE_TYPE : bit (6) == 0 01011 > < Packet Cell Change Continue message content > ¦ < MESSAGE_TYPE : bit (6) == 0 01100 > < Packet Neighbour Cell Data message content > ¦ < MESSAGE_TYPE : bit (6) == 0 01101 > < Packet Serving Cell Data message content > ¦ < MESSAGE_TYPE : bit (6) == 0 01110 > < Packet DBPSCH Assignment message content > ¦ < MESSAGE_TYPE : bit (6) == 0 01111 > < Multiple TBF Downlink Assignment message content > ¦ < MESSAGE_TYPE : bit (6) == 0 10000 > < Multiple TBF Uplink Assignment message content > ¦ < MESSAGE_TYPE : bit (6) == 0 10001 > < Multiple TBF Timeslot Reconfigure message content > ¦ < MESSAGE_TYPE : bit (6) == 0 10011 > < MBMS MS_ID Assignment message content > ¦ -- PACCH only < MESSAGE_TYPE : bit (6) == 0 10100 > < MBMS Assignment (Non-distribution) message content > ¦ < MESSAGE_TYPE : bit (6) == 0 10101 > < PS Handover Command message content > ¦ -- PACCH only < MESSAGE_TYPE : bit (6) == 0 10110 > < Packet Physical Information message content > ¦ -- PACCH only < MESSAGE_TYPE : bit (6) == 0 10111 > < DTM Handover Command message content > ¦ -- PACCH only < MESSAGE_TYPE : bit (6) == 1 00101 > < Packet Downlink Dummy Control Block message content > ¦ < MESSAGE_TYPE : bit (6) == 1 10001 > < PSI1 message content > ¦ < MESSAGE_TYPE : bit (6) == 1 10010 > < PSI2 message content > ¦ < MESSAGE_TYPE : bit (6) == 1 10011 > < PSI3 message content > ¦ < MESSAGE_TYPE : bit (6) == 1 10100 > < PSI3 bis message content > ¦ < MESSAGE_TYPE : bit (6) == 1 10101 > reserved ¦ -- this value was allocated in an earlier -- version of the protocol and shall not be used < MESSAGE_TYPE : bit (6) == 1 10110 > < PSI5 message content > ¦ < MESSAGE_TYPE : bit (6) == 1 10000 > < PSI6 message content > ¦ < MESSAGE_TYPE : bit (6) == 1 11000 > < PSI7 message content > ¦ < MESSAGE_TYPE : bit (6) == 1 11001 > < PSI8 message content > ¦ < MESSAGE_TYPE : bit (6) == 1 10111 > < PSI13 message content > ¦ < MESSAGE_TYPE : bit (6) == 1 11010 > < PSI14 message content > ¦ < MESSAGE_TYPE : bit (6) == 1 11100 > < PSI3 ter message content > ¦ < MESSAGE_TYPE : bit (6) == 1 11101 > < PSI3 quater message content > ¦ < MESSAGE_TYPE : bit (6) == 1 11110 > < PSI15 message content > ¦ < MESSAGE_TYPE : bit (6) == 1 01000 > < PSI16 message content > ¦ < MESSAGE_TYPE : bit (6) == 1 00000 > < Packet Serving Cell SI message content > ¦ < MESSAGE_TYPE : bit (6) == 1 00111 > < Packet CS Command message content > ¦ < MESSAGE_TYPE : bit (6) == 1 01001 > < Packet CS Release Indication message content > ¦ < MESSAGE_TYPE : bit (6) == 1 01010 > < MBMS Assignment (Distribution) message content > ¦ < MESSAGE_TYPE : bit (6) == 1 01011 > < MBMS Neighbouring Cell Information message content >¦ < MESSAGE_TYPE : bit (6) == 1 01100 > < Packet MBMS Announcement message content > < MESSAGE_TYPE : bit (6) == 1 01101 > < Packet Application Information message content > ! < Unknown message type : { bit (6) = < no string > } < Default downlink message content > > ;

NOTE: 	the MESSAGE_TYPE "010010" is reserved for the PHYSICAL INFORMATION message on DBPSCH only.

The 'Default downlink message contents', for messages that are not sent on the EC-PACCH, consists of the Page Mode information and an unspecified bit string that expands to the end of the message.

< Default downlink message content > ::= < PAGE_MODE : bit (2) > bit (*) = < no string > ;

The encoding of the Page Mode information is defined in sub-clause 12.20.

Downlink RLC/MAC control messages sent on EC-PACCH are received in RLC/MAC control block format. The different types of messages on the EC-PACCH are distinguished by the MESSAGE_TYPE field.

< Downlink RLC/MAC control message on EC-PACCH > ::= < MESSAGE_TYPE : bit (5) == 1 0001 > < EC Packet Access Reject message content > ¦ < MESSAGE_TYPE : bit (5) == 0 0001 > < EC Packet Downlink Assignment message content > ¦ < MESSAGE_TYPE : bit (5) == 0 0010 > < EC Packet Polling Request message content > ¦ < MESSAGE_TYPE : bit (5) == 0 0011 > < EC Packet Power Control/Timing Advance message content > ¦ < MESSAGE_TYPE : bit (5) == 0 0100 > < EC Packet TBF Release message content > ¦ < MESSAGE_TYPE : bit (5) == 0 0101 > < EC Packet Uplink Ack/Nack message content > ¦ < MESSAGE_TYPE : bit (5) == 0 0110 > < EC Packet Uplink Assignment message content > ¦ < MESSAGE_TYPE : bit (5) == 0 0111 > < EC Packet Uplink Ack/Nack and Contention Resolution message content > ¦ < MESSAGE_TYPE : bit (5) == 1 0010 > < EC Packet Downlink Dummy Control Block message content > ! < Unknown message type : { bit (5) = < no string > } < Default downlink message content > > ;

The 'Default downlink message contents' for messages that are sent on the EC-PACCH consists of the Used DL Coverage Class information and an unspecified bit string that expands to the end of the message.

< Default downlink message content > ::= < USED_DL_COVERAGE_CLASS : bit (2) > bit (*) = < no string > ;

The encoding of the Used DL Coverage Class information element is defined in sub-clause 12.71.

***** Next modification *****

#### 11.2.0.2	Uplink RLC/MAC messages

Uplink RLC/MAC control messages, except those using the access burst formats, are received in the RLC/MAC control block format. The different types of messages are distinguished by the MESSAGE_TYPE field.

< Uplink RLC/MAC control message > ::= < MESSAGE_TYPE : bit (6) == 000000 > < Packet Cell Change Failure message content > ¦ < MESSAGE_TYPE : bit (6) == 000001 > < Packet Control Acknowledgement message content > ¦ < MESSAGE_TYPE : bit (6) == 000010 > < Packet Downlink Ack/Nack message content > ¦ -- PACCH only < MESSAGE_TYPE : bit (6) == 000011 > < Packet Uplink Dummy Control Block message content > ¦ < MESSAGE_TYPE : bit (6) == 000100 > < Packet Measurement Report message content > ¦ < MESSAGE_TYPE : bit (6) == 001010 > < Packet Enhanced Measurement Report message content > ¦ < MESSAGE_TYPE : bit (6) == 000101 > < Packet Resource Request message content > ¦ < MESSAGE_TYPE : bit (6) == 000110 > < Packet Mobile TBF Status message content > ¦ < MESSAGE_TYPE : bit (6) == 000111 > < Packet PSI Status message content > ¦ < MESSAGE_TYPE : bit (6) == 001000 > < EGPRS Packet Downlink Ack/Nack message content > ¦ < MESSAGE_TYPE : bit (6) == 010001 > < EGPRS Packet Downlink Ack/Nack Type 2 message content > ¦ < MESSAGE_TYPE : bit (6) == 010010 > < EGPRS Packet Downlink Ack/Nack Type 3 message content > ¦ < MESSAGE_TYPE : bit (6) == 001001 > < Packet Pause message content > ¦ < MESSAGE_TYPE : bit (6) == 001011 > < Additional MS Radio Access Capabilities message content > ¦ < MESSAGE_TYPE : bit (6) == 001100 > < Packet Cell Change Notification message content > ¦ < MESSAGE_TYPE : bit (6) == 001101 > < Packet SI Status message content > ¦ < MESSAGE_TYPE : bit (6) == 001110 > < Packet CS Request message content > ¦ < MESSAGE_TYPE : bit (6) == 001111 > < MBMS Service Request message content > ¦ < MESSAGE_TYPE : bit (6) == 010000 > < MBMS Downlink Ack/Nack message content > ¦ < MESSAGE_TYPE : bit (6) == 010011 > < EGPRS Packet Downlink Ack/Nack DLMC message content >;

Messages using the access burst formats (11-bit and 8-bit formats) are defined in sub-clauses 11.2.2 11.2.5, 11.2.5a, 11.2.5c, 11.2.33 and 11.2.44.

Uplink RLC/MAC control messages sent on EC-PACCH, except those using the access burst formats, are received in the RLC/MAC control block format. The different types of messages on the EC-PACCH are distinguished by the MESSAGE_TYPE field.

< Uplink RLC/MAC control message on EC-PACCH > ::= < MESSAGE_TYPE : bit (5) == 00001 > < EC Packet Control Acknowledgement message content > ¦ < MESSAGE_TYPE : bit (5) == 00010 > < EC Packet Downlink Ack/Nack message content >;

The EC Packet Control Acknowledgement message can be sent both using the normal burst format and the access burst format (11-bit format). The access burst format is defined in sub-clause 11.2.50.

***** Next modification *****

### 11.2.49	EC Packet Access Reject

This message is sent on the EC-PACCH by the network to the mobile station to indicate that the network has rejected the request from the mobile station. This message may contain fields addressing more than one mobile station.

Message type:	EC PACKET ACCESS REJECT

Direction:	network to mobile station

Classification:	distribution message

Table 11.2.49.1: EC Packet ACCESS REJECT information elements

< EC Packet Access Reject message content > ::= < USED_DL_COVERAGE_CLASS : < Used DL Coverage Class IE > > < Reject : < EC Reject struct > > { { 1 < Additional Reject: < EC Reject struct > > } ** 0 < padding bits > } // -- truncation at end of message allowed, bits '0' assumed ! < Distribution part error : bit (*) = < no string > > ;

< EC Reject struct > ::= < Downlink_TFI : bit (5) > { 0 ¦ 1 < WAIT_INDICATION : bit (8) > < WAIT _INDICATION_SIZE : bit (1) > } ! < Ignore : bit (*) = <no string> > ;

Table 11.2.49.2: EC Packet ACCESS REJECT information element details

USED_DL_COVERAGE_CLASS This information element is defined in sub-clause 12.71. EC Reject struct The mobile station shall only accept the first Reject struct addressed to it and ignore all other Reject structs.

WAIT_INDICATION (8 bit field) The Wait Indication field indicates the time the mobile station shall wait before attempting another channel request. This field is coded as the binary representation of the T3172 timeout value in units of 20 ms or in units of seconds. The units are indicated in the WAIT_INDICATION_SIZE field. Range 0 to 255.

WAIT_INDICATION_SIZE (1 bit field) This field indicates the units of the WAIT_INDICATION field. 0 the WAIT_INDICATION field is coded in units of s 1 the WAIT_INDICATION field is coded in units of 20 ms

***** Next modification *****

### 11.2.50	EC Packet Control Acknowledgement

This message is sent on the EC-PACCH from the mobile station to the network. The message is formatted either as an RLC/MAC control block using the EC-PACCH block format defined in 3GPP TS 44.004 or as a number of identical access bursts, according to the assigned uplink CC (see 3GPP TS 45.002), using the EC-PACCH short acknowledgement block format defined in 3GPP TS 44.004. If the message is sent as response to an EC Packet Polling Request message this latter message shall specify the format of the EC Packet Control Acknowledgement message. The order of bit transmission is defined in 3GPP TS 44.004. The numbering, assembling and field mapping conventions defined for RLC/MAC control blocks in sub-clause 10.0b shall apply.

The RLC/MAC control block format is shown in table 11.2.50.1 and table 11.2.50.2.

The access burst format is 11-bits and is coded as shown in table 11.2.50.1. The mobile station shall transmit the access burst four times, one time in each TDMA frame of the uplink radio block. The transmission of the four access bursts shall then be repeated in each of the uplink EC-PACCH resources assigned with the RRBP field according to the assigned uplink Coverage Class for the mobile station.

Message type:	EC PACKET CONTROL ACKNOWLEDGEMENT

Direction:	mobile station to network

Table 11.2.50.1: EC PACKET CONTROL ACKNOWLEDGEMENT information elements

< EC Packet Control Acknowledgement message content > ::= -- RLC/MAC control block format < TLLI : bit (32) > < CTRL_ACK : bit (2) > < DL_CC_EST : bit (4) > < padding bits >;

< EC Packet Control Acknowledgement 11 bit message > ::= -- 11-bit access burst format < MESSAGE_TYPE : bit (7) == 1110 111 > < DL_CC_EST : bit (4) > ;

Table 11.2.50.2: EC PACKET CONTROL ACKNOWLEDGEMENT information element details

TLLI (32 bit field) This field contains the TLLI of the mobile station. This field is encoded as defined in sub-clause 12.16.

CTRL_ACK (2 bit field) This field contains acknowledgement information for the group of RLC/MAC control blocks that make up an RLC/MAC control message. The mobile station shall set the CTRL_ACK field to indicate which segments of an RLC/MAC control message have been received by the time of transmission of the EC PACKET CONTROL ACKNOWLEDGEMENT message. The CTRL_ACK field shall be set according to the following table: bit 2 1 0 0 Reserved. 0 1 the MS received an RLC/MAC control block addressed to itself and with RBSN = 1, and did not receive an RLC/MAC control block with RBSN = 0 for the same RLC/MAC control message. 1 0 the MS received an RLC/MAC control block addressed to itself and with RBSN = 0, and did not receive an RLC/MAC control block with RBSN = 1 for the same RLC/MAC control message. This value is sent irrespective of the value of the FS bit. 1 1 the MS received two RLC/MAC blocks that contain different segments of the same RLC/MAC control message, one with RBSN = 0 and the other with RBSN = 1.

DL_CC_EST (4 bit field) This field contains the latest downlink Coverage Class estimation and the latest estimated C value range in case the C value is above BT_Threshold_DL, see 3GPP TS 45.008. The BT_Threshold_DL value is broadcast in EC SI 2, see 3GPP TS 44.018. The DL_CC_EST field shall be set according to the following table: bit 4 3 2 1 0 0 0 0 DL CC4 0 0 0 1 DL CC3 0 0 1 0 DL CC2 0 0 1 1 DL CC1; BT_Threshold_DL ≤ C Value < BT_Threshold_DL+3 dB 0 1 0 0 DL CC1; BT_Threshold_DL+3 dB ≤ C Value < BT_Threshold_DL+6 dB 0 1 0 1 DL CC1; BT_Threshold_DL+6 dB ≤ C Value < BT_Threshold_DL+9 dB 0 1 1 0 DL CC1; BT_Threshold_DL+9 dB ≤ C Value < BT_Threshold_DL+12 dB 0 1 1 1 DL CC1; BT_Threshold_DL+12 dB ≤ C Value < BT_Threshold_DL+15 dB 1 0 0 0 DL CC1; BT_Threshold_DL+15 dB ≤ C Value < BT_Threshold_DL+18 dB 1 0 0 1 DL CC1; BT_Threshold_DL+18 dB ≤ C Value < BT_Threshold_DL+21 dB 1 0 1 0 DL CC1; BT_Threshold_DL+21 dB ≤ C Value < BT_Threshold_DL+24 dB 1 0 1 1 DL CC1; BT_Threshold_DL+24 dB ≤ C Value < BT_Threshold_DL+27 dB 1 1 0 0 DL CC1; BT_Threshold_DL+27 dB ≤ C Value < BT_Threshold_DL+30 dB 1 1 0 1 DL CC1; BT_Threshold_DL+30 dB ≤ C Value < BT_Threshold_DL+33 dB 1 1 1 0 DL CC1; BT_Threshold_DL+33 dB ≤ C Value < BT_Threshold_DL+36 dB 1 1 1 1 DL CC1; BT_Threshold_DL+36 dB ≤ C Value < BT_Threshold_DL+39 dB

***** Next modification *****

### 11.2.51	EC Packet Downlink Ack/Nack

This message is sent on the EC-PACCH from the mobile station to the network to indicate the status of downlink RLC data blocks received and to report the channel quality of the downlink. The mobile station may optionally initiate an uplink TBF by including the EC Channel Request Description in the message.

Message type:	EC Packet Downlink Ack/Nack

Direction:	mobile station to network

Table 11.2.51.1: EC Packet Downlink Ack/Nack information elements

< EC Packet Downlink Ack/Nack message content > ::= < DOWNLINK_TFI : bit (5) > < MS OUT OF MEMORY : bit(1)> { 0 < EC Ack/Nack Description : < EC Ack/Nack Description IE >> -- Final Ack Indicator = 0 ¦ 1 -- Final Ack Indicator = 1 } { 0 ¦ 1 { < EC Channel Quality Report : < EC-EGPRS Channel Quality Report IE > > < DL_CC_EST : bit (4) > } } { 0 ¦ 1 < EC Channel Request Description : < EC Channel Request Description struct > > } <padding bits >; < EC Channel Request Description struct > ::= < PRIORITY : bit (1) > < NUMBER_OF_UL_DATA_BLOCKS : bit (4) >;

Table 11.2.51.2: EC Packet Downlink Ack/Nack information element details

DOWNLINK_TFI (5 bit field) This field contains the TFI of the mobile station's downlink TBF. This field is defined in sub-clause 12.15.

EC Ack/Nack Description IE This information element is defined in sub-clause 12.72. EC Channel Quality Report IE This information element is defined in sub-clause 12.75.

MS OUT OF MEMORY (1 bit field) This field indicates if the MS no more has enough memory to perform Incremental Redundancy. 0 The MS has enough memory 1 The MS is running out of memory

PRIORITY This field indicates the priority of the transfer for which the mobile station is requesting uplink resources. 0 Normal priority 1 High priority NUMBER_OF_UL_DATA_BLOCKS This field indicates the number of UL RLC Data blocks, using MCS-1, that are requested by the mobile station. The field is coded as shown in the following table: bit 4 3 2 1 0 0 0 0 1 RLC data block 0 0 0 1 2 RLC data blocks . . . 1 1 1 0 15 RLC data blocks 1 1 1 1 >15 RLC data blocks

DL_CC_EST (4 bit field) This field contains the latest downlink Coverage Class estimation and the latest estimated C value range in case the C value is above BT_Threshold_DL, see 3GPP TS 45.008. The BT_Threshold_DL value is broadcast in EC SI 2, see 3GPP TS 44.018. The DL_CC_EST field shall be set according to the following table: bit 4 3 2 1 0 0 0 0 DL CC4 0 0 0 1 DL CC3 0 0 1 0 DL CC2 0 0 1 1 DL CC1; BT_Threshold_DL ≤ C Value < BT_Threshold_DL+3 dB 0 1 0 0 DL CC1; BT_Threshold_DL+3 dB ≤ C Value < BT_Threshold_DL+6 dB 0 1 0 1 DL CC1; BT_Threshold_DL+6 dB ≤ C Value < BT_Threshold_DL+9 dB 0 1 1 0 DL CC1; BT_Threshold_DL+9 dB ≤ C Value < BT_Threshold_DL+12 dB 0 1 1 1 DL CC1; BT_Threshold_DL+12 dB ≤ C Value < BT_Threshold_DL+15 dB 1 0 0 0 DL CC1; BT_Threshold_DL+15 dB ≤ C Value < BT_Threshold_DL+18 dB 1 0 0 1 DL CC1; BT_Threshold_DL+18 dB ≤ C Value < BT_Threshold_DL+21 dB 1 0 1 0 DL CC1; BT_Threshold_DL+21 dB ≤ C Value < BT_Threshold_DL+24 dB 1 0 1 1 DL CC1; BT_Threshold_DL+24 dB ≤ C Value < BT_Threshold_DL+27 dB 1 1 0 0 DL CC1; BT_Threshold_DL+27 dB ≤ C Value < BT_Threshold_DL+30 dB 1 1 0 1 DL CC1; BT_Threshold_DL+30 dB ≤ C Value < BT_Threshold_DL+33 dB 1 1 1 0 DL CC1; BT_Threshold_DL+33 dB ≤ C Value < BT_Threshold_DL+36 dB 1 1 1 1 DL CC1; BT_Threshold_DL+36 dB ≤ C Value < BT_Threshold_DL+39 dB

***** Next modification *****

### 11.2.52	EC Packet Downlink Assignment

This message is sent on the EC-PACCH by the network to the mobile station to assign resources for a downlink EC TBF.

Frequency parameters received as part of this assignment message shall be valid until a new assignment is received or each TBF of the mobile station are terminated.

Message type:	EC PACKET DOWNLINK ASSIGNMENT

Direction:	network to mobile station

Classification:	non-distribution message

Table 11.2.52.1: EC Packet Downlink ASSIGNMENT information elements

< EC Packet Downlink Assignment message content > ::= < USED_DL_COVERAGE_CLASS : < Used DL Coverage Class IE >> { 0 < Global TFI : < Global TFI IE > > { < CONTROL_ACK : bit (1) > { 0 ¦ 1 < Frequency Parameters : < Frequency Parameters struct > > } < DL_COVERAGE_CLASS : bit (2) > < STARTING_DL_TIMESLOT : bit (3) > < TIMESLOT_MULTIPLICATOR : bit (3) > < DOWNLINK_TFI_ASSIGNMENT : bit (5) > < UL_COVERAGE_CLASS : bit (2) > < STARTING_UL_TIMESLOT_OFFSET : bit (2) > { 0 ¦ 1 < EC Packet Timing Advance : < EC Packet Timing Advance IE > > } { 0 ¦ 1 < P0 : bit (4) > < PR_MODE : bit (1) > } { 0 ¦ 1 < GAMMA : bit (5) > < ALPHA Enable : bit (1) > }

< padding bits > // -- truncation at end of message allowed, bits '0' assumed ! < Non-distribution part error : bit (*) = < no string > > } ! < Address information part error : bit (*) = < no string > > ! < Message escape : { 1 } bit (*) = <no string> > } -- Extended for future changes ! < Distribution part error : bit (*) = < no string > > ;

< Frequency Parameters struct > ::= < EC_MA_NUMBER : bit (5) > < TSC : bit (3) > < Primary TSC Set : bit (1) >;

Table 11.2.52.2: EC PACKET Downlink ASSIGNMENT information element details

USED_DL_COVERAGE_CLASS (2 bit field) This information element is defined in sub-clause 12.71.

Global TFI This information element contains the TFI of one of the mobile station's downlink TBFs or uplink TBFs. This field is defined in sub-clause 12.10.

CONTROL_ACK (1 bit field) This field shall be set to '1' if the network establishes a new downlink TBF for the mobile station whose timer T3192 is running. Otherwise this field shall be set to '0'. DL_COVERAGE_CLASS (2 bit field) This field indicates the DL Coverage Class that is assigned for the DL EC TBF. The DL_COVERAGE_CLASS field shall be set according to the following table: bit 2 1 0 0 DL Coverage Class = CC1 0 1 DL Coverage Class = CC2 1 0 DL Coverage Class = CC3 1 1 DL Coverage Class = CC4

STARTING_DL_TIMESLOT (3 bit field) This field defines the timeslot number of the lowest timeslot included in the downlink assignment for the DL EC TBF. The number of additional timeslots that are included in the downlink assignment for the TBF depends on the assigned DL Coverage Class and the Timeslot Multiplicator setting. The assigned timeslots are contiguous, starting with the timeslot number indicated in the STARTING_DL_TIMESLOT field. The field is encoded according to the following table: bit 3 2 1 0 0 0 Timeslot 0 is the lowest assigned downlink timeslot 0 0 1 Timeslot 1 is the lowest assigned downlink timeslot 0 1 0 Timeslot 2 is the lowest assigned downlink timeslot 0 1 1 Timeslot 3 is the lowest assigned downlink timeslot 1 0 0 Timeslot 4 is the lowest assigned downlink timeslot 1 0 1 Timeslot 5 is the lowest assigned downlink timeslot 1 1 0 Timeslot 6 is the lowest assigned downlink timeslot 1 1 1 Timeslot 7 is the lowest assigned downlink timeslot A mobile station assigned CC2, CC3 or CC4 is always assigned 4 consecutive timeslots. Therefore only timeslot 0, 1, 2, 3 or 4 may be indicated as the lowest assigned downlink timeslot when the assignment is for CC2, CC3 or CC4 in the downlink. TIMESLOT_MULTIPLICATOR (3 bit field) This field defines how many DL timeslots that the assignment contains when the assigned DL Coverage Class, according to the DL_COVERAGE_CLASS field, is CC1. The field is coded according to the following table: bit 3 2 1 0 0 0 1 timeslot assigned 0 0 1 2 timeslots assigned 0 1 0 3 timeslots assigned 0 1 1 4 timeslots assigned 1 0 0 5 timeslots assigned 1 0 1 6 timeslots assigned 1 1 0 7 timeslots assigned 1 1 1 8 timeslots assigned Values other than ‘000’ can only be used if supported by the mobile station, as indicated by its multislot capability, see 3GPP TS 45.002. If the assigned downlink Coverage Class > CC1 (according to the DL_COVERAGE_CLASS field), the mobile station shall consider the TIMESLOT_MULTIPLICATOR field as not valid. UL_COVERAGE_CLASS (2 bit field) This field indicates the UL Coverage Class that is assigned for transfer of uplink RLC/MAC control messages on the EC-PACCH during the DL EC TBF. The UL_COVERAGE_CLASS field shall be set according to the following table: bit 2 1 0 0 UL Coverage Class = CC1 0 1 UL Coverage Class = CC2 1 0 UL Coverage Class = CC3 1 1 UL Coverage Class = CC4 EC Packet Timing Advance This information element is defined in sub-clause 12.74.

STARTING_UL_TIMESLOT_OFFSET (2 bit field) This field defines the timeslot number of the lowest timeslot included in the allocation that is used for transfer of uplink RLC/MAC control messages on the EC-PACCH during the DL EC TBF. The number of additional timeslots that are included in the uplink allocation depends on the assigned UL Coverage Class. The allocated timeslots are contiguous, starting with the timeslot number indicated in the STARTING_UL_TIMESLOT_OFFSET field. The STARTING_UL_TIMESLOT_OFFSET field is encoded as an offset to the timeslot assigned with the STARTING_DL_TIMESLOT field. The encoding of the field is dependent of the value of the DL_COVERAGE_CLASS and UL_COVERAGE_CLASS fields in the same message. If both DL_COVERAGE_CLASS and UL_COVERAGE_CLASS > CC1, the same timeslots are assigned in both the uplink and the downlink directions. The STARTING_UL_TIMESLOT_OFFSET field can then be considered as not valid by the mobile station. If DL_COVERAGE_CLASS = CC1 and UL_COVERAGE_CLASS > CC1, the STARTING_UL_TIMESLOT_OFFSET field is encoded according to the following table: bit 2 1 0 0 same timeslot as indicated by STARTING_DL_TIMESLOT 0 1 STARTING_DL_TIMESLOT - 1 1 0 STARTING_DL_TIMESLOT - 2 1 1 STARTING_DL_TIMESLOT - 3 If DL_COVERAGE_CLASS > CC1 and UL_COVERAGE_CLASS = CC1, the STARTING_UL_TIMESLOT_OFFSET field is encoded according to the following table: bit 2 1 0 0 same timeslot as indicated by STARTING_DL_TIMESLOT 0 1 STARTING_DL_TIMESLOT + 1 1 0 STARTING_DL_TIMESLOT + 2 1 1 STARTING_DL_TIMESLOT + 3 If both DL_COVERAGE_CLASS and UL_COVERAGE_CLASS = CC1, the uplink is assigned to the timeslot indicated by the STARTING_DL_TIMESLOT field. The STARTING_UL_TIMESLOT_OFFSET field can then be considered as not valid by the mobile station.

P0 (4 bit field) This field is an optional downlink power control parameter. It is encoded as follows: bit 4 3 2 1 0 0 0 0 P0 = 0 dB 0 0 0 1 P0 = 2 dB 0 0 1 0 P0 = 4 dB : 1 1 1 1 P0 = 30 dB

PR_MODE (1 bit field) This field indicates the PR Management mode, as defined in 3GPP TS 45.008. The field is encoded as follows: 0 PR mode A: for one addressed MS 1 PR mode B: for all MSs GAMMA (5 bit field) This field is the binary representation of the parameter GCH for MS output power control in units of 2 dB, see 3GPP TS 45.008. The field is coded according to the following table: bit 5 4 3 2 1 0 0 0 0 0 GCH = 0 dB 0 0 0 0 1 GCH = 2 dB : : : : 1 1 1 1 0 GCH = 60 dB 1 1 1 1 1 GCH = 62 dB ALPHA Enable (1 bit field) This field indicates whether or not the mobile station shall use the ALPHA parameter (if sent as part of EC SI) for uplink power control (see 3GPP TS 45.008 [34]). The field is coded as follows: 0 The mobile station shall not use the ALPHA parameter 1 The mobile station shall use the ALPHA parameter EC_MA_NUMBER (5 bit field) This field indicates what EC Mobile Allocation set, as defined in the EC SYSTEM INFORMATION TYPE 1 message, the TBF is allocated to. Each EC Mobile Allocation set contains frequency parameters, consisting of: - radio frequencies (ARFCNs), HSN and MAIO, for a hopping radio frequency channel; or - a single ARFCN, for a non-hopping radio frequency channel. The field is encoded as a binary number with a range 0 to 31.

DOWNLINK_TFI_ASSIGNMENT (5 bit field) This information element, if present, assigns the TFI to the mobile station to identify the downlink TBF described by this message. TFI is encoded as defined in sub-clause 12.15. Primary TSC Set (1 bit field) This field indicates to which TSC set the TSC field in the same message, for use on the assigned resources, shall be associated. This field is coded as follows: 0 TSC set 1 1 Indicates TSC set 2 for 8PSK modulation and TSC set 3 (which is identical to TSC set 3 used for the CS domain) for GMSK modulation.

***** Next modification *****

### 11.2.53	EC Packet Downlink Dummy Control Block

This message is sent on the EC-PACCH by the network to the mobile station as a fill message with just the USED_DL_COVERAGE_CLASS parameter.

Message type:	EC PACKET DOWNLINK DUMMY CONTROL BLOCK

Direction:	network to mobile station

Classification:	distribution message

Table 11.2.53.1: EC Packet DOWNLINK DUMMY CONTROL BLOCK information elements

< EC Packet Downlink Dummy Control Block message content > ::= < USED_DL_COVERAGE_CLASS : < Used DL Coverage Class IE >> < padding bits > ! < Distribution part error : bit (*) = < no string > > ;

Table 11.2.53.2: EC Packet DOWNLINK DUMMY CONTROL BLOCK information element details

USED_DL_COVERAGE_CLASS (2 bit field) This information element is defined in sub-clause 12.71.

***** Next modification *****

### 11.2.54	EC Packet Polling Request

This message is sent on the EC-PACCH by the network to the mobile station to solicit an EC PACKET CONTROL ACKNOWLEDGEMENT message from the mobile station.

Message type:	EC PACKET POLLING REQUEST

Direction:	network to mobile station

Classification:	non-distribution message

Table 11.2.54.1: EC Packet POLLING REQUEST information elements

< EC Packet Polling Request message content > ::= < USED_DL_COVERAGE_CLASS : < Used DL Coverage Class IE >> { 0 < Global TFI : < Global TFI IE > > { < TYPE_OF_ACK : bit (1) > < padding bits > ! < Non-distribution part error : bit (*) = < no string > > } ! < Address information part error : bit (*) = < no string > > ! < Message escape : { 1 } bit (*) = <no string> > } -- Extended for future changes ! < Distribution part error : bit (*) = < no string > > ;

Table 11.2.54.2: EC Packet POLLING REQUEST information element details

USED_DL_COVERAGE_CLASS (2 bit field) This information element is defined in sub-clause 12.71.

Global TFI This information element contains the TFI of the mobile station's downlink TBF or uplink TBF. This field is defined in sub-clause 12.10.

TYPE_OF_ACK This field indicates the format of the EC PACKET CONTROL ACKNOWLEDGEMENT message requested from the mobile station by the EC PACKET POLLING REQUEST message. 0 EC PACKET CONTROL ACKNOWLEDGEMENT message format shall be sent as four access bursts 1 EC PACKET CONTROL ACKNOWLEDGEMENT message format shall be an RLC/MAC control block Depending on the UL Coverage Class assigned to the mobile station, the transmission of the EC PACKET CONTROL ACKNOWLEDGEMENT message shall be sent using a specific number of blind physical layer transmissions. This is valid both when the message format is four access bursts and when it is an RLC/MAC control block.

***** Next modification *****

### 11.2.55	EC Packet Power Control/Timing Advance

This message is sent on EC-PACCH by the network to the mobile station in order to update the mobile station timing advance or power control parameters.

Message type:	EC PACKET POWER CONTROL/TIMING ADVANCE

Direction:	network to mobile station

Classification:	non-distribution message

Table 11.2.55.1: EC Packet POwer CONTROL/TIMING ADVANCE information elements

< EC Packet Power Control/Timing Advance message content > ::= < USED_DL_COVERAGE_CLASS : < Used DL Coverage Class IE >> { < Global TFI : < Global TFI IE > > { { 0 ¦ 1 < T_AVG_T : bit (5) > } { 0 ¦ 1 < EC Packet Timing Advance : < EC Packet Timing Advance IE >> } { 0 ¦ 1 < GAMMA : bit (5) > } < padding bits > ! < Non-distribution part error : bit (*) = < no string > > } ! < Address information part error : bit (*) = < no string > > } ! < Distribution part error : bit (*) = < no string > > ;

Table 11.2.55.2: EC Packet POwer CONTROL/TIMING ADVANCE information element details

USED_DL_COVERAGE_CLASS (2 bit field) This information element is defined in sub-clause 12.71. Global TFI IE This information element contains the TFI of the mobile station's downlink TBF or uplink TBF. This field is defined in sub-clause 12.10. T_AVG_T (5 bit field) The T_AVG_T parameter is a signal strength filter period for power control in packet transfer mode. 2(k/2) / 6 multiframes, k = 0, 1, 2, ..., 25 (see 3GPP TS 45.008). Values greater than 25 shall be interpreted as 25 by the mobile station. EC Packet Timing Advance This information element is defined in sub-clause 12.74. GAMMA (5 bit field) This field is the binary representation of the parameter GCH for MS output power control in units of 2 dB, see 3GPP TS 45.008. The field is coded according to the following table: bit 5 4 3 2 1 0 0 0 0 0 GCH = 0 dB 0 0 0 0 1 GCH = 2 dB : : : : 1 1 1 1 0 GCH = 60 dB 1 1 1 1 1 GCH = 62 dB

***** Next modification *****

### 11.2.56	EC Packet TBF Release

This message is sent on the EC-PACCH by the network to the mobile station to initiate release of an uplink or downlink EC TBF.

Message type:	EC PACKET TBF RELEASE

Direction:	network to mobile station

Classification:	non-distribution message

Table 11.2.56.1: EC Packet TBF RELEASE information elements

< EC Packet TBF Release message content > ::= < USED_DL_COVERAGE_CLASS : < Used DL Coverage Class IE >> { 0 < GLOBAL_TFI : Global TFI IE > { < TBF_RELEASE_CAUSE : bit (4) = { 0000 ¦ 0010 } > { 0 ¦ 1 < WAIT_INDICATION : bit (8) > < WAIT _INDICATION_SIZE : bit (1) > } < padding bits > ! < Non-distribution part error : bit (*) = < no string > > } ! < Address information part error : bit (*) = < no string > > } ! < Distribution part error : bit (*) = < no string > > ;

Table 11.2.56.2: EC Packet TBF RELEASE information element details

USED_DL_COVERAGE_CLASS (2 bit field) This information element is defined in sub-clause 12.71.

Global TFI IE This information element contains the TFI of uplink or downlink TBF that is to be released. This field is defined in sub-clause 12.10.

TBF_RELEASE_CAUSE (4 bit field) This field indicates the reason for the release of the TBF. This field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 Normal release 0 0 1 0 Abnormal release All other values are reserved, the same behaviour in reception as if 'Abnormal release'.

WAIT_INDICATION (8 bit field) The Wait Indication field indicates the time the mobile station shall wait before attempting another channel request. This field is coded as the binary representation of the T3172 timeout value in units of 20 ms or in units of seconds. The units are indicated in the WAIT_INDICATION_SIZE field. Range 0 to 255.

WAIT_INDICATION_SIZE (1 bit field) This field indicates the units of the WAIT_INDICATION field. 0 the WAIT_INDICATION field is coded in units of s 1 the WAIT_INDICATION field is coded in units of 20 ms

***** Next modification *****

### 11.2.57	EC Packet Uplink Ack/Nack

This message is sent on the EC-PACCH by the network to the mobile station to indicate the status of the received RLC data blocks. This message may also include a fixed uplink allocation for the mobile station to (re)transmit not yet acknowledged/transmitted RLC Data blocks. This message may also update the timing advance and power control parameters.

If the EGPRS Channel Coding Command parameter is not included in the message, the mobile station shall continue to use the previous EGPRS Modulation and Coding Scheme.

Message type:	EC PACKET UPLINK ACK/NACK

Direction:	network to mobile station

Classification:	non-distribution message

Table 11.2.57.1: EC Packet UPlink Ack/Nack information elements

< EC Packet Uplink Ack/Nack message content > ::= < USED_DL_COVERAGE_CLASS : < Used DL Coverage Class IE >> { < UPLINK_TFI : bit (5) > { 0 { -- Final Ack Indicator = 0 { 00 < EC Ack/Nack Description : < EC Ack/Nack Description IE >> ¦ 01 { < CONTENTION_RESOLUTION_TLLI : bit (32) > < EC Primary Ack/Nack Description : < EC Primary Ack/Nack Description IE >> } ¦ 10 { < CONTENTION_RESOLUTION_rTLLI : bit (4) > < EC Primary Ack/Nack Description : < EC Primary Ack/Nack Description IE >> } } < PUAN Fixed Uplink Allocation : < PUAN Fixed Uplink Allocation struct > > < RESEGMENT : bit (1) > { 0 ¦ 1 < EGPRS Channel Coding Command : < EGPRS Modulation and Coding Scheme IE >> } { 0 ¦ 1 < UL_COVERAGE_CLASS : bit (2) > < STARTING_UL_TIMESLOT : bit (3) > < DL_COVERAGE_CLASS : bit (2) > < STARTING_DL_TIMESLOT_OFFSET : bit (2) > < TIMESLOT_MULTIPLICATOR : bit (3) > } } ¦ 1 { -- Final Ack Indicator = 1 { 0 ¦ 1 < CONTENTION_RESOLUTION_TLLI : bit (32) > } { 0 -- the mobile station shall not continue monitoring the EC-PACCH ¦ 1 { < T3238 : bit (3) > -- the mobile station shall continue monitoring the EC-PACCH < Initial Waiting Time : bit (2) > < EC-PACCH Monitoring Pattern : bit (2) > } } } { 0 ¦ 1 < EC Packet Timing Advance : < EC Packet Timing Advance IE > > } { 0 ¦ 1 < GAMMA : bit (5) > < ALPHA Enable : bit (1) > }

< padding bits > } ! < Non-distribution part error : bit (*) = <no string> > } ! < Address information part error : bit (*) = <no string> > } ! < Distribution part error : bit (*) = <no string> > ; < PUAN Fixed Uplink Allocation struct > ::= { 0 ¦ 1 < BSN_OFFSET : bit (2) > } < START_FIRST_UL_RLC_DATA_BLOCK : bit (4) > { 1 { 0 < DELAY_NEXT_UL_RLC_DATA_BLOCK : bit (3) > -- delay until start of next UL RLC Data block ¦ 1 -- next UL RLC Data block starts at first possible occasion after previous UL RLC Data block } } ** 0;

Table 11.2.57.2: EC Packet UPlink Ack/Nack information element details

USED_DL_COVERAGE_CLASS (2 bit field) This information element is defined in sub-clause 12.71.

UPLINK_TFI (5 bit field) This field identifies the uplink TBF to which this message applies. This field is coded the same as the TFI field defined in sub-clause 12.15.

EGPRS Modulation and Coding Scheme The EGPRS Modulation and Coding Scheme information element is defined in sub-clause 12.10d.

RESEGMENT (1 bit field) This field is defined in sub-clause 12.10e.

EC Ack/Nack Description IE This information element is defined in sub-clause 12.72.

EC Primary Ack/Nack Description IE This information element is defined in sub-clause 12.73.

UL_COVERAGE_CLASS (2 bit field) This field indicates the new UL Coverage Class that is assigned for the UL EC TBF. The UL_COVERAGE_CLASS field shall be set according to the following table: bit 2 1 0 0 UL Coverage Class = CC1 0 1 UL Coverage Class = CC2 1 0 UL Coverage Class = CC3 1 1 UL Coverage Class = CC4 In case the UL Coverage Class is changed to CC1, the assignment is limited to 1 timeslot.

STARTING_UL_TIMESLOT (3 bit field) This field defines the timeslot number of the lowest timeslot included in the uplink assignment for the UL EC TBF. The number of additional timeslots that are included in the uplink assignment for the TBF depends on the assigned UL Coverage Class. The assigned timeslots are contiguous, starting with the timeslot number indicated in the STARTING_UL_TIMESLOT field. The field is encoded according to the following table: bit 3 2 1 0 0 0 Timeslot 0 is the lowest assigned uplink timeslot 0 0 1 Timeslot 1 is the lowest assigned uplink timeslot 0 1 0 Timeslot 2 is the lowest assigned uplink timeslot 0 1 1 Timeslot 3 is the lowest assigned uplink timeslot 1 0 0 Timeslot 4 is the lowest assigned uplink timeslot 1 0 1 Timeslot 5 is the lowest assigned uplink timeslot 1 1 0 Timeslot 6 is the lowest assigned uplink timeslot 1 1 1 Timeslot 7 is the lowest assigned uplink timeslot A mobile station assigned CC2, CC3 or CC4 is always assigned 4 consecutive timeslots. Therefore only timeslot 0, 1, 2, 3 or 4 may be indicated as the lowest assigned uplink timeslot when the assignment is for CC2, CC3 or CC4 in the uplink.

DL_COVERAGE_CLASS (2 bit field) This field indicates the new DL Coverage Class that is assigned for transfer of downlink RLC/MAC control messages on the EC-PACCH during the UL EC TBF. The DL_COVERAGE_CLASS field shall be set according to the following table: bit 2 1 0 0 DL Coverage Class = CC1 0 1 DL Coverage Class = CC2 1 0 DL Coverage Class = CC3 1 1 DL Coverage Class = CC4

STARTING_DL_TIMESLOT_OFFSET (2 bit field) This field defines the timeslot number of the lowest timeslot included in the new assignment that is used for transfer of downlink RLC/MAC control messages on the EC-PACCH during the UL EC TBF. The number of additional timeslots that are included in the downlink assignment depends on the assigned DL Coverage Class. The assigned timeslots are contiguous, starting with the timeslot number indicated in the STARTING_DL_TIMESLOT_OFFSET field. The STARTING_DL_TIMESLOT_OFFSET field is encoded as an offset to the timeslot assigned with the STARTING_UL_TIMESLOT field. The encoding of the field is dependent of the value of the UL_COVERAGE_CLASS and DL_COVERAGE_CLASS fields in the same message. If both UL_COVERAGE_CLASS and DL_COVERAGE_CLASS > CC1, the same timeslots are assigned in both the uplink and the downlink directions. The STARTING_DL_TIMESLOT_OFFSET field can then be considered as not valid by the mobile station. If UL_COVERAGE_CLASS = CC1 and DL_COVERAGE_CLASS > CC1, the STARTING_DL_TIMESLOT_OFFSET field is encoded according to the following table: bit 2 1 0 0 same timeslot as indicated by STARTING_UL_TIMESLOT 0 1 STARTING_UL_TIMESLOT - 1 1 0 STARTING_UL_TIMESLOT - 2 1 1 STARTING_UL_TIMESLOT - 3 If UL_COVERAGE_CLASS > CC1 and DL_COVERAGE_CLASS = CC1, the STARTING_DL_TIMESLOT_OFFSET field is encoded according to the following table: bit 2 1 0 0 same timeslot as indicated by STARTING_UL_TIMESLOT 0 1 STARTING_UL_TIMESLOT + 1 1 0 STARTING_UL_TIMESLOT + 2 1 1 STARTING_UL_TIMESLOT + 3 If both UL_COVERAGE_CLASS and DL_COVERAGE_CLASS = CC1, the downlink is assigned to the timeslot indicated by the STARTING_UL_TIMESLOT field. If timer T3248 (sent in EC SI) is used, the STARTING_DL_TIMESLOT_OFFSET field defines the lowest assigned timeslot for EC-PACCH transfers using CC2 while the timer is running. The STARTING_DL_TIMESLOT_OFFSET field is then encoded according to the following table: bit 2 1 0 0 same timeslot as indicated by STARTING_UL_TIMESLOT 0 1 STARTING_UL_TIMESLOT - 1 1 0 STARTING_UL_TIMESLOT - 2 1 1 no valid CC2 mapping applies while T3248 is running, CC1 to be used

TIMESLOT_MULTIPLICATOR (3 bit field) This field defines how many UL timeslots that the assignment contains when the assigned UL Coverage Class, according to the UL_COVERAGE_CLASS field, is CC1. The field is coded according to the following table: bit 3 2 1 0 0 0 1 timeslot assigned 0 0 1 2 timeslots assigned 0 1 0 3 timeslots assigned 0 1 1 4 timeslots assigned 1 0 0 5 timeslots assigned 1 0 1 6 timeslots assigned 1 1 0 7 timeslots assigned 1 1 1 8 timeslots assigned Values other than ‘000’ can only be used if supported by the mobile station, as indicated by its multislot capability, see 3GPP TS 45.002. If the assigned uplink Coverage Class > CC1 (according to the UL_COVERAGE_CLASS field), the mobile station shall consider the TIMESLOT_MULTIPLICATOR field as not valid

T3238 (3 bit field) This field is the binary representation of the timeout value of timer T3238. Range: 0 to 7. The timeout value is given as the binary value plus one in units of 1 second. Initial Waiting Time (2 bit field) This field gives the time that the mobile station shall wait, after the reception of this EC Packet Uplink Ack/Nack message, before it shall start to monitor the DL EC-PACCH. The waiting time is specified as the 52-multiframe, counted from the one containing the EC Packet Uplink Ack/Nack message, where the mobile station shall start monitoring the DL EC-PACCH. The field is coded according to the following table: Bit 2 1 0 0 2nd 52-multiframe 0 1 4th 52-multiframe 1 0 6th 52-multiframe 1 1 8th 52-multiframe EC-PACCH Monitoring Pattern (2 bit field) The EC-PACCH Monitoring Pattern field indicates what pattern the mobile station shall use when monitoring the downlink EC-PACCH while timer T3238 is running. The pattern shall be applied after first monitoring the 52-multiframe as indicated by the Initial Waiting Time field. The EC-PACCH Monitoring Pattern field is coded according to the following table: bit 2 1 0 0 Continuous monitoring 0 1 Monitor every 2nd 52-multiframe 1 0 Monitor every 4th 52-multiframe 1 1 Monitor every 8th 52-multiframe

CONTENTION_RESOLUTION_TLLI (32 bit field) The CONTENTION_RESOLUTION_TLLI field is present only if the network has decoded one of the uplink RLC data blocks containing the TLLI. The mobile station shall perform the contention resolution function if the TLLI information element is present. This field contains a TLLI, which is defined in sub-clause 12.16. CONTENTION_RESOLUTION_rTLLI (4 bit field) The CONTENTION_RESOLUTION_rTLLI field is present only if the network has decoded one of the uplink RLC data blocks containing the rTLLI, and not any uplink RLC data block containing the TLLI. This field contains an rTLLI, which consists of the 4 least significant bits of the TLLI.

EC Packet Timing Advance This information element is defined in sub-clause 12.74.

GAMMA (5 bit field) This field is the binary representation of the parameter GCH for MS output power control in units of 2 dB, see 3GPP TS 45.008. The field is coded according to the following table: bit 5 4 3 2 1 0 0 0 0 0 GCH = 0 dB 0 0 0 0 1 GCH = 2 dB : : : : 1 1 1 1 0 GCH = 60 dB 1 1 1 1 1 GCH = 62 dB ALPHA Enable (1 bit field) This field indicates whether or not the mobile station shall use the ALPHA parameter (if sent as part of EC SI) for uplink power control (see 3GPP TS 45.008 [34]). The field is coded as follows: 0 The mobile station shall not use the ALPHA parameter 1 The mobile station shall use the ALPHA parameter BSN_OFFSET (2 bit field) This field defines the offset, in the list of available RLC Data blocks for transmission, that shall be applied by the mobile station when selecting the RLC Data blocks to transmit in the fixed uplink allocation. If the BSN_OFFSET field is not present, the mobile station shall select the RLC Data blocks to transmit in the resources within the fixed uplink allocation as described in sub-clause 9.1.3.4. It shall e.g. start with the oldest NACKED block, if available, in the resources for the first RLC Data block within the fixed uplink allocation. If the BSN_OFFSET field is present, the mobile station shall apply an offset in RLC Data blocks and skip one, or several, of the RLC Data blocks that would be transmitted first according to the procedures in sub-clause 9.1.3.4. This field is encoded according to the following table: bit 2 1 0 0 Start with the 2nd oldest RLC Data, to be transmitted, in the first resource within the fixed uplink allocation 0 1 Start with the 3rd oldest RLC Data, to be transmitted, in the first resource within the fixed uplink allocation 1 0 Start with the 4th oldest RLC Data, to be transmitted, in the first resource within the fixed uplink allocation 1 1 Start with the 5th oldest RLC Data, to be transmitted, in the first resource within the fixed uplink allocation

START_FIRST_UL_RLC_DATA_BLOCK (4 bit field) This field indicates the starting position of the resources for the first allocated RLC Data block in the fixed uplink allocation. The starting position is relative to the first TDMA frame (N) of the downlink block containing the last blind physical layer transmission of this EC PACKET UPLINK ACK/NACK message, according to the USED_DL_COVERAGE_CLASS field. The encoding of the field is dependent of the uplink Coverage Class and TIMESLOT_MULTIPLICATOR assigned for the uplink EC TBF. If uplink Coverage Class = CC1 and TIMESLOT_MULTIPLICATOR = 0 or if uplink Coverage Class = CC2, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the TDMA frame (N+13) mod 2715648 0 0 0 1 In the TDMA frame (N+17 or N+18) mod 2715648 0 0 1 0 In the TDMA frame (N+21 or N+22) mod 2715648 0 0 1 1 In the TDMA frame (N+26) mod 2715648 0 1 0 0 In the TDMA frame (N+30 or N+31) mod 2715648 …. 1 1 1 0 In the TDMA frame (N+73 or N+74) mod 2715648 1 1 1 1 In the TDMA frame (N+78) mod 2715648 If uplink Coverage Class = CC1 and TIMESLOT_MULTIPLICATOR = 1, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 0 1 1 In the timeslot with the highest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the lowest TN in TDMA frame (N+43 or N+44) mod 2715648 1 1 1 1 In the timeslot with the highest TN in TDMA frame (N+43 or N+44) mod 2715648 If uplink Coverage Class = CC1 and TIMESLOT_MULTIPLICATOR = 2, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 1 0 0 In the timeslot with the 2nd lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 1 0 1 In the timeslot with the highest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the highest TN in TDMA frame (N+30 or N+31) mod 2715648 1 1 1 1 In the timeslot with the lowest TN in TDMA frame (N+34 or N+35) mod 2715648 If uplink Coverage Class = CC1 and TIMESLOT_MULTIPLICATOR = 3, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 1 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+26) mod 2715648 1 1 1 1 In the timeslot with the highest TN in TDMA frame (N+26) mod 2715648

If uplink_Coverage_Class = CC1 and TIMESLOT_MULTIPLICATOR = 4, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the highest TN in TDMA frame (N+21 or N+22) mod 2715648 1 1 1 1 In the timeslot with the lowest TN in TDMA frame (N+26) mod 2715648 If uplink_Coverage_Class = CC1 and TIMESLOT_MULTIPLICATOR = 5, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the 5th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 1 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+21 or N+22) mod 2715648 1 1 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+21 or N+22) mod 2715648 If uplink_Coverage_Class = CC1 and TIMESLOT_MULTIPLICATOR = 6, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the 5th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the 6th lowest TN in TDMA frame (N+13) mod 2715648 0 1 1 0 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 1 1 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the lowest TN in TDMA frame (N+21 or N+22) mod 2715648 1 1 1 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+21 or N+22) mod 2715648 If uplink_Coverage_Class = CC1 and TIMESLOT_MULTIPLICATOR = 7, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the 5th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the 6th lowest TN in TDMA frame (N+13) mod 2715648 0 1 1 0 In the timeslot with the 7th lowest TN in TDMA frame (N+13) mod 2715648 0 1 1 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 1 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the 7th lowest TN in TDMA frame (N+17 or N+18) mod 2715648 1 1 1 1 In the timeslot with the highest TN in TDMA frame (N+17 or N+18) mod 2715648

If uplink Coverage Class = CC3, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the TDMA frame (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 0 0 0 1 In the TDMA frame (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 0 0 1 0 In the TDMA frame (N+30 or N+31) mod 2715648 or (N+34 or N+35) mod 2715648 0 0 1 1 In the TDMA frame (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 0 1 0 0 In the TDMA frame (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 …. 1 1 1 0 In the TDMA frame (N+134 or N+135) mod 2715648 or (N+138 or N+139) mod 2715648 1 1 1 1 In the TDMA frame (N+143) mod 2715648 or (N+147 or N+148) mod 2715648 If uplink Coverage Class = CC4, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the TDMA frame (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 or (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 0 0 0 1 In the TDMA frame (N+30 or N+31) mod 2715648 or (N+34 or N+35) mod 2715648 or (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 0 0 1 0 In the TDMA frame (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 or (N+56 or N+57) mod 2715648 or (N+60 or N+61) mod 2715648 0 0 1 1 In the TDMA frame (N+65) mod 2715648 or (N+69 or N+70) mod 2715648 or (N+73 or N+74) mod 2715648 or (N+78) mod 2715648 0 1 0 0 In the TDMA frame (N+82 or N+83) mod 2715648 or (N+86 or N+87) mod 2715648 or (N+91) mod 2715648 or (N+95 or N+96) mod 2715648 …. 1 1 1 0 In the TDMA frame (N+255 or N+256) mod 2715648 or (N+260) mod 2715648 or (N+264 or N+265) mod 2715648 or (N+268 or N+269) mod 2715648 1 1 1 1 In the TDMA frame (N+273) mod 2715648 or (N+277 or N+278) mod 2715648 or (N+281 or N+282) mod 2715648 or (N+286) mod 2715648

DELAY_NEXT_UL_RLC_DATA_BLOCK (3 bit field) This field indicates the position of the next allocated RLC Data block in the fixed uplink allocation. The position is calculated as the number of transmission opportunities from the previous uplink allocation, according to the assigned UL_COVERAGE_CLASS and TIMESLOT_MULTIPLICATOR, where the next uplink allocation for the mobile station is placed and where the mobile station thus shall transmit the next UL RLC data block. In the transmission opportunity or opportunities between the previous uplink allocation and this allocation the mobile station is not allocated any resource, and shall thus not transmit. This field is encoded according to the following table: bit 3 2 1 0 0 0 In the 2nd next transmission opportunity 0 0 1 In the 3rd next transmission opportunity 0 1 0 In the 4th next transmission opportunity …. 1 1 0 In the 8th next transmission opportunity 1 1 1 In the 9th next transmission opportunity

***** Next modification *****

### 11.2.58	EC Packet Uplink Assignment

This message is sent on the EC-PACCH by the network to the mobile station to assign an uplink EC TBF, including a fixed uplink allocation. Frequency parameters received as part of this assignment message shall be valid until a new assignment is received or until each TBF of the mobile station is terminated.

If the EGPRS Channel Coding Command parameter is not included in the message, the mobile station is commanded to use MCS-1.

Message type:	EC PACKET UPLINK ASSIGNMENT

Direction:	network to mobile station

Classification:	non-distribution message

Table 11.2.58.1: EC Packet UPlink ASSIGNMENT information elements

< EC Packet Uplink Assignment message content > ::= < USED_DL_COVERAGE_CLASS : < Used DL Coverage Class IE >> { 0 < Global TFI : < Global TFI IE > >

{ { 0 ¦ 1 < UPLINK_TFI_ASSIGNMENT : bit (5) > } { 0 ¦ 1 < EGPRS Channel Coding Command : < EGPRS Modulation and Coding Scheme IE >> } < OVERLAID_CDMA_CODE : bit (2) > { 0 ¦ 1 < EC Packet Timing Advance : < EC Packet Timing Advance IE > > } { 0 ¦ 1 < Frequency Parameters : < Frequency Parameters struct > > } < UL_COVERAGE_CLASS : bit (2) > < STARTING_UL_TIMESLOT : bit (3) > < TIMESLOT_MULTIPLICATOR : bit (3) > < Fixed Uplink Allocation : < Fixed Uplink Allocation struct > > { 0 ¦ 1 < P0 : bit (4) > < PR_MODE : bit (1) > } { 0 ¦ 1 < GAMMA : bit (5) > < ALPHA Enable : bit (1) > } < DL_COVERAGE_CLASS : bit (2) > < STARTING_DL_TIMESLOT_OFFSET : bit (2) >

< padding bits > // -- truncation at end of message allowed, bits '0' assumed ! < Non-distribution part error : bit (*) = < no string > > } ! < Address information part error : bit (*) = < no string > > ! < Message escape : { 1 } bit (*) = <no string> > } -- Extended for future changes ! < Distribution part error : bit (*) = < no string > > ; < Frequency Parameters struct > ::= < EC_MA_NUMBER : bit (5) > < TSC : bit (3) > < Primary TSC Set : bit (1) >;

< Fixed Uplink Allocation struct > ::= < START_FIRST_UL_RLC_DATA_BLOCK : bit (4) > { 1 { 0 < DELAY_NEXT_UL_RLC_DATA_BLOCK : bit (3) > -- delay until start of next UL RLC Data block ¦ 1 -- next UL RLC Data block starts at first possible occasion after previous UL RLC Data block } } ** 0;

Table 11.2.58.2: EC Packet UPlink ASSIGNMENT information element details

USED_DL_COVERAGE_CLASS (2 bit field) This information element is defined in sub-clause 12.71.

Global TFI This information element identifies the uplink TFI, if available, or the downlink TFI, to which this message applies. This field is defined in sub-clause 12.10.

EGPRS Modulation and Coding Scheme The EGPRS Modulation and Coding Scheme information element is defined in sub-clause 12.10d. OVERLAID_CDMA_CODE (2 bit field) The Overlaid CDMA Code field indicates the Overlaid CDMA Code that shall be used at transmissions during the uplink EC TBF, see 3GPP TS 45.002. This field is encoded according to the following table, see 3GPP TS 45.002 for definition of the codes: bit 2 1 0 0 Overlaid CDMA Code 0 0 1 Overlaid CDMA Code 1 1 0 Overlaid CDMA Code 2 1 1 Overlaid CDMA Code 3 Note: Overlaid CDMA Code 0 corresponds to that the bursts are transmitted as if no code was applied.

UPLINK_TFI_ASSIGNMENT (5 bit field) This information element, if present, assigns the contained TFI to the mobile station to identify the uplink TBF described by this message. This field is coded the same as the TFI field defined in sub-clause 12.15.

EC Packet Timing Advance This information element is defined in sub-clause 12.74.

EC_MA_NUMBER (5 bit field) This field indicates what EC Mobile Allocation set, as defined in the EC SYSTEM INFORMATION TYPE 1 message, the TBF is allocated to. Each EC Mobile Allocation set contains frequency parameters, consisting of: - radio frequencies (ARFCNs), HSN and MAIO, for a hopping radio frequency channel; or - a single ARFCN, for a non-hopping radio frequency channel. The field is encoded as a binary number with a range 0 to 31.

UL_COVERAGE_CLASS (2 bit field) This field indicates the UL Coverage Class that is assigned for the UL EC TBF. The UL_COVERAGE_CLASS field shall be set according to the following table: bit 2 1 0 0 UL Coverage Class = CC1 0 1 UL Coverage Class = CC2 1 0 UL Coverage Class = CC3 1 1 UL Coverage Class = CC4

STARTING_UL_TIMESLOT (3 bit field) This field defines the timeslot number of the lowest timeslot included in the uplink assignment for the UL EC TBF. The number of additional timeslots that are included in the uplink assignment for the TBF depends on the assigned UL Coverage Class and the Timeslot Multiplicator setting. The assigned timeslots are contiguous, starting with the timeslot number indicated in the STARTING_UL_TIMESLOT field. The field is encoded according to the following table: bit 3 2 1 0 0 0 Timeslot 0 is the lowest assigned uplink timeslot 0 0 1 Timeslot 1 is the lowest assigned uplink timeslot 0 1 0 Timeslot 2 is the lowest assigned uplink timeslot 0 1 1 Timeslot 3 is the lowest assigned uplink timeslot 1 0 0 Timeslot 4 is the lowest assigned uplink timeslot 1 0 1 Timeslot 5 is the lowest assigned uplink timeslot 1 1 0 Timeslot 6 is the lowest assigned uplink timeslot 1 1 1 Timeslot 7 is the lowest assigned uplink timeslot A mobile station assigned CC2, CC3 or CC4 is always assigned 4 consecutive timeslots. Therefore only timeslot 0, 1, 2, 3 or 4 may be indicated as the lowest assigned uplink timeslot when the assignment is for CC2, CC3 or CC4 in the uplink.

TIMESLOT_MULTIPLICATOR (3 bit field) This field defines how many UL timeslots that the assignment contains when the assigned UL Coverage Class, according to the UL_COVERAGE_CLASS field, is CC1. The field is coded according to the following table: bit 3 2 1 0 0 0 1 timeslot assigned 0 0 1 2 timeslots assigned 0 1 0 3 timeslots assigned 0 1 1 4 timeslots assigned 1 0 0 5 timeslots assigned 1 0 1 6 timeslots assigned 1 1 0 7 timeslots assigned 1 1 1 8 timeslots assigned Values other than ‘000’ can only be used if supported by the mobile station, as indicated by its multislot capability, see 3GPP TS 45.002. If the assigned uplink Coverage Class > CC1 (according to the UL_COVERAGE_CLASS field), the mobile station shall consider the TIMESLOT_MULTIPLICATOR field as not valid

START_FIRST_UL_RLC_DATA_BLOCK (4 bit field) This field indicates the starting position of the resources for the first allocated RLC Data block in the fixed uplink allocation. The starting position is relative to the first TDMA frame (N) of the downlink block containing the last blind physical layer transmission of this EC PACKET UPLINK ASSIGNMENT message, according to the USED_DL_COVERAGE_CLASS field. The encoding of the field is dependent of the value of the UL_COVERAGE_CLASS and the TIMESLOT_MULTIPLICATOR fields in the same message. If UL_COVERAGE_CLASS = CC1 and TIMESLOT_MULTIPLICATOR = 0 or if UL_COVERAGE_CLASS = CC2, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the TDMA frame (N+13) mod 2715648 0 0 0 1 In the TDMA frame (N+17 or N+18) mod 2715648 0 0 1 0 In the TDMA frame (N+21 or N+22) mod 2715648 0 0 1 1 In the TDMA frame (N+26) mod 2715648 0 1 0 0 In the TDMA frame (N+30 or N+31) mod 2715648 …. 1 1 1 0 In the TDMA frame (N+73 or N+74) mod 2715648 1 1 1 1 In the TDMA frame (N+78) mod 2715648 If UL_COVERAGE_CLASS = CC1 and TIMESLOT_MULTIPLICATOR = 1, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 0 1 1 In the timeslot with the highest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the lowest TN in TDMA frame (N+43 or N+44) mod 2715648 1 1 1 1 In the timeslot with the highest TN in TDMA frame (N+43 or N+44) mod 2715648 If UL_COVERAGE_CLASS = CC1 and TIMESLOT_MULTIPLICATOR = 2, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 1 0 0 In the timeslot with the 2nd lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 1 0 1 In the timeslot with the highest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the highest TN in TDMA frame (N+30 or N+31) mod 2715648 1 1 1 1 In the timeslot with the lowest TN in TDMA frame (N+34 or N+35) mod 2715648 If UL_COVERAGE_CLASS = CC1 and TIMESLOT_MULTIPLICATOR = 3, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 1 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+26) mod 2715648 1 1 1 1 In the timeslot with the highest TN in TDMA frame (N+26) mod 2715648

If UL_COVERAGE_CLASS = CC1 and TIMESLOT_MULTIPLICATOR = 4, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the highest TN in TDMA frame (N+21 or N+22) mod 2715648 1 1 1 1 In the timeslot with the lowest TN in TDMA frame (N+26) mod 2715648 If UL_COVERAGE_CLASS = CC1 and TIMESLOT_MULTIPLICATOR = 5, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the 5th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 1 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+21 or N+22) mod 2715648 1 1 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+21 or N+22) mod 2715648 If UL_COVERAGE_CLASS = CC1 and TIMESLOT_MULTIPLICATOR = 6, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the 5th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the 6th lowest TN in TDMA frame (N+13) mod 2715648 0 1 1 0 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 1 1 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the lowest TN in TDMA frame (N+21 or N+22) mod 2715648 1 1 1 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+21 or N+22) mod 2715648 If UL_COVERAGE_CLASS = CC1 and TIMESLOT_MULTIPLICATOR = 7, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the 5th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the 6th lowest TN in TDMA frame (N+13) mod 2715648 0 1 1 0 In the timeslot with the 7th lowest TN in TDMA frame (N+13) mod 2715648 0 1 1 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 1 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the 7th lowest TN in TDMA frame (N+17 or N+18) mod 2715648 1 1 1 1 In the timeslot with the highest TN in TDMA frame (N+17 or N+18) mod 2715648

If UL_COVERAGE_CLASS = CC3, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the TDMA frame (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 0 0 0 1 In the TDMA frame (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 0 0 1 0 In the TDMA frame (N+30 or N+31) mod 2715648 or (N+34 or N+35) mod 2715648 0 0 1 1 In the TDMA frame (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 0 1 0 0 In the TDMA frame (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 …. 1 1 1 0 In the TDMA frame (N+134 or N+135) mod 2715648 or (N+138 or N+139) mod 2715648 1 1 1 1 In the TDMA frame (N+143) mod 2715648 or (N+147 or N+148) mod 2715648 If UL_COVERAGE_CLASS = CC4, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the TDMA frame (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 or (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 0 0 0 1 In the TDMA frame (N+30 or N+31) mod 2715648 or (N+34 or N+35) mod 2715648 or (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 0 0 1 0 In the TDMA frame (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 or (N+56 or N+57) mod 2715648 or (N+60 or N+61) mod 2715648 0 0 1 1 In the TDMA frame (N+65) mod 2715648 or (N+69 or N+70) mod 2715648 or (N+73 or N+74) mod 2715648 or (N+78) mod 2715648 0 1 0 0 In the TDMA frame (N+82 or N+83) mod 2715648 or (N+86 or N+87) mod 2715648 or (N+91) mod 2715648 or (N+95 or N+96) mod 2715648 …. 1 1 1 0 In the TDMA frame (N+255 or N+256) mod 2715648 or (N+260) mod 2715648 or (N+264 or N+265) mod 2715648 or (N+268 or N+269) mod 2715648 1 1 1 1 In the TDMA frame (N+273) mod 2715648 or (N+277 or N+278) mod 2715648 or (N+281 or N+282) mod 2715648 or (N+286) mod 2715648

DELAY_NEXT_UL_RLC_DATA_BLOCK (3 bit field) This field indicates the position of the next allocated RLC Data block in the fixed uplink allocation. The position is calculated as the number of transmission opportunities from the previous uplink allocation, according to the UL_COVERAGE_CLASS and TIMESLOT_MULTIPLICATOR fields in the same assignment message, where the next uplink allocation for the mobile station is placed and where the mobile station thus shall transmit the next RLC data block. In the transmission opportunity/ies between the previous uplink allocation and this allocation the mobile station is not allocated any resource, and shall thus not transmit. This field is encoded according to the following table: bit 3 2 1 0 0 0 In the 2nd next transmission opportunity 0 0 1 In the 3rd next transmission opportunity 0 1 0 In the 4th next transmission opportunity …. 1 1 0 In the 8th next transmission opportunity 1 1 1 In the 9th next transmission opportunity

GAMMA (5 bit field) This field is the binary representation of the parameter GCH for MS output power control in units of 2 dB, see 3GPP TS 45.008. The field is coded according to the following table: bit 5 4 3 2 1 0 0 0 0 0 GCH = 0 dB 0 0 0 0 1 GCH = 2 dB : : : : 1 1 1 1 0 GCH = 60 dB 1 1 1 1 1 GCH = 62 dB ALPHA Enable (1 bit field) This field indicates whether or not the mobile station shall use the ALPHA parameter (if sent as part of EC SI) for uplink power control (see 3GPP TS 45.008 [34]). The field is coded as follows: 0 The mobile station shall not use the ALPHA parameter 1 The mobile station shall use the ALPHA parameter P0 (4 bit field) This field is an optional downlink power control parameter. It is encoded as follows: bit 4 3 2 1 0 0 0 0 P0 = 0 dB 0 0 0 1 P0 = 2 dB 0 0 1 0 P0 = 4 dB : 1 1 1 1 P0 = 30 dB

PR_MODE (1 bit field) This field indicates the PR Management mode, as defined in 3GPP TS 45.008. The field is encoded as follows: 0 PR mode A: for one addressed MS 1 PR mode B: for all MSs

Primary TSC Set (1 bit field) This field indicates to which TSC set the TSC field in the same message, for use on the assigned resources, shall be associated. This field is coded as follows: 0 TSC set 1 1 Indicates TSC set 2 for 8PSK modulation and TSC set 3 (which is identical to TSC set 3 used for the CS domain) for GMSK modulation. DL_COVERAGE_CLASS (2 bit field) This field indicates the DL Coverage Class that is assigned for transfer of downlink RLC/MAC control messages on the EC-PACCH during the UL EC TBF. The DL_COVERAGE_CLASS field shall be set according to the following table: bit 2 1 0 0 DL Coverage Class = CC1 0 1 DL Coverage Class = CC2 1 0 DL Coverage Class = CC3 1 1 DL Coverage Class = CC4

STARTING_DL_TIMESLOT_OFFSET (2 bit field) This field defines the timeslot number of the lowest timeslot included in the assignment that is used for transfer of downlink RLC/MAC control messages on the EC-PACCH during the UL EC TBF. The number of additional timeslots that are included in the downlink assignment depends on the assigned DL Coverage Class. The assigned timeslots are contiguous, starting with the timeslot number indicated in the STARTING_DL_TIMESLOT_OFFSET field. The STARTING_DL_TIMESLOT_OFFSET field is encoded as an offset to the timeslot assigned with the STARTING_UL_TIMESLOT field. The encoding of the field is dependent of the value of the UL_COVERAGE_CLASS and DL_COVERAGE_CLASS fields in the same message. If both UL_COVERAGE_CLASS and DL_COVERAGE_CLASS > CC1, the same timeslots are assigned in both the uplink and the downlink directions. The STARTING_DL_TIMESLOT_OFFSET field can then be considered as not valid by the mobile station. If UL_COVERAGE_CLASS = CC1 and DL_COVERAGE_CLASS > CC1, the STARTING_DL_TIMESLOT_OFFSET field is encoded according to the following table: bit 2 1 0 0 same timeslot as indicated by STARTING_UL_TIMESLOT 0 1 STARTING_UL_TIMESLOT - 1 1 0 STARTING_UL_TIMESLOT - 2 1 1 STARTING_UL_TIMESLOT - 3 If UL_COVERAGE_CLASS > CC1 and DL_COVERAGE_CLASS = CC1, the STARTING_DL_TIMESLOT_OFFSET field is encoded according to the following table: bit 2 1 0 0 same timeslot as indicated by STARTING_UL_TIMESLOT 0 1 STARTING_UL_TIMESLOT + 1 1 0 STARTING_UL_TIMESLOT + 2 1 1 STARTING_UL_TIMESLOT + 3 If both UL_COVERAGE_CLASS and DL_COVERAGE_CLASS = CC1, the downlink is assigned to the timeslot indicated by the STARTING_UL_TIMESLOT field. If timer T3248 (sent in EC SI) is used, the STARTING_DL_TIMESLOT_OFFSET field defines the lowest assigned timeslot for EC-PACCH transfers using CC2 while the timer is running. The STARTING_DL_TIMESLOT_OFFSET field is then encoded according to the following table: bit 2 1 0 0 same timeslot as indicated by STARTING_UL_TIMESLOT 0 1 STARTING_UL_TIMESLOT - 1 1 0 STARTING_UL_TIMESLOT - 2 1 1 no valid CC2 mapping applies while T3248 is running, CC1 to be used

***** Next modification *****

### 11.2.59	EC Packet Uplink Ack/Nack and Contention Resolution

This message is sent on the EC-PACCH by the network to the mobile station to indicate the status of the received RLC data blocks, include a fixed uplink allocation for the mobile station to (re)transmit not yet acknowledged/transmitted RLC Data blocks. The message includes the TLLI for contention resolution.

The mobile station shall continue to use the previous EGPRS Modulation and Coding Scheme when transmitting in the resources allocated in this message.

Message type:	EC PACKET UPLINK ACK/NACK AND CONTENTION RESOLUTION

Direction:	network to mobile station

Classification:	non-distribution message

Table 11.2.59.1: EC Packet UPlink ACK/NACK AND CONTENTION RESOLUTION information elements

< EC Packet Uplink Ack/Nack and Contention Resolution message content > ::= < USED_DL_COVERAGE_CLASS : < Used DL Coverage Class IE >> { < UPLINK_TFI : bit (5) > { < CONTENTION_RESOLUTION_TLLI : bit (32) > < EC Primary Ack/Nack Description : < EC Primary Ack/Nack Description IE >> < PUANCR Fixed Uplink Allocation : < PUANCR Fixed Uplink Allocation struct > > < RESEGMENT : bit (1) > < padding bits > } ! < Non-distribution part error : bit (*) = <no string> > } ! < Address information part error : bit (*) = <no string> > } ! < Distribution part error : bit (*) = <no string> > ; < PUANCR Fixed Uplink Allocation struct > ::= < START_FIRST_UL_RLC_DATA_BLOCK : bit (4) > { 1 { 0 < DELAY_NEXT_UL_RLC_DATA_BLOCK : bit (3) > -- delay until start of next UL RLC Data block ¦ 1 -- next UL RLC Data block starts at first possible occasion after previous UL RLC Data block } } ** 0;

Table 11.2.59.2: EC Packet UPlink ACK/NACK AND CONTENTION RESOLUTION information element details

USED_DL_COVERAGE_CLASS (2 bit field) This information element is defined in sub-clause 12.71.

UPLINK_TFI (5 bit field) This field identifies the uplink TBF to which this message applies. This field is coded the same as the TFI field defined in sub-clause 12.15.

CONTENTION_RESOLUTION_TLLI (32 bit field) The CONTENTION_RESOLUTION_TLLI field contains the TLLI that the network has received in uplink RLC data block(s). The mobile station shall perform the contention resolution function using the TLLI information in this field. This coding of the field is defined in sub-clause 12.16.

EC Primary Ack/Nack Description IE This information element is defined in sub-clause 12.73.

RESEGMENT (1 bit field) This field is defined in sub-clause 12.10e.

START_FIRST_UL_RLC_DATA_BLOCK (4 bit field) This field indicates the starting position of the resources for the first allocated RLC Data block in the fixed uplink allocation. The starting position is relative to the first TDMA frame (N) of the downlink block containing the last blind physical layer transmission of this EC PACKET UPLINK ACK/NACK AND CONTENTION RESOLUTION message, according to the USED_DL_COVERAGE_CLASS field. The encoding of the field is dependent of the uplink Coverage Class and TIMESLOT_MULTIPLICATOR assigned for the uplink EC TBF. If uplink Coverage Class = CC1 and TIMESLOT_MULTIPLICATOR = 0 or if uplink Coverage Class = CC2, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the TDMA frame (N+13) mod 2715648 0 0 0 1 In the TDMA frame (N+17 or N+18) mod 2715648 0 0 1 0 In the TDMA frame (N+21 or N+22) mod 2715648 0 0 1 1 In the TDMA frame (N+26) mod 2715648 0 1 0 0 In the TDMA frame (N+30 or N+31) mod 2715648 …. 1 1 1 0 In the TDMA frame (N+73 or N+74) mod 2715648 1 1 1 1 In the TDMA frame (N+78) mod 2715648 If uplink Coverage Class = CC1 and TIMESLOT_MULTIPLICATOR = 1, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 0 1 1 In the timeslot with the highest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the lowest TN in TDMA frame (N+43 or N+44) mod 2715648 1 1 1 1 In the timeslot with the highest TN in TDMA frame (N+43 or N+44) mod 2715648 If uplink Coverage Class = CC1 and TIMESLOT_MULTIPLICATOR = 2, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 1 0 0 In the timeslot with the 2nd lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 1 0 1 In the timeslot with the highest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the highest TN in TDMA frame (N+30 or N+31) mod 2715648 1 1 1 1 In the timeslot with the lowest TN in TDMA frame (N+34 or N+35) mod 2715648 If uplink Coverage Class = CC1 and TIMESLOT_MULTIPLICATOR = 3, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 0 1 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+26) mod 2715648 1 1 1 1 In the timeslot with the highest TN in TDMA frame (N+26) mod 2715648

If uplink_Coverage_Class = CC1 and TIMESLOT_MULTIPLICATOR = 4, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the highest TN in TDMA frame (N+21 or N+22) mod 2715648 1 1 1 1 In the timeslot with the lowest TN in TDMA frame (N+26) mod 2715648 If uplink_Coverage_Class = CC1 and TIMESLOT_MULTIPLICATOR = 5, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the 5th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 1 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+21 or N+22) mod 2715648 1 1 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+21 or N+22) mod 2715648 If uplink_Coverage_Class = CC1 and TIMESLOT_MULTIPLICATOR = 6, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the 5th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the 6th lowest TN in TDMA frame (N+13) mod 2715648 0 1 1 0 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 0 1 1 1 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the lowest TN in TDMA frame (N+21 or N+22) mod 2715648 1 1 1 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+21 or N+22) mod 2715648

If uplink_Coverage_Class = CC1 and TIMESLOT_MULTIPLICATOR = 7, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+13) mod 2715648 0 0 0 1 In the timeslot with the 2nd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 0 In the timeslot with the 3rd lowest TN in TDMA frame (N+13) mod 2715648 0 0 1 1 In the timeslot with the 4th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 0 In the timeslot with the 5th lowest TN in TDMA frame (N+13) mod 2715648 0 1 0 1 In the timeslot with the 6th lowest TN in TDMA frame (N+13) mod 2715648 0 1 1 0 In the timeslot with the 7th lowest TN in TDMA frame (N+13) mod 2715648 0 1 1 1 In the timeslot with the highest TN in TDMA frame (N+13) mod 2715648 1 0 0 0 In the timeslot with the lowest TN in TDMA frame (N+17 or N+18) mod 2715648 …. 1 1 1 0 In the timeslot with the 7th lowest TN in TDMA frame (N+17 or N+18) mod 2715648 1 1 1 1 In the timeslot with the highest TN in TDMA frame (N+17 or N+18) mod 2715648 If uplink Coverage Class = CC3, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the TDMA frame (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 0 0 0 1 In the TDMA frame (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 0 0 1 0 In the TDMA frame (N+30 or N+31) mod 2715648 or (N+34 or N+35) mod 2715648 0 0 1 1 In the TDMA frame (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 0 1 0 0 In the TDMA frame (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 …. 1 1 1 0 In the TDMA frame (N+134 or N+135) mod 2715648 or (N+138 or N+139) mod 2715648 1 1 1 1 In the TDMA frame (N+143) mod 2715648 or (N+147 or N+148) mod 2715648 If uplink Coverage Class = CC4, the START_FN_FIRST_UL_RLC_DATA_BLOCK field is encoded according to the following table: bit 4 3 2 1 0 0 0 0 In the TDMA frame (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 or (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 0 0 0 1 In the TDMA frame (N+30 or N+31) mod 2715648 or (N+34 or N+35) mod 2715648 or (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 0 0 1 0 In the TDMA frame (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 or (N+56 or N+57) mod 2715648 or (N+60 or N+61) mod 2715648 0 0 1 1 In the TDMA frame (N+65) mod 2715648 or (N+69 or N+70) mod 2715648 or (N+73 or N+74) mod 2715648 or (N+78) mod 2715648 0 1 0 0 In the TDMA frame (N+82 or N+83) mod 2715648 or (N+86 or N+87) mod 2715648 or (N+91) mod 2715648 or (N+95 or N+96) mod 2715648 …. 1 1 1 0 In the TDMA frame (N+255 or N+256) mod 2715648 or (N+260) mod 2715648 or (N+264 or N+265) mod 2715648 or (N+268 or N+269) mod 2715648 1 1 1 1 In the TDMA frame (N+273) mod 2715648 or (N+277 or N+278) mod 2715648 or (N+281 or N+282) mod 2715648 or (N+286) mod 2715648

DELAY_NEXT_UL_RLC_DATA_BLOCK (3 bit field) This field indicates the position of the next allocated RLC Data block in the fixed uplink allocation. The position is calculated as the number of transmission opportunities from the previous uplink allocation, according to the assigned UL_COVERAGE_CLASS and TIMESLOT_MULTIPLICATOR, where the next uplink allocation for the mobile station is placed and where the mobile station thus shall transmit the next UL RLC data block. In the transmission opportunity/ies between the previous uplink allocation and this allocation the mobile station is not allocated any resource, and shall thus not transmit. This field is encoded according to the following table: bit 3 2 1 0 0 0 In the 2nd next transmission opportunity 0 0 1 In the 3rd next transmission opportunity 0 1 0 In the 4th next transmission opportunity …. 1 1 0 In the 8th next transmission opportunity 1 1 1 In the 9th next transmission opportunity

***** Next modification *****

## 12.72	EC Ack/Nack Description

The EC Ack/Nack Description information element contains the RLC parameters used to acknowledge or negatively acknowledge a group of RLC data blocks.

Table 12.72.1: EC Ack/Nack Description information element details

< EC Ack/Nack Description IE > ::= < STARTING_SEQUENCE_NUMBER : bit (5) > < RECEIVED_BLOCK_BITMAP : bit (16) >;

***** Next modification *****

## 12.73	EC Primary Ack/Nack Description

The EC Primary Ack/Nack Description information element contains the RLC parameters used to acknowledge or negatively acknowledge a group of RLC data blocks in an EC Packet Uplink Ack/Nack message, including the TLLI or the rTLLI (for contention resolution), or in an EC Packet Uplink Ack/Nack and Contention Resolution message.

Table 12.73.1: EC Primary Ack/Nack Description information element details

< EC Primary Ack/Nack Description IE > ::= < STARTING_SEQUENCE_NUMBER : bit (5) > < RECEIVED_BLOCK_BITMAP : bit (8) >;

***** Next modification *****

## 12.74	EC Packet Timing Advance

The EC Packet Timing Advance field describes the timing advance value assigned to the mobile station for use during an uplink or downlink EC TBF.

Table 12.74.1: EC Packet Timing Advance information element details

< EC Packet Timing Advance IE > ::= < TIMING_ADVANCE_VALUE : bit (6) >;

Table 12.74.2: EC Packet Timing Advance information element details

TIMING_ADVANCE_VALUE (6 bit field) The Timing Advance value field is encoded the same as the Timing Advance value of the Timing Advance information element defined in 3GPP TS 44.018, with the value range 0 to 63.

***** Next modification *****

## 12.75	EC Channel Quality Report

The EC Channel Quality Report information element is defined in tables 12.75.1 and 12.75.2.

Table 12.75.1: EC Channel Quality Report Information elements

< EC Channel Quality Report IE> ::= { 0 ¦ 1 < GMSK_MEAN_BEP : bit (5) > < GMSK_CV_BEP : bit (3) > } { 0 ¦ 1 < 8PSK_MEAN_BEP : bit (5) > < 8PSK_CV_BEP : bit (3) > }; < C_VALUE : bit (6) >;

Table 12.75.2: EC Channel Quality Report Information Elements details

GMSK_MEAN_BEP (5 bit field) This field contains the mean value of the Bit Error Probability of the channel averaged over all time slots in the TBF for GMSK, refer to 3GPP TS 45.008.

GMSK_CV_BEP (3 bit field) This field contains the variation co-efficient for the Bit Error Probability averaged over all time slots of the TBF for GMSK, refer to 3GPP TS 45.008.

8PSK_MEAN_BEP (5 bit field) This field contains the mean value of the Bit Error Probability of the channel averaged over all time slots in the TBF for 8 PSK, refer to 3GPP TS 45.008.

8PSK_CV_BEP (3 bit field) This field contains the variation co-efficient for the Bit Error Probability averaged over all time slots of the TBF for 8 PSK, refer to 3GPP TS 45.008.

C_VALUE (6 bits) This field contains the value of the C parameter calculated by the mobile station (see 3GPP TS 45.008). This field is encoded as the binary representation of the C value parameter value defined in 3GPP TS 45.008. Range 0 to 63