3GPP TSG GERAN#70	GP-160338

# Nanjing, China, 23 – 27 May, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.005 | CR | 0579 | rev | - | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Single Burst EC-PCH Message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2016-05-22 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | A new EC-PCH message mapped to a single burst of the EC-CCCH/D block is proposed and evaluated in contribution GP-160332 to enable multiplexing of paging messages to two or more mobile stations in the lower coverage class (CC1) with a mobile station in a higher coverage class (CC2 to CC4) in the same EC-CCCH/D blocks used by the higher coverage class. According to performance evaluation, this new message format is expected to improve the capacity of the EC-PCH channel compared to the two burst EC-CCCH/D message. This new message format EC-PCH/S (EC-PCH/Short) is always mapped on a single burst and results in BTS power saving by enabling transmission of reduced power on one of the bursts where no transmission is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Performance requirements for paging messages conveyed over the two burst EC-PCH block are extended to paging messages using the single burst message format EC-PCH/S. Selection of TSC according to TS 45.002 is added for EC-CCCH/D. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Improved multiplexing efficiency and BTS power saving cannot be achieved, if the new EC-PCH message is not supported at MS and BTS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.2.4a, 6.5, performance tables 1aa, 2ai, 2am |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.064 CR 0102 TS 45.001 CR 0087 TS 45.002 CR 0196 TS 45.003 CR 0140 TS 44.018 CR 1032 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | 51.010 CR XXXX |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes in this CR are based on changes to respective clauses in the baseline CR 45.005, submitted in GP-160281, and are highlighted in turquoise color. |  |  |  |  |  |  |  |  |

First Modification

### 6.2.2	Packet-switched channels

i) For packet switched channels, the minimum input signal level for which the reference performance shall be met in case of normal symbol-rate, Basic Transmit-Time-Interval (BTTI) and no Piggy-backed ACK/NACK reporting (PAN) used, is specified in table 1a for GMSK modulated input signals, tables 1b and 1c for 8-PSK modulated input signals (convolutional coding), table 1l for input signals modulated by 8-PSK (turbo coding), tables 1k and 1l for input signals modulated by 16-QAM and 32-QAM respectively, according to the type of channel and the propagation condition. In case higher symbol rate, Basic Transmit-Time-Interval (BTTI) and no Piggy-backed ACK/NACK reporting (PAN) are used, the minimum input signal level for which the reference performance shall be met, is specified in table 1m and 1n for input signals modulated by QPSK or 16-QAM or 32-QAM respectively, according to the type of channel and the propagation condition. For Reduced Transmit-Time-Interval (RTTI) the minimum performance requirements are the same as for Basic Transmit-Time-Interval (BTTI) on a static channel. When Piggy-backed ACK/NACK reporting (PAN) is used, the minimum performance requirements of tables 1o and 1p apply. The minimum input signal level for which the reference performance shall be met for PAN is specified in table 1q and 1r. Tables 1p and 1q apply to BTS for input signals with wide pulse shaping filter in the case of higher symbol rate. The performance requirements for GSM 400 and GSM 700 systems are as for GSM 900, except that the GSM 400 MS speed is doubled from that of GSM 900, e.g. TU50 becomes TU100, and the GSM 700 MS speed is increased by a factor of 1.2, e.g. TU50 becomes TU60. The levels are given for normal BTS for GMSK modulated signals. For QPSK, 8-PSK, 16-QAM and 32-QAM modulated signals, the required levels are given for normal BTS and MS separately. When in EC operation the requirements in table 1z and 1aa (for BTS and MS respectively) for GMSK modulated EC-channels and 1ab and 1ac (for BTS and EC-GSM-IoT MS respectively) for 8-PSK modulated EC-PDTCH channels shall be met only in case of BTTI, no PAN, and in the downlink randomly altered USF in each of the blind physical layer transmissions, assuming one user per timeslot. In case that Overlaid CDMA subchannels are multiplexed on the same time slot in coverage classes CC2, CC3 and CC4, the requirements in table 1ad shall be met, when the model of frequency offsets defined in Annex R is applied.

The levels shall be corrected by the following values:

Table 6.2-4 Correction factor of performance requirements for different equipments

|  | MS, GMSK modulated signals |  |
| --- | --- | --- |
| - | for DCS 1 800 class 1 or class 2 MS | +2/+4 dB** |
| - | for DCS 1 800 class 3 MS | +2 dB |
| - | for GSM 400 small MS, GSM 900 small MS, ER-GSM 900 small MS, GSM 850 small MS and GSM 700 small MS | +2 dB |
| - | for other GSM 400, GSM 900 MS, ER-GSM 900 and GSM 850 MS and GSM 700 MS | 0 dB |
|  | for PCS 1900 class 1 or class 2 MS | +2 dB |
|  | for other PCS 1900 MS | 0 dB |
|  | MS, QPSK, 8-PSK, 16-QAM and 32-QAM modulated signals |  |
| - | for GSM 400, GSM 900, ER-GSM 900, GSM 850 and GSM 700 small MS | 0 dB |
| - | for other GSM 400, GSM 900, ER-GSM 900, GSM 850 and GSM 700 MS | -2 dB |
| - | for DCS 1 800 and PCS 1900 class 1 or class 2 MS | 0 dB |
| - | for other DCS 1 800 and PCS 1900 MS | -2 dB |
|  | BTS |  |
| - | for normal BTS | 0 dB |
| - | for GSM 900, ER-GSM 900, GSM 850, MXM 850 and GSM 700 micro BTS M1 | +7 dB |
| - | for GSM 900, ER-GSM 900, GSM 850, MXM 850 and GSM 700 micro BTS M2 | +12 dB |
| - | for GSM 900, ER-GSM 900, GSM 850, MXM 850 and GSM 700 micro BTS M3 | +17 dB |
| - | for GSM 900, ER-GSM 900, GSM 850, MXM 850 and GSM 700 pico BTS P1 | +16 dB |
| - | for DCS 1 800, PCS 1900 and MXM 1900 micro BTS M1 | +2 dB |
| - | for DCS 1 800, PCS 1900 and MXM 1900 micro BTS M2 | +7 dB |
| - | for DCS 1 800, PCS 1900 and MXM 1900 micro BTS M3 | +12 dB |
| - | for DCS 1 800, PCS 1900 and MXM 1900 pico BTS P1 | +9 dB |
| - | for Wide Area multicarrier BTS | 0 dB |
| - | for Medium Range multicarrier BTS | +6 dB |
| - | for Local Area multicarrier BTS | +14 dB |

**	For DCS 1 800 class 1 and class 2 MS, a correction offset of +2dB shall apply for the reference sensitivity performance as specified in table 1a for the normal conditions defined in Annex D and an offset of +4 dB shall be used to determine all other MS performances.

The reference performance shall be according to Table 6.2-5:

Table 6.2-5 Reference performance for Packet channels

| Logical channel | Requirement |
| --- | --- |
| Packet Data Channels (PDCH) | BLER £ 10% unless otherwise stated |
| Uplink State Flags (USF) | BLER £ 1% |
| Packet Random Access Channels (PRACH) | BLER £ 15% |
| Piggy-backed ACK/NACK report (PAN) | PAN error rate £ 5% |
| Extended Coverage Packet Data Traffic Channels Uplink (EC-PDTCH/U) | BLER £ 50% |
| Extended Coverage Packet Data Traffic Channels Downlink (EC-PDTCH/D) | BLER £ 20% |
| Extended Coverage Random Access Channels (EC-RACH) | BLER £ 20% |
| Extended Coverage Control Channels (EC-CCCH/D1, EC-PACCH and EC-BCCH) | BLER £ 10% |
| Extended Coverage Synchronization Channel (EC-SCH) | BLER £ 10% |
| NOTE1: EC-AGCH, EC-PCH and EC-PCH/S |  |

| - |  |  |
| --- | --- | --- |
| - |  |  |
| - |  |  |

where BLER is the Block Error Rate, referring to all erroneously decoded data blocks including any headers, stealing flags, parity bits as well as any implicit information in the training sequence, or any downlink block discrimination error between EC-PDTCH and EC-PACCH.

For PDCH the BLER refers to RLC blocks, and hence there can be up to two block errors per 20ms radio block for EGPRS MCS7, MCS8, MCS9, UAS-7, UAS-8, UAS-9, UBS-7, UBS-8, DAS-8, DAS-9, DAS-10, DBS-7 and DBS-8, up to three block errors per radio block for UAS-10, UAS-11, UBS-9, UBS-10, DAS-11, DAS-12, DBS-9 and DBS-10, and up to four block errors per radio block for UBS-11, UBS-12, DBS-11 and DBS-12. The BLER refers to the initial transmission of RLC blocks, i.e. the channel decoding without incremental redundancy. For USF, the BLER only refers to the USF value. 
For EC-PDTCH and EC-PACCH channels in downlink, the BLER shall be evaluated with random USF in each of the blind physical layer transmissions.

ii) If BTTI USF mode is used when sending downlink data blocks in RTTI configuration and different modulations are used in the two data blocks sent in a 20 ms block period, the USF will be sent with mixed modulation. In this case, the performance of the mixed modulation USF shall meet the less stringent requirement of the two modulations for static propagation conditions according to:

Table 6.2-6 USF performance in mixed modulation

| GMSK | USF/MCS-1 to 4 | (table 1a) |
| --- | --- | --- |
| 8PSK | USF/MCS5 to 9 | (table 1c) |
| 16QAM (EGPRS2-A) | USF/DAS-8 to 9 | (table 1l) |
| 32QAM (EGPRS2-A) | USF/DAS-10 to 12 | (table 1l) |
| QPSK (EGPRS2-B) | USF/DBS-5 to 6 | (table 1n) |
| 16QAM (EGPRS2-B) | USF/DBS-7 to 9 | (table 1n) |
| 32QAM (EGPRS2-B) | USF/DBS-10 to 12 | (table 1n) |

For PDCH channels, the performance requirements for some modulation and coding schemes and propagation conditions are specified at higher BLER. Where applicable, the BLER value noted in the respective performance requirement table applies.

Second Modification

### 6.2.4a	Extended Coverage control channel and data channel performance for EC-GSM-IoT

For EC-channels in Coverage Classes higher than CC1 (i.e. CC2, CC3 and CC4) blind physical layer transmissions are used (EC-PDTCH, EC-RACH, EC-SCH, EC-PACCH, EC-BCCH, EC-AGCH, EC-PCH and EC-PCH/S).

The minimum input signal level for which the reference performance shall be met is specified in table 1z, 1aa, 1ab, 1ac and 1ad, according to the propagation condition and type of equipment, assuming the input signal fulfils the coherency requirements in subclause 4.6.3, where applicable.

Third Modification

## 6.5	Random access and paging performance at high input levels

a)	MS requirements

Under static propagation conditions with a received input level from 20 dB above the reference sensitivity level or from 20 dB above the input signal level at reference performance (CC1) for EC-GSM-IoT.

-	up to -15 dBm for GSM 400, GSM 700, GSM 850 and GSM900 and

-	up to -23 dBm for DCS1800 and PCS 1 900,

the MS FER shall be less than 0,1% for PCH and MS BLER when in EC operation less than [0,1%] for EC-PCH and EC-PCH/S.

b)	BTS requirements

Under static propagation conditions with a received input level from 20 dB above the reference sensitivity level or from 20 dB above the input signal level at reference performance (CC1) for EC-GSM-IoT.

-	up to -15 dBm for GSM 400, GSM 700, GSM 850 and GSM900 normal BTS and multicarrier BTS excluding multicarrier BTS equipped with multicarrier receiver,

-	up to -16 dBm for GSM400, GSM 850 and GSM900 multicarrier BTS equipped with multicarrier receiver and

-	up to -23 dBm for DCS1800 and PCS 1 900, normal BTS and multicarrier BTS

and a single MS sending an access burst, the BTS FER shall be less than 0,5% for RACH and BTS BLER less than [0,5%] for EC-RACH.

For multicarrier BTS equipped with multicarrier receiver, input levels shall be adjusted by 0 dB, +4 dB or +9 dB for the Wide Area, Medium Range and Local Area classes, respectively.

Fourth Modification

# Table 1aa: Input signal level (for EC-GSM-IoT MS) at reference performance for GMSK modulated signals for different Coverage Classes (CC)

| GSM 900 and GSM 850 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |
| Channel |  |  | Static |  | TU1.2 (no FH) |  | TU1.21) (ideal FH) |  | TU50 (no FH) |  |
| EC-SCH | - | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-BCCH | - | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-PACCH/D | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/4 | CC2 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/8 | CC3 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/16 | CC4 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-CCCH/D2) | CC1 | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-CCCH/D/8 | CC2 | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-CCCH/D/16 | CC3 | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-CCCH/D/32 | CC4 | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-PDTCH/MCS-14) | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/4 | CC2 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/8 | CC3 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/16 | CC4 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |
| channel |  |  | Static |  | TU1.2 (no FH) |  | TU1.21) (ideal FH) |  | TU50 (no FH) |  |
| EC-SCH | - | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-BCCH | - | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-PACCH/D | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/4 | CC2 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/8 | CC3 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/16 | CC4 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-CCCH/D2) | CC1 | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-CCCH/D/8 | CC2 | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-CCCH/D/16 | CC3 | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-CCCH/D/32 | CC4 | dBm |  | [tbd] |  | [tbd] |  | - |  | [tbd] |  |
| EC-PDTCH/MCS-14) | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/4 | CC2 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/8 | CC3 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/16 | CC4 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test NOTE 2: The performance requirements for EC-CCCH/D apply for EC-PCH, EC-PCH/S and EC-AGCH with the TSC selected according to 3GPP TS 45.002 for the wanted signal. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 1a apply for Static and TU50 (no FH) and TU50 (ideal FH) propagation conditions |  |  |  |  |  |  |  |  |  |  |  |  |

Fifth Modification

Table 2ai: Cochannel interference ratio at reference performance (for EC-GSM-IoT MS) for GMSK modulated signals

| GSM 900 and GSM 850 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  |  |  |  |  | Propagation conditions |  |  |
| Channel |  |  |  |  |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-SCH | - |  |  | dB |  |  | [tbd] | - | [tbd] |
| EC-BCCH | - |  |  | dB |  |  | [tbd] | - | [tbd] |
| EC-PACCH/D | CC1 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/4 | CC2 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/8 | CC3 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/16 | CC4 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D2) | CC1 |  |  | dB |  |  | [tbd] | - | [tbd] |
| EC-CCCH/D/8 | CC2 |  |  | dB |  |  | [tbd] | - | [tbd] |
| EC-CCCH/D/16 | CC3 |  |  | dB |  |  | [tbd] | - | [tbd] |
| EC-CCCH/D/32 | CC4 |  |  | dB |  |  | [tbd] | - | [tbd] |
| EC-PDTCH/MCS-14) | CC1 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |  |  |
| Type of |  |  |  |  |  |  | Propagation conditions |  |  |
| Channel |  |  |  |  |  |  | TU1.2 (no FH) | TU1.21) (Ideal FH) | TU50 (no FH) |
| EC-SCH |  | - |  |  | dB |  | [tbd] | - | [tbd] |
| EC-BCCH |  | - |  |  | dB |  | [tbd] | - | [tbd] |
| EC-PACCH/D |  | CC1 |  |  | dB |  | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/4 |  | CC2 |  |  | dB |  | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/8 |  | CC3 |  |  | dB |  | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/16 |  | CC4 |  |  | dB |  | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D2) |  |  | CC1 |  |  | dB | [tbd] | - | [tbd] |
| EC-CCCH/D/8 |  |  | CC2 |  |  | dB | [tbd] | - | [tbd] |
| EC-CCCH/D/16 |  |  | CC3 |  |  | dB | [tbd] | - | [tbd] |
| EC-CCCH/D/32 |  |  | CC4 |  |  | dB | [tbd] | - | [tbd] |
| EC-PDTCH/MCS-14) |  | CC1 |  |  | dB |  | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 |  | CC2 |  |  | dB |  | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 |  | CC3 |  |  | dB |  | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 |  | CC4 |  |  | dB |  | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test NOTE 2: The performance requirements for EC-CCCH/D apply for EC-PCH, EC-PCH/S and EC-AGCH with the TSC selected according to 3GPP TS 45.002 for the wanted signal. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 2a apply for TU3 (low band) and TU1,5 (high band), and, TU50 (no FH) propagation conditions together with the conditions for EGPRS in section 6.3.4. |  |  |  |  |  |  |  |  |  |

Sixth Modification

Table 2am: Adjacent channel interference ratio at reference performance (for EC-GSM-IoT MS) for GMSK modulated signals

| GSM 900 and GSM 850 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21),5) (ideal FH) | TU50 (no FH) |
| EC-SCH | - | dB | [tbd] | - | [tbd] |
| EC-BCCH | - | dB | [tbd] | - | [tbd] |
| EC-PACCH/D | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D2) | CC1 | dB | [tbd] | - | [tbd] |
| EC-CCCH/D/8 | CC2 | dB | [tbd] | - | [tbd] |
| EC-CCCH/D/16 | CC3 | dB | [tbd] | - | [tbd] |
| EC-CCCH/D/32 | CC4 | dB | [tbd] | - | [tbd] |
| EC-PDTCH/MCS-14) | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21),5) (Ideal FH) | TU50 (no FH) |
| EC-SCH | - | dB | [tbd] | - | [tbd] |
| EC-BCCH | - | dB | [tbd] | - | [tbd] |
| EC-PACCH/D | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D2) | CC1 | dB | [tbd] | - | [tbd] |
| EC-CCCH/D/8 | CC2 | dB | [tbd] | - | [tbd] |
| EC-CCCH/D/16 | CC3 | dB | [tbd] | - | [tbd] |
| EC-CCCH/D/32 | CC4 | dB | [tbd] | - | [tbd] |
| EC-PDTCH/MCS-14) | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: The performance requirements for EC-CCCH/D apply for EC-PCH, EC-PCH/S and EC-AGCH with the TSC selected according to 3GPP TS 45.002 for the wanted signal. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 2a apply for TU3 (low band) and TU1,5 (high band) and TU50 (no FH) propagation conditions together with the conditions for EGPRS in table 6.3-1a and subclause 6.3.4. |  |  |  |  |  |

End of Modifications