3GPP TSG-GERAN #70 	GP-160349

# Nanjing, PR China, May 23rd – 27th 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 44.060 | CR | 1627 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EC-GSM-IoT SINR based measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G2 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2016-05-17 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To allow for SINR based measurements when estimating downlink coverage class and received signal levels, see GP-160266 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the possibility to report downlink coverage class and received signal levels using SINR based measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SINR based measurements in packet transfer mode will not be possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS xxx CR xxxx |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS 51.021 CR 0276 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The sub-clause affected is based CR 44.060-1625 which is pending discussion. The track changes from sub-clause 10.4.29 in CR 44.060-1625 is in this CR accepted to allow for the diff to be visible. Hence, the two CRs need to be merged in some way to solve the issue of conflicting changes |  |  |  |  |  |  |  |  |

***** First modification *****

### 10.4.29	Downlink Coverage Class Estimate (DCCE)

In EC operation, a 4-bit field is included in the uplink RLC data block header to signal the estimated downlink Coverage Class, see 3GPP TS 43.064, and, when CC1 is estimated, the extent to which the estimated C value (see 3GPP TS 45.008) exceeds the value indicated by the BT_Threshold_DL parameter sent in EC SI2 (see 3GPP TS 44.018). The C value will be reported in steps of 3 dB above the BT_Threshold_DL where BT_Threshold_DL indicates when blind physical layer transmissions are used. The value Y in Table 10.4.29.1 is defined as:

- Y = C value.

An exception is when the field is included in uplink RLC data blocks allocated in an EC IMMEDIATE ASSIGNMENT TYPE 1 or an EC IMMEDIATE ASSIGNMENT TYPE 2 message, in which case no measurements on EC-PDTCH/EC-PACCH are available. In that case, received signal level based measurements (RLA) or SINR based measurements (SLA) are used instead of the C value, see 3GPP TS 45.008. Whether RLA or SLA shall be used is determined by DL_CC_Selection broadcast in EC SI (see 3GPP TS 44.018). The value Y in Table 10.4.29.1 is then defined as:

- Y = RLA if DL_CC_Selection=0;

- Y = SLA if DL_CC_Selection=1.

Table 10.4.29.1: DCCE field

| bits 4 3 2 1 | DCCE | bits 4 3 2 1 | DCCE |
| --- | --- | --- | --- |
| 0 0 0 0 | DL CC4 | 1 0 0 0 | DL CC1; BT_Threshold_DL+15 dB ≤ Y < BT_Threshold_DL+18 dB |
| 0 0 0 1 | DL CC3 | 1 0 0 1 | DL CC1; BT_Threshold_DL+18 dB ≤ Y < BT_Threshold_DL+21 dB |
| 0 0 1 0 | DL CC2 | 1 0 1 0 | DL CC1; BT_Threshold_DL+21 dB ≤ Y < BT_Threshold_DL+24 dB |
| 0 0 1 1 | DL CC1; BT_Threshold_DL ≤ Y < BT_Threshold_DL+3 dB | 1 0 1 1 | DL CC1; BT_Threshold_DL+24 dB ≤ Y < BT_Threshold_DL+27 dB |
| 0 1 0 0 | DL CC1; BT_Threshold_DL+3 dB ≤ Y < BT_Threshold_DL+6 dB | 1 1 0 0 | DL CC1; BT_Threshold_DL+27 dB ≤ Y < BT_Threshold_DL+30 dB |
| 0 1 0 1 | DL CC1; BT_Threshold_DL+6 dB ≤ Y < BT_Threshold_DL+9 dB | 1 1 0 1 | DL CC1; BT_Threshold_DL+30 dB ≤ Y < BT_Threshold_DL+33 dB |
| 0 1 1 0 | DL CC1; BT_Threshold_DL+9 dB ≤ Y < BT_Threshold_DL+12 dB | 1 1 1 0 | DL CC1; BT_Threshold_DL+33 dB ≤ Y < BT_Threshold_DL+36 dB |
| 0 1 1 1 | DL CC1; BT_Threshold_DL+12 dB ≤ Y < BT_Threshold_DL+15 dB | 1 1 1 1 | DL CC1; BT_Threshold_DL+36 dB ≤ Y < BT_Threshold_DL+39 dB |
| NOTE: The bit numbering is relative to the field position. |  |  |  |