3GPP TSG-GERAN #70 	GP-160469

# Nanjing, PR China, May 23rd – 27th 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.005 | CR | 0581 | rev | - | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BTS sensitivity performance requirements for EC-GSM-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2016-05-24 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Performance requirements for the BTS when supporting EC-GSM-IoT is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Sensitivity performance requirements for the BTS when supporting EC-GSM-IoT is added, excluding 2TS EC-RACH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No performance requirements will exist for the BTS when supporting EC-GSM-IoT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 1z, Table 1ab |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.022 CR 0037 TS 43.059 CR 0080 TS 43.064 CR 0104 TS 44.004 CR 0019 TS 44.014 CR 0024 TS 44.018 CR 1035 TS 44.018 CR 1036 TS 44.060 CR 1624 TS 44.060 CR 1625 TS 45.001 CR 0084 TS 45.002 CR 0192 TS 45.003 CR 0138 TS 45.004 CR 0024 TS 45.005 CR 0578 TS 45.008 CR 0635 TS 45.008 CR 0636 TS 45.010 CR 0069 TS 48.018 CR 0425 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.021 CR 0276 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modification

# Table 1z: Input signal level (for normal BTS) at reference performance for GMSK modulated signals for different Coverage Classes (CC)

| GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dBm | [-117] | [-115] | [-114] | [-114] |
| EC-PACCH/U/4 | CC2 | dBm | [-123] | [-121] | [-120] | [-120] |
| EC-PACCH/U/8 | CC3 | dBm | [-126] | [-124] | [-123] | [-123] |
| EC-PACCH/U/16 | CC4 | dBm | [-129] | [-127] | [-126] | [-126] |
| EC-PDTCH/MCS-14) | CC1 | dBm | [-116] | [-114,5] | [-114] | [-114] |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | [-122] | [-120,5] | [-120] | [-120] |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | [-125] | [-123,5] | [-123] | [-123] |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | [-127,5] | [-126,5] | [-126] | [-126] |
| 1 TS EC-RACH2) | CC1 | dBm | [-116] | [-114] | - | [-113] |
| 1 TS EC-RACH/4 | CC2 | dBm | [-119] | [-117] | - | [-116] |
| 1 TS EC-RACH/16 | CC3 | dBm | [-124,5] | [-122,5] | - | [-122] |
| 1 TS EC-RACH/48 | CC4 | dBm | [-127] | [-125] | - | [-126] |
| 2 TS EC-RACH/4 | CC2 | dBm | [tbd] | [tbd] | - | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dBm | [tbd] | [tbd] | - | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dBm | [tbd] | [tbd] | - | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dBm | [-116] | [-114,5] | [-113,5] | [-113,5] |
| EC-PACCH/U/4 | CC2 | dBm | [-122] | [-120,5] | [-119,5] | [-119,5] |
| EC-PACCH/U/8 | CC3 | dBm | [-125] | [-123,5] | [-122,5] | [-122,5] |
| EC-PACCH/U/16 | CC4 | dBm | [-128] | [-126,5] | [-125,5] | [-125,5] |
| EC-PDTCH/MCS-14) | CC1 | dBm | [-116] | [-114,5] | [-113,5] | [-113,5] |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | [-122] | [-120,5] | [-119,5] | [-119,5] |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | [-125] | [-123,5] | [-122,5] | [-122,5] |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | [-127,5] | [-126,5] | [-125,5] | [-125,5] |
| 1 TS EC-RACH2) | CC1 | dBm | [-115] | [-113,5] | - | [-112,5] |
| 1 TS EC-RACH/4 | CC2 | dBm | [-118] | [-116,5] | - | [-115,5] |
| 1 TS EC-RACH/16 | CC3 | dBm | [-124] | [-122] | - | [-121,5] |
| 1 TS EC-RACH/48 | CC4 | dBm | [-127] | [-125] | - | [-125,5] |
| 2 TS EC-RACH/4 | CC2 | dBm | [tbd] | [tbd] | - | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dBm | [tbd] | [tbd] | - | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dBm | [tbd] | [tbd] | - | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 1a apply for Static and TU50 (no FH) and TU50 (ideal FH) propagation conditions |  |  |  |  |  |  |

2nd modification

# Table 1ab: Input signal level (for normal BTS) at reference performance for 8-PSK modulated signals for Coverage Class 1 (CC1)

| GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PDTCH/MCS-52) | CC1 | dBm | [-104] | [-101,0] | [-100,5] | [-100,5] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PDTCH/MCS-52) | CC1 | dBm | [-104] | [-100,5] | [-99,5] | [-100,5] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: For MCS-6, MCS-7, MCS-8 and MCS-9 in CC1 the requirements in table 1b apply for Static and TU50 (no FH) and TU50 (ideal FH) propagation conditions |  |  |  |  |  |  |

End of modifications