3GPP TSG-RAN5 Meeting #99	R5-233103

# Incheon , Korea, 22nd – 26th May. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2298 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating MPR for PC2 configuration CA_n78C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_PC2_CA_R17_2BDL_2BUL-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 6.2A.2 MPR for CA need to be updated for CA_n78C for power class 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing CA_n78C PC2 requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PC2 test requirement for CA_n78C is absent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2A.2.1	UE maximum output power reduction for CA (2UL CA)

<Unchanged Skipped>

6.2A.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4.

Table 6.2A.2.1.4.3-1: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA Test IDs 1, 4-13, 16, 19, 23, 26, 27, 30, 37 |
|  | 17 |  | Power class 3 and Inter-band CA Test IDs 14, 15, 17, 18, 33 |
|  | 16 |  | Power class 3 and Inter-band CA Test IDs 2, 3, 22, 24, 25, 28, 29, 31, 32 |
|  | 14 |  | Power class 3 and Inter-band CA Test IDs 20, 21, 36 |
|  | 13 |  | Power class 3 and Inter-band CA Test IDs 34, 35 |

Table 6.2A.2.1.4.3-2: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n41C, CA_n78C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 18 |  | Power class 3 and Test IDs 1, 3 ,5 |
|  | 16 |  | Power class 3 and Test IDs 11, 13 |
|  | 14 |  | Power class 3 and Test IDs 7, 15 |
|  | 11 |  | Power class 3 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 10 |  | Power class 3 and Test IDs 10, 12, 14, 16, 18 |
|  | 20 |  | Power class 2 and Test IDs 1, 3, 5 |
|  | 19 |  | Power class 2 and Test IDs 11, 13 |
|  | 18 |  | Power class 2 and Test IDs 7, 15 |
|  | 15 |  | Power class 2 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 14 |  | Power class 2 and Test IDs 10, 12, 14, 16, 18 |

<Unchanged Skipped>

Table 6.2A.2.1.5-4: UE Output Power for intra-band contiguous 2 UL CA of PC2 (contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 2 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 26 | 2.0 | 2.5 | 0 | 24 | 23.5 | 3 | 3 | 3 | 28+TT | 21.-TT | 28+TT | 20.5-TT |
| 2 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 3 | 26 | 2.0 | 2.5 | 0 | 24 | 23.5 | 3 | 3 | 3 | 28+TT | 21.-TT | 28+TT | 20.5-TT |
| 4 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 5 | 26 | 2.5 | 2.5 | 0 | 23.5 | 23.5 | 3 | 3 | 3 | 28+TT | 20.5-TT | 28+TT | 20.5-TT |
| 6 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 7 | 26 | 3.0 | 5.0 | 0 | 23 | 21.0 | 3 | 2 | 2 | 25+TT | 20.-TT | 25+TT | 18.-TT |
| 8 | 26 | 4.5 | 7.0 | 0 | 21.5 | 19.0 | 3 | 2 | 3.5 | 25+TT | 18.5-TT | 25+TT | 15.5-TT |
| 9 | 26 | 5.5 | 7.0 | 0 | 20.5 | 19.0 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 15.5-TT |
| 10 | 26 | 6.0 | 7.5 | 0 | 20 | 18.5 | 3 | 2.5 | 4 | 25+TT | 17.-TT | 25+TT | 14.5-TT |
| 11 | 26 | 2.5 | 3.5 | 0 | 23.5 | 22.5 | 3 | 3 | 2 | 25+TT | 20.5-TT | 25+TT | 19.5-TT |
| 12 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 13 | 26 | 3.0 | 3.5 | 0 | 23 | 22.5 | 3 | 2 | 2 | 25+TT | 20.-TT | 25+TT | 19.5-TT |
| 14 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 15 | 26 | 3.5 | 5.0 | 0 | 22.5 | 21.0 | 3 | 2 | 2 | 25+TT | 19.5-TT | 25+TT | 18.-TT |
| 16 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 17 | 26 | 6.5 | 7.0 | 0 | 19.5 | 19.0 | 3 | 3.5 | 3.5 | 25+TT | 16.-TT | 25+TT | 15.5-TT |
| 18 | 26 | 6.5 | 8.0 | 0 | 19.5 | 18.0 | 3 | 3.5 | 4 | 25+TT | 16.-TT | 25+TT | 14.-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-5: UE Output Power for intra-band contiguous 2 UL CA of PC2 with TxD (contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

FFS

Table 6.2A.2.1.5-6: UE Output Power for intra-band contiguous 2 UL CA of PC2 (non-contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 2 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 26 | 3.0 | 3 | 0 | 23.0 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 2 | 26 | 6.5 (4.5)1 | 7.5 (5.5)1 | 0 | 19.5 (21.5)1 | 18.5 (20.5)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 16.-TT (18.5-TT)1 | 28+TT | 14.5-TT (17.5-TT)1 |
| 3 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 4 | 26 | 3.0 | 3.0 | 0 | 23 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 5 | 26 | 6.5 (4.5)1 | 7.5 (5.5)1 | 0 | 19.5 (21.5)1 | 18.5 (20.5)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 16.-TT (18.5-TT)1 | 28+TT | 14.5-TT (17.5-TT)1 |
| 6 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 7 | 26 | 3.0 | 3.0 | 0 | 23 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 8 | 26 | 6.5 | 7.5 | 0 | 19.5 | 18.5 | 3 | 3.5 | 4 | 28+TT | 16.-TT | 28+TT | 14.5-TT |
| 9 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 10 | 26 | 5.0 | 5.0 | 0 | 21 | 21.0 | 3 | 2 | 2 | 28+TT | 18.-TT | 28+TT | 18.-TT |
| 11 | 26 | 6.5 | 7.5 | 0 | 19.5 | 18.5 | 3 | 3.5 | 4 | 28+TT | 16.-TT | 28+TT | 14.5-TT |
| 12 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 13 | 26 | 6.5 | 6.5 | 0 | 19.5 | 19.5 | 3 | 3.5 | 3.5 | 28+TT | 16.-TT | 28+TT | 16.-TT |
| 14 | 26 | 7.0 | 7.5 | 0 | 19 | 18.5 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.5-TT |
| 15 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 16 | 26 | 3.5 | 3.5 | 0 | 22.5 | 22.5 | 3 | 2 | 2 | 28+TT | 19.5-TT | 28+TT | 19.5-TT |
| 17 | 26 | 7.0 (5.0)1 | 8.0 (6.0)1 | 0 | 19 (21.0)1 | 18.0 (20.0)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 15.5-TT (18.0-TT)1 | 28+TT | 14.-TT (17.0-TT)1 |
| 18 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 19 | 26 | 3.5 | 3.5 | 0 | 22.5 | 22.5 | 3 | 2 | 2 | 28+TT | 19.5-TT | 28+TT | 19.5-TT |
| 20 | 26 | 7.0 | 8.0 | 0 | 19 | 18.0 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.-TT |
| 21 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 22 | 26 | 5.0 | 5.0 | 0 | 21 | 21.0 | 3 | 2 | 2 | 28+TT | 18.-TT | 28+TT | 18.-TT |
| 23 | 26 | 7.0 | 8.0 | 0 | 19 | 18.0 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.-TT |
| 24 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 25 | 26 | 7.5 | 7.5 | 0 | 18.5 | 18.5 | 3 | 4 | 4 | 28+TT | 14.5-TT | 28+TT | 14.5-TT |
| 26 | 26 | 7.5 | 8.0 | 0 | 18.5 | 18.0 | 3 | 4 | 4 | 28+TT | 14.5-TT | 28+TT | 14.-TT |
| 27 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-7: UE Output Power for intra-band contiguous 2 UL CA of PC2 with TxD (non-contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

FFS

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

6.2A.3	UE additional maximum output power reduction for CA

<End of Changes>