3GPP TSG-RAN5 Meeting #99	R5-232350

# Incheon, Korea, 22th May 2023 – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2758 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RF message exceptions for K1 and number of HARQ processes in CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Anritsu Limited, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For FR1 CA RF test cases, new Table A.3.1-2 and Table A.3.1-3 defined in 38.521-1, define the specific number of HARQ process and the specific values of K1 (dl-DataToUL-ACK) to use in every carrier depending on the CA combination. However, the corresponding messages exceptions required are not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In specific section for RF test cases, defined the message exception for the number of HARQ process and the values of K1 (dl-DataToUL-ACK) to be according to specific RF requirements in case of FR1 CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on RAN4 R4-2307035 and corresponding mirror CRs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 5.4.1	Radio resource control information elements

As defined in clause 4.6.3 with the following exceptions:

## <<< Skip unchanged sections >>>

PDCCH Configuration

PDCCH-config

Table 5.4.1-16: PDCCH ControlResourceSet

| Derivation Path: Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP | RFACS AND AL8 |
|  | 11111111 11111111 00000000 00000000 00000000 00000 | CORESET to use the least significant 96 RBs of the BWP | RFACS AND AL16 |

| Condition | Explanation |
| --- | --- |
| RFACS | Used in RF Adjacent Channel Selectivity testing |
| AL8 | PDCCH aggregationLevel 8 is configured |
| AL16 | PDCCH aggregationLevel 16 is configured |

Table 5.4.1-17: PDCCH Search Space

| Derivation Path: Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n2 |  | RFACS |
| aggregationLevel8 | n2 |  | RFACS |
| aggregationLevel16 | n2 |  | RFACS |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RFACS | Used in RF Adjacent Channel Selectivity testing |

PDSCH-ServingCellConfig

Table 5.4.1-18: PDSCH-ServingCellConfig

| Derivation Path: Table 4.6.3-102 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | Set according to definition in 38.521-1 [14], Table A.3.1-3 |  | FR1 AND CA |
| } |  |  |  |

PUCCH-Config

Table 5.4.1-19: PUCCH-Config

| Derivation Path: Table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 8 entries |  | FR1 AND CA |
| INTEGER[1] | 2 | entry 1 |  |
| INTEGER[2] | 3 | entry 2 |  |
| INTEGER[3] | 4 | entry 3 |  |
| INTEGER[4] | 5 | entry 4 |  |
| INTEGER[5] | 6 | entry 5 |  |
| INTEGER[6] | 7 | entry 6 |  |
| INTEGER[7] | 8 | entry 7 |  |
| INTEGER[8] | 10 | entry 8 |  |
| } |  |  |  |
| } |  |  |  |

## <<< END OF CHANGES >>>