3GPP TSG-RAN5 Meeting #99 	R5-232996

# Incheon, South Korea, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2279 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Annex D.2 for interference signals lower than 2700 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Table D.2-4 in TS 38.101-1 is missing in TS 38.521-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table D.2-4 was added according to TS 38.101-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-band contiguous CA will not be tested correctly for frequencies lower than 2700MHz. Editorial corrections to font style |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

D.2	Interference signals

Table D.2-1 and Table D.2-4 describes the modulated interferer for different channel bandwidth options for NR band lower than 2700MHz.

Table D.2-1: Description of modulated NR interferer for NR bands with FDL_high< 2700 MHz and FUL_high< 2700 MHz

|  | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz |
| RB | NOTE1 |  |  |  |  |  |  |
| BWInterferer | 5 MHz |  |  |  |  |  |  |
|  | Channel bandwidth |  |  |  |  |  |  |
|  | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| RB | NOTE1 |  |  |  |  |  |  |
| BWInterferer | 5 MHz |  |  |  |  |  |  |
| NOTE 1: The RB configured for interfering signal is the same as maximum RB number defined in Table 5.3.2-1 for each sub-carrier spacing. |  |  |  |  |  |  |  |

Table D.2-2 and Table D.2-3 describe the modulated interferer for different channel bandwidth options for NR band higher than 3300MHz.

Table D.2-2: Description of modulated NR interferer for NR bands with FDL_low≥ 3300 MHz and FUL_low≥ 3300 MHz

|  | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| RB | NOTE1 |  |  |  |  |  |  |  |  |  |  |
| BWInterferer | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| NOTE 1: The RB configured for interfering signal is the same as maximum RB number defined in Table 5.3.2-1 for each sub-carrier spacing. |  |  |  |  |  |  |  |  |  |  |  |

Table D.2-3: Description of modulated NR interferer for NR bands with FDL_low≥ 3300 MHz and FUL_low≥ 3300 MHz for Intra-band contiguous CA

|  | Aggregated Channel bandwidth of Bandwidth Class C |  |  |  |  |  |  |  | Bandwidth Class D/E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 110 MHz | 120 MHz | 130 MHz | 140 MHz | 150 MHz | 160 MHz | 180 MHz | 200 MHz |  |
| RB(SCS=30 kHz) | Note 1 |  |  |  |  |  |  |  | 133 |
| RB(SCS=60 kHz) | Note 1 |  |  |  |  |  |  |  | 65 |
| BWInterferer | 110 MHz | 120 MHz | 130 MHz | 140 MHz | 150 MHz | 160 MHz | 180 MHz | 200 MHz | 50MHz |
| NOTE 1: The interfering signal shall be configured in the same way as the aggregated bandwidth of the wanted signal. The RB configurations for each component carrier are defined in Table 5.3.2-1 for each sub-carrier spacing. |  |  |  |  |  |  |  |  |  |

Table D.2-4: Description of modulated NR interferer for NR bands with FDL_low < 2700 MHz and FUL_low < 2700 MHz for Intra-band contiguous CA

|  | Bandwidth Class B | Bandwidth Class C |
| --- | --- | --- |
| RB | NOTE 1 | NOTE 1 |
| BWInterferer | 5 MHz | 5 MHz |
| NOTE 1: The RB configured for interfering signal is the same as maximum RB number defined in Table 5.3.2-1 for each sub-carrier spacing. |  |  |

<<End of change>>