3GPP TSG RAN #100	                                         	      RP-231040

Taipei, June 12-14, 2023

Agenda Item:	9.3.2.9

Source:	Samsung

Title:	On PDCCH monitoring resumption after UL NACK

Document for:	Discussion/Decision

# Introduction

This contribution provides a brief overview of discussions on PDCCH monitoring resumption after UL NACK in conjunction with PDCCH skipping and proposes conclusions.

# Discussion on PDCCH monitoring resumption after UL NACK

PDCCH monitoring resumption after UL NACK was discussed in RAN#99 with the following conclusion.

It is understood that this functionality is rather minor. Interested companies are encouraged to bring in CR proposals to RAN1 to show the impacts of adding this functionality. RAN1 chair confirmed to handle these proposals in the XR session, and RAN1 is expected to make a decision on these proposals as per normal process.

RAN1 considered the topic in RAN1#112bis and in RAN1#113 without conclusion. Two text proposals (TPs) were provided in RAN1#113 [1, 2] and a modified TP (below) was subsequently discussed online. The modified TP deviates from the TP in [1] in the addition of “threshold [PdcchSkippingThreshold]”. This additional feature was justified based on results from a single company showing increased UE power consumption from supporting PDCCH monitoring resumption after NACK when the skipping duration is small and, therefore, proposes to condition the UE behavior for resuming PDCCH monitoring after NACK on the PDCCH skipping duration being larger than a configured threshold. However, without going into specific analysis of UE power savings depending on the PDCCH skipping duration and possible PDCCH monitoring resumption after UL NACK, it is noted that the feature is conditioned on gNB configuration (“When the UE is configured with [PdcchMornitoringResumptionAfterNack]”). Consequently, a gNB that configured small PDCCH skipping duration values to a UE can always choose to not configure PDCCH monitoring resumption after UL NACK for the UE (and the same largely applies for a mixture of small and large values although that would be atypical). Therefore, introducing the additional threshold condition for the feature would only be an over-optimization, if any at all, and make the feature of PDCCH monitoring resumption after NACK more complex for a UE to support.

Without considering the “threshold [PdcchSkippingThreshold]” modifications, the TP also introduces an artificial restriction for when a UE resumes PDCCH monitoring after an UL NACK since that is conditioned on a PDSCH reception being scheduled by a DCI format (i.e. “due to incorrectly decoding a PDSCH scheduled by a DCI format received from the serving cell”) and SPS PDSCHs are therefore excluded. It is noted that “scheduled” (for DG-PDSCH) is not same as “activated” (for SPS PDSCH). If the feature is to be supported, any potential benefits should not be limited, for no apparent reason, by how the UE was informed to receive a PDSCH.

10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR before the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE shall monitor PDCCH regardless of PDCCH skipping indication on all serving cells of the corresponding Cell Group when the SR is pending [11, TS 38.321]. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group. When the UE is configured with [PdcchMornitoringResumptionAfterNack] and the skipping duration threshold [PdcchSkippingTheshold], after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration larger than the configured threshold [PdcchSkippingThreshold] on the active DL BWP of a serving cell, if the UE transmits a PUCCH or a PUSCH providing a NACK value due to incorrectly decoding a PDSCH scheduled by a DCI format received from the serving cell, the UE terminates PDCCH skipping, starting from the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH transmission in the serving cell. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a SpCell, when contention resolution is successful [11, TS 38.321], the UE resumes PDCCH monitoring on the SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a serving cell, when a pending SR is cancelled [11, TS 38.321], the UE resumes PDCCH monitoring in all serving cells of the corresponding Cell Group. If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell. *** Unchanged text is omitted ***

Considering the above, and that substantial time has already been allocated on discussions for PDCCH monitoring resumptions after UL NACK, despite the fact that the feature is not essential and is not different than any other feature that was not agreed in WGs and was not further discussed, we propose one of the following as a conclusion.

Proposal: For PDCCH monitoring resumption after UL NACK in conjunction with PDCCH skipping, conclude on one of the following.

- No further discussion on PDCCH skipping suspension (PDCCH monitoring resumption) after UL NACK in Rel-18.

- Endorse the above TP from RAN1#113 by removing the text related to “threshold [PdcchSkippingThreshold]” and adding “activated” to “due to incorrectly decoding a PDSCH scheduled/activated by a DCI format received from the serving cell”.

# Conclusion

This contribution considered the feature of PDCCH monitoring resumption after UL NACK in conjunction with PDCCH skipping and proposes the following.

Proposal: For PDCCH monitoring resumption after UL NACK in conjunction with PDCCH skipping, conclude on one of the following.

- No further discussion on PDCCH skipping suspension (PDCCH monitoring resumption) after UL NACK in Rel-18.

- Endorse the above TP from RAN1#113 by removing the text related to “threshold [PdcchSkippingThreshold]” and adding “activated” to “due to incorrectly decoding a PDSCH scheduled/activated by a DCI format received from the serving cell”.

# References

[1] R1-2304494	Draft CR on PDCCH monitoring resumption after NACK	vivo, MediaTek, Ericsson, Xiaomi, ZTE, Sanechips, China Telecom, China Unicom, Qualcomm, LGE, Huawei, HiSi, Google, Meta, Apple

[2] R1-2305864	Draft CR for Introducing PDCCH monitoring resumption after UL NACK	Nokia