3GPP TSG RAN Meeting #100			RP-231045

Taipei, Taiwan, June 12th – 14th, 2023

Agenda item:	9.2.2

Source:	KT Corp.

Title:	Discussion on latency requirements for Ambient IoT use cases

Document for:	Discussion

# Introduction

Study on Ambient IoT was approved in RAN#97e [1]. In the last plenary meeting, it was agreed to add Latency in RAN design targets for Ambient IoT TR 38.848 [2]. This contribution discuss how to capture latency requirements from KT’s contribution in RAN#99 [3] according to the use case mapping agreed as [4]

# Latency requirements for representative use cases

In the last plenary, representative use cases (rUCs) are agreed as below:

- rUC1: Indoor inventory

- rUC2: Indoor sensors

- rUC3: Indoor positioning

- rUC4: Indoor command

- rUC5: Outdoor inventory

- rUC6: Outdoor sensors

- rUC7: Outdoor positioning

- rUC8: Outdoor command

SA1 use cases had been mapped into 8 representative use cases as [4] and max. allowed end-to-end latency can be mapped as Table 2.1. It can be noticed from the SA1 results that the latency requirements for Ambient IoT is not too stringent as 5G NR eMBB or URLLC use cases and by capturing the shortest allowed end-to-end latency for representative use cases this can be summarized as below:

- rUC1: Indoor inventory – 100ms

- rUC2: Indoor sensors – 1s

- rUC3: Indoor positioning – 0.5s

- rUC4: Indoor command – hundreds ms level

- rUC5: Outdoor inventory – hundreds ms level

- rUC6: Outdoor sensors – 1s

- rUC7: Outdoor positioning – 1s

- rUC8: Outdoor command – hundreds ms level

Observation 1: For 8 representative use cases, max. allowed end-to-end latency is not too stringent as 5G NR eMBB or URLLC use cases

Table 2.1: Mapping between RAN representative use cases and SA1 use cases with max. allowed end-to-end latency

| rUC | Applicable SA1 UCs / traffic scenarios | Max. allowed end-to-end latency |
| --- | --- | --- |
| rUC1: Indoor inventory | 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction 6.3 Electronic shelf label | 1s Hundreds ms level 1s >100ms >1sec >10s >10s >1minute NA [<10s] 1s |
| rUC2: Indoor sensor | 5.6 Smart homes 5.13 Base station machine room environmental supervision 5.15 Smart laundry 5.20 Smart agriculture 5.23 Smart pig farm 6.2 Cow stable | 20s 1s >10s >1s >10s 1s |
| rUC3: Indoor positioning | 5.8 Finding Remote Lost Item 5.9 Location service 5.10 Ranging in a home 5.12 Personal belongings finding 5.14 Positioning in shopping centre 5.21 Museum Guide | >5s NA NA 1s 0.5s [2]s |
| rUC4: Indoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.29 Device Permanent Deactivation 6.3 Electronic shelf label | Hundreds ms level NA NA NA 1s |
| rUC5: Outdoor inventory | 5.2 Medical instruments inventory management and positioning 5.4 Non-public network for logistics 5.7 Airport terminal / shipping port 5.16 Automated supply chain distribution | Hundreds ms level NA >1sec >10s |
| rUC6: Outdoor sensor | 5.3 Smart grids 5.19 Forest Fire Monitoring 5.22 Dairy farming 5.24 Smart manhole cover safety monitoring 5.25 Smart bridge health monitoring | 1s >10sec >1s 10-30s 10s |
| rUC7: Outdoor positioning | 5.8 Finding remote lost item 5.9 Location service 5.12 Personal belongings finding | >5s NA 1s |
| rUC8: Outdoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.30 Controller in smart agriculture | Hundreds ms level NA NA Hundreds ms level |

Observation 2: From the 8 representative use cases in TR 38.848, end-to-end latency requirements of 100ms can support all the use cases in TR 38.848

From the above observations, KT proposes to capture Ambient IoT latency requirements to be less than 100ms which will be able to support all the use cases described in TR 38.848.

Proposal 1: Target design for ambient IoT maximum allowed end-to-end latency shall be less than 100ms

Proposal 2: Proposal 1 to be captured in 5.6 Latency sub-clause

# Conclusion

Based on the observations found in the above section, the following proposals are made as below:

Observation 1: For 8 representative use cases, max. allowed end-to-end latency is not too stringent as 5G NR eMBB or URLLC use cases

Observation 2: From the 8 representative use cases in TR 38.848, end-to-end latency requirements of 100ms can support all the use cases in TR 38.848

Proposal 1: Target design for ambient IoT maximum allowed end-to-end latency shall be less than 100ms

Proposal 2: Proposal 1 to be captured in 5.6 Latency sub-clause

# Reference

- RP-222685, New SID: Study on Ambient IoT, RAN#97e

- RP-230808, pCR for TR 38.848 “Study on Ambient IoT (Internet of Things) in RAN” – RAN#99 decisions, RAN#99

- RP-230575, Considerations for Ambient IoT latency requirements, RAN#99

- RP-230803, pCR for TR 38.848 “Study on Ambient IoT (Internet of Things) in RAN” – Use case mapping, RAN#99