3GPP TSG RAN Meeting #100	RP-231065

Taipei, June 12-14, 2023

Status Report to TSG

Agenda item:			9.3.4.10

| WI / SI Name | # Support of intra-band non-collocated EN-DC/NR-CA deployment |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NonCol_intraB_ENDC_NR_CA |  |  |  |  |
| Unique ID | 950081 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-230742 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 09/2023 | Performance part: 12/2023 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 90% | Performance Part: 20% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Yasuki Suzuki |
|  | Company | KDDI |
|  | Email | ui-suzuki@kddi.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#106bis
<UE RF>

- WF in [39] captured the following agreements:

  - Type 1 and Type 2 means different UE capability.

  - Type 1 is mandatory and Type 2 is optional.

  - Postpone to discuss whether UE supports to configure between Type 1 and Type 3 on a request from BS and also Type 1 shall be default when no request comes from BS to future release.

  - Postpone to discuss whether for UE supporting Type-3, clarification is required to reconfigure to Type-2 or not to future release.

  - RAN4 suggests to postpone Type 3a/3b to future release.

  - Postpone to discuss whether to decouple discussion on MRTD and power imbalance requirement for Type 3a/3b to future release.

  - Postpone to discuss whether to keep TAE/network synchronization requirement for Type 3a/3b to future release.

  - Postpone to discuss whether to cope with both 25dB power imbalance (including the relaxation<25dB) and MRTD>CP Length for Type 3a/3b to future release.

  - Postpone to discuss with the assumption MRTD>CP, whether to discuss the RF requirement/performance for 25dB power imbalance (including the relaxation<25dB) for type3a/3b based on companies’ input for Type 3a/3b to future release.

  - RAN4 suggests to postpone Type 4a/4b to future release.

  - Postpone to discuss whether UE Type 2 power imbalance is re-used for Type 4a/4b to future release.

  - Postpone to discuss whether BS TAE is 3us to future release.

  - Postpone to discuss whether MRTD=33us for Type 4a/4b to future release.

  - Postpone to discuss whether to introduce UE Capability for Type 4a/4b to future release.

  - RAN4 suggests to postpone non-collocated contiguous 2CC and non-contiguous 3CC for intra-band NR-CA (Type 2/3/4) and inter-band EN-DC (Type 3/4) to future release.

<RRM>

- WF in [35] captured the following agreements:

  - CR on scheduling Restriction for intra-band non-collocated CA type 2 UE was endorsed [36].

  - Draft CR to TS38.133 on MTTD/MRTD and interruption requirements for Rel-18 intra-band CA Type-2 UE was endorsed [37].

  - Draft CR on interruption requirements for supporting non-collocated FR1 intra-band CA was endorsed [38].

  - For non-collocated FR1 intra-band non-contiguous CA

    - The existing activation delay requirements (clause 8.3.2) for a known FR1 SCell can be re-used for Type 2 UE.

    - The existing activation delay requirements (clause 8.3.2) for an unknown FR1 SCell for non-contiguous intra-band CA can be re-used for Type 2 UE.

    - FFS whether any CRs are needed to clarify the requirements applicability.

  - For non-collocated FR1 intra-band non-contiguous CA, Type 2 UE can be configured with BFD/CBD measurements on up to two serving cells in the same band

<Demodulation performance>

- Work plan for demodulation performance part of WI intra-band non-colocated EN-DC/NR-CA deployment was approved [4].

- WF in [29] captured the following agreements:

  - Agree with the following work plan. Update work plan, if necessary, according to the RF/RRM core discussion.

- RAN4#106bis-e, April 2023 (0.25 TU) - Discuss the scope of UE demodulation requirements. - RAN4#107, May 2023 (0.25 TU) - Continue to discuss the scope of UE demodulation requirements, according to the core part conclusion. - Agree with the initial simulation assumption. - RAN4#108, August 2023 (0.25 TU) - Collect the initial simulation results. - Finalize the test parameters for UE demodulation requirements, and update the simulation assumption if necessary. - Decide the CR work split. - RAN4#108bis, October 2023 (0.25 TU) - Update the simulation assumption if necessary. - Continue the alignment of the simulation results. - Review the draft CRs for TS38.101-4. - RAN4#109, November 2023 (0.25 TU) - Collect the simulation results if necessary. - Endorse the draft CRs for TS38.101-4.

  - Discuss the UE demodulation requirements for Type 2 UE first.

    - Test scope covers both EN-DC and NR-CA

  - Not define the UE demodulation requirements for Type 3 UE.

  - For UE demodulation requirements for Type 4 UE, depending on the progress of RF discussion.

RAN4#107
<UE RF>

- WF in [69] captured the following agreements:

  - Propose revised WID R4-2310481 to next RAN Plenary.

  - Introduce an optional new IE for NR CA, [intraBandNonColocatedCA-r18] for UE supporting type 2 UE capability on NR CA operation.

- Revised WID was endorsed [71].

- CR on UE RF requirement for intra-band non-collocated CA for 2MIMO layer case was agreed [44].

<RRM>

- RRM Chair note captured the following agreements:

  - For Type 2 UE, reuse the values of TSMTC_MAX and TFirstSSB_MAX used for defining FR1 inter-band CA SCell activation delay requirements for FR1 intra-band non-contiguous CA.

  - Do not define RRM requirements to support Type 3a/3b and Type 4a/4b UEs in Rel-18

- Draft CR on Scell activation and BFD/CBD requirements for Type 2 UE was endorsed [67].

- Draft CR to TS38.133 on BFD/CBD Requirement for Rel-18 intra-band CA Type-2 UE was endorsed [68].

- Draft Big CR on RRM was agreed [72].

<Demodulation performance>

- WF in [64] captured the following agreements:

  - RAN4 should discuss the need for demodulation requirements only for Type-2 UEs.

    - Preclude Type-3a/3b and Type-4a/4b UE from the UE demodulation performance requirements discussion.

  - Define new Type 2 UE demodulation requirements with NR-CA scenario only.

  - Assume the power difference of 25dB and received time difference of 33us under the assumption that requirements introduced under static channel.

2.4.2	Remaining Open issues

<UE RF>

- How UE supporting Type 2 NR-CA/EN-DC behaves between Type 1 and Type 2

<Demodulation performance>

- Specify PDSCH demodulation requirements for non-collocated scenarios for intra-band non-contiguous NR-CA.

- Discuss the necessary RRM test cases.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R4-2304307	LS on signaling support for intra-band non-collocated CA

- R4-2304041	Discussion on viable UE architectures

- R4-2304085	Discussion on non-collocated EN-DC,NR-CA

- R4-2304086	Work plan for demodulation performance part of WI intra-band non-colocated EN-DC/NR-CA deployment

- R4-2304102	Demodulation requirements for non-collocated FR1 intra-band EN-DC/NR-CA

- R4-2304157	Dinamically Restricted PDSCH Scheduling for Intra-band Non-colocated NR CA Scenarios

- R4-2304160	UE configuration assumptions for Type-2/3

- R4-2304308	On type 3a/3b UE for intra-band non-collocated CA/EN-DC

- R4-2304309	CR on scheduling Restriction for intra-band non-collocated CA type 2 UE

- R4-2304693	Discussion on demodulation requirement for non-collocated  Intra-Band EN-DC NR-CA and inter-band EN-DC

- R4-2304694	Discussion on UE RF of non-collocated EN-DC and NR-CA

- R4-2304698	Discussion on RRM requirements for non-collocated intra-band scenario

- R4-2304719	Views on non-collocated deployment

- R4-2304817	Discussion on RRM requirements for FR1 non-co-located EN-DC and NR-CA

- R4-2304835	MRTD and MTTD for UE architecture Type 4

- R4-2304836	MRTD/MTTD requirement for non-collocated inter-band EN-DC with overlapping bands and intra band non-contiguous NR-CA

- R4-2304872	Discussion on intra-band non-co-located

- R4-2305099	Discussion on Type-3 UE RF requirement

- R4-2305169	Discussion on RRM requirements for non-collocated EN-DC and NR-CA

- R4-2305181	UE demodulation requirements for non-colocated NR-CA/EN-DC deployment scenario

- R4-2305200	Discussion on non-collocated FR1 inter-band EN-DC with overlapping DL bands and FR1 intra-band NR-CA deployment

- R4-2305201	Draft CR to TS38.133 on MTTD/MRTD and interruption requirements for Rel-18 intra-band CA Type-2 UE

- R4-2305317	Discussion on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment

- R4-2305318	DraftCR on interruption requirements for supporting non-collocated FR1 intra-band CA

- R4-2305469	Discussions on demodulation requirements for non-collocated EN-DC/NR-CA deployment

- R4-2305530	Performance issues for Non-collocated Deployments

- R4-2305657	UE Architectures Suitable for Non-Collocated Intra-Band CA/EN-DC Rel18

- R4-2305866 Email discussion summary for [106bis-e][320] NonCol_intraB_ENDC_NR_CA_Demod

- R4-2305891	WF on UE demodulation requirements for intra-band non-collocated EN-DC/NR-CA deployment scenario

- R4-2305989 Email discussion summary for [106bis-e][320] NonCol_intraB_ENDC_NR_CA_Demod

- R4-2306157	Topic summary for [106-bis-e][209] NonCol_intraB_ENDC_NR_CA

- R4-2306210	Topic summary for [106-bis-e][131] NonCol_intraB

- R4-2306240	Topic summary for [106-bis-e][209] NonCol_intraB_ENDC_NR_CA

- R4-2306293	Topic summary for [106-bis-e][131] NonCol_intraB

- R4-2306335	WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA

- R4-2306336	CR on scheduling Restriction for intra-band non-collocated CA type 2 UE

- R4-2306337	Draft CR to TS38.133 on MTTD/MRTD and interruption requirements for Rel-18 intra-band CA Type-2 UE

- R4-2306338	DraftCR on interruption requirements for supporting non-collocated FR1 intra-band CA

- R4-2306605	WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment

- R4-2307033	Discussion on UE Demodulation for non-collocated FR1 intra-band EN-DC/NR-CA

- R4-2307127	revised WID for intra-band non-collocated EN-DC/NR-CA deployment

- R4-2307128	Views on the Type 2 UE for Non-collocated deployment

- R4-2307665	On dynamic configuration for Type 2 UE

- R4-2317666	WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment

- R4-2307854	Discussion on UE demodulation requirements for intra-band non-collocated EN-DC/NR-CA deployment scenario

- R4-2307891	Discussion on RRM requirements for FR1 non-co-located EN-DC and NR-CA

- R4-2308404	Discussion on demodulation requirement for intra-band non-collocated EN-DCNR-CA deployment

- R4-2308123	Views on the remaining issue for Non-collocated deployment

- R4-2308169	Discussion on UE RF of intra-band non-collocated EN-DC and NR-CA

- R4-2308430	UE demodulation requirements for non-colocated NR-CA/EN-DC deployment scenario

- R4-2308703	Discussion on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment

- R4-2308704	DraftCR on Scell activation and BFD/CBD requirements for Type 2 UE

- R4-2308862	Discussions on non-collocated NR-CA/EN-DC PDSCH requirements

- R4-2308924	On timing requirements for non-colocated intra band

- R4-2308928	Discussion on intra-band non-co-located

- R4-2309114	RRM requirements for non-collocated FR1 inter-band EN-DC with overlapping bands

- R4-2309115	MRTD/MTTD requirement for non-collocated inter-band EN-DC with overlapping bands

- R4-2309116	38133CR on interruption requirement for FR1 non-collocated EN-DC

- R4-2309297	Demodulation Tests for non-collocated intra-band CA Deployments

- R4-2309318	On Remaining issues for FR1 intra-band NR-CA Type-2 UE RRM Requirement

- R4-2309319	Draft CR to TS38.133 on BFD/CBD Requirement for Rel-18 intra-band CA Type-2 UE

- R4-2309372	Further Discussion on Demodulation Requirements of Type-2 UEs in Intra-band Non-collocated EN-DC/NR-CA

- R4-2309451	Enabling Network to change UE reception to switch between collocated and non-collocated modes

- R4-2309807	WF for NonCol_intraB_ENDC_NR_CA_Demod

- R4-2309959	Topic summary for [107][214] NonCol_intraB_ENDC_NR_CA

- R4-2310017	Topic summary for [107][134] NonCol_intraB

- R4-2310048	DraftCR on Scell activation and BFD/CBD requirements for Type 2 UE

- R4-2310049	Draft CR to TS38.133 on BFD/CBD Requirement for Rel-18 intra-band CA Type-2 UE

- R4-2310300	On RF requirements for intra-band non-collocated CA

- R4-2310460 Topic summary for [107][323] NonCol_intraB_ENDC_NR_CA_Demod

- R4-2310481	revised WID for intra-band non-collocated EN-DC/NR-CA deployment

- R4-2310179 Draft Big CR on RRM requirements for support of intra-band non-collocated EN-DC NR-CA deployment