3GPP TSG-RAN WG3 Meeting #120	R3-232664

# Incheon, KR, 22nd – 26th May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1909 | rev | 4 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for SubcarrierSpacingSSB [NR_ext_to_71GHz-Core] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Qualcomm, Huawei, China Telecom, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is a misalignment between TS36.413 and TS36.331 on SubcarrierSpacingSSB - TS36.413 defines it as ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, ...) - Rel-17 TS36.331 defines 2 new codepoints kHz480 and kHz960 that was introduced in TS36.331 per NR_ext_to_71GHz-Core WI subcarrierSpacingSSB-r15 ENUMERATED {kHz15, kHz30, kHz120, kHz240}, subcarrierSpacingSSB-r17 ENUMERATED {kHz480, kHz960} OPTIONAL, -- Need OR kHz60 is not used in TS36.331. TS36.413 does not include the new Rel-17 codepoints. This is to support the RAN1 lead WI [NR_ext_to_71GHz-Core] in RP-222656 which was completed in September 2022. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are as follows: - Clarify kHz60 is not used in the speficication - Add kHz480 and kHz960 - Correct the RRC reference Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has impact under functional point of view. The impact can be considered isolated because it only affect the SON feature related to NR using subcarrierSpacingSSB kHz480 and kHz960. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies and inaccuracies remain in the S1AP specification, and not align with TS36.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.151, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: updated the WI code. Rev 2: resubmit to RAN3#119bis-e. Rev 3: updated the cover page during RAN3#119bis-e. Rev 4: resubmit for RAN3#120 |  |  |  |  |  |  |  |  |

Start of Modification

#### 9.2.1.151	Inter System Measurement Configuration

The Inter-System Measurement Configuration IE contains information for instructing the incoming UE to continue measuring the cells of the NR RAT after a successful inter-system handover to LTE network.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RSRP | O |  | INTEGER (0.. 127) | Threshold of RSRP. | - | - |
| RSRQ | O |  | INTEGER (0.. 127) | Threshold of RSRQ. | - | - |
| SINR | O |  | INTEGER (0..127) | Threshold of SINR | - | - |
| Inter-System Measurement Parameters | M |  |  |  | - | - |
| >Measurement Duration | M |  | INTEGER (1..100) | The period of time following the successful IRAT handover, during which the target RAT instructs the UE to measure cells of the source RAT. Unit: [second]. | - | - |
| >Inter-System Measurement List |  | 0..1 |  |  | EACH | reject |
| >>Inter-System Measurement Item |  | 1.. <maxnooffrequencies> |  | List of inter-system measurements configured | YES | reject |
| >>>FreqBandIndicatorNR | M |  | INTEGER (1..1024) | The frequency band in which the ssbFrequency is located and according to which the UE shall perform the RRC measurements. |  |  |
| >>>SSB frequencies | M |  | INTEGER (0..maxNARFCN) | Designates the specific SSB frequencies i.e., ARFCN-ValueNR which the target RAT may instruct the UE to measure. | - | - |
| >>>SubcarrierSpacingSSB | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, ..., kHz480, kHz960) | Corresponds to subcarrierSpacingSSB IE contained in the MeasObjectNR IE as defined in TS 36.331 [16]. The value kHz60 is not used in this release. |  |  |
| >>>maxRS-IndexCellQual | O |  | INTEGER (1..maxRS-IndexCellQual) | Indicates the maximum number of RS indices to be considered/ averaged to derive the cell quality for RRM. Corresponds to the MaxRS-IndexCellQualNR IE as defined in TS 36.331 [16] |  |  |
| >>>SMTC | O |  | OCTET STRING | Includes the MTC-SSB-NR IE as defined in TS 36.331 [16]. |  |  |
| >>>threshRS-Index-r15 | O |  | OCTET STRING | Includes the threshRS-Index contained in the MeasObjectNR IE as defined in TS 36.331 [16], List of thresholds for consolidation of L1 measurements per RS index |  |  |
| >>>SSB-ToMeasure | O |  | OCTET STRING | Includes the SSB-ToMeasure IE as defined in TS 36.331 [16]. |  |  |
| >>>SS-RSSI-Measurements | O |  | OCTET STRING | Includes the SS-RSSI-Measurement IE as defined in TS 36.331 [16]. |  |  |
| >>>quantityConfigNR-R15 | O |  | OCTET STRING | Includes the quantityConfigNR contained in the QuantityConfig IE as defined in TS 36.331 [16]. |  |  |
| >>>excludedCellsToAddModList | O |  | OCTET STRING | Includes the excludedCellsToAddModList contained in the MeasObjectNR IE as defined in TS 36.331 [16]. It applies only to SSB resources. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnooffrequencies | Maximum no. of frequencies. Value is 64. |
| maxNARFCN | Maximum value of NR carrier frequency, corresponds to the maxNARFCN as defined in TS 38.331 [50]. Value is 3279165 |
| maxRS-IndexCellQual | Maximum number of RS indices averaged to derive cell quality for RRM, corresponds to the maxRS-IndexCellQual-r15 as defined in TS 36.331 [16]. Value is 16 |

Next Modification

### 9.3.4	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

** Unchanged text skipped **

InterSystemMeasurementList ::= SEQUENCE (SIZE(1.. maxnooffrequencies)) OF InterSystemMeasurementItem

InterSystemMeasurementItem ::= SEQUENCE {

freqBandIndicatorNR			INTEGER (1..1024),

sSBfrequencies				INTEGER (0..maxNARFCN),

subcarrierSpacingSSB		ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, ..., kHz480, kHz960},

maxRSIndexCellQual			INTEGER (1..maxRS-IndexCellQual)  	OPTIONAL,

sMTC						OCTET STRING 						OPTIONAL,

threshRS-Index-r15			OCTET STRING 						OPTIONAL,

sSBToMeasure				OCTET STRING						OPTIONAL,

sSRSSIMeasurement			OCTET STRING						OPTIONAL,

quantityConfigNR-R15		OCTET STRING						OPTIONAL,

excludedCellsToAddModList		OCTET STRING						OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { InterSystemMeasurementItem-ExtIEs} } OPTIONAL

}

InterSystemMeasurementItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

End of Modification