3GPP TSG-RAN #100	Tdoc RP-231110

Taipei, June 12th – 14th, 2023

Agenda Item:	9.3.2.1

Source:	Ericsson

Title:	Down-scoping of Rel-18 NR Mobility enhancements

Document for:	Discussion

# 1	Introduction

The Rel-18 work item further NR Mobility enhancements [1] is large in scope, and especially the amount of RAN2 work is overwhelming and will be a challenge to complete in time. To make sure all the objectives in the Rel-18 mobility WI are completed and with high quality, we think there are items that can be explicitly down-scoped and/or removed from the WI.

# 2	Discussion

For reference, we have copied the detailed objectives in [1]  to the annex of this contribution. We go through each of these objectives one by one and identify items for removal.

Objective 1: 	L1/L2-triggered mobility (LTM)

We think that it is of outmost importance to secure completion of this objective and with high quality. There are some aspects that are being discussed in RAN WGs not explicitly mentioned in the WI scope and not absolutely required for the feature to work. Also, some other aspects mentioned in the WID we see as less important. The items for this objective that can be removed in Rel-18 are the following:

- Use of reference and delta configuration. We think that this is an optimization not needed for the feature to work. It could easily become very complex in the specifications and in the implementation. Even without a reference configuration, the network will be capable to configure multiple candidate cells with the only difference that these LTM candidate cells are required to be a “complete” configuration with respect the current UE configuration. On top of this, the reference configuration has zero impact on the connectivity interruption due to an LTM cell switch and we believe that the main focus for this objective should be to work only on the delay components. We suggest that in Rel-18, an LTM candidate cell configuration should always be a “complete” configuration and the use of reference configuration is not supported.

- Support for NR-DC. We think that LTM for SCG is not critical in Rel-18 and we can remove this from the scope. Also, we think that during LTM for the MCG, the simplest would be to release SCG when LTM is triggered for the MCG to avoid complex cases when SCG is to be reconfigured. Therefore, we think that we can remove NR-DC support for LTM completely in Rel-18 to reduce complexity and free up time for more critical things.

For LTM, we think the above two bullets are potentially the most time consuming and complex from specification and especially RAN2 workload perspective.  Additionally, we think the following objectives can be discussed for possible down-scoping:

- CSI-RS L1 measurements. With only one RAN1 meeting left, RAN1 has not yet agreed a Rel-18 solution for CSI-RS measurements for LTM. SSB measurements should be enough in Rel-18.

- MAC CE measurement reporting for LTM. Recently, the option of transmitting the LTM lower-layer measurement reports with MAC CE was brought up in RAN2 to overcome some limitation with L1 signalling. We think that in Rel-18, as RAN1 already agreed, the LTM measurement reporting can be limited to L1 reports.

For LTM objective (Objective 1): Use of reference and delta configuration is down-prioritized (or alternatively, down-scoped and removed from the WI scope).

Remove support for NR-DC from Objective 1.

Objective 2:		NR-DC with selective activation of the cell groups (Subsequent CPAC)

The items for this objective that can be removed in Rel-18 are the following:

- Inter-SN scenarios. Limit the scope to only support intra-SN PSCell changes. This avoids:

  - issues due to the different UE and network configurations and procedures for intra-SN CPC and inter-SN CPC;

  - issues with security key updates at inter-SN PSCell changes;

  - a major part of the inter-node signaling impact for the objective.

- Use of reference and delta configuration. If we remove this for LTM, it should be removed here too for the same reasons. Only “complete” configuration to be supported in Rel-18.

- MN-initiated PSCell change. Support only the SN-initiated PSCell change in Rel-18. SN control of PSCell mobility is more relevant, and there is today e.g. no support for MN-initiated intra-SN CPC. To limit the scope of the objective we could therefore remove the MN-initiated PSCell changes.

- Support for CPA. Limit the scope to only support the configuration of subsequent CPC. Thereby, the additional complexity and specification efforts that would be required to reuse a CPA configuration for CPC, with additional triggering conditions, can be avoided.

Remove the following from Objective 2: Use of reference and delta configuration, inter-SN scenarios, MN-initiated PSCell changes and support for CPA.

Objective 3: 	CHO including target MCG and target SCG

We think this objective can remain as is. The basic feature was included in Rel-17 and the things added in Rel-18 (RAN3) makes this feature complete. The basic function with a single target SCG and with no further data forwarding optimizations is enough in Rel-18.

Objective 4: 	CHO including target MCG and candidate SCGs

Keep as is. The progress for this objective has been rather good so it should be possible to conclude it in time for the Rel-18.

Objectives 5-6: 	Applicable RRM and RF requirements for the above objectives

Based on the down-scoping of objectives 1-4, the corresponding RAN4 work is to be removed as well.

Remove corresponding RAN4 work from objectives 5-6 based on down-scoping of objectives 1-4.

Objective 7: 	Idle/inactive mode measurement enhancements

Based on the recent RAN4 progress there are still two solutions discussed in parallel.  We suggest RAN4 and RAN2 to focus on introducing indication of availability and/or validity status of measurement results from UE to NW into the (existing) EMR reports. Alternative is to remove this objective completely.

For Objective 7, focus on to specify a solution for the measurement validity issue based on existing measurements. Otherwise, the objective should be removed completely.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For LTM objective (Objective 1): Use of reference and delta configuration is down-prioritized (or alternatively, down-scoped and removed from the WI scope).

Proposal 2	Remove support for NR-DC from Objective 1.

Proposal 3	Remove the following from Objective 2: Use of reference and delta configuration, inter-SN scenarios, MN-initiated PSCell changes and support for CPA.

Proposal 4	Remove corresponding RAN4 work from objectives 5-6 based on down-scoping of objectives 1-4.

Proposal 5	For Objective 7, focus on to specify a solution for the measurement validity issue based on existing measurements. Otherwise, the objective should be removed completely.

# 4	References

RP-223520, 3GPP work item description: Further NR mobility enhancements, MediaTek Inc, Apple, 3GPP TSG RAN Meeting #98-e, Electronic Meeting, December 12-16, 2022

# Annex for reference only: Detailed WI objectives from [1]

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2]

  - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet

- Timing Advance management [RAN1, RAN2]

- CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3]

Note 2: FR2 specific enhancements are not precluded, if any.

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

- For CHO including target MCG and target SCG in NR-DC [RAN3]:

  - to specify data forwarding optimizations; and

  - to specify, if needed, a solution to avoid unnecessary signaling exchange between source MN and target SN.

- To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2]

- CHO including target MCG and target SCG is used as the baseline

- To specify RRM core requirements for the following, as necessary [RAN4]:

- L1/L2-based inter-cell mobility

- Enhanced CHO configurations addressed by this WI

- To specify RF requirements to cover inter-frequency L1/L2-based mobility, as necessary [RAN4].

- To study and specify how to reuse the IDLE/INACTIVE mode measurement results which are to be reported during and/or after RRC connection setup/resume in order to improve SCell/SCG setup delay [RAN4, RAN2], including:

  - Availability and validation of the IDLE/INACTIVE mode measurement results to be reported [RAN4]; and

  - Definition of corresponding RRM requirements [RAN4]; and

  - If necessary based on RAN4 outcome, definition of corresponding signalling support [RAN2].

Note 5: RAN4 will coordinate in due course with RAN2 to start the work.

Note 6: R4-2220415 serves as baseline for future work in RAN4

Note 7: With exception of the above scenarios, enhancements on IDLE/INACTIVE mode measurements and on UE behavior in IDLE/INACTIVE mode are not in scope.