3GPP TSG RAN Meeting #100				                 RP-231163

Taipei, June 12-14, 2023

# Source:	CMCC

# Title:	Discussion on support of UE configured with two serving cells, each with SUL

# Document for:	Decision

Agenda Item:	7

- Introduction

In RAN#99 meeting, how to support the CA band combinations with two SUL cells were discussed [1]. It was tasked by RAN that RAN4 should assess the additional RAN4 specification impact and UE implementation impact to support the UE configured with two serving cells, each with SUL, the example band combinations are referred to [2].

Following RAN task, RAN4 discussed this issue and reply LS to TSG RAN in [3].

RAN4 discussed RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL. The agreements are: - For a UE configured with two serving cells, each with SUL, UL Tx switching between two SUL carriers and between SUL carrier and non-corresponding NUL carrier can be supported with UL Tx switching framework. Time mask requirements for switching across three or four uplink bands can be applied. No RAN4 specification impact is observed. No UE implementation issue is observed. - For a UE configured with two serving cells, each with SUL, back to back transmission between two SUL carriers and back-to back transmission between SUL carrier and non-corresponding NUL carrier without any switching period can be supported by some UE implementations but enabling such feature will require RAN4 work.

- Discussion

According to RAN task, there are two approaches to support UE configured with two serving cell, each with SUL:

- UL Tx switching framework with UE capability based switching period.

- back-to-back transmission between two SUL carriers and back-to-back transmission between SUL carrier and non-corresponding NUL carrier.

In RAN4#106bis and RAN4#107 meeting, RAN4 discussed the UE implementation issue and specification impact for the above two cases respectively. As captured in RAN4 LS, the agreements are:

- For a UE configured with two serving cells, each with SUL, UL Tx switching between two SUL carriers and between SUL carrier and non-corresponding NUL carrier can be supported with UL Tx switching framework. Time mask requirements for switching across three or four uplink bands can be applied. No RAN4 specification impact is observed. No UE implementation issue is observed.

- For a UE configured with two serving cells, each with SUL, back to back transmission between two SUL carriers and back-to back transmission between SUL carrier and non-corresponding NUL carrier without any switching period can be supported by some UE implementations but enabling such feature will require RAN4 work.

According to RAN4 agreements, UL Tx switching framework can support UE configured with two serving cells, each with SUL without additional spec impact and UE implementation impact.

For back-to-back transmission, it can be supported by some UE implementation, but regarding the potential RAN4 spec impact, there is no consensus in RAN4 on the exact RAN4 spec impact. Hence, in LS, it is simply captured that enabling such feature will require RAN4 work.

Observation 1: UL Tx switching framework can be applied to UE configured with two serving cell, each with SUL, no RAN4 spec impact and UE implementation issue is observed.

During the RAN1#113 meeting, the draft CR for R18 Tx switching for TS 38.214 was discussed, which the following issue#1 is about the clarification of Simultaneous UL Tx when SUL(s) is configured.

- Issue #1: Simultaneous UL Tx when SUL is part of the configuration The following alternatives can be identified when at least one cell with a SUL carrier is part of the UE’s configuration Alt 1: Transmission may take place only on one uplink band at a time Alt 2: One or more of the following simultaneous transmission cases is allowed - Transmission on two non-SUL UL bands may take place at the same time if UE reported ‘DualUL’ for that band pair - Transmission on one cell’s SUL carrier and another cell’s non-SUL band may take place at the same time if the UE reported ‘DualUL’ for that band pair - Transmission on one cell’s SUL carrier and another cell’s SUL carrier may take place at the same time if the UE reported ‘DualUL’ for that band pair

The current endorsed RAN1 specification has supported the Tx switching in 2SUL cells scenario,  the only difference between Alt 1 and Alt 2 is that whether to further capture such restriction in RAN1’ spec as the suggestion from TS 38.214 editor:

Alt.1 impact to 38.214 is along the square-bracketed text discussed in RAN1#112bis: - For a band combination including supplementary uplink bands, for all band pairs only the uplinkTxSwitchingOption set to 'switchedUL' is supported. Alt.2 with all the simultaneous transmission cases supported would have the following RAN1 specification impact: - For a band combination including supplementary uplink bands, for the band pairs where SUL band and non-SUL band belong to the same cell, only the uplinkTxSwitchingOption set to 'switchedUL' is supported.

Observation 2: There is no additional RAN1 work to support UL Tx switching for UE configured with two serving cell, each with SUL.

Based on the above discussion, both from RAN1 and RAN4 side, it is feasible to support Tx switching for UE configured with two SUL serving cells without specification impact. Thus Tx switching for UE configured with two serving cells, each with SUL should be supported in Rel-18 and the following three simultaneous transmission cases as in Alt 2 discussed in RAN1#113 should also be supported.

- Transmission on two non-SUL UL bands

- Transmission on one cell’s SUL carrier and another cell’s non-SUL band

- Transmission on one cell’s SUL carrier and another cell’s SUL carrier

Proposal: It is proposed to remove the NOTE 2 in WID on NR CA band combinations with two SUL cells in Rel-18 [4] and add the CA configurations for SUL and corresponding NUL, SUL and SUL.

- Conclusion

In this contribution, we discuss the support of UE configured with two serving cells, each with SUL. The observations and proposals are as follows:

Observation 1: UL Tx switching framework can be applied to UE configured with two serving cell, each with SUL, no RAN4 spec impact and UE implementation issue is observed.

Observation 2: There is no additional RAN1 work to support UL Tx switching for UE configured with two serving cell, each with SUL.

Proposal: It is proposed to remove the NOTE 2 in WID on NR CA band combinations with two SUL cells in Rel-18 [4] and add the CA configurations for SUL and corresponding NUL, SUL and SUL.

- Reference

RP-230810, Offline discussion summary on RAN1-MultiCarrier.

RP-223553, New WID on NR CA band combinations with two SUL cells in Rel-18, RAN#98

R4-2310394, LS on support of UE configured with two serving cells, each with SUL, RAN4#107.

RP-231162, Revised WID on Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity).

R4-2306651, WF on RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL

R1-2306256,Summary on the [113-R18-Others-02] Email discussion on MC-Enh draft CR for TS38.214.