3GPP TSG RAN Meeting #100				                 RP-231165

Taipei, June 12-14, 2023

# Source:	CMCC

# Title:	Discussion on scope of WI on complete the specification support for BandWidth Part operation without restriction in NR

# Document for:	Decision

Agenda Item:	9.3.4.14

- Introduction

In RAN#99 meeting, a new WID on complete the specification support for Bandwidth Part operation without restriction in NR was approved.

In the WID, only support of BM/RLM/BFD is mentioned in all the options. In RAN4#106bis and 107 meeting, whether to also specify the L3 related requirements for all the options were discussed and LS was sent to TSG RAN [2].

- Discussion

In previous RAN4 meeting and LS to RAN (R4-2220437), RAN4 agreed that no measurement gap is needed for intra frequency L3 measurement and can help the measurement. Based on this LS, RAN plenary made the decision on choose of options to support BWP without restriction and approve the WID.

Also, for UE support of B-1-1 and B-1-2, no measurement gap for intra-frequency L3 measurement does not require additional UE capability, and can be supported with minor spec changes. Hence, we support to include L3 measurements for option B-1-1 and B-1-2.

Proposal 1: Update the objectives to include L3 SSB-based intra-frequency measurements for option B-1-1 and B-1-2.

Table 3: High-level analysis of candidate options on Mobility performance impact

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) |  Measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements with gap requirements apply.  Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option B-1-1) |  No measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements without gap requirements apply.  Measurement gap is needed for inter-frequency L3 measurement  Can also help L3 intra-frequency measurement | Low |
| Option B-1-2) |  No measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements without gap requirements apply.  Measurement gap is needed for inter-frequency L3 measurement  Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option B-2-2) |  Measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements with gap requirements apply.  Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option C) |  Measurement gap may or may not be needed for intra-frequency L3 measurement depending on whether or not NCD-SSBs from all neighbor cells are present within UE active BWP.  Measurement gap is needed for inter-frequency L3 measurement. |  |

In addition, in the RAN4#106-bis-e meeting it was agreed that the support of L3 measurements based on NCD-SSB for Option C can be beneficial. Considering that NCD-SSB was already specified for RedCap UE in Rel-17, apply the corresponding requirements to normal UE should be straightforward and the spec impacts are not that much.

Proposal 2: Update the objectives to include L3 SSB-based intra-frequency measurements for option C.

- Conclusion

In this contribution, we discuss the L3 measurements for options to support BWP without restriction, and the proposals are:

Proposal 1: Update the objectives to include L3 SSB-based intra-frequency measurements for option B-1-1 and B-1-2.

Proposal 2: Update the objectives to include L3 SSB-based intra-frequency measurements for option C.

- Reference

RP-230805, New WID: Complete the specification support for BandWidth Part, Vodafone.

R4-2310177, LS on L3 related requirements for the options for BWP operation without restriction