3GPP TSG-RAN Meeting #100		RP-231183

Taipei, Taiwan, June 12-16, 2023

Agenda item:	9.3.2.9

Source:	Apple

Title:	On further enhancement of UE TRP and TRS requirements and test methodologies including XR UEs

Release:	Rel-18

Document for:	Discussion

# 1	Introduction

The recent achievement by RAN4 in defining Rel-17 TRP and TRS requirements for handheld UEs [1], [2] and the steady progress in the working group on Rel-18 TRP and TRS enhancements [3], [4] offer a unique opportunity for 3GPP RAN to resolve the currently ongoing discussion in relation to the potentially new XR UE device type [5].  Based on the proposed way forward (not yet approved) in [6] and the related discussion [7], it appears that one of the key concerns raised by operators is the network performance impact due to potential coverage reduction of XR devices which support 2 Rx antennas instead of 4 (in bands where 4 Rx are required).  There exists the potential to leverage OTA requirements for XR devices to address operator concerns and, coupled with the confidence we have in RAN4’s ability to conclude OTA requirements, can become an effective way to move forward on the 2Rx for XR discussion.

# 2	Discussion

The proposed way forward on handling the new XR device type, discussed in the RAN #98 meeting, was as follows [6]:

- Proposed Way Forward: For 3GPP Rel-18 XR Work Item, include the following objective - Specify mechanism to support new device type (XR) that is only mandated to support 2Rx antenna ports in the NR bands in which 4Rx antenna ports is mandated in legacy [RAN4, RAN2]

The discussion of this proposal was captured in the RAN #98 report [7]:

RP-223208 Way forward on the support of 2Rx for XR device for Rel-18 XR WI Type: discussion For: Decision Source: Apple, ABS, APT, ASR, CATT, FGI, Google, HONOR, Lenovo, Meta, New H3C, Qualcomm, Samsung, Tencent, Verizon, vivo, Xiaomi Discussion: handled in discussion [98e-07-R18-XR]; Apple: KT Corp., Microsoft and Spark New Zealand are supporting this Tdoc RP-223208 handled in Mon GTW Proposed Way Forward: For 3GPP Rel-18 XR Work Item, include the following objective Specify mechanism to support new device type (XR) that is only mandated to support 2Rx antenna ports in the NR bands in which 4Rx antenna ports is mandated in legacy [RAN4, RAN2] Qualcomm: we support this, topic is controversial, decision should come from TSG RAN avoiding to spend more time in WGs Huawei: overall scope of the WI needs to be discussed before going in details, no all form factors have limitations; at first we should define which devices have problems with 4Rx CMCC: 2RX with 100MHz on smart phone will reduce network performance, defining a new redcap device may be considered Vodafone: we do not support 2Rx without further explanation, glasses have enough space for 4Rx so it is not about form factor China Unicom: same view as Vodafone Telecom Italia: strong concern about the proposal and would not like to see redcap used for this DT: similar view as other operators and start of an XR WI shall be shifted Orange: XR needs high capacity and going for 2RX is contradicting so too early to agree on anything there AT&T: we are fine to extend the SI by 1 quarter but scope shall not increase; form factors and use cases need to be clarified before abandoning 4Rx can be discussed Verizon: we can discuss the form factor; we were accomodating 1RX redcap so we are open for discussions T-Mobile: no industrial design criteria is given; difference between XR and redcap is that redcap is not supposed to be data heavy; reducing perf. criteria for XR is not appropriate without more detailed explanations/discussions Decision: The document was noted.

Observation 1:	Operators voiced concerns with the coverage of a 2Rx XR device potentially impacting network performance.

Observation 2:	In field operation, it is the radiated performance of devices that impacts network RF planning and performance.

Given Observations 1 and 2, the 2Rx XR discussion provides a clear motivation for 3GPP to define radiated requirements for XR devices and to study whether it is feasible to unify the OTA requirements for all XR devices, regardless of whether such devices support 2Rx or 4Rx.

Proposal 1:	To resolve operator concerns with the coverage of 2Rx XR devices, 3GPP should introduce OTA requirements for XR devices and study whether it is feasible to unify the OTA requirements for all XR devices, regardless of whether such devices support 2Rx or 4Rx.

During the RAN #99 meeting this issue was discussed, with the following notes captured by the topic moderator (note that these are not formally agreed by RAN) [8]:

Potential way forward on 2Rx XR devices - Step-1: Recognize that form factor limitations of a subset of XR devices can make it impossible for these devices to support 4 antenna ports - Exact characterization and form factor limitations of these devices are to be studied in RAN plenary. Contributions are invited to TSG-RAN#100 to address this. - Step-2: Contingent on the outcome of the Step-1: Recognize the importance of identifying these devices in the network, and potential network impact (e.g. performance, coverage, capacity) and UE impact - Details of Step-2 will be further defined following the maturity of Step-1

One consideration of the impact of 2Rx XR devices on network performance is related to the radiated performance of these devices, and our Proposal 1 could introduce a potential unification of radiated performance for all XR UEs.  This, in turn, can alleviate the network’s RF planning for coverage.

Observation 3:	The potential to unify XR radiated requirements can alleviate the networks RF planning for coverage.

With industry-wide recognition of the importance of OTA requirements in terms of conformance of devices to common radiated limits, device certification, and network planning, RAN4  has completed the Rel-17 test methods and requirements and is making steady progress with the Rel-18 work scope.  Table 1 below captures this in a color-coded format:  completed Rel-17 test methods and requirements in green, the latest snapshot of Rel-18 work scope in orange, and potential scope of Rel-19 work in red.  This table is provided for information just to visualize how the XR related objectives fit into the potential OTA work scope overall.

Table 1: Status of TRP/TRS objectives by release

| Aspect / Feature |  | TRS |  |  |  | 1Tx TRP PC3 |  |  |  | 1Tx TRP PC2 |  | 2Tx Rad Power |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Browse | Talk | RedCap | XR | Browse | Talk | RedCap | XR | Browse | Talk | Browse | Talk |
| Test method and prelim MU |  | R17 | R17 | R18 | R19 | R17 | R17 | R18 | R19 | R17 | R17 | R18 | R18 |
| n1 | 72 < w ≤ 92 mm | R18 | R18 |  |  | R18 | R18 |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |
| n3 | 72 < w ≤ 92 mm |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |
| n5 | 72 < w ≤ 92 mm | R19 | R19 |  |  | R19 | R19 |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 72 < w ≤ 92 mm | R19 | R19 |  |  | R19 | R19 |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |
| n8 | 72 < w ≤ 92 mm |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 72 < w ≤ 92 mm | R18 | R18 |  |  | R18 | R18 |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |
| n41 | 72 < w ≤ 92 mm | R17 | R18 | R19 | R19 | R18 | R18 | R19 | R19 | R17 | R18 | R19 | R19 |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 72 < w ≤ 92 mm |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |
| n78 | 72 < w ≤ 92 mm | R17 | R18 | R19 | R19 | R18 | R18 | R19 | R19 | R17 | R18 | R19 | R19 |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |
| n79 | 72 < w ≤ 92 mm |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |

The potential OTA work can be classified into three categories:  XR related aspects (test method and requirements), more bands for handheld TRP/TRS (no test methodology impact), and the first set of bands for RedCap devices.

Observation 4:	As RAN considers how to handle the proposed 2Rx relaxation for XR UEs, it is useful to visualize the handling of XR OTA requirements in terms of RAN4 work scope and potential release intercept.

# Conclusions

In this contribution we have proposed an approach to resolve operator concerns associated with the 2Rx relaxation for the new XR device type via OTA requirements and described how this work can fit into the overall OTA scope of work in RAN4.  The following observations and proposals have been made:

Observation 1:	Operators voiced concerns with the coverage of a 2Rx XR device potentially impacting network performance.

Observation 2:	In field operation, it is the radiated performance of devices that impacts network RF planning and performance.

Observation 3:	The potential to unify XR radiated requirements can alleviate the networks RF planning for coverage.

Observation 4:	As RAN considers how to handle the proposed 2Rx relaxation for XR UEs, it is useful to visualize the handling of XR OTA requirements in terms of RAN4 work scope and potential release intercept.

Proposal 1:	To resolve operator concerns with the coverage of 2Rx XR devices, 3GPP should introduce OTA requirements for XR devices and study whether it is feasible to unify the OTA requirements for all XR devices, regardless of whether such devices support 2Rx or 4Rx.

# 4	References

- RP-211158, "Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)," vivo, OPPO, CMCC, 3GPP RAN #92, June 2021

- RP-222152, "Status report for WI: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC); rapporteur: vivo," 3GPP RAN #97, September 2022

- RP-222669, “New WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies,” vivo, 3GPP RAN #97, September 2022

- RP-223112, “Revised WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC),” vivo, 3GPP RAN #98, December 2022

- RP-223502, “New WID on XR Enhancements for NR,” Nokia, Qualcomm, 3GPP RAN# 98, December 2022

- RP-223208, “Way forward on the support of 2Rx for XR device for Rel-18 XR WI,” Apple, ABS, APT, ASR, CATT, FGI, Google, HONOR, Lenovo, Meta, New H3C, Qualcomm, Samsung, Tencent, Verizon, vivo, Xiaomi, KT Corp., Microsoft and Spark New Zealand, 3GPP RAN #98, December 2022

- RP-230052, “Meeting Report for electronic meeting TSG RAN meeting #98-e,” 3GPP RAN #98

- RP-230740, “Summary and Way Forward on XR Awareness and 2Rx (offline drafting),” Nokia, 3GPP RAN #99, March 2023