3GPP TSG-RAN WG1 #112bis-e	R1-2304157

# e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0474 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Type1 HARQ-ACK CB issue with more than one PDSCH per slot |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Qualcomm), Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_newRAT_Core |  |  |  |  |  | Date: |  |  | 2023-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For Type 1 HARQ-ACK codebook generation in 38.213 clause 9.1.2.1, UE determines a condition as to whether it is capable of receiving more than one unicast PDSCH or multicast PDSCH per slot. This condition impacts the codebook generation procedures as well as the codebook size. However, there are multiple such UE capabilities that can indicate UE is capable of receiving more than one PDSCH per slot, and the granularity of these UE capabilities is per FS. This may lead to ambiguity between UE and gNB with respect to Type 1 HARQ-ACK codebook generation. Additionally, this condition is not under gNB’s control, which means that even when gNB does not intend to transmit more than one PDSCH per slot, the codebook size may become unnecessarily large. To address these issues, a new RRC parameter is introduced, which controls the above-mentioned condition for Type 1 HARQ-ACK codebook generation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Generating Type 1 HARQ-ACK codebook as a function of a new RRC parameter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing Type 1 HARQ-ACK codebook generation depnds on whether UE is capable of receiving more than one unicast PDSCH or multicast PDSCH per slot or not, which can lead to ambiguity due to multiple such UE capabilities and the granularity of such UE capabilities. Furthermore, without the CR, the size of Type 1 HARQ-ACK codebook is not under gNB’s control, and hence, may be unnecesarily large. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<unchanged text omitted>

if the UE is not provided with multiPDSCHperSlot-Type1CB and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with multiPDSCHperSlot-Type1CB = ‘disabled’ and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

<unchanged text omitted>

end if

<unchanged text omitted>

if the UE is not provided with multiPDSCHperSlot-Type1CB and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with multiPDSCHperSlot-Type1CB = ‘disabled’ and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

<unchanged text omitted>

end if

<unchanged text omitted>