3GPP TSG RAN meeting #100										RP-231302

Taipei, June 12-16, 2023

## Status Report to TSG

Agenda item:			10.3.5

| WI / SI Name | Introduction of the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) for IoT NTN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_extLband |  |  |  |  |
| Unique ID | 991041 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-230790 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2023 | Performance part: 12/2023 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 10% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Luca Lodigiani |
|  | Company | Inmarsat |
|  | Email | Luca.lodigiani@inmarsat.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

## 2.2	RAN2

## 2.3	RAN3

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#106bis-e

- Agree on Band [253] as the band numbering.

- Consider if Option 1 can be agreed as a starting point, pending further checking of the frequency range.

  - Option1: Agree on Default TX-RX separation of -150 MHz

- Companies to further check ECC Report 263 and evaluate, based on current blocking requirements for NB1, NB2 and M1, whether additional blocking requirements need to be considered.

- For the Extended L-band, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Table 2.5-1 and 2.5-2, respectively (ZTE)
 
Table 2.5-1: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| [253] |  |  | 23 | +/-2 | [20] | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table 2.5-2: UE Power Class for category NB1 and NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| [253] | 23 | +/-2 | [20] | +/-2 |

- Reuse existing MPR requirements from 36.102 at least as a starting point for M1 and NB1/NB2. Different MPR tables for category M1 and NB1/NB2 separately

- For the Extended L-band, use UE channel bandwidth and transmission bandwidth configuration for eMTC NTN UL operation defined in Table 2.2-1 below.

Table 2.2-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 |
| --- | --- |
| Transmission bandwidth configuration NRB | 6 |

- Further discuss whether A-MPR requirements are needed

- Further discuss spurious emissions requirements.

- For the Extended L-band, use UE channel bandwidth and transmission bandwidth configuration for NB-IoT NTN UL operation defined in Table 2.2-2 below as a starting point.

Table 2.2-2: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

- Consider in-band blocking requirements from 36.102 as a starting point for further discussion, pending clarifications on system parameters.

- Further discuss out-of-band blocking requirements.

- Draft running CR for TS 36.102: Postpone discussion of CRs to later meetings when agreement is reached on system parameters and general requirements

- For the Extended L-band, use SAN channel bandwidth and transmission bandwidth configuration for eMTC NTN operation defined in Table 2.2-1 below as a starting point, and further discuss to align with UE.

Table 2.2-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 |
| --- | --- |
| Transmission bandwidth configuration NRB | 6 |

- For the Extended L-band, use SAN channel bandwidth and transmission bandwidth configuration for NB-IoT NTN operation should defined in Table 2.2-2 as a starting point and further discuss to make sure it is aligned with agreements on the UE side.

Table 2.2-2: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

- Draft running CR for TS 36.108:  Postpone discussion of CRs to later meetings when agreement is reached on system parameters and general requirements

RAN4#107

- Work Plan was Approved: R4-2309660 - Draft Work plan for IoT NTN Extended L-band

- For the extended L-band, the spurious emissions requirements of TS36.102 can be reused as a starting point.

- Agree on option 1: For the extended L-band, the UE co-existence emission requirements for band 255 can be reused as starting point.

  - FFS on how to capture ETSI requirements

#### Remaining Open issues

- Band definition: DL frequency range.

- General system parameters

- Majority of SAN RF requirements

- Majority of UE RF requirements.

- Alignment to common TR with other IoT NTN band (e.g. LSband).

The main open issue remains the DL frequency range.  The MSS Extended L-band band is recognized internationally at ITU-R, WRC and regional regulatory level, and moreover agreed in RAN#99, to have the following range:

- UL 1668-1675MHz (Earth-to-space), DL 1518-1525MHz (space-to-Earth)

Therefore, the following frequency table has been proposed:

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 253 | 1668 MHz | – | 1675 MHz | 1518 MHz | – | 1525 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

However, one company does not agree with the DL frequency range and would like to introduce a 3 MHz guard band.  The issue was discussed and left open in RAN4#106-bis-e.  During discussion in RAN4#107, a proposal was discussed on whether to define one single band with the full DL frequency range, or define two bands, one with the full DL frequency range, one with a reduced range including TBD guard band.

Agreement could not be reached.

The issue remains open and is blocking progress.

As a secondary open issue, the question remains on how to handle UE blocking requirements from ECC Report 263.

ACTION: Moderator respectfully asks RAN to clarify the issue of DL frequency range in order to unblock progress of the WI.

## 2.5	RAN5

## 2.6	RAN6

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN WG4#106bis-e meeting

- R4-2304492	Discussion on NTN FDD band (extended L band) UE RF requirements	Mediatek India Technology Pvt.

- R4-2304798	Discussion on UE RF requirements for the Extended L-band	ZTE Corporation

- R4-2304799	Draft CR to TS36.102 Introduction of the Extended L-band	ZTE Corporation

- R4-2304800	Discussion on SAN RF requirements for the Extended L-band	ZTE Corporation

- R4-2304801	Draft CR to TS36.108 Introduction of the Extended L-band	ZTE Corporation

- R4-2305825	Extended L band for LTE NB-IoT	Qualcomm Incorporated

- R4-2306198	Topic summary for [106-bis-e][118] IoT_NTN_extLband	Moderator (Inmarsat)

- R4-2306281	Topic summary for [106-bis-e][118] IoT_NTN_extLband	Moderator (Inmarsat)

- R4-2306658	WF on IoT NTN Extended L-band		Moderator (Inmarsat)

## 4.2	RAN WG4#107 meeting

- R4-2308590	Discussion on UE RF requirements for the Extended L-band	ZTE Corporation

- R4-2308591	Draft CR to TS36.102 Introduction of the Extended L-band	ZTE Corporation

- R4-2308592	Discussion on SAN RF requirements for the Extended L-band	ZTE Corporation

- R4-2308593	Draft CR to TS36.108 Introduction of the Extended L-band	ZTE Corporation

- R4-2309506	Guard band discussion for the extended L band	Qualcomm Incorporated

- R4-2309507	UE blocking requirements for the Extended L band	Qualcomm Incorporated

- R4-2309660	Draft Work plan for IoT NTN Extended L-band	Inmarsat

- R4-2309661	Clarification on IoT NTN Extended L-band Parameters	Inmarsat

- R4-2309721	IoT NTN Extended L-band UE requirements	Inmarsat

- R4-2310005	Topic summary for [107][122] IoT_NTN_extLband	Moderator (Inmarsat)

- R4-2310295	WF on IoT_NTN_extLband		Moderator (Inmarsat)