3GPP TSG-RAN Meeting #100	RP-231326

Taipei, June 12 – 15, 2023

Source:	Huawei, HiSilicon

Title:	Discussion on intraBand ENDC-support

Agenda item:			9.9

Document for:	Discussion and Decision

# 1. Introduction

In the contribution, we discuss the inconsistency issue of intra-band EN-DC configuration. The case3 and case4 have been confirmed to be valid in RAN4.

- Case 3: All CCs are contiguous in DL but neither carrier is contiguous to each other in UL, including

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |

- Case 4: One of LTE carriers and the NR carrier are contiguous in DL, contiguous and non-contiguous are both supported in UL:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |

# 2. Discussion

2.1. Progress in RAN2 and RAN4

The inconsistency issue of intra-band EN-DC configuration for band 48+n48 was raised for the first time in [1] RAN4#98-e, dating bake to January, 2021.

In RAN#97-e, September 2022, [2] was submitted to RAN plenary, and the inconsistency issue was concluded as RAN task to RAN2 and RAN4.

In RAN2#121, February 2023, RAN2 agreed to introduce the new signaling intraBandENDC-Support-UL for UL contiguity capability in intra-band ENDC configurations and included the solution in the LS [3] to RAN4. When the intraBandENDC-Support-UL is not included, the legacy capability (i.e. intrabandENDC-Support) indicates the capability for DL when the intra-band EN-DC is only supported in DL, or the common capability for both DL and UL when intra-band EN-DC is supported in DL and UL. When the intraBandENDC-Support-UL is included, the legacy capability indicates the DL capability. The capability intraBandENDC-Support-UL exists only when there is different contiguity capability between DL and UL. The RAN2 CRs are agreed in [4]-[9].

The signaling design in RAN2 for all possible cases is as follows.

Table 1 intra-band EN-DC scenarios indicated by intraBandENDC-Support/ intraBandENDC-Support-UL in [3]

| Scenario | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 1 | Absent (Contiguous) | Absent | - Contiguous/Contiguous |
| 2 | Absent (Contiguous) | Non-contiguous | - Contiguous/Non-contiguous NOTE: “Case 3” |
| 3 | Non-contiguous | Absent | - Non-contiguous/Non-contiguous |
| 4 | Both | Absent | - Contiguous/contiguous - Non-contiguous/Non-contiguous |
| 5 | Both | Non-contiguous | - Contiguous/Non-contiguous - Non-contiguous/Non-contiguous |
| 6 | Absent (Contiguous) | Both | - Contiguous/Contiguous - Contiguous/Non-contiguous NOTE: “Case 4” |
| 7 | Both | Both | - Contiguous/Contiguous - Non-contiguous/Non-contiguous - Contiguous/Non-contiguous |

In RAN4#107, most companies (8 sources) co-signed the CR to 38.101-3 [10] regarding the feasible solution introduced in RAN2. Only one company had different view for the CR, and provided alternative solution which would cause un-alignment in RAN2 and RAN4 specification at least for different releases, and further confusion.

Therefore, we discuss the feasibility of the solution that can be accepted by both RAN2 and RAN4 in this paper, and provide company CRs to RAN plenary. We attempt to complete the RAN task, and avoid leaving the issue as endless discussion that consumes much much RAN2 and RAN4’s time.

2.2. ‘Both’ indicated by intraBandENDC-Support

When the UE supports both contiguous and non-contiguous intra-band EN-DC for the same band entries, it reports ‘both’ by intraBandENDC-Support to save signalling overhead, as illustrated in Table 1 with scenario #4. RAN2 is not expecting other interpretations for ‘both’ apart from its own meaning, such as, different capability between DL and UL.

Furthermore, if ‘both’ is interpreted for the mixed contiguity capability for DL and UL, there would be multiple cases implied by such indication, which brings confusion for the network which case the UE really supports. In RAN2’s understanding, this is not backward compatible, and not a future-proof signaling design. Thus, the new signaling for UL capability was agreed to be introduced in RAN2.

Observation 1: UE reports ‘both’ by intraBandENDC-Support to indicate the support for both contiguous and non-contiguous intra-band EN-DC for the same band entries. Extension of the meaning on ‘both’ is not backward compatible or future-proof in RAN2.

In this case, taking 48C+n48A as an example, the when reporting ‘both’ with intraBandENDC-Support, the UE supports two pairs UL/DL configurations as shown in Figure 1:

- Config #1: Contiguous/contiguous           DL DC_(n)48CA, UL DC_(n)48AA

- Config #2: Non-contiguous/Non-contiguous   DL DC_48C_n48A, UL DC_48A_n48A

Figure 1 UE supports both C and NC intra-band EN-DC for the same band entries with ‘both’

And the network would not configure DL in Config#1 and UL in Config#2 as a pair to the UE.

Thus, Option 2c, as existing IE for Case 3 in [11] does not work.

- Option 2c: From Rel-16, UE shall also support non-contiguous operation in the DL (DC_48C_n48A or DC_48D_48A), then the network can configure DL_(n)48CA or DC_(n)48DA with the middle LTE cell DL-only and the UL with a gap (non-contiguous)

Observation 2: In RAN2’s understanding, if the UE reports ‘both’ with intraBandENDC-Support for 48C+n48A, it supports

- Config#1: DL DC_(n)48CA, UL DC_(n)48AA, and

- Config#2: DL DC_48C_n48A, UL DC_48A_n48A.

And the network would not configure DL in Config#1 and UL in Config#2 as a pair to the UE.

Observation 3: Existing IE is not applicable to support DL DC_(n)48CA, UL DC_48A_n48A, since the network would not configure DL in config#1 and UL in config#2 in Observation 2.

2.3. The support of Case 3 in RAN4 specification

Case 3 is supported as illustrated in Table 1 scenario #2 with ‘non-contiguous’ by intraBandENDC-Support-UL and absence of intraBandENDC-Support.

Figure 2 UE supports Case 3 with ‘non-contiguous’ indicated by intraBandENDC-Support-UL

To implement the special intra-band EN-DC configuration for CBRS band, and to implement the middle channel 48A to be turned off by SAS due to Tier-1 incumbent access users or higher interference, for the UE supports Case 3, it should additionally support DC_48A_n48A at least for DL configuration.

Proposal 1: For the UE supports Case 3, e.g., DL DC_(n)48CA/ DC_(n)48DA and UL DC_48A_n48A, it should additionally support DC_48A_n48A at least for DL configuration in RAN4 specification, to implement the special intra-band EN-DC configuration for CBRS band.

2.4. The support of Case 4 in RAN2 and RAN4 specification

Case 4 is supported as illustrated in Table 1 scenario #6 with ‘both’ by intraBandENDC-Support-UL and absence of intraBandENDC-Support.

In RAN2’s perspective, the capability signaling (i.e. intrabandENDC-Support/intrabandENDC-Support-UL) is intended to indicate the contiguity for adjacent LTE carrier and NR carrier. As illustrated in Figure 3 and Figure 4, in current capability signaling structure, for intra-RAT intra-band CA, the non-contiguous carriers can be indicated implicitly by multiple band entries.

Observation 4: RAN2 has confirmed that the contiguity of intra-band EN-DC depends on the contiguity of adjacent LTE carrier and NR carrier no matter whether PCC or SCC.

Figure 3 UE supports DL DC_48A_(n)48AA, UL DC_(n)48AA as intra-band contiguous EN-DC configuration

Figure 4 UE supports DL DC_48A_(n)48AA, DC_48A_n48A with ‘non-contiguous’ indicated by intraBandENDC-Support-UL

In scenario#6, there are two pairs of UL/DL configuration for EN-DC band combination DC_48A_(n)48AA.

- Config#1: Contiguous/contiguous       DL DC_48A_(n)48AA, UL DC_(n)48AA.

- Config#2: Contiguous/Non-contiguous   DL DC_48A_(n)48AA, UL DC_48A_n48A.

For the configuration#1 in DC_48A_(n)48AA, it can be covered by the legacy signalling intraBandENDC-Support given that the contiguity of UL and DL is consistent in contiguous same as scenario#1.

Observation 5: DL DC_48A_(n)48AA, UL DC_(n)48AA is a contiguous EN-DC configuration according to RAN2 LS.

Proposal 2: The configuration of DL DC_48A_(n)48AA, UL DC_(n)48AA should be moved to the tables defined for intra-band contiguous EN-DC configuration Table 5.3B.1.2-1 and Table 5.5B.2-1 in 38.101-3.

For the configuration#2, the contiguity for UL and DL is same as that in Case 3, so is the signalling indication in scenario#2. For the UE supports DL DC_48A_(n)48AA, UL DC_48A_n48A, it should additionally support DC_48A_n48A at least for DL configuration in RAN4 specification, to implement the special intra-band EN-DC configuration for CBRS band.

Proposal 3: For the UE supports DL DC_48A_(n)48AA, UL DC_48A_n48A, it should additionally support DC_48A_n48A at least for DL configuration in RAN4 specification, to implement the special intra-band EN-DC configuration for CBRS band.

Proposal 4: Agree the CRs to TS38.101-3 in [12][13][14] to align the same understanding of intraBandENDC-Support signalling between RAN4 and RAN2 specifications.

# 3. Conclusions

This contribution presents the following observations and proposals:

Observation 1: UE reports ‘both’ by intraBandENDC-Support to indicate the support for both contiguous and non-contiguous intra-band EN-DC for the same band entries. Extension of the meaning on ‘both’ is not backward compatible or future-proof in RAN2.

Observation 2: In RAN2’s understanding, if the UE reports ‘both’ with intraBandENDC-Support for 48C+n48A, it supports

- Config#1: DL DC_(n)48CA, UL DC_(n)48AA, and

- Config#2: DL DC_48C_n48A, UL DC_48A_n48A.

And the network would not configure DL in Config#1 and UL in Config#2 as a pair to the UE.

Observation 3: Existing IE is not applicable to support DL DC_(n)48CA, UL DC_48A_n48A, since the network would not configure DL in config#1 and UL in config#2 in Observation 2.

Observation 4: RAN2 has confirmed that the contiguity of intra-band EN-DC depends on the contiguity of adjacent LTE carrier and NR carrier no matter whether PCC or SCC.

Observation 5: DL DC_48A_(n)48AA, UL DC_(n)48AA is a contiguous EN-DC configuration according to RAN2 LS.

Proposal 1: For the UE supports Case 3, e.g., DL DC_(n)48CA/ DC_(n)48DA and UL DC_48A_n48A, it should additionally support DC_48A_n48A at least for DL configuration in RAN4 specification, to implement the special intra-band EN-DC configuration for CBRS band.

Proposal 2: The configuration of DL DC_48A_(n)48AA, UL DC_(n)48AA should be moved to the tables defined for intra-band contiguous EN-DC configuration Table 5.3B.1.2-1 and Table 5.5B.2-1 in 38.101-3.

Proposal 3: For the UE supports DL DC_48A_(n)48AA, UL DC_48A_n48A, it should additionally support DC_48A_n48A at least for DL configuration in RAN4 specification, to implement the special intra-band EN-DC configuration for CBRS band.

Proposal 4: Agree the CRs to TS38.101-3 in [12][13][14] to align the same understanding of intraBandENDC-Support signalling between RAN4 and RAN2 specifications.

# 4. Reference

[1]   R4-2102628, On UE capability for intra-band ENDC, Huawei, Hisilicon, TSG RAN4#98-e.

[2]   RP-222646, Discussion on intra-band EN-DC combination, Huawei, Hisilicon, TSG RAN#97-e.

[3]   R2- 2304431, “Reply LS on intraBandENDC-Support”, TSG RAN2 Meeting #121bis-e.

[4]	R2-2306507	Introduction of intra-band EN-DC contiguous capability for UL	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Intel Corporation, ZTE Corporation, Sanechips, MediaTek inc	CR	Rel-15	38.306	15.20.0	0927	-	B	TEI17, NR_newRAT-Core

[5]	R2-2306508	Introduction of intra-band EN-DC contiguous capability for UL	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Intel Corporation, ZTE Corporation, Sanechips, MediaTek inc	CR	Rel-16	38.306	16.12.0	0928	-	A	TEI17, NR_newRAT-Core

[6]	R2-2306509 Introduction of intra-band EN-DC contiguous capability for UL	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Intel Corporation, ZTE Corporation, Sanechips, MediaTek inc	CR	Rel-17	38.306	17.4.0	0929	-	A	TEI17, NR_newRAT-Core

[7]	R2-2306885	Introduction of intra-band EN-DC contiguous capability for UL	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Intel Corporation, ZTE Corporation, Sanechips, MediaTek inc	CR	Rel-15	38.331	15.21.0	4156	1	B	TEI17, NR_newRAT-Core

[8]	R2-2306886	Introduction of intra-band EN-DC contiguous capability for UL	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Intel Corporation, ZTE Corporation, Sanechips, MediaTek inc	CR	Rel-16	38.331	16.12.0	4157	1	A	TEI17, NR_newRAT-Core

[9]	R2-2306921	Introduction of intra-band EN-DC contiguous capability for UL	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Intel Corporation, ZTE Corporation, Sanechips, MediaTek inc	CR	Rel-17	38.331	17.4.0	4158	2	A	TEI17, NR_newRAT-Core

[10]  R4-2310493, “CR to 38.101-3 Correction to intra-band EN-DC configuration”, Huawei, Hisilicon, Nokia, Nokia Shanghai Bell, Xiaomi, OPPO, MediaTek Inc., ZTE Corporation, TSG RAN4 Meeting #107.

[11]  R4-2303635, “WF on inconsistency issue for intra-band EN-DC band combinations” TSG RAN4 Meeting #106.

[12]  RP-231327, “CR to 38.101-3 Correction on intra-band EN-DC configuration (R16)”, TSG RAN Meeting#100.

[13]  RP-231328, “CR to 38.101-3 Correction on intra-band EN-DC configuration (R17)”, TSG RAN Meeting#100.

[14]  RP-231329, “CR to 38.101-3 Correction on intra-band EN-DC configuration (R18)”, TSG RAN Meeting#100.