3GPP TSG RAN Meeting #100                                             RP-231399

Taipei, June 12-14, 2023

Agenda item:     9.2.2

Source:          ZTE Corporation, Sanechips

Title:             Further discussion on deployment scenarios of Ambient IoT

Document for:    Discussion and Decision

Introduction

In RAN#97e meeting, a new SID “Study on Ambient IoT” has been agreed and further revised in RAN#98e meeting [1].

In RAN#98e, the following agreements were reached:

Agreement: - ‘Environment of device’ can be ‘indoor’, ‘outdoor’, ‘indoor or outdoor’. Agreement: ‘Basestation characteristic’ can be: macro-cell based deployment, micro-cell based deployment, pico-cell base deployment, or none. - Companies are encouraged to discuss if there are additional necessary details of these descriptions in following meetings. Agreement: The study considers Ambient IoT deployment in-band to NR, in guard-band of NR, and standalone band from NR, and FFS: relationship to deployment scenarios. - Note: Prioritization among them can be discussed in later meetings. Agreement: For basestation deployments (when present), “Coexistence with existing 3GPP technologies” can be: - Deployed on the same sites as an existing 3GPP deployment corresponding to the basestation type - Deployed on new sites without an assumption of an existing 3GPP deployment Agreement: - Topology (1): BS <-> Ambient IoT device - NOTE 1: Includes the possibility of BS Rx and BS Tx in different BSs - Topology (2): BS <-> intermediate node <-> Ambient IoT device - NOTE 1: Intermediate node can be relay, IAB, UE, repeater, etc. which is capable of ambient IoT - Topology (3): BS <-> assisting node <-> Ambient IoT device <-> BS - NOTE 1: Assisting node can be relay, IAB, UE, repeater, etc. which is capable of ambient IoT - FFS: If the two BS can be different - Topology (4): UE <-> Ambient IoT device - FFS: Topology (5) UE <-> Ambient IoT device <-> {BS or UE} NOTE: For potential topology (5), discuss its relation with other topologies, its necessity, etc. in RAN#99. NOTE for all topologies: The Ambient IoT device may be provided with carrier wave from another node(s) either inside or outside the topology NOTE for all topologies: The links in each topology may be bidirectional or unidirectional FFS: Whether to consider combination of different topologies in the study. FFS: BS, UE, or assisting node could be multiple BSs, UEs or assisting nodes, respectively. Agreement: Spectrum in a deployment scenario is: licensed FDD, licensed TDD, unlicensed. - Note: Further discuss if the study should apply any limitations to the cases for which unlicensed spectrum is studied.

In RAN#99, the following proposals are endorsed for the deployment scenarios and connectivity topology aspects and incorporated in the pCRs. Furthermore, some details are discussed and included for the deployment scenario tables:

- Add device characteristics into the deployment scenario table, with possible values of ‘Device A’, ‘Device B’, ‘Device C’. - All rows in the deployment scenario tables can be indicated with more than one value. - Do not include Topology (5) in the study. - Note: it is understood this topology can be realized via combinations of other topologies

As there are still some unfilled parts for some deployment scenario table, in this contribution, we will further discuss some factors for deployment scenarios of Ambient IoT and give our proposals for the rows in the incomplete deployment scenario tables.

Discussion

2.1 Connectivity Topology

In general, the discussion in RAN should start with a generic design for Uu interface considering the limited TUs. Based on the study till now, for Topology (1), a new Uu design between BS and Ambient IoT device will be needed. We assume it also can be further applied to other Topologies, e.g., Topology (2), Topology (3) etc. For Topology (2), it not only requires a new Uu design between intermediate node and Ambient IoT device, but also requires additional interface between BS and intermediate node to support the relay communication. This relay function design can also be applied to other Topologies, e.g., Topologies (3). Therefore, we think the discussion on Topology (1) and Topology (2) is kind of basic and should be prioritized.

Proposal 1: It’s suggested RAN to prioritize the study of Topology (1) and Topology (2).

2.2 Basestation characteristics

According to the discussion in previous meetings, Ambient IoT can be co-deployed with the existing 3GPP infrastructure and therefore, the relevant basestation characteristics can be described according to the coverage range of macro BS, micro BS and pico BS.

- Macro-based cell: Due to a wide area coverage, macro-based cell would be the suitable option for the Ambient IoT use cases with outdoor environment, e.g., farms, pastures or streets, etc. Moreover, as the Ambient IoT device may be far away from the macro BS in some use cases, it’s possible that only the active device and semi-passive device can be directly connected to the macro BS. Meanwhile, the intermediate node or assisting node may be needed for passive devices.

- Micro-based cell: Micro-based cell with a medium coverage can be applied to the Ambient IoT use cases with outdoor or large indoor environment, e.g., airport, hospital or shopping mall, etc. Considering the medium communication range, it’s still possible that the active and semi-passive devices can be directly connected to micro BS and the passive device is connected to the network through intermediate node or assisting node.

- Pico-based cell: The coverage range of pico BS is from 10m to 50m. So, some Ambient IoT use cases with relatively small indoor environment (e.g., office building, library or cinema, etc.) are suitable for being served with pico-based cell, where one or several sites may be sufficient to cover the service area. If the inter-site distance of pico cell is less than 30m, all of the Ambient IoT device types can be directly connected to pico BS. For the Pico-based cell with large ISD, the passive device may need intermediate node or assisting node to connect to the network.

Based on the above discussion, the details of BS characteristics are summarized in Table 1.

Table 1. BS characteristics for Ambient IoT

| BS Type | Coverage | Environment | Coexistence |
| --- | --- | --- | --- |
| Macro BS | > 200 m | Outdoor E.g., farms, pastures or streets, etc. | Inband, guardband and standalone to existing 3GPP sites |
| Micro BS | 50-200 m | Outdoor and indoor E.g., airport, hospital or shopping mall, etc. | Inband, guardband and standalone to existing 3GPP sites |
| Pico BS | 10-50 m | Indoor E.g., office building, library or cinema, etc. | Inband, guardband and standalone to existing 3GPP sites Standalone as new sites |

Proposal 2: The following basestation characteristics can be assumed when applying them to the Ambient IoT deployment scenarios:

- Macro BS: Coverage range: > 200m; Applicable environment: outdoor. Inband, guardband and standalone to existing 3GPP sites

- Micro BS: Coverage range: 50 ~ 200m; Applicable environment: outdoor and indoor. Inband, guardband and standalone to existing 3GPP sites

- Pico BS: Coverage range: 10 ~ 50m; Applicable environment: indoor. Inband, guardband and standalone to existing 3GPP sites or Standalone as new sites.

In order to control deployment cost, Ambient IoT can be deployed in the shared macro, micro and pico sites with 3GPP and some new specific pico sites. Therefore, Ambient IoT should support inband, guardband and standalone deployments to existing 3GPP NR sites and standalone deployment as new pico sites in frequency domain. Wherein, the study on inband and standalone deployments should be prioritized.

Moreover, some additional coexistence impacts need to be studied for inband or guardband deployment compared to standalone deployment. For example, new waveform may need to be considered with the assumption of the coexistence with OFDM system for backscattering transmission.

Proposal 3: The study on Ambient IoT deployment inband to NR and standalone from NR should be prioritized in RAN SI.

Furthermore, if Ambient IoT is deployed in FR2, a signal energy attenuation will further reduce the communication range. As a result, denser deployment of base stations or nodes will be required, especially for passive or semi-passive devises, which will cause significant increase in the network deployment cost. So, we suggest to deprioritize the study of Ambient IoT in the FR2 band.

Proposal 4: It’s suggested RAN to deprioritize the study of Ambient IoT in the FR2 band.

2.3 To complete the deployment scenarios

In this section, we will discuss the deployment scenarios, especially the pending parts and give our suggestions.

For Deployment scenario 1 with Device indoors, basestation indoors, the characteristics have already been captured into TR38.848. However, considering that the recommended connectivity topologies have the involvement of basestation in this scenario, and interference caused by the use of the unlicensed spectrum may be noticeable due to that many Ambient IoT devices may perform transmission over relative short distances, we think there is no need to use the unlicensed spectrum. Hence, we suggest to exclude unlicensed spectrum from Deployment scenario 1.

Hence, the characteristics of deployment scenario 1 in Table 4.2.2.1-1 in TR 38.848 can be modified as the Table 2 below.

Table 2: Characteristics of deployment scenario 1

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico-cell |
|  | Connectivity topology | Topology (1), (2), (3) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device A or Device B or Device C |

For Deployment scenario 2 with Device indoors, basestation outdoors, the signal strength from BS is reduced due to the building penetration loss. According to our analysis in [2], the communication distance of Topology (1) and (3) is limited and not suitable for this scenario, hence, Topology (2) is preferred. For Topology (2), BS could communicate with Device B or Device C, and also Device A with the help of an intermediate node, e.g., repeater or UE. Moreover, with the similar reason as that for deployment scenario 1, it is also suggested that the unlicensed spectrum isn’t utilized in this BS-dominated scenario.

Hence, the characteristics of deployment scenario 2 in Table 4.2.2.1-2 in TR 38.848 can be filled as the Table 3 below.

Table 3: Characteristics of deployment scenario 2

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Macro- or Micro-cell |
|  | Connectivity topology | Topology (2) |
|  | Spectrum | Licensed FDD, licensed TDD, |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device A or Device B or Device C |

For Deployment scenario 3 with Device indoors, UE-based reader, this scenario is out of the control of BS. In this scenario, only Topology (4) is suitable to be applied. For Topology (4), UE could communicate with Device A or Device B or Device C directly as it’s assumed the communication range is short. And such UE should have the capability of communication via the new Uu interface for Ambient IoT, hence, new UE type would be required. Moreover, if UE indoor communicates with AIoT devices indoor by using the licensed spectrum, due to the absent of the resource coordination from BS, it may cause the interference to the surrounding UEs. Hence, the unlicensed spectrum would be more suitable in this scenario 3.

Hence, the characteristics of deployment scenario 3 in Table 4.2.2.1-3 in TR 38.848 can be filled as the Table 4 below.

Table 4: Characteristics of deployment scenario 3

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | unlicensed spectrum |
|  | Coexistence with existing 3GPP technologies | new UE |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device A or Device B or Device C |

For Deployment scenario 4 with Device outdoors, basestation outdoor, the whole communication environment is outdoor. If there is no obstacle, according to the analysis in [2], the basestation could provide sufficient coverage. Hence, Topology (1) is preferred. Meanwhile, as generally the required communication range would be large, Device B or Device C is more suitable to be used in Topology (1). However, if the AIoT devices are blocked from the coverage of gNB, gNB couldn’t communicate with AIoT devices directly. In this case, if some intermediate nodes play as relay, Topology (2) is also applicable and Device A or Device B or Device C can be used. If the AIoT devices are in the coverage of gNB, but the UL/DL communication distance is unbalanced, with the help of assisting node, e.g., UE, the communication distance in a certain direction can also be extended. And then Topology (3) is also applicable and Device A or Device B or Device C can be used.

Moreover, with the similar reason as above, it is suggested that the unlicensed spectrum isn’t utilized in this scenario.

Hence, the characteristics of deployment scenario 4 in Table 4.2.2.1-4 in TR 38.848 can be filled as the Table 5 below.

Table 5: Characteristics of deployment scenario 4

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| Outdoor inventory Outdoor sensor Outdoor positioning Outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro-cell |
|  | Connectivity topology | Topology (1), (2), (3) |
|  | Spectrum | Licensed FDD, licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device A or Device B or Device C |

For Deployment scenario 5 with Device outdoors, UE-based reader, this scenario is also out of the control of BS. So its characteristics can be similar as that of Deployment scenario 3.

Hence, the characteristics of deployment scenario 5 in Table 4.2.2.1-5 in TR 38.848 can be filled as the Table 6 below.

Table 6: Characteristics of deployment scenario 5

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| Outdoor inventory Outdoor sensor Outdoor positioning Outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | unlicensed Spectrum |
|  | Coexistence with existing 3GPP technologies | new UE |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device A or Device B or Device C |

Proposal 5: It is suggested RAN to capture the deployment scenarios in Table 2 ~ Table 6 in TR 38.848.

Conclusions

In this contribution, we give some further analysis on deployment scenarios of Ambient IoT. Based on the discussion, the following proposals are given:

Proposal 1: It’s suggested RAN to prioritize the study of Topology (1) and Topology (2).

Proposal 2: The following basestation characteristics can be assumed when applying them to the Ambient IoT deployment scenarios:

- Macro BS: Coverage range: > 200m; Applicable environment: outdoor. Inband, guardband and standalone to existing 3GPP sites

- Micro BS: Coverage range: 50 ~ 200m; Applicable environment: outdoor and indoor. Inband, guardband and standalone to existing 3GPP sites

- Pico BS: Coverage range: 10 ~ 50m; Applicable environment: indoor. Inband, guardband and standalone to existing 3GPP sites or Standalone as new sites.

Proposal 3: The study on Ambient IoT deployment inband to NR and standalone from NR should be prioritized in RAN SI.

Proposal 4: It’s suggested RAN to deprioritize the study of Ambient IoT in the FR2 band.

Proposal 5: It is suggested RAN to capture the deployment scenarios in Table 2 ~ Table 6 in TR 38.848.

References

- RP-223396, SID revision: Study on Ambient IoT (Internet of Things) in RAN, RAN#98e, December, 2022

- RP-231398, The feasibility analysis of Ambient IoT, RAN#100, June, 2023