3GPP TSG RAN Meeting #100		   RP-231474

Taipei, June 12-14, 2023

Agenda Item:	9.3.2.1

Source: 	MediaTek Inc.

Title:  	Report of Rel-18 Mobility WI Offline Discussion

Document for:	Discussion and decision

# Introduction

This is the report of offline discussion on potential revision for Rel-18 Mobility WID.

# Discussion

Objective 1

Issue #1.1: Deprioritize / preclude “reference configuration”

(Ericsson, RP-231110) For LTM objective (Objective 1): Use of reference and delta configuration is down-prioritized (or alternatively, down-scoped and removed from the WI scope).

(MediaTek, RP-231312) Revise Objective 1 to preclude ‘reference configuration’ in LTM. Also mention that a candidate configuration should be either a complete configuration, or a delta configuration that are applied on top of current RRC configuration upon cell switch.

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM |  | If down-scope, only complete config |
| Nokia |  | No ready to down-scope |
| Apple |  | Too early to down-scope |
| KDDI |  | No down-scope |
| Ericsson |  | (Proponent) |

Down-scope: Ericsson, MTK, Fujitsu, QCOM, NEC, Docomo, Verizon

Keep it: Nokia, Apple, KDDI, Intel, IDT, LG, Samsung

RAN2 Chair: Decision made too early in RAN2. Ref config is signaling optimization.

=> Continue discussion in WG

Proposal 1: For LTM reference and delta configuration, continue discussion in WG.

Issue #1.2: Remove NR-DC support for LTM

(Ericsson, RP-231110) Remove support for NR-DC from Objective 1.

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG |  | Need clarification (If UE in NR-DC, can LTM be applied only for SCG or both?) |
| IDT |  | The issue is to have LTM in only one CG or can be on both CGs |
| Ericsson |  | Intention: LTM only for MCG |
| NEC |  | Support |
| Nokia |  | Support LTM on PCell only. Concern on RAN4 workload |
| QCOM |  | FR2 for SCG, maybe limit to SCG |
| Apple |  | OK to limit to PCell |
| TMO |  | Rel-19? |

RAN2 Chair: NR-DC support is for higher freq

=> Prioritize MCG for Rel-18 LTM (for all RAN WGs)

Proposal 2:	Revise Objective#1 to prioritize MCG for Rel-18 LTM (for all RAN WGs)

Issue #1.3: Deprioritize CSI-RS L1 measurement

(Ericsson, RP-231110) CSI-RS L1 measurements. With only one RAN1 meeting left, RAN1 has not yet agreed a Rel-18 solution for CSI-RS measurements for LTM. SSB measurements should be enough in Rel-18.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Fujitsu |  | Dropping CSI-RS will help RAN1 progress. |

=> For LTM L1 measurements, only SSB measurement is supported in Rel-18.

Proposal 3:	Revise Objective#1 to state that for LTM L1 measurements, only SSB measurement is supported in Rel-18.

Objective 2

Issue #2.1: Deprioritize / preclude “reference configuration”

(Ericsson, RP-231110) Remove the following from Objective 2: Use of reference and delta configuration, inter-SN scenarios, MN-initiated PSCell changes and support for CPA.

(MediaTek, RP-231312) Revise Objective 2 to preclude ‘reference configuration’ for selective activation of SCG, and mention that a candidate configuration should be either a complete configuration, or a delta configuration that are applied on top of current RRC configuration upon cell switch.

| Company | Yes/No | Comment |
| --- | --- | --- |

=> Continue discussion in WG

Proposal 4: For reference and delta configuration in SCG selective activation, continue discussion in WGs.

Issue #2.2: Limit the scope to SN-initiated, intra-SN

(Ericsson, RP-231110) Remove the following from Objective 2: Use of reference and delta configuration, inter-SN scenarios, MN-initiated PSCell changes and support for CPA.

(MediaTek, RP-231312) Revise Objective 2 to limit the applicable scenario to SN-initiated, intra-SN selective activation of SCG.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Nokia | No | compromise: Down-prioritize MN-init PSCell change, keep inter-SN and intra-SN |
| Apple |  | share Nokia’s view |
| LG |  | Prioritize MN-init |

=> Continue discussion in WGs

Issue #2.3: Remove CPA support

(Ericsson, RP-231110) Remove the following from Objective 2: Use of reference and delta configuration, inter-SN scenarios, MN-initiated PSCell changes and support for CPA.

| Company | Yes/No | Comment |
| --- | --- | --- |

=> Continue discussion in WGs

Proposal 5:	For the supported scenarios of SCG selective activation, continue discussion in WGs.

Objective 5

Issue #5.1: Prioritize intra-frequency and PCell-SCell role-change scenarios for LTM

(MediaTek, RP-231312) Revise Objective 5 to mention that when specifying RRM core requirements for L1 measurements in LTM, we prioritize intra-frequency and PCell-SCell role-change scenarios.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Fujitsu |  | RAN1 already has agreement on inter-freq |
| Apple |  | Can wait for RAN4 |
| QCOM |  | For measurement or mobility? |
| MTK |  | It’s for measurement (mainly gap). Inter-freq LTM can be triggered based on L3 meas |
| TMO |  | Inter-freq is important use case for LTM |
| Verizon |  | Inter-freq is important use case for LTM |

RAN2 Chair: No need to discuss role-change case (it’s intra-freq meas)

=> Continue RAN4 work

Proposal 6: Continue RAN4 works on RRM requirements for LTM.

Objective 7

Issue #7.1: Focus on solutions based on existing measurements

(Ericsson, RP-231110) For Objective 7, focus on to specify a solution for the measurement validity issue based on existing measurements. Otherwise, the objective should be removed completely.

(MediaTek, RP-231312) Revise Objective 7 to prioritize solutions based on existing measurement.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Nokia |  | “existing measurements” means no enhancements |
| MediaTek |  | RAN4 situation: Diverged solutions for “additional measurements” |
| Intel |  | Need Plenary guidance |
| Ericsson |  | Due to RAN4 workload, it’s not feasible to add new measurements |

=> No conclusion offline, continue online.

Proposal 7:	Online discussion for Objective#7 is needed.

Note: Objective 5 and 6 will be updated according to our conclusion on down-scoping.

WID revision

Based on the proposals above, Objective 1 shall be revised to take into account Proposal 2 and 3. Objective 7 may also be revised based on the online discussion conclusion.

# Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1:	For LTM reference and delta configuration, continue discussion in WGs.

Proposal 2:	Revise Objective#1 to prioritize MCG for Rel-18 LTM (for all RAN WGs)

Proposal 3:	Revise Objective#1 to state that for LTM L1 measurements, only SSB measurement is supported in Rel-18.

Proposal 4:	For reference and delta configuration in SCG selective activation, continue discussion in WGs.

Proposal 5:	For the supported scenarios of SCG selective activation, continue discussion in WGs.

Proposal 6:	Continue RAN4 works on RRM requirements for LTM.

Proposal 7:	Online discussion for Objective#7 is needed.