3GPP TSG-RAN #101	 RP-231619

Bangalore, India, September 11-15, 2023

Title:		Ambient IoT and required RAN functionalities

Agenda Item:    		9.2.2

Source:	     	Nokia, Nokia Shanghai Bell

Document for:	Discussion and decision

# 1	Introduction

Ambient IoT (A-IoT) RAN led Study Item Rel-18 was approved in RAN#97e [1]. The motivation of Ambient IoT study is to support ultra-low cost and ultra-low power devices for the IoT applications. SA1 completed a study item on ambient power enabled IoT [2], which focuses on use cases and service requirements, and it includes the objective of defining performance requirements and KPIs as well.

Currently, there is on-going RAN study item on Ambient IoT (Internet of Things) in RAN [2], which focuses on 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study provides clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including reduced peak Tx power.

In terms of energy storage, the study will consider the following device characteristics:

- Battery less devices with no energy storage capability at all, and completely dependent on the availability of an external source of energy

- Devices with limited energy storage capability that do not need to be replaced or recharged manually.

Device categorization based on corresponding characteristics (e.g. energy source, energy storage capability, passive/active transmission, etc.) may be discussed during the study, in relation with the relevant use cases. The device’s peak power consumption shall be limited by its practical form factor for the intended use cases and shall consider its energy source.

The Ambient IoT devices support communication with reader via reflection or active transmission which can be supported by very low complexity hardware. The Ambient IoT device can collect energy from environment, such as RF signals, solar energy, vibration, heat, etc., and can potentially also be equipped with a small capacity battery. Hence, Ambient IoT is a promising technique to achieve even lower cost/power requirements compared to existing cellular IoT technologies, e.g. NB-IoT, LTE-M, RedCap, etc.

The RAN level study will compare and assess the feasibility of meeting the design targets for relevant use cases on the basis of the deployment scenario(s) appropriate to it and identify assumptions on required functionality to be supported.

An essential aspect for ambient IoT is the link budget limitations and the required UE and gNB grid to enable seamless discovery and communications with ambient IoT devices.

Note that in this document the Ambient IoT device is also referred to as A-IoT device, A-IoT tag or simply tag.

# 2	A-IoT Security

Security is a very important aspect of any wired and wireless communications system, and already recognized as one of the main open challenges for A-IoT. Similarly, to RFID systems, for A-IoT two main functionalities are needed:

- Authentication:

  - to confirm device identities i.e., ensuring that the tag responds only when it is queried by a legitimate activator.

  - to prevent tag cloning, i.e., against attackers that try to send data while claiming to be a different device.

- Confidentiality/privacy: to ensure protection of data against:

  - Eavesdropping. Eavesdropping occurs when a malicious device listens to the detection procedure and is able to get hold of the tag reply and decode its contents.

  - Jamming. The phenomenon occurs when a malicious device purposely interferes the NR NW communication with the tag, either by preventing the NR element to activate the tag, or by preventing the tag reading by the NR element itself.

NR is a very secure system and many algorithms and protocols have been introduced to guarantee not only authentication and confidentiality/privacy, but also other security functionalities like data integrity and availability. These protocols are implemented in different layers and locations, but mainly at PDCP and above layers, and rely on cryptography. The issue is that the approach used by NR that relies on cryptography cannot be directly applied to A-IoT devices, as these passive and semi-passive tags have such low power availability and processing power that even basic cryptographic algorithms are not applicable.

In sum, offering even a basic level of security for passive radio communication in NR is a challenging task for two main reasons:

- The tag is limited in terms of energy storage (in many cases such storage is entirely absent), therefore it cannot perform heavy cryptographic operations for securing the communication link.

- Spending energy on securing the communication reduces the link budget for passive devices and increases thus the risk that the passive radio is not heard by the reader.

- For UE based activation/reading, a further challenge is that UEs are terminal devices and not network devices and we need further enhancements in authentication protocols that allow both UE and tags authenticate each other for a short session time and a new reading session with a different UE activator may require repeating authentication procedure again.

Therefore, there is need to design novel solutions and protocols to address those security functionalities.

Proposal 1: RAN to study physical layer solutions for authentication for A-IoT and prevention of attacks.

# 3	Communication protocols aspects

## 3.1	Topology specific considerations

Considering the topology 3.a as depicted in Figure 1(a), and the associated stage-1 signaling flow, as depicted in Figure 1(b), it can be observed that the interaction between the gNB, Activator and A-IoT device is composed by the following steps:

- The gNB requests for the assistance device (i.e. the Activator) to activate the A-IoT device;

- The Activator sends the activation signal, which is a signal carrier suitable for being backscattered by the A-IoT device;

- The AIoT device replies to the gNB using a backscattered signal.

| (a) | (b) |
| --- | --- |

Figure : (a) Topology 3.a and the (b) associated stage-1 signaling flow description.

The challenge with this interaction is that for any of these steps to occur there is the need to:

- Discover AIoT devices in the coverage area

For AIoT device discovery, Activator can send an activation signal and wait for the response. The AIoT devices in its coverage can backscatter their identification and authentication information to the activator which could be used to discover AIoT devices in Activator coverage area and establish a communication session.

- Set activation signal power based on the proximity of AIoT devices

The activation signal must be transmitted with appropriate power in order for the reader to receive the backscattered signal while avoiding generating excessive interference.

- Allocate resources for the activation and reception of the backscattered signal.

One approach would be to have the gNB together with its request, to the assistance device to act as an activator, to also allocate the resources (i.e. the resource allocation would be provided in step 1 of Figure 1(b) that would allow the activation and backscattered signal to be transmitted. However, as the activator might not be in RRC Connected state it becomes necessary to decouple the request for assistance from the actual resource reservation for the activation/backscattered signal transmission. One potential approach would be to extend the RACH procedure, allowing an assisting device (i.e. the activator) and AIoT device to determine the set of resources to use for the activation/backscattering signal transmission and at the same indicate to the gNB when to expect the reply of the AIoT Device. The use of the RACH procedure would serve also as means to share the resource allocation and power control to the activator, AIoT device and reader (when the reader is not the gNB).

Proposal 2: RAN to consider the use of the RACH procedure as means to enable a A-IoT communication session.

For topology 4, depicted in Figure 2, it can be observed that the interaction between the UE taking the role of activator, the AIoT device and the UE taking the role of reader is composed by the following steps:

- The Activator sends the activation signal, which is a signal carrier suitable for being backscattered by the A-IoT device;

- The AIoT device replies to the Reader using a backscattered signal.

| (a) | (b) |
| --- | --- |

Figure : (a) Topology 4 and the (b) associated stage-1 signaling flow description.

In this case the challenge of this interaction is that for any of these steps to occur there is the need to:

- Select UEs taking the role of an activator/reader in a coverage area

There could be several UEs in a coverage area and not all of them can be selected for providing activation/reading service for AIoT devices. A selection mechanism is required to collect information on UEs’ suitability to perform this service without affecting their own service requirements with minimum overhead and interference.

- Discover which AIoT devices are covered by the selected UE activators/readers.

- If activator and readers are different UEs (bistatic) allocate resources for the activation and reception of the backscattered signal, i.e. a coordination between the UEs taking the role of activator and reader need to have a protocol in place that enable the coordination between themselves.

One potential approach would be to enable sidelink communications between these UEs and have the associated resource allocation being controlled by the network or by UEs taking the role of Activator and Reader themselves.

Proposal 3: RAN to consider the use of the sidelink communication as means to coordinate between UEs taking the role of activator and reader on a A-IoT communication session.

## 3.2	Session maintenance and handover aspects

An A-IoT system is in its simplest form composed by an AIoT activator an AIoT reader and an AIoT device. In topologies termed as monostatic, the AIoT activator and reader are the same network element (or device). This implies, for AIoT devices type A and B where the AIoT device reply is a backscattered transmission, that the network element (or device) taking the role of AIoT activator and reader is able to operate in a full duplex fashion being able to transmit an activation signal while simultaneously receiving the reply from the AIoT device. In contrast, for topologies termed as bistatic (or multi-static), the AIoT activator and the AIoT reader are two different network elements (or devices) and therefore there is no longer the need for full duplex operation in the AIoT activation and reader devices. Therefore, bistatic topologies, from a NW and non-AIoT supporting device point of view, are seen as less technically challenging and therefore will be most likely the ones to be adopted first.

The main challenge associated with bi-static topologies is that the activator and reader require coordination (e.g. at least in terms of agreement of the time and frequency resources for the transmission of the activation signal towards the AIoT device and the associated reception at the reader) and frequent monitoring if this coordination is still up to date. However, as A-IoT systems are inherently dynamic for the following reasons:

- In some applications the A-IoT device is mobile, since it is most often attached to items that need to be localized or tracked. For example, in many use cases, such as manufacturing, logistics, air and sea ports, and inventory of medical devices, need ‘on demand’ tracking of the objects to which A-IoT devices are attached. It would then be possible to track the movement of AIoT devices by exploiting the known indoor topology.

- In some applications, the AIoT device may not be mobile but activators and/or readers are mobile UEs, which may enter or leave backscattering service area.

- The A-IoT activator needs to be in the immediate proximity of the A-IoT device, so that the latter device receives sufficient energy to power on its corresponding transmission/reflection modulation. Given this limitation, the activator (which itself may be a moving UE) needs to closely follow the potentially mobile A-IoT device;

- The A-IoT reader may be yet another NR UE, therefore also mobile. For a successful reading, the reader should be no more than hundreds of meters away from the A-IoT device or closer dependent on the Activator distance;

- The monitoring of the link conditions of the reading session is particularly challenging in AIoT systems since the AIoT device itself cannot be expected to monitor any link parameters and be able to provide this information.

Given the above challenges, setting up a periodic reading session is extremely challenging, since the entity controlling the session needs to anticipate whether the activator and reader pairs remain valid (are still sufficiently close to the A-IoT device) from one reading to the next, and if not, trigger the reconfiguration (i.e., to hand over) of the future reading attempt so as to ensure the seamless reading. How to detect and hand-over the reading session to another activator/reader or activator-reader pair is particularly challenging in A-IoT systems since the A-IoT device itself cannot monitor any link parameters and trigger a handover, like a UE would do in legacy NR systems. Thus, the A-IoT device’s handover (to a new activator, a new reader, or a new activator-reader pair) must be realized without any explicit/direct help from the A-IoT device. Conversely, the handover task could be enabled by allowing the activators/readers to adapt their roles to the new AIoT topology.

Proposal 4: RAN to consider A-IoT session maintenance and handover procedures to allow the A-IoT session to be handed over to another activator and/or reader without the active involvement of the A-IoT device itself i.e., without requesting the A-IoT device to monitor/report its own link parameters.

## 3.3	Tag Deactivation and Activation

AIoT devices are deployed in the network to provide information on the uplink. As A-IoT devices are large in number and 3GPP is targeting a variety of use cases, some use cases may require temporarily deactivating A-IoT devices and reactivating them later. In certain scenarios, the application server would like to deactivate them for longer periods such as,

- No need to operate them in winter (a few weeks/months)

- No need to operate at a particular time of the day (a few minutes/hours)

With thousands of tags deployed, it is highly likely that illumination signal and queries will be of broadcast nature, and therefore even tags that are not required to respond will respond whenever they receive excitation signal, thereby causing unnecessary interference, network traffic, and system processing. RAN should be able to provide deactivation functionality to reduce unnecessary information flow from the tags when not needed and ability to reactivate tags when desired by the application.

Proposal 5: RAN to consider A-IoT procedures for temporary deactivation of tags when information is not required and be able to activate them later.

If not explicitly deactivated as described above an A-IoT device will activate when activation power is above a certain threshold. As such activation power level can be adjusted to only trigger activation of A-IoT devices within a certain distance from the activator. Assuming the A-IoT device has capability to trigger a response only once while the activation power is above a threshold or trigger a response only when received activation power is within a power window then the number of A-IoT devices responding can be mapped via activation power ramping and filtered by the activation power level, reducing the number of A-IoT devices responding and associated interference.

Proposal 6: RAN to consider adjustable Activator power level to filter the number of A-IoT devices responding.

## 3.4	AIoT Devices with multiple operating modes

In the Ambient IoT discussions so far three types of AIoT devices were identified:

- Device Type A: No energy storage, no independent signal generation, i.e. backscattering transmission.

- Device Type B: Has energy storage, no independent signal generation, i.e. backscattering transmission. Use of stored energy can potentially include amplification for reflected signals (to be confirmed feasibility).

- Device Type C: Has energy storage, has independent signal generation, i.e. active RF component for transmission.

The advantage of type C devices over type A and B devices, is the type C devices being able to perform active transmissions which can allow for the support of higher modulations orders and therefore higher throughput, while type A and B devices are restricted to only backscattered transmissions which normally have lower order modulations. However, an A-IoT device should be able to transition between A-IoT device types depending on the traffic needs and availability of energy. Additionally, the NW should be made aware of which type of AIoT it is handling and provide mechanisms that allow them to report their traffic and energy needs.

Proposal 7: RAN to consider that A-IoT devices can operate and transition between A-IoT device types.

# 4	Interference and Coexistence aspects

## 4.1	Spectrum

Discovering an A-IoT device is a challenging task due to the inherent nature of the passive radio and the variability of the wireless environment. More precisely, the A-IoT device:

- (Device A/B) does not have a power source, but it harvests energy from e.g., radio waves,

- can hear other radios only in its own proximity and

- can hear other radios only if said radios operate on a set of specific time-frequency resources.

Furthermore, the A-IoT device charge and payload are transparent to the NR NW.

The NR NW can thus discover the A-IoT device only if:

- the activator and reader are available (i.e., do not perform other tasks),

- the activator and reader are close enough to the A-IoT device,

- the activator and reader are using free resources to illuminate and respectively read the A-IoT device.

Solving 1-3 is not a trivial task since the NW must ensure both spatial (condition 1-2) and NR time-frequency (condition 3) resources are simultaneously available for the A-IoT device discovery. However, in most cases, the NR spectrum in the vicinity of an A-IoT device may be used for standard NR UE communication and/or NR UE positioning and may be thus unavailable to the activator/reader pairs for A-IoT discovery, even if the later entities are available.

Additionally, any device targeting operation in unlicensed bands in 2.4 GHz, 5GHz and 6GHz needs to complete a channel access procedure in order to coexist with other systems in unlicensed spectrum. This channel access procedure is typically Listen Before Talk and will be applicable to both activator and A-IoT device.

Proposal 8: RAN to consider that channel coexistence procedures associated with operation in licensed and unlicensed bands need to be considered during the A-IoT system design.

## 4.2	A-IoT activator interference

Furthermore, when the activator transmits its illumination signal, there is a risk that it will create interference in the cell and neighbor cells. To avoid degradation of the normal communication in the cell and neighbor cells, interference coordination and/or mitigation is needed. Take the following examples:

In a TDD system, the interference scenario from a UE A-IoT device illumination session will look like the picture in Figure 3 when illuminating the A-IoT device (tag) during the normal synchronized TDD timeslots.

Figure  Tag Illumination interference TDD synchronized, UE Illumination in UL slots

This interference scenario is similar to the normal communication as the UE transmits during the synchronized UL timeslots, the only difference is that there is no DL communication from the primary gNB. This would not be DL communications from the gNB in the normal case since the UE is illuminating in the UL slot. The gNB is as normal in receive mode and may thus potentially be part of the tag readers. Since the primary gNB is might not active during the tag illumination process, another option is to explore using the DL synchronized timeslots for the tag illumination. This changes the interference scenario as depicted in Figure 6.

Figure  Tag Illumination interference non-TDD synchronized, where the illuminating UE may also interfere with other receiving UEs in the same cell

Depending on the illumination power level and the location of active devices in own cell and neighbor cells the interference scenario will be very different.

Proposal 9: RAN to consider interference coordination scheme for activator signals and A-IoT device replies to co-exist with normal communication in the cell and neighbor cells for a deployment where A-IoT devices are activated using 3GPP NR spectrum.

## 4.3	A-IoT device interference

Reading of one or more A-IoT devices (tags) is associated with the following problem:

- Multiple A-IoT devices may reply on the same time-frequency resources, thus interfering with each other during their response. This type of interference is called A-IoT-to-A-IoT device interference and it affects the ability of the reader to distinguish between A-IoT devices which respond concurrently by using the same time-frequency resources.

- An A-IoT device typically reply/reflect on the same carrier as that used by the activation signal. Since the activation signal is several tens of dBs (up to 60 dB in LoS scenarios) stronger than the A-IoT device reply, the phenomenon called activator-to-A-IoT device interference occurs. In other words, the tag reply is drowned by the activation signal and the reader must be able to isolate the activation signal before attempting to detect the tag.

To prevent problem 2, methods to align the activation signal away from the tag signal at the reader may be considered. To that end,One  method is to make the reader would need to learn  the channel response between itself and the activator and thus the reader can use the learning to suppress the activation signal at the reader side; an alternative method is to defer transmission of the tag reply after the A-IoT device receives the activation signal, so as to avoid simultaneous reception of the activation signal and A-IoT device reply at the reader.

To remove the activator-to-A-IoT device interference in problem 2, the activator can be configured to repeat transmission of the activation signal within a transmission pattern and the A-IoT device can determine whether to mute the reply of the activation signal based on configuration in the pattern. Finally, the reader can. follow the pattern to remove the activator-to-A-IoT device interference from the receiving signal by differential operation.

Specifically for A-IoT device type C with active response transmission capability power control is needed to ensure proper A-IoT signal reception at the reader at lowest possible A-IoT device transmission power level to minimize A-IoT device interference and power consumption. The same applies to the A-IoT activator transmission power which needs to be set just adequate for activating the A-IoT device.

An alternative approach to overcome this issue is to deploy A-IoT devices designed to emit/reflect high levels of harmonic components (2nd order harmonic, 3rd order harmonic, etc), which can be used by reader to read responses from A-IoT devices. Such an approach will reduce the power level of the backscattered signal from the A-IoT device, but at the same time remove the activator-to-A-IoT device interference, which can result in a better SINR at the reader for some use cases.

Proposal 9: RAN to consider solutions for cross interference between A-IoT devices and between activators and A-IoT devices, including options such as muting an A-IoT device, grouping multiple devices, power control, orthogonal resource allocation in time, frequency or space, high level of harmonic back scattering.

# 5	Conclusion

This contribution proposes the following:

Proposal 1: RAN to study physical layer solutions for authentication for A-IoT and prevention of attacks.

Proposal 2: RAN to consider the use of the RACH procedure as means to enable a A-IoT communication session.

Proposal 3: RAN to consider the use of the sidelink communication as means to coordinate between UEs taking the role of activator and reader on a A-IoT communication session.

Proposal 4: RAN to consider A-IoT session maintenance and handover procedures to allow the A-IoT session to be handed over to another activator and/or reader without the active involvement of the A-IoT device itself i.e., without requesting the A-IoT device to monitor/report its own link parameters.

Proposal 5: RAN to consider A-IoT procedures for temporary deactivation of tags when information is not required and be able to activate them later.

Proposal 6: RAN to consider adjustable Activator power level to filter the number of A-IoT devices responding.

Proposal 7: RAN to consider that A-IoT devices can operate and transition between A-IoT device types.

Proposal 8: RAN to consider that channel coexistence procedures associated with operation in licensed and unlicensed bands need to be considered during the A-IoT system design.

Proposal 9: RAN to consider interference coordination scheme for activator signals and A-IoT device replies to co-exist with normal communication in the cell and neighbor cells for a deployment where A-IoT devices are activated using 3GPP NR spectrum.

# 6	References

- RP-222685, Study on Ambient IoT.

- S1-220192, New SID: Study on Ambient power-enabled Internet of Things.

- RP-223396 Study on Ambient IoT (Internet of Things) in RAN