3GPP TSG-RAN5 Meeting #100	R5-235668

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0984 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to FR2 RF test case 6.2.5 for EIRP with UL-Gaps |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Benelux B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | EIRP with UL-Gaps test is still ongoing development with several missing details |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating contents of EIRP with UL-Gaps test to update some references and other test config details |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete details in the test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 - updates references within the test procedure - updates the UL RMC table in Annex A.2.3 to align with core specifications, - updates test requirements sub-clause R5-235225r1 -> R5-235668 |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

### 6.2.5	UE Maximum Output Power – EIRP with UL Gaps

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The test case is incomplete for band n259.

-

-	Initial conditions are pending analysis for PC1, PC5, PC6 and PC7.

-	Impact of polarization on the measurements is still FFS.

-	MU and TT and structure of the Test Requirements Section are pending further analysis

6.2.5.1	Test purpose

The objective of this test is to determine the impact of UL-gaps on TX power management by measuring the EIRP with and without UL-Gaps configured.

6.2.5.2	Test applicability

This test case applies to all types of NR UEs release 17 and forward supporting ul-GapFR2-r17 and tdd-MPE-P-MPR-Reporting-r16

6.2.5.3	Minimum conformance requirements

The difference of the measured peak EIRP PUMAX,f,c_GAP_ON when UL gap for TX power management is configured and activated, and the measured peak EIRP PUMAX,f,c_GAP_OFF when UL gap is not configured or de-activated, shall meet the following requirement:

PUMAX,f,c_GAP_ON - PUMAX,f,c_GAP_OFF ≥max((EIRPmeas_peak – 23) + 10 * log10(Z/20), 3)dB

where EIRPmeas_peak  is the measured UE peak EIRP with zero MPR/A-MPR/P-MPR as specified in clause 6.2.1 for the corresponding power class, and Z% is duty cycle of the reference measurement channel. PUMAX,f,c_GAP_ON shall be measured outside of the UL gap symbol(s). The period of measurement shall be at least 4s. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle) and in the test Z is set to 20 when maxUplinkDutyCycle-FR2 is less than 20 or not reported, and should be larger than maxUplinkDutyCycle-FR2 when maxUplinkDutyCycle-FR2 is equal to or greater than 20. The reference measurement channel is specified in Annex A.2.3.

When UL gap for Tx power management is configured and activated, the reported P-MPRf,c shall be less than 3dB. When UL gap for Tx power management is not configured and activated, UE shall set the P bit in PHR to 1 in the test when PHR is configured.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.5.

NOTE 1:	As mentioned in 6.2.4.3 - for UE conformance testing P-MPRf,c shall be 0 dB, except for the testing of UL gap for Tx power management, where P-MPRf,c may be non-zero dB – which is relevant to this test case

6.2.5.4	Test description

6.2.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in Table 6.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.5.4.1-1: Test Configuration Table for power class 2,3 and 4

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid Range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, 100 MHz, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw (NOTE 2) | SCS | Downlink Configuration | Uplink Configuration |  |  |
| 1 | 100 | Default | - | Modulation |  | RB allocation (NOTE 1) |
|  |  |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3 |
|  |  |  |  |  |  | and PC4 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3, PC4 and PC7 or Table 6.1-2 for PC1. NOTE 2: The 200MHz and 400MHz bandwidths are not applicable to PC7 RedCap UEs |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [10] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 38.521-1 [2] Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Annex A.2.3-1 with the uplink duty cycle Z set to 20%.

5.	Propagation conditions are set according to TS 38.521-1 [2] Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED  with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.2.5.4.2	Test procedure

- to schedule the UL RMC according to Table 6.2.1.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2.1.1.4.3.

1a.	If the UE does not support beamCorrespondenceWithoutULBeamSweeping, the side conditions for SSB-based and CSI-RS based L1-RSRP measurements are applied as per Table 6.6.1.3.3.1.1-1 and Table 6.6.1.3.3.1.1-2 respectively.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

ACTIVATE Uplink Gaps

5. SS configures and activates UL-gaps via message contents defined in section 6.2.5.4.3-1. P-MPR reporting is also enabled via the message contents defined in 6.2.5.4.3-2.

6.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration. EIRP test procedure is defined in Annex K.1.3. The period of measurement shall be at least 4 seconds. EIRP is calculated considering both polarizations, theta and phi. Record this as peak EIRP PUMAX,f,c_GAP_ON

7.	SS detects and record the value within the P-MPR reports. Call this value P-MPRULgapON

DE-ACTIVATE Uplink Gaps

8. SS de-activates UL-gaps via message contents defined in section 6.2.5.4.3-1.

9.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration. EIRP test procedure is defined in Annex K.1.3. The period of measurement shall be at least 4 seconds. EIRP is calculated considering both polarizations, theta and phi. Record this value as peak EIRP PUMAX,f,c_GAP_OFF

10.	SS detects and record the value of the P bit within the PHR.

11.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

12.		Compute the difference between PUMAX,f,c_GAP_ON and PUMAX,f,c_GAP_OFF

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

6.2.5.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config and with the following exception

Table 6.2.5.4.3-1: UE UL-GapFR2-Config (FR2 UL-Gap Activation)

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-200B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UL-GapFR2-Config-r17 ::= CHOICE { |  |  |  |
| Setup SEQUENCE { |  |  |  |
| gapOffset-r17 | 0 |  |  |
| ugl-r17 | ms1 |  |  |
| ugrp-r17 | ms40 |  |  |
| } |  |  |  |

Table 6.2.5.4.3-2: PHR Config (P-MPR Report Activation)

| Derivation Path: TS 38.508 [10], clause 4.6.3-104 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PHR-Config ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mpe-Reporting-FR2-r16:: CHOICE { |  |  |  |
| Setup SEQUENCE { |  |  |  |
| Mpe-ProhibitTimer-r16 | [sf10] |  |  |
| Mpe-Threshold-r16 | dB3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.5.4.3-3: UE UL-GapFR2-Config (FR2 UL-Gap De-activation)

| Derivation Path: TS 38.508 [10], clause 4.6.3-200BB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UL-GapFR2-Config-r17 ::= CHOICE { |  |  |  |
| release |  |  |  |
| } |  |  |  |

6.2.5.5	Test requirement

The difference between PUMAX,f,c_GAP_ON and PUMAX,f,c_GAP_OFF computed in Step 12 and the UE reported P-MPRULgapON  and P-bit within PHR value in Steps 7 and 10 respectively shall meet the requirements defined in Table 6.2.5.5-1

Table 6.2.5.5-1: Test Requirements for EIRP with UL Gaps (for Power class 3)

| Test Metric | Requirement |
| --- | --- |
| PUMAX,f,c_GAP_ON - PUMAX,f,c_GAP_OFF | ≥max((EIRPmeas_peak – 23) + 10 * log10(Z/20), 3)dB - TT |
| P-MPRULgapON | < 3dB + TT |
| P bit reported within PHR report (when UL-Gaps OFF) | 1 |
| NOTE 1: Z is the uplink duty cycle set within the test procedure |  |

Table 6.2.5.5-2: TT for EIRP with UL Gaps (for Power class 3)

FFS

<Unchanged sections skipped>

## A.2.3	Reference measurement channels for TDD

For UL RMCs defined below, TDD slot pattern defined in Table A.2.3-1 will be used for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, TDD slot patterns defined for reference sensitivity tests in Table A.3.3.1-1 will be used.

Table A.2.3-1: Additional reference channels parameters for TDD

| Parameter |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS 60 kHz (µ=2) | SCS 120 kHz (µ=3) | SCS 480 kHz (µ=5) | SCS 960 kHz (µ=6) |
| TDD Slot Configuration pattern (Note 1) |  | DDDSUUUU | 7DS8U | 31DS32U | 63DS64U |
| Special Slot Configuration (Note 2) |  | S=4D+6G+4U | S=12D+2G | S=2D+12G | S=2D+12G |
| referenceSubcarrierSpacing |  | 60 kHz | 120 kHz | 480 kHz | 960 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 2 ms | 2 ms | 2 ms | 2ms |
|  | nrofDownlinkSlots | 3 | 7 | 31 | 63 |
|  | nrofDownlinkSymbols | 4 | 12 | 2 | 2 |
|  | nrofUplinkSlot | 4 | 8 | 32 | 64 |
|  | nrofUplinkSymbols | 4 | 0 | 0 | 0 |
| Indexes of active UL slots |  | mod(slot index, 40) = {36,…,39} | mod(slot index, 80) = {72,…,79} | mod(slot index, 320) = {288,…,319} | mod(slot index, 640) = {576,…,639} |
| Indexes of active UL slots for UL Gap test |  | mod(slot index, 40) = {12,…,15, 36,…,39} | mod(slot index, 80) = {24,…,31 ,72,…,79} |  |  |
| Indexes of the UL slots for UL Gap when UL gap pattern configuration 3 (IE UL-GapFR2-Config-r17) is configured |  | mod(slot index,40)={7, 28} | mod(slot index, 80) = {15,56} |  |  |
| Indexes of the UL slots for UL Gap when UL gap pattern configuration 1 (IE UL-GapFR2-Config-r17) is configured |  | mod(slot index,160)={20, 21, 22,23, 28, 29,30,31} | mod(slot index, 320) = {8, … ,15} |  |  |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. |  |  |  |  |  |

<End of changes>