3GPP TSG RAN Meeting #101	 RP-231787

Bangalore, India, September 11-15, 2023

Source:	KDDI Corporation

Title:	Discussion on Network Energy Saving Enhancements

Tdoc Type:	Discussion

Agenda Item:	8A.2.5 Network Energy Saving Enhancements

Document for:  Discussion and decision

Release:       Rel-19

- Introduction

At the RAN Release-19 Workshop, many contributions featuring enhancements on network energy savings were submitted. This shows a significant interest in the techniques making operators' networks more efficient and less power consuming. However, the companies' views seem to be somewhat ambiguous and, to some extent, diverging. In this contribution, we analyze/categorize the contributions so that everyone has the same understanding of each enhancement. Additionally, we propose some ideas that we believe are beneficial to developing justifications and objectives for Release-19 NES WI.

- Discussion

Overview

On a high level, we understand that three main directions are being considered as enhancements for Rel-19, namely SIB-less/WUS, Cell DRX/DTX for idle mode, Adaptation of common channels/signals, and dynamic Adaptation of TRPs.

- SIB-less/WUS: The first direction concerns techniques that are applicable to both connected and idle UEs in this area. For connected UEs (CA), inter-band CA with SSB-less will be supported in Rel-18 only for FR1, but not for FR2. For idle UEs, no enhancements were made in Rel-18 due to backward compatibility issues caused by the techniques. Therefore, this is a potential area for Rel-19 enhancements.

- Cell DRX/DTX for idle mode: The second direction includes Cell DRX/DTX for idle mode. In Rel-18, Cell DRX/DTX is only available to connected UEs because of the backward compatibility. With this technique, common channels/signals are re-designed to be handled within a short period of time, so that gNB can have longer consecutive period to sleep. This makes legacy UEs cannot camp on the cell which activate Cell DRX/DTX for idle mode, thus may need solutions for legacy UEs to avoid the cell.

- Adaptation of common channels/signals: The third direction involves techniques enable gNBs to reduce or omit occasions for common channels and signals depending on their traffic load. Specifically, for the downlink, the SSB, SI, paging, and cell common PDCCH can be considered, while for the uplink, RACH can be considered.

- Dynamic Adaptation of TRPs: Forth direction concerns techniques aim to support TRP activation/deactivation that can be informed to the UE when a UE is configured with multiple TRPs. The technique aims to dynamically adapt the number of TRPs transmitting and/or receiving signals and channels.

There are some contributions that propose further study for power domain techniques, such as Technique D-2 (Over the Air Digital Pre-Distortion) and Technique D-5 (UE Post-Distortion) [4][16]. These techniques can address high load scenarios and are thus attractive to operators. However, according to [4], it is not clear how they can work properly in base stations that are equipped with multiple RATs or multiple carriers. Further study is required, but it seems not feasible to start such a study within the Rel-19 timeframe as there is not enough time available.

Proposal1: RAN discuss the following three directions to identify motivation/justification and objectives of Rel-19 NES,

- SIB-less/WUS

- Cell DRX/DTX for idle mode

- Adaptation of common channels/signals

- Dynamic Adaptation of TRPs

SIB-less/WUS

For the first direction, SIB-less/WUS, we think that proposals submitted to Rel-19 WS can be classified into the following three categories:

- Category 1: SSB-less for CA (SCell) [1][2][4][10][13][16]: This category includes techniques for CA that shut off the secondary cell's SSB. In Rel-15, intra-band CA with SSB-less carriers was supported. In Rel-18, Inter-band CA with SSB-less is supported only for FR1. Inter-band CA with SSB-less for FR2 is a possible enhancement for Rel-19. This may require an indication (WUS) from UEs for the case where UE obtains SSBs such as for Scell activation or measurements.

- Category 2: SSB-less for idle mode UE (multi carrier operation) [1][2][6][8][10][12][13][16][18]: Up until Rel-18, a cell needed to regularly transmit SSB and SIB1 to be able to serve RRC IDLE and RRC INACTIVE UEs. Therefore, dropping SSB and SIB1 for idle mode UE is a possible enhancement for Rel-19. With this solution requires, for Intra-band multi carrier scenario, UE acquires time/frequency synchronization for the SSB/SIB1-less cell and system information from anchor cell and initiates UL/DL data transfer with the SSB/SIB1-less cell. For inter-band multi carrier scenario, UE acquires system information from anchor cell but may not acquire time/frequency synchronization for the SSB/SIB1-less cell. So, SSB/SIB1-less cell may need to transmit the Discovery Reference Signal to notice its presence to UE. With this solution, UE may receive paging on anchor cell when UE establish a call, it initiates RACH procedure on SSB/SIB1-less cell. This technique may include NES Cell without SIB/SSB (RAN2) mentioned in TR38.864.

- Category 3: SSB-less for idle mode UE (WUS) [1][2][6][9][10][11][12][13][15][16][17][18][19]: Another solution for idle mode UE, having an advantage not requires broadcasting system information in anchor cell, but anchor cell may provide only the information UE uses for WUS transmission. The UE transmits WUS to request SSB/SIB1-less cell to re-active omitted SSB/SIB1 transmission. This solution may need the SSB-less cell to transmit Discovery Reference Signal DRS (e.g., simplified SSBs) to announce its presence. This technique may include Technique A-3 (UE wake up signal (WUS) for gNB) and Technique A-5 (adaptation of SSB/SIB1 including on-demand SSB/SIB1), as mentioned in TR38.864.

For SSB-less in connected mode (CA Scell), many companies have proposed an enhancement for inter-band CA with SSB-less for FR2 in Rel-19. Therefore, we propose the following:

Proposal2: For SIB-less for CA, RAN discuss whether to include inter-band CA with SSB-less for FR2 in Rel-19 objectives.

Both Category 2 and Category 3 aim to achieve SSB-less operation for idle mode UE, and the two techniques are somewhat similar. From our understanding, the main difference between Category 2 and Category 3 is the cell from which UEs receive SIB/SIB1/paging. With Category 2, UEs receive these signals from the anchor cell, while with Category 3, UEs receive them from the SSB/SIB1-less cell after WUS transmission. Table 1 summarizes the differences between Category 2 and Category 3.

Proposal3: For SIB-less for idle mode, RAN discuss whether to include both category 2 (SSB-less for idle mode UE (multi carrier operation)) and category 3 (SSB-less for idle mode UE (WUS)) or either of them in Rel-19 objectives.

|  | Category 2 SSB-less for idle mode UE (multi carrier operation) | Category3 SSB-less for idle mode UE (WUS) |
| --- | --- | --- |
| SSB/SIB1 | Anchor cell | SSB/SIB1 less cell |
| Paging | Anchor cell | SSB/SIB1 less cell |
| Wake Up Signal | Potentially using current on demand SI acquition to acquire SI of SSB-less cell on anchor cell | SSB/SIB1 less cell |
| WUS configuration | NA | Other than SSB/SIB1 less cell |
| RACH | SSB/SIB1 less cell | SSB/SIB1 less cell |
| Discovery Reference Signal | SSB/SIB1 less cell | SSB/SIB1 less cell |

Table 1. Category 2 vs Category 3

Cell DRX/DTX for idle mode

The second direction includes Cell DRX/DTX for idle mode. In Rel-18, Cell DRX/DTX is only available to connected UEs because of the backward compatibility. So, in Rel-19 Cell DRX/DTX for idle mode can be considered.

- Category 4: Cell DRX/DTX for idle mode [3][10][19]: Common channels and signals are redesigned to be handled within a short period of time so that the gNB can have a longer consecutive period of sleep. Without Cell DRX/DTX, channels and signals are scattered on the time axis, which leaves no time for the gNB to enter deep sleep.

Proposal4: RAN discuss whether to include Cell DTX/DRX for idle mode UE in Rel-19 objectives.

Adaptation of common channels/signals

The third direction concerns techniques captured as Technique A-1 (Adapting Transmission/Reception of Common Channels/Signals) in TR38.864. This involves adapting the transmission pattern (when applicable) of downlink common and broadcast signals, such as SSB/SI/paging, and/or the transmission pattern/availability of uplink random access opportunities. Adaptation of the transmission pattern includes changes to periodicity, time resource locations, and omitting of specific signals/channels. Regarding adaptation of common channels/signals, some companies provide a high-level view [5][7][15], while others are interested in some specific channels/signals.

- Category 5: Paging enhancements for low loads (technique A-1-4) [1][11][15]: Currently UEs’ paging occasions are distributed in the time domain, which negatively impacts the RAN sleeping opportunities. The paging can be clustered in time for Rel19 UEs.

- Category 6: Dynamic Adaptation of PRACH/SR Occasions [4][10][16]: PRACH configuration change using more light-weight mechanisms than SI update, this is captured as (technique A-1-5) in TR38.864.

Proposal5: RAN discuss and identify which common channels/signals are considered for adaptation in UE in Rel-19 objectives,

- Paging

- PRACH

- SSB/SIB1

  - Dynamic Adaptation of TRPs

The last direction concerns techniques captured as Technique C-2 (Adaptation of TRPs in mTRP operation) in TR38.864. This technique aims to support TRP activation/deactivation that can be informed to the UE when a UE is configured with multiple TRPs. The technique aims to dynamically adapt the number of TRPs transmitting and/or receiving signals and channels.

- Category 8: Adaptation of TRPs in mTRP operation [2][5][7][8][13][14][15][16][17][18]: This includes support for link adaptation of dedicated channels with dynamic adaptation of the number of antenna ports, beam management, power control, radio link monitoring, cell (re)selection, handover, and initial access in a multi-TRP configuration.

Proposal6: RAN discuss objectives for adaptation of TRPs in mTRP operation using the text captured in TR38.864 as a baseline.

  - UE-specific/group-level/cell common signaling for indicating adaptation of TRPs and TRP-related parameters (e.g. TRP index or CORESET pool index) in mTRP,

  - Enhancements to UE behaviours due to dynamic adaptation of TRPs, e.g., measurements, CSI feedback, power control, PDCCH/PUCCH/PUSCH/PDSCH repetition, single-DCI based scheduling, multi-DCI based scheduling, SRS transmission, TCI configuration, beam management, beam failure recovery, radio link monitoring, cell (re)selection, handover, initial access, etc.

3	Conclusion

We make the following observations and proposals:

Proposal1: RAN discuss the following three directions to identify motivation/justification and objectives of Rel-19 NES,

- SIB-less/WUS

- Cell DRX/DTX for idle mode

- Adaptation of common channels/signals

- Dynamic Adaptation of TRPs

Proposal2: For SIB-less for CA, RAN discuss whether to include inter-band CA with SSB-less for FR2 in Rel-19 objectives.

Proposal3: For SIB-less for idle mode, RAN discuss whether to include both category 2 (SSB-less for idle mode UE (multi carrier operation)) and category 3 (SSB-less for idle mode UE (WUS)) or either of them in Rel-19 objectives.

Proposal4: RAN discuss whether to include Cell DTX/DRX for idle mode UE in Rel-19 objectives.

Proposal5: RAN discuss and identify which common channels/signals are considered for adaptation in UE in Rel-19 objectives,

- Paging

- PRACH

- SSB/SIB1

Proposal6: RAN discuss objectives for adaptation of TRPs in mTRP operation using the text captured in TR38.864 as a baseline.

  - UE-specific/group-level/cell common signaling for indicating adaptation of TRPs and TRP-related parameters (e.g. TRP index or CORESET pool index) in mTRP,

  - Enhancements to UE behaviours due to dynamic adaptation of TRPs, e.g., measurements, CSI feedback, power control, PDCCH/PUCCH/PUSCH/PDSCH repetition, single-DCI based scheduling, multi-DCI based scheduling, SRS transmission, TCI configuration, beam management, beam failure recovery, radio link monitoring, cell (re)selection, handover, initial access, etc.

3	Reference

[1] RWS-230013	Network energy saving enhancements	Nokia, Nokia Shanghai Bell

[2] RWS-230180	NW Energy Savings	Qualcomm Incorporated

[3] RWS-230267	Network Energy Saving in Rel-19	Fujitsu Limited

[4] RWS-230156	On NW Energy Savings for Rel-19	Ericsson

[5] RWS-230208	Network Energy Saving enhancements in Rel-19	Samsung

[6] RWS-230364	Network energy savings enhancements in Rel-19	Huawei, HiSilicon

[7] RWS-230379	Network Energy Saving Enhancement	CATT

[8] RWS-230113	[RAN1-led] System Energy Efficiency Enhancement for NR	MediaTek Inc.

[9] RWS-230062	Views on NR network energy saving in Rel-19	vivo

[10] RWS-230139	Network Energy Saving (NES) enhancement	NEC

[11] RWS-230456	Rel-19 Enhancements for Network Energy Savings (NES)	Intel Corporation

[12] RWS-230423	Further enhancements on network energy saving	CMCC

[13] RWS-230062	Views on NR network energy saving in Rel-19	vivo

[14] RWS-230197	Proposal on network energy saving in Rel-19	LG Electronics

[15] RWS-230239	Network energy saving for Rel-19	China Telecom

[16] RWS-230281	Views on Network Energy Saving in Release 19	KDDI Corporation

[17] RWS-230300	Views on network energy saving in Rel-19	ZTE, Sanechips

[18] RWS-230334	Discussion on enhancement of network energy savings	Rakuten Mobile, Inc

[19] RWS-230440	Views on Enhanced Network Energy Saving	Fraunhofer IIS, Fraunhofer HHI