3GPP TSG RAN Meeting #101			                                                       RP-231923

Bangalore, India, September 11-15, 2023

Agenda item:	9.3.4.14

Source: 	Vodafone, vivo

Title:	On the WID revision of NR_BWP_wor

Document for:	Decision, Discussion

# 1	Introduction

This contribution aims to propose revisions for the “Complete the specification support for BandWidth Part operation without restriction in NR” Work Item approved in [1], with the following objectives:

### The work item includes following objectives for non-Redcap UE: 1) Specify support for BM/RLM/BFD: - For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based onSSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD-SSB and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to Interoperability Device Testing (IoDT) availability). - The interruption related requirements will be decided and specified in RAN4. 2) Specify the necessary requirements to support L3 intra-frequency measurements without gaps (RAN4) - For Option B-1-1 - Specify applicability of legacy intra-frequency L3 measurements without gap and corresponding UE capabilities, if needed. - Note: UEs capable of Option B-1-1 BWP operation can optionally support L3 intra-frequency measurements with gaps. - For Option C - Specify requirements for L3 intra-frequency measurements based on existing RedCap NCD-SSB requirements. - Note: further check in RAN #101 on whether any work is required to support L3 intra-frequency measurements without gaps for Option B-1-2 The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

In our companion contribution [2], a document with the WID revisions is presented for further discussion and proceeding approval.

# 2	Topics under revisions

The topics we would like to address are the following:

Topic 1: L3 intra-frequency measurements for Option B-1-2

Topic 2: Handover support for Option C

## 2.1 L3 intra-frequency measurements for Option B-1-2

One of the open points of discussion after RAN#100 was whether a similar objective on L3 intra-frequency measurements that was added for Option B-1-1 and C, should also be added for Option B-1-2.

Currently, in terms of progress, our assessment [3] is that Options A, B-1-1, and C are already quite advanced when compared to Option B-1-2. From the last RAN4 WF [4], one can see that there are still several issues to be discussed on the interruption requirements for Option B-1-2 just considering L1 measurements (BM/RLM/BFD). As we expect that further discussion, somewhat controversial, for Option B-1-2 is still required, in our view there should not be any further up-scoping of Option B-1-2. With this reasoning we propose the following:

Proposal 1: No further up-scoping of Option B-1-2 should be made. Note on the support L3 intra-frequency measurements without gaps for Option B-1-2 should be removed.

## 2.2 Handover support for Option C

From the feedback received from multiple companies in RAN4#108, we find the inclusion of this objective beneficial for non-RedCap UEs supporting Option C. As captured in the contribution we co-signed [5], we do not see a reason why direct handover to a BWP with NCD-SSB should not be supported, as without this support a 2-stage procedure would need to be performed which would deem additional signalling overhead, delay, and throughput loss. Given that the specification would be based on the existing one for RedCap UEs, our understanding is that workload to define the additional handover cases ought to be reduced. With this said, we propose the following:

Proposal 2: For non-RedCap UEs supporting Option C, specify the necessary requirements to support the additional handover cases as for RedCap directly to BWP with NCD-SSB.

# 4	Conclusions

In this contribution, the following observation and proposals are made:

Proposal 1: No further up-scoping of Option B-1-2 should be made. Note on the support L3 intra-frequency measurements without gaps for Option B-1-2 should be removed.

Proposal 2: For non-RedCap UEs supporting Option C, specify the necessary requirements to support the additional handover cases as for RedCap directly to BWP with NCD-SSB.

# References

- RP-231486, Revised WID: Complete the specification support for BandWidth Part operation without restriction in NR, Moderator (Intel), Vodafone, vivo, 3GPP RAN#100 meeting, June 2023.

- RP-231922, “Further revised WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo, 3GPP RAN#101, September 2023.

- RP-231924, “Status Report to TSG: NR_BWP_wor”, Vodafone, 3GPP RAN#101, September 2023.

- R4-2314339, “WF on support for bandwidth part operation without restriction”, vivo, 3GPP RAN4#108, August 2023.

- RP-231823, “Scope of BWP operation without restriction WI”, vivo, Vodafone, Mediatek Inc., CATT, (…), 3GPP RAN#101, September 2023