3GPP TSG-RAN Meeting #101	RP-232030

Bangalore, India, 11th – 15th September, 2023

Agenda item:	8A.2.13.1

Source:	Samsung

Title:	Scope of Network Controlled Repeaters in Rel-19

Document for:	Discussion and Decision

# Introduction

From the Rel-19 Workshop, the following are on the agenda of potentially RAN2-led candidate topics [1]:

##### 8A.1.13 Additional RAN2-led Candidate Topics ##### 8A.2.13.1 Network-Controled Repeaters (NCR) ##### 8A.2.13.2 Sidelink (SL) Relay Enhancements ##### 8A.2.13.3 Uncrewed Aerial Vehicles (UAV)/Urban Air Mobility (UAM) ##### 8A.2.13.4 Multi-SIM (MU-SIM) ##### 8A.2.13.5 Broadcast/multicast ##### 8A.2.13.6 UE aggregation, collaboration, and backup

Rel-18 Work Item on Network-Controlled Repeaters (NCR) introduced a simplified amplify-and-forward repeater that is controlled by the network [2][3].

- During Rel-19 Workshop, Rel-19 enhanced NCR was discussed by several companies. In this contribution, we present our view on enhanced NCR for Release 19.

# Views on NCR support in NR

- During Rel-18 study item on NCR, power control for NCR was extensively discussed. Although there was considerable support for NCR power control among some companies, the use-case, justifications, and simulation results were not compelling. Accordingly, power control was left out of the scope of the Rel-18 work item on NCR. That assessment has not changed, so there is no need to re-open the discussion for NCR support. Since the key benefit of NCR is lower cost intermediated node than IAB, any enhancements on NCR in the physical layer bring additional cost and complexity for deployments.

- Proposal 1: Do not introduce power control for NCR to support under NR and no enhancement in physical layer for future releases

- Coming to RAN2 discussion in Rel-18, several RAN2-specific issues were discussed but RAN2 was not able to conclude them given limited TU allocation and simplicity.

- For instance, due to requirement to introduce a simplified NCR-MT, it was agreed to not support connected mode RRM and connected mode mobility. On the other hand, idle and inactive mode mobility is supported in RAN2. This brings the potential “lock-in mobility”, whereby an NCR selects “a wrong cell” and donor gNB lacks good mechanisms to transfer the NCR to the “right cell”.

- Another issue in Release 18, is that when an NCR fails it was specified that the NCR shall cease forwarding operation. Due to this limitation, we think it can be valuable for network-optimization to enable an NCR failure report for network optimization and diagnostics.

- Since these RAN2-specific enhancements would not bring additional cost and complexity for deployments–even helpful to minimize the deployment efforts–they can be considered as part of Rel-19 if they receive support from companies.

- Proposal 2: The possible objectives for Rel-19 can be:

- Specify connected mode RRM and mobility [RAN2, RAN3, RAN4] - Specify NCR failure reporting to allow for network optimization and diagnostics [RAN2, RAN3]

# Conclusion

In this contribution we presented our views on network-controlled repeater (NCR) and necessarily of NCR enhancement.

- Proposal 1:	Do not introduce power control for NCR to support under NR and no enhancement in physical layer for future releases.

- Proposal 2:	The possible objectives for Rel-19 eNCR can be:

- Specify connected mode RRM and mobility [RAN2, RAN3, RAN4]

Specify NCR failure reporting to allow for network optimization and diagnostics [RAN2, RAN3]

# Reference

RWS-230488, RAN Chair’s Summary of Rel-19 Workshop, RAN Chair, June 2023.

RP-213700, New SI: Study on NR Network-controlled Repeaters, ZTE Corporation, RAN#94-e, December 2021.

RP-223505, Revised WID on NR network-controlled repeaters, ZTE, Sanechips, RAN#98-e, December 2022