3GPP TSG RAN Meeting #101                                                          RP- 232099

# Bangalore, IN, Sep 11 - 15, 2023

Agenda Item:	9.2.5

Source:	Apple Inc.

Title:	View on R18 AI/ML for NR air interface SI status

Document for:	Discussion/Decision

# Introduction

The R18 RAN1 led study item mainly focuses on three use cases: CSI enhancement, beam management and positioning. In general agenda, the high-level LCM procedure was discussed. In RAN1 #113, the initial draft TR was approved [1].

In this contribution, we discuss the status of the R18 SI.

# Discussion

General aspects

Both functionality-based LCM and model ID based LCM have been agreed. The applicability of functionality-based LCM and model ID based LCM is open.

It was concluded that the applicable functionalities/models can be reported by UE. It is desirable to narrow down which reporting framework can be used in SI, such as UE capability report framework, UAI framework etc, to facilitate the signaling design in WI phase.

For model transfer, model transfer case z1 to z5 are defined, and comparison with level y model transfer were discussed and not concluded yet.

In RAN1 114 meeting, the reply LS for part A of data collection were discussed and agreed. Part B of the LS were not discussed. An email discussion for part B has been scheduled, however it is not clear whether RAN1 can converge over email, as part B is more challenging to answer.

CSI enhancement

For CSI enhancement, two sub-use cases have been studied:

- Spatial-frequency domain CSI compression using two-sided AI model.

- Time domain CSI prediction using UE sided model

For CSI compression sub-use case, the potential specification impact has been identified including data collection, inferencing and performance monitoring. For training, it was concluded that training collaboration type 2 will be deprioritized for 3GPP discussion. For training collaboration type 1, model transfer will be the main specific impact. For training collaboration type 3 which requires additional dataset alignment and delivery, there is no agreement whether this is outside of 3GPP scope or not. For procedure to align the paring information between UE side CSI generation model and NW side CSI reconstructive model, further discussion is needed.

For training collaboration type 1, 2 and 3 pros/cons discussion, two tables will be used to capture the high-level comparison. The table structure was agreed. The detailed comparison still requires further discussion to converge.

For CSI prediction, the SI scope is limited to data collection and performance monitoring. In RAN1 114, the functionality-based performance monitoring was agreed. Depending on general aspect discussion, in case model ID based LCM is used for UE side model as well, further discussion on model ID based CSI prediction LCM might be needed.

Observation: Both general aspect and CSI compression use case requires extension for SI conclusion.

Proposal 1: RAN1 extend the R18 AI/ML SI with limited scope to 9.2.1 and 9.2.2.2.

# Conclusion

In this contribution, we discussed the status of the RAN1 led R18 AI/ML SI. Based on the discussion, the following proposals have been proposed.

Observation: Both general aspect and CSI compression use case requires extension for SI conclusion.

Proposal 1: RAN1 extend the R18 AI/ML SI with limited scope to 9.2.1 and 9.2.2.2.

# Reference

[1] TR 38.843, v0.1.0, Study on AI/ML for NR air interface, June 2023