| 3GPP TR 38.858 V2.0.0 (2023-12) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Evolution of NR Duplex Operation (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

# B.3	System level simulation results of schemes for SBFD

## B.3.1	SLS results for dynamic SBFD

The detailed evaluation results for dynamic SBFD can be found in the attached document "B.3.1_Dynamic SBFD.zip".

#### B.3.1.1	Indoor office (FR1)

B.3.1.1.1	Dynamic SBFD vs. dynamic TDD

Table B.3.1.1.1-1: Results of Indoor (FR1) dynamic SBFD vs. dynamic TDD, {XXXXU} vs. {FFFFU}

(dynamic SBFD Option 2, large packet size)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), dynamic SBFD slot configuration {XXXXU}, dynamic TDD slot configuration {FFFFU}, dynamic SBFD Option 2, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [41] |  |  | [40] |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 3.16% | 23.45% | 45.01% | 4.95% | 15.76% | 32.13% |
|  |  | Type-2 |  |  |  | 8.53% | 27.17% | 55.40% |
|  | UL RU | Type-1 | 0.95% | 8.73% | 16.16% | 3.34% | 10.01% | 21.73% |
|  |  | Type-2 |  |  |  | 7.94% | 23.84% | 51.74% |
| Dynamic SBFD | DL RU | Type-1 | 2.78% | 11.69% | 27.57% | 5.44% | 16.64% | 32.53% |
|  |  | Type-2 |  |  |  | 8.78% | 26.84% | 52.46% |
|  | UL RU | Type-1 | 1.56% | 5.64% | 11.81% | 2.76% | 9.26% | 19.72% |
|  |  | Type-2 |  |  |  | 7.27% | 24.36% | 51.89% |
| Baseline | DL average-UPT CDF | Mean | 608.21 | 385.84 | 240.26 | 303.37 | 205.25 | 81.48 |
|  |  | 5% | 562.16 | 279.01 | 112.08 | 225.92 | 141.90 | 60.81 |
|  |  | 50% | 609.12 | 380.08 | 227.00 | 306.31 | 210.01 | 83.13 |
|  |  | 95% | 653.78 | 520.68 | 420.67 | 377.35 | 258.92 | 97.25 |
|  | UL average-UPT CDF | Mean | 259.40 | 134.81 | 71.42 | 171.77 | 121.72 | 40.31 |
|  |  | 5% | 235.27 | 96.27 | 37.28 | 139.99 | 97.71 | 33.12 |
|  |  | 50% | 259.97 | 133.12 | 67.94 | 170.64 | 118.47 | 39.61 |
|  |  | 95% | 281.33 | 179.33 | 115.81 | 208.79 | 151.93 | 50.45 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 600.27 | 339.02 | 153.37 | 321.38 | 215.23 | 84.08 |
|  |  | 5% | 552.83 | 229.14 | 16.15 | 233.13 | 145.89 | 62.17 |
|  |  | 50% | 601.23 | 332.28 | 143.75 | 329.31 | 221.87 | 86.66 |
|  |  | 95% | 643.08 | 477.88 | 335.57 | 392.73 | 278.89 | 100.23 |
|  | UL average-UPT CDF | Mean | 156.54 | 95.91 | 66.30 | 180.66 | 128.23 | 41.84 |
|  |  | 5% | 141.52 | 72.59 | 37.67 | 148.89 | 104.97 | 34.25 |
|  |  | 50% | 156.47 | 94.65 | 64.93 | 180.56 | 124.50 | 40.97 |
|  |  | 95% | 170.59 | 123.74 | 97.27 | 225.85 | 165.89 | 52.61 |
| Baseline | DL packet-Latency CDF | Mean | 6.88 | 12.96 | 36.66 | 13.31 | 19.26 | 50.12 |
|  |  | 5% | 5.21 | 6.13 | 7.54 | 9.12 | 13.87 | 39.07 |
|  |  | 50% | 6.37 | 10.64 | 19.11 | 12.82 | 18.23 | 47.90 |
|  |  | 95% | 10.30 | 27.30 | 77.54 | 18.68 | 27.11 | 67.92 |
|  | UL packet-Latency CDF | Mean | 4.14 | 9.49 | 22.92 | 5.75 | 8.70 | 25.75 |
|  |  | 5% | 3.06 | 3.74 | 6.37 | 2.73 | 4.18 | 16.39 |
|  |  | 50% | 3.55 | 8.23 | 16.10 | 5.80 | 8.81 | 25.35 |
|  |  | 95% | 6.95 | 19.65 | 56.66 | 7.82 | 11.88 | 32.59 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 6.99 | 15.60 | 126.59 | 12.39 | 18.29 | 47.93 |
|  |  | 5% | 5.24 | 6.53 | 9.32 | 8.76 | 13.33 | 37.13 |
|  |  | 50% | 6.47 | 12.39 | 33.64 | 12.15 | 17.25 | 45.93 |
|  |  | 95% | 10.47 | 35.11 | 419.15 | 16.99 | 25.96 | 65.10 |
|  | UL packet-Latency CDF | Mean | 6.68 | 12.43 | 21.64 | 5.51 | 8.30 | 24.98 |
|  |  | 5% | 4.78 | 6.24 | 7.81 | 3.33 | 5.16 | 18.08 |
|  |  | 50% | 6.38 | 10.70 | 16.20 | 5.49 | 8.12 | 24.11 |
|  |  | 95% | 9.51 | 24.39 | 51.33 | 7.26 | 10.86 | 30.85 |

Table B.3.1.1.1-2: Results of Indoor (FR1) dynamic SBFD vs. dynamic TDD, {XXXXU} vs. {FFFFU}

(dynamic SBFD Option 3, large packet size)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), dynamic SBFD slot configuration {XXXXU}, dynamic TDD slot configuration {FFFFU}, dynamic SBFD Option 3, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [41] |  |  | [40] |  |  | [33] |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 3.16% | 23.45% | 45.01% | 4.95% | 15.76% | 32.13% | 7.61% | 23.12% | 54.89% |
|  |  | Type-2 |  |  |  | 8.53% | 27.17% | 55.40% | 11.57% | 35.40% | 81.38% |
|  | UL RU | Type-1 | 0.95% | 8.73% | 16.16% | 3.34% | 10.01% | 21.73% | 1.79% | 4.59% | 10.41% |
|  |  | Type-2 |  |  |  | 7.94% | 23.84% | 51.74% | 5.70% | 14.40% | 35.05% |
| Dynamic SBFD | DL RU | Type-1 | 2.75% | 11.27% | 26.93% | 4.70% | 16.29% | 35.09% | 7.31% | 21.30% | 52.44% |
|  |  | Type-2 |  |  |  | 7.11% | 24.69% | 53.17% | 10.25% | 30.53% | 73.65% |
|  | UL RU | Type-1 | 1.61% | 6.64% | 13.96% | 2.78% | 9.07% | 18.30% | 1.77% | 4.55% | 10.50% |
|  |  | Type-2 |  |  |  | 8.19% | 26.66% | 53.84% | 6.83% | 16.45% | 39.91% |
| Baseline | DL average-UPT CDF | Mean | 608.21 | 385.84 | 240.26 | 303.37 | 205.25 | 81.48 | 523.29 | 396.08 | 256.13 |
|  |  | 5% | 562.16 | 279.01 | 112.08 | 225.92 | 141.90 | 60.81 | 352.19 | 215.81 | 62.26 |
|  |  | 50% | 609.12 | 380.08 | 227.00 | 306.31 | 210.01 | 83.13 | 533.84 | 392.70 | 231.76 |
|  |  | 95% | 653.78 | 520.68 | 420.67 | 377.35 | 258.92 | 97.25 | 628.38 | 576.58 | 518.81 |
|  | UL average-UPT CDF | Mean | 259.40 | 134.81 | 71.42 | 171.77 | 121.72 | 40.31 | 338.46 | 246.14 | 156.22 |
|  |  | 5% | 235.27 | 96.27 | 37.28 | 139.99 | 97.71 | 33.12 | 297.23 | 218.19 | 125.96 |
|  |  | 50% | 259.97 | 133.12 | 67.94 | 170.64 | 118.47 | 39.61 | 340.54 | 247.10 | 156.77 |
|  |  | 95% | 281.33 | 179.33 | 115.81 | 208.79 | 151.93 | 50.45 | 363.79 | 271.19 | 186.47 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 602.93 | 348.36 | 162.59 | 293.1457 | 197.68 | 80.59 | 617.36 | 473.57 | 312.35 |
|  |  | 5% | 558.63 | 238.37 | 20.34 | 217.1864 | 135.57 | 59.93 | 405.50 | 257.50 | 102.17 |
|  |  | 50% | 603.09 | 341.71 | 152.33 | 297.4999 | 203.00 | 82.93 | 642.59 | 451.63 | 287.45 |
|  |  | 95% | 645.22 | 485.78 | 344.86 | 372.0556 | 248.77 | 95.52 | 744.08 | 694.07 | 617.45 |
|  | UL average-UPT CDF | Mean | 259.62 | 133.72 | 77.20 | 182.8996 | 129.92 | 42.34 | 347.74 | 258.88 | 154.82 |
|  |  | 5% | 234.63 | 96.19 | 44.93 | 153.2426 | 108.00 | 35.78 | 326.69 | 230.26 | 105.60 |
|  |  | 50% | 260.77 | 132.26 | 75.16 | 182.8024 | 125.05 | 41.78 | 348.23 | 259.25 | 157.76 |
|  |  | 95% | 281.68 | 177.52 | 119.73 | 232.5812 | 169.78 | 54.71 | 365.87 | 285.52 | 196.30 |
| Baseline | DL packet-Latency CDF | Mean | 6.88 | 12.96 | 36.66 | 13.31 | 19.26 | 50.12 | 8.59 | 13.79 | 55.32 |
|  |  | 5% | 5.21 | 6.13 | 7.54 | 9.12 | 13.87 | 39.07 | 5.61 | 5.79 | 6.17 |
|  |  | 50% | 6.37 | 10.64 | 19.11 | 12.82 | 18.23 | 47.90 | 7.17 | 10.40 | 19.29 |
|  |  | 95% | 10.30 | 27.30 | 77.54 | 18.68 | 27.11 | 67.92 | 22.97 | 55.03 | 680.83 |
|  | UL packet-Latency CDF | Mean | 4.14 | 9.49 | 22.92 | 5.75 | 8.70 | 25.75 | 3.38 | 4.70 | 7.65 |
|  |  | 5% | 3.06 | 3.74 | 6.37 | 2.73 | 4.18 | 16.39 | 1.75 | 2.36 | 3.47 |
|  |  | 50% | 3.55 | 8.23 | 16.10 | 5.80 | 8.81 | 25.35 | 3.16 | 4.31 | 6.68 |
|  |  | 95% | 6.95 | 19.65 | 56.66 | 7.82 | 11.88 | 32.59 | 7.53 | 11.09 | 22.71 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 6.96 | 15.07 | 110.60 | 13.5668 | 20.045 | 51.1411 | 7.33 | 11.04 | 29.60 |
|  |  | 5% | 5.23 | 6.41 | 9.03 | 9.5954 | 14.6248 | 39.5611 | 4.66 | 5.01 | 5.36 |
|  |  | 50% | 6.44 | 12.03 | 31.17 | 13.3037 | 18.9096 | 48.8513 | 6.04 | 8.77 | 14.99 |
|  |  | 95% | 10.45 | 33.64 | 361.25 | 18.5924 | 28.4302 | 70.1284 | 19.12 | 40.38 | 217.80 |
|  | UL packet-Latency CDF | Mean | 4.11 | 9.25 | 17.96 | 5.4809 | 8.0292 | 24.3161 | 3.29 | 4.52 | 9.04 |
|  |  | 5% | 3.06 | 3.91 | 6.79 | 3.3222 | 4.9659 | 17.1724 | 1.72 | 2.24 | 3.02 |
|  |  | 50% | 3.57 | 8.18 | 14.04 | 5.3921 | 7.8355 | 23.871 | 3.07 | 3.99 | 7.12 |
|  |  | 95% | 6.69 | 18.39 | 41.22 | 7.061 | 10.5007 | 29.3367 | 7.51 | 11.72 | 36.71 |

Evaluation results of dynamic SBFD Option 3 assuming small packet size are summarized in Table 7.4.1.1.1.3.

Table B.3.1.1.1-3: Results of Indoor (FR1) dynamic SBFD vs. dynamic TDD, {XXXXU} vs. {FFFFU}

(dynamic SBFD Option 3, small packet size)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), dynamic SBFD slot configuration {XXXXU}, dynamic TDD slot configuration {FFFFU}, dynamic SBFD Option 3, DL: 4Kbytes, UL: 1Kbyte) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | [33] |  |  |
|  |  |  | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 6.63% | 21.12% | 43.58% |
|  |  | Type-2 | 9.66% | 30.79% | 63.54% |
|  | UL RU | Type-1 | 1.78% | 5.09% | 10.63% |
|  |  | Type-2 | 6.24% | 17.83% | 37.21% |
| Dynamic SBFD | DL RU | Type-1 | 6.63% | 19.47% | 41.58% |
|  |  | Type-2 | 9.18% | 27.02% | 58.52% |
|  | UL RU | Type-1 | 1.88% | 5.47% | 11.58% |
|  |  | Type-2 | 7.25% | 20.65% | 41.58% |
| Baseline | DL average-UPT CDF | Mean | 44.19 | 43.04 | 40.15 |
|  |  | 5% | 40.60 | 40.34 | 31.94 |
|  |  | 50% | 43.95 | 42.68 | 41.03 |
|  |  | 95% | 48.50 | 47.34 | 46.96 |
|  | UL average-UPT CDF | Mean | 2.43 | 2.43 | 2.38 |
|  |  | 5% | 2.32 | 2.37 | 2.25 |
|  |  | 50% | 2.42 | 2.42 | 2.39 |
|  |  | 95% | 2.54 | 2.50 | 2.47 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 39.38 | 38.46 | 36.61 |
|  |  | 5% | 37.25 | 36.77 | 33.05 |
|  |  | 50% | 39.24 | 38.05 | 36.56 |
|  |  | 95% | 42.31 | 41.37 | 40.39 |
|  | UL average-UPT CDF | Mean | 3.31 | 3.22 | 3.23 |
|  |  | 5% | 3.04 | 2.90 | 2.70 |
|  |  | 50% | 3.31 | 3.22 | 3.26 |
|  |  | 95% | 3.56 | 3.58 | 3.62 |
| Baseline | DL packet-Latency CDF | Mean | 1.03 | 1.08 | 1.26 |
|  |  | 5% | 0.35 | 0.36 | 0.39 |
|  |  | 50% | 0.79 | 0.80 | 0.85 |
|  |  | 95% | 4.38 | 4.41 | 5.16 |
|  | UL packet-Latency CDF | Mean | 3.58 | 3.59 | 3.65 |
|  |  | 5% | 2.20 | 2.20 | 2.23 |
|  |  | 50% | 3.46 | 3.47 | 3.54 |
|  |  | 95% | 6.98 | 6.96 | 6.94 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 1.04 | 1.05 | 1.13 |
|  |  | 5% | 0.52 | 0.52 | 0.53 |
|  |  | 50% | 0.82 | 0.84 | 0.87 |
|  |  | 95% | 4.17 | 3.98 | 4.12 |
|  | UL packet-Latency CDF | Mean | 2.94 | 3.05 | 3.12 |
|  |  | 5% | 1.22 | 1.25 | 1.24 |
|  |  | 50% | 2.81 | 2.85 | 2.81 |
|  |  | 95% | 6.90 | 7.44 | 8.69 |

Table B.3.1.1.1-4: Results of Indoor (FR1) dynamic SBFD vs. dynamic TDD, {XXXXX} vs. {FFFFF}

(dynamic SBFD Option 3, large & small packet size)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), dynamic SBFD slot configuration {XXXXX}, dynamic TDD slot configuration {FFFFF}, dynamic SBFD Option 3) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [33] |  |  |  |  |  |
|  |  |  | DL: 4Kbytes, UL: 1Kbyte |  |  | DL: 0.5Mbytes, UL: 0.125Mbyte |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 6.59% | 20.16% | 41.19% | 7.07% | 20.82% | 51.42% |
|  |  | Type-2 | 7.68% | 23.54% | 50.17% | 8.38% | 25.40% | 63.27% |
|  | UL RU | Type-1 | 1.75% | 5.04% | 10.27% | 1.76% | 4.53% | 10.27% |
|  |  | Type-2 | 15.41% | 43.84% | 68.28% | 13.88% | 29.76% | 64.68% |
| Dynamic SBFD | DL RU | Type-1 | 6.62% | 19.32% | 40.77% | 7.05% | 19.70% | 47.40% |
|  |  | Type-2 | 8.51% | 25.96% | 58.08% | 9.17% | 25.55% | 60.79% |
|  | UL RU | Type-1 | 2.02% | 5.97% | 11.86% | 1.77% | 4.61% | 10.62% |
|  |  | Type-2 | 9.83% | 25.03% | 42.16% | 8.35% | 21.76% | 51.53% |
| Baseline | DL average-UPT CDF | Mean | 46.76 | 45.34 | 42.89 | 716.79 | 556.74 | 376.87 |
|  |  | 5% | 43.23 | 42.21 | 37.14 | 518.61 | 312.52 | 144.71 |
|  |  | 50% | 46.41 | 44.73 | 42.79 | 737.23 | 550.36 | 359.71 |
|  |  | 95% | 51.28 | 50.49 | 49.43 | 841.36 | 776.57 | 682.54 |
|  | UL average-UPT CDF | Mean | 2.14 | 2.06 | 1.95 | 343.71 | 245.36 | 143.11 |
|  |  | 5% | 2.04 | 1.93 | 1.56 | 324.88 | 221.03 | 90.98 |
|  |  | 50% | 2.13 | 2.06 | 1.99 | 343.91 | 246.96 | 144.31 |
|  |  | 95% | 2.25 | 2.18 | 2.19 | 361.23 | 265.78 | 185.22 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 41.36 | 40.41 | 37.87 | 723.63 | 587.23 | 423.80 |
|  |  | 5% | 39.38 | 39.09 | 34.34 | 525.48 | 367.89 | 175.00 |
|  |  | 50% | 41.18 | 40.09 | 38.05 | 736.23 | 587.52 | 407.40 |
|  |  | 95% | 43.82 | 42.80 | 41.22 | 848.71 | 782.22 | 702.63 |
|  | UL average-UPT CDF | Mean | 5.00 | 4.53 | 3.85 | 349.28 | 265.01 | 169.29 |
|  |  | 5% | 4.73 | 4.31 | 3.41 | 328.90 | 238.59 | 119.28 |
|  |  | 50% | 4.98 | 4.49 | 3.86 | 349.88 | 264.97 | 169.32 |
|  |  | 95% | 5.33 | 4.83 | 4.20 | 366.09 | 289.76 | 218.66 |
| Baseline | DL packet-Latency CDF | Mean | 1.09 | 1.18 | 1.26 | 6.28 | 9.05 | 19.25 |
|  |  | 5% | 0.36 | 0.37 | 0.39 | 4.14 | 4.33 | 4.83 |
|  |  | 50% | 0.72 | 0.74 | 0.78 | 4.98 | 7.43 | 11.79 |
|  |  | 95% | 6.39 | 6.44 | 6.46 | 16.34 | 29.76 | 118.36 |
|  | UL packet-Latency CDF | Mean | 4.04 | 4.28 | 4.85 | 3.45 | 5.26 | 14.09 |
|  |  | 5% | 2.60 | 2.63 | 2.58 | 1.73 | 2.30 | 3.01 |
|  |  | 50% | 3.89 | 4.02 | 4.23 | 3.09 | 4.14 | 8.14 |
|  |  | 95% | 7.72 | 9.46 | 14.61 | 10.81 | 16.95 | 98.24 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 0.88 | 0.90 | 1.00 | 6.11 | 8.27 | 15.79 |
|  |  | 5% | 0.53 | 0.53 | 0.54 | 4.14 | 4.30 | 4.69 |
|  |  | 50% | 0.79 | 0.80 | 0.85 | 5.00 | 6.85 | 10.20 |
|  |  | 95% | 2.66 | 2.43 | 2.74 | 14.65 | 25.57 | 86.06 |
|  | UL packet-Latency CDF | Mean | 1.84 | 2.18 | 2.70 | 3.30 | 4.69 | 10.75 |
|  |  | 5% | 1.11 | 1.12 | 1.15 | 1.72 | 2.23 | 2.82 |
|  |  | 50% | 1.51 | 1.72 | 2.24 | 3.09 | 3.72 | 6.45 |
|  |  | 95% | 5.05 | 6.78 | 8.53 | 9.11 | 17.56 | 71.46 |

B.3.1.1.2	Dynamic SBFD vs. semi-static SBFD

Table B.3.1.1.2-1: Results of Indoor (FR1) dynamic SBFD vs. semi-static SBFD, {XXXXU}

(dynamic SBFD Option 2, large packet size)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), SBFD slot configuration {XXXXU}, dynamic SBFD Option 2, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [21] |  |  | [24] |  |  | [41] |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 8.10% | 15.90% | 41.40% | 5.17% | 20.78% | 49.78% |  |  |  |
|  |  | Type-2 | 13.30% | 26.10% | 67.90% | 8.42% | 33.97% | 81.61% | 5.34% | 45.95% | 79.35% |
|  | UL RU | Type-1 | 1.70% | 3.40% | 9.90% | 1.51% | 5.19% | 12.92% |  |  |  |
|  |  | Type-2 | 4.70% | 9.50% | 27.40% | 4.22% | 14.52% | 36.14% | 2.34% | 14.75% | 23.69% |
| Dynamic SBFD | DL RU | Type-1 |  |  |  | 5.12% | 18.35% | 44.24% | 2.78% | 11.69% | 27.57% |
|  |  | Type-2 |  |  |  | 6.47% | 23.75% | 60.52% |  |  |  |
|  | UL RU | Type-1 |  |  |  | 1.52% | 5.18% | 12.96% | 1.56% | 5.64% | 11.81% |
|  |  | Type-2 |  |  |  | 4.24% | 14.48% | 36.25% |  |  |  |
| Baseline | DL average-UPT CDF | Mean | 751.4 | 606.5 | 233.5 | 502.71 | 379.23 | 218.77 | 453.69 | 233.72 | 104.10 |
|  |  | 5% | 598.11 | 440.35 | 56.41 | 447.50 | 234.20 | 54.00 | 418.87 | 138.26 | 27.18 |
|  |  | 50% | 752.94 | 597.21 | 229.82 | 506.50 | 390.50 | 206.60 | 456.24 | 226.09 | 84.15 |
|  |  | 95% |  |  |  | 543.90 | 482.95 | 404.00 | 483.93 | 362.21 | 240.11 |
|  | UL average-UPT CDF | Mean | 314.4 | 245 | 138.5 | 153.02 | 145.59 | 130.54 | 163.90 | 135.16 | 116.56 |
|  |  | 5% | 217.39 | 176.45 | 94.97 | 138.80 | 130.95 | 114.90 | 152.54 | 115.37 | 90.40 |
|  |  | 50% | 307.69 | 241.38 | 133.92 | 155.78 | 145.87 | 130.66 | 164.05 | 135.23 | 116.16 |
|  |  | 95% |  |  |  | 159.40 | 157.85 | 146.10 | 175.17 | 154.91 | 143.41 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 1013.1 | 819.5 | 354 | 677.91 | 547.20 | 329.03 | 600.27 | 339.02 | 153.37 |
|  |  | 5% | 810.1 | 604.4 | 76.4 | 605.55 | 391.40 | 132.10 | 552.83 | 229.14 | 16.15 |
|  |  | 50% | 1012.7 | 809.7 | 352.5 | 682.90 | 560.35 | 327.20 | 601.23 | 332.28 | 143.75 |
|  |  | 95% |  |  |  | 730.80 | 658.10 | 519.60 | 643.08 | 477.88 | 335.57 |
|  | UL average-UPT CDF | Mean | 263.8 | 183.8 | 86.5 | 152.92 | 145.76 | 130.89 | 156.54 | 95.91 | 66.30 |
|  |  | 5% | 181.8 | 128.2 | 51.5 | 138.73 | 130.23 | 114.55 | 141.52 | 72.59 | 37.67 |
|  |  | 50% | 258.8 | 179.6 | 82.5 | 155.82 | 146.58 | 131.27 | 156.47 | 94.65 | 64.93 |
|  |  | 95% |  |  |  | 159.33 | 157.78 | 146.67 | 170.59 | 123.74 | 97.27 |
| Baseline | DL packet-Latency CDF | Mean | 5.4 | 6.8 | 23.5 | 8.33 | 14.25 | 54.62 | 9.23 | 25.63 | 184.31 |
|  |  | 5% | 4 | 4 | 5 | 6.99 | 7.08 | 7.50 | 7.18 | 8.56 | 12.61 |
|  |  | 50% | 4.5 | 6 | 14 | 7.59 | 10.09 | 23.26 | 8.50 | 18.88 | 46.24 |
|  |  | 95% |  |  |  | 13.69 | 34.80 | 207.01 | 14.05 | 60.25 | 661.88 |
|  | UL packet-Latency CDF | Mean | 3.3 | 4.2 | 7.6 | 6.62 | 7.24 | 8.92 | 6.30 | 8.24 | 10.24 |
|  |  | 5% | 2 | 2 | 3 | 6.16 | 6.17 | 6.18 | 4.72 | 5.06 | 5.33 |
|  |  | 50% | 3 | 3.5 | 6.5 | 6.38 | 6.42 | 6.52 | 6.12 | 7.28 | 8.64 |
|  |  | 95% |  |  |  | 8.82 | 11.48 | 18.21 | 8.59 | 14.44 | 20.56 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 4 | 5 | 16.5 | 6.13 | 8.86 | 23.12 | 6.99 | 15.60 | 126.59 |
|  |  | 5% | 3 | 3 | 4 | 5.07 | 5.23 | 5.68 | 5.24 | 6.53 | 9.32 |
|  |  | 50% | 3.5 | 4.5 | 9.5 | 5.68 | 6.63 | 14.12 | 6.47 | 12.39 | 33.64 |
|  |  | 95% |  |  |  | 10.08 | 19.45 | 67.61 | 10.47 | 35.11 | 419.15 |
|  | UL packet-Latency CDF | Mean | 4 | 5.7 | 12.9 | 6.64 | 7.25 | 8.93 | 6.68 | 12.43 | 21.64 |
|  |  | 5% | 2 | 2.5 | 5 | 6.16 | 6.17 | 6.18 | 4.78 | 6.24 | 7.81 |
|  |  | 50% | 3.5 | 5 | 10.5 | 6.38 | 6.42 | 6.52 | 6.38 | 10.70 | 16.20 |
|  |  | 95% |  |  |  | 9.03 | 11.50 | 18.67 | 9.51 | 24.39 | 51.33 |

Table B.3.1.1.2-2: Results of Indoor (FR1) dynamic SBFD vs. semi-static SBFD, {XXXXU}

(dynamic SBFD Option 3, large packet size)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), SBFD slot configuration {XXXXU}, dynamic SBFD Option 3, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [33] |  |  | [41] |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 7.02% | 20.38% | 48.20% |  |  |  |
|  |  | Type-2 | 11.19% | 32.51% | 76.86% | 5.34% | 45.95% | 79.35% |
|  | UL RU | Type-1 | 1.79% | 4.70% | 9.78% |  |  |  |
|  |  | Type-2 | 4.99% | 13.11% | 27.30% | 2.34% | 14.75% | 23.69% |
| Dynamic SBFD | DL RU | Type-1 | 7.31% | 21.30% | 52.44% | 2.75% | 11.27% | 26.39% |
|  |  | Type-2 | 10.25% | 30.53% | 73.65% |  |  |  |
|  | UL RU | Type-1 | 1.77% | 4.55% | 10.50% | 1.61% | 6.64% | 13.96% |
|  |  | Type-2 | 6.83% | 16.45% | 39.91% |  |  |  |
| Baseline | DL average-UPT CDF | Mean | 517.48 | 399.86 | 263.14 | 453.69 | 233.72 | 104.10 |
|  |  | 5% | 398.42 | 243.01 | 73.87 | 418.87 | 138.26 | 27.18 |
|  |  | 50% | 535.09 | 397.73 | 249.25 | 456.24 | 226.09 | 84.15 |
|  |  | 95% | 588.76 | 542.22 | 482.53 | 483.93 | 362.21 | 240.11 |
|  | UL average-UPT CDF | Mean | 273.65 | 240.01 | 199.86 | 163.90 | 135.16 | 116.56 |
|  |  | 5% | 254.74 | 201.60 | 145.17 | 152.54 | 115.37 | 90.40 |
|  |  | 50% | 275.68 | 241.23 | 198.32 | 164.05 | 135.23 | 116.16 |
|  |  | 95% | 286.94 | 274.18 | 255.45 | 175.17 | 154.91 | 143.41 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 617.36 | 473.57 | 312.35 | 602.93 | 348.36 | 162.59 |
|  |  | 5% | 405.50 | 257.50 | 102.17 | 558.63 | 238.37 | 20.34 |
|  |  | 50% | 642.59 | 451.63 | 287.45 | 603.09 | 341.71 | 152.33 |
|  |  | 95% | 744.08 | 694.07 | 617.45 | 645.22 | 485.78 | 344.86 |
|  | UL average-UPT CDF | Mean | 347.74 | 258.88 | 154.82 | 259.62 | 133.72 | 77.20 |
|  |  | 5% | 326.69 | 230.26 | 105.60 | 234.63 | 96.19 | 44.93 |
|  |  | 50% | 348.23 | 259.25 | 157.76 | 260.77 | 132.26 | 75.16 |
|  |  | 95% | 365.87 | 285.52 | 196.30 | 281.68 | 177.52 | 119.73 |
| Baseline | DL packet-Latency CDF | Mean | 8.56 | 12.99 | 42.15 | 9.23 | 25.63 | 184.31 |
|  |  | 5% | 6.08 | 6.21 | 6.44 | 7.18 | 8.56 | 12.61 |
|  |  | 50% | 6.92 | 10.01 | 18.00 | 8.50 | 18.88 | 46.24 |
|  |  | 95% | 22.32 | 48.43 | 350.63 | 14.05 | 60.25 | 661.88 |
|  | UL packet-Latency CDF | Mean | 3.74 | 4.44 | 5.79 | 6.30 | 8.24 | 10.24 |
|  |  | 5% | 3.11 | 3.13 | 3.19 | 4.72 | 5.06 | 5.33 |
|  |  | 50% | 3.46 | 3.82 | 4.72 | 6.12 | 7.28 | 8.64 |
|  |  | 95% | 7.51 | 10.78 | 18.38 | 8.59 | 14.44 | 20.56 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 7.33 | 11.04 | 29.60 | 6.96 | 15.07 | 110.60 |
|  |  | 5% | 4.66 | 5.01 | 5.36 | 5.23 | 6.41 | 9.03 |
|  |  | 50% | 6.04 | 8.77 | 14.99 | 6.44 | 12.03 | 31.17 |
|  |  | 95% | 19.12 | 40.38 | 217.80 | 10.45 | 33.64 | 361.25 |
|  | UL packet-Latency CDF | Mean | 3.29 | 4.52 | 9.04 | 4.11 | 9.25 | 17.96 |
|  |  | 5% | 1.72 | 2.24 | 3.02 | 3.06 | 3.91 | 6.79 |
|  |  | 50% | 3.07 | 3.99 | 7.12 | 3.57 | 8.18 | 14.04 |
|  |  | 95% | 7.51 | 11.72 | 36.71 | 6.69 | 18.39 | 41.22 |

Table B.3.1.1.2-3: Results of Indoor (FR1) dynamic SBFD vs. semi-static SBFD, {XXXXU}

(dynamic SBFD Option 2, small packet size)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), SBFD slot configuration {XXXXU}, dynamic SBFD Option 3, DL: 4Kbytes, UL: 1Kbyte) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | [33] |  |  |
|  |  |  | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 6.65% | 21.30% | 43.47% |
|  |  | Type-2 | 10.61% | 33.96% | 69.32% |
|  | UL RU | Type-1 | 2.06% | 6.35% | 12.49% |
|  |  | Type-2 | 5.75% | 17.74% | 34.86% |
| Dynamic SBFD | DL RU | Type-1 | 6.63% | 19.47% | 41.58% |
|  |  | Type-2 | 9.18% | 27.02% | 58.52% |
|  | UL RU | Type-1 | 1.88% | 5.47% | 11.58% |
|  |  | Type-2 | 7.25% | 20.65% | 41.58% |
| Baseline | DL average-UPT CDF | Mean | 38.75 | 38.02 | 35.27 |
|  |  | 5% | 36.80 | 36.59 | 27.98 |
|  |  | 50% | 38.61 | 37.77 | 36.17 |
|  |  | 95% | 41.15 | 40.31 | 39.70 |
|  | UL average-UPT CDF | Mean | 5.19 | 4.63 | 4.09 |
|  |  | 5% | 4.95 | 4.43 | 3.86 |
|  |  | 50% | 5.18 | 4.61 | 4.08 |
|  |  | 95% | 5.47 | 4.88 | 4.36 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 39.38 | 38.46 | 36.61 |
|  |  | 5% | 37.25 | 36.77 | 33.05 |
|  |  | 50% | 39.24 | 38.05 | 36.56 |
|  |  | 95% | 42.31 | 41.37 | 40.39 |
|  | UL average-UPT CDF | Mean | 3.31 | 3.22 | 3.23 |
|  |  | 5% | 3.04 | 2.90 | 2.70 |
|  |  | 50% | 3.31 | 3.22 | 3.26 |
|  |  | 95% | 3.56 | 3.58 | 3.62 |
| Baseline | DL packet-Latency CDF | Mean | 0.95 | 0.98 | 1.18 |
|  |  | 5% | 0.53 | 0.54 | 0.54 |
|  |  | 50% | 0.84 | 0.85 | 0.91 |
|  |  | 95% | 2.47 | 2.49 | 4.75 |
|  | UL packet-Latency CDF | Mean | 1.71 | 2.01 | 2.37 |
|  |  | 5% | 1.11 | 1.12 | 1.15 |
|  |  | 50% | 1.48 | 1.72 | 2.04 |
|  |  | 95% | 4.10 | 5.42 | 6.52 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 1.04 | 1.05 | 1.13 |
|  |  | 5% | 0.52 | 0.52 | 0.53 |
|  |  | 50% | 0.82 | 0.84 | 0.87 |
|  |  | 95% | 4.17 | 3.98 | 4.12 |
|  | UL packet-Latency CDF | Mean | 2.94 | 3.05 | 3.12 |
|  |  | 5% | 1.22 | 1.25 | 1.24 |
|  |  | 50% | 2.81 | 2.85 | 2.81 |
|  |  | 95% | 6.90 | 7.44 | 8.69 |

Table B.3.1.1.2-4: Results of Indoor (FR1) dynamic SBFD vs. semi-static SBFD, {XXXXX}

(dynamic SBFD Option 3, large packet size)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), SBFD slot configuration {XXXXX}, dynamic SBFD Option 3, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [24] |  |  | [33] |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 6.73% | 24.72% | 59.81% | 6.91% | 19.40% | 45.47% |
|  |  | Type-2 | 5.13% | 18.83% | 45.57% | 8.82% | 24.75% | 58.01% |
|  | UL RU | Type-1 | 7.49% | 25.82% | 64.19% | 1.89% | 5.21% | 11.25% |
|  |  | Type-2 | 1.51% | 5.20% | 12.93% | 9.53% | 26.34% | 56.89% |
| Dynamic SBFD | DL RU | Type-1 | 5.11% | 17.78% | 40.92% | 7.05% | 19.70% | 47.40% |
|  |  | Type-2 | 5.15% | 18.41% | 46.09% | 9.17% | 25.55% | 60.79% |
|  | UL RU | Type-1 | 1.51% | 5.20% | 12.91% | 1.77% | 4.61% | 10.62% |
|  |  | Type-2 | 1.68% | 7.86% | 35.49% | 8.35% | 21.76% | 51.53% |
| Baseline | DL average-UPT CDF | Mean | 618.23 | 515.66 | 346.68 | 654.93 | 534.46 | 383.18 |
|  |  | 5% | 560.60 | 378.90 | 147.00 | 493.56 | 331.13 | 160.99 |
|  |  | 50% | 623.17 | 526.70 | 351.50 | 677.10 | 537.40 | 369.32 |
|  |  | 95% | 655.65 | 611.80 | 526.70 | 731.63 | 685.88 | 622.66 |
|  | UL average-UPT CDF | Mean | 91.59 | 83.12 | 64.31 | 166.85 | 137.36 | 95.39 |
|  |  | 5% | 79.70 | 70.70 | 49.40 | 149.65 | 112.38 | 60.86 |
|  |  | 50% | 94.28 | 83.50 | 63.83 | 168.64 | 137.40 | 92.89 |
|  |  | 95% | 97.23 | 94.10 | 79.40 | 177.12 | 161.09 | 134.39 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 820.12 | 696.13 | 457.49 | 723.63 | 587.23 | 423.80 |
|  |  | 5% | 746.90 | 527.10 | 212.70 | 525.48 | 367.89 | 175.00 |
|  |  | 50% | 825.00 | 710.40 | 463.10 | 736.23 | 587.52 | 407.40 |
|  |  | 95% | 868.75 | 808.90 | 669.00 | 848.71 | 782.22 | 702.63 |
|  | UL average-UPT CDF | Mean | 333.72 | 293.84 | 193.82 | 349.28 | 265.01 | 169.29 |
|  |  | 5% | 283.60 | 239.50 | 129.00 | 328.90 | 238.59 | 119.28 |
|  |  | 50% | 341.41 | 295.85 | 194.07 | 349.88 | 264.97 | 169.32 |
|  |  | 95% | 361.10 | 343.20 | 262.10 | 366.09 | 289.76 | 218.66 |
| Baseline | DL packet-Latency CDF | Mean | 6.68 | 9.38 | 20.82 | 6.64 | 9.14 | 18.06 |
|  |  | 5% | 5.95 | 5.97 | 6.06 | 5.04 | 5.09 | 5.23 |
|  |  | 50% | 6.21 | 6.38 | 12.90 | 5.43 | 7.31 | 11.17 |
|  |  | 95% | 10.83 | 20.65 | 63.67 | 16.48 | 30.48 | 100.52 |
|  | UL packet-Latency CDF | Mean | 11.28 | 13.47 | 23.74 | 6.41 | 8.66 | 18.04 |
|  |  | 5% | 10.17 | 10.17 | 10.20 | 5.11 | 5.16 | 5.44 |
|  |  | 50% | 10.40 | 10.49 | 15.66 | 5.45 | 6.87 | 10.96 |
|  |  | 95% | 19.01 | 26.41 | 67.40 | 15.19 | 27.89 | 99.17 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 5.01 | 6.77 | 15.44 | 6.11 | 8.27 | 15.79 |
|  |  | 5% | 4.45 | 4.47 | 4.61 | 4.14 | 4.30 | 4.69 |
|  |  | 50% | 4.71 | 4.89 | 9.30 | 5.00 | 6.85 | 10.20 |
|  |  | 95% | 7.66 | 14.62 | 45.74 | 14.65 | 25.57 | 86.06 |
|  | UL packet-Latency CDF | Mean | 3.14 | 4.18 | 10.91 | 3.30 | 4.69 | 10.75 |
|  |  | 5% | 2.66 | 2.67 | 2.70 | 1.72 | 2.23 | 2.82 |
|  |  | 50% | 2.88 | 2.97 | 6.64 | 3.09 | 3.72 | 6.45 |
|  |  | 95% | 5.21 | 10.31 | 34.58 | 9.11 | 17.56 | 71.46 |

Table B.3.1.1.2-5: Results of Indoor (FR1) dynamic SBFD vs. semi-static SBFD, {XXXXX}

(dynamic SBFD Option 3, small packet size)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), SBFD slot configuration {XXXXX}, dynamic SBFD Option 3, DL: 4Kbytes, UL: 1Kbyte) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | [33] |  |  |
|  |  |  | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 6.71% | 19.11% | 39.72% |
|  |  | Type-2 | 8.56% | 24.38% | 50.67% |
|  | UL RU | Type-1 | 1.98% | 5.90% | 11.77% |
|  |  | Type-2 | 10.02% | 29.81% | 59.53% |
| Dynamic SBFD | DL RU | Type-1 | 6.62% | 19.32% | 40.77% |
|  |  | Type-2 | 8.51% | 25.96% | 58.08% |
|  | UL RU | Type-1 | 2.02% | 5.97% | 11.86% |
|  |  | Type-2 | 9.83% | 25.03% | 42.16% |
| Baseline | DL average-UPT CDF | Mean | 41.85 | 41.24 | 39.97 |
|  |  | 5% | 40.06 | 39.90 | 37.82 |
|  |  | 50% | 41.55 | 40.87 | 39.83 |
|  |  | 95% | 44.42 | 43.78 | 42.89 |
|  | UL average-UPT CDF | Mean | 5.35 | 4.47 | 3.28 |
|  |  | 5% | 5.05 | 4.24 | 2.63 |
|  |  | 50% | 5.33 | 4.45 | 3.32 |
|  |  | 95% | 5.66 | 4.75 | 3.85 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 41.36 | 40.41 | 37.87 |
|  |  | 5% | 39.38 | 39.09 | 34.34 |
|  |  | 50% | 41.18 | 40.09 | 38.05 |
|  |  | 95% | 43.82 | 42.80 | 41.22 |
|  | UL average-UPT CDF | Mean | 5.00 | 4.53 | 3.85 |
|  |  | 5% | 4.73 | 4.31 | 3.41 |
|  |  | 50% | 4.98 | 4.49 | 3.86 |
|  |  | 95% | 5.33 | 4.83 | 4.20 |
| Baseline | DL packet-Latency CDF | Mean | 0.85 | 0.87 | 0.92 |
|  |  | 5% | 0.53 | 0.53 | 0.53 |
|  |  | 50% | 0.78 | 0.79 | 0.81 |
|  |  | 95% | 2.09 | 2.26 | 2.45 |
|  | UL packet-Latency CDF | Mean | 1.68 | 2.27 | 3.79 |
|  |  | 5% | 1.10 | 1.12 | 1.17 |
|  |  | 50% | 1.42 | 1.72 | 2.84 |
|  |  | 95% | 4.53 | 7.36 | 15.79 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 0.88 | 0.90 | 1.00 |
|  |  | 5% | 0.53 | 0.53 | 0.54 |
|  |  | 50% | 0.79 | 0.80 | 0.85 |
|  |  | 95% | 2.66 | 2.43 | 2.74 |
|  | UL packet-Latency CDF | Mean | 1.84 | 2.18 | 2.70 |
|  |  | 5% | 1.11 | 1.12 | 1.15 |
|  |  | 50% | 1.51 | 1.72 | 2.24 |
|  |  | 95% | 5.05 | 6.78 | 8.53 |

#### B.3.1.2	Urban Macro (FR1)

B.3.1.2.1	Dynamic SBFD vs. dynamic TDD

Table B.3.1.2.1-1: Results of Urban Macro (FR1) dynamic SBFD vs. dynamic TDD, {XXXXU} vs. {FFFFU}

(dynamic SBFD Option 2, w/o CLI handling)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), dynamic SBFD slot configuration {XXXXU}, dynamic TDD slot configuration {FFFFU}, dynamic SBFD Option 2, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | [41] |  |  |
|  |  |  | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 5.49% | 21.94% | 42.03% |
|  |  | Type-2 |  |  |  |
|  | UL RU | Type-1 | 1.54% | 6.73% | 12.91% |
|  |  | Type-2 |  |  |  |
| Dynamic SBFD | DL RU | Type-1 | 3.01% | 14.79% | 28.46% |
|  |  | Type-2 |  |  |  |
|  | UL RU | Type-1 | 1.19% | 5.37% | 11.35% |
|  |  | Type-2 |  |  |  |
| Baseline | DL average-UPT CDF | Mean | 502.54 | 376.62 | 259.64 |
|  |  | 5% | 226.73 | 165.44 | 77.15 |
|  |  | 50% | 539.56 | 384.39 | 250.90 |
|  |  | 95% | 643.87 | 565.35 | 472.74 |
|  | UL average-UPT CDF | Mean | 63.94 | 30.38 | 21.70 |
|  |  | 5% | 1.21 | 0.45 | 0.29 |
|  |  | 50% | 61.90 | 28.22 | 18.16 |
|  |  | 95% | 137.28 | 66.23 | 53.27 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 480.35 | 296.34 | 145.15 |
|  |  | 5% | 215.01 | 118.58 | 23.52 |
|  |  | 50% | 512.39 | 295.86 | 122.68 |
|  |  | 95% | 627.11 | 487.85 | 340.79 |
|  | UL average-UPT CDF | Mean | 45.83 | 32.32 | 24.09 |
|  |  | 5% | 1.42 | 0.46 | 0.27 |
|  |  | 50% | 45.58 | 30.55 | 20.35 |
|  |  | 95% | 91.65 | 70.60 | 58.89 |
| Baseline | DL packet-Latency CDF | Mean | 10.21 | 16.53 | 44.69 |
|  |  | 5% | 5.48 | 5.90 | 6.65 |
|  |  | 50% | 7.57 | 10.89 | 18.00 |
|  |  | 95% | 20.39 | 39.51 | 107.62 |
|  | UL packet-Latency CDF | Mean | 77.91 | 123.69 | 171.12 |
|  |  | 5% | 5.29 | 13.71 | 15.45 |
|  |  | 50% | 18.46 | 36.54 | 61.59 |
|  |  | 95% | 363.37 | 587.42 | 765.98 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 10.86 | 23.96 | 164.53 |
|  |  | 5% | 5.53 | 6.73 | 9.04 |
|  |  | 50% | 8.03 | 14.56 | 37.88 |
|  |  | 95% | 22.53 | 61.89 | 578.16 |
|  | UL packet-Latency CDF | Mean | 78.94 | 117.84 | 155.69 |
|  |  | 5% | 9.86 | 12.67 | 14.30 |
|  |  | 50% | 21.71 | 33.59 | 52.55 |
|  |  | 95% | 342.64 | 557.62 | 717.50 |

Table B.3.1.2.1-2: Results of Urban Macro (FR1) dynamic SBFD vs. dynamic TDD, {XXXXU} vs. {FFFFU}

(dynamic SBFD Option 3, w/o CLI handling)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), dynamic SBFD slot configuration {XXXXU}, dynamic TDD slot configuration {FFFFU}, dynamic SBFD Option 3, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [41] |  |  | [33] |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 5.49% | 21.94% | 42.03% | 7.25% | 22.06% | 42.49% |
|  |  | Type-2 |  |  |  | 11.33% | 36.16% | 70.34% |
|  | UL RU | Type-1 | 1.54% | 6.73% | 12.91% | 2.38% | 5.94% | 14.49% |
|  |  | Type-2 |  |  |  | 7.15% | 16.38% | 39.48% |
| Dynamic SBFD | DL RU | Type-1 | 2.93% | 14.32% | 28.09% | 8.58% | 24.19% | 52.23% |
|  |  | Type-2 |  |  |  | 12.89% | 39.65% | 79.51% |
|  | UL RU | Type-1 | 1.30% | 5.77% | 11.59% | 1.23% | 4.21% | 12.57% |
|  |  | Type-2 |  |  |  | 3.96% | 11.36% | 38.44% |
| Baseline | DL average-UPT CDF | Mean | 502.54 | 376.62 | 259.64 | 369.01 | 295.92 | 224.62 |
|  |  | 5% | 226.73 | 165.44 | 77.15 | 119.49 | 88.86 | 45.60 |
|  |  | 50% | 539.56 | 384.39 | 250.90 | 416.04 | 303.34 | 217.71 |
|  |  | 95% | 643.87 | 565.35 | 472.74 | 499.01 | 445.29 | 408.55 |
|  | UL average-UPT CDF | Mean | 63.94 | 30.38 | 21.70 | 143.34 | 108.35 | 79.70 |
|  |  | 5% | 1.21 | 0.45 | 0.29 | 1.02 | 0.71 | 0.29 |
|  |  | 50% | 61.90 | 28.22 | 18.16 | 103.72 | 58.06 | 41.06 |
|  |  | 95% | 137.28 | 66.23 | 53.27 | 335.34 | 318.64 | 262.83 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 479.97 | 293.77 | 144.96 | 385.43 | 301.48 | 219.20 |
|  |  | 5% | 212.32 | 114.35 | 23.25 | 108.79 | 76.96 | 30.18 |
|  |  | 50% | 513.60 | 293.50 | 122.19 | 429.58 | 310.63 | 202.74 |
|  |  | 95% | 628.21 | 485.88 | 340.30 | 534.54 | 464.10 | 435.57 |
|  | UL average-UPT CDF | Mean | 66.59 | 31.22 | 23.71 | 106.85 | 84.31 | 49.24 |
|  |  | 5% | 1.31 | 0.45 | 0.28 | 1.58 | 0.92 | 0.68 |
|  |  | 50% | 64.32 | 29.33 | 20.31 | 62.09 | 34.02 | 14.35 |
|  |  | 95% | 143.49 | 68.74 | 57.39 | 328.89 | 310.54 | 204.09 |
| Baseline | DL packet-Latency CDF | Mean | 10.21 | 16.53 | 44.69 | 17.98 | 52.60 | 110.27 |
|  |  | 5% | 5.48 | 5.90 | 6.65 | 7.03 | 7.31 | 7.60 |
|  |  | 50% | 7.57 | 10.89 | 18.00 | 9.99 | 14.54 | 20.81 |
|  |  | 95% | 20.39 | 39.51 | 107.62 | 131.37 | 261.70 | 710.02 |
|  | UL packet-Latency CDF | Mean | 77.91 | 123.69 | 171.12 | 45.55 | 64.47 | 111.82 |
|  |  | 5% | 5.29 | 13.71 | 15.45 | 2.52 | 2.77 | 3.61 |
|  |  | 50% | 18.46 | 36.54 | 61.59 | 9.39 | 17.41 | 22.76 |
|  |  | 95% | 363.37 | 587.42 | 765.98 | 413.42 | 520.75 | 1187.36 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 10.85 | 23.82 | 162.95 | 21.24 | 36.48 | 172.13 |
|  |  | 5% | 5.54 | 6.79 | 9.06 | 6.61 | 6.82 | 7.31 |
|  |  | 50% | 8.04 | 14.75 | 37.91 | 9.79 | 14.13 | 21.94 |
|  |  | 95% | 22.55 | 61.78 | 565.67 | 197.45 | 452.46 | 2977.19 |
|  | UL packet-Latency CDF | Mean | 75.02 | 120.07 | 157.32 | 89.34 | 146.63 | 390.62 |
|  |  | 5% | 5.11 | 13.30 | 14.64 | 2.77 | 3.00 | 4.88 |
|  |  | 50% | 17.92 | 34.91 | 53.80 | 14.93 | 30.20 | 70.05 |
|  |  | 95% | 342.21 | 566.72 | 718.10 | 1296.57 | 1542.02 | 4849.48 |

Table B.3.1.2.1-3: Results of Urban Macro (FR1) dynamic SBFD vs. dynamic TDD, {XXXXU} vs. {FFFFU}

(dynamic SBFD Option 3, w/ CLI handling)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), dynamic SBFD slot configuration {XXXXU}, dynamic TDD slot configuration {FFFFU}, dynamic SBFD Option 3, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | [33] |  |  |
|  |  |  | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 7.25% | 22.06% | 42.49% |
|  |  | Type-2 | 11.33% | 36.16% | 70.34% |
|  | UL RU | Type-1 | 2.38% | 5.94% | 14.49% |
|  |  | Type-2 | 7.15% | 16.38% | 39.48% |
| Dynamic SBFD | DL RU | Type-1 | 8.93% | 21.60% | 41.03% |
|  |  | Type-2 | 13.17% | 36.94% | 73.39% |
|  | UL RU | Type-1 | 0.99% | 4.07% | 13.90% |
|  |  | Type-2 | 3.30% | 10.26% | 32.53% |
| Baseline | DL average-UPT CDF | Mean | 369.01 | 295.92 | 224.62 |
|  |  | 5% | 119.49 | 88.86 | 45.60 |
|  |  | 50% | 416.04 | 303.34 | 217.71 |
|  |  | 95% | 499.01 | 445.29 | 408.55 |
|  | UL average-UPT CDF | Mean | 143.34 | 108.35 | 79.70 |
|  |  | 5% | 1.02 | 0.71 | 0.29 |
|  |  | 50% | 103.72 | 58.06 | 41.06 |
|  |  | 95% | 335.34 | 318.64 | 262.83 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 399.96 | 302.48 | 201.42 |
|  |  | 5% | 139.63 | 83.03 | 39.60 |
|  |  | 50% | 444.45 | 321.85 | 197.30 |
|  |  | 95% | 538.87 | 462.29 | 378.81 |
|  | UL average-UPT CDF | Mean | 106.79 | 84.12 | 61.22 |
|  |  | 5% | 2.14 | 0.92 | 0.61 |
|  |  | 50% | 63.34 | 34.81 | 16.60 |
|  |  | 95% | 324.93 | 317.17 | 269.63 |
| Baseline | DL packet-Latency CDF | Mean | 17.98 | 52.60 | 110.27 |
|  |  | 5% | 7.03 | 7.31 | 7.60 |
|  |  | 50% | 9.99 | 14.54 | 20.81 |
|  |  | 95% | 131.37 | 261.70 | 710.02 |
|  | UL packet-Latency CDF | Mean | 45.55 | 64.47 | 111.82 |
|  |  | 5% | 2.52 | 2.77 | 3.61 |
|  |  | 50% | 9.39 | 17.41 | 22.76 |
|  |  | 95% | 413.42 | 520.75 | 1187.36 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 18.90 | 39.80 | 107.66 |
|  |  | 5% | 6.60 | 6.81 | 7.87 |
|  |  | 50% | 9.33 | 14.28 | 24.03 |
|  |  | 95% | 203.51 | 299.22 | 1170.20 |
|  | UL packet-Latency CDF | Mean | 72.44 | 140.57 | 254.26 |
|  |  | 5% | 2.85 | 3.00 | 3.62 |
|  |  | 50% | 15.24 | 28.78 | 57.02 |
|  |  | 95% | 1051.71 | 1480.21 | 2425.40 |

Table B.3.1.2.1-4: Results of Urban Macro (FR1) dynamic SBFD vs. dynamic TDD, {XXXXX} vs. {FFFFF}

(dynamic SBFD Option 3, w/ & w/o CLI handling)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), dynamic SBFD slot configuration {XXXXX}, dynamic TDD slot configuration {FFFFF}, dynamic SBFD Option 3, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [33] |  |  |  |  |  |
|  |  |  | w/o CLI handling |  |  | w/ CLI handling |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 7.28% | 20.00% | 40.95% | 7.28% | 20.00% | 40.95% |
|  |  | Type-2 | 8.75% | 26.10% | 60.56% | 8.75% | 26.10% | 60.56% |
|  | UL RU | Type-1 | 1.97% | 5.36% | 13.78% | 1.97% | 5.36% | 13.78% |
|  |  | Type-2 | 14.00% | 25.89% | 46.67% | 14.00% | 25.89% | 46.67% |
| Dynamic SBFD | DL RU | Type-1 | 8.50% | 25.37% | 62.13% | 7.94% | 20.39% | 40.58% |
|  |  | Type-2 | 10.73% | 36.56% | 85.94% | 10.02% | 29.85% | 67.70% |
|  | UL RU | Type-1 | 1.19% | 4.51% | 14.99% | 1.12% | 3.80% | 15.32% |
|  |  | Type-2 | 6.17% | 15.51% | 58.51% | 5.78% | 12.55% | 39.84% |
| Baseline | DL average-UPT CDF | Mean | 466.17 | 383.32 | 270.69 | 466.17 | 383.32 | 270.69 |
|  |  | 5% | 178.97 | 132.44 | 67.49 | 178.97 | 132.44 | 67.49 |
|  |  | 50% | 521.93 | 406.46 | 277.80 | 521.93 | 406.46 | 277.80 |
|  |  | 95% | 611.32 | 556.55 | 445.93 | 611.32 | 556.55 | 445.93 |
|  | UL average-UPT CDF | Mean | 147.15 | 118.11 | 81.29 | 147.15 | 118.11 | 81.29 |
|  |  | 5% | 2.39 | 1.19 | 0.33 | 2.39 | 1.19 | 0.33 |
|  |  | 50% | 111.07 | 65.84 | 40.11 | 111.07 | 65.84 | 40.11 |
|  |  | 95% | 345.94 | 323.93 | 272.06 | 345.94 | 323.93 | 272.06 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 466.78 | 355.20 | 212.35 | 484.20 | 374.00 | 227.03 |
|  |  | 5% | 135.30 | 96.84 | 5.62 | 153.37 | 113.88 | 47.42 |
|  |  | 50% | 515.11 | 371.23 | 197.33 | 542.74 | 396.79 | 224.97 |
|  |  | 95% | 655.43 | 550.05 | 461.72 | 657.57 | 584.99 | 416.11 |
|  | UL average-UPT CDF | Mean | 103.20 | 80.71 | 39.33 | 110.05 | 90.84 | 60.16 |
|  |  | 5% | 1.52 | 1.02 | 0.03 | 2.14 | 1.29 | 0.53 |
|  |  | 50% | 58.99 | 31.73 | 12.63 | 71.59 | 42.31 | 17.99 |
|  |  | 95% | 331.81 | 310.18 | 161.79 | 333.32 | 318.55 | 242.89 |
| Baseline | DL packet-Latency CDF | Mean | 13.87 | 33.20 | 69.67 | 13.87 | 33.20 | 69.67 |
|  |  | 5% | 6.01 | 6.05 | 6.30 | 6.01 | 6.05 | 6.30 |
|  |  | 50% | 7.66 | 11.01 | 17.05 | 7.66 | 11.01 | 17.05 |
|  |  | 95% | 94.36 | 155.63 | 441.19 | 94.36 | 155.63 | 441.19 |
|  | UL packet-Latency CDF | Mean | 37.19 | 77.20 | 158.80 | 37.19 | 77.20 | 158.80 |
|  |  | 5% | 2.49 | 2.72 | 3.37 | 2.49 | 2.72 | 3.37 |
|  |  | 50% | 8.33 | 15.17 | 24.12 | 8.33 | 15.17 | 24.12 |
|  |  | 95% | 410.82 | 699.43 | 2306.07 | 410.82 | 699.43 | 2306.07 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 17.84 | 62.56 | 583.02 | 16.45 | 39.62 | 129.17 |
|  |  | 5% | 5.54 | 5.65 | 6.79 | 5.54 | 5.61 | 7.21 |
|  |  | 50% | 8.22 | 12.06 | 21.66 | 7.60 | 11.53 | 20.62 |
|  |  | 95% | 180.99 | 713.22 | 14870.29 | 152.06 | 225.13 | 1528.16 |
|  | UL packet-Latency CDF | Mean | 86.90 | 180.18 | 1078.56 | 75.60 | 148.70 | 313.52 |
|  |  | 5% | 2.79 | 3.16 | 6.01 | 2.75 | 2.92 | 4.12 |
|  |  | 50% | 16.53 | 31.95 | 64.96 | 13.47 | 23.69 | 54.14 |
|  |  | 95% | 1183.38 | 1629.59 | 19541.56 | 1179.99 | 1445.08 | 2952.58 |

B.3.1.2.2	Dynamic SBFD vs. semi-static SBFD

Table B.3.1.2.2-1: Results of Urban Macro (FR1) dynamic SBFD vs. semi-static SBFD, {XXXXU}

(dynamic SBFD Option 2, w/ & w/o CLI handling)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), SBFD slot configuration {XXXXU}, dynamic SBFD Option 2, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [21] (w/ CLI handling) |  |  | [41] (w/o CLI handling) |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 7.10% | 15.60% | 38.60% |  |  |  |
|  |  | Type-2 | 11.60% | 25.70% | 63.40% | 9.54% | 42.18% | 77.53% |
|  | UL RU | Type-1 | 2.20% | 4.70% | 11.90% |  |  |  |
|  |  | Type-2 | 6.20% | 13.10% | 33.00% | 3.64% | 17.69% | 37.30% |
| Dynamic SBFD | DL RU | Type-1 |  |  |  | 3.01% | 14.79% | 28.46% |
|  |  | Type-2 |  |  |  |  |  |  |
|  | UL RU | Type-1 |  |  |  | 1.19% | 5.37% | 11.35% |
|  |  | Type-2 |  |  |  |  |  |  |
| Baseline | DL average-UPT CDF | Mean | 773.1 | 660.1 | 344 | 376.47 | 244.51 | 121.49 |
|  |  | 5% | 385.54 | 337.54 | 91.88 | 166.59 | 92.09 | 23.36 |
|  |  | 50% | 827.59 | 683.42 | 347.37 | 409.11 | 243.22 | 99.50 |
|  |  | 95% |  |  |  | 475.87 | 400.40 | 289.12 |
|  | UL average-UPT CDF | Mean | 191.8 | 158.8 | 104.6 | 56.95 | 41.64 | 30.11 |
|  |  | 5% | 65.57 | 59.98 | 47.41 | 2.11 | 0.58 | 0.29 |
|  |  | 50% | 200 | 151.9 | 96.15 | 58.44 | 40.25 | 26.85 |
|  |  | 95% |  |  |  | 108.90 | 87.94 | 71.62 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 924.8 | 724.5 | 324.1 | 480.35 | 296.34 | 145.15 |
|  |  | 5% | 439.6 | 390.9 | 105.7 | 215.01 | 118.58 | 23.52 |
|  |  | 50% | 971.3 | 744.2 | 318.7 | 512.39 | 295.86 | 122.68 |
|  |  | 95% |  |  |  | 627.11 | 487.85 | 340.79 |
|  | UL average-UPT CDF | Mean | 173.6 | 152.3 | 103.3 | 45.83 | 32.32 | 24.09 |
|  |  | 5% | 63.8 | 60.8 | 47.3 | 1.42 | 0.46 | 0.27 |
|  |  | 50% | 166.7 | 144.9 | 95.2 | 45.58 | 30.55 | 20.35 |
|  |  | 95% |  |  |  | 91.65 | 70.60 | 58.89 |
| Baseline | DL packet-Latency CDF | Mean | 5.9 | 6.9 | 16.1 | 14.01 | 31.03 | 165.42 |
|  |  | 5% | 4 | 4 | 4 | 7.36 | 8.04 | 10.37 |
|  |  | 50% | 4.5 | 5 | 9.5 | 9.88 | 17.91 | 45.12 |
|  |  | 95% |  |  |  | 29.68 | 80.75 | 566.89 |
|  | UL packet-Latency CDF | Mean | 6.6 | 8 | 11.7 | 62.93 | 101.94 | 133.32 |
|  |  | 5% | 3 | 3 | 3.5 | 8.71 | 10.39 | 12.31 |
|  |  | 50% | 5 | 6.5 | 9.5 | 16.78 | 24.40 | 37.86 |
|  |  | 95% |  |  |  | 262.26 | 520.67 | 659.35 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 4.9 | 6.2 | 16.3 | 10.86 | 23.96 | 164.53 |
|  |  | 5% | 3 | 3 | 4 | 5.53 | 6.73 | 9.04 |
|  |  | 50% | 4 | 4.5 | 10 | 8.03 | 14.56 | 37.88 |
|  |  | 95% |  |  |  | 22.53 | 61.89 | 578.16 |
|  | UL packet-Latency CDF | Mean | 7.1 | 8 | 11.5 | 78.94 | 117.84 | 155.69 |
|  |  | 5% | 3 | 3.5 | 3.5 | 9.86 | 12.67 | 14.30 |
|  |  | 50% | 6 | 6.5 | 9.5 | 21.71 | 33.59 | 52.55 |
|  |  | 95% |  |  |  | 342.64 | 557.62 | 717.50 |

Table B.3.1.2.2-2: Results of Urban Macro (FR1) dynamic SBFD vs. semi-static SBFD, {XXXXU}

(dynamic SBFD Option 3, w/o CLI handling)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), SBFD slot configuration {XXXXU}, dynamic SBFD Option 3, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [33] |  |  | [41] |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 8.66% | 23.17% | 57.60% |  |  |  |
|  |  | Type-2 | 13.81% | 36.95% | 91.85% | 9.54% | 42.18% | 77.53% |
|  | UL RU | Type-1 | 1.36% | 4.52% | 18.49% |  |  |  |
|  |  | Type-2 | 3.79% | 12.62% | 51.63% | 3.64% | 17.69% | 37.30% |
| Dynamic SBFD | DL RU | Type-1 | 8.58% | 24.19% | 52.23% | 2.93% | 14.32% | 28.09% |
|  |  | Type-2 | 12.89% | 39.65% | 79.51% |  |  |  |
|  | UL RU | Type-1 | 1.23% | 4.21% | 12.57% | 1.30% | 5.77% | 11.59% |
|  |  | Type-2 | 3.96% | 11.36% | 38.44% |  |  |  |
| Baseline | DL average-UPT CDF | Mean | 357.67 | 303.61 | 166.54 | 376.47 | 244.51 | 121.49 |
|  |  | 5% | 114.20 | 88.15 | 3.25 | 166.59 | 92.09 | 23.36 |
|  |  | 50% | 399.92 | 323.17 | 138.37 | 409.11 | 243.22 | 99.50 |
|  |  | 95% | 462.57 | 433.09 | 379.70 | 475.87 | 400.40 | 289.12 |
|  | UL average-UPT CDF | Mean | 82.62 | 72.46 | 45.30 | 56.95 | 41.64 | 30.11 |
|  |  | 5% | 1.48 | 0.88 | 0.25 | 2.11 | 0.58 | 0.29 |
|  |  | 50% | 55.90 | 39.89 | 9.10 | 58.44 | 40.25 | 26.85 |
|  |  | 95% | 230.47 | 222.83 | 190.32 | 108.90 | 87.94 | 71.62 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 385.43 | 301.48 | 219.20 | 479.97 | 293.77 | 144.96 |
|  |  | 5% | 108.79 | 76.96 | 30.18 | 212.32 | 114.35 | 23.25 |
|  |  | 50% | 429.58 | 310.63 | 202.74 | 513.60 | 293.50 | 122.19 |
|  |  | 95% | 534.54 | 464.10 | 435.57 | 628.21 | 485.88 | 340.30 |
|  | UL average-UPT CDF | Mean | 106.85 | 84.31 | 49.24 | 66.59 | 31.22 | 23.71 |
|  |  | 5% | 1.58 | 0.92 | 0.68 | 1.31 | 0.45 | 0.28 |
|  |  | 50% | 62.09 | 34.02 | 14.35 | 64.32 | 29.33 | 20.31 |
|  |  | 95% | 328.89 | 310.54 | 204.09 | 143.49 | 68.74 | 57.39 |
| Baseline | DL packet-Latency CDF | Mean | 22.69 | 42.75 | 646.22 | 14.01 | 31.03 | 165.42 |
|  |  | 5% | 8.07 | 8.11 | 8.33 | 7.36 | 8.04 | 10.37 |
|  |  | 50% | 9.84 | 13.03 | 32.70 | 9.88 | 17.91 | 45.12 |
|  |  | 95% | 221.57 | 524.68 | 13753.59 | 29.68 | 80.75 | 566.89 |
|  | UL packet-Latency CDF | Mean | 91.33 | 144.44 | 615.33 | 62.93 | 101.94 | 133.32 |
|  |  | 5% | 4.21 | 4.27 | 5.17 | 8.71 | 10.39 | 12.31 |
|  |  | 50% | 17.64 | 23.85 | 105.67 | 16.78 | 24.40 | 37.86 |
|  |  | 95% | 1076.41 | 1330.78 | 7774.09 | 262.26 | 520.67 | 659.35 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 21.24 | 36.48 | 172.13 | 10.85 | 23.82 | 162.95 |
|  |  | 5% | 6.61 | 6.82 | 7.31 | 5.54 | 6.79 | 9.06 |
|  |  | 50% | 9.79 | 14.13 | 21.94 | 8.04 | 14.75 | 37.91 |
|  |  | 95% | 197.45 | 452.46 | 2977.19 | 22.55 | 61.78 | 565.67 |
|  | UL packet-Latency CDF | Mean | 89.34 | 146.63 | 390.62 | 75.02 | 120.07 | 157.32 |
|  |  | 5% | 2.77 | 3.00 | 4.88 | 5.11 | 13.30 | 14.64 |
|  |  | 50% | 14.93 | 30.20 | 70.05 | 17.92 | 34.91 | 53.80 |
|  |  | 95% | 1296.57 | 1542.02 | 4849.48 | 342.21 | 566.72 | 718.10 |

Table B.3.1.2.2-3: Results of Urban Macro (FR1) dynamic SBFD vs. semi-static SBFD, {XXXXU}

(dynamic SBFD Option 3, w/ CLI handling)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), SBFD slot configuration {XXXXU}, dynamic SBFD Option 3, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | [33] |  |  |
|  |  |  | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 8.32% | 21.56% | 38.64% |
|  |  | Type-2 | 13.27% | 34.39% | 61.62% |
|  | UL RU | Type-1 | 1.30% | 5.44% | 15.48% |
|  |  | Type-2 | 3.62% | 15.19% | 43.20% |
| Dynamic SBFD | DL RU | Type-1 | 8.93% | 21.60% | 41.03% |
|  |  | Type-2 | 13.17% | 36.94% | 73.39% |
|  | UL RU | Type-1 | 0.99% | 4.07% | 13.90% |
|  |  | Type-2 | 3.30% | 10.26% | 32.53% |
| Baseline | DL average-UPT CDF | Mean | 363.69 | 302.81 | 234.28 |
|  |  | 5% | 134.44 | 101.61 | 58.65 |
|  |  | 50% | 405.94 | 323.12 | 234.98 |
|  |  | 95% | 463.45 | 435.80 | 403.85 |
|  | UL average-UPT CDF | Mean | 80.84 | 59.16 | 50.12 |
|  |  | 5% | 1.48 | 0.77 | 0.63 |
|  |  | 50% | 51.15 | 23.06 | 16.56 |
|  |  | 95% | 229.23 | 216.45 | 201.89 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 399.96 | 302.48 | 201.42 |
|  |  | 5% | 139.63 | 83.03 | 39.60 |
|  |  | 50% | 444.45 | 321.85 | 197.30 |
|  |  | 95% | 538.87 | 462.29 | 378.81 |
|  | UL average-UPT CDF | Mean | 106.79 | 84.12 | 61.22 |
|  |  | 5% | 2.14 | 0.92 | 0.61 |
|  |  | 50% | 63.34 | 34.81 | 16.60 |
|  |  | 95% | 324.93 | 317.17 | 269.63 |
| Baseline | DL packet-Latency CDF | Mean | 22.55 | 49.81 | 64.94 |
|  |  | 5% | 8.07 | 8.12 | 8.21 |
|  |  | 50% | 9.80 | 13.26 | 19.05 |
|  |  | 95% | 127.64 | 317.97 | 611.13 |
|  | UL packet-Latency CDF | Mean | 93.93 | 195.06 | 330.36 |
|  |  | 5% | 4.21 | 4.37 | 4.74 |
|  |  | 50% | 18.52 | 43.25 | 61.07 |
|  |  | 95% | 1146.78 | 1469.23 | 2643.69 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 18.90 | 39.80 | 107.66 |
|  |  | 5% | 6.60 | 6.81 | 7.87 |
|  |  | 50% | 9.33 | 14.28 | 24.03 |
|  |  | 95% | 203.51 | 299.22 | 1170.20 |
|  | UL packet-Latency CDF | Mean | 72.44 | 140.57 | 254.26 |
|  |  | 5% | 2.85 | 3.00 | 3.62 |
|  |  | 50% | 15.24 | 28.78 | 57.02 |
|  |  | 95% | 1051.71 | 1480.21 | 2425.40 |

Table B.3.1.2.2-4: Results of Urban Macro (FR1) dynamic SBFD vs. semi-static SBFD, {XXXXX}

(dynamic SBFD Option 3, w/ & w/o CLI handling)

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), SBFD slot configuration {XXXXX}, dynamic SBFD Option 3, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [33] |  |  |  |  |  |
|  |  |  | w/o CLI handling |  |  | w/ CLI handling |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load |
| Baseline | DL RU | Type-1 | 9.38% | 35.63% | 67.11% | 8.19% | 22.41% | 39.23% |
|  |  | Type-2 | 11.97% | 45.45% | 85.61% | 10.45% | 28.58% | 50.04% |
|  | UL RU | Type-1 | 1.28% | 7.64% | 14.54% | 1.31% | 5.74% | 15.24% |
|  |  | Type-2 | 6.47% | 38.63% | 73.52% | 6.63% | 29.04% | 77.05% |
| Dynamic SBFD | DL RU | Type-1 | 8.50% | 25.37% | 62.13% | 7.94% | 20.39% | 40.58% |
|  |  | Type-2 | 10.73% | 36.56% | 85.94% | 10.02% | 29.85% | 67.70% |
|  | UL RU | Type-1 | 1.19% | 4.51% | 14.99% | 1.12% | 3.80% | 15.32% |
|  |  | Type-2 | 6.17% | 15.51% | 58.51% | 5.78% | 12.55% | 39.84% |
| Baseline | DL average-UPT CDF | Mean | 443.40 | 338.31 | 216.29 | 449.55 | 386.31 | 316.12 |
|  |  | 5% | 143.61 | 52.55 | 5.69 | 141.45 | 100.98 | 66.84 |
|  |  | 50% | 492.85 | 351.37 | 180.06 | 506.59 | 417.00 | 324.54 |
|  |  | 95% | 576.83 | 543.18 | 490.77 | 578.13 | 551.87 | 520.16 |
|  | UL average-UPT CDF | Mean | 67.57 | 42.78 | 30.24 | 68.57 | 48.47 | 36.20 |
|  |  | 5% | 1.53 | 0.04 | 0.00 | 1.19 | 0.14 | 0.00 |
|  |  | 50% | 53.83 | 19.52 | 8.10 | 53.59 | 26.30 | 9.47 |
|  |  | 95% | 158.33 | 141.52 | 114.10 | 158.13 | 148.25 | 127.27 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 466.78 | 355.20 | 212.35 | 484.20 | 374.00 | 227.03 |
|  |  | 5% | 135.30 | 96.84 | 5.62 | 153.37 | 113.88 | 47.42 |
|  |  | 50% | 515.11 | 371.23 | 197.33 | 542.74 | 396.79 | 224.97 |
|  |  | 95% | 655.43 | 550.05 | 461.72 | 657.57 | 584.99 | 416.11 |
|  | UL average-UPT CDF | Mean | 103.20 | 80.71 | 39.33 | 110.05 | 90.84 | 60.16 |
|  |  | 5% | 1.52 | 1.02 | 0.03 | 2.14 | 1.29 | 0.53 |
|  |  | 50% | 58.99 | 31.73 | 12.63 | 71.59 | 42.31 | 17.99 |
|  |  | 95% | 331.81 | 310.18 | 161.79 | 333.32 | 318.55 | 242.89 |
| Baseline | DL packet-Latency CDF | Mean | 18.70 | 207.01 | 826.66 | 15.60 | 41.56 | 57.18 |
|  |  | 5% | 6.56 | 6.61 | 6.72 | 6.56 | 6.58 | 6.63 |
|  |  | 50% | 7.96 | 11.83 | 21.71 | 7.79 | 10.19 | 13.45 |
|  |  | 95% | 148.95 | 3218.15 | 19757.61 | 145.16 | 204.46 | 355.78 |
|  | UL packet-Latency CDF | Mean | 96.15 | 461.61 | 1558.78 | 102.77 | 305.84 | 1459.73 |
|  |  | 5% | 6.20 | 6.47 | 8.00 | 6.19 | 6.35 | 6.88 |
|  |  | 50% | 19.69 | 41.90 | 74.50 | 18.68 | 39.55 | 100.95 |
|  |  | 95% | 1159.97 | 6300.95 | 25375.28 | 1199.98 | 3530.86 | 20468.41 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 17.84 | 62.56 | 583.02 | 16.45 | 39.62 | 129.17 |
|  |  | 5% | 5.54 | 5.65 | 6.79 | 5.54 | 5.61 | 7.21 |
|  |  | 50% | 8.22 | 12.06 | 21.66 | 7.60 | 11.53 | 20.62 |
|  |  | 95% | 180.99 | 713.22 | 14870.29 | 152.06 | 225.13 | 1528.16 |
|  | UL packet-Latency CDF | Mean | 86.90 | 180.18 | 1078.56 | 75.60 | 148.70 | 313.52 |
|  |  | 5% | 2.79 | 3.16 | 6.01 | 2.75 | 2.92 | 4.12 |
|  |  | 50% | 16.53 | 31.95 | 64.96 | 13.47 | 23.69 | 54.14 |
|  |  | 95% | 1183.38 | 1629.59 | 19541.56 | 1179.99 | 1445.08 | 2952.58 |

#### B.3.1.3	Dense Urban Macro layer (FR1)

Table B.3.1.3.1: Results of Dense Urban Macro layer (FR1) dynamic SBFD

| Simple description of key assumptions (RSI based on 1dB desense, Twice area & same TxRUs (Option 2), SBFD slot configuration {XXXXX}, dynamic TDD slot configuration {FFFFF}, dynamic SBFD Option 3, DL: 0.5Mbytes, UL: 0.125Mbyte) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [33] |  |  |  |  |  |
|  |  |  | w/o CLI handling |  |  | w/ CLI handling |  |  |
|  |  |  | Low load | Medium load | High load | Low load | Medium load | High load |
| Dynamic TDD | DL RU | Type-1 | 5.85% | 21.96% | 48.98% | 5.85% | 21.96% | 48.98% |
|  |  | Type-2 | 7.16% | 42.59% | 83.84% | 7.16% | 42.59% | 83.84% |
|  | UL RU | Type-1 | 1.56% | 9.20% | 20.96% | 1.56% | 9.20% | 20.96% |
|  |  | Type-2 | 10.12% | 20.42% | 54.14% | 10.12% | 20.42% | 54.14% |
| Semi-static SBFD | DL RU | Type-1 | 6.28% | 35.08% | 73.93% | 5.94% | 19.73% | 45.09% |
|  |  | Type-2 | 8.01% | 44.75% | 94.31% | 7.58% | 25.17% | 57.52% |
|  | UL RU | Type-1 | 1.73% | 12.29% | 18.18% | 1.68% | 11.06% | 18.65% |
|  |  | Type-2 | 8.76% | 62.13% | 91.90% | 8.48% | 55.89% | 94.31% |
| Dynamic SBFD | DL RU | Type-1 | 6.45% | 31.90% | 58.94% | 5.80% | 20.66% | 45.00% |
|  |  | Type-2 | 8.98% | 65.63% | 96.85% | 7.73% | 47.64% | 92.50% |
|  | UL RU | Type-1 | 1.72% | 22.24% | 31.66% | 1.62% | 15.80% | 42.17% |
|  |  | Type-2 | 6.73% | 44.27% | 83.83% | 6.95% | 28.51% | 83.95% |
| Dynamic TDD | DL average-UPT CDF | Mean | 545.25 | 270.34 | 173.23 | 545.25 | 270.34 | 173.23 |
|  |  | 5% | 364.41 | 82.20 | 28.91 | 364.41 | 82.20 | 28.91 |
|  |  | 50% | 574.23 | 257.04 | 161.03 | 574.23 | 257.04 | 161.03 |
|  |  | 95% | 616.96 | 495.81 | 365.09 | 616.96 | 495.81 | 365.09 |
|  | UL average-UPT CDF | Mean | 236.27 | 147.94 | 92.05 | 236.27 | 147.94 | 92.05 |
|  |  | 5% | 56.70 | 12.75 | 4.84 | 56.70 | 12.75 | 4.84 |
|  |  | 50% | 267.55 | 158.39 | 88.88 | 267.55 | 158.39 | 88.88 |
|  |  | 95% | 309.88 | 271.42 | 190.06 | 309.88 | 271.42 | 190.06 |
| Semi-static SBFD | DL average-UPT CDF | Mean | 504.56 | 340.75 | 175.41 | 509.39 | 406.11 | 274.71 |
|  |  | 5% | 322.43 | 106.59 | 4.05 | 320.75 | 237.41 | 105.10 |
|  |  | 50% | 537.07 | 350.76 | 138.53 | 540.12 | 416.15 | 263.75 |
|  |  | 95% | 577.59 | 523.68 | 451.23 | 576.94 | 539.88 | 490.24 |
|  | UL average-UPT CDF | Mean | 110.04 | 55.10 | 21.94 | 107.89 | 60.63 | 26.48 |
|  |  | 5% | 31.52 | 2.85 | 0.09 | 28.78 | 3.80 | 0.30 |
|  |  | 50% | 123.20 | 54.66 | 11.16 | 121.14 | 60.32 | 16.35 |
|  |  | 95% | 152.04 | 112.53 | 72.74 | 150.92 | 121.68 | 84.03 |
| Dynamic SBFD | DL average-UPT CDF | Mean | 550.21 | 208.61 | 120.72 | 578.53 | 241.68 | 104.46 |
|  |  | 5% | 298.30 | 50.53 | 3.84 | 367.42 | 78.16 | 9.09 |
|  |  | 50% | 589.05 | 201.22 | 87.28 | 611.06 | 227.17 | 84.55 |
|  |  | 95% | 662.38 | 382.14 | 327.98 | 667.68 | 457.92 | 255.96 |
|  | UL average-UPT CDF | Mean | 240.44 | 94.35 | 50.82 | 250.78 | 140.06 | 68.08 |
|  |  | 5% | 54.28 | 3.98 | 0.40 | 59.76 | 9.68 | 0.72 |
|  |  | 50% | 275.09 | 77.02 | 36.73 | 282.93 | 133.52 | 52.12 |
|  |  | 95% | 330.99 | 233.79 | 145.29 | 333.77 | 292.87 | 184.28 |
| Dynamic TDD | DL packet-Latency CDF | Mean | 8.19 | 34.78 | 99.94 | 8.19 | 34.78 | 99.94 |
|  |  | 5% | 5.96 | 6.39 | 8.07 | 5.96 | 6.39 | 8.07 |
|  |  | 50% | 6.47 | 16.88 | 31.67 | 6.47 | 16.88 | 31.67 |
|  |  | 95% | 23.40 | 284.00 | 1010.58 | 23.40 | 284.00 | 1010.58 |
|  | UL packet-Latency CDF | Mean | 8.71 | 64.07 | 86.44 | 8.71 | 64.07 | 86.44 |
|  |  | 5% | 2.34 | 2.86 | 4.32 | 2.34 | 2.86 | 4.32 |
|  |  | 50% | 4.24 | 7.29 | 12.75 | 4.24 | 7.29 | 12.75 |
|  |  | 95% | 89.15 | 781.96 | 2045.03 | 89.15 | 781.96 | 2045.03 |
| Semi-static SBFD | DL packet-Latency CDF | Mean | 9.27 | 58.93 | 255.66 | 9.07 | 12.79 | 30.11 |
|  |  | 5% | 6.54 | 6.64 | 6.89 | 6.54 | 6.59 | 6.72 |
|  |  | 50% | 6.92 | 11.97 | 35.63 | 6.93 | 9.58 | 16.97 |
|  |  | 95% | 35.00 | 463.09 | 4335.59 | 23.83 | 49.37 | 169.91 |
|  | UL packet-Latency CDF | Mean | 17.30 | 144.94 | 466.10 | 32.80 | 95.36 | 373.22 |
|  |  | 5% | 6.15 | 7.54 | 10.66 | 6.15 | 6.85 | 9.44 |
|  |  | 50% | 8.24 | 20.25 | 79.64 | 8.54 | 18.75 | 70.74 |
|  |  | 95% | 193.40 | 2319.27 | 6462.09 | 171.51 | 1346.73 | 5214.66 |
| Dynamic SBFD | DL packet-Latency CDF | Mean | 9.90 | 82.70 | 322.70 | 7.84 | 36.07 | 249.87 |
|  |  | 5% | 5.53 | 8.14 | 8.71 | 5.52 | 6.85 | 11.08 |
|  |  | 50% | 6.47 | 21.98 | 67.82 | 6.11 | 19.14 | 70.35 |
|  |  | 95% | 46.40 | 1012.09 | 4122.61 | 23.78 | 286.98 | 2786.33 |
|  | UL packet-Latency CDF | Mean | 17.68 | 92.94 | 253.00 | 7.98 | 55.43 | 178.96 |
|  |  | 5% | 2.27 | 3.82 | 6.00 | 2.25 | 2.76 | 4.85 |
|  |  | 50% | 4.09 | 14.14 | 26.83 | 3.95 | 8.44 | 20.08 |
|  |  | 95% | 186.15 | 1068.32 | 4799.21 | 88.68 | 708.94 | 3488.37 |

B.3.2	SLS results for inter-gNB/inter-UE CLI handling schemes

The detailed evaluation results for CLI handling for semi-static SBFD can be found in the attached document "B.3.2_CLI handling for SBFD.zip".