3GPP TSG-RAN Meeting #102		R1-232787

Edinburgh, GB, December 13 – 17, 2023

Agenda item:	9.3.1.5

Title:	Collection of Release 18 NR sidelink FR2 study outcomes

Source:	Rapporteur (OPPO)

Document for:	Information

Release 18 – NR Sidelink Evolution WI

According to the approved WID (RP-230077), the following objective is carried out in RAN1 and RAN2 on the study of enhanced SL operation on FR2 licensed spectrum.

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] (Determine in RAN#98-e whether to continue the study or study + specification work for FR2 until the end of R18)

  - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1]

  - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2]

    - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

## RAN1

### RAN1#110-bis-e (10 – 19 October 2022)

Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2 - Reuse indoor layout defined for SL-U with pairs topology and without WiFi nodes - FFS: total number of UEs deployed in the layout - Companies should report how UEs are paired - FFS: whether to consider the cluster-based topology defined for SL-U - Note: for the evaluation, there is no Uu link in this indoor layout Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse layout option 3 in Section A.2.1.1 of TR 36.843 with - Option 1: 7 macro sites with 3 cells per site - Option 2: a single site - Companies should report how UEs are paired - FFS: total number of UEs deployed in the layout - FFS: whether Uu and PC5 use same carrier - FFS: ISD for this layout option 3 Agreement For the indoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the total number of UEs is 12 pairs/20 MHz with scaling factors of 1, ½ or 1/3. Agreement For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the number of UEs per cell is 60 with scaling factors of 1, ½ or 1/3. Agreement For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, Uu link has different carrier as PC5 in the simulation is the baseline - Optional: Uu link has same carrier as PC5 in the simulation. Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for the outdoor layout, the channel model reuses the procedures and parameters for UMi - Street Canyon specified in TR 38.901. Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for the indoor layout, the channel model reuses the procedures and parameters for InH mixed office specified in TR 38.901. Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for UE antenna parameters, reuse the antenna element pattern and antenna array configuration for pedestrian UE and cellular UE as in Table 6.1.4-6 and Table 6.1.4-7 of TR 37.885. Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, consider at least the following parameters: - Carrier frequency: 30 GHz - Sub-carrier spacing: 120 kHz (baseline), 60 kHz (optional) - Simulation bandwidth: 100 MHz (baseline), 200 MHz (optional) - UE receiver noise figure: 13 dB (baseline), 10 dB (optional) - UE Tx power: 23 dBm (EIRP should not exceed 43 dBm) - UE speed: 3 km/h Agreement For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, ISD is 200 meters. Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, support at least the following traffic model: - Option 1: periodic traffic mode 3 - Packet size scaling factor is up to companies’ porting - Option 2: FTP model 3 with arrival rate satisfying one of the followings: - BO low load: 10%-25% - BO mid load: 35%-50% - BO high load: above 55% - Packet size is up to companies’ reporting - Option 3: XR traffic models including cloud gaming, virtual reality, and augmented reality. - It is up to each company to use either Option 1 or 2 or 3 or mixed of them. Agreement When reporting the simulation results for sidelink operation on FR2, companies should report the used resource allocation scheme. Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, performance metric includes UPT, latency and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure. - FFS: UE satisfaction as section 7.2 in TR 38.838 for XR traffic evaluation

### RAN1#111 (14 – 18 November 2022)

Conclusion: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, indoor layout with cluster-based topology is up to companies. Further discussion on the evaluations assumptions for cluster-based topology is not expected. Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for outdoor layout, do not support UE-to-UE 2D distance smaller than 10m. Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for outdoor layout, in the pathloss model for UMi – Street Canyon in TR38.901, antenna height of base station (hBS) is replaced by antenna height of UE (hUT). Agreement In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, optionally support performance metric of UE satisfaction as section 7.2 in TR38.838 for XR traffic evaluation. Conclusion When reporting the simulation results for sidelink operation on FR2, companies should report the used beamwidth. Conclusion In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the UE antenna array configurations other than the one defined in Table 6.1.4-7 of TR37.885 are not precluded.

### RAN1#112 (27 February – 03 March, 2023)

Agreement For sidelink beam management, RAN1 is to study - how transmit beam(s) training and/or receive beam(s) training is performed - whether and how spatial related information (e.g., TCI, QCL, beam ID, etc) information could be identified - the relationship between PC5 unicast link establishment and sidelink initial beam pairing (e.g., whether initial beam pairing procedure starts before, during or after sidelink unicast link establishment procedure.) Agreement RAN1 is to study sidelink beam measurement and reporting schemes (e.g., periodicity, contents, container, timing, procedure, etc.) for sidelink beam maintenance. Agreement RAN1 is to study sidelink beam indication and switching schemes (e.g., framework, general procedure, contents, signaling, timing, etc.) for sidelink beam maintenance. Agreement RAN1 is to study the information related to a sidelink beam failure instance that the PHY layer provides to the MAC layer.

### RAN1#112-bis-e (17 – 26 April, 2023)

Agreement RAN1 can study the following candidate procedure where initial beam pairing is performed before sidelink unicast link establishment, including at least the following steps and how to determine UE2: - UE1 sends reference signals via different transmit beams - Note: multiple reference signals transmissions (e.g. repetitions) from each of the beams can be studied - FFS when reference signals are sent - FFS applicable reference signal - UE2 measures the reference signals and determines a UE1 transmit beam and/or a UE2 receive beam - FFS:whether/how to determine a UE2 transmit beam - UE2 indicates to UE1 the determined UE1 transmit beam - FFS how to indicate the determined transmit beam, including its feasibility - UE1 and UE2 set up sidelink unicast link using the determined beam, following existing link establishment procedure. Agreement Consider one or more of the following items as sidelink beam reporting contents for beam maintenance: - Beam indication (e.g., CRI) - L1-RSRP - FFS: L1-SINR or other reporting contents if necessary enhancement is identified. - Note: This does not preclude performing beam maintenance without any beam reporting Agreement The container(s) of sidelink beam reporting for beam maintenance is at least selected from the following options: - Option 1: SL PHY layer signal (e.g., PSFCH, SCI) - Option 2: SL MAC CE - FFS: PC5-RRC, Signaling over Uu link (e.g., UCI) Conclusion RAN1 to prioritize the case where a UE is incapable of simultaneous transmitting or receiving PSCCH/PSSCH/PSFCH using different beams in Rel-18. Agreement For UE1 and UE2 in a unicast link, RAN1 to study the selection of both UE1’s transmit beam and UE2’s corresponding receive beam. - Note: this applies for both PSCCH/PSSCH transmission/reception and PSFCH transmission/reception. Agreement Consider using sidelink CSI-RS as a starting point for beam maintenance. - FFS: whether/how to enhance existing aperiodic and non-standalone SL CSI-RS - FFS: periodic and/or semi-persistent SL CSI-RS transmissions - FFS: standalone SL CSI-RS transmissions - Note: standalone SL CSI-RS transmission means at least no accompanying sidelink data (SL MAC SDU) transmissions in the same slot. FFS: accompanying SCI(s) or SL MAC CE transmissions or PSFCH. - FFS: one or multiple SL CSI-RS transmissions within one slot - FFS: SL CSI-RS transmissions with or without repetition on transmit beams Agreement RAN1 can study the following candidate procedure where initial beam pairing is performed during sidelink unicast link establishment - UE1 sends PSCCH/PSSCH that carries unicast link establishment message (e.g., DCR message) via different transmit beams - Note: multiple PSCCH/PSSCH transmissions (e.g., repetitions) from each of the beams can be studied. - FFS: applicable reference signals which are transmitted together with unicast link establishment message. - if UE2 successfully decodes one (or more) of the PSCCH/PSSCH(s) and UE2 determines to establish a unicast link with UE1, it indicates to UE1 one (or more) UE1 transmit beam(s) of PSCCH/PSSCH(s) which is successfully received - FFS details (e.g., implicit or explicit indication) - FFS: how to map between each PSCCH/PSSCH and UE1 transmit beam - FFS: how UE2 determines UE1 transmit beam(s) and/or UE2 transmit/receive beam(s) - UE1 uses one of the indicated beam(s) to finish the remaining sidelink unicast link establishment procedure with UE2 - FFS: how UE1 determines one of the indicated beam(s) - FFS: use of additional reference signal or additional messages or additional measurement for efficient beam pairing. Agreement RAN1 can study the following candidate procedure where initial beam pairing starts after sidelink unicast link establishment between UE1 and UE2, including studying whether and in which cases initial beam pairing after sidelink unicast link establishment is feasible. - UE1 and UE2 set up sidelink unicast link, following existing link establishment procedure - FFS the beams used for unicast link establishment. - UE1 and/or UE2 configure the resources for beam sweeping and/or beam reporting - FFS details of resources configuration - UE1 and/or UE2 use the configured resources to transmit reference signals and determine a pair of transmit beam and receive beam based on beam sweeping. - FFS applicable reference signal(s) - FFS whether/how to indicate the determined beams between UE1 and UE2 - FFS difference between initial beam pairing (after sidelink unicast link establishment) and beam maintenance Agreement To study the feasibility of adapting S-SSB for initial beam pairing between UE1 and UE2, at least the following can be considered. - Whether/how to enable UE2 to identify UE1 (e.g., source ID) from UE1’s S-SSB transmission, to enable UE1 to identify the corresponding beam measurement/reporting from UE2 - Mapping between S-SSB transmission/resource and beam related information - Allocation of beam reporting resources respectively associated with different S-SSB transmit beams - Structure and contents of S-SSB - Triggering and/or activation of S-SSB transmission, if needed - Mechanism for S-SSB monitoring and reporting/responding - Mechanism to mitigate/avoid the interference between overlapped S-SSB transmissions from different UEs, including S-SSB transmission resources - Potential impact to/from other UEs, and whether/how to avoid or mitigate this impact Agreement To study the feasibility of reusing SL CSI-RS for initial beam pairing, at least the following enhancements can be considered. - SL CSI-RS transmission with or without sidelink data transmission in the same slot - FFS: slot structure - Mapping between SL CSI-RS transmission/resource and beam related information - Periodic SL CSI-RS transmission, semi-persistent SL CSI-RS transmission, or aperiodic SL CSI-RS transmission, with or without SCI indication - Allocation of SL CSI-RS beam sweeping resources and if applicable, their associated beam reporting resources - Study the possibility to apply SL CSI-RS for initial beam pairing before, during or after unicast link establishment - FFS: How to provide SL CSI-RS resource configuration - Whether or how to mitigate/avoid the interference between overlapped SL CSI-RS transmissions from different UEs - SL CSI-RS transmission with or without repetition on transmit beams Agreement RAN1 is to study sidelink Beam Failure Recovery (BFR) mechanism at least for the scheme where SL BFI is triggered based on the measurement of reference signal for BFD (if supported), including - candidate beam(s) identification - FFS details on reference signals for candidate beam identification, including structure, procedure, timing. - sidelink BFR request (BFRQ), including resources, transmit and/or receive beams, container, timing, etc. - sidelink BFR response (BFRR), including container, procedure, timing, etc. - FFS applicability to the scheme where SL BFI is triggered based on SL HARQ feedback (if supported). Agreement Consider the following two options for determining PSFCH transmit/receive beam for a single PSFCH transmission/reception in a slot. - Option 1: PSFCH transmit beam is derived from the corresponding PSCCH/PSSCH receive beam; PSFCH receive beam is derived from the corresponding PSCCH/PSSCH transmit beam. - Note this is based on sidelink beam correspondence - Option 2: PSFCH transmit beam is derived from the PSCCH/PSSCH transmit beam for reverse data transmission; PSFCH receive beam is derived from the PSCCH/PSSCH receive beam for reverse data transmission. - Note this is based on beam training for reverse data transmission - Note: The PSFCH transmit/receive beam can be the same as PSCCH/PSSCH transmit/receive beam used for the reverse data transmission. - FFS: support both options or down select one option.

### RAN1#113 (22 – 26 May, 2023)

Agreement In the candidate procedure where initial beam pairing is performed before sidelink unicast link establishment, - In step 1, - the applicable reference signal is selected based on - Alt 1-1: S-SSB or its modified format - Alt 1-2: standalone SL CSI-RS or its modified format - Alt 1-3: non-standalone SL CSI-RS - Note: standalone SL CSI-RS transmission means at least no accompanying sidelink data (SL MAC SDU) transmissions in the same slot. FFS: accompanying SCI(s) or SL MAC CE transmissions or PSFCH. - Alt 1-4: PSCCH/PSSCH DMRS - the reference signals are sent - Alt 2-0: aperiodically - Alt 2-1: periodically - Alt 2-2: semi-persistent with activation and deactivation - FFS details of activation/deactivation - FFS resources and resource allocation of reference signal - FFS: if CSI-RS is used, whether UE1 transmits other information associated with the CSI-RS - In step 2, - UE1’s transmit beam and UE2’s receive beam are determined by UE2 as the pair with the RSRP measurement satisfying certain condition(s) - FFS details of condition(s) - FFS explicit or implicit determination of UE1 transmit beam by UE2 - UE2’s transmit beam is at least determined as the one corresponding to determined UE2’s receive beam, at least if beam correspondence is assumed - FFS other scheme - In step 3, - UE2’s beam reporting is associated with determined UE1’s transmit beam. - FFS details of association - FFS details of beam reporting - Note: this does not preclude beam reporting in the link establishment message. - FFS: how UE1 determines its transmit beam if it receives different beam reporting from different UEs - FFS: whether/how to avoid unnecessary beam measurement and reporting from multiple UEs; Agreement In the candidate procedure where initial beam pairing is performed during sidelink unicast link establishment, - In step 1, the candidate reference signal which is transmitted together with unicast link establishment message is selected based on one of the following alternatives - Alt 1-1: SL CSI-RS - Alt 1-2: PSCCH/PSSCH DMRS - In step 2, UE2 determines UE1’s transmit beam(s) and UE2’s receive beam(s) as the pair with the RSRP measurement satisfying certain condition(s). - UE2’s transmit beam is at least determined as the one corresponding to determined UE2’s receive beam, at least if beam correspondence is assumed - FFS the format of UE1’s transmit beam determined by UE2, e.g. implicit or explicit - FFS details of condition(s) - In step 2, UE2 indicates UE1’s transmit beam(s). - FFS details of beam indication, including contents (e.g., ACK/NACK, beam ID, RSRP measurement), container (e.g., PSCCH/PSSCH, PSFCH) and association (e.g. resources) - In step 3, UE1 determines UE1’s transmit beam based on one or more of the following alternatives - Alt 2-1: the latest beam indication - Alt 2-2: beam indication contents (e.g., RSRP measurement) - Alt 2-3: measurement/detection of beam indication signal Agreement In the candidate procedure where initial beam pairing starts after sidelink unicast link establishment (if feasible), the initial beam pairing follows a similar procedure as beam maintenance. Agreement For beam maintenance, consider non-standalone sidelink CSI-RS transmissions. - FFS physical layer structure, including enhancements of existing non-standalone sidelink CSI-RS - FFS signaling details - FFS timing details - FFS whether non-standalone sidelink CSI-RS transmissions use the same or different transmit beam as accompanying data - FFS whether/how to support multiple transmit beams for non-standalone sidelink CSI-RS in the same slot Agreement For beam maintenance, consider standalone sidelink CSI-RS transmissions. - FFS physical layer structure - FFS timing details - FFS whether/how the same or different transmit beams are used in the same slot of standalone sidelink CSI-RS transmissions. - FFS resources, resource allocation and resource indication - FFS signaling details - Note: standalone SL CSI-RS transmission means at least no accompanying sidelink data (SL MAC SDU) transmissions in the same slot. FFS: accompanying SCI(s) or SL MAC CE transmissions or PSFCH. Agreement For beam maintenance, consider design details for sidelink CSI-RS transmissions with or without repetition on transmit beams. Agreement For beam reporting using PSFCH (if supported) in beam maintenance, study - Whether/how there is an association rule between PSFCH for beam reporting and sidelink CSI-RS (either standalone or non-standalone) - FFS beam reporting content, - Whether/how PSFCH carries multiple beam reporting bits - Alt 1: New PSFCH format - Alt 2: PSFCH format 0 by exploring the relationship with frequency and/or code domain resources - Other alternatives are not precluded - FFS: PSFCH carries one beam reporting bit - FFS: whether beam reporting and sidelink HARQ ACK can be reported together, e.g., same or different PSFCH Agreement For beam reporting using sidelink MAC CE (if supported) in beam maintenance, study - beam reporting content - timeline, e.g., - whether/how to reuse Rel-16 sidelink CSI reporting window as baseline for the association between sidelink beam reporting and sidelink CSI-RS resources. - Periodic, aperiodic and/or semi-persistent reporting timeline Agreement For beam indication in sidelink beam maintenance, study sidelink TCI state mechanism: - Details of sidelink TCI state at least including/indicating e.g. sidelink TCI state ID, sidelink CSI-RS resource and Tx/Rx spatial filter related information - FFS: QCL types for sidelink - FFS: whether PSCCH and associated PSSCH always have the same TCI state - FFS: whether the beam indication is via sidelink CSI-RS resource - Study the following optional beam indication candidate container - SCI - sidelink MAC CE - PC5-RRC - FFS: activation time of indicated beam - FFS: whether to reuse the Rel-17 unified TCI framework - Study whether/how beam indication is sent by transmitter UE or receiver UE. - FFS study beam indication in mode 1 on Uu interface Agreement RAN1 can study the following two schemes to trigger sidelink beam failure instance (BFI) that PHY layer provides to MAC layer. - Scheme 1: Sidelink BFI is triggered based on PSFCH carrying sidelink HARQ feedback - Note: this scheme follows the principle of sidelink RLF. - FFS any other enhancements - FFS whether/how to support candidate beam identification in case of BFD - FFS criteria of triggering sidelink BFI - FFS whether there is RAN1 impact - Scheme 2: Sidelink BFI is triggered based on the measurement of reference signal for BFD - Note: this scheme follows the principle of Uu BFR. - FFS any other enhancements - Consider the following reference signals for BFD - Periodic and/or semi-persistent sidelink CSI-RS - S-SSB or its modified format - FFS criteria of triggering of sidelink BFI - FFS whether/how to trigger the transmission of reference signal for BFD - Other options are not precluded. Agreement At least for the scheme where sidelink BFI is triggered on measurement of BFD reference signal (if supported), for candidate beam determination (CBD) - Consider the following reference signals for CBD - Sidelink CSI-RS or its modified format - FFS details (e.g., resources, periodicity, transmit beams, timing, etc) - S-SSB or its modified format - FFS criteria of determining candidate beam (e.g., largest RSRP measurement, RSRP measurement larger than a threshold, pre-determined during initial beam pairing, etc) - FFS whether/how to trigger the transmission of reference signal for CBD Agreement At least for the scheme where sidelink BFI is triggered on measurement of BFD reference signal (if supported), for sidelink BFRQ, study - Resources - Option 1: associated with CBD reference signal, i.e. identified candidate beam - Option 2: within a responding window after CBD reference signal transmission - Option 3: Resources determined based on sensing and resource allocated after identification of a new beam. - Option 4: associated with BFD reference signal - Other options are not precluded - Container - Option 1: PSFCH or modified PSFCH - Option 2: SCI - Option 3: sidelink MAC CE - Option 4: new channel associated with beam reporting in initial beam pairing phase - Other options are not precluded - Transmit beam - Option 1: widest attainable beam - FFS up to UE implementation - Option 2: beam sweeping - Option 3: corresponding to the receive beam of identified candidate beam - Other options are not precluded - Content can be the BFD and/or new beam indication - FFS: whether/how to support sidelink BFRQ in mode 1 including signaling to/from the network. Agreement At least for the scheme where sidelink BFI is triggered on measurement of BFD reference signal (if supported), for sidelink BFRR, study - Container - Option 1: SCI - Option 2: sidelink MAC CE - Option 3: PSFCH - Option 4: PC5-RRC - Other options are not precluded - Transmit beam - Option 1: widest attainable beam - FFS up to UE implementation - Option 2: beam sweeping - Option 3: corresponding to the candidate beam indicated by the received BFRQ - Other options are not precluded - FFS: new beam indication (e.g., SL CRI, SL SSBRI) - FFS: activation time of new beam - FFS: whether/how to support sidelink BFRR in mode 1 including signaling to/from the network Agreement In the candidate procedure where initial beam pairing is performed before sidelink unicast link establishment, at least the following is considered for SL CSI-RS (or its modified format) (if feasible) - Aperiodic, periodic and/or semi-persistent SL CSI-RS - FFS: details - In case of standalone SL CSI-RS (if supported), candidate resources for SL CSI-RS beam sweeping are (pre-)configured - FFS: whether/how to avoid resource collision of SL CSI-RS transmissions from different UEs - FFS: Association between SL CSI-RS and beam reporting and/or initial link establishment - FFS: details of association - Identification and beam related information of SL CSI-RS for initial beam pairing - FFS: details

### RAN1#114 (21 – 25 August 2023)

Agreement For beam reporting using sidelink MAC CE (if supported) in beam maintenance, consider the following - Sidelink MAC CE explicitly indicates a list of one or more (CRI, L1-RSRP) pairs - Reuse Rel-16 sidelink CSI reporting window as baseline - FFS latency bound for sidelink beam reporting Agreement In the candidate procedure where initial beam pairing is performed before sidelink unicast link establishment, in Step 2, further consider UE2 determines UE1’s transmit beam and UE2’s receive beam whose corresponding RSRP measurement is - Alt 1: the largest among all RSRP measurements performed by UE2 for UE1 - Alt 2: larger than a threshold - FFS: how UE2 determines one if there are multiple. - FFS: beam determination if above condition is not satisfied. - Alt 3: the largest among all RSRP measurements performed by UE2 for UE1, and is larger than a threshold, - FFS: beam determination if above condition is not satisfied. - FFS time window for UE2 to determine the RSRP measurement values used for beam determination. Agreement In the candidate procedure where initial beam pairing is performed before sidelink unicast link establishment, in Step 1, further consider - The candidate resources of reference signal for S-SSB or standalone sidelink CSI-RS are FFS - The candidate resources of S-SSB (or its modified format) for initial beam pairing (if supported) are - Alt 1-1: reusing the resources of legacy S-SSB for synchronization - Alt 1-2: separate from the resources of legacy S-SSB for synchronization - Alt 1-3: reusing the resources of legacy S-SSB for synchronization, with additional resources carrying beam indication/UE ID information. - The candidate resources of standalone sidelink CSI-RS for initial beam pairing (if supported) are one or more of the following - Alt 2-1: in a dedicated resource pool - FFS relationship between dedicated resource pool and legacy resource pool - Alt 2-2: in a shared resource pool for legacy PSCCH/PSSCH transmission - FFS AGC issue - Alt 2-3: dedicated resources (pre)configured within the same pool as legacy PSCCH/PSSCH transmissions - FFS backward compatibility - UE1 determines a standalone sidelink CSI-RS resource, using one or more of the following, - Alt 3-1: legacy mode-2 PSCCH/PSSCH resource selection scheme as starting point. - FFS mode-1 - Alt 3-2: UE ID. - Alt 3-3: beam ID. - Other alternatives are not precluded. - FFS details (e.g., number of symbols for standalone sidelink CSI-RS, frequency multiplexing, etc) - The candidate reference signals are transmitted semi-persistently - FFS conditions of triggering/stopping the reference signals transmission - FFS reference signals are transmitted aperiodically Agreement For standalone sidelink CSI-RS transmission in beam maintenance, consider - Structure with one or more of the following - Alt 0-1:at least with accompanying PSCCH and/or PSSCH (carrying sidelink MAC CE and/or 2nd-stage SCI) in the same slot - Alt 0-2: without accompanying PSCCH/PSSCH in the same slot - Alt 0-2-A: the transmission of standalone sidelink CSI-RS is indicated by PSCCH/PSSCH in an earlier slot(s) - Alt 0-2-B: the transmission of standalone sidelink CSI-RS is not indicated by PSCCH/PSSCH - one or multiple transmissions in a slot - Either same or different transmit beams in a slot - FFS transmit beam of accompanying PSCCH/PSSCH (if supported) - AGC symbol is potentially transmitted prior to each sidelink CSI-RS transmission - FFS same or different transmit power as accompanying PSCCH/PSSCH (if supported) - FFS whether a PSCCH accompanies each sidelink CSI-RS transmission or a single PSCCH accompanies all sidelink CSI-RS transmissions in a slot - FFS gap symbol between sidelink CSI-RS transmissions - resources for standalone sidelink CSI-RS are based on one or more of the following - Alt 1-1: in a dedicated resource pool - Alt 1-2: in a shared resource pool for legacy PSCCH/PSSCH transmission - FFS AGC issue - Alt 1-3: dedicated resources (pre)configured within the same pool as legacy PSCCH/PSSCH transmissions - Alt 1-4: on legacy PSFCH occasions - resource allocation for standalone sidelink CSI-RS - Alt 2-1: uses mode-2 PSCCH/PSSCH resource allocation scheme as starting point - FFS enhancements - FFS mode-1 - Alt 2-2: depending on mapping rule with PSCCH/PSSCH resource (if supported) - Alt 2-3: depending on mapping rule with UE ID - Alt 2-4: depending on mapping rule with beam ID - Other solutions are not precluded. Agreement For non-standalone sidelink CSI-RS transmission in beam maintenance, consider - one or multiple non-standalone sidelink CSI-RS transmissions in a slot, - sidelink CSI-RS transmissions can have the same transmit beam as the associated PSCCH/PSSCH in a slot - FFS whether/how to support sidelink CSI-RS transmissions have different transmit beams from the associated PSCCH/PSSCH in a slot (e.g., AGC issue) - resources for non-standalone sidelink CSI-RS - Alt 1: confined in PSSCH - Alt 1-1: reuse legacy design - Alt 1-2: with enhancement from legacy design - Alt 2: dedicated symbol(s) which are TDM-ed with PSCCH/PSSCH in a slot - FFS: SCI fields modification Agreement For beam indication in sidelink beam maintenance, further consider - in sidelink TCI state configuration - Periodic or semi-persistent reference signal is a source for Tx/Rx spatial filter - At least QCL type-D is supported - FFS other QCL types - PSCCH and associated PSSCH always have the same TCI state - gap between beam indication and beam switching - FFS details - Beam indication validity duration - Alt 1: until the next beam indication - Alt 2: (pre-)configured or dynamically indicated Agreement In the candidate procedure where initial beam pairing is performed during sidelink unicast link establishment, in Step 2, further consider - UE2 determines N pair(s) of UE1’s transmit beam(s) and UE2’s receive beam(s) whose corresponding RSRP measurement(s) are: - Alt 1-1: the largest N among all RSRP measurements performed by UE2 for UE1 - FFS: value of N - Alt 1-2: larger than a threshold - FFS: beam determination if above condition is not satisfied. - FFS: how UE2 determines N beam pair(s) - Alt 1-3: the largest N among all RSRP measurements performed by UE2 for UE1, and are larger than a threshold - FFS: beam determination if above condition is not satisfied. - FFS: value of N - FFS time window for UE2 to determine the largest N RSRP measurement values - UE2 indicates UE1’s transmit beam(s), consider one or more of the following: - Alt 2-1: in explicit format with - Contents: beam related information (e.g., beam ID, L1-RSRP measurement, slot or resource of reference signal) - Container: PSCCH/PSSCH (e.g., SCI stage 2 and/or sidelink MAC CE and/or unicast link establishment response message) - Beam indication resource is selected in a response window (non-overlapping with another response window) - Alt 2-2: in implicit format with - Contents: a single bit indication - Container: PSFCH - Beam indication resource is associated with the resource for UE1’s PSCCH/PSSCH transmission with the determined UE1’s transmit beam. - For transmit beam of UE2’s beam indication - Alt 3-1: transmit beam of UE2’s beam indication is based on determined UE2’s receive beam - FFS: details - UE1 attempts to monitor a beam indication resource using a receive beam based on UE1’s transmit beam for the associated PSCCH/PSSCH transmission. - Alt 3-2: UE2 beam sweeps the message containing the beam indication - FFS receive beam of UE1 if UE1 does not have beam corrsponse. - If UE1 has beam correspondence, UE1 attempts to monitor beam indication resources using a receive beam corresponding to UE1’s transmit beam - Alt 3-3: transmit beam of UE2’s beam indication corresponds to determined UE2’s receive beam - UE1 attempts to monitor a beam indication resource using a receive beam corresponding to UE1’s transmit beam for the associated PSCCH/PSSCH transmission.

## RAN2

### RAN2#121 (27 February – 03 March 2023)

The agenda was not handled.

### RAN2#121-bis-e (17 – 26 April 2023)

[BFR Framework] Agreement: For beam failure detection, reuse Uu design of timer + counter based mechanism as baseline, and R2 further study how SL beam failure is detected. FFS on Tx or Rx UE based manner. [BFR mechanism] Agreement: Upon beam failure is detection, support BFR signaling exchange between peer UEs, and further study e.g., RLF declaration due to beam failure.

### RAN2#122 (22 – 26 May 2023)

The agenda was not handled.

### RAN2#123 (21 – 25 August 2023)

The agenda was not handled.

### RAN2#123-bis (09 – 13 October 2023)

For R18 SL-FR2 initial beam-pairing, no need for further study work at R2. => Agreed. For R18 SL-FR2 beam management, no need for further study work at R2. => Agreed. For R18 SL-FR2 beam-based RRM measurement report, no need for further study work at R2. => Agreed. For R18 SL-FR2 beam failure detection, no need for further study work (also including whether TX based or RX based detection) at R2. => Agreed. For R18 SL-FR2 beam failure report/handling, no need for further study work at R2. => Agreed. Agreements on SL-FR2 1. Study for SL FR2 for Rel-18 is completed from RAN2 perspective