3GPP TSG-RAN5 Meeting #101	R5-236632

# Chicago, USA, November 13 – 17 , 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0836 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to TP analysis for FR2 CA_NS_202 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current TP analysis of FR2 CA_NS_202, there is one test RB allocation selected as Inner_Partial for each CC. With this RB allocation, the MPR needs to comply with the non-contiguous RB allocation requirement as below: For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies: MPR = max(MPRC_CA, -10*A + 14.4) Given the Inner_Partial allocation specified in TS 38.521-2 section 6.1, the MPR would be higher than AMPR value. Since the AMPR test case usually verifies the scenarios where AMPR supersedes MPR, it’s proposed to replace the RB allocation by Outer_Full so that MPR would be smaller than AMPR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replacing the RB allocation mentioned above by Outer_Full. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR is not verified sufficiently with existing test point. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Remove the following .zip file from TR 38.905: 38.521-2_TPanalysis_6.2A.3_AMPR_CA_NS_202.zip Add the following attached .zip file to TR 38.905: 38.521-2_TPanalysis_6.2A.3_AMPR_CA_NS_202_v1.zip |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

4.2.2.2	A-MPR and A-SE FR2 test cases for CA

This section contains information on test point selection for test cases 6.2A.3.1, 6.2A.3.2, and 6.2A.3.3 in [3], UE additional maximum output power reduction for CA as well as the related spectrum emissions test case 6.5A.3.3.1, 6.5A.3.3.2, and 6.5A.3.3.3 in [3] Additional Spurious emission (A-SE) for CA.

Selection of test points should include some possible worst combinations based on the A-MPR and spectrum emissions characteristics specified for each CA_NS value. The number of test points should be realistic.

Since A-MPR is defined by RAN4 together with A-Spurious requirements, a combined analysis is required. In general, the following non-compliant UE behaviours need to be checked:

a) UE apply too much A-MPR (more than RAN4 allow)

b) UE apply to little A-MPR (causing too much spectrum emissions)

Case A can be verified in A-MPR test case

Case B can be verified in A-SE test case if it is ensured that the same test point is tested inside A-MPR test. Therefore, the test points in spectrum emissions test case must be a subset of the test points in the A-MPR test case.

Note: Even if there are identical test points in the MPR test case the A-MPR test case is still needed to verify UE output power when NS-value is signalled.

Table 4.2.2.1-1: NS value specific test points for A-MPR single carrier

| NS label | Number of test points | Justification | Comments |
| --- | --- | --- | --- |
| CA_ NS_202 | 6.2A.3.1: 4 6.2A.3.2: 4 6.2A.3.3: 4 6.5A.3.3.1:4 6.5A.3.3.2:4 6.5A.3.3.2:4 | “38.521-2_TPanalysis_6.2.3_AMPR_CA_NS_202v1.zip” | RAN5#101 |

< End of Changes >

Remove the following.zip file from TR 38.905:

38.521-2_TPanalysis_6.2A.3_AMPR_CA_NS_202.zip

Add the following attached .zip file to TR 38.905:

38.521-2_TPanalysis_6.2A.3_AMPR_CA_NS_202_v1.zip