3GPP TSG RAN Meeting #102									RP-233073

December 11–15, 2023, Edinburgh, UK

Status Report to TSG

Agenda item:		9.3.4.13

| WI / SI Name | Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FR1_TRP_TRS_enh |  |  |  |  |
| Unique ID | 970082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-223112 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Dec/2023 | Performance part: Jun/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: 50% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Wang, Ruixin |
|  | Company | vivo |
|  | Email | ruixin.wang@vivo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.4	RAN4

2.4.1	Agreements

RAN4 #108bis

- General

- TR 38.870 v0.6.0 was email approved in [10]

- Topic summary for FR1 TRP TRS WI in [30]

- Ad-hoc minutes for NR_FR1_TRP_TRS_enh was approved in [25]

- TP to TR 38.870 on phantoms was approved in [11]

- draft CR to TS 38.161 on New test configurations for Rel-18 TRP TRS was endorsed in [26]

- TP to TR 38.870 on 2Tx test configuration was approved in [29]

- Working procedure for Rel-18 TRP TRS lab alignment and harmonization campaign was approved in [28]

- Updated Schedule for Rel-18 TRP TRS measurement campaigns and requirements was approved in [15]

- (Template) Measurement results for 3GPP Rel-18 TRP TRS performance test campaign was approved in [16]

- WF for Rel-18 FR1 TRP TRS was approved in [31]

Issue 1-1-2: Test Methods for fully Coherent UE support multiple TPMI index 2~5

Agreements:

- Capture common test procedure of O1 and O2 into TR, final measured EIRPs processing can be further decided. The performance metric for each approach should also be further discussed.

  - Option 1 (averaging TRPs)

  - Option 2 (Max EIRPs)

- Test mode is captured in the TR which can be considered as one test method, only if the phase variation issue can not be resolved in O1 and O2.

  - Test mode option 1: Single antenna transmission each time

  - Test mode option 2: Two antenna transmission simultaneously

  - Other Test modes are not precluded

Issue 1-1-3: Performance metric definition for Option 1 and Option 2 methodology

Agreements:

If we cannot agree this meeting to the metric, we can identify the areas we would like to study for the next meeting to further the progress.

Topics to study for next meeting

- Identify names and metrics for option 1 and option 2

- Come up with impact to regulatory and industry for both option 1 and option 2

Issue 1-1-6: phase variation for single-layer UL-MIMO

Agreements:

- phase variation is a general issue for all 2Tx test cases, no matter TxD and UL MIMO.

- Further study the TRP results impacts based on simulation and real measurements. The impact should also be considered when developing 2Tx TRP requirements in future.

Issue 1-1-7: Measurement grid analysis for UL-MIMO

Agreements:

- In RAN4, measurement grid and preliminary MU assessment to take options 1 and 2 into account

- Final grids and corresponding MU value for 2Tx conformance testing can be decided in RAN5.

- If any measurements are presented as part of this WI, use at least the legacy ==15° measurement grids

Issue 1-4-1: RC test method for 2Tx test cases

Agreements:

- RC test method for 2Tx test cases can be studied based on contribution-driven approach.

Issue 2-1-1: LAD information update

Agreements:

  - Confirm the following updated LADs for Rel-18 AC lab alignment activity.

| Rel-18 Lab Alignment Device (LAD) |  |  |  |
| --- | --- | --- | --- |
| Volunteer lab | Owner | Contact | Note |
| LAD1 (Rel-17) | vivo | Ruixin ruixin. wang@vivo.com | For n78 testing. The lab alignment has been started |
| LAD2 (Rel-17) | vivo | Ruixin ruixin. wang@vivo.com | For n78 testing. The lab alignment has been started |
| LAD3 | vivo | Ruixin ruixin. wang@vivo.com | For n28 testing. The lab alignment has been started |
| LAD4 | TIM | Alex alessandro.trogolo@telecomitalia.it | For n28 testing. The lab alignment has been started |
| Note: To keep the test burden, most 4 test cases, i.e., two devices per band. |  |  |  |

Issue 2-1-2: Updated Working procedure for Rel-18 AC lab alignment activity

Agreements:

  - Approve the following working procedure for Rel-18 AC lab alignment activity:

Working procedure for Rel-18 AC Lab Alignment Campaign

- The purpose of Lab Alignment Campaign is to ensure there is no unexpected lab deviation and establish full trust and confidence on the results.

- Test labs are invited to participate to the lab alignment and test campaign, the following conditions should be fulfilled:

  - Participating lab should be accredited under ISO 17025 (ISO 17025 accredited labs) and have any of 3GPP TS 37.544, CCSA YD/T 1484.6, and CTIA OTA Test Plan listed on its accreditation scope.

  - Participating lab should have anechoic chamber(s) ready to support testing based on 3GPP TS 38.161.

  - Participating lab should have sufficient test resource to provide the on-time measurement results without delay.

- Test methodology:

  - Test plan: 3GPP TS 38.161;

- Test cases for Lab Alignment Campaign:

  - Test bands: n28, n78; one low and one high band is sufficient

  - Number of test cases per band: BHHL and BHHR at low/mid/high channel;

  - Use scenario: Head and Hand phantom (Talk mode), i.e., BHHL and BHHR

  - Hand Phantom: Wide Grip hand

  - Operation mode: NR Standalone (SA)

  - Number of Tx chain: UE configured with 1Tx

- Lab Alignment Device (LAD): in total 4 devices, to avoid delay of lab alignment activity, the LADs confirmed by RAN4#108 meeting will not be further added

  - Rel-17 LADs (LAD1 and LAD2) for band n78;

  - 2 more devices (LAD 3 and LAD 4) for band n28;

  - UE information listed in section 6

- Test results submitting:

  - Using the same worksheet template in [R4-2313892] to submit the measurement results

  - The measurement results of LADs should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

  - Sharing of the measurements results of a specific device from test labs is permitted only towards the 3GPP member that provided such device (i.e., device’s owner) before submitting to RAN4

- Lab alignment criteria:

  - The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

  - Confirm the reference value derived based on the per-band per-PC averaging approach (linear average with dBm) of lab alignment data pool from ≥3 labs submitted before end of [RAN4#109] as baseline.

  - Apparent outliers will not be considered in averaging process. The value deviates over 1.5*MU from all the other lab’s results should be identified as apparent outlier.

  - Pass/fail limit for lab alignment should be defined as X*MU (X is TBD) as baseline. MU value is the expanded MU for BHH (defined in Annex of TR38.870).

  - The summation form for TRP and TRS lab alignment should keep consistent during the calculation process of TRP TRS lab alignment from each company i.e. sin weights approach. Only traditional approach (15-degrees TRP and 30-degrees TRS) should be used during lab alignment activity to reduce unnecessary uncertainty.

  - The timeline of AC alignment activity is in [R4-2315843]. How to treat late submission results and confirm the alignment: TBD

- Test lab procedures:

  - LAD delivery scheme

    - Decide LAD delivery scheme after all the test lab and LAD information being confirmed (after the confirmation of volunteered labs and LADs).

  - LAD measurement time in each test lab: finalize LAD measurement within [5] workdays, and deliver to next lab ASAP with LAD delivery In/Out information shared in reflector.

  - Encourage test labs to share resulting combined MU based on their own systems.

Issue 2-1-3: Lab alignment for 2Tx in Rel-18

Agreements:

  - Skip lab alignment campaign and performance requirement for 2Tx TRP in Rel-18, this can be considered as Rel-19 scope.

Issue 2-2-1: updated Working procedure for Rel-18 RC harmonization

Agreements:

  - Approve the following working procedure for Rel-18 RC harmonization activity:

Working procedure for Rel-18 RC harmonization Campaign

- The purpose of RC harmonization activity is to ensure there is no unexpected lab deviation among different RC systems, and the RC harmonization results will be compared with reference AC results to demonstrate the gap between different methodologies.

- RC Test labs are invited to participate to the lab alignment and test campaign, the following conditions should be fulfilled:

  - Participating lab should have Reverberation chamber(s) ready to support testing based on latest version of 3GPP TR 38.870.

  - Participating lab should have sufficient test resource to provide the on-time measurement results without delay.

- Test methodology:

  - Test plan: latest version of 3GPP TR 38.870;

- Test cases for RC harmonization Campaign:

  - Test bands: n28, n78; one low and one high band is sufficient

  - Number of test cases per band for each scenario: left and right at low/mid/high channel, total 6 test cases;

  - Use scenario: Both Head and Hand phantom (Talk mode, BHHL and BHHR) and Hand phantom only (Browsing mode, HL and HR);

  - Hand Phantom: Wide Grip hand

  - Operation mode: NR Standalone (SA)

  - Number of Tx chain: UE with 1Tx.

- Harmonization Devices:

  - Same devices as Rel-18 AC lab alignment activity, listed in section 6

- Test results submitting:

  - Using the same worksheet template in [R4-2313893] to submit the measurement results

  - The measurement results of LADs should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

- Harmonization RC alignment criteria:

  - The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

  - Confirm the reference value derived based on the per-band per-PC averaging approach (linear average with dBm) of harmonization data pool from ≥3 labs submitted before end of [RAN4#109] as baseline.

  - Apparent outliers will not be considered in averaging process. The value deviates over 1.5*MU from all the other lab’s results should be identified as apparent outlier.

  - Pass/fail limit for RC lab alignment should be defined as X*MU (X is TBD) as baseline. MU value is the preliminary expanded MU for talk mode and/or browsing mode (to be defined in RAN5).

  - The timeline of RC harmonization activity is in [R4-2315843]. How to treat late submission results and confirm the alignment: TBD

- RC vs AC test methods harmonization criteria:

  - How to compare the RC measurement results with AC measurements results is FFS, e.g.;

    - Option 1: comparison of each reference value of RC and AC as starting point (per-band per-PC averaging approach (linear average with dBm) of each method from all test labs those are not apparent outliers)

      - The stand Dev of each test method should be studied and potentially considered

    - Option 2: other approaches are TBA

  - The pass/fail criteria are defined as TBD

- Test lab procedures:

- LADs delivery scheme

- LAD delivery scheme will be decided after the confirmation of all the RC volunteer labs.

- LAD delivery will also consider the parallel Rel-18 AC lab alignment activity to ensure efficiency

- LAD measurement time in each test lab: finalize LAD measurement within [5] workdays, and deliver to next lab ASAP with LAD delivery In/Out information shared in reflector.

- Encourage test labs to share resulting combined MU based on their own systems

Issue 2-2-2: RC vs AC harmonization criteria

Agreements:

  - Encourage companies to further discuss reasonable criteria and pass/fail limits for RC vs AC harmonization.

Issue 2-3-1: updated working procedure for Rel-18 TRP TRS Performance Test Campaign to define requirements

Agreements:

  - Approve the following working procedure for Rel-18 TRP TRS Performance Test Campaign.

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: Complete full coverage of band n1, n28, n41, and n78 requirements, including performance objectives which were not concluded in Rel-17;

  - Use scenarios: Both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR

  - Hand Phantom: Wide Grip Hand as first priority, PDA hand is not precluded if sufficient devices are available

  - Operation mode: SA with 1Tx, study how to specify EN-DC requirements based on SA measurement results.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: Size 1(width >72mm and ≤92mm)

  - DUT capability: support for Bands those listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - Year of production: from [second-half 2021 to 2024] this will be confirmed next meeting

  - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

- Devices provisioning:

  - Any 3GPP member can work with the selected test labs to provide devices

    - A test lab shall measure only one UE model in case different samples are provided

    - Same UE model supporting different set of bands can be measured. For this case, the UE model should be marked as different model, e.g., model A-1, model A-2. (guidance on how to manage this case are provided in the spreadsheet in [TBD])

  - The 3GPP member providing the DUTs should contact one of the selected labs to check their availability to receive the DUTs and define together the related provisioning aspects

    - Any issue should be reported to the rapporteur in a timely manner to discuss for an alternative solution

    - To plan properly the measurement campaign, the following actions are requested for the RAN4 Nov meeting:

      - The rapporteur check with the volunteer labs the number of DUTs (minimum 3, maximum 15) they expect to be able to measure AND how many DUTs they can accommodate from 3GPP members

      - The 3GPP member providing the DUTs check how many samples they intend to provide (in terms of maximum number)

      - Planning of the measurement campaign could be reviewed based on the above points

- Test results submitting:

  - RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure

  - UE information disclosure: laboratories use the spreadsheet in [TBD] to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

  - Laboratories use the worksheet template in [R4-2315844] to submit the measurement results for Rel-18 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed):

    - The minimum number of submitted devices from each lab is 3

    - Volunteer labs provide the device information sheet ONLY to the RAN4 Secretary and the sheet used to submit measurement results to the rapporteur

  - RAN4 Secretary ONLY publishes to 3GPP RAN4 the following summary of statistical information after anonymizing the sensitive UE information data, i.e., UE model name and vendor name:

    - Total number of devices

    - Total number of models

    - Total number of devices vendors

    - Percentage of devices per vendor

    - Percentage of devices per Power Class

    - Percentage of devices per each supported band

    - Percentage of devices per year of production

    - Percentage of the devices that are certified by at least one of certification bodies as following: PTCRB ,GCF, and NAL/CTA (Chinese network access licensed test)] /FCC/CE

      - Once the device gets the above certification, for RAN4 discussion that means the device is commercially available

    - Percentage of devices that are commercially available

  - The progress in each lab are encouraged to be shared on the RAN4 reflector (for example - how many devices have been measured and on which bands)

    - Information of the devices that are going to be measured should be shared with the RAN4 Secretary as soon as available in order to monitor the achievement of the thresholds defined in point 8.b.

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Only the results from aligned labs will be considered for specifying requirements

  - Requirements will not be specified if the following thresholds [further check in RAN4#109] are not satisfied by the devices pool:

    - Minimum number of devices for each band, each device size, each power class: [40]

    - Minimum number of device models: [25~30]

    - Minimum number of devices' vendors: 5

    - Percentage of devices from [second-half 2021 to 2024]: TBD%:

    - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: TBD%

    - Percentage of devices that are commercially available: [100] %

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-18.

  - Method of limits derivation: per-band Data driven approach

  - The value at [TBD] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - For a band supporting both PC2 and PC3, specify PC3 requirement based on finalized PC2 requirements, e.g.,with [X]dB offset

- Test lab procedures

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - Newly defined Coarser measurement grid in TR 38.870 can be used for TRP TRS measurement

Issue 2-3-2: Rel-18 device information template for Device information collection and statistical analysis

Agreements:

  - Further check and confirm the UE information template based on updated working procedure next meeting.

Issue 2-4-1: Timeline for Rel-18 AC lab alignment and RC harmonization activity

Agreements:

  - Approve the updated schedule for Rel-18 TRP TRS measurement activities and requirement development in R4-2315843.

Issue 2-4-2: number of receive antennas for n1 or n3 requirements

Agreements:

  - Further discuss whether reduced number of device for 2Rx of n1/n3 should be considered.

  - Further discuss and decide 2Rx or 4Rx requirements for n1/n3 in Rel-18.

RAN4 #109

- General

- TR 38.870 v0.7.0 was email approved in [14]

- Topic summary for FR1 TRP TRS WI in [5]

- Ad-hoc minutes for NR_FR1_TRP_TRS_enh was approved in [44]

- TP to TR 38.870 on TRP TRS test procedure for CA was approved in [45]

- CR to TS 38.161 on New test configurations for Rel-18 TRP TRS was agreed in [21]

- TP to TR 38.870 on TRP TRS test method was approved in [46]

- TP to TR 38.870 on contents for Annex B was approved in [43]

- Template for TRP TRS and MIMO OTA Device Information Collection was approved in [48]

- WF for Rel-18 FR1 TRP TRS was approved in [49]

Issue 1-1-1: For fully Coherent UE support multiple TPMI index 2~5

- Proposals

  - Option 1 (averaging TRPs)

  - Option 2 (Max EIRPs)

Agreements:

  - Focus on performance metric discussion of two options with a goal to select a single metric as baseline in Rel-18.

    - Comparison criteria to assist in down-selection should be discussed in the next meetings

Issue 1-1-3: Requirements work for Option 1 and Option 2 methodology

Agreements:

- The intention is a single metric and a single requirement, which means one test methodology.

Issue 1-1-4: Test procedure of Option 2 for fully Coherent UE

- Proposals

  - Proposal 1: In the test procedure of Option 2, the TPMI at each test point should be selected by system simulator based on UE’s SRS rather than sweeping all the applicable TPMIs.

Agreement:

- The proposal 1 is not considered in Rel-18

Issue 1-1-5: Phase variation for single-layer UL-MIMO

Agreements:

- The amplitude and phase relative error behaviour longer than 20ms should be considered in coherent UE simulation. The UL-MIMO simulation should be updated based on updated assumption of amplitude and phase.

- Details of amplitude and phase assumptions in Annex part of this WF can be used.

Issue 1-1-6: Test mode for 2Tx UE configuration (including coherent/non-coherent UE)

Agreements:

- Further discuss test mode as a backup solution, the corresponding new metric can also be considered.

Issue 1-1-7: Measurement grid analysis for UL-MIMO

Agreements:

- Further discuss whether coarse grid can be used for UL-MIMO TRP testing. Both measurement and simulation are allowed. Outcome of measurement grid for single antenna TRP, if any, can be considered.

Issue 1-1-9: Common test procedure for coherent UE

Agreements:

- The common test procedure is endorsed in R4-2321098.

Issue 2-1-1: AC lab alignment activity status

Agreements:

- Try to resolve LAD4 connection issue, and meanwhile perform measurements with additional LAD5 to move forward.

Issue 2-1-2: BHH Lab alignment pass/fail limits in Rel-18

Agreement:

- Similar to Rel-17 lab alignment approach, setting pass/fail limits as 0.75*MU (talk mode) for both TRP and TRS, i.e., ±1.5dB for TRP, and ±1.76dB for TRS as starting point.

Issue 2-1-3: Analysis of phase 1 AC Lab alignment activity

Agreements:

- Based on agreed pass/fail limits and analysis in R4-2318969, RAN4 conclude phase 1 AC lab alignment activity for band n78 BHH. All the 6 labs have been well aligned with AC test system.

Issue 2-2-1: RC Lab alignment criteria and outcome

Agreements:

- Setting RC lab alignment pass/fail limits as 0.75*MU for both TRP and TRS, as starting point. Keep further discussion of RC alignment next RAN4 meeting, based on RAN5 outcome of RC MU.

Issue 2-2-2: RC vs AC harmonization criteria

Agreements:

- FFS how to define harmonization criteria of AC vs RC. Encourage companies input on this topic next meeting.

Issue 2-3-1: updated working procedure for Rel-18 TRP TRS Performance Test Campaign to define requirements

Agreements:

On top of agreed RAN4 working procedure R4-2316995, the following update is agreed to be updated (updated part is only the bullets with change mark)

- Test results submitting:

  - RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure

  - UE information disclosure: laboratories use the spreadsheet in [TBD] to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

    - Information of the devices that are going to be measured shall be shared with the RAN4 Secretary as soon as available (i.e., before the measurement activity on such devices starts)

    - The RAN4 Secretary updates the summary of statistical information (see point 5.e) and publishes it to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds defined in point 6.b in a timely manner and take the proper actions if these are not met

  - Skip unchanged part.

  - Skip unchanged part:

  - Skip unchanged part:

- Percentage of the devices that are certified by at least one of certification bodies as following: PTCRB ,GCF, and NAL/CTA (Chinese network access licensed test)] /FCC/CE

      - Once the device gets the above certification, for RAN4 discussion that means the device is not a prototype

- Percentage of the devices that are certified for each certification body (for information only)

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Only the results from aligned labs will be considered for specifying requirements

  - Requirements will not be specified if the following thresholds [further check in RAN4#109] are not satisfied by the devices pool:

    - Minimum number of devices for each band, each device size, each power class: [40]

    - Minimum number of device models: [30]

    - Minimum number of devices' vendors: 5

    - Percentage of devices from [2021 to 2024]: [100%]:

    - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: [100%]

Issue 2-4-1: Device Information Collection

Agreements:

- The Device Information Collection template is agreed in R4-2321191.

Issue 2-4-2: Rel-18 TRP TRS measurement data pool

Agreements:

Rapporteur will initiate an activity among test labs and interested companies those are going to provide devices for measurement campaign, to collect the following information.

- The number of DUTs (minimum 3, maximum 15) for each band the aligned test lab expected to be able to measure and submit to RAN4.

- Check how many samples interested companies planned to provide for each band (with support of UE pre-configuration for measurements).

- Whether test lab will measure the provided device based on further decisions between two sides

Issue 2-4-3: number of receive antennas for n1 or n3 requirements

Agreements:

- RAN4 should develop 4Rx requirements based on measurements of 4Rx devices for n1 in Rel-18. 2Rx requirement can also be defined with no 2Rx measurement campaign. Offset approach is considered as a solution to define 2Rx requirements.

Issue 2-4-4: How to identify receive antennas for n1 or n3 UE

Agreement:

- Contacting with corresponding OEM for seeking help is the most efficient way.

- RAN4 will further discuss and provide other proper solutions.

Issue 2-4-5: How to define PC3 requirements based on PC2

Agreement:

- For a band supporting both PC2 and PC3, specify PC3 requirements based on finalized PC2 requirements, with [2.5~3] dB offset for TRP as a starting point. Whether single offset to all bands is FFS. Further study impacts on TRS under PC2 vs PC3.

Issue 2-4-6: New CBW for band n28/n41/n77/n78 requirements

Agreements:

- Further check next meetings on adding CBW for the above bands for both TRP and TRS testing.

- RAN4 should consider the following aspects:

- Whether 2 set of requirements will be defined in RAN4 and other certification bodies?

- Whether UE should pass both requirements? The logic on requirements applicability.

- What is the consistency of these two sets of requirements, can use exactly the same of conducted REFSENS RB scaling?

- Consider other techniques for testing time reduction? e.g., Single point offset method. Whether there is MU impacts?

- What is the test parameter (e.g., aligned with conducted test case, or pure new parameter) for these CBW?

- Whether a new measurement activity is needed for this?

- Whether RAN4 will consider additional bands in the future?

Annex: coherent UL-MIMO simulation assumption (for information)

During TRP measurement, maximum power is transmitted all the time as “power up” command is sent to UE constantly. In other words, there is no need to simulate power control.

Assume the total available power is PT = 10(26/10) mW, PA and PB are conducted power from Antenna A and B, respectively, PV is a random variable between [0, 10(4/10)] with a uniform distribution (to be agreed). Then we have

PA + PB = PT    (1)

PA = PB * PV    (2)

PA and PB can be solved by putting equation (2) to (1)

PB = PT / (1 + PV)         (3)

PA = PT * PV / (1 + PV)    (4)

During simulations, PV can be generated every T ms. The value T represents how quickly relative power level between antenna A and B changes.

The phase variation for coherent UL MIMO has two scenarios due to different implementations.

Case A: the change of relative phase between two antennas is a random variable [0, 40] degrees with a uniform distribution.

Case B: the change of relative phase between two antennas is a random variable [0, 360] degrees with a uniform distribution.

The reason for the two use cases is that there is about 200ms measurement time including dwell period, etc. Some implementations may send TPMI at the beginning of 200ms, others could send TMPI closer to the actual uplink transmission, e.g. within 20ms of uplink transmission. As those uncertainties cannot be resolved, it is therefore prudent to simulate both scenarios to effectively assess best and worst cases.

2.4.2	Remaining Open issues

- Core part

    - None

- Performance part

    - Concluding RC harmonization activity

    - Concluding AC lab alignment activity

    - Rel-18 TRP TRS measurement campaign

    - Specify TRP TRS Requirements

2.5	RAN5

2.5.1	Agreements

RAN5 #101

- RC MU update was endorsed in [12].

- Pending MU values for AC MU were finalized as endorsed in [6].

- TP on forearm phantom and other MU updates was endorsed in [13], including the agreed updates to TRP TRS measurement grids in [14]

2.4.2	Remaining Open issues

- None

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

4.	References

RAN4 #108bis

- R4-2315156	on RC and AC harmonization criteria	Huawei, HiSilicon

- R4-2315157	on TRP and TRS issues	Huawei, HiSilicon

- R4-2315158	TP to TR 38.870 on TRP TRS test procedure for CA	Huawei, HiSilicon, Rohde & Schwarz

- R4-2315381	TP to TR38.870 on UL MIMO radiated output power metric	Apple

- R4-2315382	On multi-Tx radiated output power methodologies	Apple

- R4-2315383	On the rationale for CA TRP/TRS	Apple

- R4-2315384	TRP TRS device pool information	Apple, Telecom Italia

- R4-2315432	Further discussion on 2TX configuration	Xiaomi

- R4-2315557	Discussion on TRP test method for UL MIMO and TxD	Samsung

- R4-2315838	3GPP TR 38.870 v0.6.0	vivo

- R4-2315839	TP to TR 38.870 on phantoms	vivo, CTIA Certification

- R4-2315840	TP to TR 38.870 on 2Tx test configuration	vivo

- R4-2315841	Discussions on AC test method	vivo

- R4-2315842	Working procedure for Rel-18 TRP TRS lab alignment and harmonization campaign	vivo

- R4-2315843	Updated Schedule for Rel-18 TRP TRS measurement campaigns and requirements	vivo

- R4-2315844	(Template) Measurement results for 3GPP Rel-18 TRP TRS performance test campaign	vivo

- R4-2315845	CR to 38.161 on New test configurations for Rel-18 TRP TRS	vivo

- R4-2315897	draft CR to TS 38.161 on New test configurations for Rel-18 TRP TRS	vivo

- R4-2316225	Discussion on single-layer UL MIMO test methodology	OPPO

- R4-2316319	On TxD and Single-Layer UL MIMO TRP Measurements	Keysight Technologies UK Ltd

- R4-2316506	Further discussion on FR1 2Tx TRP test method	Qualcomm Incorporated

- R4-2316702	Discussion on 2TX test methodology	CAICT

- R4-2316803	Discussion on TRP test method for Single-Layer UL-MIMO and TxD	Google Inc.

- R4-2316835	Further updates to the working procedure for TRP TRS Performance Test Campaign	Telecom Italia, Vodafone

- R4-2316928	Ad-hoc meeting minutes for NR_FR1_TRP_TRS_enh	Vivo

- R4-2316993	draft CR to TS 38.161 on New test configurations for Rel-18 TRP TRS	Vivo

- R4-2316994	TP to TR 38.870 on 2Tx test configuration	Vivo

- R4-2316995	Working procedure for Rel-18 TRP TRS lab alignment and harmonization campaign	Vivo

- R4-2317003	TP to TR 38.870 on 2Tx test configuration	Vivo

- R4-2317961	Topic summary for [108bis][330] NR_FR1_TRP_TRS_enh	Moderator(Vivo)

- R4-2317004	WF on NR_FR1_TRP_TRS_enh vivo

RAN4 #109

- R4-2318103	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity-Huawei	Huawei, HiSilicon

- R4-2318104	Measurement results for 3GPP Rel-18 TRP TRS RC harmonization-Huawei	Huawei, HiSilicon

- R4-2318105	on TxD phase drift and number of receive chains	Huawei, HiSilicon

- R4-2318106	TP to TR 38.870 on TRP TRS test procedure for CA	Huawei, HiSilicon, Rohde & Schwarz, Orange, OPPO, vivo, Vodafone, CAICT

- R4-2318227	Topic summary for [109][335] NR_FR1_TRP_TRS_enh	Moderator (vivo)

- R4-2318231	Adding NR channel bandwidths for OTA TRS testing	Orange

- R4-2318427	TP to TR38.870 on MIMO radiated output power metric	Apple, T-Mobile USA

- R4-2318428	Template for TRP TRS and MIMO OTA Device Information Collection	Apple, Telecom Italia

- R4-2318429	On TRP test time reduction adopting reduced grids	Apple, Vodafone, AT&T, ETS Lindgren, Telecom Italia

- R4-2318431	TP to TR 38.870 on test time reduction adopting reduced grids	Apple

- R4-2318432	On UL MIMO test methodology	Apple, Vodafone, T-Mobile USA

- R4-2318834	On Single-Layer UL MIMO for non-coherent/coherent UEs	Keysight Technologies UK Ltd

- R4-2318835	On Single-Layer UL MIMO TRP Measurement Grid Uncertainties for the Considered Metric Options 1 and 2	Keysight Technologies UK Ltd

- R4-2318965	3GPP TR 38.870 v0.7.0	vivo

- R4-2318966	TP to TR 38.870 on TRP TRS test method	vivo

- R4-2318967	Proposals for concluding the core part work of TRP TRS WI	vivo

- R4-2318968	Discussions on TRP TRS requirement related work	vivo

- R4-2318969	Analysis of 3GPP TRP TRS AC lab alignment and RC harmonization measurement results	vivo

- R4-2318970	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity	vivo

- R4-2318971	Measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity	vivo

- R4-2318972	CR to TS 38.161 on New test configurations for Rel-18 TRP TRS	vivo

- R4-2319270	Discussion on TRP TRS measurement grid for coherent UL MIMO	Samsung

- R4-2319288	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity	SGS Wireless

- R4-2319635	3GPP Rel-18 TRP TRS RC harmonization from SRTC	SRTC

- R4-2319641	3GPP Rel-18 TRP TRS AC lab alignment activity from SRTC	SRTC

- R4-2319776	Adding NR channel bandwidths for OTA TRP/TRS testing 	Orange

- R4-2319915	Discussion on single-layer UL MIMO test methodology	OPPO

- R4-2319920	3GPP Rel-18 TRP TRS LAD measurement for AC lab alignment_n78	OPPO

- R4-2319921	3GPP Rel-18 TRP TRS LAD measurement for RC harmonization_n78	OPPO

- R4-2320175	Discussion on 2TX test methodology	CAICT

- R4-2320176	Discussion on 2Tx measurement grids	CAICT

- R4-2320177	CAICT measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity	CAICT

- R4-2320178	CAICT measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity	CAICT

- R4-2320246	Discussion on TRP test method for Single-Layer UL-MIMO	Google Inc.

- R4-2320379	Discussion on FR1 2Tx TRP test method	Qualcomm Incorporated

- R4-2320380	TP to TR 38.870 on 2Tx TRP test method	Qualcomm Incorporated

- R4-2320394	Adding NR channel bandwidths for OTA TRP/TRS testing 	Orange

- R4-2320413	Adding NR channel bandwidths for OTA TRP/TRS testing	Orange, Vodafone, AT&T, T-Mobile USA, Verizon, DISH Network, BT plc

- R4-2320600	Orange, Vodafone, AT&T, T-Mobile USA, Verizon, DISH Network, BT plc, Telecom Italia	Orange

- R4-2320617	Updates to the working procedure for TRP TRS Performance Test Campaign	TELECOM ITALIA S.p.A., Vodafone, China Telecom, Orange, T-Mobile USA

- R4-2320626	On usage of coarse sampling grid for TRP/TRS measurement	TELECOM ITALIA S.p.A., Vodafone, China Telecom, Orange, T-Mobile USA

- R4-2320627	On device provisioning for TRP TRS Performance Test Campaign	TELECOM ITALIA S.p.A.

- R4-2320707	TP to TR 38.870 on contents for Annex B	ROHDE & SCHWARZ

- R4-2321092	Ad-hoc meeting minutes for [109][335] NR_FR1_TRP_TRS_enh	vivo

- R4-2321097	TP to TR 38.870 on TRP TRS test procedure for CA	Huawei, HiSilicon, Rohde & Schwarz, Orange, OPPO, vivo, Vodafone, CAICT

- R4-2321098	TP to TR 38.870 on TRP TRS test method	vivo

- R4-2321099	WF on [109][335] NR_FR1_TRP_TRS_enh	vivo

- R4-2321191	Template for TRP TRS and MIMO OTA Device Information Collection	Apple, Telecom Italia

- R4-2321209	WF on [109][335] NR_FR1_TRP_TRS_enh	vivo

RAN5 #101

- R5-236833	TP to TR 38.870 Annex B on changes to MU tables	Apple Benelux B.V.

- R5-236837	Discussion on FR1 TRP TRS OTA spec scope & goals	Qualcomm Korea

- R5-237135	On Alternate EIS Search Procedure with Interpolation	Keysight Technologies UK Ltd

- R5-237137	Validation of TRP/TRS measurement grids with empirical data	Keysight Technologies, CAICT, SAICT, Samsung, vivo, Oppo, Huawei, HiSilicon, Sony

- R5-237151	On TRP test time reduction adopting reduced grids	Apple,Vodafone, AT&T, ETS Lindgren, Telecom Italia

- R5-237152	AC MU Analysis for NR FR1 TRP-TRS (Rel.17)	ROHDE & SCHWARZ

- R5-237209	Updates to RC MU Analysis for NR FR1 TRP-TRS Enhancement	Bluetest AB

- R5-237210	RAN4 progress update and MU impact analysis for Enhanced NR FR1 TRP-TRS test methods (Rel-18)	ROHDE & SCHWARZ

- R5-237211	TP to TR 38.870 Annex B on forearm phantom and other MU changes	ROHDE & SCHWARZ

- R5-237233	On FR1 TRP TRS MU TT Milestones	Apple Benelux B.V.

- R5-237839	On Alternate EIS Search Procedure with Interpolation	Keysight Technologies UK Ltd

- R5-237841	Updates to RC MU Analysis for NR FR1 TRP-TRS Enhancement	Bluetest AB

- R5-237842	TP to TR 38.870 Annex B on forearm phantom and other MU changes	ROHDE & SCHWARZ

- R5-237947	pCR with TRP/TRS MU Updates	Keysight Technologies UK Ltd

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template