3GPP TSG RAN meeting #102										RP-233077

Edinburgh, GB, Dec. 2023

## Status Report to TSG

Agenda item:			9.3.4.4

Title		:	Status report for WI NR_FR2_multiRx_DL; rapporteur: Qualcomm, vivo

| WI / SI Name | Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes | Testing part: No |
| Acronym | NR_FR2_multiRX_DL |  |  |  |
| Unique ID | 950077 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-233076 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: - | Core part: 12/2023 | Performance part: 03/2024 | Testing part: - |
| Overall Completion level | Study Item: - | Core part: 100% | Performance Part: 50% | Testing part: - |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Sumant Iyer |
|  | Company | Qualcomm Inc. |
|  | Email | sumanti@qti.qualcomm.com |
| Rapporteur | Name | Qian Yang |
|  | Company | vivo |
|  | Email | qian9.yang@vivo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

Void

## 2.2	RAN2

Void

## 2.3	RAN3

Void

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.1.1	UE RF Core

##### 2.4.1.1.1	From RAN4#104-e: WF R4-2214457

- To support 4L DL MIMO reception at the UE when configured with 2 active TCI states, polarization multiplex (2 layers/direction) + spatial multiplex (2 directions) is assumed at the UE.

  - Note: This proposal is for general deployment assumption, not aimed at UE RF assumption

- UE RF requirements for simultaneous reception from different directions shall be based on single-layer reception for each DL direction with dual TCI configuration, i.e., total 2 layers for both directions.

- For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline.

- Work plan:

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#104 | Aug. 2022 | Discussion on system assumptions, UE assumptions | Agree on system assumptions |
| RAN4#104-Bis | Oct. 2022 | UE implementation assumptions and requirement concept |  |
| RAN4#105 | Nov. 2022 | UE implementation assumptions and requirement concept | Agree on UE assumptions |
| RAN4#106 | Feb. 2023 | Requirement concept and simulation discussion | Agree on details pertaining to simulation effort |
| RAN4#106-Bis | Apr. 2023 | Simulation results discussion | Draft CR? |
| RAN4#107 | May 2023 | Requirement parameter details | Draft CR |
| RAN4#108 | Aug. 2023 | Requirement parameter details | Feature CR |

##### 2.4.1.1.2	From RAN4#104-Bis-e:

###### 2.4.1.1.2.1	WF R4-2217731

- ‘Panel’ is not referenced in the final UE RF requirements

- Take the following panel assumption for DL reception, noting panel is a logical construct:

- ‘Panel’ is defined as one or multiple as combination of below depending on different UE implementation:

  - Unit of antenna group to control beam independently

  - Within a panel, one beam can be selected and used for DL reception.

  - Across different panels, multiple beams [(each selected per panel)] may be used for DL reception.

  - ‘Beam’ is assumed to mean spatial filter associated with reception.

- [The scenario where a single antenna module is used to receive two AoAs simultaneously should not be excluded. Whether a UE with single antenna module can satisfy the requirement or not will be an implementation issue].

- [If an antenna module can be used to receive two AoAs simultaneously, it is considered to consist of at least two panels, where the understanding of “panel” is based on 1.2 ]

- Further study the power imbalance together with how to specify 2AoA sensitivity

- For single DCI, take the RMC in Annex 1 as starting point for further discussion. (see WF)

###### 2.4.1.1.2.2	WF R4-2217732

- On UE capabilities for the feature ”simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO

  - For UEs supporting single DCI, assume the following set of UE capabilities as pre-requisites:

    - Support of simultaneousReceptionDiffTypeD-r16

    - Support of singleDCI-SDM-scheme-r16

    - Support of 4L DL MIMO

  - For UEs supporting multi-DCIs, assume the following set of UE capabilities as pre-requisites:

    - Support of simultaneousReceptionDiffTypeD-r16

    - Support of multiDCI-MultiTRP-r16

    - Support of 4L DL MIMO

  - Further consider the following options:

    - Option 1: For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline, if the UE supports single DCI scheme. If the UE only support multi-DCI scheme, multi-DCI is used.

    - Option 2: UE RF requirements shall not preclude UEs that do not support the single DCI scheme.

- For core requirement and testing, it is assumed that both SSB and CSI-RS are configured to UE for beam management

- UE panel assumption should follow implementation agonistic manner.

  - The term “panel” is not used in test configuration.

- Requirement discussions need to consider testability issue so that the defined requirement can be properly verified.

##### 2.4.1.1.3	From RAN4#105: WF R4-2220553

- ‘The UE RF requirement is based on a test system that can support multiple fixed relative AoA locations during test.’ is the starting point:

  - Multiple fixed orientation of the AoAs or single fixed orientation of AoA can be considered for test

  - Multiple fixed AoA offset values or single fixed AoA offset value can be considered for core requirement

- Candidate proposals for UE RF requirement concept (others not precluded):

  - Option 1: Requirement is based on 2AoA directional sensitivity statistics

  - Option 2: Requirement is based on the spherical coverage EIS of AoA1 (which is swept over the full sphere), and a fixed/pre-defined power level for AoA2

  - Option 3: Only verify the UE functionality (e.g., go or no-go) under two AoAs with a fixed DL power level

  - Option 4: RAN4 considers specifying the demod requirements of multi-Rx in FR2 with pre-defined side condition, instead of defining the two-AoA spherical coverage requirement

- No more than 2AoA cases are considered in this release as specified in the WID.

- DL polarizations:

  - Option 1: Multiple aspects may be chosen:

    - Aspect 1: The RF requirement can be defined for any AoA pair with assumption that TRP1 uses  polarization when TRP2 uses  polarization and vice-versa without any limitation of angular separation between two AOAs. (R4-2219125)

    - Aspect 2: Limit the polarization combinations (from 4 possible:   and )) for the 2-DL spherical coverage test case (R4-2219852)

  - Option 2: Take the mTRP system level assumptions in TR38.802 as the baseline and further discuss whether any additional constraints on DL polarization are necessary for the RF requirement definition.

  - Other options are not precluded.

- Strive to define single set of requirements for both sDCI and mDCI.

- The terms ‘antenna module’ and “panel” are not referenced in the final UE RF requirement and test configuration

- The scenario where a single antenna module is used to receive two AoAs simultaneously should not be excluded. If an antenna module can be used to receive two AoAs simultaneously, it is considered to consist of at least two panels, where the understanding of “panel” is based on Proposal 1 of 1.2.11

- On “panel”,

  - ‘Panel’ is defined as a group of antenna element that controls beam independently and has the following attributes

    - Within a panel, one beam can be selected and used for DL reception.

    - Across different panels, multiple beams (each selected per panel) may be used for DL reception.

    - ‘Beam’ is assumed to mean spatial filter associated with reception.

  - Confirm that a physical panel with dual polarization is assumed as two “panels”.

  - [Assuming four antenna panels per UE as the reference architecture for deriving the spherical coverage requirement of the spherical coverage requirement for the multi-Rx chain DL reception. The four panels are separated into two pairs, and each pair is composed of two antenna panels with orthogonal polarizations that are co-located.

    - Antenna panels used to receive two AoAs simultaneously is up to UE implementation ]

- For the UE supporting multi-DCI, the RMC of the single carrier can be reused for each layer

- test time optimization can be further discussed and decided in OTA SI

- Test procedures for sDCI and mDCI can be further discussed and decided in OTA SI once the requirement concept and test methodologies are confirmed. Test procedure discussion should be also aligned with requirement discussion and decision including requirement concept

- Consider AoA setup and deployment scenario as a package.

  - the mTRP assumptions achieved by RAN1 in the physical layer design (including TR38.802) can be taken as one of the references in deployment scenario discussion. Interested companies are encouraged to provide their analysis on deployment scenario and associated AoA setup in next meeting.

- The same numerology (SCS and CP) is assumed for the two AoAs in Rel-18.

##### 2.4.1.1.4	From RAN4#106: WF R4-2303708

- 2TRP grid for UE performance evaluation:

  - The 2TRP data set is only complete when each TRP traverses the entire surface of the test sphere.

  - For optimized AoA1 and AoA2 test point/perceived DL direction coverage, apply a full rotation in  and a half rotation in . For optimized AoA1 and AoA2 test point/perceived DL direction coverage, utilize constant-step size grids only.

  - In the coordination system of z-axis pointing to AoA1 (P0), the two AoAs (probes) shall be located in xz plane.

  - Testing both +AoA offset and –AoA offset for each test point shall be the starting point for simulation. The intention is to accommodate the impact from directionality of the AoA1-AoA2 DL orientation vectors. In other words we want to ensure no testing bias is introduced.

- Requirement Concept for UE RF (baseline): Only verify the UE functionality (e.g., go or no-go) under two AoAs with a fixed DL power level. In other words, the UE can achieve EIS performance not worse than YdBm on the test point pair (corresponding to 2 AoAs) and the ratio of qualified test points over the whole sphere is M%.

- UE orientation during verification or simulation: UE orientation w.r.t P0 position (z-axis) is part of core requirement consideration.

- DL pol. assumption for derivation of the UE RF requirement: The UE RF requirement is derived assuming the worst case polarization match between the 2 TRPs. The requirement applies for any combination of DL polarizations from each TRP.

- AoA separation for UE RF requirement:

  - AoA offset value should be an integer multiple of the step size of the constant step size measurement test grid.

  - In the simulation, all AoA separation values in the list {30°, 60°, 90°, 120°, 150°, 180°} shall be simulated.

- Further details for baseline RF requirement: Simulation assumption starting point: Re-use the legacy spherical coverage receiver sensitivity level (single probe) as the DL power for each probe to set the core requirement.

- sDCI vs mDCI:

  - Define the RF requirement based on mDCI with understanding that UE supporting sDCI can also meet the requirement.

  - To have the unified requirement concept for UEs supporting multi-DCIs as for UEs supporting single DCI, RAN4 can focus on fully overlapping in time and in frequency, supported by UE capabilities “multiDCI-MultiTRP-r16” and “overlapPDSCHsFullyFreqTime-r16.”

  - No “joint detect/decode” is considered in simulation for mDCI. In other words, Process only 2 TRP-RX pairs– In this case we only consider two TRP to RX pairs - TRP1-RX1 and TRP2-RX2. The signal from TRP2 to RX1 and TRP1 to RX2 is treated as interference.

- UE simulation assumptions:

  - Simulation output: Statistics per TRP (further combining not precluded from final requirement)

    - Y% @legacy EIS spherical coverage value in dBm as baseline

    - X dBm for legacy coverage fraction (example PC1 = 15%, PC3=50%) in 2TRP operation is encouraged based on contribution driven for the sake of comparison

    - For mDCI, if any one of the two test directions in a pair fails, the result is a fail for the AoA pair.

  - antenna module assumptions.:

    - (2 dual-pol antenna modules, 4x1 elements, combination of front/back, left/right, top/bottom)

    - company choice on bias, antenna module location, module count, antenna gain, but legacy spherical coverage requirement must be met.

- UE RF simulation calibration step: calibrate the simulation baseline with legacy peak EIS spec and legacy spherical EIS spec.

##### 2.4.1.1.5	From RAN4#106-Bis-e: WF R4-2306604

- Baseline assumption for simulation: UE assigns ‘first’ module to track TRP that yields highest RSRP among all combinations of modules and TRPs. The best of the other modules is assigned to track the other TRP

- Module splitting up to UE implementation.

- Same requirement applies to UEs that support multiDCI-MultiTRP-r16 and one of overlapPDSCHsFullyFreqTime-r16 and overlapPDSCHsInTimePartiallyFreq-r16. No need to discuss and define the RF requirement for the UEs only supporting multiDCI-MultiTRP-r16.

- It is up to UE implementation how many panels are activated for multi-Rx UEs during legacy (before R18) requirement verification.

- It is up to UE implementation what receiver is used meeting the RF requirement. In the simulation, take min[SINR_AoA1, SINR_AoA2] >= -1dB as Pass/Fail criterion

- Confirm the baseline requirement concept (e.g. go or no-go) as the agreed requirement concept.

- Data bias in agree TE assumptions:

  - A TE system that uses a legacy 2-axis positioner as measurement setup for multi-Rx UE RF testing will introduce different measurement results for different UE orientations because test AoA pairs only lie along meridians of the UE centric grid. i.e., no test AoA pairs in other directions.

    - The work on the enhanced positioner, including motivation, conceptual solution, implementation aspects shall be recorded in the accompanying SI TR38.871 as record of having considered the technical aspects associated with the legacy positioner.

  - To balance pragmatic considerations against the goal of removing the different measurement results for different UE orientations, the enhanced positioner is not adopted in testing and the requirement is agreed to be framed based on the previously agreed legacy positioner.

  - The requirement applies for one of the FFS options below:

    - Option 1: one or more UE-declared orientations in the positioner. FFS if UE orientation is packaged with an AoA separation angle as a pair.

    - Option 2: one or more standard-specified UE orientations in the positioner.

    - Option 3: the requirement applies for two fixed AoA configurations, the UE orientation can be declared (no capability needed) for each of these two directions.

- Baseline requirement metric: For a specific angular separation between 2 TRPs and a specific UE orientation under standardized DL power level which is equal between 2 TRPs, the result at each test point is constructed based on two AoA pairs containing that test point, i.e., AoA+ pair and AoA- pair. Overall result (probability to support 2TRP DL) is by averaging regional results.

- Both DL powers are same as the legacy spherical coverage EIS requirement as starting point.

- UE implementations to study for requirement derivation:

  - Baseline: Adjust the beam shape or scale the antenna gain to make UE align with both peak EIS and spherical coverage.

- UE RF requirements shall assume that a dual-polarized (V/H) receiver pair is used to receive the DL signal from each direction.

- Define complete RF requirements for 28GHz bands first. 39 GHz and 47 GHz bands can be evaluated if time allows.

- Core requirement for multiRx will be defined for all supported channel bandwidths. Single CBW is selected for core requirement verification, FFS which CBW is chosen.

- (simulation flow agreed, see WF for details)

##### 2.4.1.1.5	From RAN4#107: WF R4-2310491

- The two-AoA spherical coverage requirement defined under this WI does not apply to UEs only supporting multiDCI-MultiTRP-r16.

- For multi-Rx DL reception of PC3, confirm that the legacy spherical power level is used as the fixed DL power.

- sDCI and mDCI share the same requirement under the current fixed DL power scheme, and no need to define additional requirements for sDCI UE.

- The UE RF requirement is defined as the average (arithmetic mean) of the metric values for two DL polarization test conditions: the first condition is when the DL polarizations at both TRPs are ‘’ and the second condition is when they are ‘’

- No UE capability will be specified for different levels of performance. The implementations of two equally and unequally capable antenna modules could be considered when specifying the requirements.

- The UE RF requirement applies to UE declared orientation(s). The UE RF requirement is derived assuming each UE is evaluated in the orientation that yields the best metric value. Candidate orientations for UE to choose from are all the ‘Alignment Options’ in Annex J (J.2) of 38.101-2.

- A new clause with dedicated suffix 7.3X is agreed. (X is placeholder, not agreed suffix)

- ‘Scale the antenna gain’ method to make UE align with both peak EIS and spherical coverage applies only to schemes that scale all modules equally for simulation.

##### 2.4.1.1.6	From RAN4#108: WF R4-2314668

- The UE only needs to meet the requirement for 1 AoA offset.

- Alternative calibration method for UE models:

  - The DL power level used in simulation will be adjusted to UE’s EIS CCDF@50%-xile.

  - This means the simulation results for PC3 provided by companies should not exceed 50% for any AoA offset.

  - The performance of UE model must meet the legacy REFSENS and EIS spherical coverage requirement

- Procedure to calculate signal and interference levels when gain imbalance exists between V-pol and H-pol in UE module, along with the alternative calibration method for UEs is introduced (see WF)

##### 2.4.1.1.7	From RAN4#108-Bis: WF R4-2314668

- Data processing of company inputs:

- There are 9 starting UE orientation options per annex J of TS38.101-2 for multiRX.

- Companies are encouraged to consider realistic packaging including metal and plastic housings, as well as H and V imbalance.

- Three types of reference UE implementation (two panels on the same side, two panels on the adjacent side and two panels on the opposite side) will be used to determine the core requirement:

    - If the AoA offset would be declared by UE

| AoA offset (degrees) | 30 | 60 | 90 | 120 | 150 |
| --- | --- | --- | --- | --- | --- |
| Reference UE | same | same | adjacent | opposite | opposite |

    - If the AoA offset would be specified in the standard.

| AoA offset (degrees) | 30 | 60 | 90 | 120 | 150 |
| --- | --- | --- | --- | --- | --- |
| Reference UE | Min (same, adjacent, opposite) | Min (same, adjacent, opposite ) | Min (same, adjacent, opposite ) | Min (same, adjacent, opposite ) | Min (same, adjacent, opposite ) |

- NTC vs ETC applicability of core requirement: Do not use the note in the multi-RX spherical coverage requirement, and RAN4 agrees the requirement shall be verified under NTC only. Also, RAN4 ask RAN5 to verify it for NTC only.

##### 2.4.1.1.8	From RAN4#109: Feature CR R4-2321722

(See CR)

#### 2.4.1.2	UE RRM Core

##### 2.4.1.2.1	From RAN4#104-e: WF R4-2214344

Number of searchers for cell detection and measurements

  - 2 searchers are assumed.

Feasibility/necessity of enhancing requirements for L1 measurements

  - Following L1 measurements related requirements are considered in the multi-Rx chain WI

    - L1-RSRP measurement

    - BFD/CBD

    - RLM

    - L1-SINR

    - TCI state switching with dual TCI

    - Scheduling/measurement restrictions

- RRM  core part work plan

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#104 | Aug. 2022 | Study and initial discussion on the feasibility/necessity of specifying RRM core requirements for the following procedures/measurements, and corresponding core requirements - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) | Conclusion on the study/feasibility/necessity of at least some of the requirements Approve the RRM work plan |
| RAN4#104-Bis | Oct. 2022 | Continue discussion on the feasibility/necessity of specifying RRM core requirements for the following procedures/measurements, and corresponding core requirements - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) | Conclusion on the study/feasibility/necessity of all the requirements Agreements on assumptions of receive timing difference between different directions Initial agreements on the confirmed RRM requirements |
| RAN4#105 | Nov. 2022 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements |
| RAN4#106 | Feb. 2023 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements |
| RAN4#106-Bis | Apr. 2023 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements Work split on the Draft CRs |
| RAN4#107 | May 2023 | Continue discussion on the remaining issues for the RRM core requirements | Initial discussion on the Draft CRs Further agreements on the confirmed RRM requirements |
| RAN4#108 | Aug. 2023 | Continue discussion on the remaining issues for the RRM core requirements | Conclude all the RRM requirements Endorse all the Draft CRs for core part of the WI |

##### 2.4.1.2.2	From RAN4#104-Bis-e:

###### 2.4.1.2.2.1	WF R4-2217246

- Scope and scenarios:

  - Joint combination of multi-Rx chain with NR-U, RedCap and NTN are not in the scope of the WI.

  - RRM requirement discussion shall be focused on the case with different QCL TypeD RSs on a single component carrier.

  - Clarify the SSB and CSI-RS based simultaneous L1 measurement scenario before making conclusion on scenarios for Rel-18 multi-Rx DL reception, i.e., for simultaneous reception of different QCL type D RSs, what RSs combination(s) can be considered and which scenario(s) it is aimed at.

- General aspects:

  - UE behaviour and capability of handling Rx signal level difference between two channels may be discussed in RF/demodulation session.

  - Enhanced RRM requirements for multi-Rx chain UE should maintain the existing accuracy requirements.

  - Whether to define RRM requirements for activation delay from a single antenna panel to multi-antenna panels should be decided after RF conclusion.

- Receive timing difference:

  - Not to discuss UL transmission in R18 FR2 multi-Rx WI.

- Applicability and conditions:

  - RRM requirement of simultaneous DL reception from different directions shall be defined based on the applicable condition to be specified in UE RF session.

- To clarify the requirement in new section that R18 multi-Rx is applicable for FR2-1 only, i.e., no FR2-2.

###### 2.4.1.2.2.2	WF R4-2217587

- L1 RSRP/SINR Measurements: The number of samples assumed to define the accuracy requirements should not be changed

- BFD/BFR: CSI-RS based BFD/BFR is in the scope of the current WI

- Multi-Rx and other features: Do not discuss join requirements for multi-Rx and NR-U, NTN and RedCap at least until all the requirements for multi-Rx are finalized

###### 2.4.1.2.2.3	WF R4-2217248

- Requirements and Scenarios: Define DL TCI state switch requirements. UL TCI state switching, and UL spatial relation info switch are not in the scope of the WI.

- Scenarios to be considered w.r.t Intra-cell/Inter-cell multi-TRP: Follow conclusion from thread 211

- Scenarios to be considered w.r.t single DCI/multi-DCI: Follow conclusion from thread 211.

- TCI Switching:

  - Dual TCI state switching requirements shall be based on Rel-15/Rel-16 TCI framework

  - Can the TCI switch is assumed to be independent on each RX chain?: For each of the two TCI states, the TCI state switch is assumed to be independent

  - Switch command for dual TCI state switch

  - When two TCI states are switched simultaneously, assumption on the switch commands: Wait for conclusion on scenarios in other thread w.r.t sDCI vs mDCI.

  - TCI state switch scenarios to be considered: Wait for conclusion on scenarios in other thread w.r.t sDCI vs mDCI.

  - Does the cross-panel switch time needs to be defined: RRM not to define additional TCI state switching delay for cross panel TCI state switching. If RF session achieves a new conclusion on panels ON/OFF switch time, RRM session may revisit the issue if required

##### 2.4.1.2.3	From RAN4#105:

###### 2.4.1.2.3.2	WF R4-2220742

- Assumption on supporting 4-layer MIMO with simultaneous DL reception with two different QCL TypeD RSs

  - group based reporting is one of the means to enable simultaneous reception

  - companies proposing to enable simultaneous reception should bring more analysis on how this can be done

- Consider both sDCI and mDCI

- L3 measurements in RRC Connected mode requirements enhancements should be studied and specified if necessary

  - Note: other enhancements are deprioritized

###### WF R4-2220743

- Overall handling of L1 measurements: Discussion on Rx beam sweeping scaling factor, simultaneous reception of DL data and RS, and other multi-Rx related topics can be done jointly for L1-RSRP, RLM, L1-SINR and CBD/BFD.

  - Note: issues specific to each of the above measurements can be discussed separately.

###### 2.4.1.2.3.3	WF R4-2220402

- Regarding group-based reporting and simultaneous reception with different QCL type-D: group based reporting is one of the means to enable simultaneous reception

- As per RAN1 agreements, group-based reporting is supported for single DCI only. If group-based reporting is a prerequisite, this AI can focus on single DCI case only.

- Definition of Dual TCI state switch

  - Dual TCI state switching refers to the case when two active TCI states with QCL type D for simultaneous reception of PDCCH/PDSCH are involved before and/or after TCI switching command.

  - In other words, following are considered as dual TCI state switching to discuss the requirements

    - Single DCI to Dual TCI

    - Dual TCI to Dual TCI

    - FFS: Dual TCI to Single TCI

- TCI state switch indication method for PDCCH and PDSCH

  - PDCCH non-SFN

    - Only single PDCCH is supported. So Single MAC CE should be used

  - FFS: PDCCH SFN

    - One MAC CE is used to indicate both TCI states

  - PDSCH

    - For both SFN and non-SFN, only one DCI code point is indicated using single DCI and the code point may indicate one or two TCI states

- Known and unknown states combination: Dual TCI state switching requirements are defined for simultaneous reception. It is FFS whether we need to define any new requirements for non-simultaneous reception. The dual TCI states known, or unknown are FFS.

##### 2.4.1.2.4	From RAN4#106:

###### 2.4.1.2.4.1	WF R4-2303302

- Scope of the WI

  - No further discussion is needed for Option 1.

  - Discuss the requirements for the objectives in the WID directly.

- m-TRP scenarios for Rel-18 multi-Rx DL reception

  - Focus on intra-cell multi-TRP operation scenario

  - No requirements will be introduced for inter-cell multi-TRP operation in R18.

- Necessity of group-based beam reporting for simultaneous reception

  - Rel-17 group-based reporting is used as a prerequisite to define requirement for simultaneous reception

  - Note: Simultaneous reception term above includes Data/Data, RS/RS and Data/RS simultaneous reception cases.

- Simultaneous L3 measurements and L1 measurements

  - Simultaneous L3 and L1 measurements is not supported in this release for multi-Rx.

  - Note: The agreement has no impact on scheduling restriction discussion for L3 measurements. Feasibility and benefit of scheduling restriction relaxation for L3 measurements is FFS.

- Baseline L1 measurement requirements for potential multi-Rx enhancement

  - For cell specific RLM: section 8.1

  - For cell specific link recovery: section 8.5

  - For TRP specific link recovery: section 8.18

  - For L1-RSRP measurement: section 9.5.

###### 2.4.1.2.4.2	WF R4-2220743

- TCI State Switching

  - Group Based Reporting and s/mDCI

    - Group based reporting applies to both sDCI and mDCI

  - Investigate and if needed specify requirements for the following TCI state switching scenarios

    - single TCI to dual TCI

    - dual TCI to dual TCI

    - dual TCI to single TCI

  - TCI Switch triggering method:

    - MAC-CE based dual TCI state switch for PDCCH reception

    - DCI based dual TCI state switch for PDCSH reception

- MRTD Handling:

  - Define RRM requirements for MRTD < CP

  - Note: the MRTD above is the maximum receive time difference between the signals coming from two directions on the same carrier

##### 2.4.1.2.5	From RAN4#106-Bis-e:

###### 2.4.1.2.5.1	WF R4-2306318

- Single (component) carrier for defining RRM requirements

  - Deprioritize discussions on RRM requirements for CA/DC scenarios till single carrier requirements are completed.

- Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel

  - Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel to receive two independent signals from the same or nearly the same direction is not further discussed in RRM session.

  - Note: It can be revisited in case any RRM impact is identified.

- Scenarios/assumption for “simultaneous reception”

  - Scenarios for mTRP simultaneous reception is to be covered as part of scheduling restrictions/measurement restrictions, L1 measurement, and TCI state switching agenda items

- Whether to define power saving related requirements

  - No power saving specific requirements, e.g., L1 measurements relaxation for multi-Rx operation, are considered in the WI.

- UE architectures

  - Multi-Rx architecture impact to RRM requirements can be considered on a case-by-case basis, if needed

- Clarification/understanding on R16 UE capability simultaneousReceptionDiffTypeD

  - It is RAN4 understanding that the R16 UE capability simultaneousReceptionDiffTypeD is only applicable for simultaneous PDSCH reception.

- UE capability for simultaneous reception in Rel-18

  - The issue is postponed until there is sufficient progress on requirements.

- UE capability of simultaneousRxDataSSB-DiffNumerology-Inter-r16

  - No need to discuss simultaneousRxDataSSB-DiffNumerology-Inter-r16 in R18 multi-Rx chains WI

- RLM

  - No further discussion on TRP specific RLM as there is no such procedure in existing specification.

- Number of samples

  - For RLM with multi-Rx chain operation, the same number of samples Mout and Min are used as in Rel-17.

  - For link recovery with multi-Rx chain operation, the same number of samples MBFD and MCBD are used as in Rel-17.

- PDCCH configuration

  - For RLM with multi-Rx chain operation, the same PDCCH transmission configuration is used as in Rel-17.

  - For link recovery with multi-Rx chain operation, the same PDCCH transmission configuration is used as in Rel-17.

###### 2.4.1.2.5.2	WF R4-2306319

- Multi-Rx L1 measurement requirements are defined under assumption of the following reference signals availability

  - CSI-RS + CSI-RS: CSI-RS reference signals are transmitted from the two TRPs

  - SSB + SSB: SSB signals are transmitted from the two TRPs

  - FFS

    - SSB + CSI-RS: SSB and CSI-RS signals are transmitted from different TRPs

  - FFS if same or different RS combinations would apply for different RRM requirements

  - FFS whether to consider simultaneous and/or non-simultaneous RS transmission from different TRP to define RRM requirement

- Introduce enhanced RRM requirements based on faster beam sweeping with multi-Rx chains based on UE capabilities

  - Option 1: Introduce beam sweeping factor reduction

  - Other options not precluded

  - FFS on specific conditions when enhancement apply

  - FFS if any signalling is needed

  - FFS on details of UE capabilities

How to capture the enhanced requirements (based on the agreement of enhanced requirements) in the spec due to faster beam sweeping can be discussed during CR stage.

- Whether to consider reducing the sharing factor for L3 and L1

  - Do not consider enhancement to sharing factor for L3 and L1 in this WI

- Issue 1-4-3: L1-RSRP measurement accuracy requirements

  - It was agreed to discuss in performance part

- Issue 2-1-1: General conditions

  - RAN4 to define TCI state switch delay requirements only for the case where to-be-activated/switched TCI state upon the TCI switch command(s) reception includes two different QCL-TypeD reference resources and the reference resources of the two TCI states are included in the group-based L1-RSRP measurement report.

    - Note: This is one of the condition and other conditions are not precluded

    - Note: Only for single TCI to dual TCI and dual TCI to dual TCI.

- RAN4 not to discuss TCI state switching requirements for following cases in this WI

  - Dual TCI switching in SFN

  - FFS: PDCCH with repetition.

- Single DCI SFN

  - No discussion in this WI.

- Multi-DCI SFN

  - No discussion in this WI.

- Requirements to be consider

  - Requirements for DCI based dual TCI states switch delay for PDSCH reception are defined for known case only.

  - Requirements for MAC CE based dual TCI states switch delay for PDCCH reception are defined for known case. FFS if it is to be limited to known case only.

- Active TCI state list update

  - For active TCI state list update for addition of a new dual TCI states, legacy requirements can be used as baseline. FFS if T/F tracking for the dual TCI states are based on different SSBs.

- Whether to consider RTD larger than CP in multi-RX WI

  - Agreed to postpone the discussion of MRTD > CP until RAN4#107.

- MRTD for UE not supporting RTD>CP

  - Agreed to postpone the discussion of MRTD > CP until RAN4#107.

##### 2.4.1.2.6	From RAN4#107:

###### 2.4.1.2.6.1	WF R4-2310046

- Definition of “simultaneous reception”

  - No need to discuss definition of ‘simultaneous reception’. The condition can be discussed during CR phase.

- Beam management related

  - Not to use the principle of independent beam management to define the requirement for R18 multi-Rx chains WI. Instead, RAN4 to directly discuss individual detailed requirements.

- RRM impact of group-based beam reporting for simultaneous reception

  - The requirements for GBBR are discussed under L1-RSRP enhancement.

- Applicability of new requirements to different QCL types

  - Scenarios where QCL type D is configured in combination with QCL type A/C are not precluded in the requirements for multi-rx operation.

- Beam sweeping factor for cell specific RLM and BFD/CBD for multi-Rx

  - Faster beam sweeping is not applicable for CSI-RS based RLM and BFD measurements, due to N=1 in the existing requirements.

- Conditions/cases that scheduling restriction for L1 measurements can be relaxed for multi-Rx

  - In multi-TRP operation scenario, for the case PDCCH/PDSCHs are transmitted from two TRPs simultaneously, and CSI-RS is transmitted from anyone of the TRPs, scheduling restriction relaxation can be made for CSI-RS based L1 measurements with multi-Rx when following conditions are met

    - The CSI-RS is not in a CSI-RS resource set with repetition ON

    - The CSI-RS and one of the PDCCH/PDSCHs are transmitted through different TRPs at the same time

    - CSI-RS has same QCL source as the active TCI state of any one of the PDCCH/PDSCHs

    - Resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair

    - UE is activated with multi-Rx operation

Note: The wording on the 2nd and 3rd bullet can be further polished.

- Conditions/cases that scheduling restriction can be relaxed for SSB based measurements for multi-Rx

  - For SSB based L1 measurements, scheduling restrictions relaxation is not considered for UE supporting Rel-18 multi-Rx capability.

- CR work split

| No. | Requirements | Remarks | Company |
| --- | --- | --- | --- |
| 1 | RLM in clause 8.1 | RLM evaluation period RLM measurement restriction Scheduling restriction for RLM | Ericsson |
| 2 | BFD in clause 8.5 | BFD evaluation period BFD measurement restriction | OPPO |
| 3 | CBD in clause 8.5 | CBD evaluation period CBD measurement restriction Scheduling restriction for CBD | ZTE |
| 4 | TRP specific BFD in clause 8.18 | TRP specific BFD evaluation period TRP specific BFD measurement restriction TRP specific BFD scheduling restriction Clause 8.18 | Nokia |
| 5 | TRP specific CBD in clause 8.18 | TRP specific CBD evaluation period TRP specific CBD measurement restriction TRP specific CBD scheduling restriction Clause 8.18 | ZTE |
| 6 | L1-RSRP measurement in clause 9.5 | Intra-frequency L1-RSRP measurement period L1-RSRP measurement restriction [Applicable condition for GBBR] [Reporting requirements for GBBR] Scheduling restriction for L1-RSRP | Huawei |
| 7 | MAC CE based TCI switch [RRC based TCI switch] | MAC CE based TCI switch [RRC based TCI switch] | Ericsson |
| 8 | Known condition DCI based TCI switch Active TCI state list update | Known condition DCI based TCI switch Active TCI state list update | Apple |
| 9 | [L1-SINR in clause 9.8] | L1-SINR measurement period L1-SINR measurement restriction Scheduling restriction for L1-SINR | Nokia |

###### 2.4.1.2.6.1	WF R4-2310047

- Conditions for beam pair selection:

  - Do not introduce additional conditions for beam pair reporting in GBBR on top of the conditions defined in RAN1 specifications in Rel-18

- L1-RSRP for GBBR:

  - GBBR measurement delay requirements will be defined under assumption that UE uses a single Rx panel for measurements at one time instance

Companies are requested to bring further analysis on L1-RSRP requirements for GBBR in next meeting

- Methods to achieve faster beam sweeping

  - RAN4 to define new optional UE capability for beam sweeping factor reduction for SSB-based L1-RSRP measurement if the UE is capable of multi-Rx operation.

- Candidate values for beam sweeping factor reduction:

  - The candidate number can be {2, 4, 6} for FR2-1.

- Issue 2-1-1: The TCI state reference signals reception for T/F tracking

  - Tfirst-SSB defined for the existing TCI state switch delay requirements can be reused for dual TCI switch in mTRP if the definition of Tfirst-SSB is redefined to account for two TDM’ed source SSBs in the QCL chains with two TRPs

    - FFS which requirements, e.g., MAC-CE based, active TCI state list update, can be applied.

    - Tfirst-SSB1 is the first SSB for one TCI state of dual TCI states, and Tfirst-SSB2 is the first SSB for the other TCI state of dual TCI states after TCI state switch command.

- sDCI non-SFN without PDCCH repetition

  - For MAC-CE based PDCCH TCI state switch for s-DCI scenario, legacy TCI state switching requirements apply for MAC-CE based PDCCH TCI indication method for PDCCH.

- sDCI PDCCH repetition

  - For MAC-CE based PDCCH TCI state switch for s-DCI PDCCH repetition, the requirement is defined with the delay in current requirement [+ [250]us additional delay].

- Multi-DCI (mDCI) non-SFN

  - For MAC-CE based PDCCH TCI state switch for m-DCI scenario, reusing legacy requirements for MAC-CE based PDCCH TCI state switch and it applies per TRP

    - FFS if the two PDSCHs carrying the two MAC-CEs are in the same slot. If the two PDSCHs carrying the two MAC-CEs are in the same slot, consider [250]us additional delay.

- RRC based TCI state switch

  - The requirements for multi-RX operation on RRC based PDCCH TCI state switch will be considered only if specifications support the procedure.

    - FFS: The procedure can include TCI state switch to single TCI, or switch to Dual TCI.

- Known conditions

  - Dual TCI states are known if the

    - dual TCI states are QCL-ed to reported beam pair (i.e., RS resources pair) within one group

      - All the RSs in the QCL chain remain detectable

    - The dual TCI states remains detectable during the TCI state switching period

    - RSs configured for dual TCI states are reported in last [1280]ms

Note: FFS whether additional conditions are needed for tests.

- Active TCI state list update delay requirement

  - Use following agreement to derive the equation for TCI state list update

    - Tfirst-SSB defined for the existing TCI state switch delay requirements can be reused for dual TCI switch in mTRP if the definition of Tfirst-SSB is redefined to account for two TDM’ed source SSBs in the QCL chains with two TRPs

    - Tfirst-SSB1 is the first SSB for one TCI state of dual TCI states, and Tfirst-SSB2 is the first SSB for the other TCI state of dual TCI states after TCI state switch command.

##### 2.4.1.2.7	From RAN4#108:

###### 2.4.1.2.7.1	WF R4-2314477

- Rx beam assumption for “simultaneous reception”

  - No need to specify AoA separation conditions in RRM core requirements for simultaneous reception from 2 directions.

    - It may be captured as side condition in the test for multi-Rx operation.

  - How Rx beams are used to receive two AoAs simultaneously is up to UE implementation.

- Indication of multi-Rx operation

  - Introduce a new UE assistance information to indicate to network of the UE preference on multi-Rx operation due to [power saving or] overheating purposes (i.e., UE preference on not supporting simultaneous reception with different QCL-typeD).

    - FFS on the detailed set of associated UE capabilities

  - The signalling details are up to RAN2.

  - Send LS to RAN2.

- UE behaviour when a condition becomes violated during a measurement

  - FFS if it is to define particular behavior when side conditions for multi-Rx operation are violated

- UE capability for indication of whether additional delay is needed in dual TCI state switching for multi-Rx

  - FFS: To indicate whether additional delay is needed in dual TCI state switching. By default, a UE should support dual TCI state switching without the additional delay.

- UE feature list

  - UE capabilities related to multi-Rx operations shall be discussed in UE feature list.

- Measurements on two RS resources simultaneously for cell specific RLM and BFD/CBD for multi-Rx

  - The UE is able to perform RLM and cell specific BFD measurements on two overlapping RSs from different directions simultaneously under certain conditions.

- Evaluation period requirements

  - For CSI-RS based RLM/BFD/CBD measurements in FR2, the existing evaluation periods are reused under multi-Rx operations.

  - For UE capable of faster beam sweeping, the reduced beam sweeping factor can be applied for defining the evaluation period of SSB based RLM/BFD measurements in FR2.

- Other aspects of RLM and cell specific BFD/CBD requirements for multi-Rx

  - For s-DCI scenario, reuse RLM requirements in section 8.1 of TS 38.133.

  - For s-DCI scenario, reuse BFD/CBD requirements in section 8.5 of TS.38.133.

  - FFS: For m-DCI scenario, reuse RLM requirements in section 8.1 of TS 38.133, with the clarification that in multi-TRP scenario, the RLM-RS for evaluating out-of-sync and in-sync may originate from two different TRPs.

  - For m-DCI scenario, reuse link recovery requirements in section 8.18 of TS 38.133.

- TRP specific BFD requirements enhancement for multi-Rx

  - PTRP=1 for CSI-RS based TRP specific BFD requirements for multi-Rx operation under following conditions,

    - Both CSI-RS are not in any CSI-RS resource set with repetition ON

    - UE is activated with multi-Rx operation

    - Resources of the active TCI states for the two PDCCHs or PDSCHs and one PDCCH and PDSCH have been reported via GBBR in a pair

    - Each CSI-RS has same QCL source as the active TCI state of the PDCCH/PDSCH [from the same TRP]

    - The two CSI-RS resources in the two sets q ̅_0,0 and q ̅_0,1for beam failure detection is transmitted through different [TRPs or QCL sources] at the same time

    - [Both of the CSI-RSs and both of the PDCCH/PDSCHs are transmitted at the same time]

    - Note: additional conditions are not precluded

- TRP specific CBD requirements enhancement for multi-Rx

  - TRP sharing factor for TRP specific CBD requirements for multi-Rx operation is not enhanced.

- Whether to enhance TRP specific BFD/CBD procedure

  - For TRP specific link recovery, enhancement on BFD/CBD procedure and requirements is not considered in Rel-18.

- Update conditions/cases for scheduling restriction that can be relaxed for CSI-RS based L1 measurements for multi-Rx

  - Discussed with CR by taking agreements on Issue 2-2-1 into consideration.

- Cases whether scheduling restriction can be relaxed for L1 measurements for multi-Rx

  - Introduce scheduling restriction relaxation (enhancement) for CSI-RS based RLM, BFD which includes cell specific BFD and TRP specific BFD, and non-group-based and group-based L1-RSRP measurements with repetition OFF. Detailed conditions are FFS.

  - FFS whether scheduling restrictions are needed for UEs supporting simultaneous reception with mixed numerologies

  - No scheduling restriction relaxation (enhancement) for other cases.

- Conditions/cases that scheduling restriction for L3 measurements can be relaxed for multi-Rx

  - Scheduling restriction/measurement restriction relaxation (enhancement) for L3 measurements are not further discussed in Rel-18 multi-Rx WI.

- Cases whether measurement restriction for SSB based L1 measurements can be relaxed for multi-Rx

  - Introduce measurement restriction relaxation (enhancement) for CSI-RS based RLM, BFD which includes cell specific BFD and TRP specific BFD, and non-group-based and group-based L1-RSRP measurements with repetition OFF. Detailed conditions are FFS.

  - FFS whether measurement restrictions are needed for UEs supporting simultaneous reception with mixed numerologies

  - No measurement restriction relaxation (enhancement) for other cases.

- Conditions that measurement restriction for CSI-RS based L1 measurements can be relaxed for multi-Rx

  - Reuse the same principle of scheduling restriction, if possible, and necessary revisions and additional conditions could be considered.

###### 2.4.1.2.7.2	WF R4-2314283

- RS to be considered for L1-RSRP reported via GBBR

  - Do not introduce requirements for Rel-17 GBBR measurements involving both SSB and CSI-RS measurements

- Should the RS configured for measurements reported via GBBR and used for multi-rx be configured based on L3 report?

  - How to configure RS for measurements reported via GBBR is NW implementation. RAN4 not to introduce any configuration restrictions.

- Consideration of non-simultaneous RS measurements for GBBR

  - RAN4 to also consider non-simultaneous RS measurements from different TRPs for Rel-17 group-based L1-RSRP report requirements.

- Measurement period for L1-RSRP configured for GBBR

  - For SSB based L1-RSRP measurements in FR2 the measurement period for GBBR defined as:

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: N is FFS |  |

  - N is FFS

  - FFS: For CSI-RS based L1-RSRP measurements in FR2, the existing L1-RSRP measurement period is reused when configured for GBBR.

- Conditions for to measurement period reduction for L1-RSRP configured for GBBR

  - Set of conditions to be considered in the L1 measurement requirements should be discussed in the corresponding requirements by taking agreements we have already made into consideration. No general agreements are needed.

  - No need to define conditions of full overlap or partial overlap of RSs in the two resource sets for defining requirements for L1-RSRP configured for group-based beam reporting

- Shall L1-SINR requirements be defined for the multi-RX UE

  - FFS: Changes in non-group-based L1-RSRP measurement delay due to multi Rx operation are also considered for L1-SINR

- When UE did not find any beam pair to report

  - No new signalling or requirements are defined for the scenario where the UE could not find any pair of beams to report in Rel-18 multi-Rx WI.

- Issue 1-3-4: Other proposals:

  - SSB can be configured as QCL source for CSI-RS based Rel-17 group-based beam reporting.

- The TCI state reference signals reception for T/F tracking

  - For sDCI mTRP case, T/F tracking for dual TCI states switching is applicable in requirements for MAC-CE based TCI state switch delay and active TCI state list update

    - (Tfirst-SSB) is updated to max(Tfirst-SSB1, Tfirst-SSB2)

- UE behaviour when dual TCI states are not supported

  - No UE behavior is defined in Rel-18 multi-Rx WI when UE cannot receive dual TCI states simultaneously

- Other proposals for further discussion

  - FFS: For DCI-based TCI state switching for sDCI, there is no TCI state switching delay for the case from dual TCI to single TCI state switch when the target TCI is one of the source TCI (e.g. [RS1, RS2] to [RS1]), when UE is configured with GBBR and is NOT configured with non-GBBR.

- Detectable condition

  - For detectable condition, all RSs in the same TCI chain for the target TCI state should remain detectable during the entire measurement/evaluation/TCI state switch period

- DCI based dual TCI state switch for sDCI scenario

  - Reuse Rel-16 requirements as baseline

  - FFS if additional delay is introduced on top of Rel-16 requirements

- DCI based dual TCI state switch for mDCI scenario

- DCI based dual TCI state switch delay for mDCI is independent provided the following conditions are met:

  - For m-DCI based dual TCI states switch, reuse Re-16 requirements for each TCI state switch delay

    - if gap between (DCI0, PDSCH0), (DCI1, PDSCH1) is not less than timeDurationForQCL

  - if gap between (DCI0, PDSCH1), (DCI1, PDSCH0) is not less than timeDurationForQCL, requirements applicability and UE behaviour is subject to RAN1 confirmation.

- MAC-CE based PDCCH TCI state switch for s-DCI PDCCH repetition

  - For MAC-CE based PDCCH TCI state switch for s-DCI PDCCH repetition [where two MAC-CEs are received in one slot], the legacy delay requirements apply if following conditions are met.

    - Target dual TCI states are in the active TCI state list; or

    - If target dual TCI states are NOT in the active TCI state list and [Tfirst_SSB] is longer than [125]us, where Tfirst_SSB is the shorter one between Tfirst-SSB1 and Tfirst_SSB2.

      - FFS if requirements should be defined for the case.

  - Otherwise, [125] µs additional delay is considered

- MAC CE based PDCCH TCI state switch for m-DCI scenario

  - For MAC-CE based PDCCH TCI state switch when PDCCHs are configured with different CoresetPoolIndex separately for m-DCI scenario, the legacy delay requirements apply for each TCI state switch.

    - Note: new beam pair can be used only after both the two TCI states switch are completed.

      - Simultaneous PDDCH reception for m-DCI scenario should be checked with RAN1.

- Whether to define requirements for RRC based TCI state switch

  - Further check with RAN1 if this is supported.

    - For RRC based PDCCH TCI state switch for m-DCI scenario, legacy delay requirements apply when there is single TCI configured by RRC for a CORESET other than a CORESET with index 0.

- Requirements to be considered

  - For MAC-CE based dual TCI state switch delay, delay requirements are only defined for known TCI states in Rel-18.

- Active TCI state list update Active TCI state list update delay requirement

  - For s-DCI case,

    - The existing requirement for active TCI state list update can be reused with the update to T/F tracking.

  - For m-DCI case,

    - FFS: The existing requirement for active TCI state list update can be reused with the update that it is for each TRP.

- Whether to consider RTD larger than CP in multi-RX WI

  - The requirements defined for multi-Rx are only applicable for receive timing difference (RTD) less than CP in Rel-18 multi-Rx WI.

    - RAN4 understands it is important to support RTD>CP and recommend to consider in future release.

##### 2.4.1.2.8	From RAN4#108bis:

###### 2.4.1.2.8.1	WF R4-2317332

- UE capability of simultaneousRxDataSSB-DiffNumerology

  - It is RAN4 common understanding that there is no scenario of simultaneous reception of mix numerologies for: SSB (TRP1) + data (TRP2), and for SSB (TRP1) + CSI-RS (TRP2) in Rel-18.

  - No new UE capability for mix numerologies is needed in Rel-18

- Updated conditions for TRP specific BFD requirements enhancement for multi-Rx

  - Both CSI-RSs are not in any CSI-RS resource set with repetition ON

  - The two CSI-RS resources in the two sets q ̅_0,0 and q ̅_0,1 for beam failure detection [and both PDSCHs] are overlapped on the same OFDM symbol.

  - [The CSI-RS in set q ̅_0,0 has same QCL source as the active TCI state of one PDSCH, and the CSI-RS in set q ̅_0,1 has same QCL source as the active TCI state of the other PDSCH]

  - Resources of the active TCI states for the two PDCCHs, or two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

  - [FFS how to capture UE is activated with multi-Rx operation]

- Updated conditions/cases for scheduling restriction that can be relaxed for CSI-RS based L1 measurements for multi-Rx

  - The CSI-RS is not in a CSI-RS resource set with repetition ON.

  - The CSI-RS has same QCL source as the active TCI state of one of the PDSCHs and has different QCL-TypeD from the other PDSCH.

  - The CSI-RS and both of the PDSCHs are on the same OFDM symbol(s).

    - FFS: The CSI-RS and only one of the PDSCHs with different QCLed typeD are on the same OFDM symbol(s)

  - Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

  - [FFS how to capture UE is activated with multi-Rx operation]

- Cases whether scheduling restriction can be relaxed for L1 measurements for multi-Rx

  - No mixed numerology case is considered for scheduling restriction for L1 measurements with multi-Rx.

- Cases whether measurement restriction for SSB based L1 measurements can be relaxed for multi-Rx

  - It is RAN4 common understanding that Measurement restriction shall be applied for SSB based L1 measurement (RLM/BFD /L1-RSRP), i.e., no relaxation, regardless of supporting mixed numerology in Rel-18.

  - No mixed numerology case is considered for measurement restriction for L1 measurements (CSI-RS + CSI-RS) for multi-Rx in Rel-18.

- Conditions that measurement restriction for CSI-RS based L1 measurements can be relaxed for multi-Rx

  - Both CSI-RSs are not in any CSI-RS resource set with repetition ON

  - [The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol].

  - One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH

  - Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

  - [FFS how to capture UE is activated with multi-Rx operation]

###### 2.4.1.2.8.2	WF R4-2317427

- UE behaviour when dual TCI states are not supported

  - It is proposed to capture in 38.133 the agreement that no UE behaviour is defined in Rel-18 multi-Rx WI when UE cannot receive dual TCI states simultaneously

- DCI based dual TCI state switch for sDCI scenario

  - No additional delay on top of Rel-16 requirements is introduced for s-DCI based dual TCI states switch

- For sDCI, for dual TCI to single TCI state switch, when the target TCI is one of the source TCI (e.g. [RS1,RS2] to [RS1]),

  - There is no TCI switching delay when UE is configured with GBBR and is NOT configured with non-GBBR

- Other proposals for mDCI

  - In mDCI scenario, TCI switching with one CORESETpoolindex does not cause interruptions on TCI states with another CORESETpoolindex. However, there is no specification impact is there for this proposal

- MAC-CE based PDCCH TCI state switch for s-DCI PDCCH repetition

  - No additional delay is needed

  - For the following case, do not define test.

    - If target dual TCI states are NOT in the active TCI state list;

    - [Tfirst_SSB] arrives with [125]us after MAC CE processing time, where Tfirst_SSB is the shorter one between Tfirst-SSB1 and Tfirst_SSB2

- Active TCI state list update delay requirement for mDCI

  - The existing requirement for active TCI state list update can be reused with the update that it is for each new TCI state being added

##### 2.4.1.2.9	From RAN4#109:

###### 2.4.1.2.9.1	WF R4-2321602

- Measurement period for L1-RSRP configured for GBBR, N = 8

- L1-RSRP non-GBBR measurement enhancement for multi-Rx is applied to L1-SINR non-GBBR measurement.

- For MAC CE based PDCCH TCI state switch for m-DCI scenario, no RRM requirement is defined for simultaneous PDCCH reception with different QCL typeD in Rel-18 multi-Rx WI.

- Requirements for RRC based dual TCI state switch are defined.

- For the requirement defined in the WI excepting the requirement for beam sweeping factor reduction:

  - When the side conditions are not fulfilled, the corresponding multi-Rx requirement is not applicable.

- Power class for new UE capability for simultaneous reception with different QCL typeD for L1 measurements.

  - All power classes expect PC6 for multi-Rx.

  - Further discussion is not precluded if this agreement is not aligned with the RF decision.

- Prerequisite features for new UE capability for simultaneous reception with different QCL typeD for L1 measurements.

  - 16-2c, 23-5-1, [at least one of 16-2a, 16-2b-1, 16-2b-2 and 16-2b-3]

- UE feature list

#### 2.4.1.3	UE Performance

##### 2.4.1.3.1	From RAN4#104-e: WF R4-2214344

- UE RRM performance part work plan

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#106 | Feb. 2023 | Initial discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Initial discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused |  |
| RAN4#106-Bis | Apr. 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused Initial discussion on list of test cases |  |
| RAN4#107 | May 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused | Initial agreement on list of test cases |
| RAN4#108 | Aug. 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused | Work split on the Conclude the list of test cases Work split on the Draft CRs for performance requirements and test cases |
| RAN4#108-Bis | Oct. 2023 | Continue discussion on the remaining issues for performance requirements and test cases Initial discussion on the Draft CRs for performance requirements and test cases |  |
| RAN4#109 | Nov. 2023 | Continue discussion on the remaining issues for performance requirements and test cases Continue discussion on the Draft CRs for performance requirements and test cases |  |
| RAN4#110 | Feb. 2024 | Continue discussion on the remaining issues for performance requirements and test cases | Conclude all the performance requirements and test cases Endorse all the Draft CRs for the performance part of the WI |

##### 2.4.1.3.2	From RAN4#106: WF R4-2302941

- General Aspects:

  - Use the existing agreement from RF in [R4-2220533] for definition of the terms of antenna module and antenna panel.

  - Rel-18 FR2 Multi-RX chain UE WI, Focus on multi-TRP transmission schemes with intra-cell mTRP scenarios only.

  - Whether to introduce a new correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements: For initial simulation purpose, MIMO correlation matrix approach considered as starting point.

  - Whether to introduce requirements for both single-DCI and multi-DCI scenarios: For initial evaluation purpose, below transmission schemes with layer combinations considered

    - Single DCI with SDM scheme: 1+1 and 2+2 as layer combination

    - Multi-DCI with fully overlapping scheme: 1+1 and 2+2(high priority) as layer combination

    - Multi-DCI with non-overlapping scheme: 2+2 as layer combination

    - Receiver assumption: MMSE-IRC

      - Option 1: UE joint processing with 4x4

      - Option 2: UE processing with 2x2 per TRP

  - No PDCCH requirements will be introduced for Rel-18 FR2 Multi-Rx chain UE WI.

  - No PBCH performance requirements will be introduced.

  - No SDR performance requirements will be introduced.

  - Whether to introduce performance requirements for CSI reporting: Introduce PMI reporting requirements for single-DCI transmission scheme.

  - Whether to introduce performance requirements for CA: Prioritize single carrier performance requirements.

  - PN model and PN compensation (Tentative Agreement): For evaluation, consider PN impact for the decision of MCS, based on expected PN degradation of [X] dB; Interested companies can bring performance simulations with and without PN modelling in the next meeting.

    - Consider Example 2 with priority;

    - Consider PN Models for FC = [30, 47] GHz;

- Test scope for PDSCH demodulation requirements

  - Channel model (Tentative agreement): Include TDLA30-75. Other channel models are not precluded.

  - SCS/Bandwidth (Tentative agreement):

    - Encourage interested companies to provide simulation results with 120kHz/100 MHz with priority;

    - Interested companies are welcome to provide simulations with 120kHz/200MHz, to investigate SNR limitations for this BW based on evaluation results;

##### 2.4.1.3.3	From RAN4#106bis-e: WF R4-2306000

- General Aspects:

  - Whether to study cross-talk between TRPs after Rx beamforming is achieved: Consider cross-talk between TRPs captured via the correlation matrix.

- Assumption on UE Antenna module: Don’t explicitly include antenna module placement in demod discussion; can be considered through correlation modelling (if needed).

  - Whether to consider MCS reduction for IFF method: Discuss later once achievable MCS is agreed.

  - Whether to introduce requirements for both single-DCI and multi-DCI scenarios: Keep the current agreement of transmissions schemes and layer combinations for evaluation for companies to provide initial simulation results. Consider both sDCI with SDM and mDCI with fully overlapping resources, prioritize mDCI with fully overlapping.

  - Performance requirement for multi- DCI non-overlapping:  Define at least one set of performance requirements for multi-DCI non-overlapping (if agreed to introduce requirements).

  - Whether to study isolation level between two parallel TRP-to-RX links for 2x2 independent UE processing: No need to consider isolation level separately. The isolation level between two parallel TRP-to-Rx links is inherently considered as part of the correlation model discussed under Issue 1-1-1

  - PN model and PN compensation:

    - Leave PN impact and modelling to companies’ implementation, the impact can be considered into results submitted by companies and the impact on test parameters i.e., MCS selection subject to companies’ results.

- Assumption on FFT window: Consider single FFT window as baseline assumption.

- PDSCH Aspects:

- AoA assumption

      - Do not define demodulation requirements for all AoA offsets considered in RF requirements.

      - Define demodulation requirements for 1 or 2 correlation types based on feasibility and performance evaluation.

      - Power imbalance between TRPs: Do not introduce power imbalance between the TRPs for FR2 multi-Rx demod requirements.

    - SCS/BW: Define 120kHz/100MHz for SCS/Bandwidth as priority.

  - For initial evaluation purpose, below transmission schemes with layer combinations considered

      - Single DCI with SDM scheme: 1+1 and 2+2 as layer combination

      - Multi-DCI with fully overlapping scheme: 2+2 as layer combination

      - Multi-DCI with non-overlapping scheme: 2+2 as layer combination

    - MCS: Consider the following MCS for initial evaluation-

      - Use MCS 13 or lower for defining rank 2+2, higher inter-TRP interference cases with SDM/fully overlapping scheduling.

      - Use MCS 17 for rank 1+1 cases or lower inter-TRP interference cases with SDM or non-overlapping case.

  - PTRS Pattern: Consider K=2, L=1 as the frequency and time density, respectively.

  - TDD slot pattern: Define the FR2 multi-Rx UE demodulation requirements for PDSCH using FR2.120-1 TDD UL-DL pattern, given by DDDSU and S: 10D+2G+2U.

  - Performance metric: Define the FR2 multi-Rx UE demodulation requirements for PDSCH with 70% of the peak throughput metric only.

  - Whether to consider additional margin for 47GHz carrier frequency: Discuss later when requirements are defined.

- CSI Aspects:

      - Whether to evaluate feasibility of introducing PMI reporting requirement for sDCI SDM scheme: Evaluate feasibility of introducing PMI reporting requirement for sDCI SDM scheme.

      - Whether to introduce PMI reporting requirement for mDCI scheme: Don’t introduce PMI reporting requirement for mDCI scheme.

      - MIMO layer: 1 MIMO layer per TRP

    - MCS: Consider MCS13 for initial evaluation.

    - Whether to assume time and frequency offsets: Don’t consider time and frequency offsets for CSI reporting requirements.

##### 2.4.1.3.4	From RAN4#107: WF R4-2309824

- General Aspects:

  - Test method for demodulation requirements for FR2 multi-Rx: RAN4 agreed to adopt virtual cable setup to test the FR2 multi-Rx demodulation requirements.

- On new correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements:

- Consider the baseline correlation model assuming cross-talk between the TRPs and UE Rx antenna as below

R16×16=Eh16×1h16×1H=11+ρ∙diag1  ρ  ρ  1⊗RMIMO

=11+ρ∙RMIMOρRMIMOρRMIMORMIMO,

- where h16×1=vec(H4x4) is the re-arranged channel values and

    - ⊗ denotes the Kronecker product, diag denotes the diagonal matrix.

    - RMIMO is the 4×4 correlation matrix for each link between TRPs to UE panels generated with parameters α (Tx correlation coefficient), β (Rx correlation coefficient), and γ (cross-polarization correlation coefficient) as stated in 38.101-4 B2.3.2

    - ρ is the cross-talk power

  - Candidate cross-talk power values for initial simulation purpose:

        - -15, -12, -9, -6 dB

        - Other options are not precluded.

- Candidate cross-polarization correlation coefficient values for initial simulation purpose:

      - γ = 0.125

      - Other options are not precluded.

  - Number of receive antennas per Rx chain: 2 Rx per chain.

  - Duplex mode for demodulation requirements: TDD mode only.

  - Receiver assumption:

    - Keep both options (independent and joint processing) open at this point of evaluation.

    - Companies are encouraged to bring results evaluating both independent and joint processing in August meeting.

- PDSCH Aspects:

      - Channel model

        - At least consider TDLA30-75 for 100 MHz/120 kHz

        - FFS TDLA30-300 for 100 MHz/120 kHz.

      - PTRS Port for mDCI schemes: One PTRS port per TRP for mDCI schemes

      - Time offset between TRPs:

  - Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us}.

  - Further discuss which time offset to be used with certain configurations.

- CSI Aspects:

    - Number of CSI-RS ports: 2-port per TRP

    - Whether to assume LI in report quantity for multi-Rx FR2: Assume no LI in report quantity for multi-Rx FR2

##### 2.4.1.3.5	From RAN4#108: WF R4-2313977

- General Aspects:

  - Further evaluate the performance for 2+2 with fully overlapping case with joint processing and sperate processing

  - Demod tests will adopt OTA agreements for demod tests..

  - For now, don’t consider 0 dB and -3dB cross-talk levels as the SNR requirements could be quite high

  - Select α=β=0 in the correlation matrix for all cases

  - Consider Tx EVM at 6% since we are considering up to 64QAM modulation

  - adopt NT FR2 OTA enhancements when defining demodulation requirements

  - Set cross-polarization coefficient = 0.0625

  - consider ρ to be cross-talk power ratio

- PDSCH demod requirements

  - Further evaluate both cases: one PTRS port across TRPs and one PTRS port for each TRP

  - PDSCH shall rate matching around PTRS from same TRP only

  - Time frequency offsets between TRPs:

    - sDCI SDM: (-0.0625us, 600Hz) and (0.25us, 0Hz)

    - mDCI non-overlapping: (-0.0625us, 600Hz)

    - mDCI fully-overlapping: (-0.0625us, 600Hz) and (0.25us, 0Hz)

  - Evaluate PDSCH demodulation performance with the following configurations.

    - Multi-DCI with fully-overlapping PDSCH, 1 layer per TRP

    - Multi-DCI with fully-overlapping PDSCH, 2 layers per TRP

    - Multi-DCI with non-overlapping PDSCH, 2 layers per TRP

    - Single-DCI SDM with 1+1

    - Single-DCI SDM with 2+2

  - Simulation params for initial evaluation agreed (see WF for details)

  - consider a scaling factor for transmitted signal in mDCI fully-overlapping case. SNR corresponds to SNR of TRxP #1 and TRxP #2 with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP

  - Select {MCS17, ρ = -6dB, rank 1+1} and/or {MCS13, ρ = -12dB, rank 2+2} for sDCI SDM cases.

  - Select {MCS17, ρ = -6dB, rank 1+1} and/or {MCS13, ρ = -12dB, rank 2+2} for mDCI fully-overlapping cases

- CSI reporting requirements

  - Use TDLA30-35 propagation channel for PMI reporting requirement for sDCI SDM scheme.

##### 2.4.1.2.8	From RAN4#108bis: WF R4-2317332

- AoA selection in RRM test cases

  - Companies to check the progress in FR2 OTA WI, and whether to define test cases for dual TCI state from dual TCI to dual TCI (e.g. [RS1, RS2] to [RS3, RS4]) is to be decided in RRM session.

  - Note: The feasibility of the test with 4 active probes simultaneously received by the same UE is pending on the discussion in FR2 OTA WI.

##### 2.4.1.2.9	From RAN4#109:

###### 2.4.1.2.9.1	WF R4-2321140

- Receiver assumption for mDCI fully and non-overlapping case:

  - Consider separate processing for mDCI in Rel-18.

  - RAN4 may consider joint processing in future releases.

- Receiver assumption for sDCI SDM case.

  - Consider separate processing for sDCI in Rel-18.

  - RAN4 may consider joint processing in future releases.

- MCS/layer/cross-talk for sDCI SDM case.

  - 1+1, MCS17, ρ=-12dB

  - Do not define requirements with 2+2

- MCS/cross-talk for mDCI fully overlapping 1+1 case:

  - 1+1, MCS17, ρ=-12dB

- MCS/cross-talk for mDCI fully overlapping 2+2 case:

  - Do not define requirements with 2+2

- MCS/layer/cross-talk for mDCI non overlapping case:

  - 2+2, MCS13, ρ=-12dB

- Assumption on correlation model:

  - Use agreed correlation model from RAN4#107.

- Whether to adopt NR FR2 OTA enhancements when defining demodulation requirements:

  - Defer this discussion to FR2 OTA.

- UE feature for joint processing (if introduced):

  - Not applicable since requirement with joint processing will not be introduced.

- TR update:

  - RAN4 will expand the scope of TR 38.751 to include demod evaluation.

    - Content and work split to be discussed over email (post meeting RAN4#109)

- PT-RS EPRE Ratio:

- Define EPRE-Ratio state ‘0’ for FR2 multi-panel RX simulation assumptions.

- Number of DMRS CDM groups without data:

- Confirm that the number of CDM groups without data is 2 for DMRS configuration for mDCI.

- Time/frequency offsets for sDCI SDM:

  - 600 Hz, -0.0625us

- Time/frequency offsets for mDCI fully overlapping:

  - 600 Hz, -0.0625us

- Time/frequency offsets for mDCI non overlapping:

  - 0.25us, 0Hz

- General PDSCH/PDCCH configuration for mDCI:

    - Configure PDCCH from each TRP non-overlapping in time for mDCI transmission mode.

    - PDCCH from TRP1 is transmitted on symbol 0 and PDCCH from TRP2 is transmitted on symbol 1 of the slot.

    - PDSCH transmission starts from symbol 2 (No of PDSCH symbols is 12)

- General PDSCH/PDCCH configuration for sDCI SDM:

  - For sDCI SDM transmission scheme, PDCCH is transmitted on symbol 0 and PDSCH is

- Receiver assumption for PMI Reporting:

  - Consider separate processing for PMI reporting with sDCI SDM.

- MCS:

  - MCS13 with 1+1.

- Time/frequency offsets for PMI reporting:

  - Set FO=0Hz and TO=0us for PMI reporting test

- Performance Metric:

  - X= [90%]

- Throughput ratio (γ) value:

  - Consider γ = [1.3] in the next meeting, other values are not precluded. Consider some margin (to be discussed).

- Reference Channel <way forward>

  - Option 1: Use reference channels as presented in Table 2.3-1 for FR2 multi-Rx PMI reporting requirements.

  - Option 2: Reuse the reference channel from TS38.101-4, section 6.3.2.1.7 with adaptation to 4 ports as starting point. Configure the codebook for 1 panel.

###### 2.4.1.2.9.2	WF R4-2321602

- Measurement period for L1-RSRP configured for GBBR, N = 8

- L1-RSRP non-GBBR measurement enhancement	for multi-Rx is applied to L1-SINR non-GBBR measurement.

- For MAC CE based PDCCH TCI state switch for m-DCI scenario, no RRM requirement is defined for simultaneous PDCCH reception with different QCL typeD in Rel-18 multi-Rx WI.

- Requirements for RRC based dual TCI state switch are defined.

- For the requirement defined in the WI excepting the requirement for beam sweeping factor reduction:

  - When the side conditions are not fulfilled, the corresponding multi-Rx requirement is not applicable.

- Power class for new UE capability for simultaneous reception with different QCL typeD for L1 measurements.

  - All power classes expect PC6 for multi-Rx.

  - Further discussion is not precluded if this agreement is not aligned with the RF decision.

- Prerequisite features for new UE capability for simultaneous reception with different QCL typeD for L1 measurements.

  - 16-2c, 23-5-1, [at least one of 16-2a, 16-2b-1, 16-2b-2 and 16-2b-3]

- UE feature list (see WF)

- Accuracy requirements for multi-Rx in Rel-18

  - Option 1: The legacy accuracy requirements for L1-RSRP measurement apply for L1-RSRP measurements with group-based beam reporting.

  - Option 2: The legacy accuracy requirements in section 10.1.20 of TS 38.133 apply for L1-RSRP measurements under multi-rx operation, with a clarification that multi-rx chain L1-RSRP accuracy requirements apply for FR2-1.

  - Option 3:

    - The legacy accuracy requirements in section 10.1.20 of TS 38.133 apply for L1-RSRP measurements under multi-rx operation, with a clarification that multi-rx chain L1-RSRP accuracy requirements apply for FR2-1.

    - No new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation.

#### 2.4.2	Remaining Open issues

#### 2.4.2.1	Objective of Core part WI

(no open issues)

#### 2.4.2.1	Objective of Performance part WI

- Introduce necessary requirement(s) for enhanced FR2 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier

  - RRM Performance requirements and test cases corresponding to the new core requirements

  - RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused

  - UE demodulation requirements:

    - Formal CR and TR update

NOTE:

− For the dual-TCI demodulation requirement case, focus on UEs supporting “simultaneousReceptionDiffTypeD-r16” UE capability, and for the 2- or 4-layer downlink MIMO reception, focus on UEs supporting the basic mTRP CSI reporting capability (FG 23-7-1/1b of NR FeMIMO).

## 2.5	RAN5

Void

## 2.6	RAN6

Void

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A.

## 4.	References

## 4.1	Discussion Tdocs from RAN4#104-e:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2211642 | Discussion on Rel-18 multi Rx pannel RRM | CATT |
| R4-2211768 | Views on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2211814 | Proposal on spherical coverage requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2211815 | Enhanced spherical coverage requirements for multi-rx chains UEs | Nokia, Nokia Shanghai Bell |
| R4-2211882 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2211883 | RRM requirements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2211940 | Discussion on RRM requirements for simultaneous DL reception from different directions | CMCC |
| R4-2211978 | On multi-RX chain | Xiaomi |
| R4-2211993 | Requirements for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2212062 | Initial discussion on RRM requirements for simultaneous DL reception from different directions | OPPO |
| R4-2212180 | Impacts on RRM to support FR2 multi-Rx chain DL reception from multi-TRP | Qualcomm Incorporated |
| R4-2212185 | Discussion on UE RF requirements for simultaneous DL reception with up to 4 layer MIMO | LG Electronics |
| R4-2212219 | Discussion on RRM for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2212332 | System assumptions for multi-Rx chain DL reception | Qualcomm Incorporated |
| R4-2212333 | Work plan for FR2 UE multi-Rx chain DL reception | Qualcomm Incorporated, vivo |
| R4-2212466 | Discussion on RRM requirements for simultaneous DL reception from different directions | Samsung |
| R4-2212512 | Discussion on simultaneous DL reception from different directions | MediaTek Inc. |
| R4-2212593 | Discussion on simultaneous multi-Rx chain DL reception | Xiaomi |
| R4-2212688 | Discussion on RRM requirements for FR2 multi-Rx chain reception | Nokia, Nokia Shanghai Bell |
| R4-2212806 | General analysis on Multi-Rx requirements | vivo |
| R4-2213053 | Work plan for RRM requirement for NR FR2 multi-Rx chain DL reception | vivo, Qualcomm |
| R4-2213054 | Discussion on RRM requirements for multi-chain DL reception | vivo |
| R4-2213289 | Discussion on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2213495 | Discussion on RRM impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2213568 | Views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2213737 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2213872 | Discussion on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2213957 | Discussion on general RRM issues for simultaneous DL reception from different directions | Ericsson |
| R4-2214245 | Email Discussion Summary for [104-e][134] FR2_multiRx_UERF | Moderator (Qualcomm) |
| R4-2214282 | Email Discussion Summary for [104-e][233] FR2_multiRx_RRM | Moderator (Vivo) |

## 4.2	Discussion Tdocs from RAN4#104-Bis-e:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2215581 | Proposal on spherical coverage requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2215621 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215781 | Discussion on UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2216127 | Discussion on RF requirement for multi-Rx DL reception | vivo |
| R4-2216353 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2216786 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2216253 | Views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2216589 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216875 | On Defining Coverage Requirements for Multi-Rx Chain Downlink Reception | Lenovo |
| R4-2215579 | Proposals on consideration of UE architecture for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2215620 | UE implementation assumptions for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215701 | System assumption and UE assumption for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2215778 | Discussion for FR2 multi-Rx chain DL reception | Murata Manufacturing Co Ltd. |
| R4-2216125 | Discussion on UE implementation assumption of multi-Rx DL reception | vivo |
| R4-2216352 | Discussion on UE architecture to support simultaneous DL reception | Xiaomi |
| R4-2216445 | Considerations on the UE assumptions for FR2 multi-Rx chain DL reception | OPPO |
| R4-2217784 | Email discussion summary for [104-bis-e][132] FR2_multiRx_UERF_part1 | Moderator (Qualcomm) |
| R4-2215360 | Discussion on FR2 multi Rx chain RRM impacts and general aspects | Intel Corporation |
| R4-2215361 | Discussion on RRM impacts for L1 measurement based on FR2 multi Rx chain | Intel Corporation |
| R4-2215362 | Discussion on RRM impacts for TCI state switching based on FR2 multi Rx chain | Intel Corporation |
| R4-2215462 | on the multi-RX chain general aspects | Xiaomi |
| R4-2215463 | on the multi-RX chain L1 measurement | Xiaomi |
| R4-2215464 | on the multi-RX chain L3 measurement | Xiaomi |
| R4-2215465 | on the multi-RX chain TCI state switching | Xiaomi |
| R4-2215622 | General aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215623 | On L3 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215624 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215710 | Discussions on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2215720 | Discussion on general aspects for FR2 multi-Rx chain DL reception | CMCC |
| R4-2215721 | Discussion on L1 measurement for FR2 multi-Rx chain DL reception | CMCC |
| R4-2215722 | Discussion on L3 measurement for FR2 multi-Rx chain DL reception | CMCC |
| R4-2215759 | Discussion on simultaneous DL reception from different directions for general issues | MediaTek Inc. |
| R4-2215760 | Discussion on simultaneous DL reception from different directions for L3 measurement | MediaTek Inc. |
| R4-2215761 | Discussion on simultaneous DL reception from different directions for L1 measurement | MediaTek Inc. |
| R4-2215762 | Discussion on simultaneous DL reception from different directions for TCI state switching | MediaTek Inc. |
| R4-2215803 | Discussion on general aspects of RRM for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2215804 | Discussion on L3 measurement related RRM for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2215805 | Discussion on L1 measurement related to RRM for simultaneous DL reception from different | LG Electronics Inc. |
| R4-2215806 | Discussion on TCI state switching for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2215812 | Discussion on general requirements for FR2_multiRX_DL | OPPO |
| R4-2215813 | Discussion on L3 requirements for FR2_multiRX_DL | OPPO |
| R4-2215814 | Discussion on L1 requirements for FR2_multiRX_DL | OPPO |
| R4-2215815 | Discussion on TCI state switching for FR2_multiRX_DL | OPPO |
| R4-2215867 | Further analysis on RRM impacts and general aspects | vivo |
| R4-2215868 | On L3 measurement for multi-Rx chain | vivo |
| R4-2215869 | On L1 measurement for multi-Rx chain | vivo |
| R4-2215870 | On TCI state switching for multi-Rx chain | vivo |
| R4-2216277 | Discussion RRM requirements of TCI state switching for multi-Rx | Huawei, HiSilicon |
| R4-2216285 | Discussion on RRM general impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216286 | Discussion on L3 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216287 | Discussion on L1 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216474 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216475 | Discussion on L1 part RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216476 | Discussion on L3 part RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216477 | Discussion on TCI state related RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216578 | General considerations on RRM requirements for multi-RX RRM | Nokia, Nokia Shanghai Bell |
| R4-2216579 | Discussion on RRM L3 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2216580 | Discussion on RRM L1 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2216581 | Discussion on RRM TCI State Switching for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2216713 | Further Analysis of RRM requirement impacts for simultaneous DL reception from different directions | Samsung |
| R4-2216824 | Discussion on scenarios for simultaneous DL reception from different directions | Ericsson |
| R4-2216825 | Discussion on L3 measurements and procedures | Ericsson |
| R4-2216826 | Discussion on L1 measurements and procedures | Ericsson |
| R4-2216827 | Discussion on active TCI state requirements | Ericsson |
| R4-2216866 | Impacts on RRM to support FR2 multi-Rx chain based 4 layer DL reception from multi-TRP | Qualcomm Incorporated |
| R4-2217246 | WF on RRM impacts and general aspects for multi-Rx | Vivo |
| R4-2217248 | WF on TCI state switching for multi-RX chain DL reception | Ericsson |
| R4-2217587 | WF on L1 measurements, beam sweeping factors and simultaneous reception | Qualcomm |

## 4.3	Discussion Tdocs from RAN4#105:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2218042 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2218043 | On specifying requirements for mDCI UEs | Qualcomm Incorporated |
| R4-2218165 | System parameter and UE assumption for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218166 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218329 | Proposal on UE RF System parameter assumption, UE architecture and conditions of UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2218330 | Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2218528 | Discussion on UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2218556 | System and UE assumptions for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2218557 | RF requirements for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2218755 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2218873 | Further consideration on system parameter of multi-Rx DL reception | vivo |
| R4-2218874 | Discussion on RF requirement for multi-Rx DL reception | vivo |
| R4-2219124 | Discussion on UE RF system assumption supporting simultaneous DL reception | Xiaomi |
| R4-2219125 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2219192 | Discussion on System parameter and UE architecture of UE RF requirements | ZTE Corporation |
| R4-2219193 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2219497 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219600 | Consideration on small AoA angular separation | OPPO |
| R4-2219852 | System Parameter Assumptions for Multi AoA Rx Testing | Keysight Technologies UK Ltd |
| R4-2219868 | Discussion on RF test methods for UEs with multi-Rx | ROHDE & SCHWARZ |
| R4-2220017 | Complexity Reduction for Multi-Rx Multi-AOA Coverage Evaluation | Lenovo |
| R4-2220529 | Discussion on System parameter and UE architecture of UE RF requirements | ZTE Corporation |
| R4-2220530 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2220111 | Topic summary for [105][132] FR2_multiRx_UERF_part1 | Moderator (Qualcomm) |
| R4-2220112 | Topic summary for [105][132] FR2_multiRx_UERF_part2 | Moderator (Apple) |
| R4-2218167 | General aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218168 | On L3 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218169 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218170 | TCI state switching for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218335 | Discussion on FR2 multi Rx chain scope and scenarios | Intel Corporation |
| R4-2218336 | Discussion on FR2 multi Rx chain general aspects | Intel Corporation |
| R4-2218337 | Discussion on RRM impacts for L1 measurement based on FR2 multi Rx chain | Intel Corporation |
| R4-2218393 | Discussion on scope and scenarios for FR2 multi-Rx chain DL reception | CMCC |
| R4-2218394 | Discussion on L3 measurement for FR2 multi-Rx chain DL reception | CMCC |
| R4-2218408 | Analysis of scope and scenarios | Qualcomm Incorporated |
| R4-2218409 | Analysis of general aspects | Qualcomm Incorporated |
| R4-2218410 | L3 measurement specifically related | Qualcomm Incorporated |
| R4-2218586 | on the multi-RX chain scenarios | Xiaomi |
| R4-2218587 | on the multi-RX chain general aspects | Xiaomi |
| R4-2218588 | on the multi-RX chain L3 measurement | Xiaomi |
| R4-2218589 | on the multi-RX chain L1 measurement | Xiaomi |
| R4-2218590 | on the multi-RX chain TCI state switching | Xiaomi |
| R4-2218596 | Discussion on scope and scenario of RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218597 | Discussion on general aspects of RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218598 | Discussion on L3 measurement related for RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218599 | Discussion on L1 measurement related for RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218738 | Discussion on simultaneous DL reception from different directions for scope and scenarios | MediaTek Inc. |
| R4-2218739 | Discussion on simultaneous DL reception from different directions for general aspects | MediaTek Inc. |
| R4-2218740 | Discussion on simultaneous DL reception from different directions for L3 measurement | MediaTek Inc. |
| R4-2218741 | Discussion on simultaneous DL reception from different directions for L1 measurement | MediaTek Inc. |
| R4-2218742 | Discussion on simultaneous DL reception from different directions for TCI state switching | MediaTek Inc. |
| R4-2218775 | On scope and scenarios for FR2 multi-Rx | vivo |
| R4-2218776 | On general aspects for FR2 multi-Rx | vivo |
| R4-2218777 | On L3 measurement for multi-Rx | vivo |
| R4-2218778 | On L1 measurement for multi-Rx | vivo |
| R4-2218779 | On TCI state switching for multi-Rx | vivo |
| R4-2218977 | On scope and scenarios for FR2_multiRX_DL | OPPO |
| R4-2218978 | On general requirements for FR2_multiRX_DL | OPPO |
| R4-2218979 | Discussion on L3 requirements for FR2_multiRX_DL | OPPO |
| R4-2218980 | Discussion on L1 requirements for FR2_multiRX_DL | OPPO |
| R4-2218981 | Discussion on TCI state switching for FR2_multiRX_DL | OPPO |
| R4-2219008 | Scope and scenarios for FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219009 | General aspects discussions for FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219010 | Discussions for L3 measurements on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219011 | Discussions for L1 measurements on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219242 | Discussion RRM requirements of TCI state switching for multi-Rx | Huawei, HiSilicon |
| R4-2219250 | Discussion on scope and scenarios for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219251 | Discussion on RRM general aspects for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219252 | Discussion on L3 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219253 | Discussion on L1 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219290 | Discussion on scope and general scenarios for FR2 multi-RX | Samsung |
| R4-2219291 | Analysis on general aspects for FR2 multi-RX | Samsung |
| R4-2219434 | Discussion on scope and scenarios for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2219435 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2219436 | Discussion on L3 measurement related aspects for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2219739 | Discussion on scope and scenarios for the requirements for multi-RX RRM | Nokia, Nokia Shanghai Bell |
| R4-2219740 | Discussion on general aspects for multi Rx DL | Nokia, Nokia Shanghai Bell |
| R4-2219741 | Discussion on RRM L3 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2219742 | Discussion on RRM L1 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2219743 | Discussion on TCI state switching requirements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2219946 | Discussion on scope and UE capabilities | Ericsson |
| R4-2219947 | Discussion on general aspects | Ericsson |
| R4-2219948 | Discussion on L3 measurements and procedures | Ericsson |
| R4-2219949 | Discussion on L1 measurements and procedures | Ericsson |
| R4-2219950 | Discussion on active TCI state requirements | Ericsson |
| R4-2220370 | Ad-hoc meeting minutes: Rel-18 NR FR2 multi-Rx chain DL reception WI | Qualcomm |
| R4-2220402 | WF on NR FR2 multi-Rx chain DL reception – Part 3 | Ericsson |
| R4-2220742 | WF on NR FR2 multi-Rx chain DL reception – Part 1 | Vivo |
| R4-2220743 | WF on NR FR2 multi-Rx chain DL reception – Part 2 | Qualcomm |

## 4.4	Discussion Tdocs from RAN4#106:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2300101 | Overall procedure to enable dual TCI based PDSCH reception | Qualcomm Incorporated |
| R4-2300102 | L1 measurement requirements for support of simultaneous receptions from two TRPs | Qualcomm Incorporated |
| R4-2300103 | RLM and BM requirements for sDCI | Qualcomm Incorporated |
| R4-2300104 | SSB based scheduling and measurement restrictions | Qualcomm Incorporated |
| R4-2300105 | TCI and UE beam switching delay requirements | Qualcomm Incorporated |
| R4-2300106 | Maximum reception timing difference for support of simultaneous receptions from two TRPs | Qualcomm Incorporated |
| R4-2300146 | Discussion for FR2 multi-Rx FOM | Murata Manufacturing Co Ltd. |
| R4-2300195 | Discussion on System parameter assumption, UE architecture and conditions of UE RF requirements | Nokia, Nokia Shanghai Bell |
| R4-2300196 | Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2300255 | Discussion on Demodulation Requirements for FR2 with multi-RX chain reception | Apple |
| R4-2300267 | System parameter and UE assumption for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300268 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300269 | On general aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300270 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300271 | On RLM and BFD/CBD for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300272 | On TCI state switching for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300273 | On Receive timing difference between different directions for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300285 | On scheduling/measurement restrictions for multiple Rx chains | Apple |
| R4-2300453 | Discussion on general aspects for FR2 multi-RX DL reception | Samsung |
| R4-2300454 | Discussion on L1-RSRP measurement delay for FR2 multi-RX | Samsung |
| R4-2300455 | Discussion on receive timing difference between different directions for FR2 multi-RX | Samsung |
| R4-2300461 | Discussion on FR2 multi Rx chain general aspects | Intel Corporation |
| R4-2300462 | Discussion on RRM impacts for L1 measurement based on FR2 multi Rx chain | Intel Corporation |
| R4-2300463 | Discussion on RRM impacts for scheduling/measurement restriction on FR2 multi Rx chain | Intel Corporation |
| R4-2300464 | Discussion on TCI state switching requirement based on FR2 multi Rx chain | Intel Corporation |
| R4-2300465 | Discussion on FR2 multi Rx chain timing offset | Intel Corporation |
| R4-2300522 | General Aspects on RRM requirements for multi Rx | Nokia, Nokia Shanghai Bell |
| R4-2300523 | L1-RSRP Measurement Delays for multi Rx | Nokia, Nokia Shanghai Bell |
| R4-2300524 | Discussion RLM and BFD/CBD requirements for FR2 Multi Rx chain reception | Nokia, Nokia Shanghai Bell |
| R4-2300525 | Discussion on Scheduling Restrictions for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2300526 | Discussion on TCI state switching requirements for FR2 Multi Rx chain reception | Nokia, Nokia Shanghai Bell |
| R4-2300527 | Discussion on Multi-Rx Receive Time Difference (MRTD) | Nokia, Nokia Shanghai Bell |
| R4-2300701 | On MultiRx Demodulation performance and CSI requirements | Nokia, Nokia Shanghai Bell |
| R4-2300709 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2300832 | discussion on demodulation performance and CSI requirements for NR FR2 multi-Rx chain DL reception | Samsung |
| R4-2300910 | on the L1-RSRP measurement delay | Xiaomi |
| R4-2300911 | on the RLM and BFD CBD requirement. | Xiaomi |
| R4-2300912 | on the scheduling measurement restrictions | Xiaomi |
| R4-2300913 | on the TCI state switching delay with dual TCI | Xiaomi |
| R4-2300914 | on the RTD between different directions | Xiaomi |
| R4-2300940 | Discussion on general aspects for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2300941 | Discussion on L1-RSRP measurement for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2300942 | Discussion on scheduling restriction for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2300943 | Discussion on RTD for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2300949 | Discussion on UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2300970 | General aspects discussions for FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2300975 | Views on Performance Requirements for FR2 Multi-Rx DL Tests | Qualcomm Incorporated |
| R4-2300987 | Discussion on FR2 Multi-RX DL UE RF requirements | Samsung |
| R4-2301040 | Discussion on general aspects for multi-Rx UE | MediaTek inc. |
| R4-2301041 | Discussion on L1 measurement for multi-Rx UE | MediaTek inc. |
| R4-2301042 | Discussion on RLM/BFD/CBD requirements for multi-Rx UE | MediaTek inc. |
| R4-2301043 | Discussion on scheduling and measurement restriction for multi-Rx UE | MediaTek inc. |
| R4-2301044 | Discussion on dual TCI state switch for multi-Rx UE | MediaTek inc. |
| R4-2301045 | Discussion on receive timing difference for multi-Rx UE | MediaTek inc. |
| R4-2301233 | Discussion on System parameter and UE architecture of UE RF requirements | ZTE Corporation |
| R4-2301234 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2301326 | On general aspects for FR2 multi-Rx | vivo |
| R4-2301327 | On L1-RSRP measurement delay for multi-Rx | vivo |
| R4-2301328 | On RLM and BFD/CBD requiremets for FR2 multi-Rx | vivo |
| R4-2301329 | On scheduling and measurement restrictions for multi-Rx | vivo |
| R4-2301330 | On TCI state switching delay for multi-Rx | vivo |
| R4-2301331 | On RTD between different directions for multi-Rx | vivo |
| R4-2301363 | Views on NR FR2 multi-Rx chain DL reception requirements | NTT DOCOMO, INC. |
| R4-2301409 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2301410 | Discussion on L1-RSRP measurement requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2301411 | Discussion on Receive timing difference between different directions | ZTE Corporation |
| R4-2301412 | Discussion on scheduling restriction and measurement restriction for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2301461 | On AoA separation range and value for RF requirement | OPPO |
| R4-2301572 | Evaluation on UE requirement of multiRx DL reception | vivo |
| R4-2301573 | Discussion on AoA separation for multiRx DL reception | vivo |
| R4-2301622 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2301650 | On general aspects for FR2_multiRX_DL | OPPO |
| R4-2301651 | On L1 measurement for FR2_multiRX_DL | OPPO |
| R4-2301652 | On measurement restriction for FR2_multiRX_DL | OPPO |
| R4-2301653 | On TCI state switching for FR2 multi-Rx DL | OPPO |
| R4-2301759 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2301812 | Discussion on TCI state switching delay with dual TCI for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2301813 | Discussion on receiving timing difference between different directions for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2301895 | UE demodulation and CSI reporting requirements for FR2 DL multi-Rx chain | Ericsson |
| R4-2301899 | On timing requirements regarding MRTD | Ericsson |
| R4-2302005 | Discussion on RRM general impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302006 | Discussion on L1-RSRP measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302007 | Discussion on RLM/BFD/CBD measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302008 | Discussion on scheduling/measurement restrictions for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302155 | Discussion on TCI state switching for FR2 multi-RX DL reception | Samsung |
| R4-2302201 | Discussion on UE demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302250 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2302522 | System Parameter Assumptions for Multi-AoA Rx Testing | Keysight Technologies UK Ltd |
| R4-2302650 | Discussion on general aspects | Ericsson |
| R4-2302651 | Discussion on L1 measurements | Ericsson |
| R4-2302652 | Discussion on RLM and beam management | Ericsson |
| R4-2302653 | Discussion on Scheduling and measurement restrictions | Ericsson |
| R4-2302654 | Discussion on dual active TCI state switching requirements | Ericsson |
| R4-2302767 | Topic summary for [106][209] FR2_multiRx_part1 | Moderator (Vivo) |
| R4-2302768 | Topic summary for [106][210] FR2_multiRx_part2 | Moderator (Qualcomm) |
| R4-2302824 | Topic summary for [106][131] FR2_multiRx_UERF_part1 | Moderator (Qualcomm) |
| R4-2302825 | Topic summary for [106][132] FR2_multiRx_UERF_part2 | Moderator (Apple) |
| R4-2303773 | Topic summary for [106][328] NR_FR2_multiRX_DL_Demod | Moderator - Qualcomm |
| R4-2300146 | Discussion for FR2 multi-Rx FOM | Murata Manufacturing Co Ltd. |
| R4-2300195 | Discussion on System parameter assumption, UE architecture and conditions of UE RF requirements | Nokia, Nokia Shanghai Bell |
| R4-2300196 | Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2300267 | System parameter and UE assumption for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300268 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300709 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2300949 | Discussion on UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2300987 | Discussion on FR2 Multi-RX DL UE RF requirements | Samsung |
| R4-2301233 | Discussion on System parameter and UE architecture of UE RF requirements | ZTE Corporation |
| R4-2301234 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2301461 | On AoA separation range and value for RF requirement | OPPO |
| R4-2301572 | Evaluation on UE requirement of multiRx DL reception | vivo |
| R4-2301573 | Discussion on AoA separation for multiRx DL reception | vivo |
| R4-2301622 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2301759 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302250 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2302522 | System Parameter Assumptions for Multi-AoA Rx Testing | Keysight Technologies UK Ltd |
| R4-2302824 | Topic summary for [106][131] FR2_multiRx_part1 | Moderator (Qualcomm) |
| R4-2302825 | Topic summary for [106][132] FR2_multiRx_part2 | Moderator (Apple) |
| R4-2303708 | WF on NR FR2 multi-Rx chain DL reception | Samsung, Apple |
| R4-2300255 | Discussion on Demodulation Requirements for FR2 with multi-RX chain reception | Apple |
| R4-2300701 | On MultiRx Demodulation performance and CSI requirements | Nokia, Nokia Shanghai Bell |
| R4-2300832 | discussion on demodulation performance and CSI requirements for NR FR2 multi-Rx chain DL reception | Samsung |
| R4-2300975 | Views on Performance Requirements for FR2 Multi-Rx DL Tests | Qualcomm Incorporated |
| R4-2301363 | Views on NR FR2 multi-Rx chain DL reception requirements | NTT DOCOMO, INC. |
| R4-2301895 | UE demodulation and CSI reporting requirements for FR2 DL multi-Rx chain | Ericsson |
| R4-2302201 | Discussion on UE demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2303773 | Topic summary for [106][328] NR_FR2_multiRX_DL_Demod | Moderator - Qualcomm |
| R4-2302941 | WF for UE demodulation and CSI requirements of Rel-18 NR FR2 multi-Rx chain DL reception | Moderator - Qualcomm |

## 4.5	Discussion Tdocs from RAN4#106-Bis-e:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2304120 | Discussion on System parameter assumption, UE architecture and conditions of UE RF requirements | Nokia, Nokia Shanghai Bell |
| R4-2304121 | Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2304129 | System parameter and UE assumption for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2304130 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2304163 | Discussion on simulation assumption for FR2-1 multi-Rx | Murata Manufacturing Co Ltd. |
| R4-2304603 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2304608 | Discussion on UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2304797 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2304824 | FR2 Multi-RX 2AoA spherical coverage simulation and requirements | Samsung |
| R4-2305071 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2305097 | Discussion on system parameters of multi-Rx DL reception | vivo |
| R4-2305098 | Further evaluation on multi-Rx DL reception | vivo |
| R4-2305599 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2305611 | On AoA separation | OPPO |
| R4-2305616 | On the RF requirement of multi DL reception | OPPO |
| R4-2305750 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2305788 | On UE RF Requirements for Multi-AoA Rx Testing | Keysight Technologies UK Ltd |
| R4-2304049 | On Multi Rx RLM and BFD/CBD requirements | Nokia, Nokia Shanghai Bell |
| R4-2304050 | On Multi Rx TCI state switching requirements | Nokia, Nokia Shanghai Bell |
| R4-2304051 | Discussion on general RRM aspects for multi Rx | Nokia, Nokia Shanghai Bell |
| R4-2304052 | Discussion on MultiRx L1 measurements | Nokia, Nokia Shanghai Bell |
| R4-2304053 | Discussion on MultiRx scheduling restrictions | Nokia, Nokia Shanghai Bell |
| R4-2304054 | Discussion on Receive timing difference between different directions | Nokia, Nokia Shanghai Bell |
| R4-2304131 | On general aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2304132 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2304133 | On RLM and BFD/CBD for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2304134 | On TCI state switching for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2304152 | On scheduling/measurement restrictions for multiple Rx chains | Apple |
| R4-2304242 | Discussion on FR2 multi Rx chain general aspects | Intel Corporation |
| R4-2304243 | Discussion on RRM impacts for L1 measurement based on FR2 multi Rx chain | Intel Corporation |
| R4-2304244 | Discussion on RRM impacts for RLM BFD on FR2 multi Rx chain | Intel Corporation |
| R4-2304245 | Discussion on RRM impacts for scheduling/measurement restriction on FR2 multi Rx chain | Intel Corporation |
| R4-2304246 | Discussion on TCI state switching requirement based on FR2 multi Rx chain | Intel Corporation |
| R4-2304369 | Conditions of UE multiple Rx-beam based DL reception | Qualcomm Korea |
| R4-2304370 | L1 measurement requirements for support of simultaneous receptions from two TRPs | Qualcomm Korea |
| R4-2304371 | RLM and BM requirements for sDCI | Qualcomm Korea |
| R4-2304372 | SSB based scheduling and measurement restrictions | Qualcomm Korea |
| R4-2304373 | TCI and UE beam switching delay requirements | Qualcomm Korea |
| R4-2304706 | Discussion on general aspects of RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2304707 | Discussion on scheduling restriction for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2304708 | Discussion on L1-RSRP measurement for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2304789 | on the L1-RSRP measurement delay | Xiaomi |
| R4-2304790 | on the RLM and BFD CBD requirement | Xiaomi |
| R4-2304791 | on the scheduling measurement restrictions | Xiaomi |
| R4-2304792 | on the TCI state switching delay with dual TCI | Xiaomi |
| R4-2304793 | on the RTD between different directions | Xiaomi |
| R4-2304833 | On timing issues for multi RX in FR2 | Ericsson |
| R4-2304855 | Discussion on L1-RSRP measurement delay for FR2 multi-Rx chain DL reception | CMCC |
| R4-2304856 | Discussion on scheduling/measurement restrictions for FR2 multi-Rx chain DL reception | CMCC |
| R4-2304982 | Discussion on RLM/BFD/CBD requirements for FR2 multi-Rx chain DL reception | CMCC |
| R4-2304995 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2304996 | Discussion on L1-RSRP measurement requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2304997 | Discussion on RLM, BFD and CBD for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2304998 | Discussion on scheduling restriction and measurement restriction for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2304999 | Discussion on Receive timing difference between different directions | ZTE Corporation |
| R4-2305040 | On general aspects for FR2 multi-Rx | vivo |
| R4-2305041 | On L1-RSRP measurement delay for multi-Rx | vivo |
| R4-2305042 | On RLM and BFD/CBD requiremets for FR2 multi-Rx | vivo |
| R4-2305043 | On scheduling and measurement restrictions for multi-Rx | vivo |
| R4-2305044 | On TCI state switching delay for multi-Rx | vivo |
| R4-2305161 | Discussion on general aspects for multi-Rx UEs | MediaTek inc. |
| R4-2305162 | Discussion on L1 measurements for multi-Rx UEs | MediaTek inc. |
| R4-2305163 | Discussion on RLM/BFD/CBD for multi-Rx UEs | MediaTek inc. |
| R4-2305164 | Discussion on scheduling and measurement restriction for multi-Rx UEs | MediaTek inc. |
| R4-2305165 | Discussion on dual TCI state switch for multi-Rx UEs | MediaTek inc. |
| R4-2305166 | Discussion on receive timing difference for multi-Rx UEs | MediaTek inc. |
| R4-2305196 | General aspects discussions for FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2305197 | BFD/CBD requirements for FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2305208 | Discussion on general aspects for FR2 multi-RX DL reception | Samsung |
| R4-2305209 | Discussion on L1-RSRP measurement delay for FR2 multi-RX DL reception | Samsung |
| R4-2305210 | Discussion on RLM and BFD/CBD requirements for FR2 multi-RX DL reception | Samsung |
| R4-2305228 | On general aspects for FR2_multiRX_DL | OPPO |
| R4-2305229 | On L1 measurement for FR2_multiRX_DL | OPPO |
| R4-2305230 | On measurement restriction for FR2_multiRX_DL | OPPO |
| R4-2305231 | On TCI state switching for FR2 multi-Rx DL | OPPO |
| R4-2305268 | Discussion on general aspects for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2305269 | Discussion on TCI state switching delay with dual TCI for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2305270 | Discussion on receiving timing difference between different directions for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2305314 | Discussion on L1-RSRP measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2305315 | Discussion on RLM and BFD/CBD measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2305316 | Discussion on scheduling/measurement restrictions for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2305434 | Discussion on scheduling/measurement restrictions for FR2 multi-RX DL reception | Samsung |
| R4-2305754 | Discussion on Scheduling and measurement restrictions | Ericsson |
| R4-2305755 | Discussion on TCI state switching delay with dual TCI | Ericsson |
| R4-2305768 | On general aspects | Ericsson |
| R4-2305769 | On L1-RSRP measurements | Ericsson |
| R4-2305770 | On RLM and beam management | Ericsson |
| R4-2306209 | Topic summary for [106-bis-e][130] FR2_multiRx_UERF_part2 | Moderator (Apple) |
| R4-2306234 | Topic summary for [106-bis-e][203] FR2_multiRx_part2 | Moderator (Ericsson) |
| R4-2306291 | Topic summary for [106-bis-e][129] FR2_multiRx_UERF_part1 | Moderator (Qualcomm) |
| R4-2306318 | WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1) | Vivo |
| R4-2306319 | WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 2) | Ericsson |
| R4-2306392 | Topic summary for [106-bis-e][202] FR2_multiRx_part1 | Moderator (Vivo) |
| R4-2306394 | LS on RS supported for group-based reporting | Ericsson |
| R4-2306604 | WF on FR2-1 UERF requirements for FR2 multi-Rx | Vivo |
| R4-2304104 | On MultiRx Demodulation performance and CSI requirements - General aspects | Nokia |
| R4-2304106 | On MultiRx Demodulation performance and CSI requirements - PMI | Nokia |
| R4-2304105 | On MultiRx Demodulation performance and CSI requirements - PDSCH | Nokia |
| R4-2304135 | On General aspects for Multi-RX in FR2 requirements | Apple |
| R4-2304136 | On PDSCH demodulation requirements with multi-RX in FR2 | Apple |
| R4-2304137 | On PDSCH PMI reporting requirements with multi-RX in FR2 | Apple |
| R4-2304258 | General aspects on NR FR2 multi-Rx DL demodulation | Intel |
| R4-2304394 | Views on General Aspects for FR2 Multi-Rx | Qualcomm |
| R4-2304394 | Views on PDSCH Performance Requirements for FR2 Multi-Rx | Qualcomm |
| R4-2304393 | Views on PMI Reporting Requirements for FR2 Multi-Rx | Qualcomm |
| R4-2305479 | Discussion on general issues for UE demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2305481 | Discussion on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2305482 | Simulation results on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2305514 | General aspects for FR2 multi-Rx DL chain | Ericsson |
| R4-2305515 | PDSCH demodulation requirements for FR2 multi-Rx chain | Ericsson |
| R4-2305516 | PMI reporting for FR2 multi-Rx DL reception | Ericsson |
| R4-2305003 | Views on NR FR2 multi-Rx chain DL reception requirements for PDSCH | NTT DOCOMO |
| R4-2306291 | Topic summary for [106bis-e][129] FR2_multiRx_UERF_part1 | Moderator - Qualcomm |
| R4-2305865 | Topic summary for [106bis-e][319] NR_FR2_multiRX_DL_Demod | Moderator - Qualcomm |

## 4.6	Discussion Tdocs from RAN4#107:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2307232 | Discussion on System parameter assumption, UE architecture and conditions of UE RF requirements | Nokia, Nokia Shanghai Bell |
| R4-2307233 | Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2307345 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2307482 | UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2307511 | Discussion for FR2-1 multi-Rx | Murata Manufacturing Co Ltd. |
| R4-2307932 | Further discussion on FR2 Multi-RX DL 2AoA spherical coverage | Samsung |
| R4-2308231 | Requesting a new TR for FR2-1 multi-Rx DL reception RF requirement | vivo, Qualcomm |
| R4-2308232 | Discussion on the impact of power imbalance for sDCI UE | vivo |
| R4-2308233 | Discussion on the RF requirement for multi-Rx DL reception | vivo |
| R4-2308589 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2308807 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2308947 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2308978 | On AoA separation and UE orientation | OPPO |
| R4-2309031 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2309283 | Draft feature CR for FR2 multi-Rx | Qualcomm Incorporated |
| R4-2309284 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2307129 | Discussion on MultiRx RRM General Aspects | Nokia, Nokia Shanghai Bell |
| R4-2307130 | On MultiRx RLM and BFD/CBD requirements | Nokia, Nokia Shanghai Bell |
| R4-2307131 | On MultiRx TCI state switching requirements | Nokia, Nokia Shanghai Bell |
| R4-2307188 | Discussion on Multi-Rx L1 measurements | Nokia, Nokia Shanghai Bell |
| R4-2307189 | Discussion on Multi-Rx scheduling restrictions | Nokia, Nokia Shanghai Bell |
| R4-2307190 | Discussion on receive timing difference between different directions | Nokia, Nokia Shanghai Bell |
| R4-2307279 | Conditions of UE multiple Rx-beam based DL reception | Qualcomm Incorporated |
| R4-2307280 | L1 measurement requirements for support of simultaneous receptions from two TRPs | Qualcomm Incorporated |
| R4-2307281 | RLM and BM requirements | Qualcomm Incorporated |
| R4-2307282 | SSB based scheduling and measurement restrictions | Qualcomm Incorporated |
| R4-2307283 | TCI and UE beam switching delay requirements | Qualcomm Incorporated |
| R4-2307325 | On scheduling/measurement restrictions for multiple Rx chains | Apple |
| R4-2307346 | On general aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2307347 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2307348 | On RLM and BFD/CBD for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2307349 | On TCI state switching for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2307350 | On Receive timing difference between different directions for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2307574 | Discussion on L1-RSRP measurement delay for FR2 multi-Rx chain DL reception | CMCC |
| R4-2307575 | Discussion on scheduling/measurement restrictions for FR2 multi-Rx chain DL reception | CMCC |
| R4-2307576 | Discussion on RLM/BFD/CBD requirements for FR2 multi-Rx chain DL reception | CMCC |
| R4-2307717 | On general aspects for FR2 multi-Rx | vivo |
| R4-2307718 | On L1-RSRP measurement delay for multi-Rx | vivo |
| R4-2307719 | On RLM and BFD/CBD requirements for FR2 multi-Rx | vivo |
| R4-2307720 | On scheduling and measurement restrictions for multi-Rx | vivo |
| R4-2307721 | On TCI state switching delay for multi-Rx | vivo |
| R4-2307810 | Discussion on general aspects of RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2307811 | Discussion on L1-RSRP measurement for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2307821 | Discussion on scheduling restriction for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2307943 | General aspects discussions for FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2307953 | Discussion on the L1-RSRP measurement delay | Xiaomi |
| R4-2307954 | Discussion on the RLM and BFD CBD requirement | Xiaomi |
| R4-2307955 | Discussion on the scheduling measurement restriction | Xiaomi |
| R4-2307956 | Discussion on the TCI state switching delay with dual TCI | Xiaomi |
| R4-2308023 | On general aspects | Ericsson |
| R4-2308024 | On L1-RSRP measurements | Ericsson |
| R4-2308025 | On RLM and beam management | Ericsson |
| R4-2308321 | Discussion on general aspects for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2308322 | Discussion on TCI state switching delay with dual TCI for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2308323 | Discussion on receiving timing difference between different directions for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2308476 | On general aspects for FR2_multiRX_DL | OPPO |
| R4-2308477 | On L1 measurement for FR2_multiRX_DL | OPPO |
| R4-2308478 | On measurement restriction for FR2_multiRX_DL | OPPO |
| R4-2308479 | On TCI state switching for FR2 multi-Rx DL | OPPO |
| R4-2308503 | Discussion on general aspects for multi-Rx UEs | MediaTek inc. |
| R4-2308504 | Discussion on L1 measurements for multi-Rx UEs | MediaTek inc. |
| R4-2308505 | Discussion on RLM/BFD/CBD for multi-Rx UEs | MediaTek inc. |
| R4-2308506 | Discussion on scheduling and measurement restriction for multi-Rx UEs | MediaTek inc. |
| R4-2308507 | Discussion on dual TCI state switch for multi-Rx UEs | MediaTek inc. |
| R4-2308508 | Discussion on receive timing difference for multi-Rx UEs | MediaTek inc. |
| R4-2308700 | Discussion on L1-RSRP measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2308701 | Discussion on RLM/BFD/CBD measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2308702 | Discussion on scheduling/measurement restrictions for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2308725 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2308726 | Discussion on L1-RSRP measurement requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2308727 | Discussion on RLM, BFD and CBD for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2308728 | Discussion on scheduling restriction and measurement restriction for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2308729 | Discussion on Receive timing difference between different directions | ZTE Corporation |
| R4-2308922 | On timing requirements in FR2 multi-RX | Ericsson |
| R4-2309590 | Discussion on Scheduling and measurement restrictions | Ericsson |
| R4-2309591 | Discussion on TCI state switching delay with dual TCI | Ericsson |
| R4-2309655 | Discussion on general aspects for FR2 multi-RX DL reception | Samsung |
| R4-2309656 | Discussion on scheduling/measurement restrictions for FR2 multi-RX DL | Samsung |
| R4-2309657 | Discussion on L1-RSRP measurement delay for FR2 multi-RX DL reception | Samsung |
| R4-2310155 | Ad-hoc minutes for FR2 Multi-Rx chain RRM requirements | vivo |
| R4-2310046 | WF on NR FR2 Multi-Rx chain DL reception RRM requirements (part 1) | vivo |
| R4-2310164 | LS on overheating indication for NR FR2 multi-Rx chain DL reception | Ericsson |
| R4-2310047 | WF on NR FR2 Multi-Rx chain DL reception RRM requirements (part 2) | Ericsson |
| R4-2307332 | On General aspects for Multi-RX in FR2 demod requirements | Apple |
| R4-2307333 | On PDSCH demodulation requirements with multi-RX in FR2 | Apple |
| R4-2307334 | On PDSCH PMI reporting requirements with multi-RX in FR2 | Apple |
| R4-2308885 | Discussion on general issues for UE demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2308887 | Discussion on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2308886 | Discussion on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2308944 | General aspects for FR2 multi-Rx DL chain | Ericsson |
| R4-2308945 | PDSCH demodulation requirements for FR2 multi-Rx chain | Ericsson |
| R4-2308946 | PMI reporting for FR2 multi-Rx DL reception | Ericsson |
| R4-2307472 | On MultiRx Demodulation performance and CSI requirements - General aspects | Nokia |
| R4-2307473 | On MultiRx Demodulation performance and CSI requirements - PDSCH | Nokia |
| R4-2307474 | On MultiRx Demodulation performance and CSI requirements - PMI | Nokia |
| R4-2307851 | Discussion on general aspects of FR2 Multi-Rx DL Demod | MediaTek |
| R4-2307852 | Discussion on PDSCH requirements of FR2 Multi-Rx DL Demod | MediaTek |
| R4-2307853 | Discussion on PMI requirements of FR2 Multi-Rx DL Demod | MediaTek |
| R4-2309684 | Views on General Aspects for FR2 Multi-Rx | Qualcomm |
| R4-2309686 | Views on PDSCH Performance Requirements for FR2 Multi-Rx | Qualcomm |
| R4-2309687 | Views on PMI Reporting Requirements for FR2 Multi-Rx | Qualcomm |
| R4-2308119 | Views on NR FR2 multi-Rx chain DL reception requirements | NTT DoCoMo |
| R5-2310015 | Topic summary for [107][132] FR2_multiRx_UERF_part1 | Moderator - Qualcomm |
| R4-2310459 | Topic summary for [107][322] NR_FR2_multiRX_DL_Demod | Moderator - Qualcomm |

## 4.7	Discussion Tdocs from RAN4#108:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2311333 | TP on DL power for TR 38.751 | Apple |
| R4-2311334 | TP on DL polarization combination for TR 38.751 | Apple |
| R4-2311863 | TR skeleton for UE RF requirements for NR frequency range 2 multi-Rx chain DL reception | vivo |
| R4-2311284 | Feature CR for FR2 multi-Rx | Qualcomm Incorporated |
| R4-2311285 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2311332 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2311430 | RF requirement for FR2-1 multi-Rx | Murata Manufacturing Co Ltd. |
| R4-2311870 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2312268 | UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2312505 | Discussion on remaining issues and draft LS on test condition | Samsung |
| R4-2312506 | Text proposal to Multi-RX DL TR 38.751 on RMC and UE orientation | Samsung |
| R4-2312577 | TP to TR 38.751 on system assumption and UE RF requirement | vivo, Qualcomm |
| R4-2312578 | Further evaluation on FR2 multi-Rx RF requirement | vivo |
| R4-2312917 | Simulation of 2 AoA reception performance | OPPO |
| R4-2312918 | On requirement metric for 2 AoA reception | OPPO |
| R4-2313095 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2313198 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2313418 | Discussion on RF requirement for NR FR2 multi-Rx chain DL reception | MediaTek (Shenzhen) Inc. |
| R4-2313712 | Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2313713 | TP to TR 38.751 on inter-beam interference in multi-Rx DL reception | Nokia, Nokia Shanghai Bell |
| R4-2313714 | Draft CR for FR2 Multi-Rx | Nokia, Nokia Shanghai Bell |
| R4-2314668 | WF on FR2_multiRx_UERF | vivo |
| R4-2311175 | Discussion on general aspects for multi-Rx UEs | MediaTek inc. |
| R4-2311176 | Discussion on L1 measurements for multi-Rx UEs | MediaTek inc. |
| R4-2311177 | Discussion on RLM/BFD/CBD for multi-Rx UEs | MediaTek inc. |
| R4-2311178 | Discussion on scheduling and measurement restriction for multi-Rx UEs | MediaTek inc. |
| R4-2311179 | Discussion on dual TCI state switch for multi-Rx UEs | MediaTek inc. |
| R4-2311180 | Discussion on receive timing difference for multi-Rx UEs | MediaTek inc. |
| R4-2311322 | On scheduling/measurement restrictions for multiple Rx chains | Apple |
| R4-2311335 | On general aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2311336 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2311337 | On RLM and BFD/CBD for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2311338 | On TCI state switching for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2311339 | On Receive timing difference between different directions for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2311340 | draft CR on DCI based TCI state switch and active TCI state list update | Apple |
| R4-2311897 | Discussion on TCI state switching delay for FR2 multi-Rx chain DL reception | CMCC |
| R4-2311898 | Discussion on L1-RSRP measurement delay for FR2 multi-Rx chain DL reception | CMCC |
| R4-2311899 | Discussion on scheduling/measurement restrictions for FR2 multi-Rx chain DL reception | CMCC |
| R4-2311900 | Discussion on RLM/BFD/CBD requirements for FR2 multi-Rx chain DL reception | CMCC |
| R4-2312144 | On general aspects for FR2 multi-Rx | vivo |
| R4-2312145 | On L1-RSRP measurement delay for FR2 multi-Rx | vivo |
| R4-2312146 | On receive timing difference for FR2 multi-Rx | vivo |
| R4-2312147 | On RLM and BFD/CBD requirements for FR2 multi-Rx | vivo |
| R4-2312148 | On scheduling and measurement restrictions for multi-Rx | vivo |
| R4-2312149 | On TCI state switch delay for multi-Rx | vivo |
| R4-2312153 | Big CR for RRM requirements for NR FR2 multi-Rx chain DL reception | vivo |
| R4-2312265 | Discussion on general aspects of RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2312267 | Discussion on mDCI based TCI state switching for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2312414 | Discussion on general aspects for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2312415 | Discussion on TCI state switching delay with dual TCI for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2312416 | Discussion on receiving timing difference between different directions for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2312517 | Discussion on Multi-RX RRM General Aspects | Nokia, Nokia Shanghai Bell |
| R4-2312518 | On MultiRx RLM and BFD/CBD requirements | Nokia, Nokia Shanghai Bell |
| R4-2312519 | Draft CR on MultiRx TRP specific Link Recovery Procedures | Nokia, Nokia Shanghai Bell |
| R4-2312520 | On Multi Rx TCI state switching requirements | Nokia, Nokia Shanghai Bell |
| R4-2312649 | On general aspects for FR2_multiRX_DL | OPPO |
| R4-2312650 | On L1-RSRP measurement for FR2_multiRX_DL | OPPO |
| R4-2312651 | On RLM and BFD CBD requirement for FR2_multiRX_DL | OPPO |
| R4-2312652 | Draft CR for BFD related requirements of R18 multi-Rx DL | OPPO |
| R4-2312653 | On scheduling and measurement restriction for FR2_multiRX_DL | OPPO |
| R4-2312654 | On TCI state switching for FR2 multi-Rx DL | OPPO |
| R4-2312725 | On general aspects | Ericsson |
| R4-2312726 | On L1-RSRP measurements | Ericsson |
| R4-2312727 | On RLM and beam management | Ericsson |
| R4-2312728 | RLM requirements for UE with multi-rx chain in FR2 | Ericsson |
| R4-2312866 | Discussion on L1-RSRP measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2312867 | DraftCR on L1-RSRP measurement requirements for FR2 multi-Rx enhancement | Huawei, HiSilicon |
| R4-2312868 | Discussion on RLM/BFD/CBD measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2312869 | Discussion on scheduling/measurement restrictions for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2312938 | Discussion on Multi-Rx L1 measurements | Nokia, Nokia Shanghai Bell |
| R4-2312939 | Draft CR on MultiRx L1-SINR | Nokia, Nokia Shanghai Bell |
| R4-2312940 | Discussion on measurement and scheduling restrictions | Nokia, Nokia Shanghai Bell |
| R4-2312941 | Discussion on maximum receive time difference | Nokia, Nokia Shanghai Bell |
| R4-2313112 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2313113 | Discussion on L1-RSRP measurement requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2313114 | Discussion on RLM, BFD and CBD for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2313115 | Discussion on scheduling restriction and measurement restriction for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2313116 | Discussion on Receive timing difference between different directions | ZTE Corporation |
| R4-2313382 | Discussion on general aspects for FR2 multi-RX DL reception | Samsung |
| R4-2313383 | Discussion on TCI state switching for FR2 multi-RX DL reception | Samsung |
| R4-2313686 | Discussion on scheduling restriction and measurements restirctions | Ericsson |
| R4-2313687 | On dual TCI state switch requirements | Ericsson |
| R4-2313688 | Draft CR on MAC-CE based dual TCI state switch | Ericsson |
| R4-2313759 | [NR_FR2_multiRX_DL-Core] Signaling of UE multiple Rx-beam operation mode | Qualcomm Incorporated |
| R4-2313760 | [NR_FR2_multiRX_DL-Core] L1 measurement requirements for support of simultaneous receptions from two TRPs | Qualcomm Incorporated |
| R4-2313761 | [NR_FR2_multiRX_DL-Core] RLM and BM requirements | Qualcomm Incorporated |
| R4-2313762 | [NR_FR2_multiRX_DL-Core] CSI-RS based scheduling and measurement restrictions | Qualcomm Incorporated |
| R4-2313763 | [NR_FR2_multiRX_DL-Core] TCI state switching delay requirements | Qualcomm Incorporated |
| R4-2314477 | WF on FR2 multi-RX operation RRM requirements (part 1) | vivo |
| R4-2314478 | LS out on UE indication of FR2 multi-RX operation | Apple |
| R4-2314283 | WF on FR2 multi-RX operation RRM requirements (part 2) | Ericsson |
| R4-2314479 | LS on Dual TCI state switching in mDCI | Ericsson |
| R4-2311349 | On General aspects for Multi-RX in FR2 requirements | Apple |
| R4-2311350 | Performance Evaluation of PDSCH with multi-RX in FR2 | Apple |
| R4-2311351 | On PDSCH PMI reporting requirements with multi-RX in FR2 | Apple |
| R4-2311743 | On MultiRx Demodulation performance and CSI requirements - General aspects | Nokia, Nokia Shanghai Bell |
| R4-2311744 | On MultiRx Demodulation performance and CSI requirements - PDSCH | Nokia, Nokia Shanghai Bell |
| R4-2311745 | On MultiRx Demodulation performance and CSI requirements - simulation results | Nokia, Nokia Shanghai Bell |
| R4-2311746 | On MultiRx Demodulation performance and CSI requirements - PMI | Nokia, Nokia Shanghai Bell |
| R4-2311756 | Views on General Aspects for FR2 Multi-Rx | Qualcomm India Pvt Ltd |
| R4-2311757 | Views on PDSCH Performance Requirements for FR2 Multi-Rx | Qualcomm India Pvt Ltd |
| R4-2311758 | Simulation Results on PDSCH Performance Requirements for FR2 Multi-Rx | Qualcomm India Pvt Ltd |
| R4-2311992 | Discussion on general aspects of FR2 multiRX DL | MediaTek inc. |
| R4-2311993 | Discussion on PDSCH requirements of FR2 multiRX DL | MediaTek inc. |
| R4-2311994 | Simulation results of PDSCH requirements of FR2 multiRX DL | MediaTek inc. |
| R4-2311995 | Discussion on PMI requirements of FR2 multiRX DL | MediaTek inc. |
| R4-2312353 | discussion on FR2 Multi-Rx PDSCH demodulation requirements | Samsung |
| R4-2313315 | General aspects for FR2 multi-Rx DL chain | Ericsson |
| R4-2313316 | PDSCH demodulation requirements for FR2 multi-Rx chain | Ericsson |
| R4-2313317 | Simulation results for PDSCH of FR2 Multi-Rx UE | Ericsson |
| R4-2313318 | PMI reporting for FR2 multi-Rx DL reception | Ericsson |
| R4-2313651 | Discussion on general issues for UE demodulation requirements for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2313652 | Discussion on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2313653 | Discussion on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2314152 | Topic summary for [108][207] FR2_multiRx_part1 | Moderator (Vivo) |
| R4-2314153 | Topic summary for [108][208] FR2_multiRx_part2 | Moderator (Ericsson) |
| R4-2314214 | Topic summary for [108][132] FR2_multiRx_UERF_part1 | Moderator (Qualcomm) |
| R4-2314215 | Topic summary for [108][133] FR2_multiRx_UERF_part2 | Moderator (Apple) |
| R4-2314258 | Topic summary for [108][322] NR_FR2_multiRX_DL_Demod | Moderator (Qualcomm) |
| R4-2313977 | WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception | Qualcomm |

## 4.8	Discussion Tdocs from RAN4#108bis:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2315564 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2316504 | Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2315086 | TR 38.751v1.1.0 for UE RF requirement for NR FR2 multi-Rx chain DL reception | vivo Japan KK |
| R4-2316223 | Discussion on UE RF requirements | OPPO |
| R4-2317591 | LS to RAN5 on Multi-Rx DL 2AoA spherical coverage test | Samsung, Apple |
| R4-2315057 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2315292 | Feature CR for FR2 multi-Rx | Qualcomm Incorporated |
| R4-2315502 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2315537 | UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2315552 | Multi-RX DL 2AoA spherical coverage simulation and requirements | Samsung |
| R4-2315812 | Discussion on remaining issue on FR2 multi-Rx | vivo |
| R4-2316224 | Simulation results of 2 AoA reception performance | OPPO |
| R4-2316332 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2316367 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2317592 | TP to 38.751 on collection of simultion results | vivo |
| R4-2315412 | Discussion on general aspects for Multi-RX | xiaomi |
| R4-2315504 | On general aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2315638 | Discussion on general aspects for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2315748 | On general aspects for FR2 multi-Rx | vivo |
| R4-2315899 | On general aspects | Ericsson |
| R4-2315920 | Discussion on multi-Rx general aspects | Nokia, Nokia Shanghai Bell |
| R4-2316155 | On general aspects for FR2_multiRX_DL | OPPO |
| R4-2316447 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2317335 | Correction to big CR on FR2 multi-RX | Ericsson |
| R4-2315413 | Discussion on L1-RSRP measurement for Multi-RX | xiaomi |
| R4-2315505 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2315749 | On L1-RSRP measurement delay for FR2 multi-Rx | vivo |
| R4-2315900 | On L1-RSRP measurements | Ericsson |
| R4-2315921 | Discussion on multi-Rx L1-measurements | Nokia, Nokia Shanghai Bell |
| R4-2315922 | DraftCR for multi-Rx L1-SINR requirements | Nokia, Nokia Shanghai Bell |
| R4-2316021 | Discussion on L1-RSRP measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2316156 | On L1-RSRP measurement for FR2_multiRX_DL | OPPO |
| R4-2316448 | Discussion on L1-RSRP measurement requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2316771 | Discussion on L1-RSRP measurements | Samsung |
| R4-2317376 | DraftCR on L1-RSRP measurement requirements for FR2 multi-Rx enhancement | Huawei, HiSilicon |
| R4-2315506 | On RLM and BFD/CBD for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2315750 | On RLM and BFD/CBD requirements for FR2 multi-Rx | vivo |
| R4-2315901 | On RLM and beam management | Ericsson |
| R4-2315923 | Discussion on multi-Rx RLM and BFD/CBD requirements | Nokia, Nokia Shanghai Bell |
| R4-2316157 | On RLM and BFD CBD requirement for FR2_multiRX_DL | OPPO |
| R4-2316449 | Discussion on RLM, BFD and CBD for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2317333 | DraftCR for multi-Rx TRP-specific BFD requirements | Nokia, Nokia Shanghai Bell |
| R4-2317334 | Draft CR for BFD related requirements of R18 multi-Rx DL | OPPO |
| R4-2315343 | Discussion on scheduling/measurement restrictions for FR2 multi-Rx chain DL reception | CMCC |
| R4-2315751 | On scheduling and measurement restrictions for multi-Rx | vivo |
| R4-2315925 | Discussion on multi-Rx scheduling and measurement restrictions | Nokia, Nokia Shanghai Bell |
| R4-2316023 | Discussion on scheduling/measurement restrictions for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2316159 | On scheduling and measurement restriction for FR2_multiRX_DL | OPPO |
| R4-2316450 | Discussion on scheduling restriction and measurement restriction for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2316582 | On scheduling/measurement restrictions for multiple Rx chains | Apple |
| R4-2316770 | Discussion on scheduling and measurement restrictions | Samsung |
| R4-2316812 | Scheduling restriction and sharing factor for layer-3 measurements | Ericsson |
| R4-2315414 | Discussion on TCI activation for Multi-RX | xiaomi |
| R4-2315415 | DraftCR on MAC CE based TCI state list update delay for mDCI | xiaomi |
| R4-2315507 | On TCI state switching for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2315639 | Discussion on TCI state switching delay with dual TCI for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2315640 | Draft CR on TCI state switching requirements for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2315752 | On TCI state switch delay for multi-Rx | vivo |
| R4-2315926 | Discussion on multi-Rx TCI state switching requirements | Nokia, Nokia Shanghai Bell |
| R4-2316160 | On TCI state switching for FR2 multi-Rx DL | OPPO |
| R4-2316813 | On dual TCI state switch requireements | Ericsson |
| R4-2317403 | draft CR on DCI based TCI state switch and active TCI state list update | Apple |
| R4-2315509 | On Receive timing difference between different directions for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2315510 | draft CR on capturing RTD < CP as a condition for multi-RX RRM requirements | Apple |
| R4-2316814 | on extending GBBR L1-RSRP requirements to RTD>CP | Ericsson |
| R4-2315641 | Discussion on performance requirements for FR2 multi-RX | Huawei, HiSilicon |
| R4-2315902 | On RRM performance aspects | Ericsson |
| R4-2315927 | Discussion on multi-Rx performance requirements | Nokia, Nokia Shanghai Bell |
| R4-2316024 | Discussion on RRM performance for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2316874 | Draft Simulation Results Summary for FR2 Multi-Rx Demodulation and CSI requirements | Qualcomm Inc |
| R4-2315349 | Views on General Aspects for FR2 Multi-Rx Performance Requirements | Qualcomm Inc |
| R4-2315474 | On General aspects for Multi-RX in FR2 requirements | Apple |
| R4-2315913 | On MultiRx Demodulation performance and CSI requirements - General aspects | Nokia, Nokia Shanghai Bell |
| R4-2315936 | Discussion on general aspects of FR2 multiRX DL | MediaTek inc. |
| R4-2316016 | Discussion on general issues for UE demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei,HiSilicon |
| R4-2315350 | Views on PDSCH Performance Requirements for FR2 Multi-Rx | Qualcomm Inc |
| R4-2315352 | Simulation Results on PDSCH Performance Requirements for FR2 Multi-Rx | Qualcomm Inc |
| R4-2315475 | Simulation results for PDSCH with multi-RX in FR2 | Apple |
| R4-2315476 | On PDSCH demod requirements with multi-RX in FR2 | Apple |
| R4-2315914 | On MultiRx Demodulation performance and CSI requirements - PDSCH | Nokia, Nokia Shanghai Bell |
| R4-2315915 | On MultiRx Demodulation performance and CSI requirements - Simulation Results | Nokia, Nokia Shanghai Bell |
| R4-2315937 | Discussion on PDSCH requirements of FR2 multiRX DL | MediaTek inc. |
| R4-2315938 | Simulation results of PDSCH requirements of FR2 multiRX DL | MediaTek inc. |
| R4-2316017 | Discussion on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei,HiSilicon |
| R4-2316081 | PDSCH demodulation requirements for FR2 UE multi-Rx reception | Ericsson |
| R4-2315477 | On PMI reporting requirements with multi-RX in FR2 | Apple |
| R4-2315916 | On MultiRx Demodulation performance and CSI requirements - PMI | Nokia, Nokia Shanghai Bell |
| R4-2315939 | Discussion on PMI requirements of FR2 multiRX DL | MediaTek inc. |
| R4-2316018 | Discussion on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception | Huawei,HiSilicon |
| R4-2316082 | PMI reporting requirements for FR2 UE multi-Rx reception | Ericsson |
| R4-2316929 | Ad-hoc meeting minutes on NR_FR2_multiRX_DL_Demod | Qualcomm |
| R4-2316963 | WF on NR_FR2_multiRX_DL_Demod | Qualcomm |
| R4-2317194 | Topic summary for [108-bis][202] FR2_multiRx_part1 | Moderator (Vivo) |
| R4-2317195 | Topic summary for [108-bis][203] FR2_multiRx_part2 | Moderator (Ericsson) |
| R4-2317251 | Topic summary for [108-bis][127] FR2_multiRx_UERF_part1 | Moderator (Qualcomm) |
| R4-2317276 | Ad-hoc minutes for FR2_multiRx WI | Vivo |
| R4-2317332 | WF on FR2 multi-RX operation RRM requirements (part 1) | Vivo |
| R4-2317392 | WF on FR2 HST RRM core requirement maintenance | Nokia |
| R4-2317427 | WF on FR2_multiRx_part2 | Ericsson |
| R4-2317593 | WF on requirement for NR FR2 multi-Rx | Sony, Qualcomm |
| R4-2317947 | Topic summary for [108bis][316] NR_FR2_multiRX_DL_Demod | Moderator(Qualcomm) |

## 4.9	Discussion Tdocs from RAN4#109:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2318361 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2318685 | On associated UE capability for RF requirement | Apple |
| R4-2321722 | Feature CR for FR2 multi-Rx | Qualcomm Incorporated |
| R4-2321723 | TP on NTC vs. ETC for TR 38.751 | Apple |
| R4-2321724 | TP on Annex <A>:Simulation results for TR 38.751 | Apple |
| R4-2318492 | Further views/Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2318686 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2318771 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2318812 | UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2318983 | Discussion on remaining issue on FR2 multi-Rx RF requirement | vivo |
| R4-2319267 | Multi-RX DL 2AoA spherical coverage simulation and requirements | Samsung |
| R4-2319565 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2320078 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2321725 | TP to 38.751 on further evaluation of gain difference between V-pol and H-pol | vivo |
| R4-2321726 | TP to 38.751 on RF requirement construction | vivo |
| R4-2318499 | Discussion on feature list for multi-Rx UEs | MediaTek inc. |
| R4-2318689 | On general aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2318690 | Draft LS on associated UE capabilities for UE indication of FR2 multi-RX operation | Apple |
| R4-2318851 | Discussion on general aspects for Multi-RX | xiaomi |
| R4-2319041 | Remaining issues on general aspects for FR2 multi-Rx | vivo |
| R4-2319272 | On general aspects of multi-Rx | Nokia, Nokia Shanghai Bell |
| R4-2319358 | Discussion on general aspects for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2319463 | On general aspects for FR2_multiRX_DL | OPPO |
| R4-2319724 | Discussion on UE capability for Multi-Rx | Samsung |
| R4-2320424 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2320461 | On general aspects | Ericsson |
| R4-2318691 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2318692 | draft CR on L1 measurement | Apple |
| R4-2319042 | Remaining issues on L1-RSRP measurement delay for FR2 multi-Rx | vivo |
| R4-2319273 | On multi-Rx L1-RSRP measurements | Nokia, Nokia Shanghai Bell |
| R4-2319464 | On L1-RSRP measurement for FR2_multiRX_DL | OPPO |
| R4-2319722 | Discussion on L1-RSRP measurement for Multi-Rx | Samsung |
| R4-2319956 | Discussion on L1-RSRP measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2320425 | Discussion on L1-RSRP measurement requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2320462 | On L1-RSRP measurements | Ericsson |
| R4-2321592 | DraftCR on L1-RSRP measurement requirements for FR2 multi-Rx enhancement | Huawei, HiSilicon |
| R4-2321593 | Draft CR on multi-Rx L1-SINR requirements | Nokia, Nokia Shanghai Bell |
| R4-2318693 | On RLM and BFD/CBD for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2319043 | Remaining issues on RLM and BFD/CBD requirements for FR2 multi-Rx | vivo |
| R4-2319275 | On multi-Rx RLM, BFD and CBD requirements | Nokia, Nokia Shanghai Bell |
| R4-2319276 | Draft CR on multi-Rx TRP-specific BFD requirements | Nokia, Nokia Shanghai Bell |
| R4-2319465 | On RLM and BFD CBD requirement for FR2_multiRX_DL | OPPO |
| R4-2320426 | Discussion on RLM, BFD and CBD for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2320463 | On RLM and beam management | Ericsson |
| R4-2321595 | Draft CR for BFD&CBD related requirements of R18 multi-Rx DL(8.5 in 38.133) | OPPO |
| R4-2321596 | [NR_FR2_multiRX_DL-Core] Draft CR on TRP specific link recovery for multi-Rx | ZTE Corporation |
| R4-2321601 | Draft CR on RLM requirements for UE with multi-rx chain in FR2 | Ericsson |
| R4-2318500 | Discussion on scheduling and measurement restriction for multi-Rx UEs | MediaTek inc. |
| R4-2318653 | On scheduling/measurement restrictions for multiple Rx chains | Apple |
| R4-2319044 | Remaining issues on scheduling and measurement restrictions for FR2 multi-Rx | vivo |
| R4-2319277 | On multi-Rx scheduling and measurement restrictions | Nokia, Nokia Shanghai Bell |
| R4-2319723 | Discussion on measurement and scheduling restriction for Multi-Rx | Samsung |
| R4-2320760 | Scheduling restriction and sharing factor for layer-3 measurements | Ericsson |
| R4-2318694 | On TCI state switching for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2318852 | Discussion on TCI activation for Multi-RX | xiaomi |
| R4-2319045 | Remaining issues on TCI state switch delay for FR2 multi-Rx | vivo |
| R4-2319278 | On multi-Rx TCI state switching requirements | Nokia, Nokia Shanghai Bell |
| R4-2319279 | Draft CR on multi-Rx TCI state switching requirements | Nokia, Nokia Shanghai Bell |
| R4-2319359 | Discussion on TCI state switching delay with dual TCI for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2319360 | Draft CR on TCI state switching requirements for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2319467 | On TCI state switching for FR2 multi-Rx DL | OPPO |
| R4-2320761 | On dual TCI state switch requirements | Ericsson |
| R4-2321598 | draft CR on DCI based TCI state switch and active TCI state list update | Apple |
| R4-2318696 | On Receive timing difference between different directions for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2318697 | draft CR on capturing RTD < CP as a condition for multi-RX RRM requirements | Apple |
| R4-2319046 | Remaining issues on RTD requirements for FR2 multi-Rx | vivo |
| R4-2320427 | Discussion on receiving time difference | ZTE Corporation |
| R4-2320724 | Discussion on Multi-RX RRM MRTD requirements | Nokia, Nokia Shanghai Bell |
| R4-2320725 | Draft CR MRTD requirements for Multi Rx | Nokia, Nokia Shanghai Bell |
| R4-2320763 | On extending GBBR L1-RSRP to RTD>CP | Ericsson |
| R4-2318698 | On RRM performance requirements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2319047 | Discussion on performance requirements for FR2 multi-Rx | vivo |
| R4-2319280 | On multi-Rx performance requirements | Nokia, Nokia Shanghai Bell |
| R4-2319361 | Discussion on performance requirements for FR2 multi-RX | Huawei, HiSilicon |
| R4-2319468 | Discussion on test cases for FR2 multi-Rx | OPPO |
| R4-2320464 | On RRM performance aspects | Ericsson |
| R4-2320513 | Discussion on performance requirements multi-Rx | ZTE Corporation |
| R4-2318733 | Simulation results summary for FR2 multi-Rx performance requirements | Qualcomm India Pvt Ltd, Apple |
| R4-2321206 | Draft CR to include the FR2 multi-rx correlation model in the specification | Qualcomm India Pvt Ltd |
| R4-2318549 | Discussion on general aspects of FR2 multiRX DL | MediaTek inc. |
| R4-2318570 | On General aspects for Multi-RX in FR2 requirements | Apple |
| R4-2318730 | Views on General Aspects for FR2 Multi-Rx Performance Requirements | Qualcomm India Pvt Ltd |
| R4-2318790 | On MultiRx Demodulation performance and CSI requirements - General aspects | Nokia, Nokia Shanghai Bell |
| R4-2320233 | Discussion on general issues for UE demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei,HiSilicon |
| R4-2318550 | Discussion on PDSCH requirements of FR2 multiRX DL | MediaTek inc. |
| R4-2318551 | Simulation results of PDSCH requirements of FR2 multiRX DL | MediaTek inc. |
| R4-2318571 | Simulation results for PDSCH with multi-RX in FR2 | Apple |
| R4-2318573 | On PDSCH demod requirements with multi-RX in FR2 | Apple |
| R4-2318731 | Views on PDSCH Aspects for FR2 Multi-Rx Performance Requirements | Qualcomm India Pvt Ltd |
| R4-2318732 | Simulation Results on PDSCH Performance Requirements for FR2 Multi-Rx | Qualcomm India Pvt Ltd |
| R4-2318791 | On MultiRx Demodulation performance and CSI requirements - PDSCH | Nokia, Nokia Shanghai Bell |
| R4-2318792 | On MultiRx Demodulation performance and CSI requirements - Simulation Results | Nokia, Nokia Shanghai Bell |
| R4-2319743 | PDSCH demodulation requirements for FR2 UE multi-Rx reception | Ericsson |
| R4-2319744 | Simulation results for FR2 UE multi-Rx reception | Ericsson |
| R4-2321205 | Draft CR on Minimum requirements and FRC definition for sDCI SDM (TS38.101-4, Rel-18) | Huawei,HiSilicon |
| R4-2321207 | DraftCR on PDSCH demod requirements for mDCI fully-overlapping with multi-RX in FR2 | Apple |
| R4-2321208 | DraftCR on Minimum requirements and Reference Channel for mDCI non-overlapping | Nokia, Nokia Shanghai Bell |
| R4-2318552 | Discussion on PMI requirements of FR2 multiRX DL | MediaTek inc. |
| R4-2318553 | Simulation results of PMI requirements of FR2 multiRX DL | MediaTek inc. |
| R4-2318554 | Draft CR to 38.101-4 PMI requirements of FR2 multiRX DL | MediaTek inc. |
| R4-2318555 | Draft CR to 38.101-4 PMI reference measurement channel of FR2 multiRX DL | MediaTek inc. |
| R4-2318574 | On PMI reporting requirements with multi-RX in FR2 | Apple |
| R4-2318793 | On MultiRx Demodulation performance and CSI requirements - PMI | Nokia, Nokia Shanghai Bell |
| R4-2319745 | PMI reporting requirements for FR2 UE multi-Rx reception | Ericsson |
| R4-2320234 | Discussion on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception | Huawei,HiSilicon |
| R4-2318162 | Topic summary for [109][206] FR2_multiRx_part1 | Moderator (Vivo) |
| R4-2318163 | Topic summary for [109][207] FR2_multiRx_part2 | Moderator (Ericsson) |
| R4-2321111 | Ad-hoc meeting minutes on [109][318] NR_FR2_multiRX_DL_Demod | Qualcomm |
| R4-2321140 | WF on [109][318] NR_FR2_multiRX_DL_Demod | Qualcomm |
| R4-2321325 | Ad-hoc minutes on NR FR2 multi-Rx chain WI | Vivo |
| R4-2321602 | WF on core maintenance and performance part for multi-Rx WI | vivo |
| R4-2321828 | Ad hoc minutes on FR2_multiRx_UERF | Qualcomm |