3GPP TSG-RAN Meeting #102                                                               	 RP-233099

Edinburgh, UK, 11th – 15th December, 2023

Agenda item:			9.1.1.7

Source:			Nokia

Title:		Moderator's summary on new SI Study on Integrated sensing and communication (ISAC) for NR

Document for:	 	Discussion

# General

Following the outcome of RAN#101 in [1] and the RAN Chair’s summary for RAN#102 in [2], this document summarises the general view of the contributions at RAN#102 for this agenda item, with a view to further developing the draft SID provided in [3].

In particular, it is stated in [2]: “RAN to select a small number of ISAC use cases from TR22.837, and to identify appropriate corresponding information needed for the point below, i.e. scenarios, frequency ranges and sensing modes. Also consider the ongoing SA1 TS22.137”. Considering this, and the contributions already submitted to RAN#102, it seems unnecessary to initiate a RAN-led SI in order to accomplish the selection of use cases, scenarios, frequency ranges and sensing modes. These aspects can simply be discussed in RAN and the RAN1-led SID agreed accordingly.

The relevant contributions are [4]-[31].

# Potential Use Cases based on Submitted Contributions

The SA1 TS 22.137 categorizes the many use cases from TR22.837 into three broad categories:

- Object detection and tracking

- Environment monitoring

- Motion monitoring.

All companies expressing a preference focus on the “object detection and tracking” category, with some companies highlighting additional difficulty of the monitoring categories.

This category is very broad. It encompasses UAV detection/tracking, as already identified at RAN#101, but it also covers a wide range of other kinds of object to be detected, as well as both indoor and outdoor environments. It is necessary to narrow down the intended scenarios in order to bring a reasonable scope to the RAN1 SI.

In addition to UAVs, the most commonly cited use cases include:

- Indoor detection of humans (and in some cases animals)

- Outdoor automotive cases

- Automated guided vehicles (e.g. in indoor factories)

- Unspecified objects on railways

# Sensing modes

Several companies suggest including all six sensing modes (gNB monostatic, UE monostatic, gNB-gNB bistatic, UE-UE bistatic, gNB-UE bistatic, UE-gNB bistatic).

However, others focus primarily on bistatic cases, especially gNB<->UE bistatic and gNB-gNB bistatic.

It should be discussed whether the study can focus on only a subset of the six sensing modes.

# Frequencies

The greatest interest for sensing is clearly in FR1 and FR2-1, but there does not seem to be any significant reason to exclude any parts of the full frequency range (0.5-100 GHz) covered by TR 38.901.

It could be discussed whether frequencies from 0.5 to 52.6 GHz could be the primary focus, with the assumption that a similar modelling approach should scale to 100GHz.

# Summary

The RAN-led SID on ISAC can be agreed in RAN directly, without the need for a RAN-level study.

The following points should be discussed and decided:

- Whether “object detection and tracking” can be the focus of the SI, excluding environment and motion monitoring.

- Within “object detection and tracking”, which of the following (and/or others) should be included explicitly:

  - UAV detection and tracking

  - Indoor detection of humans (and in some cases animals)

  - Outdoor automotive cases

  - Automated guided vehicles (e.g. in indoor factories)

  - Unspecified objects on railways

- Whether the set of six possible sensing modes can be narrowed down for the purpose of the RAN1-led SI.

- Whether frequencies from 0.5 to 52.6 GHz could be the primary focus, with the assumption that a similar modelling approach should scale to 100GHz.

# References

- RP-232617	Moderator's summary for REL-19 RAN1 topic ISAC & Exploring Study in New Spectrum (7-24GHz)	Nokia

- RP-232745	Summary for RAN Rel-19 Package: RAN1/2/3-led	RAN chair (Qualcomm)

- RP-233100	New SID: Study on Integrated sensing and communication (ISAC) for NR	Nokia (moderator)

- RP-232751	On ISAC	Ericsson

- RP-232756	Rel-19 Study on ISAC	Lekha Wireless Solutions

- RP-232764	Consideration on Rel-19 Channel Modeling of ISAC	New H3C Technologies Co., Ltd.

- RP-232872	Views on ISAC study in Rel-19	NEC

- RP-232905	Discussion on channel modelling for ISAC in Rel-19	OPPO

- RP-232980	Channel modeling for ISAC in Rel-19	Spreadtrum Communications

- RP-233017	Consideration on study for ISAC	CATT

- RP-233047	Considerations on Rel-19 ISAC	Ruijie Network Co. Ltd

- RP-233054	Views on Rel-19 channel modeling for ISAC	vivo

- RP-233082	Study on channel models with ray tracing for integrated sensing and communication (ISAC)	NVIDIA

- RP-233157	Proposals on channel modeling for ISAC in Rel-19	LG Electronics

- RP-233271	Rel-19 ISAC: Scope	MediaTek Inc.

- RP-233286	Discussion of possible scope of SI on channel modelling for ISAC	Nokia, Nokia Shanghai Bell

- RP-233295	Scope of Integrated Sensing and Communications (ISAC) in Rel-19	Robert Bosch GmbH

- RP-233326	Discussion on Rel-19 Channel Modeling for ISAC	Intel Corporation

- RP-233381	Views on ISAC studies in Rel-19	Qualcomm Incorporated

- RP-233384	Views on Integrated Sensing and Communications for Rel-19	Apple

- RP-233401	Views on channel modelling for ISAC	InterDigital, Inc.

- RP-233423	Views on Rel-19 study on integrated sensing and communication (ISAC)	CMCC

- RP-233472	Scope on ISAC channel model	Samsung

- RP-233487	Use cases for ISAC channel modeling	Xiaomi, CAICT, China Unicom, Deutsche Telekom, Bell Mobility, TELUS, Spark NZ Ltd, ABS, Rakuten Mobile, LG Uplus, KT Corp., Reliance Jio, KPN, TCL, Inspur, Google, Nvidia, Denso, ITRI, Spreadtrum, Fraunhofer IIS, CEWiT, New H3C, ETRI, Meta, Toyota ITC, Sharp, Lenovo, UESTC, Rohde & Schwarz

- RP-233506	Study on Integrated Sensing and Communications	CEWiT

- RP-233538	Study on channel model for ISAC	NTT DOCOMO, INC.

- RP-233574	Views on ISAC channel modeling and following work in Rel-19	China Telecom, Lenovo, H3C

- RP-233598	Consideration on the Additional Techniques of Integrating Sensing and Communication	China Unicom

- RP-233614	Views on ISAC channel modeling in Rel-19	ZTE, Sanechips

- RP-233663	Integrated Sensing and Communications in Rel-19	Huawei, HiSilicon, CAICT

- RP-233862	Views on Rel-19 ISAC Study Item(s)	AT&T, FirstNet