3GPP TSG RAN meeting #102			 				                                RP-233170

Edinburgh, Scotland, December 11-15, 2023

## Status Report to TSG

Agenda item:			9.2.6

| WI / SI Name | Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No | Testing part: No |
| Acronym | FS_NR_FR2_OTA_enh |  |  |  |
| Unique ID | 950068 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-230814 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2023 | Core part: N/A | Performance part: N/A | Testing part: N/A |
| Overall Completion level | Study Item: 100% | Core part: N/A | Performance Part: N/A | Testing part: N/A |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Bin Han |
|  | Company | Qualcomm Incorporated |
|  | Email | binhan@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### Void

## 2.2	RAN2

#### Void

## 2.3	RAN3

#### Void

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#109:

The approved TPs and agreed draft TR are listed in the below table.

| T-doc number | Title | Company | Decision |
| --- | --- | --- | --- |
| R4-2320391 | 3GPP TR 38.871 v0.6.0 | Qualcomm Incorporated | Agreed |
| R4-2321102 | TP to TR38.871 for test procedure of UE RF multi-Rx | OPPO, Samsung | Approved |
| R4-2321101 | TP to TR 38.871 for UE coordinate system | OPPO, Keysight Technologies, Rohde & Schwarz, Qualcomm Incorporated | Approved |
| R4-2320390 | TP to TR 38.871 on draft summary and editorial changes | Qualcomm Incorporated | Approved |
| R4-2321103 | TP to TR 38.871 on step size of measurement grid | Qualcomm Incorporated | Approved |
| R4-2321104 | TP to TR 38.871 on RRM test method | Qualcomm Incorporated, Huawei, HiSilicon | Approved |
| R4-2321105 | TP to TR 38.871 on Demodulation test method | Qualcomm Incorporated | Approved |
| R4-2321106 | TP to TR 38.871 on MU assessment | Qualcomm Incorporated, Huawei, HiSilicon, vivo | Approved |

Agreements for test method for UE RF

Step size of the measurement grid

- Agreement:

  - Take the 10deg and 15deg as the starting point of step size for measurement grid.

Starting of UE orientations for multi-Rx UE RF testing

- Agreement:

  - Fix 0°≤ φ≤180° in the testing and add following additional alignment option should be considered on top of UE alignments and orientations in Table 1.2.2-1. The total starting UE orientations is 12.

  - The starting of UE alignments should be declared by UE vendor for the testing.

  - Include the current figure into the TP with a note that the figure will be updated in maintenance.  The figure can then be updated in the next meeting.

Positioner blocking

- Agreement:

  - RAN4 to adopt re-positioning for multi-Rx measurement to avoid AoA blockage. The following Alt 1 could be considered as the baseline.

    - Alt 1: The initial positioner/UE orientation is selected to be (-90° - ½ angular separation). The minimum angular separation between positioner and probe(s) are listed below.

| Declared Angular Separation [°] | Min Angular Separation between Positioner and Probe(s) [°] |
| --- | --- |
| 30 | 75 |
| 60 | 60 |
| 90 | 45 |
| 120 | 30 |
| 150 | 15 |

    - Details of re-positioning concept are TBD

Test procedure

- Agreements:

  - The test procedure of Introduces the 2-DL MIMO signals with TRP1&TRP2 simultaneously is agreed as the baseline. Capture the test procedure in TR 38.871.

Agreements for test method for UE RRM

Measurement setup for Dual DCI switching (i.e., Category 2)

- Agreements

  - Proposal 1 and proposal 2 in Issue 2-1-1 of [41] issues are agreed as the measurement setup for Dual TCI switching

  - To capture both two proposals in TR 38.871.

  - The illustration of test procedure shown in Figure 2.2.1-1 [41] is an example. It is not precluded for other test procedure based on 3 probe measurement setup.

Side condition for Dual TCI switching

- Agreements

  - In the period of T1, the candidate test direction is selected from legacy EIS spherical coverage. In the period of T2, the candidate test directions (AoA pairs) are selected from multi-Rx spherical coverage requirements defined in UE RF session.

Agreements for test method for UE Demodulation

Minimum isolation

- Agreements:

  - Keep -[12dB] as the starting point for the min. isolation requirements in RAN4.

  - RAN4 recommended the target of SNR delta, i.e., MU due to non-ideal isolation is 1dB.

  - The final minimum isolation and corresponding MU defer the discussion to RAN5.

  - OEMs are encouraged to provide simulation results for the number of feasible AoA pair with different min. isolation assumptions.

Agreements for MU assessment

The value of X%

- Agreements:

  - For arithmetic mean combining, take X% = 4% as starting point.

The value of Y%

- Agreements:

  - Agree an average value of 2.3% as the starting point

RAN4#108bis:

The approved TPs are listed in the below table.

| T-doc number | Title | Company | Decision |
| --- | --- | --- | --- |
| R4-2316948 | TP on TR 38.871 for RRM test method | Qualcomm Incorporated | Approved |
| R4-2316949 | TP on TR 38.871 for Demodulation test method | Qualcomm Incorporated | Approved |

Agreements for test method for UE RF

Step size of the measurement grid:

- Agreement:

  - Take the 10deg and 15deg as the starting point of step size for measurement grid.

  - Encourage companies to evaluate the worst case antenna configuration of 6x2 for further analysis.

MU framework of measurement grid:

- Agreement:

  - The percentage value could be considered as the final metric for 2AoA UE RF MU analysis.

  - The following tables are agreed as the baseline of the Multi-Rx UE RF testing MU framework. Assume the values in the table to be in [] which could be further revisited.

Table 1: Uncertainty assessment for wanted DL signal absolute power in 2AoA coverage measurement with IFF

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | [0.00] |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | [0.00] |
| 3 | Quality of Quiet Zone (NOTE 7) | 0.6 | Actual | 1.00 | [0.6] |
| 4 | Mismatch | 1.30 | Actual | 1.00 | [1.30] |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | [1.45] |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | [0.00] |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | [1.05] |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | [0.25] |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | [0.00] |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | [0.00] |
| 13 | Multiple measurement antenna uncertainty (NOTE 6) | 0.15 | Actual | 1.00 | [0.15] |
| 14 | DUT repositioning | 0.00 | Rectangular | 1.73 | [0.00] |
| 15 | Influence of spherical coverage grid (NOTE 4) | 0.12 | Actual | 1 | [0.12] |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 16 | Mismatch | 0.00 | U-shaped | 1.41 | [0.00] |
| 17 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | [0.00] |
| 18 | Misalignment of positioning System | 0.00 | Normal | 2.00 | [0.00] |
| 19 | Uncertainty of the Network Analyzer | 1.50 | Normal | 2.00 | [0.75] |
| 20 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | [0.30] |
| 21 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | [0.00] |
| 22 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | [0.00] |
| 23 | Quality of quiet zone for calibration process (NOTE 7) | 0.4 | Actual | 1.00 | [0.4] |
| 24 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 25 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | [0.07] |
| 26 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
|  | Measurement uncertainty |  |  |  | Value |
| Wanted DL signal absolute power (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.86] |

Table 2: Total uncertainty assessment for 2AoA coverage measurement with IFF

| Measurement uncertainty | Value |
| --- | --- |
| Wanted DL signal absolute power (1.96σ - confidence interval of 95 %) [%] | X% |
| Uncertainty related to measurement grid | Y% |
| Total Measurement uncertainty | Value |
| [2AoA spherical coverage] expanded uncertainty (1.96σ - confidence interval of 95 %) [%] | X+Y% |
| NOTE 1: X% is derived based on the simulations with different DL power vs percentage of 2AoA metric. NOTE 2: Y% is derived based on the simulations with measurement step size vs percentage of 2AoA metric. |  |

Other aspects of measurement grid analysis:

- Agreement:

  - MU is derived at candidate declared AoA offsets and UE orientations with the biggest delta compared to the 1deg step size.

    - MU derived at standardized single AoA offset is not precluded depending on RF session agreement

Test procedure:

- Agreement

  - The test procedure should follow the assumptions for the simulation which did not consider the connection sequence. Further updates can be made to capture the progress in RF requirements.

UE orientations/alignment options for multi-Rx RF testing:

- Agreement

  - Further discussions are needed in next meeting:

    - On the definition of UE orientation/alignment options for multi-RX RF testing.

      - Based on the decisions in RF core requirements session, the 9 orientations in J.3.X of TS 38.101-2 need to be considered as starting orientation for testing.

    - Further study whether the re-positioning concept must be adapted for multi-RX

    - The position of AoA1 with respect to the coordinate system in the starting orientation (theta=0, phi=0) can be further studied

Agreements for test method for UE RRM.

RRM testing scenarios

- Agreement:

  - RAN4 to discuss the measurement setup of multi-Rx RRM testing with the following two Categories:

    - Category 1: All Multi-Rx RRM test cases expect Dual TCI switching

    - Category 2: Dual TCI switching test case for Multi-Rx RRM

  - The above is agreed for facilitating the measurement setup discussion for multi-Rx RRM testing

Measurement setup for Category 1 scenario

- Agreement:

  - The measurement setup shown in below figure is the baseline for Category 1 scenario RRM testing.

  - This completes Category 1 scenario RRM testing.

Measurement setup for Category 2 scenario (Dual TCI switching)

- Agreement

  - Option 1:

    - This option implies the implementation of two of the same AoA offsets from RF session. This typically prevents the re-use of existing test systems. This option is recommended to be deprioritized from TE perspective.

  - Option 2:

    - This option should be prioritized from TE perspective.

  - Option 3/3a:

    - This option implies a perfect polarization alignment between DUT and TE in order to separate the beams, which is not feasible form TE perspective based on current test systems. This option is de-prioritized from TE perspective.

  - Capture all the candidate options into TR38.871 and list the pros and cons.

Agreements for test method for UE Demodulation

X value in Noc level configuration

- Agreement:

  - RAN4 to adopt X = [3] as the allowable degradation from legacy REFSENS requirements for Multi-Rx demodulation test directions selection.

Assumptions for minimum isolation simulation

- Agreement:

  - The following assumptions are adopted for simulation of minimum isolation requirements.

    - For the reference required SNR, the following assumptions could be considered:

      - Assume α = 0, β = 0, and ρ=γ = [-100, -15, -12]dB emulating by channel emulator

      - Channel model parameters

      - TDLA30-75 is assumed for 100 MHz/120 kHz

      - Time offset values: 0; Frequency offset: 0

      - MCS: MCS17 with 1+1

      - Receiver assumptions: Separate processing per Rx chain.

    - For comparison, required SNR from the testing is obtained by the simulations with considering the additional crosstalk from both two TRPs/AoAs and two polarizations introduced by TE. The candidate isolations from TE is within the range of [-8dB, 20dB]

    - Compare the offset between reference required SNR and required SNR from testing.

Agreements for MU assessment

MU assessment for UE RF testing

- Agreements

  - Agree the following tables are agreed as the baseline in square brackets

Table 3: Uncertainty assessment for wanted DL signal absolute power in 2AoA coverage measurement with IFF

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | [0.00] |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | [0.00] |
| 3 | Quality of Quiet Zone (NOTE 7) | 0.6 | Actual | 1.00 | [0.6] |
| 4 | Mismatch | 1.30 | Actual | 1.00 | [1.30] |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | [1.45] |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | [0.00] |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | [1.05] |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | [0.25] |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | [0.00] |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | [0.00] |
| 13 | Multiple measurement antenna uncertainty (NOTE 6) | 0.15 | Actual | 1.00 | [0.15] |
| 14 | DUT repositioning | 0.00 | Rectangular | 1.73 | [0.00] |
| 15 | Influence of spherical coverage grid (NOTE 4) | 0.12 | Actual | 1 | [0.12] |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 16 | Mismatch | 0.00 | U-shaped | 1.41 | [0.00] |
| 17 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | [0.00] |
| 18 | Misalignment of positioning System | 0.00 | Normal | 2.00 | [0.00] |
| 19 | Uncertainty of the Network Analyzer | 1.50 | Normal | 2.00 | [0.75] |
| 20 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | [0.30] |
| 21 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | [0.00] |
| 22 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | [0.00] |
| 23 | Quality of quiet zone for calibration process (NOTE 7) | 0.4 | Actual | 1.00 | [0.4] |
| 24 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 25 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | [0.07] |
| 26 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
|  | Measurement uncertainty |  |  |  | Value |
| Wanted DL signal absolute power (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.86] |

Table 4: Total uncertainty assessment for 2AoA coverage measurement with IFF

| Measurement uncertainty | Value |
| --- | --- |
| Wanted DL signal absolute power (1.96σ - confidence interval of 95 %) [%] | X% |
| Uncertainty related to measurement grid | Y% |
| Total Measurement uncertainty | Value |
| [2AoA spherical coverage] expanded uncertainty (1.96σ - confidence interval of 95 %) [%] | X+Y% |
| NOTE 1: X% is derived based on the simulations with different DL power vs percentage of 2AoA metric. NOTE 2: Y% is derived based on the simulations with measurement step size vs percentage of 2AoA metric. |  |

MU assessment for UE RRM testing

- Agreement:

  - Agree the following table as the baseline in square brackets

Table 5: Uncertainty assessment for Multi-Rx RRM testing with IFF

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | [0.00] |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | [0.00] |
| 3 | Quality of Quiet Zone (NOTE 4) | 0.6 | Actual | 1.00 | [0.6] |
| 4 | Mismatch | 1.30 | Actual | 1.00 | [1.30] |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | [1.45] |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | [0.00] |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | [1.05] |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | [0.25] |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | [0.00] |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | [0.00] |
| 13 | Multiple measurement antenna uncertainty (NOTE 3) | 0.15 | Actual | 1.00 | [0.15] |
| 14 | DUT repositioning | 0.08 | Rectangular | 1.73 | [0.05] |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 | Mismatch | 0.00 | U-shaped | 1.41 | [0.00] |
| 16 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | [0.00] |
| 17 | Misalignment of positioning System | 0.00 | Normal | 2.00 | [0.00] |
| 18 | Uncertainty of the Network Analyzer | 0.73 | Normal | 2.00 | [0.37] |
| 19 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | [0.30] |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | [0.00] |
| 21 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | [0.00] |
| 22 | Quality of quiet zone for calibration process (NOTE 4) | 0.4 | Actual | 1.00 | [0.4] |
| 23 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 24 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | [0.07] |
| 25 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
|  | Systematic uncertainties (NOTE 2) |  |  |  | Value |
| 26 | Systematic error related to beam peak search |  |  |  | [0.5] |
| Total measurement uncertainty |  |  |  |  | Value |
| DL AWGN absolute power expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [5.19] |
| NOTE 1: The analysis was done only for the case of operating in-band, non-CA. NOTE 2: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 3: Applies to the system which has a structure of mechanical feed antenna positioning. NOTE 4: Value based on procedure defined in Annex D.2 of TR 38.810 [13] for Quiet Zone size less or equal to 30 cm. NOTE 5: The values in this table have been derived for DL powers above and equal to REFSENS. The values might need to be revisited for power levels below REFSENS |  |  |  |  |  |

MU assessment for UE demodulation testing

- Agreement:

  - Agree the following table as the baseline in square brackets

Table 4.2.3-1: Uncertainty assessment for Multi-Rx demodulation testing with IFF

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Signal-to-noise ratio uncertainty |  |  |  |  |  |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment |  | [Normal] | [2.00] |  |
| 2 | Measure distance uncertainty |  | [Rectangular] | [1.73] |  |
| 3 | Quality of Quiet Zone |  | [Actual] | [1.00] |  |
| 4 | Mismatch |  | [Actual] | [1.00] |  |
| 5 | Standing wave between the DUT and measurement antenna |  | [U-shaped] | [1.41] |  |
| 6 | gNB emulator SNR uncertainty |  | [Normal] | [2.00] |  |
| 7 | Phase curvature |  | [U-shaped] | [1.41] |  |
| 8 | Amplifier uncertainties |  | [Normal] | [2.00] |  |
| 9 | Random uncertainty |  | [Normal] | [2.00] |  |
| 10 | Influence of the XPD |  | [U-shaped] | [1.41] |  |
| 11 | Insertion Loss Variation |  | [Rectangular] | [1.73] |  |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) |  | [Actual] | [1.00] |  |
| 13 | Multiple measurement antenna uncertainty |  | [Actual] | [1.00] |  |
| 14 | DUT repositioning |  | [Rectangular] | [1.73] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 | Mismatch |  | [U-shaped] | [1.41] |  |
| 16 | Amplifier Uncertainties |  | [Normal] | [2.00] |  |
| 17 | Misalignment of positioning System |  | [Normal] | [2.00] |  |
| 18 | Uncertainty of the Network Analyzer |  | [Normal] | [2.00] |  |
| 19 | Uncertainty of the absolute gain of the calibration antenna |  | [Normal] | [2.00] |  |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna |  | [Rectangular] | [1.73] |  |
| 21 | Phase centre offset of calibration antenna |  | [Rectangular] | [1.73] |  |
| 22 | Quality of quiet zone for calibration process |  | [Actual] | [1.00] |  |
| 23 | Standing wave between reference calibration antenna and measurement antenna |  | [U-shaped] | [1.41] |  |
| 24 | Influence of the calibration antenna feed cable |  | [Normal] | [2.00] |  |
| 25 | Insertion Loss Variation |  | [Rectangular] | [1.73] |  |
|  | Systematic uncertainties |  |  |  | Value |
| 26 | Impact on non-ideal isolation between branches for the wireless cable mode |  |  |  | FFS (Note 1) |
| Total Signal-to-Noise ratio uncertainty |  |  |  |  |  |
| Other contributors affecting test result |  |  |  |  |  |
| 27 | gNB emulator fading model impairments |  | [Normal] | [2.00] |  |
| 28 | AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig (Note 3) |  | [Actual] | 1.00 |  |
| 29 | Result variation due to finite test time |  | [Actual] | [1.00] |  |
| Note 1: FFS which is relying on the min. isolation requirements. |  |  |  |  |  |

#### Remaining Open issues

No remaining open issues

## 2.5	RAN5

#### Void

## 2.6	RAN6

#### Void

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A.

## 4.	References

RAN4#108bis

- R4-2316220, TP for UE coordinate system, OPPO

- R4-2315554, Simulation and discussion on 2AoA spherical coverage measurement grid, Samsung

- R4-2315817, Discussion on FR2 multi-Rx measurement grid, vivo

- R4-2316221, Update of test procedure, OPPO

- R4-2316507, Discussion on RF test method for FR2 multi-Rx UE, Qualcomm Incorporated

- R4-2316703, Discussion on measurement grids of FR2 2AoA, CAICT

- R4-2315569, Views on FR2 2AoA RRM test setup, Anritsu Corporation

- R4-2316377, Discussion on Test method for UE RRM, Qualcomm Incorporated

- R4-2316948, TP on TR 38.871 for RRM test method, Qualcomm Incorporated

- R4-2316378, Discussion on Test method for UE Demodulation, Huawei, HiSilicon

- R4-2316509, Discussion on demodulation test method for FR2 multi-Rx UE, Qualcomm Incorporated

- R4-2316949, TP on TR 38.871 for Demodulation test method, Qualcomm Incorporated

- R4-2316222, MU analysis of measurement grid for 2AoA reception, OPPO

- R4-2316321, On Multi-RX UE demod isolation simulations, Keysight

- R4-2316510, Discussion on MU assessment for FR2 multi-Rx UE test methodology, Qualcomm Incorporated

- R4-2317960, Topic summary for [108bis][329] FS_NR_FR2_OTA_enh, Moderator (Qualcomm)

- R4-2316927, Ad-hoc meeting minutes for FS_NR_FR2_OTA_enh, Qualcomm

- R4-2317002, WF on FS_NR_FR2_OTA_en, Qualcomm

RAN4#109

- R4-2321102, TP to TR38.871 for test procedure of UE RF multi-Rx, OPPO, Samsung

- R4-2321101, TP to TR 38.871 for UE coordinate system, OPPO

- R4-2320390, TP to TR 38.871 on draft summary and editorial changes, Qualcomm Incorporated

- R4-2320391, 3GPP TR 38.871 v0.6.0, Qualcomm Incorporated

- R4-2320408, TP on TR 38.871 on RRM Measurement uncertainty, Huawei, HiSilicon

- R4-2318838, On Multi-RX UE RF Testing and Positioning Topics, Keysight Technologies UK Ltd

- R4-2318986, Discussion on FR2 multi-Rx RF test method, vivo

- R4-2319268, Discussion on 2AoA spherical coverage measurement grid and test procedure, Samsung

- R4-2319917, Discussion on the re-positioning for multi-Rx, OPPO

- R4-2319918, Further discussion on Multi-Rx test procedure, OPPO

- R4-2320382, Views on RF test method for FR2 multi-Rx UE, Qualcomm Incorporated

- R4-2321103, TP to TR 38.871 on step size of measurement grid, Qualcomm Incorporated

- R4-2320411, Discussion on Test method for UE RF, Huawei,HiSilicon

- R4-2320383, Views on RRM test method for FR2 multi-Rx UE, Qualcomm Incorporated

- R4-2321104, TP to TR 38.871 on RRM test method, Qualcomm Incorporated, Huawei, HiSilicon

- R4-2320410, Discussion on Test method for UE RRM, Huawei,HiSilicon

- R4-2318837, On Multi-RX UE Demod Isolation MU, Keysight Technologies UK Ltd

- R4-2320384, Views on demodulation test method for FR2 multi-Rx UE, Qualcomm Incorporated

- R4-2321105, TP to TR 38.871 on Demodulation test method, Qualcomm Incorporated

- R4-2318987, Preliminary MU assessment for multi-Rx RF testing, vivo

- R4-2320385, MU assessment for FR2 multi-Rx UE test methodology, Qualcomm Incorporated

- R4-2321106, TP to TR 38.871 on MU assessment, Qualcomm Incorporated, Huawei HiSilicon, vivo

- R4-2318226, Topic summary for [109][334] FS_NR_FR2_OTA_enh, Moderator (Qualcomm)

- R4-2321093, Ad-hoc meeting minutes for [109][334] FS_NR_FR2_OTA_enh