3GPP TSG RAN Meeting #102	RP-233267

Edinburgh, Scotland, December 11th – 15th, 2023

Source: 	MediaTek Inc.

Title: 	Rel-19 Duplex Evolution: Scope (gNB SBFD)

Agenda items: 	9.1.1.3

Document for: 	Discussion and Decision

# Introduction

A study item on evolution of NR duplex operation was approved in RAN#94e [1]. In this contribution, we discuss the scope for the WI phase.

# Discussion

A new SI on evolution of NR duplex operation was approved at RAN#94-e to identify the feasibility and performance of SBFD in NR. RAN1 and RAN4 have concluded the study and TR 38.858 is submitted for approval [2] at RAN#102. In RAN#101, there was a discussion on the scope of R19 WI on gNB-SBFD, and the following potential objectives are further provided as the baseline for further discussion in RAN#102 [3].

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier - [Semi-static/dynamic] indication of time location of SBFD subbands to UEs [in RRC_CONNECTED mode] - Semi-static indication of frequency domain location of SBFD subbands to UEs [in RRC_CONNECTED mode] - UE transmission and reception behavior and procedures in SBFD symbols and/or non-SBFD symbols - Note: followings are assumed based on TR 38.858 - SBFD operation Option 4 - Coexistence between legacy UEs and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - Up to one UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - At least adjacent channel coexistence between two operators should be considered as a minimum. - Specify enhancements for CLI handling: - Support both the gNB-to-gNB co-channel CLI handling scheme(s) and UE-to-UE co-channel CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858) - The SBFD operation drives the CLI enhancements, which are expected to be applicable to the dynamic/flexible TDD operation but without dedicated optimization - RF requirements for SBFD operation at gNB

In this following, we provide our views on the priorities of some of the objectives.

## Inter-UE and Inter-gNB CLIs

In legacy TDD, UL and DL transmissions take place in distinct time slots, meaning there is no risk of interference between UL and DL signals from neighbouring UEs/gNBs. However, with SBFD, UL and DL transmissions occur during the same time slot at the gNB, leading to:

- Inter-UE CLI, as the UL transmission of one UE negatively affects the reception of DL signal at another UE.

- Inter-gNB CLI, as the DL transmission of one gNB negatively affects the reception of UL signal at another gNB.

Figure 1: Inter-UE and inter-gNB CLI in SBFD scenarios.

Figure 2 shows the impact of inter-UE CLI on DL spectral efficiency (SE). As illustrated the figure, DL SE can be reduced significantly (~19%) in the presence of inter-UE CLI. Therefore, efficient CLI handling mechanisms should be employed to mitigate the negative impact of inter-UE CLI. One such approach is CLI-aware scheduling, which schedules UE-pairs based on the expected level of inter-UE CLI. That is, UEs that cause severe CLI towards each other are not scheduled together. As shown in Figure 2 the negative impact of inter-UE CLI on DL SE can be completely reversed through CLI-aware scheduling. Here, it is assumed that the gNB has perfect knowledge of inter-UE CLI and can form UE-pairs to completely avoid its impact. However, in practice, the effectiveness of CLI handling schemes, including CLI-aware scheduling, relies on the accuracy of the knowledge of inter-UE CLI at the gNB.

Therefore, the WI should prioritize the work on specifying inter-UE CLI measurement and reporting schemes.

Observation: Inter-UE CLI has significant impact to the DL performance. CLI handling mechanisms, such as CLI-aware scheduling, can significantly improve system performance in the presence of inter-UE CLI.

Proposal: For the gNB-SBFD WI, RAN1 should prioritize the work on inter-UE CLI measurement and reporting.

Figure 2: DL SE performance in the presence of inter-UE CLI.

## PHY channels/procedures enhancements

During the study phase, the following assumption were considered as the baseline for SBFD operation. These assumptions should be the basis for the WI phase.

- SBFD at the gNB with half-duplex UEs - SBFD operation within a single TDD carrier with a single DL/UL BWP pair aligned in centre frequencies - Both time and frequency locations of subbands for SBFD operation are known to SBFD-aware UEs

In addition, during the study phase, several signalling enhancements were identified for gNB-SBFD operation. For this part of the objective, the WI should focus on enabling the transmission and reception across SBFD symbols and non-SBFD symbols in different slots.

Enhancements on physical channels and procedures across SBFD symbols and non-SBFD symbols in different slots, - Separate configurations for SRS, SR, PRACH, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters - Resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - Note: each transmission/reception within a slot has either all SBFD or all non-SBFD symbols as follows

# Conclusion

In this contribution, we discussed views on the priorities of some of the objectives, and we have the following proposal.

Proposal: For the gNB-SBFD WI, the following the objectives should be prioritized:

- Enhancements for inter-UE and inter-gNB CLI handling schemes: - Enhancements on physical channels and procedures across SBFD symbols and non-SBFD symbols in different slots, - Separate configurations for SRS, SR, PRACH, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters - Resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - Note: each transmission/reception within a slot has either all SBFD or all non-SBFD symbols as follows

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation,” CMCC, RAN#94e, December 2021.

- RP-232763, TR 38.858 v2.0.0 for the Study on evolution of NR duplex operation, CMCC.

- RP-232745, “Summary for RAN Rel-19 Package: RAN1/2/3-led,” RAN Chair, RAN#102, December 2023.