3GPP TSG RAN Meeting #102	RP-233295

Edinburgh, Scotland, December 11th-15th, 2023

Source: 	Robert Bosch GmbH

Title: 	Scope of Integrated Sensing and Communications (ISAC) in Rel-19

Agenda item:	9.1.1.7

Document for:	Discussion and Decision

# Introduction

It has been recognized by several companies during the recent 3GPP meetings in various Working Groups that Integrated Sensing and Communication (ISAC) could be one of the important features to be studied and specified in the forthcoming releases. 3GPP SA1 has recently concluded the first study/work items (FS_Sensing/Sensing). During the RAN Rel-19 Workshop in June 2023, it was already recognized by various companies that the new feature of ISAC requires extensive studies, as captured by the RAN Chairman in [1]. Further during RAN#101 in September 2023, there were more contributions from companies, and the resulting discussions were summarized by the moderator in [2]. The following was described as the possible way forward.

Regarding ISAC, the following way forward seems agreeable [2]: - If not already done in SID, a RAN-led study objective to select a small number of ISAC use cases from TR22.837, and to identify appropriate corresponding information needed for point 2 below, i.e. scenarios, frequency ranges and sensing modes. - A RAN1-led study objective to define channel modelling details for sensing using 38.901 (if applicable to the selected use cases) as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets, including, for example (if needed by the selected use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. This objective to start after completion of the first (RAN-led) objective if not already done in SID.

Observation 1: RAN plans to select representative use cases from the existing study in SA1 (TR 22.837) to identify the necessary attributes required by RAN-1 to study ISAC.

Since ISAC is a novel feature in 3GPP, parallel works in plenary and Working Groups need to be avoided to ensure that sufficient maturity is achieved for the feature at each stage of the study.

Proposal 1: RAN is requested to agree that discussions on possible Study Items in the WGs (RAN1, RAN3, etc). are to be considered only after the representative use cases, scenarios, etc. have been identified by the RAN-led study objective.

# Discussion

## Existing works in 3GPP SA1

The existing works in SA1 have been documented TR 22.837 and TS 22.137, including 32 use cases and several sensing-related performance requirements and KPIs. The described use cases have been divided into the following general scenarios, which can be utilized for use case prioritization in RAN.

## Use case categories in SA1

- Object Detection and Tracking (with 4 sensing service (sub-)categories in Rel-19).

  - Example use cases include detection of humans/UAVs in the categories involving lower KPI requirements, and tracking of humans/UAVs with higher KPI requirements.

- Environment Monitoring (with 1 service category in Rel-19).

  - For example, rainfall monitoring.

- Motion Monitoring (with 2 sensing service (sub-)categories in Rel-19).

  - For example, human motions or gesture recognition.

Observation 2: SA1 has divided the use cases into three general scenarios; Object Detection and Tracking, Environment Monitoring, and Motion Monitoring.

## Sensing Performance Requirements in SA1

SA1 has summarized the following KPI table for the use case scenarios to be considered in Rel-19. Each use case scenario is provided with target performance requirements such as accuracy of positioning estimate, sensing latency, false alarm probability, and so on.

Table 1: Sensing KPI table as defined by SA1 (Table 6.2-1 in TS 22.137)

Observation 3: SA1 has summarized the performance requirements of the three general use case scenarios into a KPI table (Table 6.2-1 in TS 22.137).

It can be observed in Table 1 that the use cases described in SA1 consider vastly different performance requirements based on need, with some requiring high-performance and low latency (tracking UAVs), whereas some other use cases like rainfall monitoring may require considerably lower requirements. Since each use case scenario as defined by SA1 provides a distinct set of use cases with varied sets of KPI performance requirements, the existing categorization can be utilized by RAN in the process of the use case prioritization for further study. Since most use cases belong to the Object Detection and Tracking Scenario (1), one or two use cases from Scenario 1 could be prioritized. Since the Environment Monitoring Scenario (2) considers varied requirements different from Scenario (1), it may be included to increase the scope of the use cases, if required. However, it must be ensured that the scope of the study is limited.

Observation 4: The use case scenarios as defined by SA1 have vastly diverse sets of performance requirements, which need to be accounted for during the RAN study.

Proposal 2: RAN is requested that the scope of the prioritized use cases is kept limited in the first phase of discussions. It is proposed to prioritize at least one case or two use cases from Scenario 1 (Object Detection and Tracking) as defined by SA1. If required, Scenario 2 (Environment Monitoring) may be included to increase the scope of the study.

## Sensing modes

To perform sensing, depending on the sensing transmitting entity and the receiving entity, the following sensing modes could be developed by 3GPP as shown in Figure 1. With the combination of mono-static sensing (same transmitting and receiving entity), and bi-static sensing (transmitting enity and receiving entity are different), six sensing modes could be envisioned as a starting point.

Possible sensing modes could be as follows.

With gNodeB as the transmitting entity

- Mode 1 (mono-static, i.e. gNodeB-1 to gNodeB-1)

- Mode 2 (bi-static, with another gNodeB as receiving entity, i.e. gNodeB-1 to gNodeB-2)

- Mode 3 (bi-static, with a UE as receiving entity, i.e. gNode-1 to UE-1)

With UE as the transmitting entity

- Mode 4 (mono-static, i.e. UE-1 to UE-1)

- Mode 5 (bi-static, with another UE as receiving entity, i.e. UE-1 to UE-2)

- Mode 6 (bi-static, with a gNodeB as receiving entity, i.e. UE-1 to gNode-1)

Figure 1: Possible sensing modes in 3GPP

Observation 5: Six sensing modes may be developed initially, with combinations of gNodeBs and UEs performing mono-static or bi-static sensing.

It has been observed in previous discussions, that companies use differing nomenclature while describing the same sensing modes as described above. Therefore, to avoid confusion, it is necessary that the sensing modes use a single naming convention to be agreed.

Proposal 3: RAN is requested to agree on a unified naming convention for properly identifying the sensing modes involving gNodeBs and UEs in subsequent discussions in 3GPP.

  - Channel modeling

The existing channel models in TR 38.901 are currently applicable for modeling communication performance. It has been noted that in order to evaluate the performance of new ISAC features, extensions to the existing channel models or new channel models appropriate for sensing is necessary. The sensing channel models should also consider attributes related to the physical characteristics of the target sensing object, such as its Radar Cross Section (RCS), mobility of the object, as well as the characteristics of the (unintended for sensing) objects in the environment which may hinder the sensing performance due to blockage, introduction of more multipath components to the reflected signal, and so on. However, the discussed sensing-related parameters are currently not defined in 3GPP. It is currently also not clear, which level of detailing (number of parameters, and which parameters) is sufficient to model a sensing system which can be used for benchmarking solutions in the downstream WGs.

Observation 6: Although initial discussions have proposed some necessary parameters (RCS, mobility, etc.) to be included in the sensing channel models, the mentioned sensing-related parameters are currently not described in 3GPP.

Proposal 4: RAN is requested to agree on a (minimum) set of sensing-related parameters for channel modeling, which may be defined/utilized by downstream WGs.

# Remarks on proposals for normative works in Rel-19

As the initial discussions in RAN (and also in SA/SA2) have showcased, ISAC is a novel feature in 3GPP which requires careful evolution of several aspects which have traditionally not been considered by 3GPP (e.g. channel models). Although the first set of use cases and requirements have been described by SA1, given the low maturity of ISAC-related concepts in 3GPP, sufficient time must be allocated to mature the discussions in the relevant working groups. As described by RAN chair in [3] further aspects beyond channel modeling may be considered in Sept’24, which can introduce additional workload. Therefore, it is imperative that there is clarity on the time units being allocated for the ISAC working scope in Rel-19 fully for a Study Item. For the same reasons, it may also be required to avoid parallel work items in different WGs from the beginning.

Observation 7: Since ISAC is a new feature with low maturity in 3GPP, sufficient time is necessary to adequately study the required enhancements to the 5GS.

Observation 8: Further ISAC sub-topics have already been mentioned as possible candidates to be considered in Sep’24, which potentially increases the workload.

Proposal 5: RAN is requested to agree that the Rel-19 package for ISAC considers only a Study Item for RAN1 to obtain sufficient time for the study, and to avoid further discussions on ISAC normative work.

# Conclusions

In this contribution, the initial scope for studying ISAC in RAN has been discussed, with the following observations and proposals.

Observation 1: RAN plans to select representative use cases from the existing study in SA1 (TR 22.837) to identify the necessary attributes required by RAN-1 to study ISAC.

Observation 2: SA1 has divided the use cases into three general scenarios; Object Detection and Tracking, Environment Monitoring, and Motion Monitoring.

Observation 3: SA1 has summarized the performance requirements of the three general use case scenarios into a KPI table (Table 6.2-1 in TS 22.137).

Observation 4: The use case scenarios as defined by SA1 have vastly diverse sets of performance requirements, which need to be accounted for during the RAN study.

Observation 5: Six sensing modes may be developed initially, with combinations of gNodeBs and UEs performing mono-static or bi-static sensing.

Observation 6: Although initial discussions have proposed some necessary parameters (RCS, mobility, etc.) to be included in the sensing channel models, the mentioned sensing-related parameters are currently not described in 3GPP.

Observation 7: Since ISAC is a new feature with low maturity in 3GPP, sufficient time is necessary to adequately study the required enhancements to the 5GS.

Observation 8: Further ISAC sub-topics have already been mentioned as possible candidates to be considered in Sep’24, which potentially increases the workload.

Proposal 1: RAN is requested to agree that discussions on possible Study Items in the WGs (RAN1, RAN3, etc). are to be considered only after the representative use cases, scenarios, etc. have been identified by the RAN-led study objective.

Proposal 2: RAN is requested that the scope of the prioritized use cases is kept limited in the first phase of discussions. It is proposed to prioritize at least one case or two use cases from Scenario 1 (Object Detection and Tracking) as defined by SA1. If required, Scenario 2 (Environment Monitoring) may be included to increase the scope of the study.

Proposal 3: RAN is requested to agree on a unified naming convention for properly identifying the sensing modes involving gNodeBs and UEs in subsequent discussions in 3GPP.

Proposal 4: RAN is requested to agree on a (minimum) set of sensing-related parameters for channel modeling, which may be defined/utilized by downstream WGs.

Proposal 5: RAN is requested to agree that the Rel-19 package for ISAC considers only a Study Item for RAN1 to obtain sufficient time for the study, and to avoid further discussions on ISAC normative work.

# References

[1] RWS-230488, RAN Chair’s Summary of Rel-19 Workshop. RAN Chair.

[2] RP-232617, Moderator's summary for REL-19 RAN1 topic ISAC & Exploring Study in New Spectrum (7-24GHz)

[3] RP-232745, RAN Chair’s Summary for RAN Rel-19 Package: RAN1/2/3-led