3GPP TSG RAN Meeting #102									RP-233315

Edinburgh, Scotland, December 11-15, 2023

Agenda item:	9.1.2.2

Source: 	Huawei, HiSilicon

Title: 	XR enhancements in Rel-19

Document for:	Discussion/Decision

- Introduction

eXtended Reality (XR) is considered to be one of the most important media applications enabled by 5G-Advanced. During the RAN workshop in June 2023, RAN has made the conclusion to consider further enhancement on XR service. In last RAN#101 meeting, companies have given lots of potential directions [1], which are further summarized in [2]. In this contribution, we provide our views on these directions and discuss the scope of XR enhancements in Rel-19.

- Discussion

Enhancements for multi-modal flows

Multi-modal XR is an advancement of XR that integrates multiple modes of perception to create a truly immersive and interactive experience. In RAN#101, many potential directions are proposed including traffic awareness, capacity and power saving.

Enhancements for Multi-Modal flows Most companies expect: - A study or study phase is needed, and the following is included: - Study of traffic case(s), with SA-groups, to establish better understanding for justifications. - Both Intra-UE and inter-UE scenarios. - Enhancements (e.g. scheduling, LCP) for Coordinated / Synchronized transmissions for multi-modal flows. - Enhancements for RAN/AS awareness of inter-dependencies in multi-modal flows. - Enhancements for multiple DRX for multiple flows (Moderator comment: may be applicable to general Some companies seem to expect to include: - Packet Discard enhancements to avoid transmissions that are not useful to the receiver. - Low latency enhancements for aperiodic haptic packets e.g. for dynamic scheduling and/or configured grant, Low Round-trip latency for application reactive transmissions. - Definition of QoE Measurement(s) or KPIs applicable to the multi-modal character, and the possibility to do real-time QoE monitoring. - Coordinated RRC connection handling (incl mobility). - Other QoS enhancements (unspecific).

Traffic-oriented scheduling enhancement

Multi-modal XR provides different kinds of devices/sensors to improve the user immersive experience. Except the conventional audio and video modality, haptic modality is one of the most representative sensations to enable user interaction with a virtual world. In Rel-18, SA has defined the requirement [6] and traffic characteristic [3] [7] of haptic data. In the following, we elaborate the traffic model of haptic data by comparing it to the video traffic.

Figure 1. Traffic model for XR with multi-modality

- Video modality: Generally, video frame arrives periodically every 1/F seconds, where F is the frame rate in FPS of the video, e.g. F=60 FPS, 90FPS as shown in Figure 1. The typical bitrate of XR video given in TR 38.838 [4] are 30Mbps and 45Mbps. The corresponding average frame size is 500K bits and 750K bits, respectively which usually need several physical-layer TBs to deliver.

- Haptic modality: Compared with video, haptic sensors usually have higher sampling frequency. The amount of haptic packets for one sensor that are generated via sampling from the analog signal may be 1K - 4K packets/second [3], as illustrated as black points in the bottom box of Figure 1. According to the compression principle in IEEE 1918.1 [5] that the difference of conveyed haptic value between two compressed packets is larger than a target threshold, only haptic packets denoted by red points in Figure 1 are transmitted. After compression, the rate of haptic packets for one sensor is 100-500 packets/second and the arrival interval between two compressed haptic packets is not regular, which can be regarded to follow some random distribution, e.g., generalized Pareto distribution [7]. As indicated in [5], the size of a haptic packet is related to the degrees of freedom (DoF) that haptic sensor supports and the data size for one DoF is 2-8 Bytes. The maximum bitrate of one haptic sensor is 500 (packets/second)*6 (DoF)*8 (Byte)*8 (bits/Byte)≈200kbps with haptic compression. For each user, typically, there are at most 60 concurrent haptic sensors [7], each of which compresses haptic packets independently. The grey rectangles in Figure 1 represent the aggregated data from multiple haptic sensors. It should be noted that the data volume of each grey rectangle varies due to independent compression among haptic sensors.

Observation 1: For a user, the data volume of haptic packets is irregular for each uplink transmission opportunity.

To provide XR user with timely interaction, it is essential to guarantee low latency during the delivery of XR video. In TR 38.838 [4], the baseline value for the air-interface delay of video frame is set as 10 ms for downlink and 30ms for uplink, while the frame-level reliability requirement is 99%. However, compared with the transmission requirement of video, haptic data has more stringent requirement on both latency and reliability. In TS 22.261 [6], the end-to-end latency for accurately completing haptic operations should be less than 25 ms and the communication delay for haptic modality can be reasonably less than 5 ms, via subtracting the delay of rendering and hardware processing. Besides, the reliability requirement for transmitting compressed haptic packets is also much higher and it should satisfy 99.999% requirement as mentioned in [6].

Observation 2: Video modality and haptic modality have quite distinct transmission requirements.

- Video modality: 99%@10ms

- Haptic modality: 99.999%@5ms

Considering the low-latency requirement of haptic data, pre-scheduling and configured grants may be more suitable for uplink transmission. As mentioned above, data volume of haptic packets varies due to independent encoding among haptic sensors. UL resource may be wasted since the frequency/time resource allocation for both pre-scheduling and configured grant is determined before the actual packet arrival.

For the physical transmission of video, the initial BLER can be set as 10%, which can achieve a good trade-off between system throughput and reliability for XR service. But considering the stringent latency requirement for haptic packets, there may be no retransmission opportunities or only one HARQ retransmission for air interface. To guarantee the reliability requirement of 99.999%, the initial BLER for haptic delivery should be set as low as possible, i.e. lower than that for video. On the other hand, considering that haptic packets arrive much more frequently than video frame, it is unavoidable to transmit video and haptic data in the same slot. RAN should consider some enhancements for better resource efficiency when delivering video and haptic data together.

Proposal 1: RAN to study and specify scheduling enhancements (including dynamic scheduling and configured grants) for multi-modal traffic, by considering distinct reliability requirements of traffic for different modalities (e.g. video and haptic), as well as traffic pattern of multi-modal traffic (e.g. irregular haptic packet size).

Flow association indication

There is a need of flow association information between haptic packets and video frames to be available at RAN, as raised by numerous companies in last meeting. Such association information can be useful for gNB to handle the multi-modal QoS flows together. For example, based on such information RAN may decide to hand over the associated QoS flows together during handover, or to put the associated QoS flows into one DRB to better support synchronization delivery.

For DL transmission, a potential enhancement is to indicate the association among multiple QoS flows, which needs SA coordination. In Rel-18, SA have introduced a signalling element “Multi-Modal Session ID” to indicate that certain traffic flows for which AF requests belong to the same application session. Similarly, RAN may need such an association information of multi-modal or multi-QoS flow to better handle the data packets in the associated QoS flows.

Proposal 2: Study and specify enhancements on interaction between RAN and SA to support the association of XR with multi-modality or multi-QoS flow for DL.

Regarding UL scenario, UE is able to identify which QoS flows requiring coordinated delivery since it knowns application-layer information. Thus, UE can notify the identified association relationship information among UL QoS flows to the gNB for UL scheduling, which is helpful to improve synchronization experience.

Proposal 3: Study and specify the mechanism where UE can notify RAN the association among multi-modal traffic or multi-QoS flow for UL.

Delay aware scheduling

Rel-18 XR supports that the UE reports DSR to the network, to assist the network’s uplink scheduling. Via receiving DSR, the network can be aware of the volume and remaining time of delay-critical data buffered in the logical channels. Delay critical data is defined as data with remaining time less than a threshold in the latest PDCP running CR. Based on the above information, the network can schedule suitable uplink grants to the UE in time, to avoid the delay-critical data from exceeding its delay budget.

However, considering the multi-modal scenario with the haptic traffic and the pose/control traffic coexist in UL, as well as the other multi-flow scenarios such as coexistence of UL video and UL audio, it is doubtful whether the scheduled grants can be perfectly used for the transmission of the delay critical data. For example, as shown in Figure 3, there is delay critical data in LCH 2 which has lower priority than LCH 1. When an UL grant is allocated by the network with the intention to serve the delay critical data in LCH 2, it is highly possible that the uplink grant will be consumed by the data from LCH 1 which has a higher LCH priority. It may result in that the delay critical data finally exceeds its delay budget and becomes useless.

Figure 3. Example of data status

The reason is that LCP mechanism does not consider the remaining time of data stored in the logical channels. Based on the current MAC specification, when an uplink grant arrives, the UE will perform the LCP procedure to generate a MAC PDU. As shown in Figure 4, in the current LCP procedure, the UE will first select the logical channels for the uplink grant based on the transmission information of the grant and the configured LCP mapping restrictions. Then the UE allocates resources to the selected logical channels based on the logical priority. So even if there is delay critical data in the lower priority LCH, it is highly possible to be blocked by the data from the other LCHs with higher priority.

Figure 4.  LCP procedure

Based on the above analyses, it can be found that even though the network schedules the grants based on the knowledge of remaining time, it is still hard to guarantee the delay budget because the LCP procedure does not take the remaining time of data into account.

Proposal 4: RAN to support uplink delay aware scheduling to reduce packets exceeding delay budget, e.g., LCP enhancements.

Aspects with SA initiated work

Dynamic QoS adaptation

The radio condition of the mobile network is time-varying, which leads to inability to provide stable latency for higher data rate transmission, especially for a user at the cell edge. How to ensure network latency certainty under poor or fluctuating radio conditions is a key problem.

In the latest SA2’s SID for Rel-19 XR, WT#4 is to study how the 5GS can expose the suitable QoS profile which can be guaranteed by the network to the application layer in a fast manner. Then the application can react to do rate adjustment to adapt to the radio condition. In the SID, note 5 clarifies the coordination with RAN WG is needed.

Copied from SA Rel-19 SID [SP-231198] WT#4 Network exposure: Study whether and how XR related network capability/information (e.g. if the QoS profile requested by AF cannot be met, network can indicate the alternative QoS profile) can be exposed towards the application layer. NOTE 5: Alignment and coordination with RAN work will be needed for the study.

Thus, in RAN Rel-19 XR enhancement, RAN should coordinate with SA closely to support the SA2’s working task, i.e. QoS profile exposure to application layer for dynamic QoS adaptation.

Proposal 5: RAN to coordinate with SA2 on QoS profile exposure to application layer for dynamic QoS adaptation.

QoS handling for dynamic traffic characteristics

The traffic pattern of an XR session could change dynamically based on the usage/applications. For example, the size of media frames can be varied due to scene changes that may occur at times such as initial start-up or due to user drags on a progress bar. SA2 SID agreed to have a WT 2.2 as below:

WT#2.2 Study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by the application in the DN.

In the corresponding Key issues agreed in SA2 #160 meeting, it was clarified RAN coordination is required on this aspect.

Proposal 6: RAN to coordinate with SA2 on QoS handling enhancement considering dynamic change of traffic characteristics.

PDU Set discard enhancement based on FEC awareness

In SA2’s SID for Rel-19 XR, WT#1 says SA2 will study enhanced PDU set related information including FEC, which has RAN dependency. RAN shall perform study on RAN mechanisms and impacts to support FEC awareness at RAN aspect.

Copied from SA Rel-19 SID [SP-231198] WT#1 Enhancement for PDU Set based QoS handling. WT#1.1 Study whether and how to enhance PDU Set related (e.g. new standardized 5QI, enhancements to Alternative QoS profiles, FEC) and PDU Set information (including Control Plane and/or User plane information provided by the AF/AS) and the corresponding PDU Set QoS handling enhancement. NOTE 1: This will require close coordination between SA4 and SA2.

In the SA4 LS [S4-220505] sent to SA2 during Rel-18 discussion, it has the following description of Application layer FEC.

Copied from SA4 LS [S4-220505] Application Layer FEC is for example used in multicast/broadcast (for details refer to 3GPP TS 26.346) or in conversational applications (see TS 26.114). Typically, for an applicational layer, source block packets from 0 to K-1 identify the source symbols of a source block in sequential order, where K is the number of source symbols in the source block. Encoding Symbol IDs K onwards identify repair symbols generated from the source symbols using an FEC encoder, e,g, Raptor. Typically, N >= K packets are sent, carrying an FEC source or repair symbols. Typically, the decoder requires only any K or only a small amount more than K packet of the N packets to recover the source packets. Based on this, the definition of a PDU Set applies to all packets of a source block (A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g., a frame or video slice for XRM Services), …) and any K packets are sufficient to recover, i.e. all packets are of same importance (which are of same importance requirement at application layer). As only K out N are required, the first definition does not hold (All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information.), but more the second (In some cases, the application layer can still recover parts of the information unit, when some PDUs are missing). Actually, note that in this example of an Application Layer FEC not only parts of the information unit but the full information unit can be recovered.

Based on the description, it can be found if the application layer FEC is applied, some PDUs within a PDU set are redundant. Even though some PDUs are lost, the application layer can still recover the whole PDU set. Then if the FEC redundancy ratio can be known by the RAN, resource efficiency could be improved by using FEC information. For example, the transmitter can choose not to transmit the remaining PDUs within a PDU set in case the successfully delivered PDUs of the PDU set are sufficient to decode the information carried by the PDU set.

In another SA4 LS [S4aV220921], SA4 clarifies it is not encouraged to apply proactive PDU discard based on FEC information.

Copied from SA4 LS [S4aV220921] - In general, SA4 discourages to apply “active” packet dropping to FEC protected information as it may negatively impact receiver operations (e.g., confuse the receiver (for example asking for even more FEC packets), result in additional delay, lead to wrong measurement of the network capacity, or harm fast decoding). The 5G System should provide the requested/expected QoS and not rely on application layer FEC.

Actually, in non-congestion case, there is no need to discard PDUs proactively. Besides, proactive discard will mislead the receiver to think the link is not good and increase the FEC redundancy, which would harm the service experience.

However, in congestion case, proactive discard is useful to alleviate the congestion. Rel-18 XR already supports to proactively discard PDU set with low PSI in congestion. It is worth noting the assumption in Rel-18 is that if one PDU is lost, the whole PDU set cannot be decoded. With FEC awareness at AS, the RAN side can improve the discard mechanism by achieving a well trade-off between the congestion alleviation and service experience. For example, if the congestion level is low, the gNB or the UE (depending on whether the transmission is in DL or UL) can discard only the redundant PDUs within a PDU set to alleviate the congestion. Such method will not affect the PDU set decoding at the application layer, and can guarantee the XR service’s experience. While in case the congestion level is high, the gNB or the UE can decide to discard the whole PDU set.

Based on the above analysis, we suggest RAN to study PDU set discard enhancement in congestion based on FEC awareness at RAN. How RAN can be aware of the FEC information of XR application depends on the SA2’s solution.

Proposal 7: RAN to coordinate with SA2/SA4 on PDU set discard enhancement in congestion based on FEC awareness at RAN.

Measurement gaps / scheduling restrictions

According to the discussion of the last meeting, most companies support to reduce the impact to capacity related to the scheduling restrictions due to the inter-frequency RRM measurements with measurement gaps and FR2 intra-frequency measurements w/o measurement gaps.

Agreeable: Measurement Gaps / Scheduling restrictions scope: Specify enhancements for reducing the impact to capacity and impact to individual UEs with respect to scheduling restrictions for FR1 and FR2 inter-frequency RRM measurements with measurement gaps and FR2 intra-frequency measurements w/o measurement gaps. [RAN1, RAN2, RAN4].

To perform inter-frequency measurements, UE will switch the receiver towards the target non-serving cell and return back after the measurements. In the current spec, gaps can be configured to guarantee that UE has enough time to do inter-frequency measurements in FR1/FR2. During the gap duration, the UE will not transmit on UL-SCH except for Msg3 or the MSGA payload/not receive on DL-SCH. This scheduling restrictions are shown as follows:

Copied from clause 9.1.2 in TS 38.133 V17.11.0 During the per-UE measurement gaps the UE: - is not required to conduct reception/transmission from/to the corresponding E-UTRAN PCell, E-UTRAN SCell(s) and NR serving cells for E-UTRA-NR dual connectivity except the reception of signals used for RRM measurement(s) and the signals used for random access procedure according to TS38.321 [7]. - is not required to conduct reception/transmission from/to the corresponding NR serving cells for SA (with single carrier or CA configured) except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7]. - is not required to conduct reception/transmission from/to the corresponding PCell, SCell(s) and E-UTRAN serving cells for NR-E-UTRA dual connectivity except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7]. - is not required to conduct reception/transmission from/to the corresponding NR serving cells for NR-DC except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7].

Hence, for delay-sensitive traffic like XR, the scheduling restrictions may challenge the capacity. An example is shown in Figure 6, for the XR traffic with 60FPS, we assumed that the measurement gap repetition period (MGRP) is 40ms and the measurement gap length (MGL) is 6ms. It can be observed that two XR frames will be interrupted within each measurement gap period.

Figure 6. Impacts of scheduling restrictions on XR traffic due to measurement gap

For intra-frequency measurements w/o measurement gaps in FR2, there also exists scheduling restrictions due to the beam scanning. As specified in clause 9.2.5.3.3 of TS 38.133, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured [9].

However, based on the evaluation results of Rel-17 [4], the UE capacity in FR2 is not a concern issue on account of the broad bandwidth. Hence, enhancements on measurement gaps/scheduling restrictions in FR1 should be prioritized and specified to improve XR capacity. If the measurement gaps/scheduling restrictions in FR2 has to be enhanced, a unified solution should be considered.

Proposal 8: Enhancements on measurement gaps/scheduling restrictions in FR1 with measurement gap should be prioritized.

Others

Further UTO-UCI extension

In the last RAN meeting, extending UTO-UCI to multiple CG configurations was proposed for Rel-19 XR topic.

Almost agreeable: UTO-UCI: support multiple CG config

This enhancement has been discussed in R18 RAN1, and there was no consensus to support it since the benefit was unclear.

To match the non-integer periodicity of XR traffic, the UE may need to be configured with multiple CG configurations. For example, let us consider that the packet arrival rate is 60fps and the packet periodicity is ~16.67ms. Hence, three CG configurations with different start offsets can be configured with CG periodicity set as 50ms, as shown in Figure 7. Aiming to address low latency and variable frame size issue in Rel-18, there are 3 configured CG PUSCHs within a CG period of each CG configuration to transmit XR frames. Extending UTO-UCI to indicate CG PUSCH occasions across multiple CG configurations means that the UTO-UCI transmitted in CG 1 not only can indicate the subsequent unused CG PUSCH occasions of CG 1 but also the subsequent CG PUSCH occasions of CG 2 and CG 3. However, when UTO-UCI is transmitted by the PUSCH of CG 1, the amount of data to be transmitted in CG#2 is not known since frame #2 has not arrived yet. Thus, there seems to be no benefit to extend UTO-UCI to indicate CG PUSCH occasions across multiple CG configurations.

Figure 7. Extend UTO-UCI to indicate multiple CG configurations

In addition, it was proposed that extending UTO-UCI to multiple CG configurations can save the bits overhead when there exists CG PUSCHs overlapped in multiple CG configurations. However, it is a common understanding that the consumption of UTO-UCI bits is not a concern in the discussion of RAN1 R18 WI. This is why the UTO-UCI is represented by a bitmap in RAN1 R18 XR WI [10].

Proposal 9: Not support to extend UTO-UCI across multiple CG configurations.

Link adaptation enhancement

In last RAN meeting, the following link adaptation techniques are proposed for Rel-19 XR topic.

CQI/CSI enhancement (7) - to support CBG-retransmission, reducing overhead (capacity) - PDSCH reception based CQI: Fast Link adaptation (latency) - CSI reporting adapted to XR Traffic Pattern, on HARQ level, Soft-HARQ: Fast Link adaptation (latency)

The purpose of some enhancements is to choose better MCS to fit to the practical channel variation, including using PDSCH decoding quality as CQI reporting, adding more information in HARQ feedback to indicate the more accurate MCS, and considering CBG-level OLLA controlling. However, these techniques were also discussed in Rel-18 XR topic and were down-prioritized as they are not XR-specific issues [8].

Copied from Chairman Notes in #110-bis-e Conclusion - Study of soft HARQ-ACK and Delta MCS in RAN1 XR SI for improving XR capacity is down-priortized. Conclusion - Study on enhanced CQI based on CBG transmission, and study on enhanced CQI based on DMRS for improving XR capacity are down-priortized in RAN1 XR SI.

Proposal 10: Not support link adaptation enhancement in Rel-19 XR.

PDCP Discard enhancements for inter-PDU set dependencies

As captured in the moderator’s summary [1], there is little support for PDCP discard enhancements based on inter-PDU set dependencies.

PDCP Discard enhancements for PDU set discard enhancements for inter-PDU Set dependencies (4) Can consider, if SA2 agrees.

In Rel-18 XR, SA2 studied inter-PDU set dependency, but it was excluded in WI. As clarified in TR 23.700-60 [11], it is not easy to identify the dependency between different frames. For example, in the H.264, the P frame depends on I frame, the B frame depends on I frame, P frame and possible other B frames. If there are multiple successive (e.g. over 3) B frames in the GOP, it is very hard to identify the dependency between these B frames. Thus, for real XR applications, the actual inter-PDU set dependency might be quite complicated. One PDU set can be encoded by referring to multiple reference PDU sets. Whether the application can provide the dependency information to the 5GS, or whether the UPF/UE can identify the dependencies of different PDU sets by implementation for DL/UL traffic are both doubtful.

On the other hand, even though the 5GS can obtain the inter-PDU set dependencies, whether the RAN node can perform effectively PDCP discard based on the dependencies is still questionable. If the inter-PDU set dependency is too complicated, e.g. one PDU set may rely on multiple previous PDU sets, the RAN node or the UE needs to remember the delivery status of multiple previous PDU sets. It is worth noting that each PDU set may comprise several PDUs. Then the RAN node or the UE needs to judge whether a newly arrived PDU set can be discarded based on the transmission status of previous PDU sets and their dependencies. This will incur considerable memory cost and processing overhead for gNB and UE. Besides, the recording accuracy of the transmission status also matters. For UM radio bearer, the transmission side is not aware of the delivery status of PDUs. This will further increase the complexity of PDCP discard solutions.

Based on the above considerations, we suggest to not support PDCP discard enhancements based on inter-PDU set dependencies in Rel-19.

Proposal 11: Not support PDCP discard enhancements based on inter-PDU set dependencies.

RLC AM enhancements

XR traffic and the future immersive Metaverse communication requires high reliability with limited delay budget. As mentioned in TR 22.856 [7], for DL control traffic in Metaverse based Tele-Operated Driving use cases, the traffic demands 99.999% reliability and 20ms latency.

Currently, RLC UM is designed for low latency traffic without high reliability guarantee. For URLLC/IIoT traffic, RLC UM is commonly used together with mechanisms to improve reliability, e.g. PDCP duplication, which is not resource friendly. RLC AM is designed for traffic requiring high reliability but which is latency tolerant at the same time. When it comes to XR/Metaverse traffic, it usually requires certain reliability within strict PDB.

For RLC UM, the reliability will mainly rely on HARQ retransmission, which is not sufficient in some cases, e.g. in case that an improper initial MCS is selected. Besides, for an FR2 carrier, channel fading or blockage may easily lead to severe decline in transmission reliability for a period of time. Increasing the HARQ retransmission times in these cases is not helpful for improving reliability. Furthermore, mechanisms like PDCP duplication is not preferable for XR/Metaverse because of the impacts on resource efficiency (the resources needed would be doubled by using PDCP duplication).

For the reasons mentioned above, we believe that RLC AM is useful for XR/Metaverse traffic to ensure high reliability.

Observation 4: RLC AM is useful for XR/Metaverse traffic to ensure high reliability requirement.

RLC AM is typically used for services which require no packet loss but without stringent latency requirement, such as Web browsing services. One issue when applying RLC AM for XR traffic is that the ARQ never gives up the retransmission until the transmission succeeds or RLF is triggered when it reaches the maximum RLC retransmissions, even though the packet transmission has exceeded the PDB. The timed out retransmissions are useless and incur unnecessary resource waste, and may even lead to RLF which further cause XR service interruption.

To address the above issue, we think RLC AM enhancement to avoid the unnecessary out-of-PDB RLC retransmission is a desirable direction for Rel-19 XR enhancements.

As captured in the moderator’s summary [1], there are visible benefits of this enhancement for XR, while the moderator was concerned about the impact on specifications and believed it should rather be a common UP discussion.

RLC AM enh - better resource efficiency by avoiding unnecessary retransmissions (1) [20] Moderator Q: is this RLC-discard? - RLC re-transmission enhancements allowing operation of RLC Acknowledged Mode (AM) with small packet delay budget (1) [2] Moderator Comment: Note that a high ambition level is not reasonable for 5G Adv - Moderator: The benefits might be significant for XR, but Very High Risk that it doesn’t work to do a limited effort for RLC-AM. Inclined to suggest to not include in Rel-19 but postpone for later. Not discussed here, it is part of UP discussion

First of all, we think the shortcomings of RLC AM is not negligible for typical XR services. Potential enhancements can be evaluated based on the XR’s traffic characteristics. It is suitable to study the direction in XR topic. Besides, whether the standard efforts can be limited or not needs further study. It is hard to say there must been considerable specification changes for the enhancement. Hence, we suggest RAN to study RLC AM enhancement for better resource efficiency by avoiding unnecessary retransmissions in Rel-19.

Proposal 12: RAN to study RLC AM enhancement for better resource efficiency by avoiding unnecessary retransmissions.

- Conclusion

In this paper, we provide our views on …

Proposal 1: RAN to study and specify scheduling enhancements (including dynamic scheduling and configured grants) for multi-modal traffic, by considering distinct reliability requirements of traffic for different modalities (e.g. video and haptic), as well as traffic pattern of multi-modal traffic (e.g. irregular haptic packet size).

Proposal 2: Study and specify enhancements on interaction between RAN and SA to support the association of XR with multi-modality or multi-QoS flow for DL.

Proposal 3: Study and specify the mechanism where UE can notify RAN the association among multi-modal traffic or multi-QoS flow for UL.

Proposal 4: RAN to support uplink delay aware scheduling to reduce packets exceeding delay budget, e.g., LCP enhancements.

Proposal 5: RAN to coordinate with SA2 on QoS profile exposure to application layer for dynamic QoS adaptation.

Proposal 6: RAN to coordinate with SA2 on QoS handling enhancement considering dynamic change of traffic characteristics.

Proposal 7: RAN to coordinate with SA2/SA4 on PDU set discard enhancement in congestion based on FEC awareness at RAN.

Proposal 8: Enhancements on measurement gaps/scheduling restrictions in FR1 with measurement gap should be prioritized.

Proposal 9: Not support to extend UTO-UCI across multiple CG configurations.

Proposal 10: Not support link adaptation enhancement in Rel-19 XR.

Proposal 11: Not support PDCP discard enhancements based on inter-PDU set dependencies.

Proposal 12: RAN to study RLC AM enhancement for better resource efficiency by avoiding unnecessary retransmissions.

- WI Scope

Base on the potential objective summarized by RAN chair in [2] and our above proposals, we give our views on the XR WI scope as follows.

- Study and if justified, specify aspects related to multi-modality(intra-UE)/multi-QoS flow (intra-UE), including

  - scheduling enhancements (including dynamic scheduling and configured grants) for multi-modal traffic for better transmission efficiency, LCP enhancement (for delay aware scheduling)

  - enhancements to ensure association between different XR modal flows (for DL coordination with SA2 is needed)

- Coordinate with SA2 on SA2 initiated work, including:

  - QoS profile exposure to application layer for dynamic QoS adaptation

  - QoS handling enhancement considering dynamic change of traffic characteristics.

  - PDU set discard enhancement in congestion based on FEC awareness at RAN

- Specify enhancements for reducing the impact to capacity and impact to individual UEs with respect to scheduling restrictions for FR1 inter-frequency RRM measurements with measurement gaps.

.

- References

RP-232619, Moderator's summary for Rel-19 RAN2 topic Enhancements for XR, RAN2 Chair, 2023

RP-232745, “Summary for RAN Rel-19 Package RAN123-led”, 2023

3GPP TR 26.847 V18.2.0, “Study on supporting tactile and multi-modality communication services,” Mar, 2022.

3GPP TR 38.838 V17.0.0, “Study on XR (Extended Reality) evaluations for NR”, Jan, 2022.

O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

3GPP TR 22.261 V19.3.0, “Service requirements for the 5G system”, June, 2023.

3GPP TR 22.856 V2.0.0, “Feasibility Study on Localized Mobile Metaverse Services”, June, 2023.

Chairman notes, RAN1#110-bis-e, Oct 14 - 19, 2022.

3GPP TS 38.133 V17.11.0, Oct, 06, 2023.

3GPP TS 38.213 V18.0.0, Sep, 2023.

3GPP TR 23.700 V17.0.0, Dec,2020

R2-2312192, Introduction of XR Enhancements, LG Electronics Inc. (Rapporteur), Nov, 2023.